A Vision for a Future AIS for e-Navigation

1.	AIS for e-Navigation	1

2.	AIS today	2

3.	Increasing use of AIS	2

3.1.	Number of ships	2

3.2.	AIS base stations	2

3.3.	Airborne AIS	2

3.4.	AIS AtoN, real or virtual, and AtoN monitoring	2

3.5.	AIS SART	2

3.6.	Binary messages	3

3.7.	New AIS messages (23, 24, 25, and 26)	3

3.8.	GNSS differential corrections	3

3.9.	FATDMA usage	3

4.	Additional future use of AIS	3

4.1.	Increased use of AIS shore to ship data transmissions	3

4.2.	Increased use of ship-shore data transfer	4

4.3.	Increased use of ship-ship data transmissions	4

4.4.	Repeater, limited base station	4

4.5.	Future messaging	4

4.6.	Satellite detection of AIS	5

4.7.	Future GMDSS platform	5

4.8.	PLB	5

4.9.	Ranging	5

4.10.	Coordinated channel management for future e-Navigation VHF digital data communications	5

5.	Strategy for future AIS	5

5.1.	General thoughts	5

5.2.	AIS Channel plan	6

5.3.	Airborne AIS	8

5.4.	Legacy strategy	8

6.	Benefits for users	8

7.	Recommendation to IALA Council	8

# AIS for e-Navigation

- AIS is a proven technology and should play a significant part in data communications for e-Navigation

  - By design AIS is not an ideal candidate for high speed and/or high volume data communications

  - High speed digital data communications for e-Navigation is better handled by using multiple 25KHz channels

  - The IMO ITU joint experts group has recognised that more than 200 Kbps will be necessary for e-Navigation

- This document proposes a future AIS to handle some data communications for e-Navigation.

# AIS today

- AIS is technically defined by ITU-R M.1371, mandatory for SOLAS vessels and for other vessels on a regional basis, and used voluntarily

- AIS has proved to be a powerful tool for various applications in the field of navigation, however these applications have not yet fully been exploited

- Appendix 15 RR (WRC-07) defines the frequencies used for GMDSS, and it includes the frequencies AIS1 and AIS2 used by AIS SART stations.

# Increasing use of AIS

### Number of ships

- Full implementation of SOLAS

- Increased use of mandatory AIS on non-SOLAS vessels

  - USA (USA commercial vessels)

  - Europe (EU Directive for fishing vessels requiring the use of Class-A AIS )

  - Europe (EU Directive for inland vessels requires the use of Inland AIS (Class-A derivative)

  - [Korea]

  - [India]

  - Mexico

  - Australia

- Increased voluntary use of AIS

  - Both Class-A and Class-B units being used on pleasure vessels

- Class-B  increasing use

  - Class-B (CSTDMA) visibility reduced due to polite behaviour

### AIS Base stations

- Coastal and inland AIS infrastructure continues to grow

  - Driven by safety, security, and environmental protection considerations

  - EU Directive 59 requires all member countries to maintain coastal AIS coverage

### Airborne AIS

- More aircraft are being fitted with AIS stations

### AIS AtoN, real or virtual, and AtoN monitoring

- Message 21 (AtoN Report) real and virtual is being used or approved by competent authorities

- A separate message for AtoN monitoring (often Message 6) is being used in most cases where an AIS AtoN station is deployed

- In addition, AIS AtoN stations are being deployed by commercial organisations to mark offshore platforms, wind farms, etc.

### AIS SART

- IMO carriage option for the AIS SART starts in January 2010

  - Broadcasts eight times per minute using Messages 1 and 14

### Binary messages

##### International

- IMO SN/Circ. 236 international messages trial is now complete

- Recommendations of the IMO CG on binary messages

  - Recommended retention of most of these international binary messages and the addition of new messages

    - E.g. a new meteorological-hydrological message with a dynamic length of 2 to 5 slots

    - E.g. an area message for navigational warnings

##### Regional

- Extensive use of safety-related binary messages for AtoN monitoring

- Binary messages being used in certain waterways

  - E.g. St Lawrence seaway

- IALA is maintaining a registry of these regional binary messages

##### Mandatory binary messaging in certain regions

- Additional binary messages are being used for inland AIS in Europe

  - Ship stations are type approved and in use

### New AIS messages (23, 24, 25, and 26)

- Messages 23, 24, 25, and 26 were added in recent editions of ITU-R.M1371

### GNSS differential corrections

- Use of Message 17 for GNSS correction data dissemination is increasing

  - E.g. already being broadcast in Gulf of Finland, Tokyo Bay

  - E.g. Germany, Netherlands, England are planning

### FATDMA usage

- Competent authorities are increasingly using FATDMA slot allocations for various uses

  - AIS AtoN

  - Base Station slot reservation broadcasts (Message 20)

  - Base Station regular broadcasts (Message 17)

  - Reserved slots for ship replies

  - Safety text messages

  - Meteorological and hydrological messages (Message 8)

- Indication that VDL will become more crowded and competent authorities resort to FATDMA reservations to ensure that their services are protected

# Additional future use of AIS

### Increased use of AIS shore to ship data transmissions

- In a future e-Navigation scenario, the transmission of AIS data from shore to ship will increase significantly with the advent of mandatory ECDIS

  - Examples

    - Ship off track warning

    - Territorial water advice

    - Environmental area warning

    - Navigational hazards

    - Weather warnings

    - Military test ranges and submarine exercises

- Some of the present voice communications between shore and ship will move to AIS messaging

  - E.g. ship reporting

  - Route information

### Increased use of ship-shore data transfer

- Ship reporting by AIS

- Requesting data

  - Weather data

  - Docking data

  - Route information

### Increased use of ship-ship data transmissions

- Reduced use of voice communications in favour of AIS messages

### Repeater, [Land Based station (formally called Limited Base station) Class-C?]

- There will be more owners of land based stations, some of these being private

- Development of limited base stations will be driven by the need for private use

  - Private ports

  - Bridges and locks

  - Canals

  - Offshore commercial facilities

- Repeaters will be used to extend coverage of shore AIS base station networks, and ship-ship AIS communications, and AIS AtoN range

  - Requiring double the VDL slot count to transfer data

  - And are deployed to extend base station coverage when the usual base station to base station data links are unavailable

### Future messaging

- Will be required as the suite of 1371 messages increases

  - Originally 22 message types, now 27

- Improved set of messages:

  - Replacement for some old messages, when they have proved to be not supporting proper data modelling (e.g. static ship data)

  - Reduction of tailored messages to essentially needed messages (“statements of the AIS device of its own”), while increasing the proportion of the transparent data container messaging (“passing information of other components/entities”): properly designed application specific message realm (e.g. also “safety related texts” would be some application specific messages)

- New flexibility for applications

### Satellite detection of AIS

- Research continues on enhanced methods of detecting AIS signal, and hence monitoring shipping, by satellite

- Satellite detection of AIS messages has been requested by administrations.

- Separate frequencies for satellite detection of AIS should be selected from within Appendix 18 because the tuning range of the shipborne AIS Class A is limited to these frequencies. Also, with respect to possible additional AIS frequencies, Report ITU-R M.2084 indicated that the interference environment resulting from the existing services in those bands must be taken into account in determining the feasibility of accommodating satellite AIS in any given band or channel, due to the large satellite antenna footprint that overlaps both land and sea. Separate operating frequencies in addition to AIS 1 and AIS 2 are needed that are not subject to terrestrial use.

- Frequencies should be considered only from Appendix 18 due to the limited tuning range of the ship-borne AIS.

- Appendix 18 contains only 3 frequencies (channels 16, 75 and 76) that are exclusively dedicated to maritime use (channels 75 and 76 are proposed to be shared with this service). This proposal meets the intent of footnote n) to Appendix 18 for interference mitigation.

### Future GMDSS platform

Is there any operational functionality in present VHF-DSC spec (M.493), which could not be fulfilled by amended AIS? (Hence, leading to AIS (V.2), see below, and replacing the present VHF DSC eventually)

- COMSAR has been tasked to modify the performance standard for the 406 MHz EPIRB to include use of AIS

- Use of AIS as a GMDSS alerting device

- Additional use of AIS as a part of GMDSS should be brought to the attention of IMO

### PLB (Personal Locating Beacon)

- There are initiatives to develop a PLB based on the AIS SART technology

### Use of AIS for PNT (Position, Navigation and Timing) and Ranging

- Improved time transfer capabilities shore to ship by using high precision time bases.

- The e-Navigation Committee envisages a “Ranging” mode for AIS

  - Where the AIS messages are used for position determination

  - By ship and shore

### Coordinated channel management for future e-Navigation VHF digital data communications

- Designation of additional AIS frequencies may enhance the coordination of regional channel management

# Strategy for future AIS

### General thoughts

- AIS is a proven maritime data system, with ships equipped and shore infrastructure established, the full potential of AIS is yet to be achieved

- AIS is the quickest path for handling the increasing data exchange needs described above

### AIS Channel plan

- Safety of Navigation purposes

  - The AIS-1 and AIS-2 are internationally allocated with sharing basis.  These frequencies should be retained and protected for safety of navigation purposes.

- Data communication purposes

  - In considering the future AIS system for terrestrial (non satellite) data communication purposes, additional two frequencies will be needed.  These frequencies should be selected within the Appendix 18.  The Appendix 18 covers 156-162.025MHz; however the current channelling arrangements are only a limited parts.  As shown in the figure below, the blue bands (sharing basis) and red bands (maritime exclusive) are channelized and the yellow bands (sharing basis) are not channelized.  The yellow bands may be available for maritime mobile service and candidate frequencies may be selected with the sharing basis.  The data communications are in the terrestrial service, the frequencies can be shared.

- Satellite detection of AIS and future GMDSS system

  - Since the satellites have very wide footprint, the frequencies for satellite detection of AIS should be exclusive for maritime mobile service.  The preliminary draft report ITU-R M.[SAT-AIS] proposes to use CH75 and CH76.  These channels are guard band channels of CH16 and are only maritime dedicated channels except CH16 and CH70.

  - In the future GMDSS system, the AIS technologies should be the basis.  The distress alerting, urgency and safety communications should be by both terrestrial and satellite communications; therefore the maritime exclusive frequencies are needed. The satellite detection of AIS is one way system (from earth to satellite); however in the GMDSS, the acknowledgement (from satellite to earth) is essential as the two way system.  The CH75 and CH76 will be used for this purpose.

- Channel management purposes

  - In order to fully use available VHF spectrum there must be a worldwide channel dedicated to manage and coordinate usage of the VHF Data Link.

  - TDMA or other protocols may share Channel 70 with minor changes to Appendix 18, footnote j) according to ITU-R M.822-1.

- Therefore, in order to meet the data communications needs set out above, IALA envisages that the present AIS system will need to develop into a future AIS

| AIS frequency pair | AIS-1 & AIS-2 | AIS-3 & AIS-4 Maritime Dedicated Channels | AIS-5 & AIS-6 | Channel 70 |
| --- | --- | --- | --- | --- |
| Purpose | - Safety of navigation - Maritime and inland distress and safety communications | - Space detection of AIS - Future GMDSS | - Marine safety information - General purpose information communication | - Management of VHF VDL in harmony with current ITU-R M. 822-1 |
| Existing/New frequency allocation | - Existing | - New | - New | - Existing |
| AIS Message types | - Present ITU-R.M1371-3 | - Message 27 for space detection of AIS - Other messages for support of future GMDSS | - S/N Circ 236 international AIS binary messages - Regional AIS binary messages | - TDMA system management messages e.g Message 20, Message 22, - Other messages for support of coordinating channel sharing |
| Typical applications | - Ship to ship collision avoidance - VTS tools - Tracking of ships - Locating in GMDSS | - Detection of vessels by coastal states beyond range of coastal AIS base stations - Two way distress communications | - Area warnings and advice - Meteorological and hydrological data - Traffic management - Channel management of AIS channels and future VHF digital data channels - Ship-shore data exchange | - Channel switching - FATDMA allocation, - Assignment |
| Proposed channels in Appendix 18 | - As now | - Channel 75 and 76 | - World-wide dedicated channels are preferred, but if this is not possible, then the non channelised band may be used for the candidate channels | - As now but requires Appendix 18, Footnote j), and ITU-R M. 822 modification |
| Comments |  | - IMO COMSAR has been tasked to study future GMDSS technology and timetable, and what changes in SOLAS are required. Two COMSAR sessions from 2010-03 are allocated (2 years). - The USA and other countries have proposed to IMO and ITU that a third AIS channel be designated to enhance the reception of AIS signals by satellite. |  | Expanded use of channel 70 to improve efficiency |

### Airborne AIS

- ITU Radio Regulations Article 51 Section III sets limits on aircraft VHF usage

  - This will be required to allow aircraft effective use for AIS message transmission in SAR cases

- It may also require change in the future to permit and/or prohibit certain AIS message transmission

### Legacy strategy

- Future advances in AIS will require updates to legacy systems

- Legacy functionality will be maintained

- Channel management, using Channel 70, already available internationally, should be maintained until it is replaced by another internationally agreed method of channel management

# Benefits of future AIS for the users

- [amend paragraph with good selling point of benefits!]

- Use of AIS for maritime information dissemination will mean that Navtex will not be needed for ships navigating in Area A1

- Existing shipboard AIS installations infrastructure could be upgraded

# Recommendation to IALA Council

- AIS has shown a convergence of navigation and communication. The challenges of this convergence may warrant  that IMO consider convergence of NAV and COMSAR into an e-Nav focused sub-committee

- Council should send a liaison statement asking IMO and ITU to consider the strategy for future AIS and its inclusion in GMDSS

- Member states to support worldwide allocation of  frequencies for future AIS at the WRC11 agenda item 1.10

- Work with ITU to amend the Radio Regulations to permit AIS airborne SAR operation above 300 metres

- Ensure that the e-Navigation communications plan includes a dedicated channel for future AIS channel management

# ANNEX A Family of AIS stations

Fixed AIS stations

AIS station Category 1 (Base stations)

Dependent

Independent

AIS station Category 2 (AtoN stations)

Type 1

Type 2

Type 3

AIS station Category 3 (Limited Base station)

AIS station Category 4 (Repeaters)

Mobile AIS stations

AIS station Class-A

SOLAS

Inland AIS

AIS station Class-B

CS

SO

AIS station Class-C

SART

[EPIRB]

[PLB (Personal Locating Beacon)]

AIS station Class-D (Airborne)

Airborne