Comments on draft IALA Guideline No. ####  On A minimum comprehensive mix of AtoN in fairways including dredged channels and canals Edition 1 [May 2009]

Date 2009-09-04

Michael Card, Zeni Lite Buoy Co Ltd

- Extract from draft guideline

6.5 AIS in aids to navigation

AIS as applied to aids to navigation (AtoN), improves and enhances services provided to mariners as far as their vessels are equipped with AIS. The AIS applied to an AtoN can provide a better:

- Integrity of the aid, because it is able to show Off-Position and Off-Function Messages to the mariner; and

- Accuracy, because it shows the accurate position on the electronic chart or on a modern radar screen.

So, AIS applied to AtoN should be considered for floating and fixed AtoN if they are:

- Major aids;

- On positions with a high risk of damage or impairment by ice, current, collision.

More Details can be found in IALA Recommendation A-126 “on the use of AIS in Marine Aids to Navigation Services”, Edition 1.3, June 2007 and the Guideline xxx”

- Comments on draft section 6.5 for consideration by the e-Navigation Committee

An AIS AtoN also

- Provides an unambiguous indication of the identity of the AtoN, by carrying the AtoN name within the AIS message data

- Operates day and night, and also in conditions of impaired visibility when visual AtoN signals may not be seen

- Will generally have a greater range than daytime visual signals and visual signals from floating AtoN

An AIS AtoN can also be used to broadcast meteorological and hydrological data in real time, and this data, along with the AIS AtoN data, can be displayed on the bridge of a ship within an electronic chart or radar display.

So, AIS applied to AtoN should also be considered for fixed and floating AtoN if they are

- In an area where clear indication of the identity of certain AtoN is essential

- In a group of AtoN, where it is advantageous to clearly identify certain more-important AtoN, for example the buoys or beacons at a turning point in a channel

- In an area where visibility may be poor at times

- Where a reliable signal range of 10 miles or more is needed, day and night

- Recommendation

I recommend that the e-Navigation Committee considers suggesting to ANM that an expansion of section 6.5 of the draft Guideline would be beneficial.