Industrial Wireless Networks

09 December 2008

Industrial Wireless Networks

Wireless networks are used industrially as a way to avoid the cost of installing and maintaining cabling. Most installed networks are of a star topology, and the network configuration and data paths are fixed during operation. Networks standards include:

IEEE 802.11 (WiFi)

IEEE 802.15.4 (ZigBee)

IEEE 802.16 (WiMax)

The following web site is an excellent resource for information about these types of networks:

http://wireless.industrial-networking.com/

Two of the biggest problems when using star topology are lack of redundant data paths and difficulty in placing hubs effectively to match environmental conditions.

Mesh Networks

Mesh topologies were developed to address the shortcomings of star topology networks. The mesh absorbs environmental problems by providing multiple data paths. Mesh networks that are self-forming or self-healing can also react to transient environmental problems or network node failures by rerouting data through other nodes. The following article gives more background on mesh networks.

http://www.commsdesign.com/showArticle.jhtml?articleID=18900233

Mesh network equipment use one of the following standards:

IEEE 802.11s (WiFi Mesh - unreleased)

IEEE 802.15.4 (ZigBee)

Proprietary

802.11s is a standard that has not yet been released. It will go into voting in early 2009. However, many vendors have released products based on the early work of 802.11s.

Both WiFi Mesh and ZigBee are generally thought of in terms of local or short distance radio networks. However, there is no technological reason ranges cannot be extended using these protocols with different media. Certainly, we can expect WiMax to have a mesh variant one day.

Proprietary Mesh Network Protocols

There are many protocols available. Some are bundled into radios and can only be used with like radios. These will not be described.

Most of the proprietary mesh networks were developed because no standard existed. Many have been used by military or police services. The WiFi Mesh standard was developed taking many good features from existing proprietary networks.

Having the network protocol available as a software product can lead to enormous flexibility. For instance, selection of media (radio, satellite, gprs, gsm, etc.) can be done on a case-by-case basis, or even on a connection-by-connection basis. Most protocols are self-forming meaning the individual nodes announce themselves and data routing maps are generated automatically. The other side of the self-forming coin is self-healing, where the network can automatically route around a failed node. Some networks can use multiple media simultaneously, choosing the least expensive route for data. In the case of complete failure of the cheap route (e.g. vhf radio), the expensive route (e.g. satellite) is automatically taken.

The following is a small selection of available vendors:

Mesh Dynamics Inc.; http://www.meshdynamics.com/

Strix Systems; http://www.strixsystems.com/

Tranzeo Wireless Technologies, Inc  (Sensoria); http://www.tranzeo.com/

[Tideland Signal Corp. is also a vendor for a mesh network protocol. The Tideland network is designed specifically for remote monitoring of aids to navigation, includes all of the features described above and makes an interesting study. However, it is not currently available as a general-purpose product. Please contact Paul Mueller paulm@tidelandsignal.com for more information.]