- Annex 2: Detailed user requirements

| Reference | User need | Current solution | Comments on current solution | Potential improvements from eNavigation |
| --- | --- | --- | --- | --- |
| ON-SHORE |  |  |  |  |
| Collection |  |  |  |  |
| MSI_REQ_1 | The MSI SP shall enable appropriate data originators to provide reports, in accordance with international standards | MSI SPs make available contact details and relevant instructions for data originators Information received from variety of third parties via published means of contact (email, fax, phone, paper, etc) | Single point of contact and well co-ordinated/structured. Duties are well specified in IMO Resolution A.706(17) Inconsistency in format of incoming data | Greater level of automation in receipt and formatting of input data |
| MSI_REQ_2 | The MSI SP shall receive reports relevant to its scope of responsibility | Separation between NAVAREA, national and coastal. Each MSI SP monitors output from neighbouring and other MSI SPs. | Strong mapping to level of geographical interest. Lack of centralised co-ordination and manpower intensive | No proposal to change scope of responsibility of each (NAVAREA, national or coastal) MSI SP. Greater efficiency in the sharing of data between MS SPs and quicker distribution of MSI through all MSI SPs. |
| MSI_REQ_3 | The MSI SP shall maintain records in accordance with the requirements of their national administration | Combination of on-line and off-line record keeping unique to each MSI SP. | No sharing of record keeping between MSI SPs. | Opportunity to centralise access to records of all MSI issued. |
| Integration |  |  |  |  |
| Analysis |  |  |  |  |
| MSI_REQ_4 | The MSI SP shall assess and validate incoming reports to identify MSI | Manual subjective process, using charts and other relevant information | Potential for inconsistencies in working practises | Increased use of standardised decision support tools |
| Presentation |  |  |  |  |
| MSI_REQ_5 | The MSI SP shall format all MSI in a form suitable for subsequent integration and presentation on-board all vessels | Guidelines published for different types of warning | Format used dictates presentation to end user. Potential for inconsistencies in working practises | Separation of data format from presentation offers flexibility in on-board presentation, which will require a published standard |
| Reference | User need | Current solution | Comments on current solution | Potential improvements from eNavigation |
| ON-SHORE |  |  |  |  |
| Exchange |  |  |  |  |
| MSI_REQ_6 | The MSI SP shall make available MSI relevant to their scope of responsibility | MSI SPs pass information to other MSI SPs that is out of their scope of responsibility (ie NAVAREA, NAVTEX service area, etc) MSI SPs monitor the on-air output of other MSI SPs | MSI SP relationships are well established Exchange of information is unstructured and ad hoc | Establish efficient and robust exchange mechanism providing information in known format |
| MSI_REQ_7 | MSI shall be made available as soon as possible, or as dictated by the timing of the event | MSI distributed on fixed schedules dictated by technologies used. Schedules can be overridden by urgent MSI. | Highest priority MSI will go out within 30 minutes of receipt. Scheduling standardised by performance of MSI SPs not standardised. |  |
| MSI_REQ_8 | The MSI SP shall continue to make available all MSI whilst valid. | In-force bulletin | Relies on users having received the message or picking up in-force bulletin in NM bulletin or on Internet | Ensure that all valid MSI is made available to user at all times. |
| MSI_REQ_9 | The MSI SP shall inform users of MSI that has ceased to be valid. | MSI includes validity date or is cancelled by additional MSI | Validity date is not available for all MSI. Users receive MSI about MSI. | Automatically cancel MSI that is no longer valid. |
| MSI_REQ_10 | MSI shall be available to all users to whom each item of MSI applies - in both location and time | Variety of different broadcast methods and schedules In-force bulletin used for MSI no longer being broadcast | Repeated transmissions necessary to ensure that users receive the broadcast | Improved availability of all valid MSI, regardless of date of issue |
| MSI_REQ_11 | MSI shall be available to all types of vessel, including both SOLAS and non-SOLAS vessels | Different technologies used for ocean and coastal MSI. Coastal NAVTEX service and RT more suitable for non-SOLAS vessels. Often rely on posters and notices for communication of MSI to non-SOLAS vessels | Needs of non-SOLAS users not well catered for | Provide MSI in a standardised form that can be used to develop solutions for all sectors of the market, removing the dependency on specific equipment |
| Reference | User need | Current solution | Comments on current solution | Potential improvements from eNavigation |
| ON-SHIP |  |  |  |  |
| Collection |  |  |  |  |
| MSI_REQ_12 | The user shall acquire all MSI appropriate to operational requirements | Messages are currently received by multiple systems (NAVTEX, SafetyNET, voice, etc) and manually logged | Risk of single person error, and lack of traceability in assessment of MSI by the user | The system should display all MSI relevant to voyage planning and/or voyage execution and save a record of all MSI received |
| MSI_REQ_13 | The user shall acquire MSI in a timely fashion appropriate to the event | Presently, it depends on the duties of the user | Possibility to distract the navigator, or to erase some relevant MSI | MSI should be displayed or signalled to the user as an additional piece of information in the navigation and alarm systems |
| Integration |  |  |  |  |
| MSI_REQ_14 | MSI information shall be integrated with on-board navigation and voyage data | Not done currently – manual process. Currently MSI is an independent and manual process | Risk of oversight. Subject to human error and ability to distinguish relevance/validity | Integration will enable improved decision making through presentation and analysis functions (see below also) |
| MSI_REQ_15 | MSI alarms shall be integrated with the vessel’s navigation alarm system | No integration with other on-board navigation alarm systems. No electronic/computerized treatment of MSI | Solely dependent on the MSI information collector presenting MSI to any concerned people | Enables consistency of presentation of alarms to user |
| Exchange |  |  |  |  |
| MSI_REQ_16 | Users shall be enabled to exchange MSI between vessels | Currently achieved by VHF transmission | No automatic procedure/system allowing the user to broadcast MSI | More effective exchange of MSI between vessels |
| Presentation |  |  |  |  |
| MSI_REQ_17 | All relevant MSI shall be presented in a user friendly manner and in conjunction with other related navigational information | Presently not integrated in terms of location and format onboard ships | Lack of integration into the navigation system doesn’t guarantee the proper treatment of the MSI | MSI information should appear like any other navigational or safety information readily available on the navigational system |
| MSI_REQ_18 | An alarm shall be presented to the user for any MSI when it represents a threat to the safe navigation of the vessel | Currently, the only alarm that is presented occurs at the moment of receipt (EGC) | Risk that MSI is not taken into account at the moment it is needed even if it has been previously acquired | System should be able to alert the user in due time and location when any MSI becomes relevant to the safe navigation of its ship |
| Reference | User need | Current solution | Comments on current solution | Potential improvements from eNavigation |
| ON-SHIP |  |  |  |  |
| Presentation |  |  |  |  |
| MSI_REQ_19 | It shall be possible to present MSI in a form suitable for inclusion in a berth to berth passage plan. | At the moment this is performed by gathering passage plan documents and relevant indentified MSI at the moment when the PP is produced | Correlated to the single person error than can occur in the integration process. Difficulty to guarantee proper updating of the PP | System should integrate a passage planning function that allows the user to recall the fulfilled template and update it with new MSI if required |
| MSI_REQ_20 | It shall be possible to present a record of any changes to relevant MSI over a specified time period. | Manual enquiry today | Again an open window to single person error that can prevent the operator to perform a perfect traceability of MSI | System should allow the user to perform queries on integrated MSI |
| MSI_REQ_21 | It shall be possible to present MSI in a form that meets record keeping requirements | The choice is given to every user | Again an open window to single person error that can prevent the operator to perform a perfect traceability of MSI | System should allow any investigator to check the alarm log |
| MSI_REQ_22 | All relevant MSI shall be readily apparent and independent of shift changeovers. | MSI can be improperly transferred during shift changeovers | Potential risk for an MSI to disappear from the navigator’s awareness | MSI information should appear like any other navigational or safety information permanently available on the navigational system |
| Analysis |  |  |  |  |
| MSI_REQ_23 | The relevance of each item of MSI shall be analysed. | Continual process of assessment, undertaken manually today | (+)The analysis is subject to a human evaluation of the relevancy, but (-) This relevance might not updated in case of any changes in speed, heading... of the ship | Could be readily automated through decision support tools within the on-board system |
| MSI_REQ_24 | The criticality of each item of MSI shall be analysed | Continual process of assessment, undertaken manually today | The user analysis is the sole agreed way to perform a practical and knowledgeable evaluation of MSI criticality | Should still depend primarily upon human analysis |
| MSI_REQ_25 | The analysis of MSI shall not be at risk of single person error | MSI is solely acquired, analysed and recorded by the watch keeper at the time of receipt | At the moment it is at risk of a single person error | eNavigation should provide a phased MSI alarm system to assist risk avoidance |