Maritime Information Systems (MIS): Scope and Focus

Working Paper – v3

IALA e-NAV6 Committee Meeting

9 - 13 March 2009, St. Germain en Laye, France

13 March 2009

Introduction

Definition:  “E-navigation is the harmonized collection, integration, exchange, presentation and analysis of marine information onboard and ashore by electronic means to enhance berth to berth navigation and related services for safety and security at sea and protection of the marine environment.”

This Working Paper deals with scope of what constitutes “Maritime Information Systems” as a component of e-Navigation.  It describes a variety shore-based and shipborne equipment, systems, services and processes.  As “work in progress”, it is not intended to be a definitive listing.  Instead, it is a basis for active discussion and further refinement by the IALA e-NAV Committee MIS Task Group.

Background

Definition:  “E-navigation is the harmonized collection, integration, exchange, presentation and analysis of marine information onboard and ashore by electronic means to enhance berth to berth navigation and related services for safety and security at sea and protection of the marine environment.”

As recorded in the Report of the IALA e-NAV4 Committee meeting:

10.6  Discussion on Maritime Information Systems (MIS)

Lee Alexander made a presentation introducing Maritime Information Systems (MIS). The committee agreed that this could be an essential component of e-Navigation. In particular, this would include dynamic operational information broadcast via AIS such as status of aids to navigation, water levels, current flow, and meteorological/hydrographic information. The committee agreed to form an ad hoc Task Group (TG) to examine this issue in more detail.

The TG discussed the scope of what would comprise MIS, both ashore and onboard ships. It was agreed that there are a wide variety of shipborne and shore based systems. In addition, there is a wide range of information types. The task group decided not to deal with S-Mode or Internet initially. Two MIS input papers will be prepared for e-NAV5 – Lee Alexander will prepare a paper on shipborne aspects and Jon Leon Ervik will prepare one on shore based aspects.

It was agreed that the work of the TG should be coordinated from within the Strategy and Operations WG and include liaison with other working groups of the e-Navigation Committee, as well as with the VTS Committee and the IHO – IEC Harmonization Group on MIOs (HGMIO).

As stated in Paragraph 4.2 of the Annex to NAV 54/WP.2 paper (Draft Proposed Strategy for the Development and Implementation of e-Navigation), the general expectations for the onboard, onshore and communications elements of e-Navigation are:

.1 Onboard:

Navigation systems that benefit from the integration of own ship sensors,

supporting information, a standard user interface, and a comprehensive system for

managing guard zones and alerts. Core elements of such a system will include,

actively engaging the mariner in the process of navigation while preventing

distraction and overburdening,

.2 Ashore:

The management of vessel traffic and related services from ashore enhanced

through better provision, co-ordination, and exchange of comprehensive data in

formats that will be more easily understood and utilized by shore-based operators

in support of vessel safety and efficiency, and

.3 Communications:

An infrastructure providing authorized seamless information transfer onboard

ship, between ships, between ship and shore and between shore authorities and

other parties with many related benefits, including a reduction of single person

error.

Since onboard and ashore elements of e-Navigation are closely related, they will be treated together.  Communications applies to both elements (ashore and shipborne) since it is the means to communicate information from shore-to-ship, ship-to-ship, and ship-to-shore.

E-Navigation User Needs  Systems/Services

Goal: 	Describe the variety of shore-based and shipborne marine navigation systems and services that would fall under the e-navigation concept of operations.

Scope: 	1) 	Develop an overview of the user need/requirements, and sources of information related to the various needs.

2) 	Breakdown the information and map it to the proposed object model. [VTS28/WG4/WP02]

Shore-based Perspective

### Information Needs (provisional)

### The following identification is based on the three primary tasks of present and future VTS’s (res. A.857(20))

### Information Service (VTS) [from what was developed by VTS Committee]

| Task/Functions | Information Needed | Source | Requirements (in addition to v128) | (U)MDM |  |
| --- | --- | --- | --- | --- | --- |
| Maintaining a traffic image |  |  |  | Traffic obj |  |
|  | ship “unique” identity | AIS | IEC exists | IMO# |  |
|  | ship identities | AIS | IEC exists | MMSI#, callsign, name |  |
|  |  | Radio voice communications | Check VDR standard for voice output |  |  |
|  |  | Electro-Optical | ? | ? |  |
|  |  | RDF/VHF | IEC exists | Bearing frequency |  |
|  | ship positions | AIS | IEC exists | LAT LON DGNS Raim Time |  |
|  |  | Retransmitted AIS | IEC exists | LAT LON DGNS Raim Time |  |
|  |  | Static Radar |  | Range/Azimut Position (TBC) Radar own position Target size Echo strength Time Quality -measure |  |
|  |  | RDF | IEC exists | Bearing frequency |  |
|  |  | LRIT |  | ID (IMO#) LAT LON Time |  |
|  |  | Mobil Radar |  | Range/Azimut Position (TBC) Radar own position (mobile radar) Target size Echo strength Time Quality -measure |  |
|  | ship intentions and destinations | Radio voice communications |  |  |  |
|  | Sailing plan | Ship (VHF) |  |  |  |
|  |  | Ship (IBS) |  | Dep_location Waypoint’s ETA’s Tide info (static/dynamic, see environmental data) Arr_location |  |
|  |  |  | Standard needed for exchange of sailing plan between VTS Centres and to/from the vessel’s master |  |  |
|  |  | AIS |  |  |  |
|  | Ship Navigational Status | Radio voice communications, AIS |  |  |  |
|  | Cargo information | Radio voice communications, Agent | User interface needed for agents to update HAZMAT, contact details |  |  |
|  |  | SafeSeaNet |  |  |  |
|  |  | AIS |  |  |  |
|  | Number of persons on board | Agent, SafeSeaNet | User interface needed for agents to update persons on board |  |  |
|  | Ship database | e.g. Lloyds, VTS database | Define minimum fields to deliver global consistency |  |  |
|  | Changes in promulgated data | Administration/IMO | Manual update |  |  |
|  | Chart data | Hydrographic office | VTS Centre to use current charts |  |  |
|  | Meteorological and hydrological conditions | Met office | UKC system requirements to be considered |  |  |
|  |  | hydro/meteo sensors | Update frequency definition |  |  |
|  | limited maneuverability (diving, dredging etcetera) | Administration |  |  |  |
|  | Navigation safety information (eg. fairway information, obstructions, infrastructure out of order, AtoN off-station) | Administration, AIS AtoN |  |  |  |
|  |  | Radio voice communications | Need to define frequency of message broadcast as a function of traffic density |  |  |
|  |  | (remote) monitoring |  |  |  |
| Interaction with traffic | Ship identity & position report | Radio voice communications | Establish reporting system (frequency etc) |  |  |
|  |  | <future means?> |  |  |  |
|  | Information to master (Notifications) | Radio voice communications |  |  |  |
|  | Request from Master | Radio voice communications | Establish service level (eg. response time) |  |  |
|  |  | <future means?> |  |  |  |
| Incident Management | Incident report | Radio voice communications, SITREP | Mechanism to escalate efficiently to responsible parties |  |  |
|  |  | Radio voice communications, SITREP | Minimum & standard data requirement for incident reports. |  |  |
|  | Implement contingency plan | Radio voice communications broadcast |  |  |  |
| Provide traffic image to allied services | VTS Centre information | Surface picture of vessels and their movements in the VTS area to be transmitted | Addressed to some degree with Data Exchange Standard |  |  |
| Archive and replay of traffic image & concurrent data (voice, CCTV, met) | VTS Centre information | Internal storage |  |  |  |
| Coordination with Allied Services | SAR incident reports | SAR authority, Coastguard | Mechanism to escalate efficiently to responsible parties |  |  |
|  | Pollution incident reports | Pollution Authority & Vessels | Mechanism to escalate efficiently to responsible parties |  |  |
|  | …many others not listed… |  |  |  |  |
| System maintenance and status monitoring | system statuses | All VTS equipment | Define archiving of the configuration changes to allow audit (eg. changes in radar settings or zone alerts) |  |  |

### Traffic Organisation Service

Same as INS, except a potentially higher quality of service for the traffic image, plus

| Task | Information Needed | Source | Requirements | (U)MDM |
| --- | --- | --- | --- | --- |
| Strategic planning | (long term) sailing plan | Agent |  |  |
|  | ship database | e.g. Lloyds |  |  |
|  | berth planning | Port Administration |  |  |
|  | route structure (traffic separation scheme) | Transport Administration |  |  |
|  | Cargo information policy | Transport Administration |  |  |
| Route adherence monitoring | Specific sailing plan | Radio voice communications |  |  |
|  | Cargo information and risk profile | HAZMAT, Risk analyses (Check the completeness of IMO Guidelines) | Review existing standards in this area |  |
|  | traffic clearances (passing of waypoints) | Radio voice communications |  |  |
| Traffic separation (operations) | Processing of information according to rules | Radio voice communications, switching of traffic lights |  |  |
| Coordination with allied services | Vessel database, Allied Services database, sailing plan | Agent, Allied Services, Lloyds |  |  |
|  |  | Radio voice communications |  |  |

### Navigational Assistance Service

Same as TOS, except a potentially higher quality of service for the traffic image.

Miscellaneous Issues

| Task | Information Needed | Source | Requirements | (U)MDM |
| --- | --- | --- | --- | --- |
| Vessels in VTS area to have adequate charts | Automated request from VTS Centre to Vessel for ENC chart version | Chart: Hydrographic office or commercial provider | Automated check that version of local ENC chart on board is adequate (future e-Nav) | xxx obj |
| Vessels in VTS area to have correct AIS data, if not, then report to responsible authority | AIS Static and voyage related data | Vessel AIS system |  |  |

Information Service (non-VTS)

| Task/Functions | Information Needed | Source | Requirements (in addition to v128) | (U)MDM |
| --- | --- | --- | --- | --- |
| Maintaining logistics | Berth Availability |  |  | xxx obj |
|  | Departure Time (ETD) |  |  |  |
|  | Tug information |  |  |  |
|  | Linesmen |  |  |  |
|  | Regular Traffic |  |  |  |
|  | Port Facilities |  |  |  |
|  | ISPS |  |  |  |
|  | Pilot |  |  |  |

Requirements Draft (to be refined by OPS working group)

Ship database -

Define minimum fields to deliver global consistency

| Description | Data Form |
| --- | --- |
| MMSI number | Std |
| Ship Name |  |
| Ship Native Name |  |
| IMO number |  |
| Flag State |  |
| Ship owner |  |
| Length |  |
| Beam |  |
| Draught |  |
| Air Draught |  |
| Type |  |
| Gross Tonnage |  |

| User requirement | Data Sources | Comments | UMD element |
| --- | --- | --- | --- |
| Position | AIS, radar, VHF, LRIT | AIS could also be from space | uoiuoi |
| Course | AIS, radar |  |  |
| Speed | AIS, radar, VHF |  |  |
| Heading | VHF |  |  |
| Draught | AIS, radar, VHF, reporting systems |  |  |
| Air draught | VHF |  |  |
| Flag | AIS, databases |  |  |
| Destination | AIS, VHF, reporting- and agent systems |  |  |
| Arrival Time (ETA (new)) |  |  |  |
| Ports of call |  |  |  |
| ROT |  |  |  |
| ISPS status |  |  |  |
| Ship Status | AIS |  |  |
| Passage Plan |  |  |  |
| HAZMAT |  |  |  |
| Marine Pollution |  |  |  |
| Pilot Status |  |  |  |
| Under-keel Clearance |  |  |  |
| CCTV |  |  |  |
| Radar Data |  | From ship, shore &space |  |
| Rebroadcast AIS |  | From ship |  |
| Statatory Information |  |  |  |
| Paris MOU status |  |  |  |
| MET/HYD & wind information |  |  |  |
| NAVAREA (new) |  | warnings (e.g., unlit lights, drifting objects, wrecks, etc.) |  |
| Limitations such as local regulations and legislations. (new) |  |  |  |
| SAR (Search and Rescue) information (new) |  |  |  |
| GEO(Group on Earth Observations)-special information such as sandbanks. (new) |  |  |  |
| Oil polluting information (new) |  |  |  |
| Dynamic MaRA information based on coastal vulnerability and weather conditions. (new) |  |  |  |
| Voyage planning (VP) information such as fairway availability, obstruction and events following the traffic behavior. (new) |  |  |  |

Logistics

| User requirement | Data Sources | Comments |
| --- | --- | --- |
| Berth Availability |  |  |
| Departure Time (ETD) |  |  |
| Tug information |  | Position & availability |
| Linesmen |  |  |
| Regular Traffic |  | Schedules services in the area such as ferries |
| Port Facilities |  | Ex. Bunker, crane, waste |
| ISPS |  |  |
| Pilot |  |  |

### Shipborne

### Information Needs (provisional)

### The following identification is based on the primary tasks of present and future shipborne navigation systems. [As listed in IMO NAV54/WP.2, Annex 1]

Collision avoidance

Charting

Positioning, Navigation and Timing

Communications

Reporting

Situational Awareness

| Task/Functions | Information Needed | Source | Existing specifications | Action Required |
| --- | --- | --- | --- | --- |
| Collision avoidance | Relative position | Radar/radar plotting (ARPA) | IEC 62388 | none |
|  |  | AIS | IEC 61993-2 | [Check with Alan Stewart] |
|  |  | Compass bearings |  |  |
|  |  | Electro-Optical (night vision) |  |  |
| Charting |  | ECDIS/ECS | IMO, IHO, IEC |  |
| Positioning, Navigation and Timing |  | GNSS (e.g., GPS/DGPS) | IMO, IEC |  |
|  |  | e-Loran |  |  |
| Communications |  | VHF |  |  |
|  |  | HF |  |  |
|  |  | AIS |  |  |
|  |  | NAVTEX |  |  |
|  |  | Satellite (e.g., INMARSAT) |  |  |
| Reporting |  | AIS, LRIT |  |  |
| Situational Awareness |  | AIS, radar, physical AtoN |  |  |
| Integrated Navigation Systems (e.g., INS & IBS) |  | Integrated Navigation System | IMO | [check with Florian Motz] |
|  |  | Integrated Bridge System | IMO | [check with Florian Motz] |

E-Navigation-related Equipment, Systems, and Services

[will become an Annex or Glossary]

Scope/goal: 	Describe the variety of shore-based and shipborne marine navigation systems that fall under the e-Navigation concept of operation.

Shore-based

Equipment    - Navigational devices, sensors or equipment that are [or could be] used for navigation.  This includes both [analog] and electronic equipment?

GMDSS (Global Maritime Distress Safety System) is based on an international agreement regarding required types of equipment (carriage) and communication protocols used to increase safety and make search and rescue (SAR) rescuing easier.

Radar/ARPA (Automatic Radar Plotting Aid) is a computer assisted radar data processing system. The system creates tracks using radar contacts to calculate the objects course, speed and CPA to avoid danger of collisions.

AIS (Automatic Identification System) is a supplement to radar that  automatically provides other ships and coastal authority with information about the ship’s dynamic (e.g., speed), static (e.g., identity), and voyage-related (e.g., destination). This information is broadcast via the VHF net.

ECDIS (Electronic Chart Display and Information System) is a computer-based navigation information system which can be used in lieu of paper nautical chart. This shipborne navigational display provides continuously integrated position information from GPS, and other navigational sensors in addition to navigation-related information All true ECDIS meets IMO`s requirement, and use vector chart with standardized content, structure and format.

CCTV (Closed-Circuit Television) is the use of video cameras to transmit a signal to a specific, limited set of monitors.

VHF Voice Radio - ?

[Others?]

System

LRIT (Long Range Identification and Tracking system). The most important distinction between LRIT and AIS is the range, and that LRIT only will be available to the recipients who are entitled to receive the information.

SSN (SafeSeaNet) is a reporting system for all ships approaching and departing Norwegian harbors.

GNSS (Global Navigation Satellite System) is the standard generic term for the global satellite navigation systems. The only fully functional GNSS in the world is NAVSTAR GPS (Global Position System), developed and operated by United States. To enhance the GPS we have DGPS (Differential GPS) which can provide the accuracy down to centimeters.

Navigational Warnings (such as unlit lights, drifting objects wrecks, etc.)

e-Loran

Service

VTS (Vessel Traffic Service) provides information service, navigational assistant’s service, and    traffic organization service. Immediately might the role of VTS change and expand if it should be incorporated in a concept on a global level, Vessel Traffic Management.

VTM (Vessel Traffic Management) is the structured framework of strategic, tactical and operational efforts to maximize the safety, security and efficiency of vessel traffic in interaction with its operating environment worldwide. Here we have VTS, pilotage, legislation, routing and reporting means.

MRCC (Maritime Rescue Coordinated Center) coordinates SAR operations.

Oil polluting response.

Computer systems such as Dynamic MaRA. Dynamic MaRA is used in specific areas with a high environmental vulnerability and difficult weather conditions.

Maritime Coastal Radio stations coordinate information between ship and shore.

Weather Service (e.g., meteorlogic/hydrographic information including wind, wave, current flow, visibility, ice, etc.).

Sea Ice (ice coverage, types of ice, icebergs, etc.)

Shipborne

Equipment

Definition - Navigational devices, sensors or equipment that are directly related to providing information that can be used to enhance navigation and decision-making onboard vessels.

Radar/ARPA – Nick Ward

Satellite-based (e.g., GNSS) – Nick Ward

Terrestrial (e.g., eLORAN) – Nick Ward

ECDIS/ECS – Lee Alexander and Michael Bergmann

AIS [check with AIS WG]

- e.g., AIS Binary Messages

System [need definition]

INS/IBS – [check with Florian Motz]

Automated Reporting Systems – [check with Michael Bergmann]

- e.g., SSN (SafeSeaNet) is a reporting system for all ships approaching and departing European harbors.

[others?]

Service

[use what is listed in NAV54/WP.2, Annex 1, Section 8 (MIS)

8 Maritime Information Systems [& Services]:

8.1 SOLAS chapter V, regulation 13, requires Contracting Governments to provide navigational information to mariners. Regulation 13, paragraph 3 states: “Contracting

Governments undertake to arrange for information relating to aids-to-navigation to be made

available to all concerned. Changes in the transmissions [i.e., status report] of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.”

8.2 The promulgation of information on navigational safety is coordinated by means of the

World-Wide Navigational Warning Service that was established jointly by IMO and IHO

in 1977.

8.3 The World-Wide Navigational Warning Service is administered through 21 AVAREAS5. Each NAVAREA has an Area Coordinator who is responsible for collecting information, analysing it, and transmitting NAVAREA Warnings.

8.4 Lists of aids to navigation are produced by (or for) most national authorities as part of the

navigational information made available to mariners in support of SOLAS chapter V,

regulation 13.

8.5 The joint IMO/IHO/WMO Manual on Maritime Safety Information (IMO COMSAR/Circ.15) provides definitions of standard terms to describe particular events

that should be used when composing navigational warnings. The terms that are relevant to the

condition of aids to navigation are defined in the circular.

Processes

Marine Information Overlays (MIOs) - Lee Alexander

- static (	IHO S-57  IHO S-100)

- dynamic (AIS Binary Messages)

Types of

e-Navigation Related Data and Information

[Add section describing how the (U)MDM could be used]

It is important to make a clear distinction between “information” and “data.”  “Information” includes voice, text, fax, graphical, geo-spatial (map or chart), etc.  If/when this information is converted into another format for digital transmission, it becomes “data” (e.g., NMEA 0183 or AIS binary message).  However, to be relevant to a mariner onboard a vessel this data must eventually be converted back into a presentation/display of “information” that can be used for situational awareness or decision-support.

Shipborne

Electronic chart data

vector (e.g., IHO S-57 ENCs, SENC Direct Distribution)

raster (e.g., UKHO Admiralty Raster Chart Service, or ARCS)

MIOs (Marine Information Overlays)

Static (e.g., IHO S-57)

Dynamic (e.g.,

AIS Binary Messages)

Presentation/Display of e-Navigation Information

Shore-based

At present, there are no specific standards related to the presentation/display of shore-based information (e.g., at a VTS Centre).  Potentially, some of the standards that apply to the display of shipborne information may be suitable for shore-based displays as well.

Shipborne

There are a number of general and equipment-specific international standards that have been adopted by IMO, IHO, and IEC that contain “guidance” related to the presentation/display of shipborne navigation-related information.

General

Performance Standards for the Presentation of Navigation-related Information on Shipborne Navigational Displays, IMO Res. MSC.191(79), 6 December 2004.

Guidelines for the Presentation of Navigation-related Symbols, Terms and Abbreviations, IMO SN/Circ.243, 15 December 2004.

Presentation of Navigation-related Information on Shipborne Navigational Displays – General Requirements, methods of testing, required test requirements. IEC 62288, Edition 1.0, July 2008.

Equipment Specific (only a partial list)

Revised Performance Standards for ECDIS, IMO MSC.232(82), 2006.

Specifications for Chart Content and Display Aspects of ECDIS, IHO S-52, Ed. 4.2, March 2004.  (also includes: IHO S-52, Appendix 2, Colour and Symbol Specifications for ECDIS, March 2004.)

Performance Standards for Radar Equipment, IMO MSC.192(79), 2004.

Performance Standards for a Universal Shipborne Automatic Identification System (AIS), IMO Res. MSC 74(69), Annex 3, 19 May 1998.

Guidelines for the Onboard Operational Use of Shipborne Automatic Identification Systems (AIS), IMO Res. A.917(22), 25 January 2002.

Display of AIS Target Information, IMO SN/Circ. 217.

Guidance of the Application of AIS Binary Messages, IMO SNCirc.236, 28 may 2004.

Performance Standards for an Integrated navigation System (INS), IMO Res. 86(70), Annex 3.

Integrated Navigation Systems (INS) - Operational and performance requirements, methods of testing and required test results.  IEC 61294, Ed. 1,

General requirements for shipborne radio equipment forming part of the GMDSS, and for electronic navigational aids, IMO A.694(17), 1991.

Harmonization Challenges and Opportunities

Currently, the performance standards and operation of most ashore and shipborne equipment/systems have been developed independent of one another.  As such, there is a need (or challenge) to describe how it will be possible to achieve the goal of e-Navigation:

“the harmonized collection, integration, exchange, presentation, and analysis of marine information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment.”

a) Shore-based considerations

The exchange of information required for berth-to-berth navigation would benefit from global standards and protocols. These standards and protocols should be in an open form so everybody can share and make use of them.  But, it is also important that they are appropriate and not subject to constant change. It is the increase in traffic density along with the diversity and enlargement of the shipping fleet that has increases the need for more reliable and up-to-date information. This information can still be presented with different symbols by different users, as long as the information content is exactly the same.  But, there is also a need to consider a mutual compatibility and interoperability of equipment, systems, symbolism and operational procedures. We need to articulate a viable and coherent framework to achieve a common and consistent format to avoid potential conflicts between users.

The first step will be to get consistent global standards and protocols.  The next challenge will be to decide on the best process for collection, integration, simplification, presentation and analysis of marine information onboard and ashore by electronic means.

b) Shipborne

Initially, there will significant challenges associated with trying to reconcile the existing shipborne equipment/system performance standards such that they conform to an e-Navigation Concept of Operation.   One way to start this harmonization process is to develop and agree upon some guiding principles.

Guiding Principles

Goal:  	Unless there is a compelling need not to do so, both shore-based and shipborne equipment/systems/services should provide information content to all users (ashore and shipborne) that is uniform or consistent in presentation/display.

General

a) There may be a need to develop new international and supra-national regulations and guidelines.

b) Potentially, there should be some “guiding principles” that are followed related to the information provided by shore/shipboard marine navigation systems.  These include:

- Consistency/uniformity -- yet flexible to allow for innovation.

- Presentation/display ashore and onboard vessels (i.e., is there a need to be different?)
- Ergonomics/human factors must be considered (e.g., too much information or cluttered display can be worse than too little information)

- Education and training of personnel – both shore-based and shipborne – will be crucial.

c) The proposal to the IMO work program on e-Navigation defines seven key components of a safe and comprehensive e-navigation policy.  These include:

1. Electronic chart and weather information

2. Electronic positioning signals

3. Electronic information on vessel route, course, maneuvering, etc.

4. Transmission of positional and navigational information

5. Display of information

6. Information reporting, prioritization and alert capability

7. Transmission of distress alert and maritime safety information

In order to be able to exchange and communicate maritime information between different systems, ashore and shipborne, new global standards may need to be developed.

There may also be a need to design an infrastructure that would enable authorized information to be transferred between shore authorities, and other parties with opportunities for improving the efficiency of transports, logistics and distributions.

Recommendations on Way Forward

Shore-based

Pursue a holistic approach of global standardization and protocol.  Specifically:

- Enhance the VTS through better provision, coordination, and exchange of comprehensive data in support of vessel safety and efficiency.

- Create a global combined integration of exchanging information within e-Navigation and therefore the VTM. The VTM concept must continue to facilitate global coverage, and continue to decrease the diversity of port services by co-ordination of allied services.

- Enhance the user-friendliness with the goal of reducing human error, while preventing distraction and overburdening by trying to reducing the range of reporting obligations on the ship-owner and ship master.

- Enhance and combine education and training for those authorities involved in the management of maritime transport.

- Continue to articulate a viable and coherent framework which sets out to achieve a common and consistent format for the use of electronic technologies.

- Keep the e-Navigation concept to be evolutionary and dynamic so that it will continue to develop as new user needs arise, and the emerging technological opportunities evolve together.

Shipborne

1. Review existing performance standards for shipborne equipment/systems/services that have been adopted by IMO, IEC, IHO, ISO, etc.  The focus should be on those that are e-Navigation-related.  Particular attention is should be made to the recent IMO and IEC performance standards regarding the “Presentation of Navigation-related Information on Shipborne Displays”.

2. Compile a list of the various performance standards that pertain to e-Navigation.  This listing/review should include specific mention of any inconsistencies and/or conflicts noted that may inhibit the implementation and conduct of e-Navigation.

3. Develop a Concept of Operation/Best Practices for Shipborne e-Navigation.  The intent is to explain what needs to be done to harmonize the wide variety of equipment, systems, and services in order to achieve the goal of e-Navigation.

4. Provide recommendations to the IALA e-Navigation Committee IMO in regard to how e-Navigation can be implemented within the context of existing international performance standards.  Ultimately, this may include recommendations to IMO for potential new standards/guidance specific to achieving the eleven (11) Core Objectives of e-Navigation.

_____________________________

Version 1 – 28 August 2008, Version 2 – 18 September 2008

See “Proposed Strategy for the Development and Implementation of e-Navigation” (NAV 54/WP.2, 11 June 2008), Annex, Section 5, p.3-4)