AISM Association Internationale de Signalisation Maritime     IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation

[e-###]

On

The e-Navigation Architecture -

the initial Shore-based Perspective

(e-NAV101)

Edition 1.0

December 2009

Draft as at 13/3/09

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01    Fax: +33 1 34 51 82 05

e-mail:  iala-aism@wanadoo.fr       Internet:  www.iala-aism.org

This document has been edited to conform to the IALA template

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

IALA Recommendation on ....... (Block text)

(Recommendation X-####)

THE COUNCIL:

RECALLING that one of the aims of the Association is to foster safe, economic and efficient movement of vessels and the protection of the environment through the improvement and harmonisation of aids to navigation, vessel traffic services and other means world-wide

RECOGNISING that the e-Navigation architecture will assist in the development and maintenance of e-Navigations applications for ship to ship, ship to shore, shore to ship, shore to shore, in particular in the following fields

- shore-based technical e-Navigation services;

- technical means for communication;

- data modelling and referential data;

- Human Machine Interface presentations;

RECOGNISING ALSO that the e-Navigation architecture will

- assist administrations in the efficient deployment of their operational and technical services to the mariner and to the maritime community;

- require administrations to maintain those operational and technical services to the mariner and to the maritime community with the service level, as published in their service portfolio;

NOTING

- IMO’s “Strategy for the development and implementation of e-Navigation” (MSC 85, Annexes 20 and 21);

- IMO has expressed an interest in the contribution of IALA to the work on e-Navigation (compare IMO MSC86/23/4, IMO MSC86/26, §23.26, and draft report of IMO NAV55, §11.11);

- that the definition of e-Navigation has been established by IMO;

- that the user requirements are not yet finally established at IMO on the other hand;

- hence there will be a need to revise any recommendation of IALA in the light of the then finally established user requirements;

- that during a transition period both “e-Navigation compliant” services and systems and legacy services and systems will co-exist, and that that co-existence must not threat the established level of safety and of efficiency and of the protection of the environment;

- an increased need for service portfolio management, both for operational and technical services provided by an IALA member;

- an increased usage of service level definitions to describe the quality parameters of those operational and technical services;

- expected variations of service portfolios and service levels;

- the work of various IALA Committees regarding user requirements within the scope of IALA;

ADOPTS The e-Navigation Architecture – the initial Shore-based Perspective in the annex of this recommendation; and,

RECOMMENDS that National Members and other appropriate authorities providing marine aids to navigation services adopt the e-Navigation design and implementation principles in accordance with the Annex to this recommendation.

Table of Contents

Document Revisions	2

Index of Tables	6

Index of Figures	6

Annex	7

1	Overview	7

2	Background	7

2.1	IALA’s general mandate	7

2.2	IMO’s e-Navigation concept	8

2.3	Consequential driving forces for IALA and IALA members due to IMO’s e-Navigation concept	8

2.4	IALA’s role in the e-Navigation concept	9

2.5	Consequences of IMO’s definition of the e-Navigation concept	10

3	The application-to-application information/data flow	10

4	From e-Navigation concept to a technical e-Navigation architecture	11

4.1	The technical point of view - introduction	11

4.2	The e-Navigation architecture	12

4.3	The importance and the features of the Universal Maritime Data Model	15

4.4	Functional and physical links	15

4.4.1	The relationship between functional and physical links	15

4.4.2	Relevant physical link technologies	16

4.5	IALA’s role applied to the e-Navigation architecture	17

5	The common shore-based e-Navigation system architecture	18

5.1	Introduction	18

5.2	Information/data flow, application interactions, and user interfaces	19

5.3	The functional approach for the common shore-based e-Navigation system	23

5.4	The shore-based e-Navigation system layout	24

5.5	Migration of legacy systems	24

5.6	Life-cycle management considerations	25

5.6.1	Overview and introduction	25

5.6.2	Information/data life-cycle management	26

5.6.3	Technical e-Navigation services and components life-cycle management	27

6	Dependency on external systems and on infrastructure	27

6.1	Dependency on GNSS for position and time	27

6.2	Dependency on infrastructure	27

7	IALA e-Navigation documentation framework	27

8	Glossary of terms	29

Index of Tables

Table 1	Derivation of information needs, information objects and data objects for technical services from user requirements	19

Index of Figures

Figure 1	e-Navigation Architecture	14

Figure 2	Scope and Impact of the Universal Maritime Data Model	15

Figure 3	A Variety of physical link technologies relevant for the e-Navigation architecture	17

Figure 4	The place of the shipboard e-Navigation system architecture within the e-Navigation architecture – simplified presentation	20

Figure 5	The e-Navigation architecture in terms of information flow, application interactions, and user interfaces	21

Figure 6	Engineering analysis of requirements to the shore-based e-Navigation system	24

Figure 7	Life-cycle management of systems and processes supporting the e-Navigation architecture	26

Figure 8	Future IALA e-Navigation documentation scheme	28

Annex

The e-Navigation Architecture –

the initial Shore-based Perspective

# Overview

This Annex describes the e-Navigation architecture from a shore-based perspective and thus provides guidance to IALA membership regarding methodology.

To that end, this Annex first provides background information on IMO’s e-Navigation concept, the driving forces which lead to its development, and IALA’s role in the e-Navigation concept. Within this context, the dual nature of IALA’s commitment to IMO’s e-Navigation concept is reflected: While there has been a long standing responsibility for architectural considerations of shore-based systems within IALA’s existing mandate, IMO’s e-Navigation concepts adds new dimensions to those architectural considerations.

The Annex goes on to introduce important principles of the e-Navigation architecture, such as the orientation towards information flow instead of technology orientation.  The Annex then deduces an e-Navigation architecture in system engineering terms from the e-Navigation concept, and establishes relevant principles for a shore-based e-Navigation system architecture. In particular a number of concepts are introduced and explained, namely

- encapsulation,

- interactions between applications,

- functional interfaces to users,

- functional and physical links ship to ship, ship to shore, shore to ship and shore to shore, and

- technical e-Navigation services.

This introduction of important principles eventually culminates in one concluding figure (Figure 4), which is a comprehensive engineering representation of the e-Navigation architecture with a focus on the shore-side.

After the principles of the shore-based e-Navigation system architecture have been introduced, this Annex applies them to appropriate technologies.  It also describes how it is possible to seamlessly derive more technical requirements for the modules and components of the shore-based e-Navigation system from stated information needs and formats of the users (“top down approach”).  It also takes into account the requirements of life-cycle management at the appropriate stage.

The e-Navigation depends on two external contributions, namely the IMO defined World Wide Radionavigation System (WWRNS) (IMO Resolution A.953(23)), and on the on-site infrastructure both at shore and on ships.  These dependencies will be addressed later on in the document.

The Annex concludes with a discussion of the bearing of the e-Navigation architecture on IALA documentation and provides an “e-Navigation compliant” structure for the IALA documentation as an overview.

# Background

## IALA’s general mandate

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) was formed in 1957 as a non-government, non-profit making, technical association that provides a framework for aids to navigation authorities, manufacturers and consultants from all parts of the world to work with a common effort to:

- harmonise standards for aids to navigation systems worldwide;

- facilitate the safe and efficient movement of shipping, and;

- enhance the protection of the maritime environment.

The functions of IALA include, among other things:

- developing international cooperation by promoting close working relationships and assistance between members;

- liaison with relevant inter-governmental, international and other organisations.  For example, the International Maritime Organisation (IMO), the International Hydrographic Organisation (IHO), the Commission on Illumination (CIE), and the International Telecommunications Union (ITU);

- liaison with organisations representing the aids to navigation users;

- address emerging navigational technologies, hydrographic matters and vessel traffic management;

- provide specialist advice or assistance on aids to navigation issues (including technical, organisational or training matters);

- encouraging IALA members to develop policies that address the social and environmental issues associated with establishing and operating aids to navigation.  This includes issues such as use of aids to navigation as a base for the collection of data or other governmental or commercial services.

For a complete list refer to IALA Constitution.

## IMO’s e-Navigation concept

IMO (MSC 85) adopted a “Strategy for the development and implementation of e-Navigation” (MSC85-report, Annexes 20 and 21).

In particular, IMO adopted the following definition of e-Navigation:

“e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment.”

Three mutually interacting parts of the e-Navigation architecture can be identified:

- shipboard systems of information/data processing devices;

- application-to-application data exchange via physical links ship to shore and shore to ship;

- shore-based e-Navigation system architecture that integrates a variety of shore-based technologies and data processing devices.

## Consequential driving forces for IALA and IALA members due to IMO’s e-Navigation concept

IMO has stated the driving forces and the consequential goal for their e-Navigation concept as follows:

“There is a clear and compelling need to equip shipboard users and those ashore responsible for the safety of shipping with modern, proven tools that are optimized for good decision making in order to make maritime navigation and communications more reliable and user friendly.  The overall goal is to improve safety of navigation and to reduce errors.  However, if current technological advances continue without proper coordination there is a risk that the future development of marine navigation systems will be hampered through a lack of standardization on board and ashore, incompatibility between vessels and an increased and unnecessary level of complexity.”  (IMO MSC 85, Annex 20, §2.1).

Consequential driving forces for IALA and IALA members due to that e-Navigation concept are:

- Increased demand for improved information/data processing in maritime services: It is a requirement that more data is collected and stored in order to facilitate data comparison, data exchange and statistic evaluation.

- Increased degree of automation: An increased degree of automation is needed to process the increased amount of data and is required to assist shore-based operators and maintenance personnel.

- Request for simplification of information sharing between information users and information providers.

- Increased demand for communication capabilities and capacity: The maritime community requires user friendly and efficient communication systems that ensure confidentiality where required, integrity and availability of information being transmitted and received ship to ship, ship to shore, shore to ship, and shore to shore.  Also, there is an increased demand for communication capacity to Search-and-Rescue and law enforcement aircrafts.

- Advent of digital information technologies: Most information is now available in digital format both onboard ships and within shore-based systems.

- Extended area coverage, up to global coverage: There is a requirement for extended area coverage which can be fulfilled more readily by technologies presently under development.

- Reduction in staffing level: A steady reduction in staffing level is a demand by national governments, administrations requires

- an optimum of enhanced technical services to support in terms of both investment and maintenance and

- an efficient life cycle management of the technical services in place.

- Demand for improved cost/benefit ratios: Administrations face an increased public awareness for cost/benefit ratios for the operation and maintenance of their shore-based systems.

- International standardisation: International standardisation has been recognised as a state-of-the-art description for technology.  Hence, there is an increased need for information and documentation for a common, international and public understanding of system functions.

- Open architecture: Modular and open system design principles should be applied striving for “plug-and-play” capabilities.  Open system architectures are more scalable and better maintainable.  When individual components need to be replaced due to life-cycle management, there is a requirement to maintain the functional requirements of a shore-based system.

## IALA’s role in the e-Navigation concept

The IMO described the overarching governance of the e-Navigation concept as follows:

“The governance of the e-Navigation concept should reside in a single institution that has the technical, operational and legal competences needed to define and enforce the overarching framework with implementation, operation and enforcement taking place at the appropriate level – global, regional, national or local – within that framework. This approach does not mean that the governing organisation has to carry out all tasks in-house – it can delegate as appropriate to competent bodies.  Being responsible for establishing mandatory standards for enhancing the safety of life at sea, maritime security and protection of the marine environment as well as having a global remit, IMO is the only organization that is capable of meeting the overall governance requirement.”  (MSC 85, Annex 20, §9.2)

Because of its general mandate, IALA has a responsibility to the maritime community to support national members providing shore-based services (ship to shore, shore to ship and shore to shore). In order to fulfil this responsibility for its membership IALA produces recommendations, guidelines and manuals.  Thus, IALA fulfils the task that IMO has stated essential for the implementation of the e-Navigation concept, namely coordination and standardization on a global scale (refer to IMO quotation in the last section above), as far as IALA’s membership is concerned.

In doing so IALA observes the definition and enforcement of the overarching e-Navigation framework as defined by IMO as the governing body (refer to above IMO quotation).

Furthermore, IMO considers IALA to be a competent body because of its natural role and experience in shore-side matters (IMO MSC86/23/4).  Also, IALA previously assisted IMO in developing the e-Navigation strategy (IMO MSC86/26, §23.26).  IALA is participating in the implementation plan of the e-Navigation strategy, by capturing user requirements, developing a shore-side architecture and performing gap and cost/risk analyses (IMO MSC/86/23/4 and IMO draft report of NAV55, §11.11).  Hence, IMO has invited IALA and other international organisations to participate in its work and provide relevant input. IALA formed the e-Navigation committee for the purpose of developing recommendations and guidelines on shore-based e-Navigation systems and services.

## Consequences of IMO’s definition of the e-Navigation concept

From the above IMO definition of e-Navigation, the following points are understood:

- Operational requirements (user requirements) will be analyzed and their impact on the e-Navigation architecture will be assessed – thereby fulfilling the demand for a “user requirement driven system architecture”.  Conversely, internationally analysed and consolidated operational requirements need to be represented in a way meaningful to the individual parts of the shore-based e-Navigation system architecture.  These two tasks are continually performed as part of the life-cycle management of the shore-based e-Navigation system architecture.

- The e-Navigation concept is envisaged over a ship’s complete voyage from berth-to-berth.  Therefore, the e-Navigation concept leads to a high degree of commonality of the shore-based e-Navigation system architectures of different national competent authorities.  Also, to exchange relevant data between shore authorities involved in that berth-to-berth voyage of the ship, interoperability of the shore-based e-Navigation systems of different national competent authorities is needed.  Both require a certain degree of international standardisation for operational requirements, system architecture considerations, and human-machine- as well as machine-to-machine interfacing.

Common service levels and standardised minimum capabilities are implied for all of the three mutually interacting parts of IMO’s e-Navigation concept (refer to section “the IMO’s e-Navigation concept” above).

# The application-to-application information/data flow

The key element of the e-Navigation concept is considered in terms of information/data flow between applications, which constitute sources and sinks for information, or both simultaneously.

In addition, storage of data over longer periods of time together with the retrieval of that data is part of the e-Navigation concept.

To achieve that end, user or operational requirements and their associated information needs can be represented in terms of appropriate state-of-the-art data models together with abstract descriptions of the processes and of the functionalities (including interactions) which are required to properly process the data to arrive at meaningful information for the stakeholders involved.  For a precise definition of information compared with data refer to the glossary.

Furthermore, the co-operation in information/data exchange ship to ship, ship to shore, shore to ship, shore to shore and other users is at the core of the e-Navigation architecture.  Hence the co-operative nature is a core value of the e-Navigation concept.  By the same token, some technical e-Navigation services are called co-operative services.

There are also so-called non-co-operative technical e-Navigation services, which for the ship-shore/shore-ship information flow do not require a specific shipboard device (e. g. radar detection in the information flow direction ship-to-shore or visual Aids-to-Navigation in the information flow direction shore-to-ship).  That means that information may be derived without the contribution of an active component on the other side.  These non-co-operative technical services are an integral part of the e-Navigation concept.

It is appropriate, when considering abstract flow of data between applications, to use the term of functional links between participating applications.

Following from the general requirement for appropriate standardisation, the interfaces of the entities involved as well as their protocols and encoding techniques used throughout the information flow chain should be standardised.

The above abstract statements constitute the core values of the e-Navigation concept in analysis and design.  Eventually the data exchange and the processes and functionalities using that data need to be manifested in physical links, e.g. a physical communication link, and physical entities and devices, e. g. computer systems.  Hence, requirements and limitations stemming from that physical world will have their impact on the e-Navigation system architecture.

Finally, the different dimensions of application-to-application data flow need to be considered to understand the e-Navigation concept.  These different dimensions include the following:

- Location of the participating applications: on board ships, on board floating AtoNs, onboard aircrafts interacting with the maritime community and on shore, etc.  This dimension highlights the requirement of a high connectivity.

- Distributed responsibilities of stake holders, e.g. different authorities ashore operating in the same area, but with different tasks.

- The large variety of the nature and amount of data to be exchanged between applications will result in a large variety of appropriate technologies to be considered.

- Quality requirements: The attributes of usability, accuracy, integrity, reliability or availability, continuity, time behaviour, maintainability, security, confidentiality etc. need to be defined when analysing the operational requirements (user requirements).

# From e-Navigation concept to a technical e-Navigation architecture

## The technical point of view - introduction

In the previous chapter e-Navigation was described as a concept in operational and management terms.  In order to implement the e-Navigation concept a technical architecture need to be developed and deployed.  This technical architecture needs to fulfil both the operational requirements and the management goals simultaneously.

The derivation of the technical architecture was developed following the so-called “spiral approach” in system engineering terms: Based on an initially defined and solid set of operational requirements (user requirements) an initial technical architecture is defined.  This may have some influences on the operational requirements, which in turn are revised accordingly.  Also new operational requirements may appear and may be accepted.  The technical architecture is then again adapted and migrated based on the then improved set of defined and solid operational requirements.  This process is reiterated as often as needed.

After every cycle, the technical solution will be documented in a way, as if originally derived from the operational requirements in a seamless and transparent, top-down development process.

To support the above processes it is essential for an IALA member to have a well established life-cycle management in place throughout the above process.

As e-Navigation affects existing systems, it is essential to consider legacy issues and develop and implement appropriate migration strategies.

It is evident that due to the overarching and complex nature of the e-Navigation concept, the technical solution is not a simple one.  By means of introduction, several steps are required to arrive at a suitable technical solution.  The first steps are being taken in this IALA Recommendation, while consecutive IALA Recommendations will build upon this.

A first step is to look at the e-Navigation architecture from a technical point of view, i.e. correlate the abstract e-Navigation concept with some well known technical concepts, systems and components while keeping in mind the whole picture and preserving the abstract qualities of the e-Navigation concept.  This step is examined in this chapter.

## The e-Navigation architecture

All the aspects introduced in the preceding chapters plus some others to be introduced need to be considered when developing the e-Navigation architecture.

One resulting representation of the e-Navigation architecture is shown in Figure 1.  The shipboard entities, the physical link(s) and the shore-based entities are included in this representation of the e-Navigation architecture.

On the left within Figure 1 is represented, for simplicity’s sake, a single “ship technology environment”.  From the e-Navigation concept’s perspective the relevant devices within the ship technology environment are the transceiver station, the data sources and the data sinks connected to the transceiver station, the Integrated Navigation System (INS) and the Integrated Bridge System (IBS).  The transceiver station is shown as a single station for simplicity’s sake, although there may be several transceiver stations.

The transceiver station interfaces with the physical link(s) to the appropriate technical e-Navigation services ashore. Examples of physical links are introduced in the next section.

The shore-based technical e-Navigation services, in their totality and by their interactions, provide the interfaces of the shore-based user applications to the physical link(s).  They also encapsulate their technology to the whole of the common shore-based e-Navigation system architecture.  The encapsulation principle hides the technology’s sophistication from the shore-based e-Navigation system as a whole and thus reduces complexity.  In Figure 1, the entities which are involved with the specifics of the link technology are confined by the dotted line. Amongst other benefits, it allows for parallel work of the appropriate experts in the particular technology of a given physical link, provided the functional interfaces of the shore-based technical e-Navigation services are well defined.

For the precise technical structure of the shore-base technical e-Navigation services see the common shore-base e-Navigation system architecture in IALA Recommendation (e-Nav 0201).

The shore-based technical e-Navigation services use data transfer network(s), which distribute(s) the data used by the shore-based technical e-Navigation services.  The requirements for data transfer are dependent on the specifics of the technology of the technical shore-based e-Navigation service.  For example, a shore-based Radar Service may require different data transfer network capabilities than a shore-based AIS Service.  Hence, the individual requirements for data transfer network of a shore-based technical e-Navigation service are also encapsulated.  This also reduces complexity.

The operators ashore, e. g. VTS operators, pilot station operators, and lock operators, need the user applications to perform their tasks in co-operation with shipboard applications.  From their perspective, it is neither the physical links nor the shore-based technical e-Navigation services that matter – they have been encapsulated for that reason.  It is the functional links between the shore-based user applications at hand and the shipboard applications which matter to the users on both sides.  A similar setup of interactions applies for ship-to-ship and shore-to-shore applications.

Figure 1 shows IMO’s World Wide Radio Navigation System (WWRNS), which includes GNSS, being presented as a system “external” to the e-Navigation architecture providing position and time information.  A further discussion of that dependency is contained in a chapter below.

In Figure 1 the also the so-called Universal Maritime Data Model was introduced.  This is because IMO specifically requested it, as follows:

“Mariners require information pertaining to the planning and execution of voyages, the assessment of navigation risk and compliance with regulation.  This information should be accessible from a single integrated system. Shore users require information pertaining to their maritime domain, including static and dynamic information on vessels and their voyages.  This information should be provided in an internationally agreed common data structure.  Such a data structure is essential for the sharing of information amongst shore authorities on a regional and international basis.”  [IMO MSC85 Annex 20 Section 8.2.1]

The features of the UMDM will be explained in the following section.

UMDM

Ship technology

environment

IBS

World Wide Radionavigation System (WWRS) of IMO, including GNSS, GNSS augmentation and terrestrial backup

Appli-cation 1

Application 2

Application 3

Application 4

Link technology (encapsulated)

Transceiver station

Ship’s

sensors

Shipboard

Applications

INS

technical

e-Nav

ser-vices

Application-to-

application

functional

connection

Physical

Link (e.g. radio link)

other

ships

other

ships

mariner

Shore-based

users

- e-Navigation Architecture

## The importance and the features of the Universal Maritime Data Model

In Figure 1 also the Universal Maritime Data Model (UMDM) is introduced:

- The UMDM is an abstract representation of the maritime domain.  Specifically, it represents the entities and relationships among the entities that exist in this domain but does not represent processes.The purpose of a data model is to give users of the UMDM, i.e. (software) engineers, a common understanding of the entities and relationships.

- The UMDM is neither a database nor an interface.  The model contains no details about physical representation of the entities within it.  However, the UMDM can be used to guide the development of databases and interfaces, which are the physical representations of entities. By defining a common reference, all implementers can be assured that their understanding of the entities required agrees with the understanding of others.

- The UMDM is flexible and extendable for meeting future requirements.  New entities can be added to the model by any e-Navigation stakeholder through a process known as registration.  Once registered, an entity is available to all stakeholders; there is open, public access.  The UMDM also has provisions for proprietary entities that do not require registration.  The model is modular and an appropriate subset is easily referenced.

- The UMDM can represent any maritime entity, can be extended by the addition of new entities, and is accessible to any e-Navigation stakeholder or implementer, thus it is considered to be universal in the maritime domain.

Figure 2 illustrates how the UMDM will be created – based on user requirements – and how it will influence the components of the e-Navigation architecture when used by (software) engineers.

UMDM

Ship

Technology environment

Shore-based e-Navigation system

WWRNS

Physical Link technologies

User requirements

User requirements

Templates + methodology

Templates + methodology

- Scope and Impact of the Universal Maritime Data Model

## Functional and physical links

### The relationship between functional and physical links

The bold arrow in Figure 1 is a graphical symbol for the functional connections for data exchange between the shore-based applications and the “ship’s technology environment”, and vice versa.  In IT terms the originating entity of data in a functional link is called “source”, while the receiving entity of that data in the same functional link is called “sink”.  The physical path of the data exchange uses the physical links and the various physical interfaces between the shore-based applications and the “ship’s technology environment”, as indicated by the small arrows.  The functional connection is the abstract statement in regard to requirement analysis for the application-to-application data exchange.  The physical path may take a completely different and more sophisticated route.  To use the abstraction of the functional connection hence facilitates the requirement analysis.

### Relevant physical link technologies

Physical links between (fixed) shore and (mobile) shipboard equipment each employ one or more appropriate mediums such as radio waves or light.

Figure 3a shows a variety of physical link technologies which are relevant for the e-Navigation architecture.  It also shows that well-known technologies such as VTS radar can be modelled as a physical link: the data exchanged is the ship’s radar echo signature.  Figure 3b shows that visual or traditional Aids-to-Navigations (either fixed or floating) can also be modelled in the same way.  In fact, “visual” Aids-to-Navigation may employ a variety of state-of-the-art physical link technologies simultaneously, not just light.

Every technical e-Navigation service of the shore-based e-Navigation system architecture employs at least one physical link technology.

While Figure 3 addresses the ship-shore/shore-ship links, it is understood that there are functional and physical connections between ship-ship and shore-shore as well.

- A Variety of physical link technologies relevant for the e-Navigation architecture

## IALA’s role applied to the e-Navigation architecture

All Aids-to-Navigation can be represented digitally allowing full evaluation and planning of Aids-to-Navigation with respect to e-Navigation.  IALA members have responsibility for marine Aids-to-Navigation, including VTS.  IALA’s role regarding the e-Navigation architecture can be derived in terms of the entities involved.  Based on IALA’s existing mandate, IALA would thus concern itself with:

- the shore-based aspects in general, including the definition of the relevant shore-based user requirements, as well as training aspects of shore-based personnel;

- the relevant shore-based user applications, e. g. VTS, electronic Aids-to-Navigation (e-ANSI), and inland waterways applications;

- the variety of individual shore-based technical e-Navigation services, such as the Radar Service, the AIS Service, the Visual Aids-to-Navigation services etc;

- the link technologies:

  - The AIS is a point in case: ITU has requested IALA to maintain the AIS VHF data link (compare Recommendation ITU-R M.1371).

  - Another point in case may be the re-engineering of existing radio navigation systems, such as the MF DGNSS system (IALA Radio Beacon system), with the goal to maximise the data transfer capacity of that radio link.

  - Yet another point in case may be the co-ordinated development and use of optimised existing and genuinely new radio communication systems shore to ship/ship to shore, as well as the development of the IALA Maritime Radio Communication Plan (MRCP).

  - IALA encourages the use of link technologies that are recommended by its Maritime Radio Communication Plan.

- the definition of certain shipboard transceiver stations, since they cannot be treated independently from their associated physical link technologies, which are under consideration at IALA.

  - Again AIS is a point in case: IALA initiated and still takes an active role in the definition of mobile AIS stations, which were or are still under consideration at IEC.

  - Also, IALA was involved in the definition of a type-approved shipboard MF DGNSS correction receiver at IEC.

- the dependency of the e-Navigation on the World Wide Radio Navigation System (WWRNS): To that end, IALA members contribute in terms of terrestrial augmentation and/or backup systems, such as the IALA developed MF DGNSS system.  The development of the IALA World Wide Radio Navigation Plan (WWRNP) is another point in case.

- the development of the IALA UMDM as  a consistent and unique data model:

To that end, IALA will publish a suite of recommendations for the definition and use of the IALA UMDM.  The UMDM requires a governing body to ensure proper management of the model.  There may be several management roles to be performed by the governing body, one of which is the Registrar.  The different management roles may be delegated at the discretion of the governing body.  Hence, the Registrar may not be the same organization as the governing body.  The tasks of the governing body always comprise supervision for the Registrar, however.

Initially, IALA will operate as the governing body and Registrar for the IALA UMDM.  Once brought to a certain degree of maturity, the IALA UMDM would be offered to IMO as a contribution to the implementation of IMO’s e-Navigation strategy.  Then IMO would assume the role as the governing body for the UMDM, thereby guaranteeing the uniqueness of the UMDM within e-Navigation.  IALA is prepared to continue as the Registrar for the IMO UMDM.  There is precedent for this within IMO, e.g. the registrar for a ship’s IMO number is Lloyd’s Register.

# The common shore-based e-Navigation system architecture

## Introduction

The previous chapter introduced the first step of how to arrive at a technical solution for the quite abstract e-Navigation concept, starting from user requirements by top-down seamless derivation and taking also into account the management goals stated above.  Also, the role of IALA was introduced in this regard, thus introducing a specific shore-based point of view.

Also, an e-Navigation architecture was described in more technical terms, part of which are well-known, part of which are necessary by the e-Navigation concept.  The e-Navigation architecture consists of three main columns:

- Shipboard equipment and shipboard applications

- Shore-based e-Navigation system, comprising its shore-based technical e-Navigation services, and shore-based user applications;

- Links ship-to-shore/shore-to-ship as functional links between applications and services, supported by physical link technologies.

Thus, certain important concepts of the e-Navigation architecture have been introduced, from a technical point of view.

This chapter focuses on that shore-side system architecture, i.e. this chapter focuses specifically on the second of the above three main columns.  To that end it necessarily takes also into account the e-Navigation architecture as a whole as introduced in the previous chapter.

Since the e-Navigation concept mainly is about information flow as introduced above, the shore-based system architecture under consideration in this chapter uses extensively Information Technology (IT) concepts and terms.

The common shore-based e-Navigation system architecture introduced in this chapter can be used to support a variety of Human Machine Interfaces (HMI) to different shore-based users.

## Information/data flow, application interactions, and user interfaces

When considering the e-Navigation architecture, one should think in terms of information/data flow, application interactions, and user interfaces, as follows:

User requirements can be described in terms of their respective information needs for their tasks to perform.  The required information items are delivered at the Human Machine Interfaces of the user applications by the shore-based e-Navigation system as fulfilment of the user requirements. These information items are transmitted, stored, and processed as data objects by the shore-based e-Navigation system.  They are exchanged using the functional links between applications, both shipboard and ashore.

From the above it follows, that when there is an information flow between users and applications, there is always a parallel data flow associated with it.

The following Table summarizes the above relationships.  For the precise definitions of information vs. data compare the glossary.

- Derivation of information needs, information objects and data objects for technical services from user requirements

  - User requirements in general (top level) 	(information domain)

  - Tasks of the user 					(information domain)

  - Stated information needs				(information domain)

  - Individual information objects 			(information domain)

__interaction by Human-Machine-Interface (Operational Presentation Surface)__

  - Maritime data objects  				(data domain)

  - Maritime data encoding-for-data exchange 	(data domain)

Figure 4 shows the complete e-Navigation architecture while the structural details of both the technical shipboard and shore-based e-Navigation system architectures are not yet shown.

- The place of the shipboard e-Navigation system architecture within the e-Navigation architecture – simplified presentation

- The e-Navigation architecture in terms of information flow, application interactions, and user interfaces

In Figure 5 the following graphical symbols are used:

- Interfaces between entities involved are symbolised by a little circle and a line leading to the system which provides the interface. The interface as such belongs to the system which provides it.

- Technical systems are symbolised by cubes, which expresses the encapsulation principle explained above (“black boxes”).

- Human-Machine-Interfaces, indicated by the acronym HMI, and Machine-Machine-Interfaces, i.e. mainly IT or computer interfaces, indicated by the acronym M2M.

- The arrows with the dotted lines indicate requirements, which are put forward by the entity at which the dotted line starts.

- Requirement arrow(s) always point to interfaces (circles). This is to indicate, that the interface(s) fulfil the requirement(s).

The following conclusions can be drawn from Figures 4 and 5:

- When designing and describing e-Navigation applications, they should be described as interactions between applications using the information/data flow concept:

- (Ultimate) sources of data should be identified together with the data objects stemming from those sources, i.e. the source data.

- Likewise, (ultimate) sinks for data should be identified, together with the data objects required by a particular sink in the information flow, i.e. recipient data.

- When there is intermediate processing of data objects involved, the appropriate algorithms should be stated.

- For all data objects should be described their relevant attributes or properties, including their constraints (such as permissible min/max values).

- Figure 4 and 5 also represents a statement regarding the distribution of responsibilities between operational stake-holders and engineers:

- Operational stake-holders state their information needs and the format (symbolised in Figure 5 by dotted arrows) at the user interfaces of the shore-based e-Navigation system.

- Engineers analyse these information needs and take into account the management goals, in particular considering the life-cycle-management aspects of any system or component. The result of their analysis is an engineering-like representation.

- This engineering-like representation is described in one or more stand-alone documents. Thus, it can and should be verified and validated by operational stake-holders and management. This engineering-like representation constitutes a kind of “Service Level Agreement (SLA)” between the operational stakeholders and the engineers, i.e. a Service Level Agreement regarding a technical service delivered to the user interfaces.

- Engineers will eventually provide the appropriate technical Human-Machine-Interface (HMI), which fulfils the stated information needs and the stated format requirements.

- A Human-Machine-Interface (HMI) can be any kind of appropriate combination of displays, keyboards, voice interfaces (microphone / loudspeaker), and other human interaction devices. The suite of those devices at one operational working position is called the Operational Presentation Surface (OPS). This comprises the concepts of modern IT human interfacing technologies, such as window techniques.

- The distribution of responsibilities is reflected in an appropriate documentation framework. As far as IALA documentation is concerned, this is further developed in the last chapter.

It should also be noted, that Figure 4 returns the usage of the term “VTS” completely to the operational realm.

## The functional approach for the common shore-based e-Navigation system

The stated information needs as described above, can then be translated into documentation of data objects and their relationship, of the functionalities of technical e-Navigation services and their interaction, and eventually of component specifications, using the appropriate engineering analysing methodology.

The shore-based technical e-Navigation services supporting the information needs of the shore-based users in turn require certain physical links (“signal-in-air”) and certain shipboard devices in most cases.  Similarly, a requirement chain exists for the shipboard user.  (Note: These statements address the requirement chains as opposed to information flow chain.)

While doing this the management goals in this process, such as life cycle management requirements, need to be taken into account as well.

There are state-of-the-art engineering methodologies available to facilitate that work, such as the Object-oriented Engineering Process (OEP) and the use case methodology.  That methodology supports IMO’s e-Navigation concept.

The most important part of the paradigm of the common shore-based e-Navigation system is the Object-oriented Engineering Process (OEP) (see Figure 6 below).  It describes the methodology applied to the engineering task at hand in the following step-by-step process.

- List and specify all stated information needs and the associated information items for the common shore-based e-Navigation system as a whole.

- List and specify the internal requirements for the common shore-based e-Navigation system as a whole from a management and technical point of view, such as life-cycle-management requirements and resources usage requirements.

- Use engineering methodologies to do an engineering analysis of requirements to the shore-based e-Navigation system

The well attested use case approach provides such engineering methodology. It is a point in case that IMO defines e-Navigation in accordance with the use case concept: “e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore (…).”

  - The five keywords highlighted above describe what tasks are to be performed with maritime information, are indeed use case designations.

  - It should further be noted, that IMO’s definition explicitly discerns the use cases for the shipboard side and the shore side (also highlighted above).

  - Hence, it can be concluded that IMO’s definition of e-Navigation is fully compatible with state-of-the-art engineering methodologies for requirement analysis, and vice versa.

- Derive the essential system requirements by exploitation of similarities between stated information needs of different users and also of internal requirements as identified above.

- Design a common shore-based e-Navigation system layout predominantly using individual technical e-Navigation services as building blocks.

- For each and every essential system requirement identify the interactions of the relevant individual technical e-Navigation services needed to fulfil that specific essential system requirement.

- Draft a precise functional description of an individual technology as a technical e-Navigation service in accordance with a generic e-Navigation service model.

- Derive all component requirements for components of an individual technical e-Navigation service.

- Capture all the above descriptions in a documentation methodology and prepare for the submission to quality management audits.

- Engineering analysis of requirements to the shore-based e-Navigation system

## The shore-based e-Navigation system layout

The shore-based e-Navigation system layout is described in detail in IALA Recommendation e-NAV 201.

## Migration of legacy systems

Legacy systems were not developed with a-priori knowledge of the e-Navigation architecture.  During a transition period both services and systems developed in accordance with the common e-Navigation system architecture as described in this and consecutive IALA recommendations, guidelines and manuals as well as legacy services and systems will co-exist.

The ultimate goal of IALA National members should be to arrive at the common shore-based e-Navigation system architecture.  Therefore, some migration of those legacy systems towards the e-Navigation system architecture may be required.

During the above mentioned period of co-existence of the emerging common shore-based e-Navigation system and the legacy systems there must not be a threat to the established level of safety and of efficiency and of the protection of the environment.

## Life-cycle management considerations

### Overview and introduction

Considering the rather complex nature of the e-Navigation architecture and the many technologies involved, IALA encourages its members to apply state-of-the-art life-cycle management techniques for both the information/data and the systems involved.  More details and references on recommended life-cycle management practices can be obtained in the future IALA document “Life-Cycle Considerations” (outlined in Figure 9 in the next chapter).

The two sections below outline some important aspects of life-cycle management in regards with the e-Navigation architecture for both the information/data and the systems involved.

It should be noted, that any life-cycle management consideration is orthogonal to the plane in which the information/data and / or the systems involved have been considered so far.  The following considerations take place on a meta-level, and are concerned with the development of the information/data and / or the systems involved over a longer period of time of their existence, i.e. their life-cycles.

The figure below (Figure 8) outlines the life-cycle management of the systems and processes that support the e-Navigation architecture.  It shows the already introduced e-Navigation architecture (Figure 5 in preceding chapter) as a plane supported by additional data processing systems and data producing processes.  These systems and processes will evolve in time in order to keep in line with the evolution of the e-Navigation architecture and its components.

It is expected that any change in the e-Navigation components will result in some changes in the underlying systems and processes but would not necessarily affect the whole chain every time.  This is represented by the small time arrows where moments in time require different changes to the associated systems and/or processes.

- Life-cycle management of systems and processes supporting the e-Navigation architecture

### Information/data life-cycle management

Information/data life-cycle comprises five phases:

Phase 1: Production of a data object (creation and receipt)

Phase 2: Distribution

Phase 3: Use

Phase 4: Maintenance (and storage)

Phase 5: Deletion

The (IALA) UMDM data objects must undergo all of the information life-cycle management phases and provide appropriate structures for dealing with each phase throughout the UMDM object’s life.

IALA members should be aware that phase 1 (production), is expected to require particular attention to ensure that the data produced is done so in a consistent and predictable manner, compliant with the definitions of data objects contained in the UMDM. This will most probably require an on-going review of methods, procedures and tools used to produce this data, as depicted in Figure 7 above.

It should also be noted that the production of data often requires other data input in the process as well as user input. Data input into the production process would come from the live e-Navigation system. A UMDM-compliant process must be applied to produce the data objects, and in particular data source duplication needs to be avoided. Figure 7 above also illustrates this concept.

In order to ensure proper life-cycle management and avoid confusion on the production and usage/interpretation of the data, proper metadata has to be provided as part of the UMDM.

### Technical e-Navigation services and components life-cycle management

As stated above, the technical e-Navigation system architecture will comprise of many technical e-Navigation services and components contributing to the overall objectives.  IALA recommends applying a life-cycle management process to all of those technical e-Navigation services and components to reduce the total cost of ownership and ensure appropriate compatibility with the common shore-based e-Navigation system architecture (see Figure 7).

For more details on recommended life-cycle management practices related to the technical e-Navigation service refer to the future IALA document “Life-Cycle Considerations” (outlined in Figure 8 in the next chapter) as well as the Information Technology Infrastructure Library (ITIL) documents for best practices on maintaining IT.

# Dependency on external systems and on infrastructure

## Dependency on GNSS for position and time

As indicated in Figure 1, there is a dependency of the e-Navigation architecture on external systems for position fixing and for timing. This may pose a certain vulnerability for all e-Navigation applications, since dynamic position information is involved in most and time information is required in every. Hence, mitigation methods are important.

## Dependency on infrastructure

Though functionally apparently “invisible”, the supporting infrastructure as a module internal to the common shore-based e-Navigation system architecture is important for the proper and efficient function of the e-Navigation applications. Hence, the dependency of the common shore-based e-Navigation system on its infrastructure.

The infrastructure consists of supporting devices, which e.g. allow application software to be executed (i.e. computers, computer periphery, operation systems, local area network components, etc.), provide a reliable energy supply, provide temperature control, provide housing for components, protection against harmful interference, tampering, unauthorised access, environmental conditions, etc.

Infrastructure becomes more and more sophisticated and in itself subject to increasing requirements.  Reduction of energy consumption, environmental friendliness, centralised management, maintenance cost, staffing level, staff responsibilities, and reliability level are just a few key words in this regard.

When setting up a shore-based e-Navigation system, the infrastructure should be planned carefully to mitigate harmful consequences of that dependency.

# IALA e-Navigation documentation framework

Considering the complexity of the e-Navigation approach and the complexity of the different technologies involved, a successful implementation of the e-Navigation strategy requires a well-structured documentation framework at IALA, which is organized in accordance with the principles of the e-Navigation, hence “e-Navigation compliant”.

While IMO as the governing body for the e-Navigation strategy will develop relevant documents for e-Navigation, such as resolutions, circulars and performance standards, IALA’s own documentation (recommendations, guidelines, manuals) needs also to be arranged in a documentation framework that reflects the principles and methods set out in this recommendation. For an overview compare Figure 8.

This documentation framework will provide a proper place also for existing IALA documents as soon as they will have been reviewed by IALA, taking into account the e-Navigation principles and methods.

This recommendation fits into the IALA e-Navigation documentation framework as indicated in Figure 8 (highlighted).  The details of the IALA e-Navigation documentation framework will be defined in a future IALA Recommendation on “Table of Contents for ‘e-Navigation compliant’ IALA Documentation” (compare also Figure 8).

| Recommendations, Guidelines, and Manuals on Generalities of the shore-based e-Navigation system architecture |  |  |  |  |
| --- | --- | --- | --- | --- |
| Table of Contents for “e-Navigation compliant” IALA Documentation |  |  |  |  |
| Shore-based e-Navigation system terminology (= glossary) |  |  |  |  |
| Recommendations, Guidelines, and Manuals on e-Navigation conceptional aspects and user requirements viewed from ashore |  |  |  |  |
| 0101: The e-Navigation Architecture – the initial Shore-Based Perspective (= this Recommendation) |  |  |  |  |
| e-Navigation system requirement analysis and e-Navigation applications descriptions: arranged in fields of e-Navigation applications |  |  |  |  |
| Recommendations, Guidelines, and Manuals on technical generics of the common shore-based e-Navigation system architecture |  |  |  |  |
| common shore-based e-Navigation system architecture |  |  |  |  |
| technical e-Navigation services (engineering model) |  |  |  |  |
| The Universal Maritime Data Model and the data model of the common shore-based e-Navigation system architecture |  |  |  |  |
| Interface considerations with shore-based systems and between shore-based systems |  |  |  |  |
| On-site Infrastructure considerations |  |  |  |  |
| Life-Cycle-Management Considerations: Technical Operation Personnel + Tasks/ Technical Development Personnel + Tasks |  |  |  |  |
| Quality Management and Certification Considerations |  |  |  |  |
| Recommendations, Guidelines, and Manuals on shore-based e-Navigation applications mapping to e-Navigation services involved - examples |  |  |  |  |
| Shore-based e-Navigation applications involving visual Aids-to-Navigation |  |  |  |  |
| Shore-based e-Navigation applications involving VTS |  |  |  |  |
| … |  |  |  |  |
| Recommendations, Guidelines, and Manuals on specific Technical e-Navigation services – examples |  |  |  |  |
| AIS Service | Radar Service | Visual AtoN Service | User Interaction Service | … other technical e-Navigation services … |

- Future IALA e-Navigation documentation scheme

# Glossary of terms

e-Navigation concept: a concept devised to provide a comprehensive, standardised ship-to-ship, ship-to-shore, shore-to-ship and shore-to-shore information exchange environment, including the appropriate data exchange and data processing features.

e-Navigation architecture: a well-structured description of all the entities involved in the e-Navigation concept.  The entities to be taken into account by the e-Navigation architecture are users and their requirements, regulations, functions and processes, technical services and systems, interactions, information and data.

“e-Navigation compliancy”: While there is presently no direct definition of “e-Navigation compliancy” or of an “e-Navigation compliant” operational or technical service or device provided by IMO, the working and therefore tentative definition can be inferred from the IMO e-Navigation strategy.  “e-Navigation compliant” would mean that an operational or technical service or device has been proven, tested, or checked by a competent body to be in conformity with relevant IMO performance standards, which were expressively created or revised as part of the implementation of IMO’s e-Navigation strategy.

Common shore-based e-Navigation system architecture: a subset of the e-Navigation architecture, composed of only the shore-based part, which is exploiting international commonalities of such shore-based systems of IALA national members.

Operational e-Navigation service:  An operational e-Navigation service, or Operational Service for short, is an activity or a combination of activities, provided by one or more stakeholders within the maritime domain, for the benefit of maritime stakeholders to support their mission at all levels, which has been harmonized and brought into accordance with the goals of IMO’s e-Navigation concept. An operational service is characterized by providing information to users.  It is such distinguished from a technical e-Navigation service, which is providing data.  Well established examples of operational e-Navigation services are: Traffic Information Service, Traffic Organisation Service, and Navigational Assistance, as part of the Vessel Traffic Services (IALA VTS Manual 2008).

Technical e-Navigation service: a fundamental building block of the common shore-based e-Navigation system.  For the technical e-Navigation service there is a generic, engineering model without reference to a specific physical implementation or a specific technology, which exploits the architectural commonalities of various technologies.

Individual technical e-Navigation service: a technology-specific technical e-Navigation service like the AIS Service, the Radar Service, the DGNSS Correction Service, the User Interaction Service, etc.

Functional link: the functional implementation of a communication link between two e-Navigation entities.

Physical link: the physical implementation of a communication link between two e-Navigation entities.

Application interactions: describes communication from the originator/sender (data ion cation interactions regarding discrimator for defining a service]55] note to IALA 00000000000000000000000000000000000000000source) to the recipient (data sink), without reference to the communication medium(s).

Information (item): “knowledge concerning objects, such as facts, events, things, processes or ideas, including concepts, that within a certain context has a particular meaning” (International Standardisation Organisation. 2002. Ships and marine technology – data transfer standard for maritime and intermodal transportation and security. ISO/PAS 16917:2002(E)).

Data (object) “representation of facts, concepts or instructions in a formalized manner, suitable for communication, interpretation or processing by humans or by automatic means” (International Standardisation Organisation, 2002.  Ships and marine technology – data transfer standard for maritime and intermodal transportation and security.  ISO/PAS 16917:2002(E))

Metadata: Metadata by definition is simply "data about data", information about the objects stored within e.g. the UMDM, whether these are in traditional or electronic formats.  In library science, catalogue records are metadata, as they contain data about the library's collection of media items, i.e. the books, journals, and electronic resources that make up its collections.  Metadata records in the traditional library fulfil several functions, including allowing users to find items, allowing them to assess their usefulness, and to allow librarians to administer them correctly.  The same principles apply to objects within the digital library.

Due to that development it is strictly speaking a misnomer to use the word “visual” aids-to-navigation.  Since “visual” aids-to-navigation is a well-known term, it will be used throughout the description of the e-Navigation concept and the common shore-based e-Nav system architecture.

Data objects are defined by the UMDM, and the UMDM eventually consists of data objects and their relationships.

Vessel Traffic Services” (VTS) strictly speaking is an operational term, as the term implies.  It comprises the three operational services: Traffic Information Service, Traffic Organisation Service, and Navigational Assistance (IALA VTS Manual 2008).  Over time, the term VTS came to be used as a synonym for the technical system, which supports the above operational services.  Common usage was to consider a “VTS” as a well defined set of certain technologies, in particular radar and VHF voice communication. When referring to the technical equipment needed to support the Vessel Traffic Services, the technical term VTS System should be used.

Note: This phrase was generalized using IALA’s VTM concept; VTS29-output-8=eNAV6-8-21.