# Draft IALA Contribution to IMO Correspondence Group on e-Navigation

This document is a contribution from IALA to the IMO Correspondence group on e-Navigation (CG) and is used to inform the CG about IALA’s work on e-Navigation Architecture.

Chapter 2 - “Initial technical e-Navigation Architecture”

# IMO’s understanding of e-Navigation Architecture

IALA has started the “Initial technical e-Navigation architecture work” based on the following understanding of IMO’s e-Navigation concept in general, and the e-Navigation architecture specifically using the following quotations taken from IMO documents:

- In order to have the e-Navigation concept implemented, the architecture is a required step [“Work programme - coordinated approach to the implementation of the e-Navigation strategy”, MSC 86/23/4, as adopted by MSC86, §5]:

“In order to prepare the strategy implementation plan several steps are required, such as capturing evolving user needs, developing an architecture and undertaking a gap analysis, a cost-benefit analysis and a risk analysis. These elements should provide a common and informed foundation for the creation of a detailed strategy implementation plan.”

- The IMO is also specific regarding the functions and components of any technical e-Navigation architecture [“Work programme - coordinated approach to the implementation of the e-Navigation strategy”; MSC 86/23/4, as adopted by MSC86, §7]:

“The overall conceptual, functional and technical architecture will need to be developed and maintained, particularly in terms of process description, data structures, information systems, communications technology and regulations. The architecture should include the hardware, data, information, communications technology and software needed to meet the user needs. The system architecture should be based on a modular and scalable concept. The system hardware and software should be based on open architectures to allow scalability of functions according to the needs of different users and to cater to continued development and enhancement.”

Further, IALA has undertaken this work, not only for its own needs, but at the invitation of IMO as stated in the following IMO documents:

- The above mentioned IMO document “Work programme - coordinated approach to the implementation of the e-Navigation strategy”; MSC 86/23/4, as adopted by MSC86, contains several times the invitation to IALA to contribute [§16, and Annex, columns entitled “external input”].

- IMO NAV55 identified in particular the contribution of IALA to the “Initial e-Navigation Architecture” [IMO NAV55/WP5 Report, Agenda item 11, §6.1] and [IMO NAV55/WP8 Report, §11.34]:

“The group also noted that development of system architecture had taken place in the interim period within IALA. Accordingly, the group invited IALA to provide the results of these developments to the correspondence group”.

# Overview

## The overarching e-Navigation architecture

This document provides an introduction to the IALA developed Initial technical e-Navigation architecture. There has been a long standing responsibility for architectural considerations of shore-based systems within IALA’s existing mandate. However, IMO’s e-Navigation concept adds new dimensions to those architectural considerations.

The following sections introduce important principles of the IALA e-Navigation architecture, such as the orientation towards information flow instead of technology development. In particular a number of concepts are introduced, namely

- The Universal Maritime Data Model,

- Service Portfolios,

- Aspects of the architecture (ship to ship, ship to shore & shore to ship,  shore to shore)

The document then goes on to give an introduction to the IALA developed “Initial technical e-Navigation architecture” in some more detail. Finally it concludes by giving a proof-of-concept example as a validation for the Initial technical architecture design.

The scope of this introduction to the IALA developed “Initial technical e-Navigation architecture” is very well represented by Figure 1 below, which shows the shipboard entities, the physical link(s) and the shore-based entities.

--- consider transforming the Figure 1 into a “portrait style” figure on this page --

UMDM

UMDM

Ship

environment

IBS

World Wide Radionavigation System (WWRNS) of IMO, including GNSS, GNSS augmentation and terrestrial backup

Transceiver station

Ship’s

sensors

Shipboard

Applications

INS

Shore-based

technical

e-Navigation

services

(= shore-based

applications)

Application-to-

application

functional

connection

Physical

Link (e.g. radio link)

other

ships

other

ships

mariner

Shore- based users

(e.g. VTS Operators)

Shore

environment

Figure 1: e-Navigation architecture

On the left within Figure 1 is represented, for simplicity’s sake, a single “ship environment”. From the e-Navigation concept’s perspective the relevant devices within the ship environment are the transceiver station, the data sources and the data sinks connected to the transceiver station, the Integrated Navigation System (INS) and the Integrated Bridge System (IBS). The transceiver station is shown as a single station for simplicity’s sake, although there may be several transceiver stations.

The transceiver station interfaces with the physical link(s) to the appropriate technical e-Navigation services ashore.

The shore-based technical e-Navigation services, in their totality and by their interactions, provide the interfaces of the shore-based user applications to the physical link(s). They also encapsulate their technology to the whole of the common shore-based e-Navigation system architecture. The encapsulation principle hides the technology’s sophistication from the shore-based e-Navigation system as a whole and thus reduces complexity. Amongst other benefits, it allows for parallel work of the appropriate experts in the particular technology of a given physical link, provided the functional interfaces of the shore-based technical e-Navigation services are defined in accordance with the requirement for open system architecture.

The operators ashore, e. g. VTS operators, pilot station operators, and lock operators, need the user applications to perform their tasks in co-operation with shipboard applications. From their perspective, it is neither the physical links nor the shore-based technical e-Navigation services that matter – they have been encapsulated for that reason. It is the functional links between the shore-based user applications and the shipboard applications which matter to the users on both sides. A similar setup of interactions applies for ship-to-ship and shore-to-shore applications.

Figure 1 also shows IMO’s World Wide Radio Navigation System (WWRNS), which includes GNSS, being presented as a system “external” to the e-Navigation architecture providing position and time information. A further discussion of that dependency is contained in a chapter below.

In Figure 1 there is also the so-called Universal Maritime Data Model introduced. The features of the UMDM will be explained in the following section.

## Functional and physical links – the means to interact

The bold arrow in Figure 1 is a graphical symbol for the functional connections for data exchange between the shore-based applications and the “ship’s environment”, and vice versa. In IT terms the originating entity of data in a functional link is called “source”, while the receiving entity of that data in the same functional link is called “sink”. The physical path of the data exchange uses the physical links and the various physical interfaces between the shore-based applications and the “ship’s environment”.

The functional connection is the abstract statement in regard to requirement analysis for the application-to-application data exchange. The physical path may take a completely different and more sophisticated route. To use the abstraction of the functional connection hence facilitates the requirement analysis by system engineers.

Physical links between (fixed) shore and (mobile) shipboard equipment each employ one or more appropriate mediums such as radio waves or light.

## Dependency on external systems and on infrastructure

  - Dependency on GNSS for position and time

As indicated in Figure 1, there is a dependency of the e-Navigation architecture on external systems for position fixing and for timing. This may pose a certain vulnerability for all e-Navigation applications, since dynamic position information is involved in most and time information is required in every. Hence, mitigation methods are important.

--- incorporate reference to former IMO NAV statements (NAV53 report) regarding required terrestrial PNT backup system (while not being specific on which system proposal at this point)

  - Dependency on infrastructure

Though functionally apparently “invisible”, the supporting infrastructure as a module internal to any shore-based e-Navigation system architecture is important for the proper and efficient function of the e-Navigation applications. The same holds true for the shipboard infrastructure. Hence, the dependency of any technical e-Navigation system architecture on infrastructure.

The infrastructure consists of supporting devices, which e.g. allow application software to be executed (i.e. computers, computer periphery, operation systems, local area network components, etc.), provide a reliable energy supply, provide temperature control, provide housing for components, protection against harmful interference, tampering, unauthorised access, environmental conditions, etc.

Infrastructure becomes more and more sophisticated and in itself subject to increasing requirements. Reduction of energy consumption, environmental friendliness, centralised management, maintenance cost, staffing level, staff responsibilities, and reliability level are just a few key words in this regard.

The infrastructure should be planned carefully to mitigate harmful consequences of that dependency.

# The concept of the Universal Maritime Data Model (UMDM)

The Universal Maritime Data Model (UMDM) was introduced in Figure 1 already.

## The derivation of required data objects from user needs

When considering the e-Navigation architecture, one should think in terms of information/data flow, application interactions, and user interfaces, as follows:

User requirements can be described in terms of their respective information needs for their tasks to perform. The required information items are delivered at the Human Machine Interfaces of the user applications by the shore-based e-Navigation system as fulfilment of the user requirements. These information items are transmitted, stored, and processed as data objects by the shore-based e-Navigation system. They are exchanged using the functional links between applications, both shipboard and ashore.

From the above it follows, that when there is an information flow between users and applications, there is always a parallel data flow associated with it.

The following Table 1 summarizes the above relationships.

  - User requirements in general (top level) 	(information domain)

  - Tasks of the user 					(information domain)

  - Stated information needs				(information domain)

  - Individual information objects 			(information domain)

__interaction by Human-Machine-Interface (Operational Presentation Surface)__

  - Maritime data objects  				(data domain)

  - Maritime data encoding-for-data exchange 	(data domain)

Table 1: Derivation of information needs, information objects and data objects for technical services from user requirements

## The justification for the UMDM

Implementation of IMO’s e-Navigation strategy leads to a larger variety and higher volume of information and increased information exchange due to globalization. Consequently there is a need to handle information more effectively in a standardized way. IALA realized that a common data model that expressly includes ship and shore data is essential. IALA designed a data model that meets these characteristics but is also extendable so that it can grow as more end user requirements are identified and captured. This data model is named the IALA Universal Maritime Data Model (UMDM).

Specifically for integration and exchange of maritime information the IMO has stated (italics added):

“Mariners require information pertaining to the planning and execution of voyages, the assessment of navigation risk and compliance with regulation. This information should be accessible from a single integrated system. Shore users require information pertaining to their maritime domain, including static and dynamic information on vessels and their voyages. This information should be provided in an internationally agreed common data structure. Such a data structure is essential for the sharing of information amongst shore authorities on a regional and international basis.”[IMO MSC85 Annex 20 Section 8.2.1]

The first step towards a common data structure is to define the meaning of each and every item in the data structure and the relationships among the items. This is done so that implementers of the data structure have common understanding of items. The means to do this is with a data model. A data model is an inventory of item definitions and blueprints for item relationships, and as such is a tool used to define data structures. The IALA UMDM is a data model specifically designed for maritime data.

## The description of the IALA UMDM

The UMDM is an abstract representation of the maritime domain. Specifically, it represents the entities and relationships among the entities that exist in this domain but does not represent processes. The purpose of a data model is to give users a common understanding of the entities and relationships.

The UMDM is neither a database nor an interface. The model contains no details about physical representation of the entities within it. However, the UMDM can be used to guide the development of databases and interfaces, which are the physical representations of entities. By defining a common reference, all implementers can be assured that their understanding of the entities required agrees with the understanding of others.

The UMDM is flexible and extendable for meeting future requirements. New entities can be added to the model by any e-Navigation stakeholder through a process known as registration. Once registered, an entity is available to all stakeholders; there is open, public access. The UMDM also has provisions for proprietary entities that do not require registration. The model is modular and an appropriate subset is easily referenced.

The UMDM can represent any maritime entity, can be extended by the addition of new entities, and is accessible to any e-Navigation stakeholder or implementer, thus it is considered to be universal in the maritime domain.

The Universal Data Model concept is composed of these elements:

- UMDM – this is the structure of the data model itself and the objects it contains. In the terminology of UMDM, maritime entities are represented by data objects.

- UDOI – Universal Maritime Data Object Identifier – this is a unique identifier assigned to each object in the UMDM. The UDOI is discussed in the following section.

- UMOR – Universal Maritime Data Object Registry – this is the mechanism for extending and maintaining the UMDM. The Registry is discussed in the second section following.

- Rules for management of the UMDM - management is discussed in the third section following.

The Figure 2 illustrates how the UMDM will be created – based on user requirements –  and how it will influence the components of the e-Navigation architecture when used by (software) engineers.

UMDM

Ship

Technology environment

Shore-based e-Navigation system

WWRNS

Physical Link technologies

User requirements

User requirements

Templates + methodology

Templates + methodology

mariner

Shore-based

users

Figure 2: Scope and Impact of the Universal Maritime Data Model

## The necessity of a data object identifier

Entities are represented in the UMDM by objects. Each object consists of (or contains) one or more attributes that represent the properties of the object. An attribute can be another object, or it can represent a single property. The property describes the attribute in terms of the values that it can contain. Other attributes may be used to further describe the object.

In order to use objects unambiguously, they must be uniquely identified. Identification of objects is accomplished using the Universal Maritime Data Object Identifier (UDOI) system where each object is given a unique UDOI. The UDOI:

- expedites access of objects,

- facilitates interchange and sharing of objects, and

- is used as an index into the Universal Maritime Data Object Registry.

## The necessity of a “registrar” for the UMDM

A system of organization is needed to keep track of objects. This is done by the Universal Maritime Data Object Registry (UMOR).

The Registry eases these tasks:

- Maintenance of the UMDM

- Finding and retrieving objects

- Prevention of redundancies within the UMDM

- Facilitation of  global implementation and exchange of objects through standardization

There is only one Registry for the UMDM. The Registry is operated by a body known as the Registrar. The Registry provides for the organization of objects and their UDOIs.

## IMO’s and IALA’s responsibilities regarding the UMDM

The UMDM should be unique. A parallel can be made between the UMDM and the Internet. Although it is possible to have multiple global networks, the true benefits of such a network would only be achieved if there is only one global network.

The parallel extends even further as the Internet provides a series of standardized network services from which the client can choose only the ones which he or she requires, e.g, email, ftp, www, etc. Hence, e-Navigation shore-based stakeholders can choose to implement only the UMDM objects that they require, as long as the objects implemented are compliant with the UMDM.

The UMDM will require a governing body to fulfil the role required for the management of the model. There may be several management roles to be performed by the governing body, one of which is the Registrar. The different management roles may be delegated at the discretion of the governing body. Hence, the Registrar may not be the same organization as the governing body. The tasks of the governing body always comprise supervision for the Registrar, however.

Initially, IALA will operate as the governing body and Registrar for the IALA UMDM. Once brought to a certain degree of maturity, the IALA UMDM would be offered to IMO as a contribution to the implementation of IMO’s e-Navigation strategy. Then IMO would assume the role as the governing body for the UMDM, thereby guaranteeing the uniqueness of the UMDM within e-Navigation. IALA is prepared to continue as the Registrar for the IMO UMDM. There is precedent for this within IMO, e.g., the registrar for a ship’s IMO number is Lloyd’s Register.

Construction of the UMDM will be a collaborative effort among many parties involved in the maritime and inland waterway environments. By having each party bring their particular expertise, the UMDM will become the accepted standard model.

# The concept of a “Service Portfolio”

When there are sets of Operational e-Navigation Services offered at a given location, this set is an Operational Service Portfolio. Similarly, this holds true for a Technical Service Portfolio, which consists of a set of Technical e-Navigation Services offered at a given location. The benefit of an expressively defined Service Portfolio is as described as follows.

## Justification for “Service Portfolio”

It was determined that scalability is one of the “key strategy elements” of the e-Navigation strategy [compare quotes from MSC86/23/4 above and in particular MSC85 report, Annex 20, §9.1.8]. This introduces by its very nature differences in service portfolios and / or service levels on a global, regional, and even national scale, both for operational e-Navigation services and for technical e-Navigation services being offered from ashore to the mariner and/or shipboard technical equipment.

## Description of a “Service Portfolio”

The following Figure 3 gives a graphical expression on how a portfolio of technical e-Navigation services of the shore-based e-Navigation system can be derived from stated user needs and user requirements.

--- add explanation, if desired ---

Figure 3: Engineering analysis of requirements to the shore-based e-Navigation system

## Relevance of “Service Portfolios”

With the implementation of the e-Navigation strategy there will be – as a matter of fact – both different sets of services provided and different levels of these services, both operational and technical, in adjacent areas throughout the same voyage of same vessel from berth to berth.

While this is the case with “legacy” services even today, the implementation of the e-Navigation strategy, will amplify the effects of this situations, both for the operational and the technical services, in the following ways:

- Due to the global nature of the e-Navigation concept the need for service portfolio management and service level management, both regarding operational and technical services, will be proliferated globally by default.

- While some aspects of the e-Navigation strategy may be implemented synchronously on a global scale, implementation of other aspects will happen asynchronously and not evenly everywhere on the world for various reasons (e.g. amount of relevant traffic, cost-benefit-considerations). Hence there will be differences both in the geographical and the introduction time domain. Ideally, the implementation of the e-Navigation strategy will eventually minimize the differences.

The concept of well defined service levels for both operational and technical e-Navigation services should be introduced deliberately to handle the associated complexity and sophistication and mitigate their consequences, both operationally and technically.

Compare by analogy as an example aviation landing approach Categories  I, II, and III at airports and their bearing on operations, technical equipment, and regulations.

When navigating from one area with one “service portfolio / service level” combo arrangement, offered by the competent shore-based authority, to an adjacent area with a combo, which may differ just in only one of those aspects “service portfolio” or “service level”, a “transition” takes place. Vessels during their voyage from one berth to another berth will regularly undergo several transitions. These transitions must be safe at all times, i.e. “safety at transitions” is of paramount importance for the navigation process, i.e. for e-Navigation. This has consequences for both operational and technical architectures. As an existing example of a technical transition, “transition zones” have been defined to maintain navigational safety within the AIS Channel Management.

--- introduce a Figure 4 of this concept for illustration: draw a ship’s passage from berth to berth (in different continents/countries) through areas with different service portfolios, also indicating the transition necessities.

Key issues like “service portfolio”, “levels of service”, “safety at transitions” firstly and prominently impact the operational e-Navigation services provided by the shore-based administrations. In order to finalize the technical shore-based e-Navigation system architecture, however, the definitions of “e-Navigation compliant” Operational Service Portfolios, Operational Service Levels, and criteria for safe transitions would be needed, because in e-Navigation the fulfilment of Operational Services will rely largely on automated machines (compare the very definition of the e-Navigation: “electronic means”). These definitions and criteria need to be developed as part of the finalization of “user requirements”.

# Aspects of the Initial technical e-Navigation Architecture

--- Introduction

## Ship-to-ship

--- introduce the following Figure

Figure 5: Ship-to-Ship aspect of the Initial technical e-Navigation system architecture

--- create new text

## Ship-to-shore and shore-to-ship

---Intro statetement, explaining that the document now turns ashore.

### The responsibility of shore-based authorities regarding e-Navigation

The responsibilities of IALA National Members regarding international and/or regional e-Navigation applications are:

- that the shore-based e-Navigation systems of National Members are embedded into local, national, regional and global topologies of systems;

- that there are data/information flow chains within the local, national, regional and global topologies of systems which support local, national, regional, and global e-Navigation applications;

- that each shore-based e-Navigation system contributes in a specific way to those data/information flow chains;

- that the disruption of this contribution would have an effect on the e-Navigation applications supported;

- that the shore-based e-Navigation system in itself consists of components, which support the data/information flow chain through that system;

- that therefore certain requirements regarding quality parameters, such as accuracy, reliability, continuity, confidentiality, and others, will need to be taken into account when designing the common shore-based e-Navigation system architecture;

### General requirements for shore-based e-Navigation system architecture

The shore-based e-Navigation system architecture will assist shore authorities in the efficient deployment of their services to the mariner and to the maritime community by

- allowing the shore-based system of an authority to participate in the harmonised data exchange ship-shore and shore-ship;

- easing the integration of standardised technical e-Navigation services and components;

- creating a global market for those standardised technical e-Navigation services and components;

- ensuring maintenance- and development-friendly architecture of the technical e-Navigation services and components;

- improving the efficiency in organisational terms by exploiting commonalities;

- improving the life cycle management;

- harmonising data exchange to other shore-based stakeholders’ systems (local, national, regional, global);

- complying with the anticipated Performance Standards, i.e. rendering the shore-based systems “e-Navigation compliant”;

- obtaining cost benefits from the above bullets;

- taking into account the sensitivity of data;

By implementing and adopting the shore-based e-Navigation system architecture the following possible benefits can be achieved. Many of them have been specifically identified also in the e-Navigation strategy [MSC85 report, Annexes 20/21].

- Quality benefits:

- Provision of user information, as stated, and in stated  format(s), including accuracy, integrity, reliability, continuity, and latency

- demonstration of “e-Navigation compliancy” (see below) to stakeholders by means of appropriate certification for service level achievements

- application of objective criteria for each technical e-Navigation service provided

- same level of service for different users with similar requirements

- extensive usage of applicable international technical standards

- improved provision in architectural terms to incorporate new functionalities at planning phase based on new stated information needs and formats

- increased dynamic adaptability to incorporate new functionalities at run-time based on new stated information needs and formats

- improved responsiveness to new or amended stated information needs and formats

- Cost related benefits:

- Full life-cycle cost evaluation facilitated by the identification of cost items, both for investment and running costs, which are known in advance

- Cost reductions for planning and development by maximum usage of applicable international standards and extensive re-use of engineering concepts

- Cost reductions for updating existing services and functionalities due to (rapid) change of technology

- Cost reductions for implementing further or changed services and/or functionalities

- Investment cost reduction: a) reduction by cost per unit due to standardization; b) reduction in numbers of units needed due to better utilization of the same kind of unit (down side: potentially increased single point of failure, hence effective mitigation strategies needed)

- Running cost reduction over full life cycle, taking also into account cut in staffing level: a) remote access/control as opposed to only-localized access/control resulting in consequential savings of working time and travel; b) automation; c) easier replacement of individual components

- Organisation and staff related benefits:

- Maintenance and widening of the expertise of technical personnel

- Achievement of common level of understanding between different, but interacting authorities, who both employ the common shore-based e-Navigation system architecture

- Optimization of the business processes and organizational procedures involved based on the use of the common shore-based e-Navigation system architecture

- potentially optimization of organizational structure

- improvement of training efficiency for technical personnel due to the transfer of knowledge from the general (i.e. the understanding of the generic model) to the specific technology under consideration (i.e. individual technical e-Navigation service);

- Public relations and societal benefits:

- Contribution to an improved public relation image of the administration by use of an internationally accepted, advanced system architecture

- Ease of public access to information by e.g. publicly accessible information portals provided by the administration, while the administration simultaneously has the benefit of a standardized method of presenting that information to the public

- Success in achieving the stated goals for improving the safety and efficiency of navigation, protection of the environment and security [IMO MSC85 report, Annex 20].

Most of the above benefits are critical to the success in achieving the stated goals for the e-Navigation strategy.

### Structural overview

As a consequence of the above discussion, the Figure 6 on the next page shows the resulting place of the shore-based e-Navigation system of a shore authority within the overarching e-Navigation system architecture.

Also, Figure 6 shows the

- interactions of shore-based applications with shipboard applications as operational e-Navigation services which are facilitated by the interaction of the shipboard technical equipment with the shore-based technical system;

- potentially user-independent, automated interaction of the shore-based technical system with the shipboard technical equipment;

- relationship between the shore-based system and its shore-based users, indicating that information needs must be satisfied by an appropriate technical service at its appropriate Human Machine Interface.

- interaction with other shore-based systems, named “third party” systems from the point of view of the shore authority’s system, both for giving and receiving data;

- existence of so-called technical e-Navigation services as one building block of the shore-based e-Navigation system.

Figure 6: The ship-shore/shore-ship interactions within the Initial technical e-Navigation architecture – simplified presentation

--- draft short to the following figure

Figure 7: Shore-based e-Navigation system architecture as embedded in the overarching e-Navigation architecture and in relation to stated information needs and formats

In Figure 7 the following graphical symbols are used:

- Interfaces between entities involved are symbolised by a little circle and a line leading to the system which provides the interface. The interface as such belongs to the system which provides it.

- Technical systems are symbolised by cubes, which expresses the encapsulation principle explained above (“black boxes”).

- Human-Machine-Interfaces, indicated by the acronym HMI, and Machine-Machine-Interfaces, i.e. mainly IT or computer interfaces, indicated by the acronym M2M.

- The arrows with the dotted lines indicate requirements, which are put forward by the entity at which the dotted line starts.

- Requirement arrow(s) always point to interfaces (circles). This is to indicate, that the interface(s) fulfil the requirement(s).

The following conclusions can be drawn from Figures 6 and 7:

- When designing and describing e-Navigation applications, they should be described as interactions between applications using the information/data flow concept:

- (Ultimate) sources of data should be identified together with the data objects stemming from those sources, i.e. the source data.

- Likewise, (ultimate) sinks for data should be identified, together with the data objects required by a particular sink in the information flow, i.e. recipient data.

- When there is intermediate processing of data objects involved, the appropriate algorithms should be stated.

- For all data objects should be described their relevant attributes or properties, including their constraints (such as permissible min/max values).

- Figure 6 and 7 also represents a statement regarding the distribution of responsibilities between operational stake-holders and engineers:

- Operational stake-holders state their information needs and the format (symbolised in Figure 7 by dotted arrows) at the user interfaces of the shore-based e-Navigation system.

- Engineers analyse these information needs and take into account the management goals, in particular considering the life-cycle-management aspects of any system or component. The result of their analysis is an engineering-like representation.

- This engineering-like representation is described in one or more stand-alone documents. Thus, it can and should be verified and validated by operational stake-holders and management. This engineering-like representation constitutes a kind of “Service Level Agreement (SLA)” between the operational stakeholders and the engineers, i.e. a Service Level Agreement regarding a technical service delivered to the user interfaces.

- Engineers will eventually provide the appropriate technical Human-Machine-Interface (HMI), which fulfils the stated information needs and the stated format requirements.

- A Human-Machine-Interface (HMI) can be any kind of appropriate combination of displays, keyboards, voice interfaces (microphone / loudspeaker), and other human interaction devices. The suite of those devices at one operational working position is called the Operational Presentation Surface (OPS). This comprises the concepts of modern IT human interfacing technologies, such as window techniques.

- The distribution of responsibilities is reflected in an appropriate documentation framework. As far as IALA documentation is concerned, this is further developed in the last chapter.

For the precise technical structure of the shore-base technical e-Navigation services see the common shore-base e-Navigation system architecture in the appropriate IALA Recommendation.

## Shore-to-shore

---Intro: The relationship between different shore-based systems within the context of e-Navigation

### Topologies for shore-based systems

Along the maritime and inland waterways there are and will be different shore-based systems operating giving services to the mariners and the shore-based users, i.e. “e-Navigation compliant” systems and legacy systems. Those different shore-based systems should interact with the systems on board of ships and other shore-based systems. This can be on a local, national, regional or global scale.

- Local systems: Local systems provide services to users in geographically confined areas of waterways. Their main goal is to serve the mariner in that confined area.

- National systems: National systems provide services to users of appropriate waterways in their respective countries.

- Regional systems: Regional systems provide services to users in a specific region of the world. Such a system could for instance serve the St-Lawrence-Seaway, the whole Baltic area, the Malacca Strait, as a region.

- Global systems: Global systems provide services on a world-wide scale.

Every system comprises a set of components which provide the specific services for the different users in their appropriate area of responsibility.

At present, connectivity between shore-based systems is sometimes limited due to present architectural constraints, i.e. at present not every shore-based system is able to interact with any other system on whatever scale in principle.

From the individual user’s perspective, both onboard and ashore, the UN/IMO concept of a “single window” (refer to e.g. the work of IMO’s Facilitation Committee) will be beneficial because a “single window” would avoid administrative overhead. In order to support this concept, the systems involved need to interact accordingly and exchange data to the other stakeholders involved. This can be achieved by distributed systems on the various scales involved (local, national, regional, and global), which are operated by independent authorities: “Single window” does not imply “single system”.

Figure 8 illustrates how the above systems communicate with each other and with the mariner. The lines show a substantial sample of the possible relations between systems and mariners. In order for these relationships to work efficiently there needs to be commonality of the data objects and their format.

An IALA National Member generally operates and maintains one of the national systems (compare highlighted national system in Figure 8). Hence, an IALA National Member needs to take into account the required connectivity to the other shore-based systems operated and maintained by other stakeholders (compare e.g. Annex 2 of [2] for a list).

Figure 8: Topology of different kind of shore-based systems

Figure 8 also provides an example of a vessel interacting with shore-based authorities using a “single window” provided by the national system No.3. National system No. 3 would be responsible to disseminate the data required by the other shore-based systems indicated by the dotted rectangle, i.e. national system No. 4 and regional system No. 2.

### Necessary Inter-System data exchange as a resulting consequence

For the shore-based systems to work in an efficient way, i.e. increasingly automated, standardized data exchange between those systems is necessary. Standardized data exchange between the shore-based systems will result in a more consistent and reliable system interaction and data.

Standardized data exchange will also minimize the burden on the mariner for giving information to the shore-based authorities and will ensure more reliable and more complete information about shipping.

Standardized data exchange requires both a standardized data model and standardized data exchange formats.

A standardized data model describes the data exchanged by using data property definitions. The standardized data model ideally is one universal data model including all kind of data properties in the maritime world, which is called Universal Maritime Data Model (U.MDM). Also, each and every data item of that universal model should be identifiable by a unique data object identifier, which is called Universal Data Object Identifier (U.DOI).

There are several data encoding options to exchange the same data item of the U.MDM by using different data exchange formats, each of which is tailored to a specific need.

On top of that there are several data transmission technology options available to transmit the encoded data.

The U.MDM/U.DOI concept, the data encoding options and the data transmission technologies are not within the scope of this Recommendation. They will be addressed in different IALA Recommendations.

## Conclusions

--- all aspects are active in parall at run-time

--- others…

# Elaboration of the Initial technical e-Navigation architecture

## The concept of the Service ID

--- taken from IMO NAV 55 report (WP.6); to be amended accordingly

--- apply the logic of “registrar” similar to UMDM to the Service ID concept

- Germany proposed to include both an “Originator Service ID” field and a “Destination Service ID” field in the beginning of the data packet portions of Application Specific Messages. This data packet portion is henceforth called Application-Specific Data Container (ASC).

- Justification

  - The justification for this proposal is the purpose of the AIS Application-Specific Messages, as the very name implies, namely to directly support user applications by appropriate data exchange, using the AIS VHF data link only as one data communication link readily available.

  - To that end, Application-Specific Data Containers as such are exchanged between e.g. some appropriate piece of ship borne electronic equipment, e.g. the INS, to some piece of electronic equipment ashore, e.g. a VTS data processing entity, using the AIS VHF data link, while the AIS devices involved forward the Application-Specific Data Containers in accordance with the AIS radio protocol without any evaluation, processing or change of any Application-Specific Data Container.

  - The Application-Specific Data Container is thus “transparently” routed through the AIS at large. The process can be performed also shore-to-ship and ship-to-ship.

  - The exchange of Application-Specific Data Container between two entities constitutes a functional link in terms of data exchange between them. This principle also holds true, if the Application-Specific Data Container is intended to be received by many different recipients, such as in the broadcast mode. Then there are point-to-multipoint functional connections established.

  - To achieve the purpose of the AIS Application Specific Messages, namely to directly support applications by appropriate data exchange via the AIS VHF data link, the actual originator of the Application-Specific Data Container, i.e. the “Originator Service”, as well as the actual destination of the Application-Specific Data Container, i.e. the “Destination Service”, must be known in the process.

  - To use the MMSI for that purpose is inappropriate because the MMSI signifies the identity of the AIS devices involved in the process and is thus intended to support the physical radio transmissions of AIS messages between the AIS devices involved. Also, the exhaustion of the MMSI number space would prevent the use of MMSI for service identity.

- Germany also stated that the notion of the service identities would require an internationally agreed scheme or table to identify the services and applications. The creation and eventual use of such international agreed scheme or table would also be a major contribution to the implementation of the e-Navigation strategy, namely a contribution to the core objective of harmonization of the e-Navigation. The above scheme or table of service identities could also be based on the existing tables of maritime stake holders contained in the e-Navigation strategy documentation (refer to Annex 2, “Potential e-Navigation users”, in Annex 12 to NAV54/25).

## Elaboration of the e-Navigation shipboard architecture vision

Point to Annexes explaining in some detail what is to be understood by AIS (V2), integrated radio navigation receiver, and the integration digital communication receiver

### Introduction

Taking into account the needs of mariners (i.e., user needs), developments in technology, and the requirements and constraints of aids to navigation authorities, IALA has begun the development of the operational services as well as a common shore-based e-navigation system architecture with a detailed description of its shore-side components.  In particular, these technical e-Navigation services must be compatible with the shipboard e-Navigation system architecture and its equipment on every level of machine-to-machine interaction.

Following this rationale, IALA has developed its vision for the shipboard e-navigation system architecture.  It addresses the fundamentals of the architectural design and the functional levels within the ship’s architecture.  It is then shown how the shipboard architecture fits with IALA’s common shore-based system architecture.

### Development of a shipboard e-Navigation system architecture

After the principle elements of the overarching architecture for e-Navigation have been introduced in the previous chapter, this chapter now focuses on a possible future architecture for the shipboard electronic system, which supports the e-Navigation concept. This shipboard architecture is called “shipboard e-Navigation system architecture”.

Since the architecture under consideration is the system architecture for an electronic system, the architecture under consideration essentially is a technical architecture and must be developed using state-of-the-art engineering principles. Some of these have been stipulated by IMO’s “strategy for the development and implementation of e-Navigation” [1], for example that the technical system should be an “open” system with standardised interfaces between modules and components.

The following sections develop the shipboard e-Navigation system architecture in a step by step fashion.

#### Overview and most fundamental ideas

According to the above IMO e-Navigation strategy, every consideration has to start with the user requirements. The user requirements inform engineers what the user wants to see on the presentation surface i.e. on the Human Machine Interface provided by the shipboard system in order to perform the user tasks. These information needs of the user lead to a set of individual information items, which in turn translate to data objects being presented by the machine via its Human Machine Interface (HMI) to the user: The machine(s) is only capable of dealing with data. Whether the data presented by the machine to the user is indeed meaningful to him in a given situation, i.e. thus becomes information, only the user can decide.

This important first step is depicted in the top of Figure 2 (overleaf): The technical functional block which is directly in contact with the user and provides him the required Human Machine Interface is depicted in the top and is called “Data Presentation and User Interaction” in accordance with its primary task. It should be noted, that “data” in this general context could be any kind of data, including digitized voice. The important distinction between information and data is introduced by the arrows at the left margin of the Figure, indicating the “information domain” in which the user resides and the “data domain”, to which all machines belong.

But where does the data to be presented to the user come from, and where does data input by the user go to? The answer to that question is given by the remaining functional blocks of the Figure 2, as far as the shipboard environment is concerned:

Transparent to the user at the next level down the data must be stored and evaluated. This in turn requires that that data must be gathered and transferred from its original source which is outside the own shipboard environment (i.e. provided from ashore and/or from other ships). These two processes are depicted by the functional blocks “Data Evaluation and Data Storage” and “Data Acquisition and Data Communications”. The functional block “Data Acquisition and Data Communication” interacts with the outside of the shipboard environment using a variety of data communication links. Again it should be noted, that data could also be voice.

Figure 2: Depiction of the fundamental technical modules for the shipboard e-Navigation system architecture

Any functional block of the shipboard e-Navigation system architecture must be able to exchange data with any other functional block, i.e. maximum connectivity is a requirement. To support this, a bus structure is introduced, which allows every functional block to connect with any other functional block.

Regarding “Communications”, i.e. transport of data (including digitized voice) through the shipboard e-Navigation system, it should be noted that there are two principle relationships for the shipboard e-Navigation system architecture:

- Internal to the shipboard environment: There is a direct connection between the Data Presentation + User Interaction level with the Data Acquisition + Data Communication level  (Therefore the bus structure allows for bypassing the Data Evaluation + Data Storage level).

- External to the shipboard environment: There are a variety of Data Communication Links in particular in the MF, HF, VHF, and even higher bands (refer for example to the IALA WWRCP), each of which caters for a specific set of functional requirements.

It should also be noted that at this stage the shipboard e-Navigation system architecture is composed of functional blocks instead of devices. Eventually there will be the need to define devices, of course, even only for the sake for required testing and for the consequential type approval. Conceptually, however, to use function blocks from the outset is the consequence of the user-requirement driven approach to start technical system development with the required functionality provided by the devices. The distribution of the total required functionality on the different devices would be done at some later stage, leading eventually to a precise definition of both devices and their functionality in detail.

#### The Data Acquisition and Data Communications level

In this section the level which interfaces the shipboard e-Navigation system to its external e-Navigation environment is introduced. From IALA’s point of view the Data Acquisition and Data Communication level contains at least the following devices (refer to Figure 3):

- an AIS (V2.0) mobile station: Building on the core functionalities of the present AIS shipboard mobile Class A station, and improving several technical features in detail, this device exhibits improved AIS related messaging capabilities and improved frequency channel utilization. Thereby, the LRIT, amongst other applications, is supported by an improved satellite tracking capability of the AIS signals. Also, the AIS (V2.0) mobile station supports the GMDSS by absorbing the relevant operational functionalities of the VHF-DSC components. Thus, this AIS (V2.0) mobile station may ultimately render the VHF-DSC shipboard component of the GMDSS obsolete. It would use the globally designated VHF Channel 70 for all AIS management purposes but with the AIS’s own, more frequency efficient TDMA radio protocol instead of the DSC radio protocol. Finally, the AIS (V2.0) mobile station would no longer require a mandatory user interface making the so-called “Minimum Keyboard and Display” of the present AIS shipboard stations obsolete. The interaction with the user would be done completely at the “Data Presentation and User Interaction” level by default.

- an  integrated digital terrestrial communication device: This devices allows for digital voice/data communication in several terrestrial frequency bands, such as MF, HF, VHF, and even higher frequencies, using efficient digital data radio transmission protocols. The device may also autonomously select the most suitable, available terrestrial frequency band and protocol for a data transmission, depending on current location, radio conditions, and costs. Thus, this integrated digital voice/data communication device may ultimately render the shipboard NAVTEX receiver and the HF-DSC shipboard component of the GMDSS obsolete. It would use the globally designated MF and HF channels with more frequency efficient radio protocols. This device also does not require a user interface, because the interaction with the user again is done completely at the “Data Presentation and User Interaction” level by default. This app
 
Note: The device is only using terrestrial frequency bands, the shipboard satellite communication device(s) is/are still necessary. Conceptually, the shipboard satellite communication device(s) also belong(s) to the “Data Acquisition and Data Communication” level (compare Figure 3).

Note: There may additional digital terrestrial communication devices on this “Data Acquisition and Data Communication” level, for which there is intention to integrate them into the above device, namely e.g. WiFi or WiMax communication devices.

- an integrated radio navigation device: This is a device using all available components of IMO’s World Wide Radionavigation System (WWRNS) simultaneously to provide the best electronic position fix for the ship. Those components of the WWRNS are the GNSS, satellite- and terrestrial augmentation systems as well as terrestrial backup systems. The shipboard receiver “integrates” signals from the different available WWRNS components by combining the pseudo ranges or TOE/TOA measurements of all received signals across the individual WWRNS component boundaries. Such a device would render the different existing individual shipboard radio navigation devices obsolete. Again, such a device also would not require a user interface, because the interaction with the user again is done completely at the “Data Presentation and User Interaction” level by default.

- a radar sensor: This is a potentially improved shipboard radar device which does not require a user presentation interface but forwards the radar-derived data to the “Data Evaluation and Data Storage” level for evaluation and eventual presentation to the user at the “Data Presentation and User Interaction” level. Even with the advent of the AIS (V2.0), shipboard radar is envisaged to remain an important data acquisition tool also in the future.

While this list of devices is not complete regarding shipboard data acquisition (“sensors”) and communications devices, it constitutes a major portion as far as the e-Navigation is concerned. Other devices which conceptually also belong to this “Data Acquisition and Data Communication” level are gyro, echo sounding device, SDME, etc. The “Data Acquisition and Data Communication” level may even include an optional high-speed land line data exchange device (when at berth).

Figure 3: Shipboard e-Navigation system architecture with functional block “Data Acquisition + Data Communications” further developed into devices

#### The Data Presentation and User Interaction level

This “Data Presentation and User Interaction” level is specialized on all functionalities (and eventually all necessary hard- and software devices) to present the available data to the user, as required, and in the user-required format, or to accept user interaction with the shipboard e-Navigation system. This level does not contain any data acquisition and data evaluation functionalities because there are other modules of the shipboard e-Navigation system architecture specialized in these functionalities.

This level thus is the Operational Presentation Surface of the whole of the shipboard e-Navigation system architecture. It will contain certain modules, which are well-known to the mariner and are therefore only introduced briefly (compare Figure 4 overleaf).

Figure 4: Shipboard e-Navigation system architecture with functional block “Data Presentation + User Interaction” further developed into functional modules

A harmonisation with the INS concept will be required.

Again it should be noted, that voice conceptually is considered a special variety of data. Hence, there will be a voice communications part.

#### The Data Evaluation and Data Storage level

This “Data Evaluation and Data Storage” level is the central data storage, evaluation and correlation core (IT kernel) of the shipboard e-Navigation system architecture. Eventually, this level will be mainly implemented by software modules running on at least two computers (for redundancy reasons). From an organisational, data modelling, IT engineering and maintenance point of view it is beneficial to subdivide the required functionality in accordance with the data objects they deal with algorithmically. Hence, a potential subdivision in functional modules would be “Traffic Object Correlation”, “Sea State and Waterways Data”, and “Maritime Services Directory” (compare Figure 5), and further modules would be added as needed.

Figure 5: Shipboard e-Navigation system architecture with functional building block „Data Evaluation + Data Storage“ further developed in functional modules

## Introduction to the common shore-based e-Navigation system architecture

Common shore-based system architecture general layout

The main building blocks of the common shore-based e-Navigation system architecture are the so-called technical e-Navigation services. The common shore-based e-Navigation system architecture describes the technical set-up of the shore-based system of an IALA National Member. Hence, any individual technical e-Navigation service contributing to the shore-based e-Navigation system is also a technical entity.

Its qualities are as follows: A technical e-Navigation service encapsulates all primary functionalities dealing with a specific technology or with a specific user, depending on the kind of the technical e-Navigation service. All technical e-Navigation services of the common shore-based e-Navigation system architecture adhere to the same object-oriented engineering model. The object-oriented model is described in a different IALA Recommendation. All technical e-Navigation services are self-contained and provide for themselves all capabilities needed for their tasks, including their own service management.

Figure 4 provides a structural overview representation of the common shore-based e-Navigation system architecture.

It should be noted, that the common shore-based e-Navigation system architecture is modelled in a client-server-fashion; the individual technical e-Navigation services can regularly assume either roles, i.e. be “clients” or “servers”, depending on their present role in a given interaction chain to support a given application-to-application data exchange within the overarching e-Navigation architecture.

Figure 4: common shore-based e-Navigation system architecture (overview)

In Figure 4 there are given some individual technical e-Navigation services (i.e. the User Interaction Service and the Gateway Service) and some groups of individual technical e-Navigation services (i.e. Data Collection and Data Transfer Services, Value Added Data Processing Services).

Basically all technical e-Navigation services shown in Figure 4 do what is implied by their name:

- The “Data Collection and Data Transfer Services” are a group of technical e-Navigation services interfacing the shore-based e-Navigation system to the physical links towards the traffic objects, the natural environment and the waterways as such. For example, the AIS Service, the Radar Service, the Visual Aids-to-Navigation Services belong to this group. Some operate bi-directionally (e.g. the AIS Service), while others operate uni-laterally on their appropriate physical links.

- The “Value Added Data Processing Services” also are a group of individual technical e-Navigation services. Their main task is to add value to (raw) data by processing, combination, comparison etc., store data and information and provide it upon request to other technical e-Navigation services.

- The “User Interaction Service” – a single individual technical e-Navigation service – is specialised to provide the Human-Machine-Interface to the primary users of the shore-based e-Navigation system, i.e. such users who are supported directly by the e-Navigation system via dedicated displays, keyboards and other human interaction devices.

- The “Gateway Service” – another individual technical e-Navigation service – is specialised on data exchange shore-to-shore. Mainly it interfaces to external systems of “third parties”, which provide data needed within the own system or which request relevant data to be forwarded to them. The “Gateway Service” can also interface different shore-based systems locally, regionally, and globally.

From Figure 4 also one further important conclusion can be derived: It is the combination of the technical e-Navigation services in their system-wide interaction, which provides the required functionality of the whole of the system to its users.

# The Initial technical e-Navigation architecture – the comprehensive picture

IMO has defined overarching e-Navigation architecture, saying that it should consist of a shipboard side, a shore-side, the communication links in-between and that it should also take into account position fixing (compare Figure 1 above). IALA has defined a so-called “common shore-based e-Navigation system architecture” as a recommendation for its national membership i.e. Aids-to-Navigation and VTS authorities.

The previous chapter introduced IALA’s vision of a shipboard e-Navigation system architecture to some level of detail. It is now required to fit that shipboard e-Navigation system architecture into the overarching e-Navigation architecture. This is done again in a step-by-step fashion, both steps building on Figure 1 (see above) and the second step adding detail to the first.

Figure 6 shows the complete e-Navigation architecture while the structural details of both the technical shipboard and shore-based e-Navigation system architectures are not yet shown.

Figure 6: The place of the shipboard e-Navigation system architecture within the e-Navigation architecture – simplified presentation

In the following Figure 7 the above details regarding the shipboard and shore-based e-Navigation system architectures have been included.

Figure 7: The place of the shipboard architecture in the overarching e-Navigation architecture – more detailed presentation

# The label “e-Navigation compliant”

Proposed working definition for the term/label “e-Navigation compliant” as opposed to (existing, then) “legacy systems”:

- The WG5 offers the following working definition for the term/label “e-Navigation compliant”:

<<While there is presently no direct definition of “e-Navigation compliancy” or of an “e-Navigation compliant” operational or technical service or device provided by IMO, the working and therefore tentative definition can be inferred from the IMO e-Navigation strategy. “e-Navigation compliant” would mean that an operational or technical service or device has been proven, tested, or checked by a competent body to be in conformity with relevant IMO performance standards, which were expressively created or revised as part of the implementation of IMO’s e-Navigation strategy.>>

- Migration from present systems, which would be rendered “legacy systems” with the start of the implementation of the e-Navigation strategy, to future “e-Navigation compliant” systems was recognized as a very important aspect.

- The label/term “e-Navigation compliant” would amongst other benefits also support the service portfolio management and the service level management, both in operational and technical regard, as described above.

# The Application-specific AIS messages as a proof of concept for the Initial technical e-Navigation Architecture

AIS has now grown to a World wide well accepted system for ship - ship communications. The main reason for using this is for safety reasons. The AIS system isn’t only used by ships but also by shore-based systems mainly operated by governmental agencies.

AIS has next to those capabilities the capability for using so called “Binary messages” and use them for Application specific reasons. In principal all the AIS messages are binary messages but some are pre defined and used all over the world for the same reason (applications).

The framework setup for AIS messages was designed to cope with future developments for applications whom need specific AIS messages. The messages specific developed are now registered by IMO and IALA. IMO register the application-specific messages used World wide and IALA registers the messages used on a Regional bases. The information registered is not how the AIS messages are but how the content of the messages are.

The architecture used for AIS could be mentioned as the first start for a technical e-Navigation Architecture,  a Proof of Concept for the e-Navigation Architecture. Therefore parts of the documentation written for AIS is now transferred to the UMDM.

--- add to this chapter the notion of the different stages of information/data flow through the chain in accordance with the architecture developed in previous chapters.

# Annexes to this chapter

## Annex A – The vision for the AIS (V.2)

[Note to WG3 “AIS TWG”: The WG5 “Architecture” has incorporated the work of the AIS TWG “A Vision for a Future AIS for e-Navigation “(eNAV7/14/6) verbatim / “as is”, except for chapter 7 “Recommendations to IALA Council”, in order to show the resulting appearance within the framework of this draft IMO CG’s report.

It is understood that the Annex A needs to be amended and finalized (!) at IALA eNAV7 in order to meet the IMO CG’s deadlines for submission to COMSAR 14.]

# AIS for e-Navigation

- AIS is a proven technology and should play a significant part in data communications for e-Navigation

  - By design AIS is not an ideal candidate for high speed and/or high volume data communications

  - High speed digital data communications for e-Navigation is better handled by using multiple 25KHz channels

  - The IMO ITU joint experts group has recognised that more than 200 Kbps will be necessary for e-Navigation

- This document proposes a future AIS to handle some data communications for e-Navigation.

# AIS today

- AIS is technically defined by ITU-R M.1371, mandatory for SOLAS vessels and for other vessels on a regional basis, and used voluntarily

- AIS has proved to be a powerful tool for various applications in the field of navigation, however these applications have not yet fully been exploited

- Appendix 15 RR (WRC-07) defines the frequencies used for GMDSS, and it includes the frequencies AIS1 and AIS2 used by AIS SART stations.

# Increasing use of AIS

### Number of ships

- Full implementation of SOLAS

- Increased use of mandatory AIS on non-SOLAS vessels

  - USA (USA commercial vessels)

  - Europe (EU Directive for fishing vessels requiring the use of Class-A AIS )

  - Europe (EU Directive for inland vessels requires the use of Inland AIS (Class-A derivative)

  - [Korea]

  - [India]

  - Mexico

  - Australia

- Increased voluntary use of AIS

  - Both Class-A and Class-B units being used on pleasure vessels

- Class-B  increasing use

  - Class-B (CSTDMA) visibility reduced due to polite behaviour

### AIS Base stations

- Coastal and inland AIS infrastructure continues to grow

  - Driven by safety, security, and environmental protection considerations

  - EU Directive 59 requires all member countries to maintain coastal AIS coverage

### Airborne AIS

- More aircraft are being fitted with AIS stations

### AIS AtoN, real or virtual, and AtoN monitoring

- Message 21 (AtoN Report) real and virtual is being used or approved by competent authorities

- A separate message for AtoN monitoring (often Message 6) is being used in most cases where an AIS AtoN station is deployed

- In addition, AIS AtoN stations are being deployed by commercial organisations to mark offshore platforms, wind farms, etc.

### AIS SART

- IMO carriage option for the AIS SART starts in January 2010

  - Broadcasts eight times per minute using Messages 1 and 14

### Binary messages

##### International

- IMO SN/Circ. 236 international messages trial is now complete

- Recommendations of the IMO CG on binary messages

  - Recommended retention of most of these international binary messages and the addition of new messages

    - E.g. a new meteorological-hydrological message with a dynamic length of 2 to 5 slots

    - E.g. an area message for navigational warnings

##### Regional

- Extensive use of safety-related binary messages for AtoN monitoring

- Binary messages being used in certain waterways

  - E.g. St Lawrence seaway

- IALA is maintaining a registry of these regional binary messages

##### Mandatory binary messaging in certain regions

- Additional binary messages are being used for inland AIS in Europe

  - Ship stations are type approved and in use

### New AIS messages (23, 24, 25, and 26)

- Messages 23, 24, 25, and 26 were added in recent editions of ITU-R.M1371

### GNSS differential corrections

- Use of Message 17 for GNSS correction data dissemination is increasing

  - E.g. already being broadcast in Gulf of Finland, Tokyo Bay

  - E.g. Germany, Netherlands, England are planning

### FATDMA usage

- Competent authorities are increasingly using FATDMA slot allocations for various uses

  - AIS AtoN

  - Base Station slot reservation broadcasts (Message 20)

  - Base Station regular broadcasts (Message 17)

  - Reserved slots for ship replies

  - Safety text messages

  - Meteorological and hydrological messages (Message 8)

- Indication that VDL will become more crowded and competent authorities resort to FATDMA reservations to ensure that their services are protected

# Additional future use of AIS

### Increased use of AIS shore to ship data transmissions

- In a future e-Navigation scenario, the transmission of AIS data from shore to ship will increase significantly with the advent of mandatory ECDIS

  - Examples

    - Ship off track warning

    - Territorial water advice

    - Environmental area warning

    - Navigational hazards

    - Weather warnings

    - Military test ranges and submarine exercises

- Some of the present voice communications between shore and ship will move to AIS messaging

  - E.g. ship reporting

  - Route information

### Increased use of ship-shore data transfer

- Ship reporting by AIS

- Requesting data

  - Weather data

  - Docking data

  - Route information

### Increased use of ship-ship data transmissions

- Reduced use of voice communications in favour of AIS messages

### Repeater, [Land Based station (formally called Limited Base station) Class-C?]

- There will be more owners of land based stations, some of these being private

- Development of limited base stations will be driven by the need for private use

  - Private ports

  - Bridges and locks

  - Canals

  - Offshore commercial facilities

- Repeaters will be used to extend coverage of shore AIS base station networks, and ship-ship AIS communications, and AIS AtoN range

  - Requiring double the VDL slot count to transfer data

  - And are deployed to extend base station coverage when the usual base station to base station data links are unavailable

### Future messaging

- Will be required as the suite of 1371 messages increases

  - Originally 22 message types, now 27

- Improved set of messages:

  - Replacement for some old messages, when they have proved to be not supporting proper data modelling (e.g. static ship data)

  - Reduction of tailored messages to essentially needed messages (“statements of the AIS device of its own”), while increasing the proportion of the transparent data container messaging (“passing information of other components/entities”): properly designed application specific message realm (e.g. also “safety related texts” would be some application specific messages)

- New flexibility for applications

### Satellite detection of AIS

- Research continues on enhanced methods of detecting AIS signal, and hence monitoring shipping, by satellite

- Satellite detection of AIS messages has been requested by administrations.

- Separate frequencies for satellite detection of AIS should be selected from within Appendix 18 because the tuning range of the shipborne AIS Class A is limited to these frequencies. Also, with respect to possible additional AIS frequencies, Report ITU-R M.2084 indicated that the interference environment resulting from the existing services in those bands must be taken into account in determining the feasibility of accommodating satellite AIS in any given band or channel, due to the large satellite antenna footprint that overlaps both land and sea. Separate operating frequencies in addition to AIS 1 and AIS 2 are needed that are not subject to terrestrial use.

- Frequencies should be considered only from Appendix 18 due to the limited tuning range of the ship-borne AIS.

- Appendix 18 contains only 3 frequencies (channels 16, 75 and 76) that are exclusively dedicated to maritime use (channels 75 and 76 are proposed to be shared with this service). This proposal meets the intent of footnote n) to Appendix 18 for interference mitigation.

### Future GMDSS platform

Is there any operational functionality in present VHF-DSC spec (M.493), which could not be fulfilled by amended AIS? (Hence, leading to AIS (V.2), see below, and replacing the present VHF DSC eventually)

- COMSAR has been tasked to modify the performance standard for the 406 MHz EPIRB to include use of AIS

- Use of AIS as a GMDSS alerting device

- Additional use of AIS as a part of GMDSS should be brought to the attention of IMO

### PLB (Personal Locating Beacon)

- There are initiatives to develop a PLB based on the AIS SART technology

### Use of AIS for PNT (Position, Navigation and Timing) and Ranging

- Improved time transfer capabilities shore to ship by using high precision time bases.

- The e-Navigation Committee envisages a “Ranging” mode for AIS

  - Where the AIS messages are used for position determination

  - By ship and shore

### Coordinated channel management for future e-Navigation VHF digital data communications

- Designation of additional AIS frequencies may enhance the coordination of regional channel management

# Strategy for future AIS

### General thoughts

- AIS is a proven maritime data system, with ships equipped and shore infrastructure established, the full potential of AIS is yet to be achieved

- AIS is the quickest path for handling the increasing data exchange needs described above

### AIS Channel plan

- Safety of Navigation purposes

  - The AIS-1 and AIS-2 are internationally allocated with sharing basis.  These frequencies should be retained and protected for safety of navigation purposes.

- Data communication purposes

  - In considering the future AIS system for terrestrial (non satellite) data communication purposes, additional two frequencies will be needed.  These frequencies should be selected within the Appendix 18.  The Appendix 18 covers 156-162.025MHz; however the current channelling arrangements are only a limited parts.  As shown in the figure below, the blue bands (sharing basis) and red bands (maritime exclusive) are channelized and the yellow bands (sharing basis) are not channelized.  The yellow bands may be available for maritime mobile service and candidate frequencies may be selected with the sharing basis.  The data communications are in the terrestrial service, the frequencies can be shared.

- Satellite detection of AIS and future GMDSS system

  - Since the satellites have very wide footprint, the frequencies for satellite detection of AIS should be exclusive for maritime mobile service.  The preliminary draft report ITU-R M.[SAT-AIS] proposes to use CH75 and CH76.  These channels are guard band channels of CH16 and are only maritime dedicated channels except CH16 and CH70.

  - In the future GMDSS system, the AIS technologies should be the basis.  The distress alerting, urgency and safety communications should be by both terrestrial and satellite communications; therefore the maritime exclusive frequencies are needed. The satellite detection of AIS is one way system (from earth to satellite); however in the GMDSS, the acknowledgement (from satellite to earth) is essential as the two way system.  The CH75 and CH76 will be used for this purpose.

- Channel management purposes

  - In order to fully use available VHF spectrum there must be a worldwide channel dedicated to manage and coordinate usage of the VHF Data Link.

  - TDMA or other protocols may share Channel 70 with minor changes to Appendix 18, footnote j) according to ITU-R M.822-1.

- Therefore, in order to meet the data communications needs set out above, IALA envisages that the present AIS system will need to develop into a future AIS

| AIS frequency pair | AIS-1 & AIS-2 | AIS-3 & AIS-4 Maritime Dedicated Channels | AIS-5 & AIS-6 | Channel 70 |
| --- | --- | --- | --- | --- |
| Purpose | - Safety of navigation - Maritime and inland distress and safety communications | - Space detection of AIS - Future GMDSS | - Marine safety information - General purpose information communication | - Management of VHF VDL in harmony with current ITU-R M. 822-1 |
| Existing/New frequency allocation | - Existing | - New | - New | - Existing |
| AIS Message types | - Present ITU-R.M1371-3 | - Message 27 for space detection of AIS - Other messages for support of future GMDSS | - S/N Circ 236 international AIS binary messages - Regional AIS binary messages | - TDMA system management messages e.g Message 20, Message 22, - Other messages for support of coordinating channel sharing |
| Typical applications | - Ship to ship collision avoidance - VTS tools - Tracking of ships - Locating in GMDSS | - Detection of vessels by coastal states beyond range of coastal AIS base stations - Two way distress communications | - Area warnings and advice - Meteorological and hydrological data - Traffic management - Channel management of AIS channels and future VHF digital data channels - Ship-shore data exchange | - Channel switching - FATDMA allocation, - Assignment |
| Proposed channels in Appendix 18 | - As now | - Channel 75 and 76 | - World-wide dedicated channels are preferred, but if this is not possible, then the non channelised band may be used for the candidate channels | - As now but requires Appendix 18, Footnote j), and ITU-R M. 822 modification |
| Comments |  | - IMO COMSAR has been tasked to study future GMDSS technology and timetable, and what changes in SOLAS are required. Two COMSAR sessions from 2010-03 are allocated (2 years). - The USA and other countries have proposed to IMO and ITU that a third AIS channel be designated to enhance the reception of AIS signals by satellite. |  | Expanded use of channel 70 to improve efficiency |

### Airborne AIS

- ITU Radio Regulations Article 51 Section III sets limits on aircraft VHF usage

  - This will be required to allow aircraft effective use for AIS message transmission in SAR cases

- It may also require change in the future to permit and/or prohibit certain AIS message transmission

### Legacy strategy

- Future advances in AIS will require updates to legacy systems

- Legacy functionality will be maintained

- Channel management, using Channel 70, already available internationally, should be maintained until it is replaced by another internationally agreed method of channel management

# Benefits of future AIS for the users

- [amend paragraph with good selling point of benefits!]

- Use of AIS for maritime information dissemination will mean that Navtex will not be needed for ships navigating in Area A1

- Existing shipboard AIS installations infrastructure could be upgraded

## Annex B – The vision for the “integrated digital terrestrial communication device”

[The WG5 “Architecture” considers it appropriate to incorporate a more or less complete description of the IALA envisaged “Integrated digital terrestrial communication device” as part of the IALA envisaged shipboard e-Navigation equipment suite in this Annex B.

This description should exhibit a proximity to a future IMO performance standards on the “integrated digital terrestrial communication device”. The Annex A regarding the envisaged AIS (V2) could be considered as an example in this regard.

The WG4 “Communications” is requested to compile such a description based on the MRCP, as developed by WG4, i.e. taking relevant material from eNAV7/10/14, eNAV7/10/15, eNAV7/10/16, eNAV7/10/17, during the e-NAV7 Committee meeting.

This should be considered a matter of priority in order to meet the IMO CG’s deadlines for submission to COMSAR 14.]

## Annex C – The vision for the “integrated radio navigation device”

[The WG5 “Architecture” considers it appropriate to incorporate a more or less complete description of the IALA envisaged “Integrated radio navigation device” as part of the IALA envisaged shipboard e-Navigation equipment suite in this Annex C.

This description should exhibit a proximity to a future IMO performance standards on the “integrated radio navigation device”. The Annex A regarding the envisaged AIS (V2) could be considered as an example in this regard.

The WG2 “Technologies for e-Navigation” is requested to compile such a description based on the WWRNP, as developed by WG2, i.e. taking relevant material from eNAV7/9/3, during the e-NAV7 Committee meeting.

This should be considered a matter of priority in order to meet the IMO CG’s deadlines for submission to COMSAR 14.]

Information - “knowledge concerning objects, such as facts, events, things, processes or ideas, including concepts, that within a certain context has a particular meaning” (ISO/PAS 16917:2002(E)) or, data made useful by interpretation or analysis.

Data - “representation of facts, concepts or instructions in a formalized manner, suitable or communication, interpretation or processing by humans or by automatic means” (ISO/PAS 16917:2002(E))

(One of the consequences of this regarding the benefits of the e-Navigation strategy is: While in the overall there will be achieved an improvement, for a period of time some anticipated benefits of the e-Navigation strategy will not materialize to the same degree everywhere.)

“Regional” in the context of this Recommendation is seen from a global perspective, i.e. a region could be a continent or a substantial part of a continent. E. g. the Baltic Sea as a whole is a region in this regard.

This implies that information needs and formats are captured and stated in appropriate documentation.

This data packet portion starts with the first field after the IAI/RAI field which contains the DAC/FI combo.

TOE: Time of Emission (of signal); TOA: Time of Arrival (of signal)

SDME: Speed and Distance Measurement Equipment

This would be in accordance with IMO’s requirement for a “common maritime information/data structure” [1, §8.2.1].

It should be noted, that by convention the individual technical e-Navigation services (i.e. the instances in the object-oriented terminology) are capitalised (“Service”) as opposed to technical e-Navigation services (using small letters) in a generic sense (i.e. the class descriptions in object-oriented terminology).

It should also be noted, that there is no such thing as a “super-service”, which would manage or control all services. Such a “super-service” is unnecessary when adhering properly to the object-oriented paradigm and would also constitute single point of failure of the e-Navigation system architecture. (This does not contradict a centralised control of the technical operation of the shore-based e-Navigation system or its centralised maintenance, however.)