| From: e-NAV Committee | e-NAV7/output/13 |
| --- | --- |
| To: ANM Committee | 25 September 2009 |

Liaison Note

Draft IALA Guideline No. #### On

A minimum comprehensive mix of AtoN in fairways including dredged channels and canals

Edition 1 [dated as December 2009]

# Introduction

The e-NAV Committee has reviewed the draft guideline (input paper e-NAV/19/5, formerly ANM13/WG1/WP1) and makes the following comments:

# General Comments

The Original task from Council is noted within the document as follows:

“Develop guidance (manual) on channel design and a minimum comprehensive mix of AtoN, promoting Marine Electronic Highways / e-Navigation especially in congested waters, including how to react to incidents that create navigation hazards.”

The document is considered to emphasise traditional (visual/physical) aids and has limited focus on e-Navigation.  There is no mention of Marine Electronic Highways or e-Navigation in the introduction.  There is currently no text on the final part of the brief although there is an editorial note highlighting this issue

# e-Navigation

This is general information which is not expanded within the text of the guideline in terms of a mix of AtoNs.

# Background

This section has much generalised text which may not be directly relevant to the particular topic of the guideline. The list of “tools” for navigating is very much a “traditional” list with e-Navigation elements only noted at the end.  These elements are referred to in the guideline only in passing.

4.1 notes:

“Safe navigation by electronic means requires a certain level of accuracy, availability and integrity in order to accurately portray the fairway/waterway, dredged channel or canal.  There should also be an adequate level of integration with the control of the ship either automatically or through the navigator.  As there are uncertainties connected with this, visual navigation eventually combined with the use of radar is in principle either a primary or a secondary means of navigation.”

This text appears to do little to “promote MEH / e-Navigation” as requested by Council.

4.2.4 A.423(XI) has been superseded by A.615(15) adopted in 1987

Suggested text for this section (taken from the resolution) is shown below, however quoting the reference may suffice:

# Accuracy

The table on which the e-NAV committee is specifically asked to comment includes all phases of navigation whereas the focus of the guideline is on fairways (restricted waters).  Furthermore, the table refers to more than accuracy which may be confusing to the reader as the additional parameters are not explained.  While there is ongoing discussion on the various parameters, especially accuracy requirements and continuity, no changes have yet been agreed at IMO.

5.2.2 on Radionavigation has the following text:

“Most Radionavigation systems provide a very good absolute accuracy for determination of the position based on a geodetic datum.  Using the navigational chart the mariner can navigate safely along a waterway.  The navigation accuracy is limited not only by the Radionavigation itself, but also by the accuracy of the navigational chart and the navigational equipment and performance on the ship.

However, there is a limitation in the safe use of GNSS position data caused by the Alert limit (or threshold value).  The maximum allowable error in the measured position – during integrity monitoring - before an alarm is triggered, is 25 m according to Appendix 2 of IMO-resolution A.915(22), which can be found in section 2.2 of this guideline.  So the user cannot rely absolutely on an accuracy of better than 25 m.  So, this is one more reason for having visual and short range AtoN for reference, also from the point of view of the Radionavigation.”

This promotes visual and short range AtoN over the use of Radionavigation rather than promoting a mix of AtoNs and does little to promote MEH / e-Navigation.  The statement whereby the user “cannot rely absolutely on an accuracy of better than 25m” is considered misleading.  The accuracy is specified as 10m (95%) for harbour entrances, harbour approaches and coastal waters.

# Perception (AIS AtoN)

The following text is proposed for the draft guideline, based on the current state of AIS AtoN developments:

## “6.3 Perception by means of additional electronic devices on AtoN

AtoN perception can be improved by electronic means utilizing AIS as an AtoN.

- Identification of the AtoN

- Longer range detectability, independent of visibility and weather conditions

- Health status of the AtoN

More Details can be found in IALA Recommendation A-126 on the use of AIS in Marine Aids to Navigation Services, Edition 1.4, December 2008.

6.3.1 - AIS on aids to navigation

AIS as applied to aids to navigation (AtoN), improves and enhances services provided to mariners as far as their vessels are equipped with AIS – and their display systems capable of displaying relevant AIS AtoN information:

- Provides an unambiguous indication of the identity of the AtoN, by carrying the AtoN name within the AIS message data;

- Operates day and night, and also in conditions of impaired visibility when visual AtoN signals may not be seen;

- Will generally have a greater range than daytime visual signals and visual signals from floating AtoN.

6.3.2 Met/hydro

- An AIS AtoN can also be used to broadcast meteorological and hydrological data and other safety related information in real time, and this data, along with the AIS AtoN data, can be displayed on the bridge of a ship on an electronic chart or radar display.

6.3.3 AIS applications to AtoN

AIS should be considered for fixed and floating AtoN if they are:

- In an area where clear indication of the identity of certain AtoN is essential;

- In a group of AtoN, where it is advantageous to clearly identify certain more-important AtoN, for example the buoys or beacons at a turning point in a channel;

- In an area where visibility may be poor at times;

- Where a reliable signal range of 10 miles or more is needed, day and night the e-Navigation committee proposes to improve the description of the potential benefits of using AIS technology for AtoN, however also bearing in mind that the operational capabilities for displaying AIS AtoN information on board vessels and recognition by mariners is currently limited.

- Verification of the integrity of the aid, including Off-Position and operational status indication / malfunction alert to the mariner; and

- Accuracy, because it shows the accurate position on the electronic chart or on a modern radar screen.

- Where clear identification of AtoN is essential, for instance at turning points

- Where long range or all weather perceptibility is required, compared with visible AtoN or RACON

- On positions with a high risk of damage, impairment or positional inaccuracy due to ice, current, collision, etc.

Administrations should however be aware of the limited display capabilities for AIS AtoN signals on current shipborne display systems, as well as limited recognition by mariners.

More Details can be found in IALA Recommendation A-126 on the use of AIS in Marine Aids to Navigation Services, Edition 1.4, December 2008.”

[the proposed text in 6.3.3 might make the present Section 7.5 redundant]

6.3.4 Virtual AtoNs

It is noted that IALA has arranged a workshop on Virtual AtoN in January 2010, which is intended to identify the future role of Virtual AtoN.  Current experience with AIS AtoN technology – for real and virtual AtoN – indicates that the role of electronic AtoN technologies may be developed significantly in the near future.  This should be taken into consideration in relation to the optimal timing of the publication of a guideline on the subject.”

# 7	Action requested

The ANM Committee is invited to consider the above comments on the draft guideline and in particular, whether material should be included on the possibility of providing virtual AtoN.