| From: e-NAV Committee | e-NAV7/output/14 |
| --- | --- |
| To: IALA Council | 25 September 2009 |

Liaison Note

Initial technical e-Navigation Architecture - the first contribution to the IMO e-Navigation Correspondence Group

# Introduction

The attached annex provides the document „Initial technical e-Navigation Architecture“.  It is the first contribution to the IMO e-Navigation Correspondence group, prepared by the IALA e-Navigation Committee for information of and approval by the IALA Council.

It is an initial description of:

- The overarching e-Navigation architecture

- Its dependency on external systems and on infrastructure

- The concept of the Universal Maritime Data Model (UMDM)

- Central elements of the UMDM i.e. Universal Data Object Identifier, Universal Maritime Data Object Register and the importance of a Registrar

The e-NAV committee continues its work on the shore based technical e-navigation system Architecture at its next meeting and at inter-sessional meetings, with the intention to provide further input to the IMO correspondence Group.

# Action requested

The Council is requested to approve the annexed document Initial technical e-Navigation Architecture as an input to the IMO Correspondence Group on e-Navigation.:

If approved the Secretariat is requested to forward the Annex to the IMO CG on e-Navigation.

### COVER LETTER TO THE IMO e-NAVIGATION CORRESPONDENCE GROUP

From:	e-NAV Committee e-Nav7

To:	The IMO e-Navigation Correspondence Group

Initial technical e-Navigation Architecture - contribution to the IMO e-Navigation Correspondence Group

The attached Annex provides the document Initial technical e-Navigation Architecture.  It is the first IALA contribution to the IMO e-navigation Correspondence group on the subject.  The document is prepared by the IALA e-NAV Committee.

The motivation behind this document is the statement from IMO on the e-Navigation strategy”; MSC 86/23/4, adopted by MSC86, §7]:

“The overall conceptual, functional and technical architecture will need to be developed and maintained, particularly in terms of process description, data structures, information systems, communications technology and regulations.  The architecture should include the hardware, data, information, communications technology and software needed to meet the user needs.  The system architecture should be based on a modular and scalable concept.  The system hardware and software should be based on open architectures to allow scalability of functions according to the needs of different users and to cater to continued development and enhancement.”

The Annex which is provided to the CG is an initial description of:

- The overarching e-Navigation architecture ;

- Its dependency on external systems and on infrastructure ;

- The concept of a Universal Maritime Data Model (UMDM) ;

- Central elements of the UMDM as; UDOI, UMOR and the importance of a Registrar.

Note that the attached annex does not, at this point in time, cover all the elements as stated in the quote above.  However, IALA continues to develop the technical e-Navigation Architecture document, and will when further developed cover the most relevant elements as stated in the quote above.

The further developed document will be provided to the IMO CG at later stage.

It is anticipated that the functional architecture will follow on from the work on user requirements.

- THE INITIAL TECHNICAL e-NAVIGATION ARCHITECTURE

# Scope of the e-Navigation Architecture

IALA has started the “Initial technical e-Navigation architecture work” based on the following understanding of IMO’s e-Navigation concept in general, and the e-Navigation architecture specifically using the following quotations taken from IMO documents:

- In order to have the e-Navigation concept implemented, the architecture is a required step [“Work programme - coordinated approach to the implementation of the e-Navigation strategy”, MSC 86/23/4, as adopted by MSC86, §5]:

“In order to prepare the strategy implementation plan several steps are required, such as capturing evolving user needs, developing an architecture and undertaking a gap analysis, a cost-benefit analysis and a risk analysis. These elements should provide a common and informed foundation for the creation of a detailed strategy implementation plan.”

- The IMO is also specific regarding the functions and components of any technical e-Navigation architecture [“Work programme - coordinated approach to the implementation of the e-Navigation strategy”; MSC 86/23/4, as adopted by MSC86, §7]:

“The overall conceptual, functional and technical architecture will need to be developed and maintained, particularly in terms of process description, data structures, information systems, communications technology and regulations. The architecture should include the hardware, data, information, communications technology and software needed to meet the user needs. The system architecture should be based on a modular and scalable concept. The system hardware and software should be based on open architectures to allow scalability of functions according to the needs of different users and to cater to continued development and enhancement.”

Further, IALA has undertaken this work, due to the invitation of IMO as stated in the following IMO documents:

- The above mentioned IMO document “Work programme - coordinated approach to the implementation of the e-Navigation strategy”; MSC 86/23/4, as adopted by MSC86, contains several times the invitation to IALA to contribute [§16, and Annex, columns entitled “external input”].

- IMO NAV55 identified in particular the contribution of IALA to the “Initial e-Navigation Architecture” [IMO NAV55/WP5 Report, Agenda item 11, §6.1] and [IMO NAV55/WP8 Report, §11.34]:

“The group also noted that development of system architecture had taken place in the interim period within IALA. Accordingly, the group invited IALA to provide the results of these developments to the correspondence group”.

## The overarching e-Navigation architecture

This document provides an introduction to the IALA developed Initial technical e-Navigation architecture.

The following sections introduce important principles of the IALA e-Navigation architecture, such as the orientation towards information flow instead of technology development.

The scope of this introduction to the IALA developed “Initial technical e-Navigation architecture” is represented by Figure 1 below, which shows the shipboard entities, the physical link(s) and the shore-based entities.

UMDM

UMDM

Ship

environment

IBS

World Wide Radionavigation System (WWRNS) of IMO, including GNSS, GNSS augmentation and terrestrial backup

Transceiver station

Ship’s

sensors

Shipboard

Applications

INS

Shore-based

technical

e-Navigation

services

(= shore-based

applications)

Application-to-

application

functional

connection

Physical

Link (e.g. radio link)

other

ships

other

ships

mariner

Shore- based users

(e.g. VTS Operators)

Shore

environment

- e-Navigation architecture

Figure 1 is an abstract representation of the e-Navigation environment.  On the left there is a single “ship environment”.  From the e-Navigation concept’s perspective the relevant devices within the ship environment are the transceiver station, the sensors and applications connected to the transceiver station, the Integrated Navigation System (INS) and the Integrated Bridge System (IBS). The transceiver station is shown as a single station, although in reality there may be several transceiver stations.

The transceiver station interfaces via the physical link(s) with the appropriate technical e-Navigation services ashore.

Within the shore-based services there are particular technical services responsible for interfacing to the ship.  All of the shore based technical services encapsulate the technology and, hence, reduce complexity.  Functional interfaces of the shore-based technical services are defined in accordance with the requirement for open system architecture.

The operators ashore, e. g. VTS operators, pilot station operators, and lock operators, need the user applications to perform their tasks in co-operation with shipboard applications.  From their perspective, it is neither the physical links nor the shore-based technical e-Navigation services that matter – they have been encapsulated for that reason.  It is the functional links between the shore-based user applications and the shipboard applications which matter to the users on both sides.  A similar setup of interactions applies for ship-to-ship and shore-to-shore applications.

Figure 1 also shows a dependency on the IMO World Wide Radio Navigation System (WWRNS), which provides position and timing information.  This dependency is discussed later on in the document.

Figure 1 also shows the Universal Maritime Data Model (UMDM).  The UMDM is an abstract and common data representation of the entities and relationships within the maritime domain.  The features of the UMDM will be explained later on in the document.

The bold arrow in Figure 1 is the representation of the functional connections for data exchange between the shore-based applications and the “ship’s environment”, and vice versa.  The physical path of the data exchange uses the physical links and the various physical interfaces between the shore-based applications and the “ship’s environment”.

The functional connection is the abstract statement in regard to requirement analysis for the application-to-application data exchange.

Physical links between (fixed) shore and (mobile) shipboard equipment each employ one or more appropriate mediums such as radio waves or light.

## Dependency on external systems and on infrastructure

- Dependency on GNSS for position and time

To have accurate position fixing and timing is a prerequisite for the whole e-Navigation concept. This information is provided through the WWRNS.

- Dependency on infrastructure

In order to implement the e-Navigation concept, a sufficiently advanced a proper infrastructure needs to be present.  This infrastructure would consist of supporting devices, which e.g. allow application software to be executed (i.e. computers, computer periphery, operation systems, local area network components, etc.)

Note that infrastructure itself becomes more and more sophisticated and is subject to its own requirements, such as reduction of energy consumption and environmental friendliness.

# The concept of the Universal Maritime Data Model (UMDM)

When considering the e-Navigation architecture, one should think in terms of information/data flow, application interactions, and user interfaces, as follows:

User requirements can be described in terms of their respective information needs for their tasks to perform.  The required information items are delivered at the Human Machine Interfaces of the user applications by the shore-based e-Navigation system as fulfilment of the user requirements.  These information items are transmitted, stored, and processed as so-called data objects by the shore-based e-Navigation system.

From this perspective the UMDM is an abstract and common data representation of all data objects and relationships occurring in the maritime domain.

Construction of the UMDM will be a collaborative effort among many parties involved in the maritime environments.  By having each party bring their particular expertise, the UMDM will become the accepted standard model.

## The justification for the UMDM

Implementation of IMO’s e-Navigation strategy leads to a larger variety and higher volume of information and increased information exchange due to globalization.  Consequently there is a need to handle information more effectively in a standardized way. IALA realized that a common data model that expressly includes ship and shore data is essential.  IALA designed a data model that meets these characteristics but is also extendable so that it can grow as more end user requirements are identified and captured.  This data model is proposed by IALA, under the name “Universal Maritime Data Model” (UMDM).  For the rest of the document this is referred to as the IALA Universal Maritime Data Model.

Specifically for integration and exchange of maritime information the IMO has stated (italics added):

“... Shore users require information pertaining to their maritime domain, including static and dynamic information on vessels and their voyages.  This information should be provided in an internationally agreed common data structure.  Such a data structure is essential for the sharing of information amongst shore authorities on a regional and international basis.”[IMO MSC85 Annex 20 Section 8.2.1]

The first step towards a common data structure is to define the meaning of each and every item in the data structure and the relationships among the items.  This is done so that implementers of the data structure have common understanding of items.  The means to do this is with a data model.  A data model is an inventory of item definitions and blueprints for item relationships, and as such is a tool used to define data structures.  The IALA UMDM is a data model specifically designed for maritime data.

## The description of the IALA UMDM

The UMDM is neither a database nor an interface.  The model contains no details about physical representation of the entities within it.  However, the UMDM can be used to guide the development of databases and interfaces, which are the physical representations of entities.

The UMDM is flexible and extendable for meeting future requirements.  New entities can be added to the model by any e-Navigation stakeholder through a process known as registration.  Once registered, an entity is available to all stakeholders; there is open, public access.  The UMDM also has provisions for proprietary entities that do not require registration. The model is modular and an appropriate subset is easily referenced.

The UMDM can represent any maritime entity, can be extended by the addition of new entities, and is accessible to any e-Navigation stakeholder or implementer, thus it is considered to be universal in the maritime domain.

The central elements of the UMDM are:

- UMDM – this is the structure of the data model itself and the data objects it contains, together with a data object rule base.  In the terminology of UMDM, maritime entities are represented by data objects.

- UDOI – Universal Maritime Data Object Identifier – Entities are represented in the UMDM by objects.  In order to use objects unambiguously, they must be uniquely identified.  Identification of objects is accomplished using the Universal Maritime Data Object Identifier (UDOI) system where each object is given a unique UDOI.

- UMOR – Universal Maritime Data Object Register – this is the mechanism for extending and maintaining the UMDM.  The Register will require some place to be hosted which is the Registry.  The organisation fulfilling the role of managing the UMDM is named the Registrar.  Initially, IALA could host the Register, and act as the Registrar.

- Rules for management of the UMDM – provided by relevant standards.

Information - “knowledge concerning objects, such as facts, events, things, processes or ideas, including concepts, that within a certain context has a particular meaning” (ISO/PAS 16917:2002(E)) or, data made useful by interpretation or analysis.

Data - “representation of facts, concepts or instructions in a formalized manner, suitable or communication, interpretation or processing by humans or by automatic means” (ISO/PAS 16917:2002(E))