Executive Summary

IALA World Wide Radio Communications Plan

International Association of Marine Aids to Navigation and Lighthouse Authorities

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) exists to:

- harmonise standards for aids to navigation systems worldwide;

- facilitate the safe and efficient movement of shipping, and;

- enhance the protection of the maritime environment.

This World Wide Radio Communication Plan (WWRCP) is being developed by IALA to assist in the selection of radio communication systems required to support e-Navigation. The current status of radio communication in the Maritime Mobile Band is discussed and forms the basis of projections for future developments needed to support e-Navigation. This WWRCP could be the basis for a possible future submission to ITU as a contribution to the continuing efficiency improvements with respect to radio spectrum.

The plan provides guidance to IALA members regarding potential future developments, which will enable members to identify areas requiring resource allocation and research activity.Table of Content

IALA World Wide Radio Communications Plan	1

International Association of Marine Aids to Navigation and Lighthouse Authorities	1

Executive Summary	2

The plan provides guidance to IALA members regarding potential future developments, which will enable members to identify areas requiring resource allocation and research activity.Table of Content	3

Glossary of terms	7

1	Introduction	9

1.1	general	9

1.2	scope and objectives	10

2	Background to Service Provision	11

2.1	general	11

2.2	institutional	11

2.3	REGULATORY	11

2.4	commercial	12

2.5	OPERATIONAL	12

2.6	technical	12

3	The IALA World Wide Radio Communication Plan	14

3.1	OVERVIEW	14

3.2	General overview of spectrum usage by band for existing communication system technologies	14

3.2.1	Low Frequency Band (LF)	14

3.2.2	Medium Frequency/ High Frequency Band (MF/HF)	14

3.2.3	Very High Frequency Band (VHF)	16

3.2.4	Ultra High Frequency Band / Super High Frequency Band (UHF/SHF)	17

3.3	Future development	19

3.3.1	Low Frequency Band (LF)	19

3.3.2	Medium Frequency/ High Frequency Band (MF/HF)	19

3.3.3	Very High Frequency Band (VHF)	19

3.3.4	Ultra High Frequency Band / Super High Frequency Band (UHF/SHF)	20

4	Application of communications within e-Navigation	22

4.1	Summary	24

5	Securing the spectrum	27

5.1	Situation with respect to existing spectrum usage	27

5.1.1	Conclusions	33

5.2	WRC-11 agenda items	33

5.2.1	Agenda Item 1.9	34

5.2.2	Agenda Item 1.10	35

5.2.3	Agenda Item 1.14	37

5.2.4	Agenda Item 1.15	37

5.2.5	Agenda Item 1.23	38

5.2.6	Conclusions	39

5.3	Using current and evolving wireless telecommunications technologies in the Maritime environment	39

5.3.1	Conclusions	48

6	Development of new characteristics	49

7	Conclusions	0

8	Recommendations	2

9	Delivering the Plan	3

Glossary of terms

| AIS | Automatic Identification System |
| --- | --- |
| ARPA | Automatic Radar Plotting Aid |
| AtoN(s) | Aid(s) to Navigation |
| CDMA | Code Division Multiple Access |
| COMPASS | China GNSS (not an acronym) |
| CS | Coastal Surveillance (Radar) |
| DGNSS | Differential Global Navigation Satellite System |
| DGPS | Differential Global Positioning System |
| eANSI | Electronic Aids to Navigation System Information |
| ECDIS | Electronic Chart Display Information System |
| EGNOS | European Geostationary Navigation Overlay System |
| eLoran | Enhanced Loran |
| ENC | Electronic Navigation Chart |
| FAA | (US) Federal Aviation Authority |
| GAGAN | GPS-Aided Geo Augmented Navigation (System) (India) |
| GALILEO | European GNSS (not an acronym) |
| GBAS | Ground Based Augmentation System |
| GEO | Geo-stationary Orbit |
| GMDSS | Global Maritime Distress & Safety System |
| GLONASS | Global Navigation Satellite System (Russian Federation) |
| GNSS | Global Navigation Satellite System |
| GPS | Global Positioning System (US) |
| HF | High frequency |
| IALA | International Association of Marine Aids to Navigation and Lighthouse Authorities |
| IBS | Integrated Bridge Systems |
| IEC | International Electrotechnical Commission |
| IGSO | Inclined Geosynchronous Orbit |
| IHO | International Hydrographic Organisation |
| IMO | International Maritime Organization |
| INS | Inertial Navigation System |
| IRNSS | Indian Regional Navigational Satellite System |
| ITU | International Telecommunication Union |
| LEO | Low Earth Orbit |
| LORAN | Long Range Navigation system |
| LRIT | Long Range Identification & Tracking |
| MBOC | Multiplex Binary Offset Carrier |
| MEO | Medium Earth Orbit |
| MF | Medium Frequency |
| MSAS | Multi-Satellite Augmentation System (Japan) |
| PNT | Position , Navigation & Timing |
| QZSS | Quasi-Zenith Satellite System |
| RACON | RAdar BeaCON |
| RAIM | Receiver Autonomous Integrity Monitoring |
| RNAV | Radionavigation |
| SBAS | Satellite Based Augmentation System |
| SDCM | System for Differential Corrections and Monitoring (Russian Federation) |
| SOLAS | Safety of Life at Sea (IMO Convention) |
| TDMA | Time Division Multiple Access |
| UHF | Ultra High Frequency |
| VDR | Voyage Data Recorder |
| VHF | Very High Frequency |
| VTS | Vessel Traffic Services |
| WAAS | Wide Area Augmentation System (US) |
| WWRNP | World Wide Radio Navigation Plan |
| WWRNS | World Wide Radio Navigation System |

- Introduction

  - general

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) was formed in 1957 as a non-government, non-profit making, technical association that provides a framework for aids to navigation authorities, manufacturers and consultants from all parts of the world to work with a common effort to:

      - harmonise standards for aids to navigation systems worldwide;

      - facilitate the safe and efficient movement of shipping, and;

      - enhance the protection of the marine environment.

The functions of IALA include, among other things:

- developing international cooperation by promoting close working relationships and assistance between members;

- collecting and circulating information on recent developments and matters of common interest;

- liaison with relevant inter-governmental, international and other organisations.  For example, the International Maritime Organization (IMO), the International Hydrographic Organisation (IHO), the International Electrotechnical Commission (IEC), and the International Telecommunication Union (ITU);

- liaison with organisations representing the aids to navigation users;

- addressing emerging navigational technologies, hydrographic matters and vessel traffic management;

e-Navigation

e-Navigation is an International Maritime Organization (IMO) led concept based on the harmonisation of marine navigation systems and supporting shore services driven by user needs.

The working definition of e-Navigation as adopted by IMO is :

“e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment”

There are 3 key elements or strands that must first be in place before e-Navigation can be realized:

- Electronic Navigation Chart (ENC) coverage of navigational areas,

- a robust electronic positioning (navigation and timing) system (with redundancy), and

- an agreed infrastructure of communications to link ship and shore.

Vision

The aim of IALA is:

Fostering the safe, economic and efficient movement of vessels by improvements and harmonisation of aids to navigation worldwide

while the Vision of IALA is

Safe marine navigation in a world of:

- Larger and faster ships

- Changing economy & technology

- Stringent standards

- Holistic approach (e-NAV)

- Changing waterway use

With this in mind, IALA has taken an initiative as part of the strategy for the future of e-Navigation by developing a World Wide Radio Navigation Plan supported by this  World Wide Radio Communication Plan.

  - scope and objectives

The 3 key elements for e-Navigation are identified above. However, this document focuses mainly on the need for an agreed infrastructure of communications between ships and ashore. It presents IALA’s view on current, developing and future Radio Communication Systems for the maritime sector.

e-Navigation will require appropriately designed radio communication systems for a robust and reliable service to the mariner. This document is aimed at assisting in the formulation of policy for National and International spectrum allocation and usage.

It is also provided for IALA members and other administrations to assist them in offering their proposed use of radio spectrum as part of the on going studies at ITU.

The core objective of this document is to state the IALA vision for the efficient use of Radio Spectrum in the Maritime Mobile Service.

- Background to Service Provision

  - general

The challenge facing radio communication in the maritime sector is pressure on the availability of spectrum from other users and the requirement to relay more and more information over fewer and fewer channels.

Spectrum as a finite resource is under increasing demand from all users globally. Recent trends from national licensing/regulatory authorities have been to use pricing/cost to drive efficiency improvements.

Scenario of private provision of MMS

The moves from analogue to digital and from voice to data show the way forward for future radio communications. Time sharing of voice and data communications on a common channel is a recent technological advance that can be applied to e-Navigation.

Recent advances with TDMA techniques, AIS is the prime example, have shown the way to have several hundred users effectively and efficiently using only two channels to share vast amounts of data.

Selective of appropriate bandwidth given the constraints of required range, data rates and channel availability means that narrow band techniques should be considered.

  - institutional

Significant changes have occurred in the world wide marine institutional environment during the last five years:

In the near future, there will be a need for ever-closer co-operation between IMO, IHO and IALA in pursuit of the emerging e-Navigation concept.

Close co-ordination with ITU will be required to achieve the step change that e-Navigation could deliver.

- 2.3	REGULATORY

The carriage requirement for radio communication equipment comes from IMO. This includes AIS, GMDSS, DSC and voice communications and radar systems.

Radio Spectrum is regulated by ITU this includes not only the frequencies but the technologies and standards for the systems employed.

The WWRCP has meet the requirements of both ITU and IMO when meeting the needs of the maritime industry.

There are moves towards charging “market rates” for the use of radio spectrum in some countries and proposals may be put forward for adoption of this approach internationally.

- 2.4	commercial

Growth of the world fleet places greater stress on the ability of the existing radio communications systems to cope with the traffic. Voice systems that occupy a channel for the duration of the call can be viewed as an inefficient means of relaying information. As the number of vessels increases, a saturation point will be reached where no more calls can be made. Systems using TDMA, such as AIS, can cope with multiple simultaneous users and are much more immune to overloading due to growth in the number of participants.

The option of more channels in the Maritime Mobile Service does not exist so alternatives have to be found to cater for future growth.

- 2.5	OPERATIONAL

There are many changes taking place in the operational environment that present new challenges including:

IMO’s and IALA’s support of e-Navigation

the widespread and growing reliance upon GNSS and its role underpinning navigation, situational awareness and communications for e-Navigation;

Growing deployment of local and specific Traffic Management Schemes to meet ever more stringent requirements at higher capacity levels; and

the balance between traditional navigation skills and the role of new technological advances such as ECDIS and IBS.

The introduction of GNSS has encouraged mariners to navigate in areas and conditions in which they would not previously have done and the introduction of e-Navigation will further change the way that ships operate.  As part of its introduction, it is essential to understand what happens when key e-Navigation components (e.g. GNSS) fail or are denied.  Getting the human factors part of this right is also critical: before adopting the technology - safety, liability, on-board training and duty of care must all be considered.

e-Navigation means that international bodies (IMO, IHO, IALA etc) must work more closely together as the concept encompasses all their areas of responsibility.

- 2.6	technical

Significant changes to underpinning services and systems are expected over the next two decades:

development of data communication systems by ITU

development of AIS satellite detection

regulatory constraints from ITU, interference

compatibility with existing systems and services

application of Wi-Fi and Wi-Max in short range (port)

adding data services to current voice channels.

Change from analogue voice to digital voice

- The IALA World Wide Radio Communication Plan

  - OVERVIEW

The maritime domain uses a wide range of communications technologies across the radio spectrum in order to support safe navigation, efficient operations and to provide commercial opportunities. However, many of these technologies have been developed with a single application in mind. As a result a vessel needs to carry many different forms of radio in order to be able to receive relevant data to allow it to undertake its voyage.

In the e-Navigation environment there is the opportunity to plan the maritime communications system architecture afresh. This requires an assessment of the likely applications of communications as well as an understanding of the current radio spectrum available to the community. The following sections consider the current radio spectrum and the use made of it and then move on to consider what applications are likely to run on top of the future communications infrastructure. This section gives a brief summary of the existing and future Radio communication systems..

.

## General overview of spectrum usage by band for existing communication system technologies

### Low Frequency Band (LF)

There is some use of the LF radio spectrum by the maritime community. The Loran-C system broadcasts at 100 kHz and is used for radio navigation.

### Medium Frequency/ High Frequency Band (MF/HF)

Various uses are made of the MF/HF radio spectrum by the maritime community for communication of voice and data, in ship-ship, shore-ship and ship-shore modes of operation:

MF/HF transmissions support both general and distress related voice communications. General voice communication takes place across the band 1.6-26.5MHz, whilst distress related communications are consigned to a small set of specific channels. Channel widths are typically 3 kHz.

#### Digital Selective Calling (DSC )

DSC is a technique using digital codes which enables a radio station to establish contact with, and transfer information to, another station or group of stations, for distress or general communications over medium to long range distances. DSC is primarily intended to initiate ship-to-ship, ship-to-shore and shore-to-ship radiotelephone and MF/HF radiotelex calls. DSC calls can also be made to individual stations, groups of stations, or "all stations" in one's reach. Each DSC-equipped ship, shore station and group is assigned a unique 9-digit Maritime Mobile Service Identity. DSC distress alerts, which consist of a preformatted distress message, are used to initiate emergency communications with ships and rescue coordination centers. DSC was intended to eliminate the need for persons on a ship's bridge or on shore to continuously guard radio receivers on voice radio channel 2182 kHz used for distress, safety and calling. 
Six specific MF/HF frequencies are also set aside for Digital Selective Calling (DSC) distress communications, one in each communication sub-band. DSC is an element of the Global Maritime Distress Safety System (GMDSS).

#### Voice Communication

Various uses are made of the MF/HF radio spectrum by the maritime community for communication of voice in ship-ship, shore-ship and ship-shore modes of operation. General voice communication takes place across the band 1.6-26.5MHz. Channel widths are typically 2.8 kHz. Digital voice communication within the MF/HF bands is relatively new technology with limited use however growing with future technologies.

#### Data Communication

Current and emerging HF digital modulation schemes are breathing new life into data transmission in this frequency band (1.6 to 26.5 MHz). Data services such as those provided by Global Wireless offer data broadcast at rates approaching 6 kbps through the use of orthogonal frequency division modulation (OFDM) with each sub-carrier over a 3 kHz channel modulated with quadrature phase shift keyed (QPSK) data.

Whilst not currently under consideration for maritime services, broadcasters have developed a digital HF (and MF/LF) technology known as Digital Radio Mondiale (DRM) which can offer data rates in excess of 20 kbps in a 10 kHz channel.  Such techniques may offer additional future capability.

#### Narrowband Direct Printing (NBDP)

NBDP is a technique which automates radio signals to telegraphy.NBDP (also known as radio telex) is FSK modulated onto HF channels of 0.5 kHz and supports low speed data transmissions.

NBDP uses specific MF/HF channels for the promulgation meteorological warnings and forecasts for ships.

NBDP is not longer supported by IMO but is still provided in some regions. (Is this still part of GMDSS?)

#### Telex over Radio (TOR/SITOR)

Telex over Radio (TOR) and Simple TOR (SITOR) are used to describe a method of sending Maritime Safety Information using text communication techniques operating as FSK modulation in the MF/HF bands and having it automatically printed. They provide for bi-directional communication using a 0.5 kHz bandwidth. This technology is nearly obsolete.

#### Navigational Telex (NAVTEX)

NAVTEX  is an international, automated system for instantly distributing maritime navigational warnings, weather forecasts and warnings, search and rescue notices and similar information to ships. A small, low-cost and self-contained "smart" printing radio receiver installed in the pilot house of a ship or boat checks each incoming message to see if it has been received during an earlier transmission, or if it is of a category of no interest to the ship's master. The frequency of transmission of these messages is 518 kHz in English, while 490 kHz is use to broadcast in local language. The messages are coded with a header code identified by the using alphabets to represent broadcasting stations, type of messages, and followed by two figures indicating the serial number of the message.

#### Differential Global Navigational Satellite System (DGNSS)

The IALA compliant Differential GNSS coastal radio beacon network broadcasts corrections and integrity information to maritime users in the LF/MF bands (between 283.5 and 325 kHz). Data rates can be 50, 100 or 200 bps. This is a one way data broadcast system from shore to ship.

### Very High Frequency Band (VHF)

Voice communication using the maritime VHF band (156.025 to 161.950 MHz) is prevalent and the primary means of ship-shore and ship-ship communication in the domain. It is used for distress, safety information and general communications. Channel spacing is currently 25 kHz although there has been consideration to reducing this to 12.5 kHz or smaller.

#### Digital Selective Calling (DSC )

DSC is a technique using digital codes which enables a radio station to establish contact with, and transfer information to, another station or group of stations, for distress or general communications within line of site using channel 70 (156.525 MHz). DSC is primarily intended to initiate ship-to-ship, ship-to-shore and shore-to-ship radiotelephone and VHF radiotelex calls. DSC calls can also be made to individual stations, groups of stations, or "all stations" in one's reach. Each DSC-equipped ship, shore station and group is assigned a unique 9-digit Maritime Mobile Service Identity. DSC distress alerts, which consist of a preformatted distress message, are used to initiate emergency communications with ships and rescue coordination centers. DSC was intended to eliminate the need for persons on a ship's bridge or on shore to continuously guard radio receivers on voice VHF channel 16 (156.8 MHz) used for distress, safety and calling.

#### Voice communication

Voice communication using the maritime VHF band (156.025 to 161.950 MHz) is prevalent and the primary means of ship-shore and ship-ship communication in the domain. It is used for distress, safety and general communications. Primary channels used for distress and safety communications are Ch 6, Ch 13, Ch 16 and Ch 70.  Channel spacing is currently 25 kHz although there has been consideration to reducing this to 12.5 kHz or smaller.

#### 121.5MHz Locating Beacon

The frequency 121.5 MHz is an aeronautical emergency frequency. 121.5 MHz radiobeacons were developed in the mid-seventies for installation on aircraft, as Emergency Locator Transmitters (ELTs). However, they can also be used on board ship as Emergency Position-Indicating Radio Beacons (EPIRBs) or as Personal Locator Beacons (PLBs).

#### Automatic Identification System (AIS)

AIS is a system used by ships and shore authorities principally for identification and locating vessels. AIS provides a means for ships to electronically exchange ship data including: identification, position, course, and speed, with other nearby ships and shore stations. This information can be displayed on a screen display. AIS is intended to assist the vessel's watch standing officers and allow maritime authorities to track and monitor vessel movements. AIS uses VHF Channels AIS 1 (161.975 MHz) and AIS 2 (162.025 MHz).

#### Automatic Identification System Aids to Navigation (AIS AtoN)

AIS is also used to exchange of navigation information from Aids to Navigation, meteorological and hydrological data, and binary messages.

#### Automatic Identification System Search and Rescue Transmitter (AIS-SART)

The AIS-SART is used to locate a survival craft or distressed vessels by sending  position reports using a standard AIS class A message 1. The position and time synchronization of the AIS-SART is derived from a built in GNSS receiver.

##### Regional Data Communication Systems

In various regions VHF data communication systems exists for data exchange between shore to ship and ship to shore. Those systems are used primarily for vessel tracking, search areas in SAR operation, etc.   An example of this technology is the Blue Box used in Italy for vessel monitoring.

### Ultra High Frequency Band / Super High Frequency Band (UHF/SHF)

Various uses are made of the UHF radio spectrum by the maritime community for communication of voice and data, in ship-ship, shore-ship and ship-shore modes of operation:

#### Emergency Position Indicating Radio Beacon (EPIRB)

Distress radio beacons, also known as emergency beacons, are tracking transmitters which aid in the detection and location of boats, aircraft, and people in distress. They are radio beacons that interface with Cospas-Sarsat, the international satellite system for search and rescue (SAR). When activated, such beacons send out a distress signal that, when detected by non-geostationary satellites, can be located by triangulation.

An Emergency Position-Indicating Radio Beacon (EPIRB) is used to signal maritime distress to the COSPAS/SARSAT satellite system for the purposes of notifying Search and Rescue (SAR) organisations. EPIRBs transmit a 144-bit message including 49 bits of identification plus position information periodically to the satellites at 406 MHz on a 170 kHz wide channel (actually centred at 406.05 MHz).

#### Voice Communication (on board communication)

UHF radios are commonly used onboard vessels for internal communications and for communications with workers on the dock or berth when alongside. These radios are typical constrained to radiating less than 2W and are for voice communication only.

#### Satellite Voice and Data Communication

Satellite communications in the UHF band are commonly deployed on vessels to fulfil a number of distress, safety and general communications purposes. Satellite solutions typically breakdown into a number of categories, Geostationary wide area satellites such as those provided by Inmarsat, Inmarsat is an international satellite communications company providing voice telephony and high speed internet connections for use on land, sea and in the air. Very Small Aperture Terminals (VSAT) within a regional area and global coverage systems namely Iridium.

Satellite communication links are capable of supporting for example digital voice, e-mail, SMS, crew calling, telex, facsimile, remote monitoring, tracking (position reporting), chart and weather updates and FleetNET services.

#### Enhanced Group Call (EGC)

The Inmarsat C maritime mobile satellite system has an inherent capability, known as Enhanced Group Call (EGC), which allows broadcast messages to be made to selected groups of ship stations located anywhere within a satellite's coverage. Four geostationary satellites provide worldwide coverage for Safety NET. SafetyNET, along with NAVTEX, is recognized by the GMDSS as the primary means for disseminating maritime safety information. Ships regulated by the Safety of Life at Sea Convention travelling outside areas covered by NAVTEX must carry an Inmarsat C SafetyNET receiver.

#### Long Range Identification and Tracking (LRIT)

LRIT is an IMO mandated scheme through which all passenger ships, cargo ships and mobile offshore drilling units engaged on international voyages must report their position on a regular basis (at least 4 times a day) to their flag administration. The data from the vessel can be augmented with additional information by the shore based authorities. Other States may then request this information from the flag administration.

#### Global Navigational Satellite System (GNSS)

Global Navigation Satellite Systems (GNSS) are used not only for positioning, navigation and timing (PNT) but also for short message communication and as an essential input into other ship systems. Current signals are in the frequency-band around 1 GHz to 2.5 GHz

#### X-band and S-band RADAR, RADAR Beacon (RACON) and Radar SART

Radar systems are commonly deployed and typically operate in two bands: X-band from 9.2 GHz to 9.5 GHz and S-band from 2.9 GHz to 3.1 GHz. The radars are used for target identification and for coastal and port navigation. These bands are also used by radar transponders, namely the Racon and radar Search and Rescue Transponder (SART) both create identifiable patterns when interrogated by vessel radars. Racons are used to highlight the location of a visual Aid to Navigation (AtoN) on a mariner’s radar screen and a radar SART is used to aid homing on to a casualty in a SAR operation. Racons operate in S- and X-band and SARTs are X-band.

The radar-SART is used to locate a survival craft or distressed vessel by creating a series of dots on a rescuing ship's radar display. A radar-SART will only respond to a 9 GHz X-band (3 cm wavelength) radar.

#### Other communication technologies using the UHF/SHF band

- Various other communication technologies in the UHF to SHF bands are being used (or considered) for general maritime communications, namely GSM/GPRS, 3G, 4G, WiFi, WiMax and ZigBee links.  These all offer the possibility of various levels of data transfer up to Mbytes/second that could support a number of applications that demand higher bandwidth. However, it should be noted that the coverage of most of these systems is limited and they would therefore be confined to supporting data transfer within a port or harbour environment.

## Future development

### Low Frequency Band (LF)

#### Enhanced Long Range Aid to Navigation (eLORAN)

In the future the proposed enhanced Loran (eLoran) system might provide both better performance and a data channel modulated onto the approximately 100 kHz signals. Two formats for this data channel are currently available, known as Eurofix and 9th pulse respectively. Both techniques offer data rates below 100 bps although higher rate concepts have been proposed.

### Medium Frequency/ High Frequency Band (MF/HF)

#### Digital data communication using MF/HF band

- The likely increase in ship traffic in polar regions, as a result of receding ice fields may also create new potential for HF communications, since geostationary satellites do not cover these areas. Global Wireless is studying increased data rates using 3 kHz channels which may produce data rates of 19.2 kbps.

### Very High Frequency Band (VHF)

#### Automatic Identification System (AIS)

##### Additional AIS Channels

- Two internationally dedicated frequencies (with improved location in the band and improved protection) reserved for exchange of navigational data proper

- One or two internationally dedicated frequencies for exchange of application specific messages, only (ship-ship, ship-shore, shore-ship)

AIS 3rd Channel

- The proposed 3rd AIS Channel would be used for improved satellite detection using a new message.

AIS 4th Channel

- The proposed 4th AIS Channel would be used for binary applications for e-Nav to avoid increasing the channel load on AIS 1 and AIS 2.

##### AIS Maritime Mobile Band channel management

- A universal method for channel management of the 3G maritime services could be archived by using VHF Ch 70 in an AIS messaging format.

- the co-existence in accordance with Recommendation ITU-R M.822)

#### Digital data communication

The use of VHF for digital data transfer has yet to be broadly implemented by the maritime community. A shore based network of digital transceivers has been installed in Norway, but has yet to see wider adoption. The Norwegian system operated by Telenor deploys a network of radio modems capable of switching between nine narrowband duplex VHF channels in the maritime mobile band or else these nine channels can be combined into one 200 kHz wideband channel. The system claims to offer the capacity for hundreds of ships serving applications ranging from providing e-mail through point of sale services to limited browsing of web pages.

Other investigations are on-going into other techniques to be applied more widely, from 25 kHz bandwidth systems providing 43 kbps data rates through to 100 kHz systems offering nearly 400 kbps. Further consideration of how best to achieve spectral efficiency in this band is worthwhile and needs to address channel spacing, modulation schemes and the integration of channels to provide broadband equivalent data rates.

#### Digital voice communication

- Digital voice communication is expected to replace the present analogue VHF voice communication service, i.e. ship-ship/ship-shore/shore-ship.

### Ultra High Frequency Band / Super High Frequency Band (UHF/SHF)

#### Satellite communication using the UHF/SHF band

In future, GNSS such as GPS3 , GALILEO and COMPASS will function as MEO SAR system in emergency, which will be connected with current GMDSS by the MEO LUTs.

#### GPRS/CDMA/3G mobile wireless communications

IALA encourages the maritime communications to be compatible with public mobile wireless communications like GPRS/CDMA/3G by adding some security measures, because the systems are developing very much, and has used in port and coastal area by mariner themselves.

#### Internet access

##### Data transfer based on Mobile IP

Based on mobile IP net, all the ship and shore devices can be seamlessly connected to the Internet with efficient data exchanges. This function is for the ships in port, coastal areas and inland waterways.

##### Shipboard wireless LAN

On board, many marine devices should be combined together with ship wireless LAN with Ad hoc techniques for the efficient data exchanges in ship, ship to shore and ship to ship.

## Geographical classification of e-Navigation data and voice communication techniques

The illustration “Geographical classification of e-Navigation data and voice communication techniques” lists, in the column headings, the communications techniques which are presently available for maritime voice and data communications. It also includes some techniques which may become available in the short to medium term.

Six geographical regions are listed at the left column, defined by the ranges of a selection from the available communications technologies. The GMDSS sea areas which approximately correspond are also noted in the second column.  Separate tables are provided for ship-shore and for ship-ship communications.

As explained elsewhere in this document, the existing and possible future technologies are characterised by differing performance parameters. These include range, bandwidth, latency, and the need or not for shore facilities.

The suggested most likely candidates for voice and data communications, for e-Navigation, in each of the six geographical areas are indicated by yellow colour, with “E” for existing technologies, and “F” for future technologies.

Bill’s seamless communication

Some existing marine communication technologies, such as AIS, automatically carry out data communications without any need for the ship’s crew to intervene in the communications process. (In the specific case of AIS, the crew must set certain voyage parameters at the start of each voyage, but the communications process is automatic.) Other existing marine communications technologies, such as VHF voice communications, require the ship’s crew to adjust the communications apparatus, in the case of VHF voice this involves deciding on and selecting the appropriate communications channel.

An ideal e-Navigation communications system would operate automatically, selecting the best communications technology, channel, and characteristics in accordance with the ship’s location, and the type of data to be exchanged.  This automatic process would be managed in accordance with rules which might include the following type of consideration.

- Need to avoid latency – such as when exchanging safety and navigation data with other vessels or receiving it from electronic systems ashore or on the water surface

- Ability to delay the sending or receipt of data – such as the sending of non-urgent administrative data, or the receipt of chart corrections for a subsequent voyage or for a port to be visited later in the present voyage

- Cost of communications technology

- Whether the communications cost is recovered from crew or passengers

- Importance of the data for commercial services on board

- Xxxxx

- Xxxxx

Some of the rules managing this seamless data communications would be programmed into the system, for example those concerned with safety of navigation. Other rules might be set by the ship’s operator in accordance with house rules and the operator’s commercial model. Others might be adjusted from time to time by the ship’s crew. Such an automatic seamless management of e-Navigation communications appears to be essential in the future.

The ship’s crew would have programmed its requirements at the start of the voyage

# Application of communications within e-Navigation

The maritime domain uses communication links for a wide range of purposes, from essential safety of life applications through routine operational activities to commercial applications.

When establishing the communication requirements under e-Navigation it is essential to include safety of life applications, preferable to include operational applications but commercial applications need only be supported if technically, financially and institutionally acceptable. In order to achieve the benefits of e-Navigation as soon as possible the design of the communications architecture will need to focus on a small set of known applications with the flexibility to grow to encompass others in the future as necessary.

Figure 4-1: Overview of maritime communication applications by type

Figure 4-1 above shows some example applications of communication links within e-Navigation broken down between the categories of essential communication application to ensure safety, important communication application for efficient operations and possible commercial communication applications.

Within the area of safety related e-Navigation communications there are a variety of applications ranging from real time data transmission, emergency use only and routine safety related data broadcasts. Applications include:

- AIS position reports and AtoN data – the broadcast of position information from the vessel using AIS messages 1, 2 and 3 as well as the reception of AIS data from the shore and AtoNs.

- Digital Selective Calling – the emergency broadcast of a vessels position. DSC is expected to be used only when required, although a rate of false alarms should be expected.

- Long Range Identification and Tracking - LRIT is an IMO mandated scheme through which all passenger ships, cargo ships and mobile offshore drilling units engaged on international voyages must report their position on a regular basis (at least 4 times a day) to their flag administration. The data from the vessel can be augmented with additional information by the shore based authorities. Other States may then request this information from the flag administration.

- Differential GNSS – is the broadcast of corrections and integrity information from the shore based DGNSS infrastructure to vessels to allow for higher accuracy and integrity GNSS based position solutions.

- NAVTEX – or navigational telex is an automated medium frequency direct-printing service for delivery of navigational and meteorological warnings and forecasts, as well as urgent marine safety information to ships.

- VTS coordination – the use of communications by VTS centres enables the safe and efficient movement of vessels in busy and/or congested waterways through the passage of traffic information and by advising on appropriate times to leave berths, etc.

- Emergency Search and Rescue (SAR) – the use of communications to coordinate SAR activities between vessels, aircraft and operators in an emergency situation can greatly facilitate rescues.

In an e-Navigation environment mariners will be able to use data communication to support a number of routine operational activities ranging from reporting through to updating their electronic charts in real time. Envisaged applications include:

- Weather data – currently weather forecasts are received in various ways, through NAVTEX broadcasts, from commercial radio broadcasts, from weather relayed via the Internet and from specific maritime weather faxes.

- Ship reporting – mandatory reporting is undertaken between a vessel and a port, or between a vessel and a state authority to fulfil various legal responsibilities including obtaining clearances, providing security information, cargo information and various other items. Some ports, such as Southampton allow all data to be provided via electronic submission of one form (via email or Internet) in other ports data will need to be exchanged using various means such as fax or telephone. Under e-Navigation all such information could be provided electronically.

- Notification to coastal States – there are reporting requirements in the event of any environmental problems or emergencies, namely oil spills, etc. that must be provided to coastal States.

- Notification of arrival to ports – clearance must be obtained from a port prior to arrival, also port facilities must be available (e.g. customs and immigration, etc.). Frequently a ships agent in a specific port or country may undertake a number of the calls necessary to secure a vessel’s entry.

- Maritime Information Objects – as foreseen by IALA, an MIO is a dynamic object for display on an ENC such that important new information can be displayed to the mariner in an easily comprehensible format.

- Port and VTS surveillance feeds – the passage of port VTS and dock video camera information to vessels could be highly beneficial both for pilots and for the master when operating in a complex environment. This application is at a very early stage with some products supporting the transmission of VTS images to portable pilot units.

- Electronic chart updates – periodic chart updates of relevance to each vessel should be able to be conveyed to the vessels in a timely fashion.

- Access to vessel & equipment manuals, documents, etc. – online access to up to date vessel and equipment manuals as well as all of the necessary report formats and documents required by the ships crew throughout their voyage could be beneficial.

- Remote maintenance and service – there is the possibility of maintenance organisations being able to provide remote servicing and repair of ship systems through the use of remote access techniques, remote diagnostics, plus being able to see live video of problems.

- Telemedicine – in the event of medical emergencies on-board the vessel it would be advantageous to be able to receive advice from a qualified medic. This could range from voice data through to a full video session.

There are other more commercially oriented use of communications relating to the provision of Internet services to crew and passengers alike as well as supporting commerce on-board passenger vessels.

- Voyage orders – vessel voyage orders are received from the ship owners or their agents dictating the plan for the vessel to maximise utilisation and profitability.

- Commercial port services – this covers a variety of services that could be provided to vessels in port as demonstrated by recent developments in the Far East. It includes, e.g. video channels and entertainment.

- Operational reports – periodic and ad-hoc reports are made to the ship owners and/or their agents. Such reports include daily updates, equipment status reports, arrival and departure estimates, etc.

- Crew personal communications – the availability of Internet and email access on-board vessels provides a new way to allow crew to maintain contact with family and friends and help shipping companies compete to retain skilled crew.

- Crew e-training – high bandwidth communications have the potential to enable crew training whilst at sea including features such as live interaction with tutors through virtual classrooms.

- Cargo telemetry – the provision of real time information regarding the status of high value and perishable cargos has the potential to provide financial benefits to the shipping companies.

- Passenger Internet access – passengers value access to the Internet, email and added value services that run on IP networks.

- Point of sale transactions – being able to support PoS transactions on ferries and other passenger vessels in an efficient fashion provides for risk free retailing.

## Summary

There are a number of different communication technologies in use in the maritime domain which may or may not prove to be beneficial in a future e-Navigation environment. All offer varying characteristics such as data rates, coverage areas, institutional ownership and cost that lend themselves to supporting different applications, see Table 4-1 below.

| Technology | Typical Data rates | Coverage | Ownership | Infrastructure cost to maritime | Vessel Cost |
| --- | --- | --- | --- | --- | --- |
| eLoran data channel | 100 bps | Regional | Public | Medium | Low |
| HF data link | 5 kbps | Regional | Public / 3rd Party | Medium | Low |
| AIS | 19.2 kbps | Global | Public | High | Low |
| VHF data link | 50 – 150 kbps | Coastal | Public / 3rd Party | High | Low |
| Satellite | 432 kbps | Global | 3rd Party | Low | High |
| Terrestrial mobile network | 384 kbps | Port | 3rd Party | Low | Medium |
| Terrestrial wireless network | 75 Mbps | Port | 3rd Party | Low | Medium |

Table 4-1: Summary of communications technologies

An examination of the future communication requirements in two busy areas around the coast of the UK under assumptions of vessel and communication traffic growth due to e-Navigation suggests that the purely safety related applications have very modest bandwidth requirements. This low bandwidth requirement means that many technologies have the potential to support them. There could be merit in considering the use of AIS as a means of conveying much of this information given its institutional ownership and operation as a safety related system in its own right.

The range of applications that could be classed as supporting the operation of the vessel are more data intensive, but still do not represent an onerous level of information to be transferred across foreseen data links. In a coastal environment a VHF data link solution has the potential to replicate current services provided via VHF voice communications with the addition of other broadcast services that currently utilise HF/MF data links. Further from the shore a solution with better coverage will be required and for practical purposes (e.g. to share costs with commercial communication applications) a satellite based solution may be preferable.

| Application | Ocean | Coastal | Port | Notes |
| --- | --- | --- | --- | --- |
| Safety | AIS, Satellite, VHF DL*, HF DL** | AIS, VHF DL | AIS, VHF DL | * for vessel to vessel comms ** where beyond satellite coverage |
| Operational | Satellite, VHF DL* | VHF DL, Satellite | VHF DL, WiMax / 3G / 4G | * for vessel to vessel comms |
| Commercial | Satellite | Satellite | Satellite, WiMax / 3G / 4G |  |

Table 4-2: Candidate technology in e-Navigation environment

Purely commercial applications of communication links often require high data rates and low latency. The data rates of those examined are too high to be supported practically by a terrestrial VHF network, but could be supported by future satellite networks, future terrestrial mobile networks and current terrestrial wireless networks. For coastal traffic, such as domestic and cross channel ferries a terrestrial wireless network could constitute a practical means of supporting the many commercial applications demanded by crew and passengers. However, it may be the case that such networks are likely to remain the preserve of ports and may not see the necessary infrastructure deployed by the operators to provide coastal coverage in light of the low levels of traffic (and therefore revenue) to be obtained from marine applications.

Appropriate candidate technologies for each class of application by the location of the vessel are shown in Table 4-2 above.

It should be noted that the scenario analysis presented is predicated on a number of assumptions and presents average order of magnitude data rates under typical conditions. Peak data rates are likely to vary from this estimate and there is therefore value in further consideration (through for example simulations) of how best to handle any additional traffic. Message scheduling techniques or separate communication channels for different data types (e.g. safety versus operational or broadcast versus bi-directional) may well be merited. Other considerations may also drive the provision of excess capacity in the shore side infrastructure in order to ensure that safety is never compromised by operational and commercial services.

# Securing the spectrum

## Situation with respect to existing spectrum usage

Section 3.2 described the existing maritime technologies which use the radio spectrum and indicated the bands in which they operate.  Whilst there is some ongoing consideration of variations to the technologies which may be used, all such variations take, as their base assumption, the ongoing use of existing spectrum allocations.  These variations may give rise to a need to change the channelisation within certain bands (this is discussed in more detail in section 5.2 as it is one of the agenda items for the forthcoming world radio conference); however there are no indications, at present, that any additional or any different spectrum is required.

The table below sets out the various current maritime frequency allocations.

| Frequency | Region 1 Allocation | Region 2 Allocation | Region 3 Allocation | Status of Maritime Use |
| --- | --- | --- | --- | --- |
| 90 – 110 kHz | RADIONAVIGATION 5.62 Fixed 5.64 |  |  | No constraints |
| 285 – 315 kHz | AERONAUTICAL RADIONAVIGATION MARITIME RADIONAVIGATION (radiobeacons) 5.73 5.72 5.74 | AERONAUTICAL RADIONAVIGATION MARITIME RADIONAVIGATION (radiobeacons) 5.73 |  | No constraints |
| 315 – 325 kHz | AERONAUTICAL RADIONAVIGATION Maritime radionavigation (radiobeacons) 5.73 5.72 5.75 | MARITIME RADIONAVIGATION (radiobeacons) 5.73 Aeronautical radionavigation | AERONAUTICAL RADIONAVIGATION MARITIME RADIONAVIGATION (radiobeacons) 5.73 | Secondary in Region 1 |
| 415 – 435 kHz | MARITIME MOBILE 5.79 AERONAUTICAL RADIONAVIGATION 5.72 | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.80 5.77 5.78 5.82 |  | No constraints |
| 435 – 495 kHz | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.72 5.82 | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.80 5.77 5.78 5.82 |  | No constraints |
| 495 – 505 kHz | MOBILE 5.82A 5.82B |  |  | No constraints |
| 505 – 510 kHz | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION 5.72 | MARITIME MOBILE 5.79 | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION Aeronautical mobile Land mobile | No constraints |
| 510 – 525 kHz | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION 5.72 | MOBILE 5.79A 5.84 AERONAUTICAL RADIONAVIGATION | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION Aeronautical mobile Land mobile | No constraints |
| 1606.5 – 1625 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 | BROADCASTING 5.89 5.90 | FIXED MOBILE RADIOLOCATION RADIONAVIGATION 5.91 | Region 1 only and subject to restrictions on permitted coverage area |
| 1635 – 1705 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 5.96 | FIXED MOBILE BROADCASTING 5.89 Radiolocation 5.90 | FIXED MOBILE RADIOLOCATION RADIONAVIGATION 5.91 | Region 1 only and subject to restrictions on permitted coverage area |
| 1705 – 1800 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 5.96 | FIXED MOBILE RADIOLOCATION AERONAUTICAL RADIONAVIGATION | FIXED MOBILE RADIOLOCATION RADIONAVIGATION 5.91 | Region 1 only and subject to restrictions on permitted coverage area |
| 2045 – 2065 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | FIXED MOBILE |  | Region 1 only |
| 2065 – 2107 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | MARITIME MOBILE 5.105 5.106 |  | No constraints |
| 2107 – 2160 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | FIXED MOBILE |  | Region 1 only |
| 2170 – 2173.5 kHz | MARITIME MOBILE |  |  | No constraints |
| 2190.5 – 2194 kHz | MARITIME MOBILE |  |  | No constraints |
| 2625 – 2650 kHz | MARITIME MOBILE MARITIME RADIONAVIGATION 5.92 | FIXED MOBILE |  | Region 1 only |
| 4000 – 4063 kHz | FIXED MARITIME MOBILE 5.127 5.126 |  |  | No constraints |
| 4063 – 4438 kHz | MARITIME MOBILE 5.79A 5.109 5.110 5.130 5.131 5.132 5.128 |  |  | No constraints |
| 6200 – 6525 kHz | MARITIME MOBILE 5.109 5.110 5.130 5.132 5.137 |  |  | No constraints |
| 8100 – 8195 kHz | FIXED MARITIME MOBILE |  |  | No constraints |
| 8195 – 8815 kHz | MARITIME MOBILE 5.109 5.110 5.132 5.145 5.111 |  |  | No constraints |
| 12230 – 13200 kHz | MARITIME MOBILE 5.109 5.110 5.132 5.145 |  |  | No constraints |
| 16360 – 17410 kHz | MARITIME MOBILE 5.109 5.110 5.132 5.145 |  |  | No constraints |
| 18780 – 18900 kHz | MARITIME MOBILE |  |  | No constraints |
| 19680 – 19800 kHz | MARITIME MOBILE 5.132 |  |  | No constraints |
| 22000 – 22855 kHz | MARITIME MOBILE 5.132 5.156 |  |  | No constraints |
| 25070 – 25210 kHz | MARITIME MOBILE |  |  | No constraints |
| 26100 – 26175 kHz | MARITIME MOBILE 5.132 |  |  | No constraints |
| 156 – 156.4875 MHz | FIXED MOBILE except aeronautical mobile (R) 5.226 | FIXED MOBILE 5.225 5.226 |  | Operation protected by footnote 5.226 |
| 156.4875 MHz – 156.5625 MHz | MARITIME MOBILE (distress and calling via DSC) 5.111 5.226 5.227 |  |  | No constraints |
| 156.5625 – 156.7625 MHz | FIXED MOBILE except aeronautical mobile (R) 5.226 | FIXED MOBILE 5.225 5.226 |  | Operation protected by footnote 5.226 |
| 156.7625 MHz – 156.8375 MHz | MARITIME MOBILE (distress and calling) 5.111 5.226 |  |  | No constraints |
| 156.8375 - 162.050 MHz | FIXED MOBILE except aeronautical mobile (R) 5.226 5.227A 5.229 | FIXED MOBILE 5.226 5.227A 5.230 5.231 5.232 |  | Operation protected by footnote 5.226 |
| 406 – 406.1 MHz | MOBILE-SATELLITE (Earth-to-space) 5.266 5.267 |  |  | No constraints |
| 456 – 459 MHz | FIXED MOBILE 5.286AA 5.271 5.287 5.288 |  |  | Footnote 5.287 specifies maritime mobile use |
| 460 – 470 MHz | FIXED MOBILE 5.286AA Meteorological-satellite (space-to-Earth) 5.287 5.288 5.289 5.290 |  |  | Footnote 5.287 specifies maritime mobile use |
| 1525 – 1530 MHz | SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Mobile except aeronautical mobile 5.349 5.341 5.342 5.350 5.351 5.352A 5.354 | SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Fixed Mobile 5.343 5.341 5.351 5.354 | SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Mobile 5.349 5.341 5.351 5.352A 5.354 | Complex |
| 1525 – 1535 MHz | SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.353A Earth exploration-satellite Fixed Mobile except aeronautical mobile 5.341 5.342 5.351 5.354 | SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.353A Earth exploration-satellite Fixed Mobile 5.343 5.341 5.351 5.354 |  | Complex |
| 1535 – 1545 MHz | MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.341 5.351 5.353A 5.354 5.355 5.356 5.357 5.357A 5.359 5.362A |  |  | Complex |
| 1616 – 1626.5 MHz | MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) 5.208B 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.371 5.372 | MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION RADIODETERMINATION- SATELLITE (Earth-to-space) Mobile-satellite (space-to-Earth) 5.208B 5.341 5.364 5.365 5.366 5.367 5.368 5.370 5.372 | MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) 5.208B Radiodetermination-satellite (Earth-to-space) 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.372 | Complex |
| 1626.5 – 1646 MHz | MOBILE-SATELLITE (Earth-to-space) 5.351A 5.341 5.351 5.353A 5.354 5.355 5.357A 5.359 5.362A 5.374 5.375 5.376 |  |  | Complex |
| 2900 – 3100 MHz | RADIOLOCATION 5.424A RADIONAVIGATION 5.426 5.425 5.427 |  |  | No constraints |
| 9200 – 9300 MHz | RADIOLOCATION MARITIME RADIONAVIGATION 5.472 5.473 5.474 |  |  | No constraints |
| 9300 – 9500 MHz | RADIONAVIGATION EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) RADIOLOCATION 5.427 5.474 5.475 5.475A 5.475B 5.476A |  |  | No constraints |

Table 5-3: Maritime frequency allocations

The key maritime mobile bands are generally well protected with no specific indication that further effort is needed to secure their continued use (other than that discussed in section 5.2 below).  The situation with respect to maritime mobile-satellite use is complex, however use for GMDSS is protected via footnote 5.535A and thus there appears no particular need to take action other than to safeguard the protection offered by this footnote.

However in one or two cases there may be pressure to modify maritime usage. Issues which may arise include:

- The on-board UHF frequencies in the range 457.525 to 457.600 and 467.525 to 467.825 MHz are directly adjacent to proposed bands for 450 MHz cellular systems and further are cited as future bands for IMT-2000 technology both by the ITU and European Commission (as per the WAPECS policy).  [ITU WP-8F].

- There is increasing pressure on administrations to permit the use of power line telecommunications (PLT) also known as broadband over power line (BPL).  This is a technology which transmits on HF frequencies over domestic power distribution networks either from sub-stations to homes or around homes.  Many of these devices emit excessive radiation on HF frequencies such that the use of such frequencies in the immediate vicinity (e.g. within 200 – 500 metres) becomes exceptionally difficult.  Such devices are not licensed through traditional radio methods as they are not radio transmitters per se, and whilst the emissions they generate should be controlled through various EMC legislations, they are seen as offering socio-economic benefits in excess of value of the loss of HF reception.

In the first of these instances, the maritime community should closely track developments for mobile communication in the 450 MHz band.  In most cases, on-board use is recognised and respected, however there may be moves to over-ride this recognition if pressure on spectrum increases further.  Whilst some of the proposed band plans for this spectrum specifically stop short of suggesting use of the maritime on-board frequencies, they do propose using immediately adjacent frequencies.  Careful consideration will therefore be very necessary in order to ensure that such systems when used, for example, along the coast or in ports, do not cause harmful interference to maritime services.

In the second instance, it is important that the maritime industry is engaged in the debate over PLT/BPL such that, should additional pressure on regulators and administrations to enforce applicable directives be required to protect HF reception, the right communication paths are open.

### Conclusions

There are three key activities which need to take place in reference to existing spectrum allocations:

- General continued protection of the status of the maritime mobile service across the RF spectrum.

- Keeping a watching brief on the (ITU) development of frequency plans for the 450 to 470 MHz band so as to protect on-board use;

- Engaging in discussions concerning the use of PLT/BPL at a national level as and when the possibility or necessity arises.

## WRC-11 agenda items

A number of the agenda items for the 2011 World Radio Conference (WRC-11) have the direct or indirect potential to affect maritime use of the spectrum, namely:

- 1.9 “to revise frequencies and channelling arrangements of Appendix 17 to the Radio Regulations, in accordance with Resolution 351 (Rev.WRC 07), in order to implement new digital technologies for the maritime mobile service”.

- 1.10 “to examine the frequency allocation requirements with regard to operation of safety systems for ships and ports and associated regulatory provisions, in accordance with Resolution 357 [COM6/10] (WRC 07)”

- 1.14 “to consider requirements for new applications in the radiolocation service and review allocations or regulatory provisions for implementation of the radiolocation service in the range 30 - 300 MHz, in accordance with Resolution 611 [COM6/14] (WRC 07)”

- 1.15 “to consider possible allocations in the range 3 - 50 MHz to the radiolocation service for oceanographic radar applications, taking into account the results of ITU R studies, in accordance with Resolution 612 [COM6/15] (WRC 07)”

- 1.23 “to consider an allocation of about 15 kHz in parts of the band 415-526.5 kHz to the amateur service on a secondary basis, taking into account the need to protect existing services”

Some preparatory work has already been done in order to address certain of these agenda items.

### Agenda Item 1.9

Appendix 17 of the ITU Radio Regulations provides a comprehensive channelling arrangement for maritime HF communications, based on the use of either single side-band (SSB), Morse telegraphy or NBDP transmissions (with channel spacings of 3 kHz, 0.5 kHz and 0.5 kHz respectively).  In recognition of the development of digital HF communication technologies and the reduction in use of frequencies for Morse telegraphy, Resolution 351 seeks to review these channelling arrangements in order to enable the use of digital technologies whilst protecting GMDSS.

Globe Wireless are already providing digital communications for shipping, using OFDM techniques which delivers around 6 kbps of data in an existing 3 kHz maritime channel (i.e. with an efficiency of 2 bits per second per Hz).  There are other digital communication technologies in use on the HF band (such as Digital Radio Mondiale, DRM), almost all of which use OFDM techniques to achieve high spectrum efficiency and coping with the complex propagation environment.  As an example, DRM offers the following channel bandwidths and associated throughput and efficiencies:

| Transmission Bandwidth (kHz) | Data Throughput (kbps) | Efficiency (bits/second/Hz) |
| --- | --- | --- |
| 4.5 | Between 6 and 11.3 | Between 1.33 and 2.51 |
| 5 | Between 6.9 and 12.8 kbps | Between 1.38 and 2.56 |
| 9 | Between 12.8 and 23.6 | Between 1.42 and 2.62 |
| 10 | Between 7.6 and 26.6 | Between 0.76 and 2.66 |
| 18 | Between 26.5 and 49.1 | Between 1.47 and 2.73 |
| 20 | Between 16.3 and 55 | Between 0.82 and 2.75 |

It is worth noting that the maximum spectrum efficiency (in bits per second per Hz) increases slightly as the transmission (or channel) bandwidth increases. This is common for digital systems as the wider the channel, the less space is needed between adjacent channels to reduce interference.  Also note that the maximum efficiency of the system is slightly higher than that achieved by the Globe Wireless technology.

The DRM standard is a good analogy for any need to re-channel maritime usage.  Its available transmission bandwidths are designed specifically to fit in with any existing channel spacing which it is likely to encounter across the world.  It is highly likely that any future maritime digital system will be able to offer a similar level of flexibility.  As such, it should be able to operate in a 3 kHz channel or in multiple adjacent 3 kHz channels (or even possibly in multiple non-contiguous channels in the same band), with increasing transmission bandwidth offering slight increases in overall efficiency.

As Morse telegraphy use declines, it would therefore appear to make sense to re-assign some of these frequencies for additional 3 kHz channels, and also modify the Appendix 17 rules to permit the combining of (adjacent) channels to permit wider bandwidth transmissions.

### Agenda Item 1.10

There is a global requirement for application of radiocommunications to enhance ships and ports security. Among the concerns are:

- management and identification of cargo;

- coordination of sensors and monitors;

- rapid detection of dangerous, unauthorised, or compromised shipments; and

- enhanced interaction with both local and national public protection resources.

The International Maritime Organization (IMO) recognised this need by its adoption of the Code on International Ship and Port Facility Security (ISPS), implemented as treaty by amendment to the Safety of Life at Sea (SOLAS) Convention.

The IMO’s Maritime Safety Committee (MSC 81) approved new provisions in Chapter V (Safety of Navigation) of SOLAS for Long Range Identification and Tracking (LRIT) following the adoption of the ISPS Code which also introduced a requirement for Ship Security Alert System (SSAS) and carriage requirements. LRIT information can be used for both security and safety (including SAR activities) and protection of the marine environment.

The Agenda Item specifically aims to address the following issues:

  - Consideration of additional allocations to support the requirements for the operation of ship and port security and maritime safety systems:

    - Below 1 GHz for the maritime mobile service.

    - Between 156 and 162.025 MHz for the maritime mobile-satellite service such as a 3rd AIS frequency dedicated to satellite reception.

  - Consider upgrading the WRC-07 secondary allocations for AIS satellite reception to primary status

  - Compatibility of proposed spectrum allocations with existing communication systems, notably in the fixed and land mobile service.

  - Identify suitable allocations for the transmission of AIS data from satellites to earth stations.

Solutions which have been proposed by CEPT include:

- the satellite detection of AIS, taking into account Resolution 357 as adopted by WRC-07;

- the possible harmonisation of technology for cargo identification and tracking through ITU Recommendations;

- the broadcasting of security levels for ports and coastal waters in the band around 500 kHz;

- the harmonised introduction of new technologies by the Maritime mobile service (in the VHF band) through possible regulatory measures (Resolution 342 rev. WRC-07)

It is the view of CEPT that frequencies, procedures and techniques used by the GMDSS will not be affected by any further developments of Agenda Item 1.10, however there is clearly a strong interlinkage between the development of eNavigation and the topics of the particular Agenda Item.  Indeed work within this Agenda Item may yield opportunities for the medium and long term development of the eNavigation concept through preparing the way for services and technologies which may add to the overall solution-space.

Some of the necessary actions (such as the introduction of new digital HF technologies) will be dealt with through other Agenda Items and Resolutions (e.g. Resolution 342 which considers methods of improving spectrum efficiency for maritime VHF communications); however some of the opportunities presented in this Agenda Item could be significant for eNavigation.  In particular, Resolution 357 aims to investigate the allocation for the maritime mobile-satellite service in the maritime VHF band as well as examining the potential for expanding the band.

It would therefore seem beneficial to support the various studies and working groups feeding into this Agenda Item in order to ensure the best possible outcome for the maritime community.

### Agenda Item 1.14

There is a recognised need to provide spectrum for radiolocation services used, in particular, to track the launch and manoeuvring of spacecraft, detecting space debris and for new aeronautical surveillance services.  Various proposals were put to WRC-07 for 2 MHz of spectrum in the frequency range 30 – 300 MHz, and in particular in the range 138 – 173 MHz (with a particular focus on 154 - 156 MHz) for such services, leading to this Agenda item for the 2011 conference.

Resolution 611 which invites the ITU to investigate this issue, specifically addresses the need to avoid the emergency, AIS and some other VHF maritime mobile channels.  Resolve 2 of the Resolution states:

“that the introduction of new systems in the radiolocation service shall be avoided in the frequency bands 156.4875-156.8375 MHz and 161.9625-162.0375 MHz”

It should be noted, however, that wider maritime VHF usage is not being specifically identified for protection.  Further, with plans to introduce additional AIS channels, the protection afforded by the resolution may not be sufficient.

The CEPT position with respect to this Agenda Item is that the frequency range 142 to 144 MHz is the most suitable candidate band, which would have minimal impact on maritime mobile use.

ITU WP 5B is responsible for this agenda item.  Clearly there is potential for the proposed new services to cause interference to maritime VHF usage over a wide area, even out to sea away from land and it would therefore be prudent to take a watching interest in the work of the group on this matter in order to ensure protection of maritime services.  Any technical studies conducted in order to analyse compatibility with existing services should include suitable compatibility criteria for maritime services.

### Agenda Item 1.15

Oceanographic radar using HF frequencies is growing in popularity as a means of determining sea currents and wave size.  Such devices typically use multiple frequencies across the HF spectrum in order to elicit responses from different wave heights.  They typically occupy over 100 kHz of radio spectrum and use transmitter powers of around 50 Watts.  Though such technology has been in use for over 30 years, improvements in signal processing and transmitter technology have made such devices simpler and cheaper than before such that their usage need no longer be confined to isolated research projects.

At present the use of such radar is authorised locally by the spectrum administration concerned on frequencies ranging from 3 to 30 MHz on a non-interference basis (i.e. they must not cause interference to any other user nor can they claim protection from interference from other spectrum users).  Due, however, to the nature of radio propagation in the HF frequency range, the emissions from such devices have the potential to cause harmful interference to stations thousands of kilometres away and indeed such interference is widely reported.

In order to minimise interference to other users, oceanographic radar users are seeking to identify an exclusive (primary) allocation for their equipment.  Three specific frequency allocations have been requested, around 4.5, 13 and 25 MHz with each band not exceeding 600 kHz.

Resolution 612 invites the ITU to

“complete the necessary studies, as a matter of urgency, taking into account the present use of the allocated band, with a view to presenting, at the appropriate time, the technical information likely to be required as a basis for the work of WRC-11”

Some of the frequency bands being considered (as per the outcome of an ITU WP5B meeting) are:

- 3.5 – 5.5 MHz

- 8 – 10 MHz

- 12 – 14 MHz

- 24 – 30 MHz and

- 39 – 45 MHz.

The amount of spectrum and the frequency ranges concerned have the potential to overlap with maritime mobile use, in particular in the bands 4063 to 4438 kHz, 8100 to 8815 kHz, 12230 to 13200 kHz and 25070 to 25210 kHz.  Clearly operating equipment which is specifically designed to provide signals over the sea has greater potential to cause interference to maritime services than ones destined for use directed over land.  Furthermore, as most installations are close to the coast, there is potential for adjacent channel interference into maritime services for vessels in the vicinity of transmitting stations.

It is unlikely that any final allocation which directly interferes with maritime operation will be selected for these devices given the nature of the conflicts between the service, even adjacent frequency use has the potential to cause problems.  However, other services within the HF spectrum are equally fervent in support of protection for their allocations and counterbalancing any maritime argument is the fact that the service delivers most benefit to the maritime community, either directly or through oceanographic research.

ITU WP 5B is responsible for the necessary work being undertaken with reference to this agenda item and it would be sensible to keep track of the results of any work so as to ensure that any resulting recommendations accurately reflect maritime needs.

### Agenda Item 1.23

The closure of various of the MF transmitter and receiver facilities around 500 kHz (in particular the closure of stations in the Western Hemisphere which monitor the frequency) and the requisite reduction in the use of this area of the spectrum has led to radio amateurs approaching various regulators in order to gain access to a portion of the band.  Whilst usage of this frequency range continues in the Pacific region, the use of other frequencies and of satellite technology are reducing reliance on LF and MF services for maritime communications.

So far the following countries have allowed amateurs (limited) access to frequencies around 500 kHz:

- United Kingdom

- Sweden

- Germany

- Belgium

- Czech Republic

- United States of America

With the exception of NBDP and NAVTEX transmissions on 490 and 518 kHz respectively, frequencies in this area of the spectrum are taking a decreasing level of importance for maritime use.  DGNSS technology is a likely candidate to make use of this band in the near future. Allowing secondary use by amateur services is not in the interest of the maritime service.

ITU WP 5A is responsible for this agenda item. ITU WP 5B should liaise with ITU WP 5A to inform of the maritime use for e-Navigation around 500 kHz.

### Conclusions

There are several important activities which need to take place in order to support maritime input into the 2011 world radio conference.  In particular:

- Support for modifications to Appendix 17 HF maritime channelisation to permit new digital systems;

- Support for a maritime mobile-satellite allocation within the VHF band to enable satellite detection of AIS;

- Careful consideration of the need for spectrum around 500 kHz given the desire to use this for the broadcasting of security levels given pressure for some of this spectrum from the amateur radio community;

- Keeping a watching brief on developments with respect to VHF radiolocation allocations so as to protect maritime VHF use;

- Engagement in the groups considering an HF allocation for oceanographic radar in order to ensure that sufficient protection is afforded to maritime services (especially in light of the likely increasing use of HF for digital technologies).

## Using current and evolving wireless telecommunications technologies in the Maritime environment

There are an ever increasing number of wireless technologies which aim to deliver voice and data connectivity to mobile users and which may be of use in the maritime environment.  Aside from their technical capabilities, there may be regulatory issues and associated technical restrictions which prevent them being fully exploited.  Such issues may include:

- the ITU radio regulations do not permit maritime mobile operation in the frequency bands concerned

- frequencies are licensed on a national basis, such that there is no continuity of operation from country to country (this may even require equipment to be switched off when entering certain jurisdictions)

- the system parameters may have the potential to cause interference to ship-borne equipment

- planning parameters may make use at sea (or even in ports) complex or difficult

The following terrestrial technologies are currently in the process of either being rolled-out or standardised on an international basis and thus may be candidates for use in the maritime environment, particularly for commercial port services:

- Digital PMR (to replace analogue PMR for on-board comms): TETRA, TETRAPOL, P25, dPMR, DMR;

- 3G Mobile: UMTS (TDD & FDD), cdma2000, TD-SCDMA, IEEE802.20 (including iBurst);

- ‘3.5G’ Mobile: WiMAX;

- 4G Mobile: LTE, UMB.

In addition to these commercial service technologies, there are various technologies which may offer potential solutions under the banner of ‘licence exempt’ or ‘low-power’ technologies such as WiFi, Bluetooth, ZigBee and UWB.  The range of such services, however, is exceptionally limited (normally to 100 metres or much less) and the frequencies employed are globally allocated for the purpose (e.g. the 2.4 GHz ISM band) such that there is no need for a specific consideration of the impact on spectrum of their usage in a maritime environment.

Table 5-4 below details, for each of the commercial technologies, the frequency band(s) in which they ‘prefer’ to operate, i.e. those for which there are known services, as well as the general range of frequencies over which they are specified to operate.

| Technology | Frequency Range | Notes |
| --- | --- | --- |
| TETRA | 380 – 400 MHz 410 – 430 MHz 450 – 470 MHz 806 – 821 // 851 – 866 MHz | EN 300 392 |
| TETRA 2 (TEDS) |  | EN 302 561 |
| TETRAPOL | 70 – 933 MHz |  |
| P25 | 136 – 870 MHz |  |
| DMR | 30 – 900 MHz | TS 102 361 |
| dPMR | 30 – 900 MHz | EN 301 166 |
| GSM (incl EDGE) | 380 – 400 MHz 410 – 430 MHz 450 – 470 MHz 478 – 496 MHz 698 – 746 MHz 747 – 792 MHz 806 – 866 MHz 824 – 894 MHz 870 – 921 MHz 876 – 925 MHz 880 – 960 MHz 1710 – 1880 MHz 1850 – 1990 MHz |  |
| cdma2000 | 450 – 470 MHz 790 – 862 MHz 824 – 894 MHz 880 – 960 MHz 1710 – 1880 MHz 1850 – 1990 MHz 1820 – 2170 MHz 2300 – 2400 MHz 2500 – 2690 MHz |  |
| W-CDMA (UMTS) |  |  |
| TD-CDMA | 1900 – 1920 MHz 2010 – 2025 MHz (2570 – 2620 MHz) | ‘TDtv’ applications have been trialled in some EU countries |
| TD-SCDMA |  |  |
| WiMAX | 2 – 11 GHz 10 – 66 GHz | 3400 – 3600 MHz is being seen as a potential target following WRC-07 and EU WAPECS moves |
| 802.20 (MBWA) | Below 3.5 GHz |  |
| WiBro | 2.3 – 2.4 GHz | Currently Korea only |
| iBurst (HC-SDMA) | Below 3.5 GHz | Often uses 1785 – 1805 MHz or 1900 – 1920 MHz |

Table 5-4: Summary of spectrum requirements of new commercial data links

LTE and UMB are 4G extensions to UMTS and cdma2000 respectively. At present, it is envisaged that existing spectrum assignments will be re-farmed such that no new spectrum will be necessary.

From the above, the bands of particular interest, and where there are most likely to be mobile communication systems providing value in maritime applications are:

- 380 to 400 MHz

- 410 to 430 MHz

- 450 to 470 MHz

- 790 to 862 MHz (the ‘Digital Dividend’)

- 824 to 849 // 869 to 894 MHz (US ‘800 MHz’ cellular band)

- 880 to 915 // 925 to 960 MHz (EU ‘900 MHz’ cellular band)

- 1710 to 1785 // 1815 to 1880 MHz (EU ‘1800 MHz’ cellular band)

- 1850 to 1910 // 1930 to 1990 MHz (US ‘1900 MHz’ cellular band)

- 1880 to 1920 MHz (EU ‘3G TDD’ band)

- 1920 to 1980 // 2110 to 2170 MHz (EU ‘3G’ band)

- 2010 to 2015 MHz (EU ‘3G TDD’ band)

- 2400 to 2483.5 MHz (‘2.4 GHz’ licence exempt, low-power band)

- 2500 to 2690 MHz (‘3G expansion’ band)

- 3400 to 3600 MHz

Table 5-5 below shows, based on the latest version of the ITU Radio Regulations, the allocations in those bands (including any footnotes), highlighting the position of maritime services in those bands.

| Frequency (MHz) | Region 1 Allocation | Region 2 Allocation | Region 3 Allocation | Status of Maritime Use |
| --- | --- | --- | --- | --- |
| 380 – 387 | FIXED MOBILE 5.254 |  |  | No constraints |
| 387 – 390 | FIXED MOBILE Mobile-satellite (space-to-Earth) 5.208A 5.208B 5.254 5.255 |  |  | No constraints |
| 390 – 399.9 | FIXED MOBILE 5.254 |  |  | No constraints |
| 410 – 420 | FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-space) 5.268 |  |  | No constraints |
| 420 – 430 | FIXED MOBILE except aeronautical mobile Radiolocation 5.269 5.270 5.271 |  |  | No constraints |
| 450 – 455 | FIXED MOBILE 5.286 AA 5.209 5.271 5.286 5.286A 5.286B 5.286C 5.286D 5.286 E |  |  | No constraints |
| 455 – 456 | FIXED MOBILE 5.286AA 5.209 5.271 5.286 5.286A 5.286B 5.286C 5.286D 5.286 E | FIXED MOBILE 5.286AA MOBILE-SATELLITE (Earth-to-space) 5.286A 5.286B 5.286C 5.209 | FIXED MOBILE 5.286AA 5.209 5.271 5.286 5.286A 5.286B 5.286C 5.286D 5.286 E | No constraints |
| 456 – 459 | FIXED MOBILE 5.286AA 5.271 5.287 5.288 |  |  | No constraints |
| 459 – 460 | FIXED MOBILE 5.286AA 5.209 5.271 5.286 5.286A 5.286B 5.286C 5.286D 5.286 E | FIXED MOBILE 5.286AA MOBILE-SATELLITE (Earth-to-space) 5.286A 5.286B 5.286C 5.209 | FIXED MOBILE 5.286AA 5.209 5.271 5.286 5.286A 5.286B 5.286C 5.286D 5.286 E | No constraints |
| 460 – 470 | FIXED MOBILE 5.286AA Meteorological-satellite (space-to-Earth) 5.287 5.288 5.289 5.290 |  |  | No constraints |
| 790 – 806 | FIXED BROADCASTING MOBILE except aeronautical mobile 5.316B 5.317A 5.312 5.314 5.315 5.316 5.316A 5.319 | BROADCASTING Fixed MOBILE 5.313B 5.317A 5.293 5.309 5.317A | FIXED MOBILE 5.313A 5.317A BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | No constraints |
| 806 – 862 | FIXED BROADCASTING MOBILE except aeronautical mobile 5.316B 5.317A 5.312 5.314 5.315 5.316 5.316A 5.319 | FIXED MOBILE 5.317A BROADCASTING 5.317 5.318 | FIXED MOBILE 5.313A 5.317A BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | No constraints |
| 862 – 890 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 5.319 5.323 | FIXED MOBILE 5.317A BROADCASTING 5.317 5.318 | FIXED MOBILE 5.313A 5.317A BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | No constraints |
| 890 – 902 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 Radiolocation 5.323 | FIXED MOBILE except aeronautical mobile 5.317A Radiolocation 5.318 5.325 | FIXED MOBILE 5.317A BROADCASTING Radiolocation 5.327 | No constraints |
| 902 – 928 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 Radiolocation 5.323 | FIXED Amateur Mobile except aeronautical mobile 5.325A Radiolocation 5.150 5.325 5.326 | FIXED MOBILE 5.317A BROADCASTING Radiolocation 5.327 | No constraints |
| 928 – 942 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 Radiolocation 5.323 | FIXED MOBILE except aeronautical mobile 5.317A Radiolocation 5.325 | FIXED MOBILE 5.317A BROADCASTING Radiolocation 5.327 | No constraints |
| 942 – 960 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 5.323 | FIXED MOBILE 5.317A | FIXED MOBILE 5.317A BROADCASTING 5.320 | No constraints |
| 1710 – 1930 | FIXED MOBILE 5.384A 5.388A 5.388B 5.149 5.341 5.385 5.386 5.387 5.388 |  |  | No constraints |
| 1930 – 1970 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE 5.388A 5.388B Mobile-satellite (Earth-to-space) 5.388 | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 1970 – 1980 | FIXED MOBILE 5.388A 5.388B 5.388 |  |  | No constraints |
| 1980 – 2010 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  | No constraints |
| 2010 – 2015 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.388 5.389C 5.389E | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 2110 – 2120 | FIXED MOBILE 5.388A 5.388B SPACE RESEARCH (deep space) (Earth-to-space) 5.388 |  |  | No constraints |
| 2120 – 2160 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE 5.388A 5.388B Mobile-satellite (space-to-Earth) 5.388 | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 2160 – 2170 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.388 5.389C 5.389E | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 2400 – 2450 | FIXED MOBILE 5.384A Amateur Radiolocation 5.150 5.282 5.395 | FIXED MOBILE 5.384A RADIOLOCATION Amateur 5.150 5.282 5.393 5.394 5.396 |  | No constraints |
| 2450 – 2483.5 | FIXED MOBILE Radiolocation 5.150 5.397 | FIXED MOBILE RADIOLOCATION 5.150 |  | No constraints |
| 2500 – 2520 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A 5.405 5.412 | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A 5.404 | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A MOBILE-SATELLITE (space-to-Earth) 5.351A 5.407 5.414 5.414A 5.404 5.415A | No constraints |
| 2520 – 2535 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.405 5.412 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.403 5.414A 5.415A | No constraints |
| 2535 – 2655 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.405 5.412 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417A 5.417B 5.417C 5.417D 5.418 5.418A 5.418B 5.418C | No constraints |
| 2655 – 2670 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.208B 5.413 5.416 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.412 | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.208B | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.208B 5.420 | No constraints |
| 2670 – 2690 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.412 | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.208B 5.415 MOBILE except aeronautical mobile 5.384A Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) 5.415 MOBILE except aeronautical mobile 5.384A MOBILE-SATELLITE (Earth-to-space) 5.351A 5.419 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 | No constraints |
| 3400 – 3500 | FIXED FIXED-SATELLITE (space-to-Earth) Mobile 5.430A Radiolocation 5.431 | FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile 5.431A Radiolocation 5.433 5.282 | FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile 5.432B Radiolocation 5.433 5.282 5.432 5.432A | Secondary |
| 3500 – 3600 | FIXED FIXED-SATELLITE (space-to-Earth) Mobile 5.430A Radiolocation 5.431 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile Radiolocation 5.433 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.433A Radiolocation 5.433 | Secondary in Region 1 |

Table 5-5: Summary of maritime radio spectrum

None of the footnotes relating to the use of these bands specifically preclude any use for maritime services.

With the exception of the band 3400 – 3600 MHz, the mobile service (which includes, unless explicitly stated, the maritime mobile service) has a primary allocation meaning that the use of these bands for maritime applications will pose no regulatory issues at the ITU.  In the band 3400 – 3600 MHz, many of the footnotes relate to the upgrading of the mobile service from secondary to primary status, however it is known that in some parts of the world (especially in high rainfall areas such as around the equator) this band is still heavily used for satellite downlinks and as such its development for mobile services is likely to remain constrained.

### Conclusions

There does not appear to be any necessity to expend significant effort on the standardisation or regulation of these mobile communication technologies to make them suitable for the maritime community.

# Development of new characteristics

The following tables summarise the status of the various ITU technical characteristics relating to maritime communication applications both for those already existing as well as for those foreseen in e-Navigation. They outline where existing characteristics are likely to need updating or else where new characteristics need to be developed. If changes are required they will take place through the appropriate ITU Study Groups.

ITU Study Groups (SG) are responsible for developing draft recommendations to be provided to the periodic World Radio Conferences (WRC). Study groups are established and assigned study questions by a Radiocommunication Assembly to prepare draft Recommendations for approval by ITU Member States. In the case of e-Navigation two SG’s are likely to be involved in the development of material to be agreed at WRC, namely SG4 and SG5.

SG4 relates to satellite services, covering systems and networks for the following service:

- Fixed-satellite service,

- Mobile-satellite service,

- Broadcasting-satellite service and

- Radiodetermination-satellite service.

SG5 relates to terrestrial services, covering systems and networks for the following services:

- Fixed,

- Mobile,

- Radiodetermination,

- Amateur and

- Amateur-satellite.

Below the SG’s are subgroups, such as Working Parties (WP) and Task Groups (TG) that are established to study the questions assigned to the different Study Groups. For example, SG4 has the following WP’s:

- Working Party 4A - Efficient orbit/spectrum utilization for FSS and BSS

- Working Party 4B - Systems, air interfaces, performance and availability objectives for FSS, BSS and MSS, including IP-based applications and satellite news gathering

- Working Party 4C - Efficient orbit/spectrum utilization for MSS and RDSS

SG5 has the following WP’s:

- Working Party 5A - Land mobile service excluding IMT; amateur and amateur-satellite service

- Working Party 5B - Maritime mobile service including Global Maritime Distress and Safety System (GMDSS); aeronautical mobile service and radiodetermination service

- Working Party 5C - Fixed wireless systems; HF systems in the Fixed and Land Mobile Services

- Working Party 5D - IMT Systems

- Joint Task Group 5-6 - Studies on the use of the band 790-862 MHz by mobile applications and by other services

For maritime e-Navigation applications any necessary revision to, or creation of, technical characteristics will take place in one of two WP’s – SG4 WP4C and SG5 WP5B.

| Application |  | Current characteristics |  | Amendments | Mechanism |
| --- | --- | --- | --- | --- | --- |
| DSC | VHF | ITU-R M.493-12 | Digital selective-calling system for use in the maritime mobile service. | No amendments are foreseen for the use of VHF DSC within the e-Navigation concept as currently envisaged. However, if the VHF voice channels are changed to make them more spectrally efficient (e.g. see VHF voice below) there may be a need to revise the VHF DSC characteristics. | N/A |
|  |  | ITU-R M.541-9 | Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service |  |  |
|  |  | ITU-R M.693 | Technical characteristics of VHF emergency position-indicating radio beacons using digital selective calling (DSC VHF EPIRB) |  |  |
|  |  | ITU-R M.689-2 | International Maritime VHF Radiotelephone Systems with Automatic Facilities Based on DSC Signalling Format |  |  |
|  |  | ITU-R M.821-1 | Optional Expansion of the Digital Selective-Calling System for Use in the Maritime Mobile Service |  |  |
|  | MF/HF | ITU-R M.493-12 | Digital selective-calling system for use in the maritime mobile service. | No amendments are foreseen for the use of HF/MF DSC within the e-Navigation concept as currently envisaged. N/A | N/A |
|  |  | ITU-R M.541-9 | Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service |  |  |
|  |  | ITU-R M.1173 | Technical characteristics of single-sideband transmitters used in the maritime mobile service for radiotelephony in the bands between 1 606.5 kHz (1 605 kHz Region 2) and 4 000 kHz and between 4 000 kHz and 27 500 kHz |  |  |
|  |  | ITU-R M.1082-1 | International maritime MF/HF radiotelephone system with automatic facilities based on DSC signalling format |  |  |
|  |  | ITU-R M.476-5 | Direct-Printing Telegraph Equipment in the Maritime Mobile Service |  |  |
|  | Satellite | ITU-R M.541-9 | Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service | DSC over satellite is not currently defined explicitly through a technical characteristic. However, it could be appropriate to develop a complementary characteristic to those available for VHF and HF. | This task should be coordinated through WP4C/SG4 under existing question 227/4. |
| LRIT | HF |  |  | The use of HF and Satellite data links to support the LRIT application should not need a new technical characteristic as they are simply applications supported by existing data links. However, there may be merit in amending the existing recommendations in relation to HF and satellite communications to explicitly recognise the use of LRIT. | The subject is on the WRC-11 agenda under Item 1.10. However, there is unlikely to be any directly relevant outcomes. The work would therefore have to be addressed through WP4C/SG4 Questions 84-4/4 (Iridium), 87-4/4 (satellite) coordinating with WP5B/SG5 for HF. |
|  | Satellite |  |  |  |  |
| AIS |  | ITU-R M.1371-3 | Technical characteristics for an automatic identification system using time division multiple access in the VHF maritime mobile band | There are a number of potential changes to AIS under e-Navigation that would require revision to the existing technical characteristics. Firstly, the possible need for a dedicated channel to enable the detection of AIS transmissions from space. Secondly the allocation of additional channels to provide extra capacity for e.g. environmental or security related messaging. Thirdly the possible need to define new AIS messages such as those relating to channel management. Fourthly, the possible use of the AIS data link as a candidate technology for future maritime VHF data communications could also be a possibility. | WRC-11 agenda item 1.10 is considering the expanded use of VHF for port safety and security. Under this item the allocation of a 3rd AIS channel for detection from space is being considered. As a result of this item there could be a need to revise technical characteristics. This task would likely fall to WP5B/SG5 under existing questions 232/5 and 96-2/5. |
| VHF Comms | Voice | ITU-R M.1084-4 | Interim solutions for improved efficiency in the use of the band 156-174 MHz by stations in the maritime mobile service | Under e-Navigation a general push to make more efficient use of the spectrum available for maritime voice communications may take place as existing channels are increasingly allocated to digital services. The existing characteristics (e.g. M.1084) propose possible interim solutions such as the use of 12.5kHz channel spacing. It may be desirable to develop a new technical characteristic to describe a permanent solutions for e.g. 12.5 kHz or 8.33 kHz voice channels in the future communication environment. | This task should be coordinated through WP5B/SG5 possibly under existing question 96-2/5. |
|  |  | ITU-R M.489-2 | Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz |  |  |
|  | Data | ITU-R M.1842 | Characteristics of VHF radio system and equipment for the exchange of data and electronic mail in the maritime mobile service RR Appendix 18 channels | A new technical characteristic will be required to support the use of VHF channels for digital data transmission – assuming that AIS is not selected as the maritime domain datalink of choice. The current characteristic for VHF data exchange refers to the specific solution implemented by Telenor in Norway. If this solution is accepted by the maritime community this characteristic could be amended. However, in the more likely scenario of a new datalink technology being proposed a new characteristic will be required. | This task should be coordinated through WP5B/SG5 possibly under existing question 96-2/5. |
|  | SAR | ITU-R M.489-2 | Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz | No e-Nav related comms change is anticipated for the use of 121.5 MHz for short range SAR purposes. However, were the VHF channels to be revised the SAR associated characteristic (M.489) would in any event be updated. | N/A |
| HF Comms | Voice | ITU-R M.1173 | Technical characteristics of single-sideband transmitters used in the maritime mobile service for radiotelephony in the bands between 1 606.5 kHz (1 605 kHz Region 2) and 4 000 kHz and between 4 000 kHz and 27 500 kHz | No e-Nav related comms change is envisaged for HF voice services. However, if there is a broad encroachment of digital techniques into the band there may be a need to revise existing voice characteristics to accommodate data. | N/A |
|  |  | ITU-R M.1082-1 | International maritime MF/HF radiotelephone system with automatic facilities based on DSC signalling format |  |  |
|  | Data | ITU-R M.1081 | Automatic HF facsimile and data system for maritime mobile users | There are a number of existing technical characteristics relating to data exchange in the HF band that include both outmoded legacy technologies as well as candidate future technologies. There needs to be either an update to M.1798, or else a new characteristic to describe the selected HF digital data scheme to be used under e-Navigation. Currently M.1798 and F.1821 describes multi-channel HF data links such as the system currently deployed by Globe Wireless for commercial services (email, etc.). | The HF channelling requirements are being addressed within WRC-11 agenda item 1.9. The outcome could lead to a need to develop new characteristics in relation to the e-Navigation preferred HF datalink. See WP5B/SG5 and questions 145-2/5 and 158-1/5. |
|  |  | ITU-R M.1798 | Characteristics of HF radio equipment for the exchange of digital data and electronic mail in the maritime mobile service |  |  |
|  |  | ITU-R M.627-1 | Technical characteristics for HF maritime radio equipment using narrow-band phase-shift keying (NBPSK) telegraphy |  |  |
|  |  | ITU-R F.1821 | Characteristics of advanced digital high frequency (HF) radiocommunication systems |  |  |
| NAVTEX / NBDP | MIO | ITU-R M.625-3 | Direct-Printing Telegraph Equipment Employing Automatic Identification in the Maritime Mobile Service | If under e-Navigation the NAVTEX data is carried across different communication links there may be a need to provide a new characteristic to accommodate it. However, if it is intended to include NAVTEX data along with other safety related information e.g. Maritime Information Objects, etc. then it may be more appropriate for a specific characteristic to encompass all maritime safety information. | This task should be coordinated through WP5B/SG5 possibly under existing question 98-5. |
|  |  | ITU-R M.540-2 | Operational and Technical Characteristics for an Automated Direct- Printing Telegraph System for Promulgation of Navigational and Meteorological Warnings and Urgent Information to Ships - Section 8B - Maritime Mobile Service; Telegraphy and Related Subjects. |  |  |
|  |  | ITU-R M.688 | Technical characteristics for a high frequency direct-printing telegraph system for promulgation of high seas and NAVTEX-type maritime safety information |  |  |
|  |  | ITU-R M.492-6 | Operational procedures for the use of direct-printing telegraph equipment in the maritime mobile service |  |  |
|  |  | ITU-R M.476-5 | Direct-Printing Telegraph Equipment in the Maritime Mobile Service |  |  |
| On board Comms | UHF | ITU-R M.1174-2 | Technical characteristics of equipment used for on-board vessel communications in the bands between 450 and 470 MHz | There is a need to update the existing characteristics to support the implementation of digital PMR. | This task should be coordinated through WP5B/SG5 most likely under a new question. |
|  | WiFi | ITU-R F.2086 | Technical and operational characteristics and applications of broadband wireless access in the fixed service | If WiFi stations are to be adopted for on-board data communications there may be a need for a new characteristic associated with is use in the maritime mobile domain. | This task should be coordinated through WP5B/SG5 most likely under a new question. |
| Satellite Comms | Broadband | ITU-R S.1709 | Technical characteristics of air interfaces for global broadband satellite systems | Broadband over satellite is an existing application of current satellite systems and does not warrant a specific characteristic for the maritime domain | N/A |
|  | MSI | ITU-R S.1709 | Technical characteristics of air interfaces for global broadband satellite systems | The transmission of MSI over broadband may warrant a characteristic to define the data layers. | This task should be coordinated through WP4C/SG4 under existing question 87-4/4. |
| DGNSS |  | ITU-R M.823-3 | Technical characteristics of differential transmissions for global navigation satellite systems from maritime radio beacons in the frequency band 283.5-315 kHz in Region 1 and 285-325 kHz in Regions 2 and 3 | The current characteristic M.823-3 will ultimately need to be updated to accommodate new developments in GNSS such as Galileo and COMPASS. | This task should be coordinated through WP5B/SG5. |
| Port Services | WiMax / 2G / 3G / 4G |  |  | The maritime community will make use of technologies developed and standardised for other applications in port. No maritime specific characteristics are required. | N/A |
| Inter-ship Broadband | WiMax | ITU-R M.1801 | Radio interface standards for broadband wireless access systems, including mobile and nomadic applications, in the mobile service operating below 6 GHz | Under e-Navigation ad-hoc mesh WiMax networks could be deployed to provide broadband coastal networks. This would require a specific form of implementation of WiMax that would necessitate a specific technical characteristic. | This task should be coordinated through WP5B/SG5 possibly under existing question 212-3/5. |
| Loran | eLoran data channel | ITU-R M.589-3 | Technical characteristics of methods of data transmission and interference protection for radionavigation services in the frequency bands between 70 and 130 kHz | The existing technical characteristic will need to be revised to accommodate the definitive 9th pulse, Eurofix and/or new modulation schemes selected for the eLoran data channel. | This task should be coordinated through WP5B/SG5 under a new question. |
| Radar | SART | ITU-R M.628-4 | Technical Characteristics for Search and Rescue Radar Transponders | Relates to 9 GHz SART, so no change required. | N/A |
|  | Racon | ITU-R M.824-3 | Technical parameters of radar beacons (Racons) | No e-Nav related comms change. | N/A |
|  | Radio determination | ITU-R M.1460 | Technical and operational characteristics and protection criteria of radiodetermination radars in the 2 900-3 100 MHz band | No e-Nav related comms change. | N/A |
| EPIRB | VHF | ITU-R M.690-1 | Technical characteristics of emergency position-indicating radio beacons (EPIRBs) operating on the carrier frequencies of 121.5 MHz and 243 MHz | No e-Nav related comms change. | N/A |
|  | Satellite (COSPAS / SARSAT) | ITU-R M.633-3 | Transmission characteristics of a satellite emergency position-indicating radio beacon (satellite EPIRB) system operating through a satellite system in the 406 MHz band | No e-Nav related comms change. | N/A |
|  | Satellite | ITU-R M.632-3 | Transmission Characteristics of a Satellite Emergency Position Indicating Radio Beacon (Satellite EPIRB) System Operating Through Geostationary Satellites in the 1.6 GHz Band | No e-Nav related comms change. | N/A |

# Conclusions

e-Navigation is the future, digital concept for the maritime sector that responds to structural changes and will have a profound and long-term impact on the way the maritime sector operates. It is foreseen to be supported by a host of current and new technologies and will be underpinned by a communications infrastructure designed to enable authorised seamless information transfer onboard ship, between ships, between ship and shore and between shore authorities and other parties. This infrastructure will have to be capable of not only supporting future e-Navigation applications, but will also have to support legacy applications – at least during the transition to e-Navigation, if not beyond. It will therefore be bandwidth intensive and possibly rely upon a range of technologies.

This study examined the range of possible applications that would need to be supported by the maritime communication infrastructure in a future e-Navigation environment (considering 2020+). These applications can be broken down into three areas: safety related, operationally related and commercial. The study then assessed the likely data levels that would be typical in two busy coastal areas around the UK, the Solent and the Dover Straits, resulting from future vessel and communication traffic.

Within the study scenario analysis, in both scenarios, the levels of safety related data that needed to be passed were negligible when compared to potential data link capacity. Operationally related data levels were more significant, but again well within the reach of current terrestrial and satellite based communications. However, the possible commercial applications of communications have the potential to provide large levels of data traffic. It is likely that current technology would act as a constraint on what can realistically be achieved in relation to commercial applications. However, there are some high bandwidth technologies being assessed for possible use under e-Navigation, albeit that they currently tend to be constrained to port and near-coastal areas.

Securing spectrum, or changing the use within existing allocations is important for the e-Navigation concept due to the planning timescales required to protect or extend the use of the radio spectrum through the ITU WRC. The study therefore considered the bands which the current and foreseen maritime communications systems will occupy to ensure both that existing communication applications are secure and also that the maritime community is free to utilise emerging multi-modal technologies without constraint.

In general it is concluded that there are few concerns relating to e-Navigation from a spectrum management perspective. There are four identified activities which need to take place in reference to existing spectrum allocations, namely:

- carry out a study of the options for improving efficiency of spectrum use in the VHF Maritime Mobile Band, including reduced channel spacing, modulation alternatives for digital transmissions and different schemes for broadband services;

- general continued protection of the status of the maritime mobile service in the HF spectrum;

- keeping a watching brief on the (ITU) development of frequency plans for the 450 to 470 MHz band so as to protect on-board use and

- engaging in discussions concerning the use of PLT/BPL at a national level as and when the possibility or necessity arises.

In specific relation to the forthcoming WRC meeting there are several activities which need to take place in order to support appropriate maritime input.  In particular:

- Consider submission of results of study on improving spectrum efficiency of the VHF Maritime Mobile Band;

- Support for modifications to Appendix 17 HF maritime channelisation to permit new digital systems;

- Support for a maritime mobile-satellite allocation within the VHF band to enable satellite detection of AIS;

- Careful consideration of the need for spectrum around 500 kHz given the desire to use this for the broadcasting of security levels given pressure for some of this spectrum from the amateur radio community;

- Keeping a watching brief on developments with respect to VHF radiolocation allocations so as to protect maritime VHF use;

- Engagement in the groups considering an HF allocation for oceanographic radar in order to ensure that sufficient protection is afforded to maritime services (especially in light of the likely increasing use of HF for digital technologies).

Finally, the study considered the need for new recommendations to the ITU for the development or amendment of technical characteristics to enable e-Navigation. The envisaged communication technology and applications have been assessed in light of the existing technical characteristics and on-going ITU activities and where necessary recommendations have been made to provide input to either Study Group 5 Work Party 5B for terrestrial services or else to Study Group 4 Work Party 4C for satellite based services.

# Recommendations

In undertaking this study a number of recommendations have emerged to allow for the finalisation of the e-Navigation communications architecture:

- It is recommended that IALA seek to develop a Concept of Operations (CONOPS) for e-Navigation. This should outline the activities that will take place within e-Navigation, describe the systems used and stakeholders involved. This will allow a much better understanding of the communication requirements within the concept. The CONOPS should be sufficiently detailed that it will allow interface requirements between the elements to be described as well as the necessary flows of information. It could potentially allow for the development of use cases and communication system requirement documents. Such a CONOPS should be developed with the input of operational staff so that assumptions made about e.g. reporting rates can be fully validated. It is possible that live bridge simulations could be used to help elaborate this concept.

- The scenarios developed in this study have necessarily taken a high level somewhat averaged view of the likely data transmission requirements in relation to each application within the e-Navigation concept. In order to be able to undertake a thorough assessment of the detailed requirements a software based simulation of specific scenarios could yield much more information including e.g. peak data levels, coverage of terrestrial systems in the coastal environment, technology specific data link utilisation. In addition software based scenario analysis would allow for much more in depth investigations of candidate datalink technology options and could account for data link overheads, bandwidths, data rates, message scheduling and so on.

- In the future architecture, consideration will be needed as to how best to support communications within the GMDSS in remote areas outside the coverage of geostationary satellites. There could be merit in considering the options through a high-level business case that would need to consider e.g. the balance between utilising a global satellite based solution such as that provided by Iridium versus the maintenance of a wide area terrestrial HF network. This could help inform the development of the comms architecture by indicating where technology development effort needs to be focussed.

- A study should be carried out of the options for improving efficiency of spectrum use in the VHF Maritime Mobile Band, including reduced channel spacing, modulation alternatives for digital transmissions and different schemes for broadband services. The study should consider the advantages and disadvantages of each option. The results should be submitted to ITU WP5B.

- 9	Delivering the Plan

http://www.globalwireless.com/network_digitalhf.php

See, for example: http://www.cdg.org/technology/3g/cdma450/band_allocation.asp

Footnote 5.286AA refers

The variation is associated with different levels of ‘robustness’, i.e. the extent to which propagation and interference can be tolerated.

See section 5.2.5 for other activity around this frequency.

Industrial, Scientific and Medical.  These bands are used by devices such as industrial heaters and driers and domestic equipments such as microwave ovens.

According to 3GPP TS 45.005