Radio Technical Commission for Maritime Services

1800 N. Kent St., Suite 1060

Arlington, Virginia 22209-2109

www.rtcm.org           hq@rtcm.org

Telephone: +1-703-527-2000                                                                 Telefax: +1-703-351-9932

March 16, 2009

RTCM Special Committee 110 on Emergency Beacons

Liaison Note to IEC, IALA and CIRM – AIS Locating Device in an EPIRB

1.	RTCM would like to thank the IEC TC 80 AIS WG for its Information paper on tx power level of AIS SART (IEC-AIS-WG-06-out-06 dated 24-10-2008).

2.	RTCM SC110 considered this information paper at its December 2008 meeting in Washington DC and approved this response at its February 2009 meeting.   It was noted that the AIS WG’s mandate is to create a standard that fulfils the performance requirement from IMO, being that the AIS SART should be detected at a range of 5NM from a ship when mounted 1m above sea level.   It was also noted that the AIS WG had determined that this requirement could be met at 0.5W EIRP.   In addition it was noted that the AIS WG had not considered the case of an EPIRB with an AIS locating device and that the Tx power of an EPIRB with an AIS locating device requires both theoretical design and further sea trials.   The AIS WG also noted that there may be reasons to increase the output power on an EPIRB with an AIS locating device as an EPIRB is (normally) operated floating in the sea (rather than being 1m above sea level).

3.	RTCM wishes to inform the AIS WG that it concurs with the observations noted by the AIS WG in its Information paper related to the inclusion of an AIS locating device in an EPIRB.   Specifically RTCM considers that:

- The location of the base of the antenna in an EPIRB, being only 4cm above sea level, is a different scenario to that of the AIS SART and as such it requires further study.

- If an EPIRB with AIS locating device becomes a work item, then the EIRP requirements of the AIS locating device in the EPIRB should be based upon the IMO performance requirement and ITU technical recommendations.

- It is possible that the IMO performance requirement for the AIS SART, of being detected at a range of 5NM from a ship, may be different for an AIS device in an EPIRB, given that the 406 MHz signal in the EPIRB already provides an initial location to SAR forces.

- There is no technical reason why the EIRP of an AIS SART has to be the same as the EIRP of an AIS locating device in an EPIRB, especially as the Battery and Physical Size constraints in an EPIRB are different to those in an AIS SART.

- Based upon the AIS-SART trials results, the range of an AIS locating device in an EPIRB to aircraft is not considered to be an issue and RTCM is of the opinion that no further work is needed in this area.

4.	RTCM wishes to inform the AIS WG that the subject of including an AIS locating device in an EPIRB was raised at the recent IMO COMSAR 13 meeting in London and that the Technical Working Group concluded that, the option of adding an AIS processor to the standard EPIRB should be permitted.

5.	RTCM also wishes to inform the AIS WG that a New Work Item proposal has been submitted to IMO MSC 86 recommending that MSC instruct COMSAR to add this subject to its future work programme.

6.	Based upon the above information RTCM requests that the IEC TC80 AIS WG undertakes both the theoretical design and necessary sea trials to establish the required minimum EIRP of an AIS locating device mounted in an EPIRB, with the base of its antenna 4cm above sea level, necessary to achieve a detection range from ships of 3NM and separately 5NM at a 50% or better probability in Sea State 4.