Description of a Limited Base Station

# Introduction

## Rationale for the definition of the Limited AIS Base Station

IMO states the purpose of the AIS as follows:

“The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

- in a ship-to-ship mode for collision avoidance;

- as a means for littoral States to obtain information about the ship and its cargo; and

- as a VTS tool, i. e. ship-to-shore (traffic management).”

This ruling applies to the member states of the IMO, i. e. the contracting governments. These states can be both port states and littoral states. By the introduction of the AIS the IMO requests shipping industry to disclose and broadcast potentially sensitive information about ship movements, ship static and voyage related data. Thus IMO stipulates that only other participating ships and competent authorities, i. e. the national competent authority of the member state, may receive and use that information.

In addition, this information may not be used for other purposes than stated above. The information on the AIS VHF data link is protected by the appropriate stipulations of the Radio Regulations, IMO Resolution MSC.140(76), and additionally by the data protection legislation of the littoral or port states.

In order to use the AIS in its area of competence a littoral or a port state needs to set up a national AIS Service in accordance with the appropriate technical standards created by ITU (Recommendation ITU-R M.1371-3) together with derived technical Recommendations issued by IALA. The most fundamental means to access the AIS VHF data link for a competent shore authority and thus provide the AIS Service is to deploy and operate AIS Base Station(s).

The AIS Service in a littoral or port state is operated by the national competent authority for the whole of the national coverage area. This national competent authority has the need to control the AIS Service and the AIS VHF data link, by employing means like channel management, protected slot allocations, and assigned mode. The AIS Base Station allows for the full control of the AIS VHF data link.

However, there are other competent authorities responsible for local operations of e. g. individual harbours, locks, marinas, off-shore structures. These regularly have an interest in monitoring the vessel traffic in their locally confined area of competence and exchange limited operational information with ships in regard to their area of competence, e. g. docking scheduling, tugs operation.

The national competent authority is in charge of the full AIS Service functionality, including the control mechanisms. However, to allow local competent authorities to participate in and benefit from AIS , there are two possible options:

- connect to the national AIS Service via a shore-based communication link,

- establish an own locally confined AIS infrastructure with limited capacities.

## General overview on the capabilities and limitations of the LBS

The LBS should be capable of receiving all AIS messages within its radio range. In the case of addressed AIS messages the LBS only receives messages addressed to itself, however.

The LBS may transmit interrogation messages, binary and safety related messages.

The LBS may transmit Base Station static report but should not provide secondary synchronization.

The LBS shall also transmit acknowledgements to addressed messages.

Should the optional Aids-to-Navigation functionality be added then the LBS shall also transmit AtoN-reports.

The maximum permissible link utilization of an LBS should be 20 slots per frame for all above messages together except acknowledgements. Interrogation messages should limited to 5 slots in a frame in total.

The LBS should be prohibited to transmit any controlling messages such as assigned mode commands, DGNSS broadcasts and to perform data link management (FATDMA reservations) and channel management. The LBS should also not provide secondary synchronisation support, i. e. it should not act as a semaphore.

Optionally LBS are subject to the channel management messages (TDMA and DSC Channel 70) and assignment messages. Therefore, the LBS reacts like a mobile station in that it reacts to received Message 20 and 22 and/or DSC-based AIS Channel Management command.

# Comparison of Limited Base Station to Class A Mobile Station and Base Station

Table 1 presents the maximum capabilities of a Limited Base Station in comparison to the Class A Mobile Station and the AIS Base Station.  Limited Base Stations that are less capable than that shown in Table 1 can be achieved by hardware or software configuration means.

Table 1 –Comparison of transmit capabilities between Limited Base Station Mobile Class A Station and Base Station

| Msg ID | Description | Class A | Limited Base Station1 | Base Station |
| --- | --- | --- | --- | --- |
| 1 | Position Report (SOTDMA) | Yes | nil | nil |
| 2 | Position Report (assigned) | Yes | nil | nil |
| 3 | Position Report (ITDMA) | Yes | nil | nil |
| 4 | Base Station report | nil | Yes2 | Yes |
| 5 | Ship static and voyage related Data | Yes | nil | nil |
| 6 | Binary Addressed Message | Yes | Yes | Yes |
| 7 | Binary Addressed Ack | Yes | Yes | Yes |
| 8 | Binary Broadcast Message | Yes | Yes | Yes |
| 9 | SAR Position Report | nil | Nil | nil |
| 10 | UTC/Date request | Yes | Nil | Yes |
| 11 | UTC/Date | Yes | Nil | nil |
| 12 | Safety Addressed Message | Yes | Yes | Yes |
| 13 | Safety Addressed Ack | Yes | Yes | Yes |
| 14 | Safety Broadcast Message | Yes | Yes | Yes |
| 15 | Interrogation | Yes | Yes | Yes |
| 16 | Assigned mode command | nil | Nil | Yes |
| 17 | DGNSS broadcast | nil | Nil | Yes |
| 18 | Class B position report | nil | Nil | nil |
| 19 | Extended Class B position report | nil | Nil | nil |
| 20 | Data Link Management | nil | Nil | Yes |
| 21 | Aids-to-navigation report | nil | Yes3 | Yes |
| 22 | Channel Management | nil | Nil | Yes |
| 23 | Group assignment command | Nil | Nil | Yes |
| 24a | Static data report | nil | Yes | Yes |
| 25 | Single slot binary message | Yes | Yes | Yes |
| 26 | Multiple slot binary message with ComStat | Yes | Yes | Yes |

Footnote 1: Total Link Load should be less than 20 slots per frame due to RATDMA access scheme.  This applies to Binary Msgs (6, 8), Safety Messages (12, 14), Interrogation (15) and ATON Report (21). Interrogation messages are limited to 5 slots in a frame.

Footnote 2: No semaphore functionality

Footnote 3: If implemented

All messages will be received and processed accordingly

Other Rules and Limitations of Limited Base Station:

- No retransmission.

- AIS Message 21: There may be a liability issue for the LBS operating authority that applies only if optional AtoN functionality is added to the Limited Base Station.

- The LBS should discard all addressed messages received that are not addressed to the LBS MMSI.  The LBS may have more than one MMSI representing different services connected to the LBS.