Background Paper on AtoN Information Exchange and Presentation

Introduction

At the XVIth IALA Conference several papers were presented that discussed the application of Geographic Information Systems (GIS) for AtoN planning and information management.  In response to these papers the Conference recommended that IALA should investigate the use of simulation and GIS in relation to risk assessment planning and evaluation of AtoN.  In late 2007 IALA held a GIS and Simulation Workshop, and one of the recommendations from this Workshop was that IALA should develop an internationally agreed AtoN Attribute and Metadata Information Standard.

Relevance to e-Navigation

IALA’s work to date on simulation and GIS aligns well with several of the identified future directions of e-Navigation, and will form a good foundation for the continuation of IALA’s work in support of e-Navigation.  It was agreed at the 4th session of the e-NAV Committee that in future there should be an ad hoc Task Group formed within WG1 to explore the use of Marine Information Systems within e-Navigation. Work is also underway in the Architecture Working Group (WG5) on a Universal Maritime Data Model, of which AtoN Information would form a part.

The first of the high-level generic user needs for e-Navigation identified at IMO NAV 54 was a Common Maritime Information/Data Structure: “Mariners require information pertaining to the planning and execution of voyages, the assessment of navigation risk and compliance with regulation.  This information should be accessible from a single integrated system.  Shore users require information pertaining to their maritime domain, including static and dynamic information on vessels and their voyages.  This information should be provided in an internationally agreed common data structure.  Such a data structure is essential for the sharing of information amongst shore authorities on a regional and international basis.”

Information about AtoN will be required as an integral component of the “information” being referred to above.

# AtoN Attribute Information

The exchange of AtoN information between any parties in a digital environment will require internationally agreed standards so that information can be automatically compiled for sending and automatically understood by systems that receive it.

Such a standard will enable harmonisation of the management of information about AtoN, and in particular information that is relevant to mariners: “situation normal” data (position, colour, shapes, light etc) and also situation abnormal (lost top mark, light on reduced range, unlit etc).  In GIS terms this sort of information can be described as attribute data (information particular to a GIS object such as an AtoN) and metadata (data about the attribute data).

Digital representations of Visual AtoNs are a sub-category of Traffic Objects in the present draft of the IALA Universal Maritime Data Model. Radio-navigations AtoNs, such as GNSS are considered as a sub-category of Physical External Entities other than traffic objects.

Relevant Standards

ISO 8879:1986, Standard Generalized Markup Language (SGML) is an ISO defined metalanguage used to define markup languages.

ISO 19115:2003, Geographic information - Metadata, defines the schema for the identification, extent, quality, spatial and temporal schema, spatial reference, and distribution of digital geographic data. These schemas are useful for the cataloguing of datasets, clearinghouse activities, and the full description of datasets; geographic datasets, dataset series, and individual geographic features and feature properties.

ISO 19119:2005, OpenGIS Service Architecture, defines a service metadata schema for use in a catalogue service as is done for dataset metadata.

ISO 11179-1 to 7:2004/5, Information Technology – Metadata Registries (MDR).

IMO Resolution MSC. 191(79), Performance Standards for the presentation of navigation-related information on shipborne navigational displays.

IMO SN/Circ. 243 Guidelines for the presentation of navigation-related symbols, terms and abbreviations.

IHO Special publication No.57 (S57).

IHO Special publication No.100 (S100).

IALA Draft Future Recommendation e-Nav 0220, Universal Maritime Data Model.

# Examples of Existing Information Management Schemes

The Australian Hydrographic Office (AHO) in cooperation with AMSA has produced an Excel application for compiling AtoN information in a format suitable for input to a GIS and also the AHO’s AtoN database - Annex 1.

The UK Hydrographic Office has customised software (POLAR) for the same purpose.  For the exchange of such data a likely message format is XML (Extensible Markup Language) – Annex 2.

The General Lighthouse Authorities have demonstrated XML schema for AtoNs (lighthouses, buoys and DGPS stations) and IALA is in the process of implementing the DGNSS application on its website – Annex 3.

The Portuguese Instituto Hidrografico has in place a web-based system supported by robust navigational databases, providing mariners with Maritime Safety Information, including tidal and wave data Notices to Mariners, NAVTEX and Chart Updates – Annex 4.

Finally a researcher at the University of Arizona, with support from the USCG R & D Center, has developed a Maritime Information Management Language (MIML) based on XML and relating to S 57.  Subject to general acceptability and copyright considerations, this could be a candidate for an international standard markup language for this application – Annex 5.

Action Proposed

Accepting the need for a harmonised approach, the first step is to investigate relevant, existing systems and standards.

It is apparent that several relevant standards already exist, therefore it may be appropriate to draft an IALA Guideline bringing these to the attention of IALA Members and advising how they can be implemented.

If the existing standards are not considered to cover the requirement adequately then an IALA Recommendation may need to be drafted, taking into account the examples above and concurrent work on the Universal Maritime Data Model and Maritime Information Systems within Working Groups 5 and 1 of the IALA e-Navigation Committee.

Noting the need for AtoN authorities to communicate information about AtoN to Hydrographic authorities, it will be paramount that any agreed IALA AtoN Attribute and Metadata Information Recommendation is compatible with the IHO’s S-57 and S-100 Standards.

Benefits

Once such standards are in use, with suitable security measures, administrations will be able to coordinate management of AtoN information in a digital environment and this will in turn facilitate sharing of information with:

- mariners in real-time or near real-time,

- shore-based operators,

- hydrographic authorities (for inclusion in official navigational products),

- Original Equipment Manufacturers (OEMs) for inclusion in non-official chart systems such as Electronic Chart Systems (ECS), chart radars and “AIS radars” and other equipment databases

ANNEX 1 - AID TO NAVIGATION INPUT TEMPLATE (AHO/AMSA)

| AID TO NAVIGATION FORM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | AtoN Owner |  |  |  |  |
|  | Date established |  | <Input Date (dd/mm/yy)> |  |  |
|  | Country |  | <Input Country> |  |  |
|  | State |  | <Input State> |  |  |
|  | AtoN owner/authority |  | <Input Aton Owner> |  |  |
|  | Aton ID |  | <Input Aton ID(State/Port Authority Asset No)> |  |  |
|  | Location ID |  | <Input Location ID (eg No 1, No 2, etc)> |  |  |
|  | AHS Ausnum |  | <Input AHS Ausnum by AHO Office Only> |  |  |
|  | ALL Vol K Number |  | <Input VolK number> |  |  |
|  | Location name |  | <Input Location Name> |  |  |
|  | Common name |  | <Input Common Name> |  |  |
|  | AtoN type |  | <Input AtoN Type (eg Racon, Light, Ramark etc)> |  |  |
|  | Mark structure |  | <Input Mark Structure (eg Tower - Lattice, Tower - Masonry etc)> |  |  |
|  | Mark type |  | <Input IALA Description (eg North Cardinal etc)> |  |  |
|  | Geographic or Grid Co-ordinates |  |  |  |  |
|  | Datum |  | <Input Datum> |  |  |
|  | Latitude degrees |  | <Input Latitude Degrees (eg 24) |  |  |
|  | Latitude minutes |  | <Input Latitude Minutes (eg mm.mmmm) |  |  |
|  | North/South |  | <Input North/South |  |  |
|  | Longitude degrees |  | <Input Longitude Degrees (eg 24)> |  |  |
|  | Longitude minutes |  | <Input Longitude Minutes (eg mm.mmmm)> |  |  |
|  | East/West |  | <Input East/West |  |  |
|  | Horizontal accuracy |  | <Input Suitable Estimated Accuracy (minutes) |  |  |
|  | Projection |  |  | <Input Name of Projection> |  |
|  | Zone |  |  | <Input Zone> |  |
|  | Easting |  |  | <Input Easting> |  |
|  | Northing |  |  | <Input Northing> |  |
|  | Positional Survey Information |  |  |  |  |
|  | Method of survey |  | <Input Method of Survey (or position fixing)> |  |  |
|  | Surveyed by |  | <Input Surveyed by> |  |  |
|  | Survey date |  | <Input Survey Date (eg dd/mm/yy)> |  |  |
|  | Light Details |  |  |  |  |
|  | Light character |  | <Input Flash Character-Light(Dark) (eg 00.50+(01.00)+00.50+(03.00)> |  |  |
|  | Light colour |  | <Input Light Colour> |  |  |
|  | Period of operation |  | <Input EXCLIT (Night, Day, 24 hour, occasional)> |  |  |
|  | Nominal light range |  | <Input Nominal Light Range (in Nautical Miles)> |  |  |
|  | Elevation |  | <Input Height of Light Focal plane> |  |  |
|  | Elevation datum |  | <Input Height Datum of Light Focal plane> |  |  |
|  | Vertical accuracy |  | <Input Suitable Estimated Accuracy (metres)> |  |  |
|  | Visible sectors |  | <Input Arc of Visibility/Sectors (From:)> |  |  |
|  | Visible sectors |  | <Input Arc of Visibility/Sectors (To:)> |  |  |
|  | Obscure sectors |  | <Input Arc of Obscurity/Sectors (From:)> |  |  |
|  | Obscure sectors |  | <Input Arc of Obscurity/Sectors (To:)> |  |  |
|  | Charted light characteristics |  | <Input Chart Light Characteristics (eg FI(2) G 5s 7m 10M)> |  |  |
|  | Beacon/Buoy/Structure Details |  |  |  |  |
|  | Situation |  | <Input Situation> |  |  |
|  | Attached to fixed structure |  | <Input Yes/No> |  |  |
|  | Shape |  | <Input shape> |  |  |
|  | Colour(s) |  | <Input Colour(s) (Top to Bottom)> |  |  |
|  | Special mark |  | <Input Special Mark (Yes/No)> |  |  |
|  | Purpose of special mark |  | <Input Purpose of Special Mark> |  |  |
|  | Daymark/topmark |  | <Input Daymark/Topmark (Yes/No)> |  |  |
|  | Daymark/topmark colour |  | <Input Daymark/Topmark Colour> |  |  |
|  | Signage |  | <Input Wording on Sign> |  |  |
|  | Photograph |  | <Input Photo Supplied (Yes/No)> |  |  |
|  | Leading Lights/Beacon Details |  |  |  |  |
|  | Orientation of leading marks/transits |  | <Input Orientation (eg 035)> |  |  |
|  | Type of navline |  | <Input Type of Navline: Recommended Track/Clearing Line/Transit Line> |  |  |
|  | Direction of traffic |  | <Input Direction of Traffic: One Way/Two way/Inbound/Outbound> |  |  |
|  | Racon/AIS Details |  |  |  |  |
|  | Racon character |  | <Input Racon Character (Morse Code)> |  |  |
|  | Racon type |  | <Input Racon Type (X/S/Frequency Agile)> |  |  |
|  | Racon range |  | <Input Racon Range> |  |  |
|  | AIS station type |  | <Input AIS Station Type> |  |  |
|  | AIS - Tx/Rx |  | <Input AIS (Transmit/Receive)> |  |  |
|  | MMSI Number |  | <Input MMSI Number> |  |  |
|  | Elevation |  | <Input height of antenna (AIS or racon, metres to 2 decimal places)> |  |  |
|  | Range (calculated) |  | <Input Calculated Range> |  |  |
|  | General Comments |  |  |  |  |
|  | Comments |  | <Input General Comments> |  |  |
|  | Record completed by |  | <Input Your Name> |  |  |
|  | Title |  | <Input Your Title> |  |  |
|  | Date completed |  | <Input Date Completed (eg dd/mm/yy)> |  |  |
|  | Email |  | <Input Your Email Address> |  |  |

ANNEX 2 – POLAR (UKHO)

View of the Admiralty Digital List of Lights display for light A0008.

Production of Lights and Radio Signals

Lights data is held as structured xml data in an Oracle database, with an editing facility. The lights object is broken down into a number of fields and sub-fields which can be edited as required. The data can be displayed against images of raster charts, ENCs and a World Vector Shoreline. The output for the CD base data and updates for Admiralty Digital List of Lights is xml. Output for the paper List of Lights and the paper updates is PDF. The application has a full audit and version trail with rule based fields / warnings.

Option Filer View and the World Vector Shoreline and Raster Chart of the Scillies with the light A0008 position displayed

Panels:- Option Filer View and the World Vector Shoreline and ENC of the Scillies with the light A0008 position displayed

Panels:- Field Option and graphic display of A0008 and book entry display and sector editing field and Raster Chart of the Scillies

ANNEX 3 – Example of XML Schema for Buoys (GLA)

<?xml version="1.0" encoding="UTF-8" standalone="yes" ?>

- <!--

The internal DTD follows

-->

<!DOCTYPE BUOYLIST (View Source for full doctype...)>

- <!--

Start of DGPS station list

-->

- <BUOYLIST>

- <STATION>

<STATION_NAME>AFAN OUTFALL INNER</STATION_NAME>

<TYPE_CODE>2S4SMSP</TYPE_CODE>

<LIT>LIGHTED BUOY</LIT>

<ACTIVE>FALSE</ACTIVE>

<AREA>SWANSEA</AREA>

<LT_CHARACTER>FL.Y.10S</LT_CHARACTER>

<RACON />

<RACON_CODE />

<LAT_DEG>51</LAT_DEG>

<LAT_MIN>33.11</LAT_MIN>

<LONG_DEG>3</LONG_DEG>

<LONG_MIN>48.72</LONG_MIN>

<LONG_EW>W</LONG_EW>

<DEPTH>8.1</DEPTH>

<NORTHING>185269</NORTHING>

<EASTING>274384</EASTING>

<TRINITY_EQUIPMENT>TRUE</TRINITY_EQUIPMENT>

<EXTRANEOUS_AUTHORITY>WELSH WATER AUTHORITY</EXTRANEOUS_AUTHORITY>

<OWNERSHIP_DETAILS>WELSH WATER AUTHORITY</OWNERSHIP_DETAILS>

</STATION>

- <STATION>

<STATION_NAME>AFAN OUTFALL OUTER</STATION_NAME>

<TYPE_CODE>2S4SMSP</TYPE_CODE>

<LIT>LIGHTED BUOY</LIT>

<ACTIVE>FALSE</ACTIVE>

<AREA>SWANSEA</AREA>

<LT_CHARACTER>FL(4)Y.15S</LT_CHARACTER>

<RACON />

<RACON_CODE />

<LAT_DEG>51</LAT_DEG>

<LAT_MIN>32.5</LAT_MIN>

<LONG_DEG>3</LONG_DEG>

<LONG_MIN>49.52</LONG_MIN>

<LONG_EW>W</LONG_EW>

<DEPTH>9.7</DEPTH>

<NORTHING>184162</NORTHING>

<EASTING>273431</EASTING>

<TRINITY_EQUIPMENT>TRUE</TRINITY_EQUIPMENT>

<EXTRANEOUS_AUTHORITY>WELSH WATER AUTHORITY</EXTRANEOUS_AUTHORITY>

<OWNERSHIP_DETAILS>WELSH WATER AUTHORITY</OWNERSHIP_DETAILS>

# Annex 4 – ANAVNET, Instituto Hidrografico, Portugal

ANAVnet was meant to be a friendly on-line application, supported by robust and secure databases, capable of providing either entire Notice to Mariners (NtM) publications, or single NtM affecting individual documents; allowing in any case consultation and printing, including entire correction pages of nautical publications and graphical annexes to glue on charts.

In order to achieve the proposed goals it was necessary to correlate NtM, coastal and local NavWarnings with the existing chart folio and nautical publications; furthermore there was a need to associate geographical areas with designated local authorities.

Concept modeling was done based on interviews with end users, navy navigators and system administrators from IHPT Navigation Division.

Entity relationship models were used in order to depict relationships between different authorities and their unique description attributes.

These models were then mapped to a set of tables in a database management system - Oracle spatial.

A relational-object logical logging was used, where the graphic components (affecting for instance charts or publications) were stored as polygons or points in SDO format (Spatial Data Option) in a table’s column which describe attributes correlated with other tables through identification keys, or primary keys.

It was necessary to develop a few applications using Oracle forms in order to log the required data, both textual and graphical (polygons and points). This option allowed specific setup requirements in terms of man-machine interface.

All graphical data was kept accessible for other applications based on database search. Furthermore, once they are given a geo-reference system they can be used in Geographic Information Systems.

Looking forward to providing new added value service to mariners, IHPT has probably found its way into e-Navigation.

A List of Lights database is currently being developed and it will only have geo-referenced objects. Once completed, this database will be linked with ANAVnet in order to be automatically updated. Furthermore, it is intended to give remote access to Direcção de Faróis (Portuguese Lighthouse Authority), surveying teams and nautical charts compilers, providing an in-office/field application that enables real

time updates.

There is still one more on-line application being developed, named “IH Skipper”. This application is meant to aid the planning stage of a short voyage along the coast (covering 150 nautical miles each time at the most); it will retrieve information from ANAVnet and List of Lights databases; it will add the real time information available from IHPT field sensors such as ODAS buoys, weather forecasts and possibly Sailing Directions information.

# Annex 5 - Vocabulary Development for Markup Languages --- A Case Study with Maritime Information, Raphael Malyankar
Arizona State University (Author holds copyright)

The long-term aim of this effort is to allow automated systems to use geospatial information better, so that it is possible for a navigational program to `understand', for example, that shorelines can be crossed by aircraft but not by surface vessels, that tidal tables are important for near shore navigation, that routes may need to stay away from restricted waters, that cutting across shipping lanes should be minimized, etc. The near-term aims are to enhance the exchange of maritime information between disparate databases, applications, and user communities, to simplify the management of the voluminous amount of data related to this domain, to lay a foundation for the next generation of data and information distribution, and to enable better location and retrieval of information from existing formats and distribution modes.

Maritime Information Markup Language (MIML) is still under development. What is described in this case study must be regarded as `first steps' in development. The most important pending issue is standardization. Imminent efforts to standardize the ontology (or ontologies) and markup language - that is, remove the straw-man status of MIML - are anticipated in conjunction with the US Coast Guard, NOAA, and other interested parties.

Work is in progress on creating markup tools for semi-automated markup of source texts with MIML. The core idea here is to exploit the inherent structure in source texts of the kind we are dealing with in the maritime information domain to recognize word patterns and other lexical and structural clues to suggest markup. Work on the other applications mentioned in the previous section will also be taken up, and efforts on enhancing the capabilities of the demonstration prototype continue. Another direction of research is concerned with combining MIML documents in accordance with semantic criteria, for example, to place weather-related information from different source MIML documents together in an output MIML document. Such combination techniques will be enhancements to current XML merging tools in that they will rely on semantic clues (for example, concept relatedness in a base ontology) for document merging, instead of lexical comparison of tags or structural comparison of elements.

Part of the ontology derived from the IHO S-57 standard object catalog. Shown is the "Wreck" class in that catalog.

Sample elements extracted from the Coast Pilot