One basic tool aimed to SAR and traffic monitoring purposes is the Blue Box System, provided by GEM Elettronica _ Italy, in the frame of the European Community project already in action from the Nineties.

More than 3,000 fishing vessels included in the Blue Box System are continuously monitored and assisted by the Main Control Center located at the Italian Coast Guard Headquarter – Rome – Italy.

The structure of the system is quite simple:

- Each participant vessel hosts a Blue Box, consisting of a processor and a GPS receiver with its antenna, connected to a Inmarsat C (Mini C) terminal with its antenna;

- The control center can communicate with each participant vessel through the Inmarsat C segment. It also represents their last messages on a large, ECDIS-like screen. All received position enter into the local Data Base for due processing and management.

- The Inmarsat segment, which consists of the gateway station at Fucino – Italy, and the couple of geostationary Inmarsat East Atlantic and Indian Ocean satellites, covering the area all the fishing vessel is spread out.

Briefly, all blue boxes send Inmarsat C Data Reports and GEM Message Reports to the Control Center, to inform about their position, time stamp, health status of the vessel.

In case of emergency, the crew can send a coded alarm message through as local user interface, that is sent through the Inmarsat segment to the control center for due assistance.

The control center can download to one or more vessels (EGC function) software upgrades and polygonals of areas restricted to navigation. Therefore, such areas shall be stored by the receiving Bluer Box(es), making the real-time, automatic comparison of the current GPS position against collision into all those areas. Should the Blue Box detect automatically the collision into one restricted area, a dedicated message shall be sent through the Inmarsat segment to the control center, without chances to the crew to intervene.

The system engagement is to work at maximum load borne by the Inmarsat C communications system. On-field tests and prolonged experience state that the optimum usage of such system is to limit the transmission rate of each vessel at 4 hours minimum a day to reduce the load at the gateway and relevant communications costs.

The Blue Box System matches as much as possible to Inmarsat C features, one of them is the Store-and-Forward management of all messages travelling bidirectionally.

Also, it is worth to mention that the Blue Box system doesn’t exploit the DSC capability which is the unique real-time communications capability available from Inmarsat C, to avoid false alarms and consequent interventions for assistance.

Experience has also said that Inmarsat C can be a useful communications means whereas VHF standard communications cannot cover the area of operation of the vessel, and now GEM Elettronica is currently producing an improved Blue Box System emboding a standard AIS and an Inmarsat C terminals, switching automatically between them in accordance with good coverage criterion. Should both of them available, AIS shall be preferred for reducing communications costs.

Another consideration is that using the Inmarsat C against such additional workload given by the Blue Box System, especially when the Blue Box targets are spread on one satellite-only covered area, could take the Inmarsat C system into saturation, that means to get sometimes unacceptable delays to the message transfers, especially in case of emergency.

Hence the current Inmarsat C technology should be referred as a “coverage filler” backing-up AIS or other dedicated data communications systems in the global or regional E-nav scenarios.