Development of the Korean land-based e-Navigation

1   Background

.1   The International Maritime Organization implied the necessity of developing the e-navigation as a long-term vision for innovation strategy in May 2006.

.1   Demand for diverse and accurate information from the field that is essential for the maritime sector in securing marine safety and raising logistics efficiency has been increasing.

.2   In July 2009, the survey result on the demand for e-navigation was released in the 55th NAV Committee. (NAV 55/11/3)

.1   Germany and Canada summarized the survey result based on demand (NAV 54/25) from high-level users in the 54th NAV.

.2    Maritime communications

.1   The dominant problem in maritime communications is the difference of language in vessel-to-vessel communication and noisy traffic communication between vessel and land.

.2   85% of the respondents preferred using satellite broadband both for vessel-to-vessel and vessel-to-land communications.

.3   Around 50% of the respondents had knowledge and experience on AIS binary message, and may asserted that AIS binary message should be used as an effective medium to share navigation information.

.3   Human Machine Interface (HMI)

.1   25% answered that user convenience should be improved and data should be simplified, followed by 20% who laid significance on standardizing equipment management and data format.

.2   More than 80% preferred the S-mode concept.

.4   Technical/Operational Enhancement

.1   83% of the respondents preferred additional GNSS, and 67% and 16% showed preference toward radar positioning and Loan-C, respectively.

.2   Most answerers wanted broader cooperation from land on traffic flow. The main elements they mentioned were relevant to the movement of large or special vessels and traffic density.

.3   71% of the participants preferred automatic checks on certain navigation routes, and 83% preferred the provision of electronic formats used to write paper documents.

.3   Korea could lead the replacement of marine safety information system with e-navigation by sharing our progress in establishing it.

.1   The introduction of the model used to build the land-based e-navigation could position Korea as a leader in the sector, and enhance the understanding of other countries.

.2   Korea could propose e-navigation centered on actual users instead of the IT-based system by reflecting the demand from users in the design.

2   Progress in establishing the e-navigation

.1   DGNSS message (No. 17) service of the AIS station

.1   Progress

.1   11 DGPS Reference Stations have been receiving DGPS modification signals since 1998.

.2   DGPS equipments were installed, and the DGPS service (every second, message No. 17) has been provided since August 2009.

.1   A total of 32 AIS stations will be built on a phased-out basis over several years.

< Current DGPS reference signal broadcasting stations>

| VTS Center | Station | DGPS Model | DGPS Station |
| --- | --- | --- | --- |
| Incheon(Pyeongtaek) | Salgoji | DusiTech DST-A01 | Palmido |
| Daesan | Mt. Humang | FURUNO GP-37 | Palmido |
| Gunsan | Osikdo | DusiTech DST-A01 | Eocheongdo |
|  | Eocheongdo | FURUNO GP-37 | Eocheongdo |
| Jeju | Seoreum | DusiTech DST-A01 | Marado |
| Yeosu | Wenareudo | FURUNO GP-37 | Geomundo |
| Masan | Mt. Yonghwa | FURUNO GP-37 | Yeongdo |
| Pohang | Mt. Seongdong | FURUNO GP-37 | Homigot |
| Donghae | Mt. Samun | FURUNO GP-37 | Jumunjin |

.2   Outline

.1   The DGPS receiver receives modification data RTCM SC-104, which is sent as Message 17 (DGNSS Broadcast binary message) defined in ITU-R M.1373-3 using the BSC and station transponder.

.2   Message 17 received by the vessel transponder is sent after it is modified with the vessel location.

< DGPS reference broadcasting service using AIS >

.3   RTCM SC-104 Data Format

.1   11 among 64 message types were designated for RTCM SC-104 Ver 2.2 received by DGPS, and broadcasting message types serviced within Korea are as follows.

| Message Type | Details |
| --- | --- |
| No. 3 | GPS Reference station parameter |
| No. 5 | GPS Constellation Health |
| No. 7 | DGPS Radiobeacon Almanac |
| No. 9 | GPS Partial Correction Set |
| No. 16 | GPS Special Message |

.2   	Message types 7 and 9 of RTCM SC-104 are utilized to service AIS Message 17 to vessels.

.4   Field study on DGNSS service

.1   Study outline

.1   Date / place: April 23 to 29, 2009 / Pyeongtaek, Gunsan, and Daesan

.2   	Method: Comparison between AIS device for vessels and DGPS detector

.3   Specific roles and researchers

| Roles | Researchers | Number of persons |
| --- | --- | --- |
| Overall management of the study and survey | Navigational Safety and Information Division | 2 |
| Monitoring the DGPS service status of the AIS station | GMT Cybernetics | 2 |
| Inspection of the modification accuracy of vessel AIS device | GMT Cybernetics | 1 |
| Inspection of accuracy of DGPS detector | DGNSS Central Office and Anse Technologies | 2 each |

.2   Subject

.1   Station: Mt. Humang of Daesan, Osikdo of Gunsan, and Salkoji of Pyeongtaek

.2   Vessel: Six vessels including JILIHU, a Chinese oil tanker

.3   Comparison: Vessel AIS device and vessel location value of the DGPS detector

.3   Result

.1   We measured the change of the mobile station through a simulation on before and after the broadcasting of AIS message 17 in Mt. Humang, Daesan. Around 85% of dynamic messages of the mobile station changed to DGNSS.

.2   The following table is the modification rate before and after the broadcasting. The number of DGPS vessels pre-broadcasting equals the number of vessels that installed the DGPS receiver.

< Modification rate pre- and post-DGPS broadcasting >

| Time | Number of received vessels | Number of DGPS vessels | DGPS change |
| --- | --- | --- | --- |
| Pre-broadcasting | 76 | 10 | Around 13% |
| Post-broadcasting | 77 | 66 | Around 86% |

.3   Vessels with DGPS modification before and after broadcasting AIS message 17

<Before>                        <During the broadcast>

.4   The DGPS service status of the AIS station turned out to be normal.

The vessel AIS device shows whether DGPS was broadcasted or not.

< ECDIS screen showing DGPS broadcasting >

| Without DGPS broadcasting | With DGPS broadcasting |
| --- | --- |

< Track with or without DGPS broadcasting >

| Without DGPS broadcasting | With DGPS broadcasting |
| --- | --- |

.5   The vessel AIS device receives message 17 for accurate modification.

The location of the AIS device is automatically transferred from GPS to DGPS, and has the same location value with the DGPS device for measurement.

< AIS screen with or without DGPS broadcasting>

| Without DGPS broadcasting | With DGPS broadcasting |
| --- | --- |

< Location with or without DGPS modification >

| Without DGPS broadcasting | With DGPS broadcasting |
| --- | --- |

.5   Problem

.1   Seafarers are unaware of the DGPS service of the AIS.

.1   Awareness on the DGPS modification of AIS device: 20%

.2   Awareness on the DGPS service of AIS station: 20%

.3   Improvement rate of location error through DGPS service: 60%

.4   Support to expanding DGPS service: 80%

.2   Malfunctioning of some vessel AIS devices

.1   Vessel AIS devices made by some manufacturers fail to receive DGPS message or malfunction.

.6   Expected outcome

.1   Improved accuracy in location vessels with AIS

.1   	Location error: 30 m before DGPS service → Within 10 m after using the service

.2   Cost reduction in purchasing DGPS receiver

.1   	Precise data on vessel location is needed when navigation the coast. However, the DGPS service eradicates the necessity to buy expensive DGPS receivers.

.2   Utilization of AIS binary message: Waste ship monitoring system

.1   Progress

.1   Establishment of the waste ship monitoring system: July 3. 2007 to September 1. 2007

.2   Test operation of AIS and interface installed on waste ship: September 2007 to October 2007

.3   Expansion of AIS and interface installment to ships carrying dredged soil: May 2008 to July 2008

.2   Main aspects

.1   Real-time monitoring of the navigation data and disposal status of waste ships by receiving the information as AIS message 6.

.1   Installment of the interface device: AIS, GPS, applied handset, linkage with the disposal valve sensor

.2   Regularly-received data from the sensor attached on the disposal valve is assembled by the handset device to in AIS binary message format. Then, this is sent to the control center using AIS. The control center receives it and sends an Ack. signal to the vessel to confirm the receipt.

.2   The web-based service uses the internet to provide disposal data and monitoring service to relevant organizations and companies that discharge waste in the ocean.

< Monitoring screen showing the waste ship >

< Waste valve usage data packet format >

< Expanded AIS message used in Korea (DAC is defined as 440 and 441) >

.3   As seen above, there are 24 types of messages used in Korea which are expanded versions of AIS message 6, and at the same time abides by the expansion standard of IEC and ITU.

.3   Expected outcome

.1   Prevention of illegal waste disposal

.1   Thanks to real-time monitoring of waste disposal

.2   Trust from the public through transparent management of waste ships

.2   Increased efficiency of waste disposal management

.1   Real-time monitoring enables immediate confirmation of illegal disposal.

.2   The disposal location can be confirmed on screen, eliminating the need to use maps.

.3   Improved convenience of the public and streamlined administration

.1   It is no longer necessary to print out the location of waste ships every 30 minutes to submit it to the coast guard.

.2   Owners of waste ships can manage the navigation inside the office by using navigation data.

.3   Safety data messaging service

.1   Progress

.1   System established in August 2009

.1   Development of the system was completed in the GICOMS situation management system based on the M-Gov service.

.2   AIS text messaging system was completed on the GICOMS website.

.2   Test operation: August 17 to 21, 2009

.1   Navigation warning: Text message sent using cell phones and AIS

.2   Main news and accident information: Cell phone message

.3   System upgrade: August 24 to 31, 2009

.1   Text messaging also enabled in the VMS system

※ User can choose a certain station or vessel to prevent too many data being sent from the AIS.

.2   Demand from users identified and reflected

.4   Service launch: September 1, 2009

.1   Navigation warning, main news, and accident information messaging service

.2   Main aspects

.1   Service

| Type | Details | Collection method |  |
| --- | --- | --- | --- |
| Navigation warning | Marine drill, shooting, research | Korea Hydrographic and Oceanographic Administration data | Data linked to GICOMS |
| Accident information | Marine accidents | Data from local agencies or the coast guard |  |
| Main news | Daily headline news | Data from Yeonhap News | Data linked to GICOMS |

.2   Subject and scope

| Type | Subject | Scope |  |
| --- | --- | --- | --- |
| Cell phone | Safety manager in the vessel owner | GICOMS registerer | Flag ship |
| AIS | Vessel | Around 100 miles of the coast | Flag ships with AIS or foreign flag ships |

.3   Service method

| Type | Data limit | Content |  |
| --- | --- | --- | --- |
| Cell phone | Korean 80 letters | Navigation warning | Navigation planning |
|  |  | Headline news | Situation management |
| AIS | English 150 letters | Navigation warning | Navigation planning |
|  |  | Accident information | Situation management |

.4   Service Time

| Type | Cell phone | AIS |  |
| --- | --- | --- | --- |
| Navigation warning | Upon warning registration | Upon warning registration |  |
| Headline news | 10:00 ~ 17:00 | - | Headline news relevant to marine safety |
| Accident information | Upon accident registration | Upon accident registration | Accidents like sinking and massive oil spill that disrupt the marine traffic |

.3   Expected outcome

.1   Seafarers can prevent additional marine accidents by swiftly receiving navigation warning or accident information.

.1   Vessels that installed AIS can receive the text message even though they do not have NAVTEX.

.2   Safety managers can also support safety management on land by swiftly receiving navigation warning or accident information.

.1   Staff on land failed to acquire information quickly since they had to depend on conventional web-sites or paper documents.

.4   Navigation data provision on traffic congestion and disaster-causing vessels

.1   Main aspects

<Traffic congestion system structure>

.1   Navigation data cells (Cell) are designated around 50 miles (AIS detection zone) off the Korean coast.

※ Cell: The smallest size of water space where disaster-causing vessels navigate or traffic is congested.

.1   The scope of the service is limited to main routes to reduce system load and minimize data amount.

.2   The cell is based on the small marine districts of the marine district map, eliminating the aversion felt by seafarers and increasing the usage.

※ Korea's marine district map consists of 1,318 large marine districts
 (30x30 miles) and 11,862 small marine districts (10x10 miles)

.2   The system automatically analyzes cells where disaster-causing vessels navigate or with traffic jams on real-time, and sends the data to nearby vessels through the AIS.

.1   Definition of a disaster-causing vessel (tentative): Passenger vessel navigation at more than 35 knots, tugboats for crane barge, vessels carrying dangerous cargo, and large vessels

.2   Definition of the traffic congestion area (tentative): An area with more than 50 vessels navigating.

<Analysis on traffic congestion area by stage>

<Congestion area and navigation warning>

.3   Data provision method

.1   Disaster-causing vessel: Notify the vessels within 10 miles around the disaster-causing vessel of the fact.

.2   Traffic congestion: Notify all vessels navigation on the Korean coast.

.2   Progress and plans

.1   May to July, 2009: System analysis and design

.2   August 2009 : Division of cells for main routes and analysis on congestion and test operation of disaster-causing vessel data system

.3   September to October, 2009: System development

.4   November 2009: Distribution of introduction pamphlets and test operation of the system

.5   December 2009: System launch

.3   Expected outcome

.1   Traffic congestion on the route can be predicted in advance to advise alternative routes if necessary. As a result, congestion in a certain route can be prevented.

.2   Prevent marine accidents by raising vigilance of seafarers with the real-time identification of the status of disaster-causing vessels.

3   Future plans

.1   Korea established and has been operating GICOMS since 2002, and has been improving it for the land-based e-navigation system since 2009.

.1   The system will provide information not only for the government but for the public and the actual users in the maritime sector.

.2   Project time and budget: Five years (2010-2014), around 14.5 billion KRW

.1   Improvement of the GICOMS information generation system for marine safety

.2   Enhancement of the GICOMS information provision method for marine safety

.3   Globalization of the GICOMS by establishing a global marine safety information network

.4   Strengthened prevention of marine accidents through reinforced marine surveillance

.5   Establishment of an advanced disaster management system for marine environments and ports that focuses on the field