Annex 1: Methodology for developing eNavigation user needs using a task-based approach - Example using Maritime Safety Information (MSI)

- Executive summary

  - Background

The IMO have defined eNavigation as “the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment”. The IMO Strategy, as agreed at MSC 85, further states that eNavigation should be led by ‘User Needs’.

The IALA eNavigation Committee have been working to address this challenge in order to support the work of the IMO, and have taken this opportunity to explore the subject of Maritime Safety Information (MSI) in terms of ‘Enhancement’ of this service based on ‘User Needs’ captured as part of the IALA User Needs Analysis.

  - Purpose

The IALA eNavigation Committee have accepted that developing eNavigation ‘User Needs’ is a highly complex, detailed, and resource intensive task. The Committee’s Working Group (WG1) on Strategy and Operations have therefore decided to initially explore the issue of MSI in detail, in order to identify areas that may be improved through the application of eNavigation. This exercise has been undertaken as an example of how a specific issue of user needs can be systematically investigated in terms of Harmonised Collection, Integration, Exchange, Presentation, and Analysis.

The purpose of this exercise was only to give an example of an approach to developing and mapping eNavigation user needs. In no way should this exercise be assumed to be a complete work programme to enhance the provision of MSI through eNavigation.

  - Recommendations

The Safety of Navigation Sub-Committee is invited to make note of the following recommendations:

- Developing eNavigation user requirements is a highly complex exercise that requires the commitment of resources and the involvement of key stakeholders.

- A systematic approach needs to be taken for this activity, boundaries must be set, and a formalised ‘mapping technique’ to track information flows, terminology and responsibilities is highly desirable. The exercise outlined in this paper used Enterprise Architect software (others are available).

- When applying this technique to the subject of MSI, it became clear that the provision of MSI has many current strengths but also provides opportunities for improvement. In brief, the shoreside organisation by the hydrographic community has been proven to be very effective - however great opportunities exist to improve the onboard analysis and presentation. Opportunities also exist to improve the accessibility of such essential information to non-SOLAS vessels.

- The improved application of MSI onboard vessels has been clearly identified as a user need by mariners, and work on this issue under the agenda of eNavigation should continue.

- The subcommittee is invited to consider how this type of approach may be applied to other areas of eNavigation development and the necessary allocation of resources.

- Proposed methodology

  - Underlying principles

The proposed methodology has been developed in order to ensure that, in all aspects of eNavigation, the system design and level of automation follows from a clear understanding of user needs. The proposed methodology is based upon well recognised system engineering principles that have been previously applied across a wide range of different domains.

- The development of clear, specific and measurable user requirements that are independent of technology.

- The mapping of user requirements to (future) system requirements enabling full traceability and determination of how requirements are met.

- User requirements based upon well recognised operational tasks, generic functions and information flows, independent of any local variance in institutional structures or responsibilities.

Of particular importance to eNavigation is the need to be independent of phase of implementation. This method is designed to develop a set of holistic ‘all-encompassing’ requirements that are equally valid regardless of how they are implemented. To this end, many of the requirements are met today with human procedures - and the extent to which these user requirements are translated into system requirements in the future will need to be determined.

The eNavigation concept is considered in terms of information flows between users, which constitute sources and sinks for information. Such ‘peer-to-peer’ applications encompass ship-to-shore, shore-to-ship, shore-to-shore and ship-to-ship information flows – all of which will need to be identified and methodically developed into user requirements.

User requirements can be described in terms of information and functionality delivered at the Human-Machine-Interfaces (HMI), both shipboard and ashore, and by the functional links in-between those peer-to-peer applications.

  - Methodology

In developing a set of preliminary user requirements, a methodology was applied broadly based upon the following sequential steps:

- Define scope of each functional application and describe how the application is undertaken today.

- Identify strengths in today’s solution (institutionally, operationally, technically, etc) and also identify weaknesses and potential means of improvement.

- Identify key external boundaries with other applications and/or third parties, clearly documenting assumptions.

- Decompose each application into a series of processes and information flows, categorising each as collection, integration, exchange, presentation and analysis, taking into account:

- Location of the participating users: onboard, onshore, etc.

- Distributed responsibilities of stake holders.

- The nature and amount of data to be exchanged.

- Quality requirements: usability, accuracy, integrity, reliability or availability, continuity, time behaviour, maintainability, security etc.

- For each individual process, determine the underlying user requirement(s), independent of the means of provision or scope of technology.

This process must be repeated for each identified functional peer-to-peer application, noting common requirements, terminology and key inter-dependencies. This process can then be readily extended to identify system requirements and undertake a formal gap analysis.

- Scope of this document

  - Maritime Safety Information (MSI)

The decision to choose MSI as the particular single function from which to start the development of eNavigation arose from the recommendations of the IALA eNav6 meeting of March 2009.

This meeting had concluded that the existing procedures and protocol currently in place as part of the World-Wide Navigational Warning Service (WWNWS) offered a good example of a shore-to-ship structure, with a clearly defined scope, containing elements of collection, exchange, integration, presentation and analysis. It was identified by the user needs analysis that the provision of MSI had the potential for tangible benefits to be identified in relation to how mariners may receive and use MSI onboard in the future.

To begin with, the definition of MSI contained in SOLAS Chapter IV, Radiocommunications, Regulation 2, 1.9 was acknowledged:

- “Maritime safety information means navigational and meteorological warnings, meteorological forecasts and other urgent safety related messages broadcast to ships.”

Due diligence and acceptance was also agreed in relation to the existing architecture for the co-ordination and promulgation of MSI as defined in the recently amended IMO documentation:

- Resolution A.705(17) – Maritime Safety Information,

- Resolution A.706(17) – World-Wide Navigational Warning Service, and

- Joint IMO/IHO/WMO Manual on Maritime Safety Information

The stepped approach of the differing types of Navigational Warnings (NAVAREA, Coastal and Local) and the underpinning on-shore institutionally structured organisations respectively responsible for the selection and distribution of the different types of warnings, was seen as a considerable strength to the current system. When looking forward to what eNavigation may provide in the future, an improved network of storage, sharing and distribution of data in a variety of formats (by what could be described as MSI Service Providers (MSI SPs)) was identified as an objective worth considering.

  - Scope of application

The scope of this exercise encompasses the provision of MSI, as defined above, by organisations identified as MSI SPs to mariners. It specifically excludes the provision of other navigational information (such as electronic charts, radar and position fixing, etc).

The provision of other types of navigational information may have very similar user requirements, potentially leading to common means of provision. It is clear that the mariner’s requirement is that all navigational information is presented in an integrated manner to support good decision making.

- Preliminary user requirements

  - Introduction and terminology

The following tables provide a two page extract from the preliminary set of MSI user requirements developed for the purpose of this document. The requirements are separated into ship-based and shore-based and are subsequently developed under Collection, Integration, Exchange, Presentation and Analysis.

In addition to each individual statement of requirement, the table includes a description of how the requirement is met today to provide the reader with a familiar frame of reference. It also demonstrates how this can be readily extended to include perceived strengths and weaknesses of the current solution and potential future improvements.

In developing the requirements, it has been necessary to develop a set of terms which should form part of a common library of terms to be used for all eNavigation user requirements.

- MSI (Maritime Safety Information) – as defined in Section 3.

- MSI SP (MSI Service Provider) – an organisation responsible for making available MSI to mariners – that today includes NAVAREA and National Co-ordinators and local port authorities, etc.

- User – all maritime users of MSI on-board SOLAS and non SOLAS vessels.

- Data Originator – any individual or organisation providing data to an MSI SP.

| Reference | User Requirement | Current solution | Strengths and weaknesses of current solution | Potential improvements from eNavigation |  |
| --- | --- | --- | --- | --- | --- |
| ON-SHORE |  |  |  |  |  |
| Collection |  |  |  |  |  |
| MSI_REQ_1 | The MSI SP shall enable appropriate data originators to provide reports, in accordance with international standards | MSI SPs make available contact details and relevant instructions for data originators Information received from variety of third parties via published means of contact (email, fax, phone, paper, etc) | Single point of contact and well co-ordinated/structured. Duties are well specified in IMO Resolution A.706(17) Inconsistency in format of incoming data | Greater level of automation in receipt and formatting of input data |  |
| Analysis |  |  |  |  |  |
| MSI_REQ_4 | The MSI SP shall assess and validate incoming reports to identify MSI | Manual subjective process, using charts and other relevant information | Potential for inconsistencies in working practises | Increased use of standardised decision support tools |  |
| Presentation |  |  |  |  |  |
| MSI_REQ_5 | The MSI SP shall format all MSI in a form suitable for subsequent integration and presentation on-board all vessels | Guidelines published for different types of warning | Format used dictates presentation to end user. Potential for inconsistencies in working practises | Separation of data format from presentation offers flexibility in on-board presentation, which will require a published standard |  |
| Exchange |  |  |  |  |  |
| MSI_REQ_6 | The MSI SP shall make available MSI relevant to their scope of responsibility | MSI SPs pass information to other MSI SPs that is out of their scope of responsibility (ie NAVAREA, NAVTEX service area, etc) MSI SPs monitor the on-air output of other MSI SPs | MSI SP relationships are well established Exchange of information is unstructured and ad hoc | Establish efficient and robust exchange mechanism providing information in known format |  |
| MSI_REQ_10 | MSI shall be available to all users to whom each item of MSI applies - in both location and time | Variety of different broadcast methods and schedules In-force bulletin used for MSI no longer being broadcast | Repeated transmissions necessary to ensure that users receive the broadcast | Improved availability of all valid MSI, regardless of date of issue |  |
| MSI_REQ_11 | MSI shall be available to all types of vessel, including both SOLAS and non-SOLAS vessels | Different technologies used for ocean and coastal MSI. Coastal NAVTEX service and RT more suitable for non-SOLAS vessels. Often rely on posters and notices for communication of MSI to non-SOLAS vessels | Needs of non-SOLAS users not well catered for | Provide MSI in a standardised form that can be used to develop solutions for all sectors of the market, removing the dependency on specific equipment |  |
| Reference | User requirement | Current solution | Strengths and weaknesses of current solution | Potential improvements from eNavigation |  |
| ON-SHIP |  |  |  |  |  |
| Collection |  |  |  |  |  |
| MSI_REQ_12 | The user shall acquire all MSI appropriate to operational requirements | Messages are currently received by multiple systems (NAVTEX, SafetyNET, voice, etc) and manually logged | Risk of single person error, and lack of traceability in assessment of MSI by the user |  | The system should display all MSI relevant to voyage planning and/or voyage execution and save a record of all MSI received |
| Integration |  |  |  |  |  |
| MSI_REQ_15 | MSI alarms shall be integrated with the vessel’s navigation alarm system | No integration with other on-board navigation alarm systems. No electronic/computerized treatment of MSI | Solely dependent on the MSI information collector presenting MSI to any concerned people |  | Enables consistency of presentation of alarms to user |
| Exchange |  |  |  |  |  |
| MSI_REQ_16 | Users shall be enabled to exchange MSI between vessels | Currently achieved by VHF transmission | No automatic procedure/system allowing the user to broadcast MSI |  | More effective exchange of MSI between vessels |
| Presentation |  |  |  |  |  |
| MSI_REQ_17 | All relevant MSI shall be presented in a user friendly manner and in conjunction with other related navigational information | Presently not integrated in terms of location and format onboard ships | Lack of integration into the navigation system doesn’t guarantee the proper treatment of the MSI |  | MSI information should appear like any other navigational or safety information readily available on the navigational system |
| MSI_REQ_18 | An alarm shall be presented to the user for any MSI when it represents a threat to the safe navigation of the vessel | Currently, the only alarm that is presented occurs at the moment of receipt (EGC) | Risk that MSI is not taken into account at the moment it is needed even if it has been previously acquired |  | System should be able to alert the user in due time and location when any MSI becomes relevant to the safe navigation of its ship |
| Analysis |  |  |  |  |  |
| MSI_REQ_24 | The criticality of each item of MSI shall be analysed | Continual process of assessment, undertaken manually today | The user analysis is the sole agreed way to perform a practical and knowledgeable evaluation of MSI criticality |  | Should still depend primarily upon human analysis |
| MSI_REQ_25 | The analysis of MSI shall not be at risk of single person error | MSI is solely acquired, analysed and recorded by the watch keeper at the time of receipt | At the moment it is at risk of a single person error |  | eNavigation should provide a phased MSI alarm system to assist risk avoidance |

The preliminary set of MSI user requirements comprises twenty five detailed user requirements.