[Draft; 2009-06-10]

IALA’s Vision for the 
Shipboard e-Navigation System Architecture

Table of contents

1	Introduction	2

2	Development of a shipboard e-Navigation system architecture	4

2.1	Overview and most fundamental ideas	4

2.2	The Data Acquisition and Data Communications level	6

2.3	The Data Presentation and User Interaction level	8

2.4	The Data Evaluation and Data Storage level	9

3	The place of the shipboard architecture in the overarching e-Navigation architecture	11

4	How to move from today’s shipboard architecture to tomorrow’s e-Navigation shipboard architecture	13

5	Conclusions	15

6	References	15

Annex 1: Existing SOLAS V mandatory carriage requirements	16

Annex 2: Existing SOLAS IV mandatory carriage requirements (example)	18

# Introduction

The purpose of IALA is to ensure that seafarers are provided with effective and harmonised marine aids to navigation services worldwide to assist in safe navigation of shipping and protection of the environment.  Comprised of navigation authorities, manufacturers and consultants, IALA’s role is to harmonize aids to navigation worldwide and to ensure that the movements of vessels are safe, expeditious and cost effective and at the same time protect the environment.

IALA represents aids to navigation authorities that will be responsible in the future for the provision of e-navigation services to the mariner.  These services are comprised of both operational and technical e-Navigation services.

Taking into account the needs of mariners (i.e., user needs), developments in technology, and the requirements and constraints of aids to navigation authorities, IALA has begun the development of the operational services as well as a common shore-based e-navigation system architecture with a detailed description of its shore-side components. This is clearly in IALA’s remit.  In particular, these technical e-Navigation services must be compatible with the shipboard e-Navigation system architecture and its equipment on every level of machine-to-machine interaction (as indicated in Figure 1 overleaf).

Following this rationale, IALA has developed this document as its vision for the shipboard e-navigation system architecture.  It addresses the fundamentals of the architectural design and the functional levels within the ship’s architecture.  It is then shown how the shipboard architecture fits with IALA’s common shore-based system architecture.  Finally, this document considers how the existing equipment installations might migrate to IMO’s overall e-navigation architecture.

Ship

environment

IBS

World Wide Radionavigation System (WWRNS) of IMO, including GNSS, GNSS augmentation and terrestrial backup

Transceiver station

Ship’s

sensors

Shipboard

Applications

INS

Shore-based

technical

e-Navigation

services

(= shore-based

applications)

Application-to-

application

functional

connection

Physical

Link (e.g. radio link)

other

ships

other

ships

mariner

Shore- based users

(e.g. VTS Operators)

Shore

environment

Figure 1: e-Nav architecture

# Development of a shipboard e-Navigation system architecture

After the principle elements of the overarching architecture for e-Navigation have been introduced in the previous chapter, this chapter now focuses on a possible future architecture for the shipboard electronic system, which supports the e-Navigation concept. This shipboard architecture is called “shipboard e-Navigation system architecture”.

Since the architecture under consideration is the system architecture for an electronic system, the architecture under consideration essentially is a technical architecture and must be developed using state-of-the-art engineering principles. Some of these have been stipulated by IMO’s “strategy for the development and implementation of e-Navigation” [1], for example that the technical system should be an “open” system with standardised interfaces between modules and components.

The following sections develop the shipboard e-Navigation system architecture in a step by step fashion.

## Overview and most fundamental ideas

According to the above IMO e-Navigation strategy, every consideration has to start with the user requirements. The user requirements inform engineers what the user wants to see on the presentation surface i.e. on the Human Machine Interface provided by the shipboard system in order to perform the user tasks. These information needs of the user lead to a set of individual information items, which in turn translate to data objects being presented by the machine via its Human Machine Interface (HMI) to the user: The machine(s) is only capable of dealing with data. Whether the data presented by the machine to the user is indeed meaningful to him in a given situation, i.e. thus becomes information, only the user can decide.

This important first step is depicted in the top of Figure 2 (overleaf): The technical functional block which is directly in contact with the user and provides him the required Human Machine Interface is depicted in the top and is called “Data Presentation and User Interaction” in accordance with its primary task. It should be noted, that “data” in this general context could be any kind of data, including digitized voice. The important distinction between information and data is introduced by the arrows at the left margin of the Figure, indicating the “information domain” in which the user resides and the “data domain”, to which all machines belong.

But where does the data to be presented to the user come from, and where does data input by the user go to? The answer to that question is given by the remaining functional blocks of the Figure 2, as far as the shipboard environment is concerned:

Transparent to the user at the next level down the data must be stored and evaluated. This in turn requires that that data must be gathered and transferred from its original source which is outside the own shipboard environment (i.e. provided from ashore and/or from other ships). These two processes are depicted by the functional blocks “Data Evaluation and Data Storage” and “Data Acquisition and Data Communications”. The functional block “Data Acquisition and Data Communication” interacts with the outside of the shipboard environment using a variety of data communication links. Again it should be noted, that data could also be voice.

Figure 2: Depiction of the fundamental technical modules for the shipboard e-Navigation system architecture

Any functional block of the shipboard e-Navigation system architecture must be able to exchange data with any other functional block, i.e. maximum connectivity is a requirement. To support this, a bus structure is introduced, which allows every functional block to connect with any other functional block.

Regarding “Communications”, i.e. transport of data (including digitized voice) through the shipboard e-Navigation system, it should be noted that there are two principle relationships for the shipboard e-Navigation system architecture:

- Internal to the shipboard environment: There is a direct connection between the Data Presentation + User Interaction level with the Data Acquisition + Data Communication level  (Therefore the bus structure allows for bypassing the Data Evaluation + Data Storage level).

- External to the shipboard environment: There are a variety of Data Communication Links in particular in the MF, HF, VHF, and even higher bands (refer for example to the IALA WWRCP), each of which caters for a specific set of functional requirements.

It should also be noted that at this stage the shipboard e-Navigation system architecture is composed of functional blocks instead of devices. Eventually there will be the need to define devices, of course, even only for the sake for required testing and for the consequential type approval. Conceptually, however, to use function blocks from the outset is the consequence of the user-requirement driven approach to start technical system development with the required functionality provided by the devices. The distribution of the total required functionality on the different devices would be done at some later stage, leading eventually to a precise definition of both devices and their functionality in detail.

## The Data Acquisition and Data Communications level

In this section the level which interfaces the shipboard e-Navigation system to its external e-Navigation environment is introduced. From IALA’s point of view the Data Acquisition and Data Communication level contains at least the following devices (refer to Figure 3):

- an AIS (V2.0) mobile station: Building on the core functionalities of the present AIS shipboard mobile Class A station, and improving several technical features in detail, this device exhibits improved AIS related messaging capabilities and improved frequency channel utilization. Thereby, the LRIT, amongst other applications, is supported by an improved satellite tracking capability of the AIS signals. Also, the AIS (V2.0) mobile station supports the GMDSS by absorbing the relevant operational functionalities of the VHF-DSC components. Thus, this AIS (V2.0) mobile station may ultimately render the VHF-DSC shipboard component of the GMDSS obsolete. It would use the globally designated VHF Channel 70 for all AIS management purposes but with the AIS’s own, more frequency efficient TDMA radio protocol instead of the DSC radio protocol. Finally, the AIS (V2.0) mobile station would no longer require a mandatory user interface making the so-called “Minimum Keyboard and Display” of the present AIS shipboard stations obsolete. The interaction with the user would be done completely at the “Data Presentation and User Interaction” level by default.

- an  integrated digital terrestrial communication device: This devices allows for digital voice/data communication in several terrestrial frequency bands, such as MF, HF, VHF, and even higher frequencies, using efficient digital data radio transmission protocols. The device may also autonomously select the most suitable, available terrestrial frequency band and protocol for a data transmission, depending on current location, radio conditions, and costs. Thus, this integrated digital voice/data communication device may ultimately render the shipboard NAVTEX receiver and the HF-DSC shipboard component of the GMDSS obsolete. It would use the globally designated MF and HF channels with more frequency efficient radio protocols. This device also does not require a user interface, because the interaction with the user again is done completely at the “Data Presentation and User Interaction” level by default. This app
 
Note: The device is only using terrestrial frequency bands, the shipboard satellite communication device(s) is/are still necessary. Conceptually, the shipboard satellite communication device(s) also belong(s) to the “Data Acquisition and Data Communication” level (compare Figure 3).

Note: There may additional digital terrestrial communication devices on this “Data Acquisition and Data Communication” level, for which there is intention to integrate them into the above device, namely e.g. WiFi or WiMax communication devices.

- an integrated radio navigation device: This is a device using all available components of IMO’s World Wide Radionavigation System (WWRNS) simultaneously to provide the best electronic position fix for the ship. Those components of the WWRNS are the GNSS, satellite- and terrestrial augmentation systems as well as terrestrial backup systems. The shipboard receiver “integrates” signals from the different available WWRNS components by combining the pseudo ranges or TOE/TOA measurements of all received signals across the individual WWRNS component boundaries. Such a device would render the different existing individual shipboard radio navigation devices obsolete. Again, such a device also would not require a user interface, because the interaction with the user again is done completely at the “Data Presentation and User Interaction” level by default.

- a radar sensor: This is a potentially improved shipboard radar device which does not require a user presentation interface but forwards the radar-derived data to the “Data Evaluation and Data Storage” level for evaluation and eventual presentation to the user at the “Data Presentation and User Interaction” level. Even with the advent of the AIS (V2.0), shipboard radar is envisaged to remain an important data acquisition tool also in the future.

While this list of devices is not complete regarding shipboard data acquisition (“sensors”) and communications devices, it constitutes a major portion as far as the e-Navigation is concerned. Other devices which conceptually also belong to this “Data Acquisition and Data Communication” level are gyro, echo sounding device, SDME, etc. The “Data Acquisition and Data Communication” level may even include an optional high-speed land line data exchange device (when at berth).

Figure 3: Shipboard e-Navigation system architecture with functional block “Data Acquisition + Data Communications” further developed into devices

## The Data Presentation and User Interaction level

This “Data Presentation and User Interaction” level is specialized on all functionalities (and eventually all necessary hard- and software devices) to present the available data to the user, as required, and in the user-required format, or to accept user interaction with the shipboard e-Navigation system. This level does not contain any data acquisition and data evaluation functionalities because there are other modules of the shipboard e-Navigation system architecture specialized in these functionalities.

This level thus is the Operational Presentation Surface of the whole of the shipboard e-Navigation system architecture. It will contain certain modules, which are well-known to the mariner and are therefore only introduced briefly (compare Figure 4 overleaf).

Figure 4: Shipboard e-Navigation system architecture with functional block “Data Presentation + User Interaction” further developed into functional modules

A harmonisation with the INS concept will be required.

Again it should be noted, that voice conceptually is considered a special variety of data. Hence, there will be a voice communications part.

## The Data Evaluation and Data Storage level

This “Data Evaluation and Data Storage” level is the central data storage, evaluation and correlation core (IT kernel) of the shipboard e-Navigation system architecture. Eventually, this level will be mainly implemented by software modules running on at least two computers (for redundancy reasons). From an organisational, data modelling, IT engineering and maintenance point of view it is beneficial to subdivide the required functionality in accordance with the data objects they deal with algorithmically. Hence, a potential subdivision in functional modules would be “Traffic Object Correlation”, “Sea State and Waterways Data”, and “Maritime Services Directory” (compare Figure 5), and further modules would be added as needed.

Figure 5: Shipboard e-Navigation system architecture with functional building block „Data Evaluation + Data Storage“ further developed in functional modules

# The place of the shipboard architecture in the overarching e-Navigation architecture

IMO has defined overarching e-Navigation architecture, saying that it should consist of a shipboard side, a shore-side, the communication links in-between and that it should also take into account position fixing (compare Figure 1 above). IALA has defined a so-called “common shore-based e-Navigation system architecture” as a recommendation for its national membership i.e. Aids-to-Navigation and VTS authorities.

The previous chapter introduced IALA’s vision of a shipboard e-Navigation system architecture to some level of detail. It is now required to fit that shipboard e-Navigation system architecture into the overarching e-Navigation architecture. This is done again in a step-by-step fashion, both steps building on Figure 1 (see above) and the second step adding detail to the first.

Figure 6 shows the complete e-Navigation architecture while the structural details of both the technical shipboard and shore-based e-Navigation system architectures are not yet shown.

Figure 6: The place of the shipboard e-Navigation system architecture within the e-Navigation architecture – simplified presentation

In the following Figure 7 the above details regarding the shipboard and shore-based e-Navigation system architectures have been included.

Figure 7: The place of the shipboard architecture in the overarching e-Navigation architecture – more detailed presentation

# How to move from today’s shipboard architecture to tomorrow’s e-Navigation shipboard architecture

IMO has recognized that e-navigation can not be implemented in a single instant.  Rather, ships would likely begin to migrate towards the shipboard e-navigation system architecture as the technology becomes available and as ship owners begin to see the benefits of such a migration. To use IMO’s own words: “Implementation itself, [should happen] in phases, perhaps based on a voluntary equipage of (integrated) existing systems to begin with, but with mandatory equipage and use of a full e-navigation solution in the longer term.” [1, §9.9.3]

Today, the user requirements for safety of navigation equipment are addressed in SOLAS V/19 and associated performance standards (see Annex 1). SOLAS IV addresses the Global Maritime Distress and Safety System requirements (see Annex 2). When comparing the requirement stated in the two above annexes with the technical capabilities of the functional modules and devices introduced above, the same technologies used may meet SOLAS Chapter V requirements as well as SOLAS Chapter IV requirements.  As an example, AIS (V.2) when introduced together with digital voice/data communications can satisfy a number of the requirements for safety of navigation equipment and GMDSS.

However, the IMO vision for e-navigation is not solely for SOLAS-class ships.  Accordingly, IMO envisions a level of scalability such that non-SOLAS class ships may attain an some level of conformity with the shipboard e-navigation system architecture, which would allow them to exchange certain e-Navigation data with those ships required to meet the full e-navigation system architecture requirements. That is, not every ship will need to migrate in accordance with the vision explained.

The focus of IALA is to provide the maximum benefit of e-Navigation services from ashore to shipboard e-Navigation users.  In order to achieve the maximum benefit, ships would need to be equipped with an e-Navigation compliant equipment suite in concert with the accepted architecture.

Therefore, the migration process from today’s numerous SOLAS V/19 equipment approvals to the shipboard e-navigation system architecture is envisaged to be a long one, perhaps out to 2020.  This timeframe roughly coincides with the phased implementation of required carriage of ECDIS (2012-2018).

The following diagram takes the above functional block diagrams of the shipboard e-navigation system architecture and identifies representative equipment and services available today that match the given functional block in some cases.  Similarly, it envisions the e-Navigation environment in which some new devices will replace one or more existing ones.

Migration

during

e-Navigation

implementation

Today (SOLAS)

Separate type approved radar, ECDIS, Conning Display, and Workstation for INS, MKD with LCD etc

Display screens which can be configured to suit need.

Separate components of radar, ECDIS, VDR etc

Central processor unit(s) handling all data processing activities

Separate type-approved components such as radar, GNSS, Gyro, SDME, Echo-sounder, AIS, VHF radio etc.

Data Presentation and User Interaction

(Includes display screens and keyboards)

data acquisition and communications

data storage, evaluation, verification, processing

data links

(radar, radio, satellite, AIS VDL, telephone cable, WiFi, etc.)

data links

radio (MF, HF, VHF),

satellite, AIS, telephone etc.

Individual “sensors” for each function, AIS (V2) mobile station, digital voice/data communication device, integrated radio navigation device, radar sensor device, WiFi device etc.

Shipboard e-Navigation suite

Shipboard e-Navigation system architecture

Data links

AIS VHF data link (V2), digital voice/data radio links (MF, HF, VHF etc), radio links of WWRNS, radar link, telephone, WiFi,  WiMax etc

# Conclusions

IALA represents Aids to Navigation authorities that will be responsible in the future for the provision of e-navigation services to the mariner.  IALA has begun the development of the operational services as well as a common shore-based e-navigation system architecture with a detailed description of its shore-side components. This is clearly in IALA’s remit.  In particular, these technical e-Navigation services must be compatible with the shipboard e-Navigation system architecture and its equipment on every level of machine-to-machine interaction

This document described the shipboard e-navigation system architecture and how that architecture fits with IALA’s common shore-based system architecture in IMO’s overall e-Navigation architecture.

The following IALA documentation provides more detailed information:

- E-Navigation Frequently Asked Questions (FAQs): These FAQ’s are maintained up-to-date by the IALA e-Navigation Committee at its biannual meetings usually held during March and September. e-Navigation is a rapidly evolving IMO concept and IALA takes all reasonable efforts to reflect the most current IMO decisions in these FAQ’s.

- IALA Recommendation e-Nav 101 – This document describes the e-Navigation concept from IALA’s shore perspective.

- IALA Recommendation e-Nav 201 – This document describes the common shore-based e-Navigation system architecture IALA developed as a recommendation to its membership.

- IALA e-Navigation World-Wide Radio Communications Plan (WWRCP) – This document identifies radio communications systems today and envisioned for the future in an e-navigation environment.

- IALA e-Navigation World-Wide Radio Navigation Plan (WWRNP) – This document identifies those radionavigation systems existing today and envisioned for the future in an e-navigation environment. It is loosely modelled on the Draft European Radio Navigation Plan and the U.S. Federal Radionavigation Plan.

# References

[1]	IMO. Maritime Safety Committee. 2008. “Strategy for the Development and Implementation of e-Navigation”, MSC85/26, Annex 20.

# Annex 1: Existing SOLAS V mandatory carriage requirements

| No. | Facilities | Note | 0 to 150 GT | 150 to 300 GT | 300 to 500 GT | 500 to 1600 GT | 1600 to 3000 | 3000 to 10K | 10K to 50K | 50K GT < |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.1.2 | Magnetic Compass |  |  |  |  |  |  |  |  |  |
| 2.1.4 | Paper chart or ECDIS |  |  |  |  |  |  |  |  |  |
| 2.1.5 | Back-up ECDIS | ECDIS in 2.1.4 |  |  |  |  |  |  |  |  |
| 2.1.6 | GNSS(GPS, DGPS) |  |  |  |  |  |  |  |  |  |
| 2.1.7 | Radar Reflectors | If possible |  |  |  |  |  |  |  |  |
| 2.1.9 | Communication for heading information | In case of emergency steering |  |  |  |  |  |  |  |  |
| 2.2.1 | Spare Magnetic Compass |  | Passenger ship |  |  |  |  |  |  |  |
| 2.3.1 | Echo Sounder |  |  |  |  |  |  |  |  |  |
| 2.3.2 | X band radar |  | Passenger ship | Passenger ship |  |  |  |  |  |  |
| 2.3.3 | Electronic Plotting Aids | EPA | Passenger ship | Passenger ship |  |  |  |  |  |  |
| 2.3.5 | THD (Transmitting Heading Device) |  | Passenger ship | Passenger ship |  |  |  |  |  |  |
| 2.4 | AIS (V1.0) |  | Passenger ship | Passenger ship | Passenger ship |  |  |  |  |  |
| 2.3.4 | SDME (through Water) |  | Passenger ship | Passenger ship |  |  |  |  |  |  |

SOLAS-V Carriage Requirement (2/2)

| No. | Facilities | Note | 0 to 150 GT | 150 to 300 GT | 300 to 500 GT | 500 to 1600 GT | 1600 to 3000 | 3000 to 10K | 10K to 50K | 50K GT < |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.5.1 | GYRO Compass |  |  |  |  |  |  |  |  |  |
| 2.5.5 | ATA (Automatic Tracking Aid) |  |  |  |  |  |  |  |  |  |
| 2.6 | 2nd Compass |  |  |  |  |  |  |  |  |  |
| 2.7.1 | S band 2nd Radar |  |  |  |  |  |  |  |  |  |
| 2.7.2 | 2nd ATA |  |  |  |  |  |  |  |  |  |
| 2.8.1 | ARPA |  |  |  |  |  |  |  |  |  |
| 2.8.2 | Track Control System |  |  |  |  |  |  |  |  |  |
| 2.9.2 | SDME (Speed over Ground) |  |  |  |  |  |  |  |  |  |
| 20 | VDR, S-VDR |  | Passenger ship | Passenger ship | Passenger ship | Passenger ship | Passenger ship |  |  |  |
| SOLAS-V 19-1 | LRIT (Long Range Identification & Tracking | On or after 31 Dec. 2008 |  |  |  |  |  |  |  |  |
| SOLAS-V | BNWAS (Bridge Navigational Watch Alarm System) | On or after 1st July 2011 |  |  |  |  |  |  |  |  |

# Annex 2: Existing SOLAS IV mandatory carriage requirements (example)

| Area | Ship (TG) | VHF + DSC | MF + DSC | MF/HF + DSC + NBDP | Inmarsat | SART | EPIRB | NAVTEX | EGC | TWO-WAY | TWO-WAY AIR | VHF DSC WKR | MF DSC WKR | MF/HF DSC WKR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 300-500 | 1 (*1) |  |  |  | 1 | 1 | 1 | [1] | 2 |  | 1 |  |  |
|  | 500 and Over | 1 (*1) |  |  |  | 2 | 1 | 1 | [1] | 3 |  | 1 |  |  |
|  | Passenger | 1 (*1) |  |  |  | 2 | 2 | 1 | [1] | 3 | 1 | 1 |  |  |
| A2 | 300-500 | 1 (*1) | [1] (*1) |  |  | 1 | 1 | 1 | [1] | 2 |  | 1 | 1 |  |
|  | 500 and Over | 1 (*1) | [1] (*1) |  |  | 2 | 1 | 1 | [1] | 3 |  | 1 | 1 |  |
|  | Passenger | 1 (*1) | [1] (*1) |  |  | 2 | 2 | 1 | [1] | 3 | 1 | 1 | 1 |  |
| A3 | 300-500 | 1 (*1) |  | [1] (*1) | [1] (*1) | 1 | 1 | 1 | [1] | 2 |  | 1 | [1] | [1] (*1) |
|  | 500 and Over | 1 (*1) |  | [1] (*1) | [1] (*1) | 2 | 1 | 1 | [1] | 3 |  | 1 | [1] | [1] (*1) |
|  | Passenger | 1 (*1) |  | [1] (*1) | [1] (*1) | 2 | 2 | 1 | [1] | 3 | 1 | 1 | [1] | [1] (*1) |
| A4 | 300-500 | 1 (*1) |  | 1 (*1) |  | 1 | 1 | 1 | [1] | 2 |  | 1 |  | 1 (*1) |
|  | 500 and Over | 1 (*1) |  | 1 (*1) |  | 2 | 1 | 1 | [1] | 3 |  | 1 |  | 1 (*1) |
|  | Passenger | 1 (*1) |  | 1 (*1) |  | 2 | 2 | 1 | [1] | 3 | 1 | 1 |  | 1 (*1) |

(*1) Duplication is required ; 
[] Brackets can be omitted by the terms and conditions.

EGC: Enhanced Group Call	TWO-WAY AIR: TWO-WAY VHF Air band	WKR: Watch keeping receiver

TOE: Time of Emission (of signal); TOA: Time of Arrival (of signal)

SDME: Speed and Distance Measurement Equipment

This would be in accordance with IMO’s requirement for a “common maritime information/data structure” [1, §8.2.1].