Liaison Note to ITU-R WP5B

on spectrum requirements for future digital communication systems in conjunction with WRC-11 Agenda item 1.10

# Introduction

Noting that ITU-R Resolution 357 (Rev. WRC-07) invites IALA (together with IMO and IEC) to contribute to studies for WRC-11 agenda item 1.10 on the future use of the spectrum, considering the future requirements envisioned by ITU-R.  Further, noting that IMO has begun the development of e-Navigation, and has commenced discussions on the next generation of GMDSS, it is clear that the new digital communication systems will be integral to the maritime community in the coming years and careful consideration must be given to the user requirements and the available technology and the spectrum needed to meet these requirements.

# Discussion

At the request of the IMO, IALA has developed an e-Navigation Plan that addresses current and future e-Navigation requirements for maritime navigation and communication systems.  IALA’s vision and strategy for maritime systems propose a significant shift from analogue to digital communications in the VHF maritime band (Radio Regulations Appendix 18), as well as advanced AIS technologies, which IALA believes will greatly contribute toward the modernization of the GMDSS.  IALA’s e-Navigation Plan remains under development and requirements continue to evolve.  IALA is evaluating the spectrum needs for e-Navigation which are not channellised within Appendix 18 and will provide further comments in due course; however, it is suggested that the recommendations listed below should be treated as a matter of urgency, under agenda item 1.10 by ITU-R WP5B for consideration at WRC-12.

# Recommendation

“e-Navigation” as defined by IMO and envisioned by IALA requires appropriate provisions within the Radio Regulations as follows:

- The protection of AIS1 and AIS2 channels as distress and safety communications.

- Spectrum resources for application of the VHF data service as described in Recommendation ITU-R M.1842.  IALA envisage e-Navigation would require a minimum capacity equivalent to four 25 kHz channels however IALA are still in the process of evaluating the relevant studies.  Consideration needs to be given to the options of Simplex/Duplex and Wide/Narrow band.  (RR App 18 Footnote “o”.)

- AIS requirements for satellite tracking (channels 75 and 76), including a future GMDSS.

- Application specific communications.  Two channels for low volume TDMA digital communications using techniques similar to AIS.

- Agenda item for WRC 2016 to review VHF mobile band.

- France and the United States are developing digital data systems to support e-Navigation around 500 kHz.  DGNSS technology and e-Navigation digital communications technologies are likely candidates to make use of this band in the near future.  Allowing secondary use by amateur services is not in the interest of the maritime service.

# Action requested

IALA invites ITU-R WP5B to consider and action the above recommendations.