## Current and evolving wireless telecommunications technologies in the Maritime environment

There are an ever increasing number of wireless technologies which aim to deliver voice and data connectivity to mobile users and which may be of use in the maritime environment.  Aside from their technical capabilities, there may be regulatory issues and associated technical restrictions which prevent them being fully exploited.  Such issues may include:

- the ITU radio regulations do not permit maritime mobile operation in the frequency bands concerned

- frequencies are licensed on a national basis, such that there is no continuity of operation from country to country (this may even require equipment to be switched off when entering certain jurisdictions)

- the system parameters may have the potential to cause interference to ship-borne equipment

- planning parameters may make use at sea (or even in ports) complex or difficult

The following terrestrial technologies are currently in the process of either being rolled-out or standardised on an international basis and thus may be candidates for use in the maritime environment, particularly for commercial port services:

- Digital PMR (to replace analogue PMR for on-board comms): TETRA, TETRAPOL, P25, dPMR, DMR;

- 3G Mobile: UMTS (TDD & FDD), cdma2000, TD-SCDMA, IEEE802.20 (including iBurst);

- ‘3.5G’ Mobile: WiMAX;

- 4G Mobile: LTE, UMB.

In addition to these commercial service technologies, there are various technologies which may offer potential solutions under the banner of ‘licence exempt’ or ‘low-power’ technologies such as WiFi, Bluetooth, ZigBee and UWB.  The range of such services, however, is exceptionally limited (normally to 100 metres or much less) and the frequencies employed are globally allocated for the purpose (e.g. the 2.4 GHz ISM band) such that there is no need for a specific consideration of the impact on spectrum of their usage in a maritime environment.

Table A3-1 below details, for each of the commercial technologies, the frequency band(s) in which they ‘prefer’ to operate, i.e. those for which there are known services, as well as the general range of frequencies over which they are specified to operate.

| Technology | Frequency Range | Notes |
| --- | --- | --- |
| TETRA | 380 – 400 MHz 410 – 430 MHz 450 – 470 MHz 806 – 821 // 851 – 866 MHz | EN 300 392 |
| TETRA 2 (TEDS) |  | EN 302 561 |
| TETRAPOL | 70 – 933 MHz |  |
| P25 | 136 – 870 MHz |  |
| DMR | 30 – 900 MHz | TS 102 361 |
| dPMR | 30 – 900 MHz | EN 301 166 |
| GSM (incl EDGE) | 380 – 400 MHz 410 – 430 MHz 450 – 470 MHz 478 – 496 MHz 698 – 746 MHz 747 – 792 MHz 806 – 866 MHz 824 – 894 MHz 870 – 921 MHz 876 – 925 MHz 880 – 960 MHz 1710 – 1880 MHz 1850 – 1990 MHz |  |
| cdma2000 | 450 – 470 MHz 790 – 862 MHz 824 – 894 MHz 880 – 960 MHz 1710 – 1880 MHz 1850 – 1990 MHz 1820 – 2170 MHz 2300 – 2400 MHz 2500 – 2690 MHz |  |
| W-CDMA (UMTS) |  |  |
| TD-CDMA | 1900 – 1920 MHz 2010 – 2025 MHz (2570 – 2620 MHz) | ‘TDtv’ applications have been trialled in some EU countries |
| TD-SCDMA |  |  |
| WiMAX | 2 – 11 GHz 10 – 66 GHz | 3400 – 3600 MHz is being seen as a potential target following WRC-07 and EU WAPECS moves |
| 802.20 (MBWA) | Below 3.5 GHz |  |
| WiBro | 2.3 – 2.4 GHz | Currently Korea only |
| iBurst (HC-SDMA) | Below 3.5 GHz | Often uses 1785 – 1805 MHz or 1900 – 1920 MHz |

Table A3-1: Summary of spectrum requirements of new commercial data links

LTE and UMB are 4G extensions to UMTS and cdma2000 respectively. At present, it is envisaged that existing spectrum assignments will be re-farmed such that no new spectrum will be necessary.

From the above, the bands of particular interest, and where there are most likely to be mobile communication systems providing value in maritime applications are:

- 380 to 400 MHz

- 410 to 430 MHz

- 450 to 470 MHz

- 790 to 862 MHz (the ‘Digital Dividend’)

- 824 to 849 // 869 to 894 MHz (US ‘800 MHz’ cellular band)

- 880 to 915 // 925 to 960 MHz (EU ‘900 MHz’ cellular band)

- 1710 to 1785 // 1815 to 1880 MHz (EU ‘1800 MHz’ cellular band)

- 1850 to 1910 // 1930 to 1990 MHz (US ‘1900 MHz’ cellular band)

- 1880 to 1920 MHz (EU ‘3G TDD’ band)

- 1920 to 1980 // 2110 to 2170 MHz (EU ‘3G’ band)

- 2010 to 2015 MHz (EU ‘3G TDD’ band)

- 2400 to 2483.5 MHz (‘2.4 GHz’ licence exempt, low-power band)

- 2500 to 2690 MHz (‘3G expansion’ band)

- 3400 to 3600 MHz

Table A3-2 unten shows, based on the latest version of the ITU Radio Regulations, the allocations in those bands (including any footnotes), highlighting the position of maritime services in those bands.

| Frequency (MHz) | Region 1 Allocation | Region 2 Allocation | Region 3 Allocation | Status of Maritime Use |
| --- | --- | --- | --- | --- |
| 380 – 387 | FIXED MOBILE 5.254 |  |  | No constraints |
| 387 – 390 | FIXED MOBILE Mobile-satellite (space-to-Earth) 5.208A 5.208B 5.254 5.255 |  |  | No constraints |
| 390 – 399.9 | FIXED MOBILE 5.254 |  |  | No constraints |
| 410 – 420 | FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-space) 5.268 |  |  | No constraints |
| 420 – 430 | FIXED MOBILE except aeronautical mobile Radiolocation 5.269 5.270 5.271 |  |  | No constraints |
| 450 – 455 | FIXED MOBILE 5.286 AA 5.209 5.271 5.286 5.286A 5.286B 5.286C 5.286D 5.286 E |  |  | No constraints |
| 455 – 456 | FIXED MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | FIXED MOBILE 5.286AA MOBILE-SATELLITE (Earth-to-space) 5.286A 5.286B 5.286C 5.209 | FIXED MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | No constraints |
| 456 – 459 | FIXED MOBILE 5.286AA 5.271 5.287 5.288 |  |  | No constraints |
| 459 – 460 | FIXED MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | FIXED MOBILE 5.286AA MOBILE-SATELLITE (Earth-to-space) 5.286A 5.286B 5.286C 5.209 | FIXED MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | No constraints |
| 460 – 470 | FIXED MOBILE 5.286AA Meteorological-satellite (space-to-Earth) 5.287 5.288 5.289 5.290 |  |  | No constraints |
| 790 – 806 | FIXED BROADCASTING MOBILE except aeronautical mobile 5.316B 5.317A 5.312 5.314 5.315 5.316 5.316A 5.319 | BROADCASTING Fixed MOBILE 5.313B 5.317A 5.293 5.309 5.311A | FIXED MOBILE 5.313A 5.317A BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | No constraints |
| 806 – 862 | FIXED BROADCASTING MOBILE except aeronautical mobile 5.316B 5.317A 5.312 5.314 5.315 5.316 5.316A 5.319 | FIXED MOBILE 5.317A BROADCASTING 5.317 5.318 | FIXED MOBILE 5.313A 5.317A BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | No constraints |
| 862 – 890 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 5.319 5.323 | FIXED MOBILE 5.317A BROADCASTING 5.317 5.318 | FIXED MOBILE 5.313A 5.317A BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | No constraints |
| 890 – 902 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 Radiolocation 5.323 | FIXED MOBILE except aeronautical mobile 5.317A Radiolocation 5.318 5.325 | FIXED MOBILE 5.317A BROADCASTING Radiolocation 5.327 | No constraints |
| 902 – 928 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 Radiolocation 5.323 | FIXED Amateur Mobile except aeronautical mobile 5.325A Radiolocation 5.150 5.325 5.326 | FIXED MOBILE 5.317A BROADCASTING Radiolocation 5.327 | No constraints |
| 928 – 942 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 Radiolocation 5.323 | FIXED MOBILE except aeronautical mobile 5.317A Radiolocation 5.325 | FIXED MOBILE 5.317A BROADCASTING Radiolocation 5.327 | No constraints |
| 942 – 960 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 5.323 | FIXED MOBILE 5.317A | FIXED MOBILE 5.317A BROADCASTING 5.320 | No constraints |
| 1710 – 1930 | FIXED MOBILE 5.384A 5.388A 5.388B 5.149 5.341 5.385 5.386 5.387 5.388 |  |  | No constraints |
| 1930 – 1970 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE 5.388A 5.388B Mobile-satellite (Earth-to-space) 5.388 | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 1970 – 1980 | FIXED MOBILE 5.388A 5.388B 5.388 |  |  | No constraints |
| 1980 – 2010 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  | No constraints |
| 2010 – 2015 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.388 5.389C 5.389E | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 2110 – 2120 | FIXED MOBILE 5.388A 5.388B SPACE RESEARCH (deep space) (Earth-to-space) 5.388 |  |  | No constraints |
| 2120 – 2160 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE 5.388A 5.388B Mobile-satellite (space-to-Earth) 5.388 | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 2160 – 2170 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.388 5.389C 5.389E | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 2400 – 2450 | FIXED MOBILE 5.384A Amateur Radiolocation 5.150 5.282 5.395 | FIXED MOBILE 5.384A RADIOLOCATION Amateur 5.150 5.282 5.393 5.394 5.396 |  | No constraints |
| 2450 – 2483.5 | FIXED MOBILE Radiolocation 5.150 5.397 | FIXED MOBILE RADIOLOCATION 5.150 |  | No constraints |
| 2500 – 2520 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A 5.405 5.412 | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A 5.404 | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A MOBILE-SATELLITE (space-to-Earth) 5.351A 5.407 5.414 5.414A 5.404 5.415A | No constraints |
| 2520 – 2535 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.405 5.412 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.403 5.414A 5.415A | No constraints |
| 2535 – 2655 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.405 5.412 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417A 5.417B 5.417C 5.417D 5.418 5.418A 5.418B 5.418C | No constraints |
| 2655 – 2670 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.208B 5.413 5.416 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.412 | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.208B | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.208B 5.420 | No constraints |
| 2670 – 2690 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.412 | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.208B 5.415 MOBILE except aeronautical mobile 5.384A Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) 5.415 MOBILE except aeronautical mobile 5.384A MOBILE-SATELLITE (Earth-to-space) 5.351A 5.419 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 | No constraints |
| 3400 – 3500 | FIXED FIXED-SATELLITE (space-to-Earth) Mobile 5.430A Radiolocation 5.431 | FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile 5.431A Radiolocation 5.433 5.282 | FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile 5.432B Radiolocation 5.433 5.282 5.432 5.432A | Secondary |
| 3500 – 3600 | FIXED FIXED-SATELLITE (space-to-Earth) Mobile 5.430A Radiolocation 5.431 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile Radiolocation 5.433 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.433A Radiolocation 5.433 | Secondary in Region 1 |

Table A3-2: Summary of maritime radio spectrum

None of the footnotes relating to the use of these bands specifically preclude any use for maritime services.

With the exception of the band 3400 – 3600 MHz, the mobile service (which includes, unless explicitly stated, the maritime mobile service) has a primary allocation meaning that the use of these bands for maritime applications will pose no regulatory issues at the ITU.  In the band 3400 – 3600 MHz, many of the footnotes relate to the upgrading of the mobile service from secondary to primary status, however it is known that in some parts of the world (especially in high rainfall areas such as around the equator) this band is still heavily used for satellite downlinks and as such its development for mobile services is likely to remain constrained.

### Conclusions

There does not appear to be any necessity to expend significant effort on the standardisation or regulation of these mobile communication technologies to make them suitable for the maritime community.

Industrial, Scientific and Medical.  These bands are used by devices such as industrial heaters and driers and domestic equipments such as microwave ovens.

According to 3GPP TS 45.005