e-NAV7/8/15

Title 	Efficient, Safe and Sustainable Traffic at Sea

Acronym			EfficienSea

Contract No. 013

Document No. W_WP4_2a_7

Document Access: 	Restricted

Simulation report

Deliverable No.  ???

Date: 11.03.2009

Part-financed by the European Union (European Regional Development Fund and European Neighbourhood and Partnership Instrument)

DOCUMENT STATUS

Authors

| Name | Organisation |
| --- | --- |
| Margareta Lützhöft | Chalmars |
| Thomas Porathe | Chalmers |

Reviewing

| Name | Organisation | Signature | Date |
| --- | --- | --- | --- |
| Helle Heckmann | DaMSA |  |  |

Approval of report

| Name | Organisation | Signature | Date |
| --- | --- | --- | --- |
| Thomas Christensen | DaMSA |  |  |

Document History

| Revision | Date | Organisation | Initials | Revised pages | Short description of changes |
| --- | --- | --- | --- | --- | --- |

Report W_WP4_2a_7

Baseline scenario: The Sound Simulation

A normal day in the Sound. The traffic situation on February 21st  2009 at 5 PM. (AIS data from http://www.marinetraffic.com/ais  )The red and the green tracks where the tracks deduced from normal track patterns and used by participants in the simulation.

# Executive summary

This report describes the results of a simulation performed at Chalmers University as part of WP 4 in the EfficienSea project. The purpose of the simulation was to make observations of human behaviour and cooperation in a complex maritime environment so as to provide information on user needs for future e-Navigation solutions and concepts. The simulation can also be used as a test bed for new systems and solutions. Bridge simulators and adjacent offices were used to recreate the various shipping actors in the Sound (Öresund). The scenario was monitored by researchers and all communication and its direction, length and frequency was recorded in a system that allowed us to replay and analyse the scenario later. Photos and observations were also collected.

The participants were all professionals, and at sea they were the passenger ship Royal Princess, with a captain, a first officer and a helmsman, the tanker T C Gleisner with a captain and an officer and the coaster Clarity 2 with a captain. Land-based participants were:  The Sound VTS (Vessel Traffic Service) with an operator, the Drogden Pilot with a pilot, Lyngby Radio with an operator, Malmö-Copenhagen Port with a port official and the Joint Rescue and Communication Centre (JRCC) with an operator (telephone contact).

Important findings were:

- Alarms and other disturbances on the bridge are increasing and pose a significant risk.

- Training. With ever more complex bridge systems, training is increasingly important and should perhaps be made within the domain of “serious gaming” in order to maintain interest and stimulate learning.

- Planning can be supported with intelligent integration of information needs regarding for instance planning a voyage.

- Communication is a two-fold issue. There is too much communication which is seen as disturbing (VTS asking ships) but in reality serves a purpose (VTS checking status and awareness of ship crew), whereas at other times there is too little communication. Situations in which the latter issue arises is both in near-misses and collisions where ship officers are reluctant to communicate. There are also examples of calls to the wrong participants and ships not answering calls.

- Trust contains several issues but two are central, AIS and new technology in general.

  - AIS is distrusted by many, in particular those who must depend on it – VTS operators and ship’s officers. At the same time, more and more support systems are suggested based on AIS – which really must be carefully considered.

  - As technology becomes more difficult to “see through” and understand, design must take into account that if a system looks trustworthy, humans will trust it. This means that talking about officers having too low or too high trust in equipment may be due to the design emitting this capacity and not a flaw in human behaviour.  Equipment design must ensure that technical systems have redundancy and reliability.

- Appendix A contains summarized findings of a parallel study performed with the Swedish Maritime Administration for reference. The issue of shore-based, or remote, pilotage is discussed in regards to the aims of e-Navigation. Appendix B contains some guidelines for the port authority.

Design suggestions
During the simulation and in the briefings and analysis sessions several suggestions of design improvements were brought forward.

- The Ping Ship Graphical Confirmation Dialogue. See main document under Discussion: Too little communication.

- VHF Channel Display. Have ships send, with the AIS information, which VHF channels their radio sets are presently set to. See main document under Discussion: Ships not answering calls

- 3D naturalistic sound conning environment.  See main document under Discussion: Too much communication and Ships not aware of each other

- On bridge WebCam. See main document under Discussion: Ships not answering calls

- Integrate AIS and SafeSeaNet. See main document under Discussion: Trust.

Contents

Executive summary	5

Introduction	8

Objective	9

Method	11

Results/issues found	16

Discussion	20

Design suggestions	25

References	27

User needs for remote pilotage	28

User needs for the port authority	33

# Introduction

This simulation scenario was undertaken as part of a large EU project, EfficienSea, lasting 3 years with 17 participating organisations from 6 different countries around the Baltic Sea. One of the work packages in the project concerns e-Navigation, which is a new concept for designing IT systems used in navigation and communication. The purpose of the simulation was to make observations of human behaviour and cooperation in a complex maritime environment. These observations would then hopefully shed some light on interaction in larger social and technical networks. The goal was to analyse how people involved in the navigation of ships work and communicate today, in order to identify the optimal solutions for navigation tomorrow.

Another goal was to see if the simulation scenario could be used as a test bed for benchmarking of technical and organizational innovations in shipping. By creating a “normal day in the Sound” the scenario could be reused as a baseline condition for one type of nautical environment. The hypothesis is that it will be possible to make comparisons between simulations with and without certain equipment, communication methods or organizational patterns.

The study was performed during two days in March 2009 at Chalmers University of Technology in Gothenburg, Sweden. We used the bridge simulators and adjacent offices to recreate the various shipping actors in the Sound (Öresund). A role-playing scenario took place in which all participants played their own respective professional role (e.g. officers on ships, VTS operators and pilots). All work stations were manned with professionals. This allowed us to omit the use of manuscript and let the events develop as they would any ordinary day in the Sound. The scenario was monitored by a group of researchers from the simulator control room, and one observer was present on every work station (ship or shore workplace). The observers took notes and asked prompting questions about performed actions. A special recording system from VSL Systems, recorded all communication and its direction, length and frequency. This allowed us to replay and analyse the scenario later. Photo documentation was also made.

# Objective

A day in The Sound. This simulation had three objectives:

- To collect user needs for the ongoing work in WP 4 with a bridge mockup,

- To provide results to the larger context on e-Navigation, i.e., the EfficienSea project and ongoing work in IMO/IALA,

- To provide a simulation scenario and a results baseline which can be used, partly or wholly, to compare future suggested solutions (technical or organisational) emerging from the EfficienSea project.

This report mainly focuses on the first objective.

Figure 1. The traffic plan for the three ships during simulation. The blue dotted line is the passenger ship Royal Princess starting from the south, the red line is the tanker T.C. Gleisner starting from the north and the green line is the coaster Clarity 2 starting from the south. The afternoon times are passage times used during the first day’s “normal” simulation, the morning times are from the second day’s “busy” simulation.

# Method

Simulation was used as a method to collect user needs and to map communications and information flow among the participants of a 3 hour long scenario in the Sound between Sweden and Denmark (see Figure 1). The bridge simulator at Chalmers University of Technology in Göteborg was used with a three bridge setup.

Simulators have been used for a long time in the maritime and the aviation industry both for training and research. Human performance in simulators and in real environment has shown good correlations; see for instance Nählinder (2009) for a recent example.  For further examples of research on navigation made in simulators see Donderi, Mercer, Hong & Skinner (2004)and Nilsson, Lützhöft, et al. (2007). For a discussion on the validity of bridge simulators in training see for instance the Marine Board (1996).

The participants (see also Figure 2)

Sea based:

1. The passenger ship Royal Princess, with a captain, a first officer and a helmsman.

2. The tanker T C Gleisner with a captain and an officer.

3. The Coaster Clarity 2 with a captain.

Land based:

1. The Sound VTS (Vessel Traffic Service) with an operator

2. The Drogden Pilot with a pilot. (Shore based pilotage on day “busy” and as an onboard pilot on the passenger ship in the “normal day” simulation.)

3. Lyngby Radio with an operator.

4. Malmö-Copenhagen Port with a port official (took little part in the simulation apart from being interviewed, see appendix B for relevant findings).

5. The Joint Rescue and Communication Centre (JRCC) with an operator (telephone contact).

Passenger ship

Royal Princess

Tanker

T C Gleisner

Coaster

Clarity 2

The Sound VTS

The Sound Pilot

Lyngby Radio

Malmö-Copenhagen Port

JRCC Rescue Center

Other actors

(e.g. ships, coast guard)

Figure 2: the work stations in the simulation

The participants in the scenario could communicate using UHF radio sets. Four channels were available: 1 – 4, where channel 1 was designated as the general VHF watch channel 16, channel 2 was the pilot channel 13, channel 3 was the VTS working channel 71 and channel 4 was used as VHF working channel 69. Each work stations also had a mobile phone that could be used to make calls between participants, and access to a telephone number list.

All communication was recorded by VSL Systems AB from Linköping who set up a system listening to, recording and synchronising all radio and phone traffic together with notes from the observers placed at each participant/work station, photos taken during the simulation and screen dumps from the electronic chart, all was placed in a time stamped database for later monitoring, replay and analysis. The database was also used during the exercise to coordinate observer activities.

The scenario
The traffic ratio in the simulation was based on a normal day scenario. We had through the Danish Maritime Safety Administration gained access to a 24 hour AIS recording of the traffic in the Sound on 2009-01-19 which was used to model the background traffic in the simulation. See Figure 3.

Figure 3. Screendump from GateHouse’s software GAD where AIS data from 24 hrs in January 2009 were analyzed to determine typical tracks in the Sound. These were then used to model the traffic in the simulation and extract three typical voyage plans for the simulation participants along with the voyage patterns of surrounding traffic.

During two three-hour long scenarios the objective was to model an ordinary day in the Sound.

Narrative: Day 1, a “normal” day in the Sound
The simulation took place on March 19th 2009 at Chalmers full bridge simulator in Göteborg. As the simulation started at 12.40 the passenger ship Royal Princess was northbound entering the Sound south of Drogden lighthouse and the cargo ship Clarity 2 entered the Sound south of the bridge northbound through the Flint Channel. Both ships were called by the VTS operator and asked for destination and draught.

In the north the tanker T.C. Gleisner was approaching the Helsingborg – Helsingör ferry crossing southbound by the Drogden channel. At 13.43 an unplanned incident occurred. Because the simulation instructor was busy in the southern part of the Sound he did not notice that one of the ferries (which was a non-playing character on “auto pilot”) was to give way to T.C. Gleisner according to Colregs.  Of course the ferry did not give way by turning starboard and passing aft of the tanker. The tanker who under rule 17a should maintain course and speed did so until the last minute when she, as required under rule 17 b, did an evasive manoeuvre by doing a full 360 degree round turn. No attempt was made from the tanker to contact the ferry before the turn was ordered.

In the southern Sound Royal Princess called for a pilot at 12.45. The pilot boarded at 13.00 to take the ship through the narrow Drogden channel (see Figure 4). As the air draught of Royal Princess (55 meters) exceeded the 35 meters allowed for the passage of Kastrup airport a call was made to the VTS and further relayed to the airport which stops starts and landings on one of the runways during the passage.

Figure 4.At 13.03 the pilot is onboard the Royal Princess. From the left: the helmsman, the observer, the pilot and the captain. The first officer is seated outside the picture to the right. Photo from the bridge simulator.

Royal Princess now proceeded uneventfully northward with pilot onboard through the Drogden channel. After having passed the channel the pilot leaves Royal Princess at 13.55 and then the ship proceeds north in the Sound, passes east of the island Ven and crosses the Helsingborg – Helsingör ferry crossing at 10.14.

The coaster Clarity 2 has an uneventful voyage through the Flint channel and north through the Sound. Just before she enters the TSS (traffic separation scheme) Helsingborg – Helsingör she is passed by the Royal Princess. Clarity 2 adjusts her position to starboard to make sure Royal Princess will overtake her on her port side.

At 15.00 the pilot boards the southbound T.C. Gleisner to take her through the Drogden channel.

The simulation is suspended at about 15.40.

Narrative: Day 2, a “busy” day in the Sound
On the second day of the simulation, March 20th 2009, the scenario started at 9 o’clock. The scenario starts out the same as the day before: the same ships, the same crew and the same routes. At 9.07 Royal Princess called Sound pilot and asks for assistance through Drogden channel. Assistance was this time given with an approximation of remote pilotage with the pilot stationed in the VTS centre. For there the pilot guided Royal Princess over VHF channel 71. At 9.25, as the Royal Princess was passing through the channel, the captain received a telephone call from the shipowner who needed to have the company inspector, who was on board the Royal Princess, put ashore. Arrangements were made to have the pilot boat pick him up as the ship passed Copenhagen. The pilot redirected Royal Princess through the inner channel closer to Copenhagen where the pilot boat picked up the inspector at about 10.20. After that Royal Princess returned to her planned route north and at 10.28 the conn was returned to the captain from the land based pilot.

From the north the tanker TC Gleisner has this time uneventfully passed the Helsingborg – Helsingör ferry crossing and approaches Drogden. At 10.26 the bridge received a telephone call from the engine room requesting a spare part to be sent out from Copenhagen. The captain arranged with the Sound pilot to have a boat sent out. Meanwhile TC Gleisner will deviate from the fairway and proceed through the anchorage at minimum speed. Soon after the problems with her main engine got worse and she had to require a berthing place in Copenhagen for repair. While waiting TC Gleisner dropped anchor at 10.55. Soon afterwards the port confirmed a berthing place on condition that the tanker was not carrying class 1 or 2 oil cargo. TC Gleisner then received shore based pilot to berth and heaved anchor at 11.05 and proceeded to port. But at 11.30 the engine room has evidently fixed the problem and TC Gleisner resumed her voyage for Riga with shore based pilot through Drogden. Royal Princess passed Helsingborg – Helsingör uneventfully at 12.00 and shortly after the simulation was suspended.

Debriefings

On both days an individual debriefing was held by each observer with their respective work station. After this a joint debriefing was held on day one. During this debriefing the key words from IALA:s e-navigation concept (collect, integrate, exchange, present and analyse) were used to focus the discussion. On day two, individual debriefings were held, followed by a grouped debriefing in which “shore stations” sat together and “ship stations” sat together. This allowed for more ship/shore related feedback, after which the two groups joined in a final debriefing and general discussion session.

# Results/issues found

This section reports our findings from this and related studies. It is followed by a discussion section in which the issues are treated in more depth and suggestions for solutions given.

For the analysis of the recorded data VSL from Linköping provided a software analysis platform, CITE Explorer and CITE Observer where all radio and telephone communication as well as the observations from the observers and photo documentation was laid out on a time line (see screen dump in Figure 5).

Figure 5. Screen dump from the VSL tool CITE Explorer showing the timeline of the first day’s simulation. Entries from the observers, link map for VHF communications and a photo from the tanker’s bridge.

The CITE Explorer was very helpful in the analysis process. Below follow a selection of results.

#### Alarms

Bridge alarms were turned off during this simulation (e.g., all alarms not relating to bridge instruments). One would think that this might pass unnoticed but several participants commented on the absence of alarms as being a relieving experience. These are yet more testimonies for the long-awaited alarms management system.

#### Training

An observation was that although one and a half hours of training on the bridge equipment in the simulators (Transas 4000) was provided, the participating professionals had problems handling the ECDIS and the radar (buttons, menus, how to handle the software). This observation correlates well with comments made in several places in The Nautical Institute’s recent guide book From paper chart to ECDIS: a practical voyage plan (Gale, 2009): that there is a need for training in using bridge software (in this case ECDIS) and that such training generally is lacking. This was also a comment in the debriefing: often a new officer on his/her first watch onboard a new ship is left alone to learn to handle an unknown brand of ECDIS or radar.

#### Planning

Information collection
One of the observers noted that the crew of the Royal Princess had difficulties of finding information for the route planning. For instance there were difficulties of finding things in the Admiralty List of Radio Signals, ALRS 6(2): VTS reporting points were found under “Ship reporting systems” and not under the expected headings “Sweden” or “Denmark”, further it was difficult to find the maximum air draught for a ship passing Kastrup airport.

An observer also reported difficulties of finding information about the VTS reporting points in the ECDIS, as well as difficulties finding the depth in the channel (Drogden). When the officer did not find it in the chart, he looked in the Pilot book instead.

#### Communication

Too much communication
During at least one occasion a player was observed to turn down the volume of the radio due to large amounts of irrelevant communication on the two radio channels listened to.

One of the observers noted that the VTS called up a ship to ask if it had pilot onboard. The observer commented that there should be an easier or automatic way of showing whether or not there was a pilot onboard (e.g. by a tag on the AIS symbol).

A lot of unnecessary communication took place where the VTS asked about things that was already displayed among the AIS information (e.g. destination and draught). See also comments under “trust” on this issue.

Too little communication
On one occasion as the tanker passed Helsingborg-Helsingör one of the ferries showed no sign of giving way although required to do so by the Colregs. In the last moment the tanker made a 360 degree turn to avoid a collision. No attempt was made from the tanker to contact the ferry.

In Managing Collision Avoidance at Sea the Nautical Institute presents results from an investigation studying 100 collisions reports from maritime authorities in UK, USA, Australia, New Zeeland and Sweden. In their findings, communication problems ranked high in the list of unsafe acts. “The most frequently made mistakes were lack of communication and misinterpreting received information.” (Lee and Parker, 2007, p. 2). The major underlying issue found was the reluctance of navigators to exchange information.

Ships not answering calls 
During the simulation it happened several times that ships did not answer calls. An example was in a meeting situation between TC Gleisner and Royal Princess on day 2 when Royal Princess tried to call T.C. Gleisner on channels 16 and 71 while T.C Gleisner was busy on the phone and talking to the pilots on channel 13.

This was sometimes due to that the ships involved in the simulation did not have radio sets with dual watch while they were attending other VHF channels. But comments from the seafarers and VTS operators involved in the simulation stated that this was not an uncommon situation in real life either.

Calls to the wrong participant
As an example of this, one ship leaving anchorage outside Copenhagen day 2 was supposed to call the VTS but instead calls Copenhagen Port on the VTS channel. Copenhagen port was busy in a call with another ship which in turn did not answer. One could call this a frustrating situation of miscommunication that was lasting for about 30 minutes.

Communication issues

In the second day simulation (busy), we used a form of remote pilotage. We found that during the remote pilotage there were issues with communication. Other stations could not get through to the ship while this was going on, probably due to the bridge crew being very concentrated on the task and not hearing other calls.

Many ships have “impossible names” for some reason. It could be business (naming your ship after the main customers, for example Stora Korsnäs Link 1, in all countries but Sweden she was called SK Link 1), tradition in a shipping company (long family names) or difficult to know how to pronounce (for example Asian names to westerners, or western names to Asians).

Another example is the incident with the Swedish coaster Sommen and the French Naval ship Somme arriving to the same German port at the same time. The mishearing of names was expected but could have resulted in a “diplomatic” incident as the port almost let the small coaster in before the French Naval ship.

#### Trust

During the simulation the VTS operator called each ship approaching the Sound asking for draught and destination. He reported that this was practice by the Sound VTS. Draught and destination is information transmitted by the AIS, however this information is often wrong or missing. A few minutes of search on the live AIS sites on the web reveal several ships lacking information among them the mysterious ship north of Bornholm in Figure 6. It is worth noting that it is a legal requirement for the data to be correct and it should be clear to a VTS what they should do if it is not.  At present, for example, Dover Strait Coastguard issues a warning.

Figure 6. This screen shot from LiveAIS shows just one example of a ship with missing information. The. MMSI number belongs to the tanker Pacific Innovator, 183 m long and built 2008 (revealed by a quick search on the internet).

But even if the AIS reported draught and destination this information would often be wrong, reported the VTS operator.  Furthermore, the way and at what speed a call from the VTS was answered would say something of the status of the ship. It was a question of “trust” according to the VTS operator, something that was also confirmed by VTS experienced personnel in the analysis group.

Cruise ship captain: “In real life I just use ARPA – I do not trust AIS information”.

The observer on the cruise ship reported “None of the navigating officers trust AIS data for other vessels. Interesting - a system that has high priority and has been pushed through”.

In a study in a VTS centre in Holland the operators used the radar picture for traffic organization. On a question why they did not use AIS information they answered that the AIS information was not reliable enough (EfficienSea WP 6 deliverable).

Usability issues
Usability (earlier called user friendliness) issues were of course present in the simulation because of the short time the bridge crews had to get acquainted with the instrumentation on the bridges. However, this reflects the real situation for an officer working on the bridge for the first time, often left to find out for him or herself how the particular brand of equipment works.

A particular difficulty that did not seem to be understood intuitively was the acquire target function on the Transas 4000 radar on the two bridges. This function is often used for anti-collision purposes and should be easy to use even for the first time.

The crew on the cruise ship commented on the screen resolution on the ECDIS where they had difficulties to see the AIS information (too small letters – too high resolution on the display).

Too much information – too many vectors on the radar display. The cruise ship crew reported having difficulties determining the heading of other ships due to too many vectors.

A solution to some of the usability issues is the S-mode function, a standard mode that some (radar, ECDIS) or all bridge equipment can be reset to by pressing an S-mode button. Work is ongoing in cooperation with the Nautical Institute. S-Mode is envisioned to be far more than a default display, it should also encompass a standardised menu system and interface device. The problem of acquiring targets on the Transas 4000 radar, might have been solved by a standard function within S-Mode. Future work includes the identification and testing of which key tasks such as target acquisition should be included in S-mode.

# Discussion

This section provides a discussion of the results above, as well as suggested solutions or ways forward.

#### Training

There is a constantly changing plethora of software used on the bridge. Training in the maritime academies can only give general knowledge on the use of e.g. radars and ECDIS’s. The specifics of an individual brand have to be learned onboard. Testimonies of the need for training have been given above. There is now a need for improved methods of training. Considering that the seafarers spend long hours onboard, the area of edutainment and serious games should be explored. For instance, games involving ship navigation where the player has to use a radar or and ECDIS of the same brand as the one on the ship’s bridge. A suggestion would be that the manufacturers of bridge equipment also had a serious game web site where mariners could log in and play and train at the same time. These games need to be developed by game designers in cooperation with seafarers and human factors experts to make sure they really attract the target group.

#### Communication

Too much communication
High frequency of communications not relevant to a particular ship may be a distraction in situations of high workload and may lead to, as observed, that the bridge crew turn down or even turn off their VHF sets. This, in turn can lead to a situation where other ships or shore cannot reach the ship. And also that important information, relevant to the particular ship is not heard. This might have been the case in the Pomerania – Rio Grande collision 2005, commented on below.

Using the bridge as a “switch board” for all incoming calls to the ship is another problem also encountered in the simulation where the captain of the cruise ship received a call from the shipping company during piloting and asked to have the call transferred to the information desk. Not all ships have “information desks.” This leads to unnecessary extra work and distraction on the bridge and could potentially compromise safety.

In the discussion with the analysis group suggestions of some kind of chat system was raised. On a graphical interface (chart, radar?) connections between ships would be visible, maybe as connecting lines, and short standard communications could be typed and saved as a threaded history, open and closable to avoid cluttering. Concerns were raised about hiding the chart under dozens of chat boxes.

Another solution discussed by the analysis group was “natural communication”. Communication should not be hidden in separate VHF channels, but instead all could talk in an open communication space, like in a large room. The voices talking should be presented on the bridge from the correct bearing, and with a low volume corresponding to the distance, i.e. 3D sound. If you would want to call another ship you would raise your voice or “ping” it by clicking its AIS symbol. Once in contact with a ship you could choose to enter into a virtual “call booth” where you would have a close person-to-person communication while still being able to hear other ships in the background. Outside a certain area calls would not be heard unless a directed call was made to the ship. If a faint engine sound, individual for each ship in the close vicinity, maybe based on the ships speed, and heard from the right bearing, was added in this natural communication environment, situation awareness might be augmented.

Addressing the problem of using the bridge as a switchboard for incoming calls, a suggestion made by the analysis group was to route all incoming calls to the ship to a land-based administrator in the ship-owners office. This person could have access to all ship and cargo data and answer questions from pilots, ports etc. and thus off-load some of the administrative work from the ship.

Too little communication
During the tanker-ferry incident on day 1 neither of the ships tried to contact each other on radio. In this case the ferry was “unmanned” (the simulation instructor was busy elsewhere) but the right-of-way tanker made no attempt of communication. The tanker otherwise behaved completely in accordance with the Colregs. However, one might wonder why there was no attempt made to radio the ferry. A comment from the observer group was about “a culture of silence” at sea: “a well run ship is a silent ship.” A comparison can be made to the different car signalling cultures in different cities of the world.

During the debriefing after the first day a suggestion was made regarding some kind of possibility to “ping” another ship, just to make sure he was alert and on stand-by without actually having to make a voice call. (“Ping” is a method of finding out if another computer is on-line and listening.) One of the observers made a comparison with road or railroad bridges using a flashing white light to signal that it had observed the approaching ship and was going to open the bridge eventually.

One of the original objectives of AIS was to reduce the amount of communication. Today we know that the navigationally relevant amount of communication has lessened but not the “rubbish communication”. The risk of this is that, before AIS, ships would communicate their intentions to other ships in a way so that others listening in could deduce who they were, and where they were (e.g., “ship passing E8 buoy”). This allowed for a “public awareness” or “joint situation awareness” which is partly lost now with less, and directed, communication which will be hard to decipher for those with no AIS – and thereby not having access to ship names (final year project by Olle Blomberg, Linkoping University). The problem is exacerbated by use of mobile phones instead of VHF, again reducing the information available to all (e.g., pilots to tugs, other pilots and VTS).

Ships not answering calls
Communication problems due to ships not answering calls on the VHF were experienced several times during the simulation. Sometimes this was due to the radio system used in the simulation which did not have dual watch although many participants had two radio sets to be able to watch two channels simultaneously. But statements from the participants involved shows that this is often the situation in real life. Often this problem is due to that ships are listening and calling on different channels. For instance during day 2 the passenger ship had difficulties contacting the tanker and had to ask the VTS for help to locate on what channels she was listening to. A design suggestion here is to have the AIS system send out the channels that the bridge VHF(s) are set to.

Furthermore, the remote pilotage situation seems to make the bridge crew very focused on this situation, and seemingly less attention is available for picking up other calls. At present we have no explicit solution, except careful testing for unwanted side effects when implementing any change to the work situation, be it new technology, procedures or training.

Sometimes calls are not answered for other reasons than the VHF being set to another channel or high workload. This could be for instance due to volume being turned down, interfering noise or sound/music on the radio or on the bridge, lack of attention or even that the bridge is unmanned or the OOW is asleep or drunk. During the weekend following the two simulation days, on Sunday 22 March 2009, a Bermuda registered container ship manned with Russian crew, Karin Schepers, grounded on Amager in The Sound. The Danish navy had for one hour tried to call the ship on VHF without success after having noticed that she was off course. A navy helicopter sent out to warn the ship observed that no one was on the bridge. After the ship had grounded the boarding pilots found the drunken officer sleeping on the bridge couch. The captain was likewise drunk. Situations similar to this frequently appear.

Another case is the collision between the Polish Ropax ferry Pomerania and the Swedish tanker Rio Grande 31 of January 2005 (Sjöfartsinspektionen, 2006). The right of way tanker several times called the ferry on the VHF but got no answer. On the ferry bridge the lookout tried to make the OOW aware of the approaching tanker but the OOW took no action.

Could a video picture from a web-cam on the bridge be transmitted to ships in the vicinity? That would definitely be an intrusion on integrity of the bridge crew but would give clear evidence to whether or not the bridge was manned in cases where no one would answer calls on the VHF.

Ships not aware of each other
In the July 2007 issue of the Nautical Institute’s Seaways Harry Gale and David Patraiko presented findings from an investigation of causes of collisions and grounding from the last 10 years. In 23 percent of the collisions the reason was poor lookout and in a surprising 13 percent of the collisions the two ships were completely unaware of the other vessel until (or just before) they collided.  The “naturalistic sound environment” suggested above might be a solution to that ships in the absolute vicinity are not aware of each other. Something to consider is what will happen in Singapore strait or other crowded places where the traffic intensity is immense. Maybe the “information horizon” then needs to be closer.

#### Trust

It was obvious to us that AIS information was not trusted. The reason seemed simple enough: the AIS information was often wrong or missing. At the same time there is another system that partly contains the same information, the SafeSeaNet (SSN). SSN is a distributed network that links the maritime administrations of the EU Member States (including Norway) in accordance with the EU directive 2002/59/EG with the purpose of increasing the efficiency of the maritime supervision. Information in the SSN is entered through the web and stored in local databases in each member state. This information can be accessed by VTS centers, pilots, ports and other authorities. Such information as destination, ETA and ETD, draught, people onboard is in this system. These were some of the most frequently asked questions by the VTS operator during the simulations. It seems obvious that there should be an automatic function so that information is taken from the SSN to the AIS. The aim should be that no information should need to be entered more than once, and by an identified official that is closest to the source of the information. We recognize that how to guarantee the authenticity of the information is an area that needs to be carefully considered.

The information should also include technical ship facts allowing shore based rescue services to make computations and recommendations on for instance stability and ballasting.

Earlier studies show that the “better the picture”, the higher the trust. This means that a system that looks and feels trustworthy will be trusted even if the underlying data is not valid. An example is ECDIS with underlying questionable depth data. This has two design implications: make sure your system is trustworthy if it looks it, and make sure that unreliable data is clearly indicated on a system.

One important question of trust concerns the interaction between an operator and the system. This interaction is of major importance if advantages are to be gained between the operator and technical system. Extensive research on the subject had been made in both the aircraft and shipping industry. In the literature eleven parameters have been identified affecting operators trust towards new technical systems. Of these parameters four have been identified as more important regarding trust to a system than the others. These four are predictability, usefulness, robustness and responsibility. One recommendation could be that any prototype system should evaluate these four factors in particular, as they seem to have a more obvious effect on users’ trust in a new system. However, this is only valid for one system, if there are two or more systems monitored the complexity increases drastically. There literature is limited considering “system trust” when systems of systems are developed, or new systems are integrated with old systems.

Usability issues
A solution to some of the usability issues is the S-mode function, a standard mode that some (radar, ECDIS) or all bridge equipment can be reset to by pressing an S-mode button. Work is ongoing in cooperation with the Nautical Institute. However it does not help for issues like the problem of acquiring targets on the Transas 4000 radar. Here the only remedy would be a standardization of the interaction process. This might be considered to be a product differentiator by manufacturers.

Exchange of intentions
Several radio calls during the simulation were made to ask for intentions. One of the observers commented that “most radio communications is about asking for intentions”. How can intentions be communicated to other ships? By, for example, relaying waypoints ahead? And how can this intended track be trusted if the ship is allowed to deviate from it, which it must in for instance collision situations? Furthermore, we mus t consider the lifespan of an intention. How long does the officer have to stand by his intention (perhaps transmitted to other ships by some means) and how will changes be transmitted, due to anti-collision manoeuvres or similar? It is easy to predict many intentions and many changes, making normal situations very hard to overview and assess at any one point in time due to the dynamics of intentions.

Method discussion

This sections contains a brief discussion on method issues that may have influenced the results.

The fact that we had no real VHF radio sets but instead communicated with a number of handheld UHF stations with only 4 channels and no dual watch had some implications to the simulation although some participants  were equipped with two UHF sets to simulate a realistic opportunity to watch more than one channel at a time. VHF channels 16, 71 (Sound VTS), 13 (pilot) and 69 (working channel) was mapped on UHF channels 1 – 4 respectively.

One of the participants, the coaster ship was placed in the simulation control room. This had some implications to the simulation because she could hear all communications that went on because the communication hub was at the instructors desk. Also, through technical limitations she was not able to con her ship in a normal fashion, instead her ship was locked in track mode following the voyage plan and when she needed to tweak or deviate from the track she had to ask the instructor for help. This resulted in a somewhat unrealistic working situation that affected the realism.

Some participants did not have access to all the tools or instruments they normally would have had but this has been taken into account in the analysis. Furthermore, it is the intention to keep gathering data on the various participants to make the picture more and more complete.

Design suggestions
During the simulation and in the briefings and analysis several suggestions of design improvements were brought forward.

- The Ping Ship Graphical Confirmation Dialogue. See above under Discussion: Too little communication.

- VHF Channel Display. Have ships send with the AIS information which VHF channels their radio sets are presently set to. See above under Discussion: Ships not answering calls

- 3D naturalistic sound conning environment.  See above under Discussion: Too much communication and Ships not aware of each other

- On bridge WebCam. See above under Discussion: Ships not answering calls

- Integrate AIS and SafeSeaNet. See above under Discussion: Trust.

# References

Donderi, D.C., Mercer, R., Hong, M.B. & Skinner, D. (2004). Simulated navigation performance with Marine Electronic Chart and Information Display Systems (ECDIS). The Journal of navigation. London: The Royal Institute of Navigation.  57, 189-202.

Gale, H. (2009). From paper chart to ECDIS: a practical voyage plan. London: The Nautical Institute.

Lee, G.W.U. & Parker, C.J. (2007). Managing Collision Avoidance at Sea. London: The Nautical Institute.

Marine Board. (1996). Simulated Voyages: Using Simulation Technology to Train and License Mariners. Washington D.C.: Commission on Engineering and Technical Systems

Nilsson, R., M. Lützhöft, et al. (2007). Fairway Navigation – Observing Safety-Related Performance in a Bridge Simulator. 7th International Navigational Symposium on Marine Navigation and Safety of Sea Transportation: Trans-Nav 2007, Gdynia, Poland.

Nählinder, S. (2009). Flight Simulator Training: Assessing the potential. Linköping Studies in Science and technology, Dissertation No. 1250. Linköping: Linköping University.

Swedish Maritime Safety Inspectorate. (2005). The Ropax Ferry POMERANIA - C6RA9 – in Collision with the Tanker RIO GRANDE - SMPA – 31 January 2005. In Sjöfartsverkets rapportserie B 2006-5. Norrköping: Swedish Maritime Safety Inspectorate.

Appendix A.

# User needs for remote pilotage

Remote pilotage is an act of pilotage (as defined by the European Maritime Pilot’s Association):

[…] carried out in a designated area by a Pilot licensed for that area from a position other than on board the vessel concerned to conduct the safe navigation of that vessel. (Hadley 1999)

This is a service that to some extent exists in several European ports today. There are also political pressures to expand and spread the concept (Bjelfvenstam 2007), largely based on the perception that new technologies (e.g. AIS (Robinson 1993)) will make extensive remote pilotage possible, with expectations of increased efficiency and economic gains. This perception is however at best a considerable simplification and at worst completely unfounded. There are fundamental problems with remote pilotage that are not technology-dependent (Bruno and Lützhöft, 2009). This means, essentially, that regardless of any realistically conceivable technological developments, it will not be possible to replace an on-board pilot with a shore-based pilot while maintaining the same level of safety. In certain contexts, these problems can likely be mitigated; in most, they cannot, at least not to a sufficient degree.

In more concrete terms, remote pilotage will only be feasible for a segment of ships in easily navigable fairways. Remote pilotage also depends heavily on the establishment of trust between the pilot and the crew onboard the ship. This has implications for user needs that are perhaps not always considered. In the following, this will be discussed and some high-level user needs will be presented. The discussion will relate to the collection, integration, exchange, presentation and analysis of marine information, as this is the foundation of IMO’s strategy for e-Navigation. The definitions of these five key areas follow IALA’s e-Navigation User Needs Capture Methodology Template (International Maritime Organization 2008).

The discussion is based on study visits, observations and interviews carried out at four different VTS centers, with shore-based pilots, VTS operators and administrative staff as informants.

User needs for collection

e-Navigation should allow the collection of all appropriate information needed to support the task of remote pilotage by all available means

The crucial points here are that the information collected should be “appropriate” and “needed to support the task”. For information needs, the standard information that is normally present in today’s VTS centers (i.e. a radar image, AIS information, various databases, cameras) should suffice. More detailed information (e.g. AIS-data about rate of turn, rudder position, heading) is not perceived as necessary and indeed has been shown to lead to information overload and less efficient work in a remote pilotage setting (Beimers 2008). Collection of such information is thus inappropriate, and therefore contrary to the principles of e-Navigation.

With regards to technology, there has to be a high reliability in the data collection, as remote pilotage is a safety-critical service and the consequences of a total loss of data could be severe.

The user needs for collection can be summarized as:

- Collect “standard” VTS information, including radar, AIS, CCTV camera pictures and databases.

- Maintain high degrees of reliability and redundancy in the collection process to ensure that sufficient information for the provision of remote pilotage always is available.

User needs for integration

e-Navigation should integrate all appropriate data and information needed to support remote pilotage

The integration user needs relates to the integration of the collected information into an actual system. This means that, apart from the collected data, there is also a need to take the status of the integration process into account. To be appropriate, the integration must be performed in a manner that is adapted to the needs of the user.

Field studies at VTS centers indicate that the integration processes are too opaque today. VTS operators find it hard to understand the sources of the information they are presented with, and in many systems it is not immediately obvious if e.g. a radar failure should occur, because the various data sources are too integrated. Further work with e-Navigation should thus, as far as integration is concerned, focus on making the integration processes more transparent. Not so transparent as to interfere with the normal work of the operator, but at least transparent enough for the operator to be fully aware of what information comes from which data source.

As with collection, high reliability is also needed, and data that is invalid should be filtered out and the operator informed of the problem. This is a problem today, when operators e.g. have seen too much incorrect AIS information in the system and as a consequence has stopped trusting the AIS at all.

The user needs for integration can be summarized as:

- The collected data should be integrated into a system for presentation.

- The integration process should be made transparent enough to enable operator awareness of how the presented information is related to the various data sources.

- The integration must have a high degree of reliability, and invalid data should be filtered out.

User needs for exchange

e-Navigation should allow for the exchange of any data or information needed to support remote pilotage

Data exchange can be considered to be the automatic or manual dissemination of data and information throughout the bigger system consisting of ships and shore stations. This includes ship-ship, ship-shore and shore-shore data/information exchange.

This is a huge area of e-Navigation that encompasses everything from normal VHF communication ship-to-ship (e.g. asking for intentions) to highly automated data transfers. The user needs are thus diverse, but most are not specific for remote pilotage. For example, access to and data transfers between databases are important for the system as a whole, including VTS operators and harbor managers. Furthermore, many important technical needs (i.e. standardization of data formats to allow for seamless exchange) are beyond the competence of this author and will not be discussed.

The primary user need for exchange in a remote pilotage context is exchange of the actual remote pilotage instructions. This can presumably be accomplished either through VHF communication or a data link system. Other user needs involve the transmission and confirmation of “background” information such as passage plans, if that is a part of the remote pilotage procedure. It is also important to consider how the exchange takes place. For example, earlier studies in this and other domains has shown that verbal communication can be very important for the establishment of trust between two people who are not co-located. It is thus likely that VHF communication should remain the primary mode of exchange when it comes to instructions. Other necessary exchanges could possibly be accomplished in a more automated manner. In any case, the format for exchanges should be carefully considered and adapted to the relevant context.

The user needs for exchange can be summarized as:

- There should be a robust method (technically as well as procedurally) for exchange of remote pilotage instructions, based on VHF communication.

- All other necessary data exchanges should be adapted to the actual work situation of the operator and should not distract from the primary task.

User needs for presentation

e-Navigation should facilitate the clear presentation of all information pertaining to remote pilotage in a manner that supports the decision making process, engages the user and minimizes any risk of distraction or over burden. It should also provide easy to use facilities for a user to interact with the system and input data

The information pertaining to remote pilotage should be presented in a clear and engaging manner that supports the decision making process and minimizes the risk of distraction or over burden. The clear presentation pertains to showing only the relevant geographical area, minimizing clutter etc. The display should preferably also be configurable according to individual operator preference. The decision making process can be supported with means such as presenting the configuration of the sea space (the division into e.g. different sectors manned by different operators) but for the most part, the decision making processes of shore-based pilots and VTS operators seem to depend more on experience and recognition than on any particular attribute of the information presentation.

Making the presentation engaging relates to distraction and over burden. The operator need to be engaged so as to not be distracted, but not so engaged as to be too overloaded to focus on the task at hand. These are however primarily organizational factors that involve e.g. shift length, general work procedure, workload etc. Trying to solve them through technology design is not the right way.

The system should also have a high degree of usability for interaction and input tasks. This can be achieved by designing the system with the help of a user-centered design methodology. This is often not the case with VTS systems today; operators often complain that their systems are hard to use and that information is hidden away in menus or dialog boxes that require many sequential steps to access.

The user needs for presentation can be summarized as:

- The information display should show the relevant geographical area with a minimum of clutter.

- The display should preferably be configurable by the operator.

- It does not seem particularly relevant to support decision making in this domain by particular way of presenting information. Decision making is facilitated by experience and recognition, things that are not directly related to how the information is presented.

- It also does not seem relevant to try and manage operator engagement, distraction or over burden through the information presentation. Those issues are better handled using non-technical methods.

- The system should be made usable for its tasks through user-centered design (this also includes physical ergonomic aspects). Important information should be easily accessed.

User needs for analysis

e-Navigation should support the user through the appropriate analysis of data and information to support remote pilotage

Supporting the user through appropriate analysis of data and information likely refers to processing data in various ways to help with e.g. decision making. As a category it is hard to distinguish from both integration and presentation, and there are few user needs here that go beyond what has been presented above. It was for example observed that the shore-based pilots in the Netherlands mainly work with raw radar data that has not been subject to any particular analysis, and neither them nor other interviewed VTS operators could perceive a need for more analysis. As it is generally important that systems that support e.g. decision making are adapted to the user’s current decision making processes, it cannot be recommended that e-Navigation introduces new extensive analysis methods in this domain.

The user needs for analysis can be summarized as:

- There is no perceived need for extensive data analysis in this domain, beyond what is done by the operators themselves.

- Any systems/algorithms that do analyze remote pilotage data should be carefully designed to support, in a non-intrusive manner, the existing decision making processes of the operator.

References

Beimers, P. B. (2008). Onderzoek optimaliseren loodsen op afstand - gebruik van AIS informatie (in Dutch). Wageningen, MARIN.

Bjelfvenstam, J. (2007). Lotsa rätt! Delbetänkande av Lotsutredningen (in Swedish). Stockholm, Statens offentliga utredningar. SOU 2007:106.

Bruno, K. and Lützhöft, M. (2009). "Shore-based pilotage: Pilot or autopilot? Piloting as a control problem." Journal of Navigation, 62 (3): 427-437.

Hadley, M. (1999). "Issues in Remote Pilotage." Journal of Navigation 52(1): 1-10.

International Maritime Organization (2008). Development of an e-Navigation strategy: Report from the e-Navigation correspondence group. Sub-committee on safety of navigation, 54th session.

Robinson, A. (1993). The Impact of New Technology on Marine Pilotage - An End to Pilotage? International Maritime Conference: The Impact of New Technology on the Marine Industries. Warsash, Southampton Institute Maritime Division.

Appendix B.

# User needs for the port authority

While the simulation was going on, further data was collected by way of an interview with one of the participants, namely the port manager of Copenhagen-Malmö Port (CMP). This person did not play a large role in the simulation, so there was plenty of time for the observer to perform the interview. The topic of the interview was the operation of the port. This was later analyzed from an e-Navigation perspective, to identify possible user needs for port authorities.

The analysis indicates that the main concern of the port authority (as represented by this particular person) relates to the e-Navigation concepts of collection and exchange, i.e. the collection of data, and the automatic or manual dissemination of data and information throughout the larger maritime system. Many of the comments related either to the direct communication with the ships or to data dissemination in networks, e.g. the SafeSeaNet. The primary needs were perceived as a more efficient way of distributing both operational and administrative information between e.g. ports, maritime authorities, agents, shipping companies etc., and a safe and efficient way to perform the actual VHF communication with the ships.

The other three focus areas of e-Navigation (integration, presentation and analysis) did not figure prominently in the interview (although it can be argued that there is some overlap between exchange/collection and the other areas – this is due to the somewhat fuzzy nature of the concept definitions). It seems likely that at least the presentation and analysis areas are more relevant for operative personnel, e.g., VTS operators or ship officers. Integration could be important for port authorities but, as noted, overlap to some extent with collection and possibly exchange.