| From: VTS30 |  |
| --- | --- |
| To: e-NAV7 | 18 September 2009 |

Liaison Note

e-Navigation Shorebased User Needs from a VTS perspective

# Introduction

IALA represents aids to navigation and VTS authorities that are responsible for the provision of services to the mariner that will be supported by e-navigation.  IALA has agreed to provide to IMO’s e-Navigation Correspondence group a compilation of user needs from the shorebased user perspective.

# Background

Many of the shorebased user requirements of e-navigation will come from vessel traffic services.  The following definitions of VTS services (Annex A) and shorebased sensors and communications equipment (Annex B) identify some of those requirements.

# Action requested

The e-NAV Committee is requested to consider VTS Committee’s views on the e-Navigation shorebased user requirements from a VTS perspective.

- VTS Services

A VTS is capable of offering one or more of the following service types.

- Information Service (INS)

  - Maintain a traffic image and interact with traffic and respond to developing traffic situations.

  - Assist the on-board decision-making process by providing:

    - Position, identity, intention and destination of vessels;

    - Amendments and changes in the VTS area such as boundaries, procedures, radio frequencies, reporting points;

    - Mandatory reporting of movements;

    - Meteorological and hydrological conditions, notices to mariners, status of aids to navigation; limited manoeuvrability that may impose restrictions on the navigation of other vessels, or any other potential hindrances.

- Traffic Organisation Service (TOS)

  - Provide essential and timely information to assist the onboard decision-making process.

  - May involve the provision of information, advice and instructions.

  - Plan movements to maintain vessel safety and to achieve efficiency.

  - Service may include:

    - Position, identity, intention and destination of vessels;

    - Amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points;

    - Mandatory reporting of movements;

    - Meteorological and hydrological conditions, notices to mariners, status of aids to navigation;

    - Traffic congestion and special vessels with limited manoeuvrability which may impose restrictions on the navigation of other vessels or any other potential hindrances;

    - Allocation of water space;

    - Establish and operate a system of traffic clearances - all or certain classes of vessels may be required to participate in this service and shall not proceed without clearance;

    - Establish routes to be followed and speed limits to be observed and other measures ;

    - Traffic congestion and advice about vessels within the VTS area;

    - Co-ordinating VTS sailing/route plans, including intended route, ETA in VTS area, departure from berth or anchorage. May include general flow of traffic, efficiency and co-ordination with allied services.

- Navigational Assistance Service (NAS)

  - Provide navigational information, advice, or instruction to assist in the on-board navigational decision-making process.

  - Provided at the request of a vessel or when a navigational situation is observed and intervention by VTS is deemed necessary. Requires positive identification and continuous communication throughout the process.

  - Agreed between the vessel and VTS providing the service.

  - Provision of information, such as:

    - Course and speed made good by a vessel;

    - Position relative to fairway axis, navigational features and/or way-points;

    - Proximity to navigational hazards;

    - Positions, identities, intentions and any restrictions of surrounding traffic.

  - Provision of advice and/or instruction, and may include or require:

    - An assessment of the suitability of the vessel to respond to the advice provided including an assessment of linguistic ability;

    - A review of vessel characteristics including manoeuvrability relative to the area in which the service is provided and any defects or deficiencies;

    - An assessment of the environmental conditions;

    - An assessment of the implications of the cargo carried;

    - A review of the proposed sailing plan;

    - Recommendations on measures to maintain the sailing plan noting that any advice on courses and speeds should be result orientated;

    - The use of message markers;

    - The use of a dedicated frequency;

    - Restriction of other traffic movement.

Along with the above VTS service types, the following equipment/services/systems can be considered to form the basis of the shoreside user requirements for e-navigation.  The following list of VTS equipment is not comprehensive and the requirements for each sensor or system are extensive, so all of those requirements cannot be replicated here.  However, an attempt was made to capture the most basic requirements (i.e. functional requirements or the data elements output by the equipment) of each below.

If shoreside user requirements are supported by e-navigation, then one can expect a benefit to VTS operators, other shoreside authorities and participating vessels.  More accurate and reliable information from the integration of sensors (e.g. LRIT) will serve to enhance the benefit through a more efficient use of existing resources.

- SHORESIDE SENSORS AND COMMUNICATIONS EQUIPMENT

In order to deliver the aforementioned services in Annex A, the following user needs may be necessary from a VTS perspective.

- Communications

  - The following general types of communication may be required by a VTS:

    - With vessels in or about to enter a VTS area;

    - Between VTSOs in the same VTS centre;

    - Between VTSOs in different VTS centres;

    - Automatic/manual communications between vessel traffic systems;

    - With Pilotage, tugboats, SAR, Port Authorities and other allied services.

  - Communications may comprise both verbal (voice) and data transmission.

  - These types of communication can be performed by public or private telephone lines, telex, facsimile and radio telephony, or automated data transfer systems.

  - Where there is a need for communication between a VTS and another VTS or an allied service the communications must have an appropriate level of reliability and security.

- Communication with Vessel Traffic in the VTS Area

  - VTS often has its own independent VHF network using specifically designated VHF Channels.  This network may be comprised of one or more VHF Channels in different sectors of the VTS Area.  A VTS Authority may require specific VHF Channels to be designated by the national telecommunications authorities for specific types of operations. The number of channels is dependent upon the number of sub-areas and sectors that will be used and the overall traffic density.

  - VHF equipment must comply with national and international regulations.

  - VHF can be used for telephony or data transmissions. (eg. AIS, DSC)

  - A VTS centre should have one or more general reporting or emergency VHF channels.

  - VHF Channel 16 is for emergencies and for calling non participating vessels.

  - The VTS should record communications with/and between vessels for accident investigation purposes.

- Communication with Vessel Traffic outside the VTS area

  - Standard maritime communication systems (e.g., MF, HF, Inmarsat) could be used where a VTS Authority requires information from vessels beyond VHF coverage.

- VTS Radar System

  - VTS radar is a sensor that does not depend on the cooperation of the target.  Detection is a complex process that depends on many factors, such as weather, atmospheric propagation and the target characteristics.

  - Irrespective of the specific circumstances, a VTS radar will need to operate with a minimum of operator adjustment. It needs to detect all specified moving or stationary targets which satisfy the detection criteria within the specified coverage areas and during all specified operating circumstances.

  - VTS radar information is processed to derive target position, speed over ground and course over ground.  This processing is called target tracking. Target tracks are used for functions such as:

    - Target labelling and identification;

    - Collision avoidance;

    - Vessel vector display;

    - Calculation of the Closest Point of Approach (CPA) and the time to CPA;

    - Path, time and track prediction;

    - Anchor watch;

    - Buoy watch.

- Automatic Identification System (AIS)

  - Ships provide AIS information autonomously and continuously with minimal involvement of ship’s personnel.

  - AIS validity is dependent on the proper installation of AIS, correctly interfaced and functioning ship’s equipment, and correct manual input of static and voyage-related data.

  - AIS provides static, dynamic, voyage-related and safety-related short messages.

    - AIS provides positional and manoeuvring information through dynamic messages at a data rate adequate to facilitate accurate tracking. It provides ship’s identity through static messages and voyage details through voyage-related messages.

    - AIS has a limited capability to transfer additional information through safety–related messages and application specific messages.

    - A shoreside station should have the ability to receive AIS data continuously.

  - AIS provides the capability to respond to safety related message with a minimum of delay.

  - AIS should help to identify vessels, assist in target tracking, simplify information exchange (i.e. reduce ship reporting using radiotelephony), and provide additional information to assist situational awareness.

  - Provide AIS information from one or more base stations or an AIS network to users. In addition to vessel data, provide status on AIS equipment and management functions.

  - Where applicable support regional Vessel Traffic Services between adjoining VTS Centres.

  - View AIS data on electronic chart, either separately or combined, with the other data sources including radar.

  - AIS should have the ability to filter the displayed, distributed and stored information.

- Radio Direction Finder (RDF)

  - RDF can be used to receive bearing information from radio transmissions and to assist in the identification of vessels.

  - All bearings should be automatically displayed on the chosen screen when signal received after a delay of no more than 3 seconds.

  - Bearings remain visible on the chosen screen as long as the vessel is transmitting a signal. If required, after the transmission has ceased, the display of the bearing can be extended by the VTS Operator.

- Hydrological/Meteorological Equipment

  - Local Hydrometeo information relevant to the VTS Area(s) should be made available to the VTS.

  - VTS should have an ability to disseminate this information to users and allied services.

  - Where a VTS Authority establishes its own monitoring stations, the individual VTS Authorities determine the accuracy and availability requirements for each VTS Centre.

  - Typical meteorological variables include air temperature and humidity, wind velocity and direction, and visibility.

  - Typical hydrological variables include tidal level, tidal stream/current direction and velocity.

  - Data can be obtained through sensors or from national authorities.

  - VTS displays should present graphical and/or numeric information for use by the VTS Operators.

  - Hydrological and meteorological information should be available to the VTS Operator for assessment of the environmental situation in the VTS area of responsibility.

  - VTS should provide information to ships to assist in assessing the waterway conditions.

- Closed circuit TV (CCTV) Cameras

  - CCTV should be integrated into VTS applications and;

    - Identify the type of vessel at a minimum range of 3 nm with nominal visibility in excess of 10 nm.

    - Identify a vessel by shape, colour and other features at a minimum range of 1 nm with nominal visibility in excess of 10 nm.

  - CCTV may be used as an independent sensor or in conjunction with other sensors.

  - CCTV may have an automatic vessel tracking function.

  - CCTV should provide recording or instant replay capability.

- Information Management

  - VTS should be able to maintain situational awareness by collecting, processing and correlating information from different sources to present an integrated traffic image, including its environment.

  - This may include:

    - Communications, internal and external;

    - Sensor data, i.e. data used to generate the traffic image such as radar, CCTV, AIS, status of AtoN;

    - Shipping information data, i.e. vessel and cargo data, including vessel movement information;

    - Meteorological and hydrological data;

    - Data from other sources if relevant. (eg. databases, sat images, Int. VTS, etc);

    - Data distribution to other shoreside users – bearing in mind that confidential data to one user may be public information to another;

    - Decision support systems;

    - Emergency response;

    - Notice to Mariners.

  - VTS should be able to record and store data for later use.

- Human Machine Interface / Traffic Situation & Information Display

  - VTS should have the ability to display charts, analytical graphics and overlay information to enable an operator to have a concise picture of the geographical features, waterways and navigational lanes, e.g. ECDIS.

  - VTS should have the ability to verify track positions and classify individual targets on the basis of sensor inputs.

  - There should be an ability to suppress radar video echoes of the coastline, making this area of the display available for other synthetic information, e.g. land blanking function.

  - If VHF RDF is included in the system, bearing lines should be shown on the VTS display, with an option for the operator to suppress.

  - Symbols should comply with IALA Recommendation V-125 On the use and presentation of symbology at a VTS Centre (including AIS).

  - There should be an ability to display and correlate the information from various sources, e.g. correlate AIS and radar data.

  - In addition to the traffic situation the ability to display textual information in the form of tables should be available.

  - A list of participating vessels, including static and dynamic information concerning the vessel, such as vessel name, call sign, IMO number, MMSI, ETA, ETD, course, speed and position should be available.

  - The VTS should have the ability to integrate database information into the traffic picture.

- Work Environment - VTS Operator Positions

  - Consoles should be provided at every VTSO position with the equipment integrated in ergonomic arrangement.

  - There should be access to database information at VTSO positions for reference purposes and for providing information to shipping.

Refer to ‘IALA Recommendation V-128 On Operational and Technical Performance Requirements

for VTS Equipment’.

The technical and system requirements of AIS are documented in IALA A-124 and ITU-R M.1371-3.