| From: e-NAV Committee | e-NAV7/output/12 |
| --- | --- |
| To: ANM Committee | 25 September 2009 |

Liaison Note

Electronic Position Fixing Systems (EPFS) requirements

# Introduction

This Liaison Note responds to input paper e-NAV7/8/23, which took into account a draft note from ANM 13, not provided officially to e-NAV 7).

# Accuracy Requirements

The e-NAV Committee did not see any technical reason why the accuracy requirements should be changed from the 10m, 10m and 100m currently specified in A.953(23).  The reason stated that higher accuracies than these are irrelevant when compared with the size of the vessel does not make sense without stating what size ship is referred to.  The system requirements do not take into account any ship size and are specified for small as well as large ships.

Various onboard systems depend on an electronic position input (AIS, autopilot etc) and there might be a requirement for even greater levels of accuracy e.g. Dynamic positioning

The antenna is normally mapped to show actual position of a ship on an ECDIS so the argument regarding where the antenna is located on a large ship and its relevance to an accuracy requirement may be false.  A larger ship may require higher accuracy due to the impact of the difference between the heading and true course over ground.  A large ship is likely to be both wide and long.  Ship dimensions should be known and the antenna position relative to the ships dimensions also needs to be known.

# Continuity Requirements

The issue of changing the period over which continuity is required from 3hours to 15minutes has been discussed previously and resulted in an output paper from e-NAV 3 with suggested amendments to A.953(23).  The continuity issue is now a work item within IMO and will be considered at NAV 56 in July 2010.

# Next Steps

The e-Navigation Committee consider that any proposed revision of IMO Resolutions A.915(22) and A.953(23) requires further deliberation, and would wish to develop such work in conjunction with the ANM Committee.  Due to the IALA meetings schedule, it is unlikely that this work could be completed to allow a submission to NAV 56.

# Action Requested

The ANM Committee is requested to work with the e-NAV Committee, in order to make a joint submission to IMO NAV 57.