DRAFT IALA TECHNICAL

CLARIFICATIONS

ON

RECOMMENDATION ITU-R M.1371-4

Edition 1.0

Table of Contents

1	Introduction	4

2	Technical Clarifications of ITU-R M.1371-4	6

2.1	A1; 4.2.1 Reporting interval; Table 2	7

2.1.1	Proposed Clarifying Text:	7

2.1.2	Rationale for Clarification:	7

2.1.3	Date of amendment:	7

2.1.4	Comment	7

2.2	A2; 4.4.1 Intentional slot reuse by the own station	7

2.2.1	Proposed Clarifying Text:	7

2.2.2	Rationale for Clarification:	10

2.2.3	Date of amendment:	10

2.2.4	Comment	10

2.3	A4; 3.3.2 Access scheme	10

2.3.1	Proposed Clarifying Text:	10

2.3.2	Rationale for Clarification:	10

2.3.3	Date of amendment:	10

2.3.4	Comment	10

2.4	A4; 3.3.3 AIS shore station qualifier	10

2.4.1	Proposed Clarifying Text:	10

2.4.2	Rationale for Clarification:	11

2.4.3	Date of amendment:	11

2.4.4	Comment	11

2.5	A5; 1.2 Definition of application identifiers	11

2.5.1	Proposed Clarifying Text:	11

2.5.2	Rationale for Clarification:	11

2.5.3	Date of amendment:	11

2.5.4	Comment	11

2.6	A8; 2 Message summary	11

2.6.1	Proposed Clarifying Text:	11

2.6.2	Rationale for Clarification:	14

2.6.3	Date of amendment:	14

2.6.4	Comment	14

2.7	A8; 3.1 Messages 1, 2, 3 Position Report	14

2.7.1	Proposed Clarifying Text:	14

2.7.2	Rationale for Clarification:	14

2.7.3	Date of amendment:	14

2.7.4	Comment	14

2.8	A8; 3.2 Message 4: Base station report; Message 11: UTC and date response	14

2.8.1	Proposed Clarifying Text:	14

2.8.2	Rationale for Clarification:	16

2.8.3	Date of amendment:	16

2.8.4	Comment	16

2.9	A8; 3.21 Message 15: Interrogation	16

2.9.1	Proposed Clarifying Text:	16

2.9.2	Rationale for Clarification:	17

2.9.3	Date of amendment:	17

2.9.4	Comment	17

2.10	A8; 3.21 Message 23: Group Assignment Command	17

2.10.1	Proposed Clarifying Text:	17

2.10.2	Rationale for Clarification:	19

2.10.3	Date of amendment:	19

2.10.4	Comment	19

2.11	A8; 3.22 Message 24: Static data report	19

2.11.1	Proposed Clarifying Text:	19

2.11.2	Rationale for Clarification:	19

2.11.3	Date of amendment:	19

2.11.4	Comment	19

Technical Clarifications of

Recommendation ITU-R M.1371-4

Edition 3.1

# Introduction

  - The purpose of AIS is to improve the maritime safety and efficiency of navigation, safety of life at sea and the protection of the marine environment.

  - It has long been realized that an automatic reporting device fitted to a ship would have the potential to increase significantly the safety of navigation and would allow the improved control and monitoring of the maritime environment.  Taking up this challenge, the International Maritime Organization (IMO) together with the International Telecommunications Union (ITU) and the International Electrotechnical Commission (IEC) developed a new navigation system called the Automatic Identification System (AIS).

  - Recommendation ITU-R M.1371-4 is the document that describes the technical characteristics for an Automatic Identification System using Time Division Multiple Access in the VHF Maritime Mobile Band. As with any technical document some concepts are subject to interpretation. This document presents a series of clarifications for some of these concepts as agreed by the IALA e-NAV Committee.

  - Figure 1 (attached) illustrates the inter-relationship of this document to IMO SOLAS Chapter V; the IMO recommendation on Performance Standards for a Universal Shipborne Automatic Identification System (AIS), (MSC.74(69)); the IMO Guidelines for the Onboard Operational Use of the Shipborne AIS (A.917(22)); the ITU Recommendation on the Technical Characteristics for an Automatic Identification System (AIS) using Time Division Multiple Access in the Maritime Mobile Band (ITU-R M.1371-4); and the IEC Standard 61993-Part 2: Universal Shipborne Automatic Identification System (AIS) Operational Performance Requirements, Methods of testing and required test results; the IALA Guidelines on Automatic Identification Systems (AIS).

  - As shown in Figure 1, each of the referenced documents in paragraph 1.4 including this document has a specific focus in relation to an overall AIS as follows.

This document, "Recommendation on Technical Clarification of ITU-1371-4", deals specifically with the VHF Data Link (VDL) of an AIS. It acknowledges the fact that some of the specific technical paragraphs in ITU-R M.1371-4 might require interpretations as to their precise, intended meaning.  Consequently, this document presents a set of unifying recommendations that will, hopefully, result in a standardized application of these paragraphs by potential manufacturers, system integrators as well as users of the systems.  Adherence to these recommendations will ensure the viability of the VDL and the interoperability among AIS units obtained from different sources.

  - However, this document also covers some other specific, technical details of various possible AIS station or device configurations that are only mentioned in general terms in ITU-R M.1371.

  - All of the identified documents have a specific focus and address specific user needs. Nevertheless, IALA also recognized that there is a need for a high level document that consolidates all of technical and operational aspects of the sub-systems described in Figure 1.  Consequently, IALA is at this time also drafting a general IALA Guideline on AIS  which will incorporate all of the elements in Figure 1 in a descriptive manner.

| IALA Documents |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAVGUIDE [AIS Guideline Overview xxxx] |  |  |  |  |  |  |  |  |  |  |  |
| Guidelines 1028-1029 AIS as an e-Nav service |  | Guideline 1059 Comparison of AIS stations |  |  |  |  |  | Guideline 1062 Establishment AIS as AtoN |  | Guideline xxx Future guidelines |  |
| A-123 Provision of shore based AIS | A-124 AIS Service |  |  |  |  | A-126 AIS in marine aids to navigation |  |  |  | A-yyy Future recommendations |  |
|  | Annex FATDMA planning |  | Annex …. |  | Annex …. |  |  |  |  |  |  |
| Non IALA Documents |  |  |  |  |  |  |  |  |  |  |  |
| IEC 61993-2 Class A | IEC 62287-1/-2 Class B “CS”/”SO” |  |  | IEC 62320-1 Base station |  |  | IEC 62320-2 AtoN station |  | IEC 61097-14 AIS-SART |  | IEC zzz Future standards |
| ITU-R documentation on AIS, e.g. ITU-R M.1371-4 |  |  |  |  |  |  |  |  |  |  | IALA Technical Clarifications on 1371-4 |
| IMO documentation on AIS, e.g. AIS Performance standards |  |  |  |  |  |  |  |  |  |  |  |

Figure 1  Documents Matrix

# Technical Clarifications of ITU-R M.1371-4

Important General Remark

Every clarification of this Chapter is presented in a uniform way:

- Every clarification is introduced in the appropriate context of Recommendation ITU-R M.1371-4, which is always quoted as a whole section. This is done to ensure, that all clarifications are to be understood in their appropriate context.

- The verbatim text of Recommendation ITU-R M.1371-4 is given in italics; additions by the clarification are given upright and underlined as of previous clarification editions and upright and double underlined in the present edition. Deleted text portions are identified with strike-out as of previous clarification editions and double strike out in the present edition.

- For every clarification a rationale is given to allow complete understanding as to why a clarification was needed also. Eventually, the clarifications will be incorporated into a future revision of Recommendation ITU-R M.1371-4).

- A date of when the clarification was drafted is given.

- A comment contains additional information on the clarification itself. For instance, some clarifications have already been incorporated in the first edition of the IEC standard for Class A shipborne mobile AIS stations (IEC 61993-2 latest edition). A future substantial change of the clarification result in the creation of a legacy issue. Any future change for those clarifications should therefore be done with the utmost caution.

## A1; 4.2.1	Reporting interval; Table 2

### Proposed Clarifying Text:

TABLE  2

Reporting intervals for equipment other than Class A shipborne mobile equipment

| Platform’s condition | Nominal reporting interval | Increased Reporting Interval for Class B SO |
| --- | --- | --- |
| Class B “SO” shipborne mobile equipment not moving faster than 2 knots | 3 min(1) | 3 min(1) |
| Class B “SO” shipborne mobile equipment moving 2-14 knots | 30 s(1) | 30 s(1) |
| Class B “SO” shipborne mobile equipment moving 14-23 knots | 15 s(1), (3) | 30 s(1), (3), (5) |
| Class B “SO” shipborne mobile equipment moving 23 knots | 5 s(1), (3) | 15 s(1), (3), (5) |
| Class B “CS” shipborne mobile equipment not moving faster than 2 knots | 3 min |  |
| Class B “CS” shipborne mobile equipment moving faster than 2 knots | 30 s |  |
| Search and rescue aircraft (airborne mobile equipment)(4) | 10 s |  |
| Aids to navigation | 3 min |  |
| AIS base station(2) | 10 s |  |
| (1) When a mobile station determines that it is the semaphore (see § 3.1.1.4, Annex 2) the reporting interval (RI) should decrease to 2 s (see § 3.1.3.3.2, Annex 2). (2) The base station’s RI should decrease to 3 1/3 s after the station detects that one or more stations are synchronizing to the base station (see § 3.1.3.3.1, Annex 2). (3) The nominal RI for Class B “CS” is 30 s. (4) Shorter RI down to 2 s could be used in the area of search and rescue operations. (5) Class B “SO”AIS shall report at the “Increased Reporting Interval” when less than 50% of the slots of each of the last four consecutive frames are Free. When more than 65% of the slots of each of the last four consecutive frames are Free, the Class B “SO” AIS shall report at the “Nominal Reporting Interval”. |  |  |

### Rationale for Clarification:

Introduction of the polite behaviour of Class B “SO” with respect to the VDL load

### Date of amendment:

### Comment

## A2; 4.4.1 Intentional slot reuse by the own station

### Proposed Clarifying Text:

A station should reuse time slots only in accordance with this paragraph and only when own position is available.

When selecting new slots for transmission, the station should select from its candidate slot set (see § 3.3.1.2) within the desired SI. When the candidate slot set has less than 4 slots, the station should intentionally reuse available slots, in order to make the candidate slot set equal to 4 slots. Slots may not be intentionally reused from stations that indicate no position available. This may result in fewer than 4 candidate slots. The intentionally reused slots should be taken from the most distant station(s) within the SI. Slots allocated or used by base stations should not be used unless the base station is located over 120 NM from the own station. When a distant station has been subject to intentional slot reuse, that station should be excluded from further intentional slot reuse during a time period equal to one frame.

Slot reuse provides candidate slots for random selection. This process attempts to increase the candidate slot set to a maximum of four. When the candidate slot set has reached four, the candidate slot selection process is complete. If four slots have not been identified after all the rules have been applied, this process may report less than four slots. Candidate slots for reuse are defined as:

Free (F):

F:  Not used;

L:  Mobile Stations (SOTMDA or ITDMA), or Base Stations reserved slot (FATMDA) beyond 120 nautical miles;

G:  Garbled

Available (A):

D:  allocated by the most distant station(s);

Unavailable (U):

R:  distant station that has been subject to intentional slot reuse, and so excluded from further intentional slot resuse during a time period equal to one frame;

E:  externally allocated (allocated by another station near own station or a station “moored” or “at anchor” with a reporting interval of 3 minutes)

B:  Base Station reserved slot (FATDMA) within 120 nautical miles, or a Mobile Station reporting without position information.

I:  internally allocated (allocated by own station, not in use)

X:  should not be used, transmitter switching time

Candidate slots for reuse should be selected using the following priorities beginning with Rule 1 0(also see the Slot selection rules flow diagram – Fig. 22) assuming channel A is the desired transmitting channel:.

Add to the Free slot set (if any) all slots that are:

Rule 1:	Free (see § 3.1.6) on selection channel and Available(1) (see § 3.1.6) on the other channel.

Rule 2:   Available(1) on selection channel and Free on the other channel.

Rule 3:   Available(1) on both channels.

Rule 4:  Free on selection channel and Unavailable(2) on the other channel.

Rule 5:  Available(1) on selection channel and Unavailable(2) on the other channel.

(1) 	Available – Mobile Station (SOTDMA or ITDMA), or Base Station reserved slot (FATDMA or Message 4) beyond 120 NM.

(2) 	Unavailable – Base Station reserved slot (FATDMA or Message 4) within 120 NM, or a Mobile Station reporting without position information.

Rule 0:  Channel A:  Free

Channel B:  Free

Rule 1:  Channel A:  Free

Channel B:   Available

Rule 2:  Channel A:  Available

Channel B:  not Unavailable

Rule 3:  Channel A:  Free

Channel B:   Unavailable

Rule 4:  Channel A:  Available

Channel B:  Unavailable

Note: When all slots are Unavailable see 3.3.1.2

|  | Rule 0 | Rule 1 | Rule 2 | Rule 3 | Rule 4 |
| --- | --- | --- | --- | --- | --- |
| CH A | F, G,L | F,G ,L | D | F,G,L | D |
| CH B | F, G,L | D | F,G,L,D | R,E,B | R,E,B |

Figure 21 is an example applying these rules.

Figure 21

Example of possible SI for slot reuse:

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CH A | F | G | F | G | L | F | D | D | D | D | F | G | L | D | D | X | X | X | I |
| CH B | F | G | L | D | D | D | F | G | L | D | R | B | R | E | R | E | I | B | F |

| Rule | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

na = not allowed for slot reuse

note:  Figure 21 needs to be a double strike through

It is intended to reuse one slot within the SI of frequency channel A. The current status of the use of the slots within the SI on both frequency channels A and B is given as follows:

F:	Free

I:	Internally allocated (allocated by own station, not in use)

E: 	Externally allocated (allocated by another station near own station)

B: 	Allocated by a base station within 120 NM of own station

T:	Another station under way that has not been received for 3 min or more

D:	Allocated by the most distant station(s)

X:	Should not be used.

The slot for intentional slot reuse should then be selected by the following priority (indicated by the number of the slot combination as given in Fig. 21):

Highest Selection Priority:	No. 1
	No. 2
	No. 5
	No. 6
	No. 3
	No. 4
	No. 7
Lowest Selection Priority	No. 8

Combinations 9, 10, 11 and 12 should not be used.

Rationale for not using slot combinations:

No. 9	Adjacent slot rule
No. 10	Opposite channel rule
No. 11	Adjacent slot rule
No. 12	Base station rule.

### Rationale for Clarification:

Garbling:

It is important to recognize that garbling does occur, however we do not have enough data to change the slot selection rules.  Therefore, until more data is gathered and analyzed a garbled slot should be recognized as garbled but treated as FREE, reflecting current Class A behaviour.

Slot selection rules and example:

The current Rule 2, Available on Channel A and Free on B, may force you to reuse the slot of a closer target only because the other channel's slot is Free, and the current Rule 3, Available on Channel A and Available on Channel B, may force you to not reuse the slot of a more distant target just because the other channel's slot is not Free.  Therefore Rule 2 and 3 should be considered together resulting in the proposed change.

The example and format of the rules were changed only to provide clarity and to reflect the combination of rules 2 and 3.

### Date of amendment:

### Comment

## A4; 3.3.2	Access scheme

### Proposed Clarifying Text:

The access scheme for transmitting the long-range AIS broadcast message should be Satellite Slot Selection Access (SSSA) RATDMA. The RATDMA algorithm, using the AIS terrestrial channels (AIS 1, AIS 2 or regional channels), will be used to select a slot but the transmission will be on the two separate designated long-range channels.  The AIS should consider only the two frequencies in the AIS VDL for qualifying the candidate slots for the transmission of the long-range AIS message. The AIS VDL is comprised only of channel A (by default AIS 1) and channel B (by default AIS 2). The long-range AIS broadcast on the two separate designated long-range channels should not be considered part of the AIS VDL for the purpose of scheduling the RATDMA transmissions of the long-range AIS broadcast messages.

Note: The purpose is to avoid transmitting during slots when the unit expects to receive messages from other AIS stations.

### Rationale for Clarification:

Original text used RATDMA incorrectly as access scheme.

### Date of amendment:

### Comment

## A4; 3.3.3	AIS shore station qualifier

### Proposed Clarifying Text:

Transmission of the long-range AIS broadcast message, when the AIS Class-A station is in the range of an AIS base station, should be left to the decision of the competent authority by Message 4 in conjunction with Message 23. When a Class A unit receives Message 23 with Station Type 10 (= Base Station Coverage Area) it will use the Lat/Lon from the Message 23 to define the “base station coverage area”;  all other fields will be ignored.  This base station coverage area should be calculated according to the rules described in section 4.1.5 Annex 2. The Class A unit must receive both the Message 4 with the control setting “Off” for the transmission of Message 27 and a Message 23 with the definition of the base station coverage area; it will then verify that it is within the base station coverage area and stop transmission of Message 27.  The control and the base station coverage area will time out within 3 minutes of the last Message 4. If the AIS Class A station does not receive a Message 4 and Message 23, it should revert to its nominal behaviour after three minutes.

### Rationale for Clarification:

Defines base station coverage area for Message 27 transmission control and the behaviour of the Class A mobile station.

### Date of amendment:

### Comment

## A5; 1.2	Definition of application identifiers

### Proposed Clarifying Text:

The application identifier uniquely identifies the message and its contents. The application identifier is a 16-bit number used to identify the meaning of the bits making up the data content. The use of application identifiers is defined in § 2.

The DAC is a 10-bit number. DAC assignments are:

–	international (DAC = 1-9), maintained by international agreement for global use;

–	regional (DAC >= 10), maintained by the regional authorities affected;

–	test (DAC = 0), used for test purposes.

It is recommended that DAC 2-9 be used to identify subsequent versions of international specific messages and that the administrator of application specific messages base the DAC selection on the maritime identification digit (MID) of the administrator’s country or region. It is the intention that any application specific message can be utilized worldwide. The choice of the DAC does not limit the area where the message can be used.

The FI is a 6-bit number assigned to uniquely identify the data content structure within an application under a DAC assignment. Each DAC can support up to 64 applications.

–	The definition of the technical characteristics, as defined in Annexes 2, 3 and 4, of any AIS station covers layers 1 to 4 of the OSI model, only (see § 1, Annex 2).

–	The layers 5 (session layer), 6 (presentation layer) and 7 (application layer, that includes the human-machine-interface) should be in accordance with the definitions and guidelines given in this Annex in order to avoid application conflicts.

### Rationale for Clarification:

Behaviour when the DAC is equal to 10 must be defined.

### Date of amendment:

### Comment

## A8; 2	Message summary

### Proposed Clarifying Text:

The defined messages are summarized in Table 43.

TABLE 43

| Message ID | Name | Description | Priority | Access scheme | Communi-cation state | M/B |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Position report | Scheduled position report; (Class A shipborne mobile equipment) | 1 | SOTDMA, RATDMA, ITDMA(1) | SOTDMA | M |
| 2 | Position report | Assigned scheduled position report; (Class A shipborne mobile equipment) | 1 | SOTDMA(9) | SOTDMA | M |
| 3 | Position report | Special position report, response to interrogation; (Class A shipborne mobile equipment) | 1 | RATDMA(1) | ITDMA | M |
| 4 | Base station report | Position, UTC, date and current slot number of base station | 1 | FATDMA(3), (7), RATDMA(2) | SOTDMA | B |
| 5 | Static and voyage related data | Scheduled static and voyage related vessel data report; (Class A shipborne mobile equipment) | 4(5) | RATDMA, ITDMA(2) | N/A | M |
| 6 | Binary addressed message | Binary data for addressed communication | 4 | RATDMA(10), FATDMA, ITDMA(2) | N/A | M/B |
| 7 | Binary acknowledgement | Acknowledgement of received addressed binary data | 1 | RATDMA, FATDMA, ITDMA(2) | N/A | M/B |
| 8 | Binary broadcast message | Binary data for broadcast communication | 4 | RATDMA(10), FATDMA, ITDMA(2) | N/A | M/B |
| 9 | Standard SAR aircraft position report | Position report for airborne stations involved in SAR operations, only | 1 | SOTDMA, RATDMA, ITDMA(1) | SOTDMA ITDMA | M |
| 10 | UTC/date inquiry | Request UTC and date | 3 | RATDMA, FATDMA, ITDMA(2) | N/A | M/B |
| 11 | UTC/date response | Current UTC and date if available | 3 | RATDMA, ITDMA(2) | SOTDMA | M |
| 12 | Addressed safety related message | Safety related data for addressed communication | 2 | RATDMA(10), FATDMA, ITDMA(2) | N/A | M/B |
| 13 | Safety related acknowledgement | Acknowledgement of received addressed safety related message | 1 | RATDMA, FATDMA, ITDMA(2) | N/A | M/B |
| 14 | Safety related broadcast message | Safety related data for broadcast communication | 2 | RATDMA(10), FATDMA, ITDMA(2) | N/A | M/B |
| 15 | Interrogation | Request for a specific message type (can result in multiple responses from one or several stations)(4) | 3 | RATDMA, FATDMA, ITDMA(2) | N/A | M/B |
| 16 | Assignment mode command | Assignment of a specific report behaviour by competent authority using a Base station | 1 | RATDMA, FATDMA(2) | N/A | B |
| 17 | DGNSS broadcast binary message | DGNSS corrections provided by a base station | 2 | FATDMA(3), RATDMA(2) | N/A | B |
| 18 | Standard Class B equipment position report | Standard position report for Class B shipborne mobile equipment to be used instead of Messages 1, 2, 3(8) | 1 | SOTDMA, ITDMA(1), CSTDMA | SOTDMA, ITDMA | M |
| 19 | Extended Class B equipment position report | Extended position report for class B shipborne mobile equipment; contains additional static information(8) | 1 | ITDMA | N/A | M |
| 20 | Data link management message | Reserve slots for Base station(s) | 1 | FATDMA(3), RATDMA | N/A | B |
| 21 | Aids-to-navigation report | Position and status report for aids-to-navigation | 1 | FATDMA(3), RATDMA(2) | N/A | M/B |
| 22 | Channel management(6) | Management of channels and transceiver modes by a Base station | 1 | FATDMA(3), RATDMA(2) | N/A | B |
| 23 | Group assignment command | Assignment of a specific report behaviour by competent authority using a Base station to a specific group of mobiles | 1 | FATDMA, RATDMA | N/A | B |
| 24 | Static data report | Additional data assigned to an MMSI Part A: Name Part B: Static Data | 4 | RATDMA, ITDMA, CSTDMA, FATDMA | N/A | M/B |
| 25 | Single slot binary message | Short unscheduled binary data transmission (Broadcast or addressed) | 4 | RATDMA, ITDMA, CSTDMA, FATDMA | N/A | M/B |
| 26 | Multiple slot binary message with Communications State | Scheduled binary data transmission (Broadcast or addressed) | 4 | SOTDMA, RATDMA, ITDMA | SOTDMA, ITDMA | M/B |
| 27 | Position report for long-range applications | Scheduled position report; (Class A shipborne mobile equipment outside base station coverage) | 1 | RATDMA SSSA | N/A | M |

### Rationale for Clarification:

Modify  the access scheme for Message 27 to be SSSA.

### Date of amendment:

### Comment

## A8; 3.1 Messages 1, 2, 3 Position Report

### Proposed Clarifying Text:

TABLE 47

Determination of position accuracy information

| Accuracy status from RAIM (for 95% of position fixes)(1) | RAIM flag | Differential correction status(2) | Resulting value of PA flag |
| --- | --- | --- | --- |
| No RAIM process available | 0 | Uncorrected | 0 = low (>10 m) |
| EXPECTED RAIM error is <= 10m | 1 |  | 1 = high (<=10 m) |
| EXPECTED RAIM error is > 10m | 1 |  | 0 = low (>10 m) |
| No RAIM process available | 0 | Corrected | 1 = high (<=10 m) |
| EXPECTED RAIM error is < =10m | 1 |  | 1 = high (<=10 m) |
| EXPECTED RAIM error is > 10m | 1 |  | 0 = low (>10 m) |

### Rationale for Clarification:

Behaviour when the RAIM error is equal to 10m must be defined.

### Date of amendment:

### Comment

## A8; 3.2	Message 4: Base station report; Message 11: UTC and date response

### Proposed Clarifying Text:

Should be used for reporting UTC time and date and, at the same time, position. A base station should use Message 4 in its periodical transmissions and should be used for the transmission control of Message 27 in conjunction with Message 23 which defines the base station coverage area.

A mobile station should output Message 11 only in response to interrogation by Message 10.

Message 11 is only transmitted as a result of a UTC request message (Message 10). The UTC and date response should be transmitted on the channel, where the UTC request message was received.

TABLE 48

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message 4 or 11 4 = UTC and position report from base station 11 = UTC and position response from mobile station |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. Refer to § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| UTC year | 14 | 1-9999; 0 = UTC year not available = default |
| UTC month | 4 | 1-12; 0 = UTC month not available = default; 13-15 not used |
| UTC day | 5 | 1-31; 0 = UTC day not available = default |
| UTC hour | 5 | 0-23; 24 = UTC hour not available = default; 25-31 not used |
| UTC minute | 6 | 0-59; 60 = UTC minute not available = default; 61-63 not used |
| UTC second | 6 | 0-59; 60 = UTC second not available = default; 61-63 not used |
| Position accuracy | 1 | 1 = high (< > 10 m) 0 = low (> < 10 m) 0 = default The PA flag should be determined in accordance with Table 47 |
| Longitude | 28 | Longitude in 1/10 000 min (180º, East = positive (as per 2’s complement), West = negative (as per 2’s complement); 181= (6791AC0h) = not available = default) |
| Latitude | 27 | Latitude in 1/10 000 min (90º, North = positive (as per 2’s complement), South = negative (as per 2’s complement); 91 = (3412140h) = not available = default) |
| Type of electronic position fixing device | 4 | Use of differential corrections is defined by field position accuracy above: 0 = undefined (default) 1 = global positioning system (GPS) 2 = GNSS (GLONASS) 3 = combined GPS/GLONASS 4 = Loran-C 5 = Chayka 6 = integrated navigation system 7 = surveyed 8 = Galileo 9-14= not used 15 = internal GNSS |
| Transmission control for long-range broadcast message | 1 | 0 = default – Class-A AIS station stops transmission of Message 27 within an AIS base station coverage area. 1 = Request Class-A station to transmit Message 27 within an AIS base station coverage area. Base station coverage area must be defined by Message 23; If Message 23, defining the base station area, is not received the Class A mobile will ignore this bit. |
| Spare | 9 | Not used. Should be set to zero. Reserved for future use |
| RAIM-flag | 1 | RAIM (Receiver autonomous integrity monitoring) flag of electronic position fixing device; 0 = RAIM not in use = default; 1 = RAIM in use see Table 47 |
| Communication state | 19 | SOTDMA communication state as described in § 3.3.7.2.1, Annex 2 |
| Number of bits | 168 |  |

### Rationale for Clarification:

The base station coverage area must be deterministic in order to:

- Avoid unpredictable behaviour from mobile stations, when conflicting commands are received from multiple base stations;

- Provide deterministic behaviour for satellite data correlation purposes;

- Provide deterministic behaviour so that anomalous behaviour by a mobile station can be detected;

### Date of amendment:

### Comment

## A8; 3.21	Message 15: Interrogation

### Proposed Clarifying Text:

This message should be used for interrogations via the TDMA (not DSC) VHF data link other than  except for requests for UTC and date. The response should be transmitted on the channel where the interrogation was received.

TABLE 62

| Interrogator | Class A | Class B-SO | Class B -CS | SAR aircraft | AtoN | Base station |
| --- | --- | --- | --- | --- | --- | --- |
| Interrogated |  |  |  |  |  |  |
| Class A | 3, 5 | N | N | 3, 5 | N | 3, 5, 24(1) |
| Class B-SO | 18, 19, | N | N | 18, 19 | N | 18, 19, 24(1) |
| Class B-CS | 18, 24(1) | N | N | 18, 24(1) | N | 18, 19, 24(1) |
| SAR-aircraft | 9, 24(1) | N | N | 9 | N | 9, 24(1) |
| AtoN | 21(2) | N | N | N | N | 21(2) |
| Base Station | 4, 24(1) | N | N | 4, 24(1) | N | 4, 24(1) |
| (1) An Interrogation for Message 24 shall be answered either with a Part A or a Part B or with both Part A and Part B and depending on the its own capability of the unit with a Part B. (2) Some AtoN stations are not able to respond due to there their operational behaviour. The parameter slot offset should be set to zero, if slot should autonomously be allocated by the responding station. An interrogating mobile station should always set the parameter “slot offset” to zero. Slot assignments for the reply to an interrogation should only be used by a base station. If a slot offset is given, it should be relative to the start slot of this transmission. A mobile station should be able to process a minimum slot offset of 10 slots. There should be the following four (4) possibilities to use this message: – One (1) station is interrogated one (1) message: The parameters destination ID1, message ID1.1 and slot offset 1.1 should be defined. All other parameters should be omitted. – One (1) station is interrogated two (2) messages: The parameters destination ID1, message ID1.1, slot offset 1.1, message ID1.2, and slot offset 1.2 should be defined. The parameters destination ID2, message ID2.1, and slot offset 2.1 should be omitted. See § 3.3.7, Annex 2 for byte boundaries. – The first station and the second station are interrogated one (1) message each: The parameters destination ID1, message ID1.1, slot offset 1.1, destination ID2, message ID2.1, and slot offset 2.1 should be defined. The parameters message ID1.2 and slot offset 1.2 should be set to zero (0). – The first station is interrogated two (2) messages, and the second station is interrogated one (1) message: All parameters should be defined. |  |  |  |  |  |  |

### Rationale for Clarification:

Provide means to interrogate the Vendor ID of the AIS unit.

### Date of amendment:

### Comment

## A8; 3.21	Message 23: Group Assignment Command

### Proposed Clarifying Text:

The Group Assignment Command is transmitted by a Base station when operating as a controlling entity(see § 4.3.3.3.2 Annex 7 and § 3.20). This message should be applied to a mobile station within the defined region and as selected by “Ship and Cargo Type” or by “Station type”. The receiving station should consider all selector fields concurrently.

It controls the following operating parameters of a mobile station:

–	transmit/ receive mode;

–	reporting interval;

–	the duration of a quiet time.

Station type 10 should be used to define the base station coverage area for control of Message 27 transmissions by Class A mobile stations. When station type is 10 only the fields latitude, longitude are used, all other fields should be ignored. This information will be relevant until three minutes after the last reception of controlling message 4 from the same base station (same MMSI).

TABLE 73

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 23; always 23 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0-3; default = 0; 3 = do not repeat any more |
| Source ID | 30 | MMSI of assigning station |
| Spare | 2 | Spare. Should be set to zero |
| Longitude 1 | 18 | Longitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (180, East = positive, West = negative) |
| Latitude 1 | 17 | Latitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (90, North = positive, South = negative) |
| Longitude 2 | 18 | Longitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (180, East = positive, West = negative) |
| Latitude 2 | 17 | Latitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (90, North = positive, South = negative) |
| Station type | 4 | 0 = all types of mobiles (default); 1 = Class A mobile stations only; 2 = all types of Class B mobile stations; 3 = SAR airborne mobile station; 4 = Class B “SO” mobile stations only; 5 = Class B “CS” shipborne mobile station only; 6 = inland waterways; 7 to 9 = regional use and; 10 = Base Station Coverage Area (see message 4 and message 27); 11 10 to 15 = for future use |
| Type of ship and cargo type | 8 | 0 = all types (default) 1...99 see TABLE 50 100...199 reserved for regional use 200...255 reserved for future use |
| Spare | 22 | Not used. Should be set to zero. Reserved for future use |
| Tx/Rx mode | 2 | This parameter commands the respective stations to one of the following modes: 0 = TxA/TxB, RxA/RxB (default); 1 = TxA, RxA/RxB , 2 = TxB, RxA/RxB, 3 = reserved for future use |
| Reporting interval | 4 | This parameter commands the respective stations to the reporting interval given in Table 74 |
| Quiet time | 4 | 0 = default = no quiet time commanded; 1-15 = quiet time of 1 to 15 min |
| Spare | 6 | Not used. Should be set to zero. Reserved for future use |
| Number of bits | 160 | Occupies one-time period |

### Rationale for Clarification:

The base station coverage area must be deterministic in order to:

- Avoid unpredictable behaviour from mobile stations, when conflicting commands are received from multiple base stations;

- Provide deterministic behaviour for satellite data correlation purposes;

- Provide deterministic behaviour so that anomalous behaviour by a mobile station can be detected;

### Date of amendment:

### Comment

## A8; 3.22	Message 24: Static data report

### Proposed Clarifying Text:

Message 24 Part A and Part B may be used by any AIS station to associate a MMSI with a name.

Message 24 Part A and Part B should be used by Class B “CS” shipborne mobile equipment. The message consists of two parts. Message 24B should be transmitted within 1 min following Message 24A.

In case of an interrogation for a Class B “CS” on a Message 24, the response should include Part A and Part B.

When requesting the transmission of a Message 24 from a Class B “CS”, the AIS station should respond with part A and part B.

When requesting the transmission of a Message 24 from a Class A or Class B “SO”, the AIS station should respond with part B, which may contain the vendor ID only.

TABLE 76A

Vendor ID field

| Bit | Information | Description |
| --- | --- | --- |
| (MSB) 41 …...... 24 (18 bits) | Manufacturer’s ID | The Manufacturer's ID bits indicate the manufacture’s mnemonic code consisting of three 6 bit ASCII characters(1) |
| 23 …...... 20 (4 bits) | Unit Model Code | The Unit Model Code bits indicate the binary coded series number of the model. The first model of the manufacture uses “1” and the number is incremented at the release of a new model. The code reverts to “1” after reaching to “15”. The “0” is not used |
| 19 …...... 0 (LSB) (20 bits) | Unit Serial Number | The Unit Serial Number bits indicate the manufacture traceable serial number. When the serial number is composed of numeric only, the binary coding should be used. If it includes figure(s), the manufacture can define the coding method. The coding method should be mentioned in the manual |
| (1) IALA is presently considering the creation of an open international register of manufacturers’ IDs. NMEA mnemonic manufacturer codes should be used for Message 24B Manufacturer ID. Manufacturers and or Vendors may request this code via NMEA at www.nmea.org. |  |  |

### Rationale for Clarification:

Clarify the holder of the register of the Manufacturer ID and where it can be requested

### Date of amendment:

### Comment