e-NAV9	Input paper

Agenda item	12

Task Number	25

Author(s)	Jon Leon Ervyk

Pierre D’Arcy

Victor Placido da Conceiçao

Maritime Information Systems (MIS): Scope and Focus

1. Summary

This Working Paper deals with scope of what constitutes “Maritime Information Systems” as a component of e-Navigation.  It describes a variety of shore-based and shipborne equipment, systems, services and processes.  As “work in progress”, it is not intended to be a definitive listing.  Instead, it is a basis for active discussion and further refinement by the IALA e-NAV Committee MIS Task Group.

2. Background

Definition:  “E-navigation is the harmonized collection, integration, exchange, presentation and analysis of marine information onboard and ashore by electronic means to enhance berth to berth navigation and related services for safety and security at sea and protection of the marine environment.”

As stated in NAV 54-25 Annex 12 “Draft Proposed Strategy for the Development and Implementation of e-Navigation”, the general expectations for the onboard, onshore and communications elements of e-Navigation are:

.1 Onboard:

Navigation systems that benefit from the integration of own ship sensors,

supporting information, a standard user interface, and a comprehensive system for

managing guard zones and alerts. Core elements of such a system will include,

actively engaging the mariner in the process of navigation while preventing

distraction and overburdening,

.2 Ashore:

The management of vessel traffic and related services from ashore enhanced

through better provision, co-ordination, and exchange of comprehensive data in

formats that will be more easily understood and utilized by shore-based operators

in support of vessel safety and efficiency, and

.3 Communications:

An infrastructure providing authorized seamless information transfer onboard

ship, between ships, between ship and shore and between shore authorities and

other parties with many related benefits, including a reduction of single person

error.

Since onboard and ashore elements of e-Navigation are closely related, they will be treated together.  Communications applies to both elements (ashore and shipborne) since it is the means to communicate information from shore-to-ship, ship-to-ship, and ship-to-shore.

E-Navigation User Needs  Systems/Services

Goal: 	Describe the variety of shore-based and shipborne marine navigation systems and services that would fall under the e-navigation concept of operations.

Scope: 	1) 	Develop an overview of the user need/requirements, and sources of information related to the various needs.

2) 	Breakdown the information and map it to the proposed object model. [VTS28/WG4/WP02]

### 3. Shore-based Information Needs

### The following identification is based on the three primary tasks of present and future VTS’s and related shore-based organizations (IMO  res. A.857(20)):

Information Service (INS). An Information Service provides essential and

timely information to assist the on-board decision-making process. An

Information Service does not participate in on-board decision-making;

Traffic Organisation Service (TOS). A Traffic Organisation Service is a

service to provide for the safe and efficient movement of traffic and to

identify and manage potentially dangerous traffic situations. A Traffic

Organisation Service provides essential and timely information to assist the

on-board decision-making process and may advise, instruct or exercise the

authority to direct movements;

Navigational Assistance Service (NAS). A Navigational Assistance Service

may be provided in addition to an Information Service and/or Traffic

Organisation Service. It is a service to assist in the on-board navigational

decision-making process and is provided at the request of a vessel, or when

deemed necessary by the VTS. A Navigational Assistance Service provides

essential and timely navigational information to assist the on-board decision-making

process and may inform, advise and/or instruct vessels accordingly.

Supportive Services may additionally be provided by VTS to support other shore-based authorities, stakeholders or services, i.e.:

Allied Services (Logistic)

- includes nautical services (e.g., linesmen)

Remedial services (SAR & Incident Management)

Law enforcement

Security

and other services

Tables 1 to 4 provide a preliminary description of elements pertaining to each category of services.

### TABLE 1.  Information Service (INS) [ref  IMO Resolution A.857(20)

| Task/Functions | Information Needed | Typical Source Shipborne (how collected | Typical Source Ashore | Communication means [WG4 should provide inputs on how they classify comms] | Maritime data modeling or data structure [to be determined] | VTS-related Information to be shown onboard vessels |
| --- | --- | --- | --- | --- | --- | --- |
| Maintaining a traffic image | Ship identity: Ship’s name MMSI number Call sign IMO number | AIS, radio voice communications AIS, LRIT, radio voice communications AIS, Radio voice communications AIS, Radio voice communications | S-AIS, AIS base station or network, radio voice communications, electro-optical equipment S-AIS, AIS base station or network, LRIT, radio voice communications (DSC) S-AIS, AIS base station or network AIS base station or network, radio, databases In general, most ship’s IDs can be obtained from several sources, including other VTSs. | VHF/MF/HF SATCOM Broadband mobile phone | Traffic obj IMO# |  |
|  | Other ship identities: Ship’s name (native and international) MMSI number Call sign | AIS, radio voice communications AIS, LRIT, radio voice communications AIS, radio voice communications | S-AIS, AIS base station or network, radio voice communications, electro-optical equipment S-AIS, AIS base station or network, LRIT, radio voice communications (DSC) S-AIS, AIS base station, Radar In general, most ship’s IDs can be obtained from several sources, including other VTSs. | VHF/MF/HF SATCOM Broadband mobile phone | MMSI#, callsign, name |  |
|  | Ship positions: - Time stamp - Time reference | GNSS (INS) AIS position(msg 17 available and INS no differentially correct) Retransmitted AIS Geonavigation | S-AIS, AIS base station or network, LRIT AIS base station or network AIS base station or network Voice, other reports Other Radio-direction finder Radar Visual Electro-Optical | VHF/MF/HF SATCOM Broadband mobile phone |  |  |
|  | Voyage plan related information | Email Web service Radio voice communications Mandatory reporting system AIS Fax | Email Web service Radio voice communications Mandatory reporting system Agent (User interface needed for agents to update HAZMAT, contact details) AIS base station or network Fax | VHF/MF/HF SATCOM Broadband mobile phone |  |  |
|  | Ship Navigational Status | AIS Radio voice communications Mandatory reporting system Visual signals (lights, etc.) | S-AIS, AIS base station or network Radio voice communications Mandatory reporting system Agent (User interface needed for agents to update HAZMAT, contact details) Visual observation, electro-optical equipment Agent | VHF/MF/HF SATCOM Broadband mobile phone Visual |  |  |
|  | Cargo information | Mandatory reporting system AIS Visual signals (lights, etc.) Email Web service Fax | Mandatory reporting system Agent (User interface needed for agents to update HAZMAT, contact details) S-AIS, AIS base station or network Visual observation, electro-optical equipment Email Web service Fax | VHF/MF/HF SATCOM Broadband mobile phone Visual |  |  |
|  | Number of persons on board | Mandatory reporting system Radio voice communications AIS Email Web service Fax | Mandatory reporting system Agent Radio voice communications S-AIS, AIS base station or network Email Web service Fax | VHF/MF/HF SATCOM Broadband mobile phone |  |  |
|  | Ship data |  | Lloyds list or other Web service Additional local service | Broadband |  |  |
|  | Chart data |  | Chart data and updates Notice to mariners | Broadband Paper format |  |  |
|  | Meteorological and hydrological conditions |  | hydro/meteo sensors Web service | Broadband AIS base station |  |  |
|  | Navigation safety information (eg. fairway information, obstructions, infrastructure out of order, AtoN off-station) | GMDSS Radio voice communications | GMDSS WWNWS AIS base station or network Radio voice communications | Broadband Paper format VHF/MF/HF SATCOM |  |  |
| Provision and Exchange of Information | Information to mariner: - METOC - Navigation hazard - Traffic situation - Ice report - Recommendation route - Special areas - Special operations - Piracy reports - Georeferenced data - PPUs infos - Etc. | Radio voice communications AIS Web service Fax | Radio voice communications AIS base station or network Web service Fax GMDSS | Broadband Paper format VHF/MF/HF SATCOM |  |  |
|  | Request from Master: - METOC - Navigation hazard - Traffic situation - Ice report - Recommendation route - Special areas - Special operations - Piracy reports - Georeferenced data - Etc. | Radio voice communications AIS Web service Fax | Radio voice communications AIS base station or network Web service Fax GMDSS | Broadband Paper format VHF/MF/HF SATCOM |  |  |
| Incident mitigation | In addition to traffic image infos: Safety incident report SAR incident report Security incident report Contingency plan Pollution incident report Alert Refugee place | GMDSS Email Fax | GMDSS Surveillance system Email Web service Operational data links Databases Fax | Broadband VHF/MF/HF SATCOM Paper format Mobile phone |  |  |

### TABLE 2.  Traffic Organization Service (TOS), in addition to INS.

| Task | Information Needed | Typical Source Shipborne | Typical Source Ashore | Interface Media | (U)MDM Data (preliminary) |
| --- | --- | --- | --- | --- | --- |
| Strategic planning | (long term) sailing plan |  | Agent | Ashore-ashore interfaces |  |
|  | ship database |  | e.g. Lloyds, VTS database | Ashore-ashore interfaces |  |
|  | berth planning |  | Port Administration | Ashore-ashore interfaces |  |
|  | route structure (traffic separation scheme) |  | Transport Administration |  |  |
|  | Cargo information policy |  | Transport Administration |  |  |
| Route adherence monitoring | Specific sailing plan |  | Radio voice communications |  |  |
|  | Cargo information and risk profile |  | HAZMAT, Risk analyses (Check the completeness of IMO Guidelines) Review existing standards in this area |  |  |
|  | traffic clearances (passing of waypoints) |  | Radio voice communications |  |  |
| Traffic separation (operations) | Processing of information according to rules |  | Radio voice communications, switching of traffic lights |  |  |
| Coordination with allied services | Vessel database, Allied Services database, sailing plan | Agent, Allied Services, Lloyds |  |  |  |

### TABLE 3.  Navigational Assistance Service (NAS), in addition to INS

Miscellaneous Issues

| Task | Information Needed | Source | Requirements | (U)MDM |
| --- | --- | --- | --- | --- |
| Vessels in VTS area to have adequate charts | Automated request from VTS Centre to Vessel for ENC chart version | Chart: Hydrographic office or commercial provider | Automated check that version of local ENC chart on board is adequate (future e-Nav) | xxx obj |
| Vessels in VTS area to have correct AIS data, if not, then report to responsible authority | AIS Static and voyage related data | Vessel AIS system |  |  |
| Navigation Safety Advisory | Tide info (static/dynamic, see environmental data) | AIS binary GMDSS |  |  |

### TABLE 4. Supportive Service

| Task/Functions | Information Needed | Source | Requirements (in addition to v128) | (U)MDM |
| --- | --- | --- | --- | --- |
| Allied Services (Maintaining logistics) | Berth Availability |  |  | xxx obj |
|  | Departure Time (ETD) |  |  |  |
|  | Port Facilities |  |  |  |
|  | ISPS |  |  |  |
|  | Pilot |  |  |  |
|  | Berth Availability |  |  |  |
|  | Departure Time (ETD) |  |  |  |
|  | Tug information |  | Position & availability |  |
|  | Linesmen |  |  |  |
|  | Regular Traffic |  | Schedules services in the area such as ferries |  |
| Provide traffic image to allied services | Surface picture of vessels and their movements in the VTS area to be transmitted | VTS Centre | Internal storage |  |
| Archive and replay of traffic image & concurrent data (voice, CCTV, met) |  |  | SAR authority, Coastguard |  |
| Coordination with Allied Services | SAR incident reports |  | Pollution Authority & Vessels |  |
|  | Pollution incident reports (Mechanism to escalate efficiently to responsible parties) |  |  | (as required) |
|  | …many others not listed… |  | All VTS equipment | n/a |

4. Requirements Draft (to be refined by VTS Committee OPS working group)

A summary of the information that should be part of the ship’s database is presented in Table 5. These information aims to identify or define the minimum fields required to deliver global consistency.

TABLE 5. Ship database -

| Description | Data Form |
| --- | --- |
| MMSI number | Std |
| Ship Name |  |
| Ship Native Name |  |
| IMO number |  |
| Flag State |  |
| Ship owner |  |
| Length |  |
| Beam |  |
| Draught |  |
| Air Draught |  |
| Type |  |
| Gross Tonnage |  |

| User requirement | Data Sources | Comments | UMD element |
| --- | --- | --- | --- |
| Position | AIS, radar, VHF, LRIT | AIS could also be from space | uoiuoi |
| Course | AIS, radar |  |  |
| Speed | AIS, radar, VHF |  |  |
| Heading | VHF |  |  |
| Draught | AIS, radar, VHF, reporting systems |  |  |
| Air draught | VHF |  |  |
| Flag | AIS, databases |  |  |
| Destination | AIS, VHF, reporting- and agent systems |  |  |
| Arrival Time (ETA (new)) |  |  |  |
| Ports of call |  |  |  |
| ROT |  |  |  |
| ISPS status |  |  |  |
| Ship Status | AIS |  |  |
| Passage Plan |  |  |  |
| HAZMAT |  |  |  |
| Marine Pollution |  |  |  |
| Pilot Status |  |  |  |
| Under-keel Clearance |  |  |  |
| CCTV |  |  |  |
| Radar Data |  | From ship, shore &space |  |
| Rebroadcast AIS |  | From ship |  |
| Statatory Information |  |  |  |
| Paris MOU status |  |  |  |
| MET/HYD & wind information |  |  |  |
| NAVAREA (new) |  | warnings (e.g., unlit lights, drifting objects, wrecks, etc.) |  |
| Limitations such as local regulations and legislations. (new) |  |  |  |
| SAR (Search and Rescue) information (new) |  |  |  |
| GEO(Group on Earth Observations)-special information such as sandbanks. (new) |  |  |  |
| Oil polluting information (new) |  |  |  |
| Dynamic MaRA information based on coastal vulnerability and weather conditions. (new) |  |  |  |
| Voyage planning (VP) information such as fairway availability, obstruction and events following the traffic behavior. (new) |  |  |  |

Logistics

| User requirement | Data Sources | Comments |
| --- | --- | --- |
| Port Facilities |  | Ex. Bunker, crane, waste |
| ISPS |  |  |
| Pilot |  |  |

### 5. Shipborne Information

### The following identification is based on the primary tasks of present and future shipborne navigation systems in order to optimize ship operations in terms of safety and security, and protection of the environment.

1. Route Planning/Grounding avoidance

2. Route Execution

3. Maintaining Situational Awareness

4. Operational Mode Awareness

5. Collision Avoidance

6. Emergency & SAR

7. Reporting

8. Special conditions (berthing, anchoring)

Details related toe ach task are presented in table 6.

### Reference documents

### IMO NAV54/WP.2, Annex 1

IMO NAV54-25 Annex 12

IMO Res A.893(21)

IMO SOLAS V/19

IMO MSC Circ. 1061 Guidance For The Operational Use Of  IBS

IMO MSC 78/11/3	Bridge Design, Equipment and Arrangements (IACS)

ISO 8468 Ship’s bridge layout and associated equipment. Requirements and guidelines

ISO 14612 Additional requirements and guidelines for centralized and integrated functions

STCW

ISM

TABLE 6. Shipborne Information

| Task/Functions | Information Needed | UMDS Information Type |  | Typical Shipborne Source | Typical External Source (Shore or other Ship ) | (U)MDM Data (preliminary) |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Route planning/Grounding avoidance [IMO Res A.893] | Route, Navigational hazards, Bathymetry, Traffic lanes, AtoN’s, Ports, etc. Under keel clearance (draft) Vertical clearance (air draft) | Charted information |  | ECDIS [ECDIS/ECS] Appropriate Charts/Chart database Chart updating/ECDIS [ECDIS/ECS] chart database update Routing charts Company orders Port state control | Chart provider Chart update service Notice to mariners MSI (e.g, NAVTEX, SafetyNet, Radio) Proprietary Routing services (Weather, other) |  |
|  | Climate information Ephemeris Currents METOC forecast Ice forecast | Tide prediction Water level forecast Wind forecast Published information or databases Updates to published info Marine Information Overlays |  | Publications or databases - Climatology - Almanac - Tide Tables - Coastal Pilots & Sailing Directions - List of aids to navigation GMDSS | Notice to mariners MSI (e.g, NAVTEX, SafetyNet, Radio) |  |
|  | Radio service information | Published information or database |  | List of radio stations & services (nautical pubs) |  |  |
|  | Environmental protection |  |  | GMDSS | Notice to mariners MIOs Nautical pubs AIS binary messages |  |
|  |  |  |  | ECDIS [ECDIS/ECS] |  |  |
|  | Ship characteristics (conditions) - static - dynamic |  |  | Onboard Ship design documentation |  |  |
|  | Ship stability condition | Visual observations Measurements Calculations |  | Onboard management | Load plan |  |
|  | Voyage constraints | Passage schedule, shore service needs, special cargo characteristics, etc. |  | Company orders | Port State control Berthing availability Pilot availability |  |
|  | Security issues | Security conditions, Warnings, precautions, procedures, etc. |  | GMDSS | Flag State Ship owners |  |
| 2. Route execution & monitoring | Voyage Plan | Voyage plan Appropriate Charts/Chart database |  | Chart or ECDIS [ECDIS/ECS] |  |  |
|  | Vessel position | Position sensors Radar fix, obs Visual fix, obs |  | GNSS (e.g., GPS/DGPS), inertial sensors Radar Compass, binoculars, pelorus, ATON’s | ATON’s |  |
|  | Sensor integrity/validity | Course Speed Heading |  | GNSS or calculated. SDME Gyrocompass, Mag compass |  |  |
|  | Time of day (HH:MM) | Time |  | GNSS, radio time broadcast clocks |  |  |
|  | VTS instructions |  |  | GMDSS/voice radio |  |  |
|  | Vessel parameter (squat) | Ship’s documentation |  | Ship’s data (e.g., table, sensors, sources, Ougi Board, etc.) | Contracted services |  |
|  | Ship stability update | Ship’s dynamic table |  |  |  |  |
|  | Heading/rudder & propulsion orders |  |  | Heading control system (HSC) Track control system Rate of turn indicator | --- |  |
|  | Set & drift Under-keel clearance Updated Vertical clearance (air draft) | Wind & current information water level salinity same |  | Sensors & instruments Track analysis Depth sounder Static draft Dynamic draft predictions ENC data Tide tables same | Broadcast Broadcast water level information same |  |
|  |  |  |  | GMDSS/NAVTEX | MSI (e.g, NAVTEX, SafetyNet, Radio) |  |
|  |  |  |  | AIS (binary data msg) | Radio Broadcast AIS Broadcast |  |
|  | Sea state | Wind, Wave height |  | Visual obs | Radio Broadcast AIS Broadcast |  |
| 3. Maintaining Situational Awareness | Route monitoring Collision Avoidance |  |  | [See #2 and #5 ] |  |  |
|  | Consider differences between various sensor outputs (e.g., comparison of multiple nav sensors) | Visual observation (traffic, AtoNs, etc.) Radar Observation (traffic AtoNs, etc. |  | [actively engaging the mariner] Appropriate Charts/Chart database ECDIS [ECDIS/ECS] Radar AIS GNSS Compass/Pelorus Electro-Optical (night vision, HSC) Speedlog Echo Sounder | Broadcast DGPS |  |
|  | Bridge Resource Management (BRM) | Voice Visual observation |  | Fatigue management plan | Training |  |
| 4. Operational Mode Awareness | Integrated Navigation Systems (e.g., INS & IBS) |  |  | IMO PS Integrated Bridge System Guidelines for |  |  |
|  | System-level integrity | Indications alarms |  | Integrated Navigation System, Centralized Alert Management, Bridge equipment design & layout |  |  |
|  | Information authentication | Indications alarms |  |  |  |  |
|  | Personnel status | BNWAS status |  | Bridge navigation watch alarm (BNWAS) |  |  |
|  | Bridge Resource Management (BRM) | Voice |  | Fatigue management | Training |  |
| 5. Collision avoidance | Relative bearing rate Range rate Relative bearing Relative position (Course of ship) Relative speed (STW) Heading of other Heading, ownship | Visual observation |  | Compass/pelorus Electro-optical (night vision, HSC) |  |  |
|  |  | Radar display |  | Radar, Speed Log (water track) | AIS |  |
|  |  |  |  | Compass, THD, TMHD |  |  |
|  |  |  |  | AIS |  |  |
|  | Visibility Condition | Visual/Radar |  | Radar Charts/ECDIS [ECDIS/ECS] Bridge team |  |  |
|  | Status of other vessel | Visual observation |  | Lights/day-shapes AIS Voice radio | AIS broadcast |  |
|  | Exchange/Confirm navigational intentions (Deviations necessary from COLREGS) | Voice comm |  | Voice VHF/ VHF-DSC AIS | AIS broadcast |  |
|  | COLREG, geographical position | Visual radar charted position |  | AIS Radar GNSS DGPS ECDIS [ECDIS/ECS] VHF radio |  |  |
|  | Avoiding contact with floating objects (i.e., icebergs) | Radar Visual |  | Radar Speed Log (water track) Compass, THD, TMHD AIS GMDSS VHF radio | AIS broadcast MSI (e.g., NAVTEX, SafetyNet) |  |
| 6. Emergency & SAR Management | Distress, Type |  | GMDSS [ RCC ] |  | Acknowledge |  |
|  | Position |  |  |  |  |  |
|  | Beacon |  |  |  |  |  |
|  | Piracy Event |  |  |  |  |  |
|  | Security Event |  | SSAS |  |  |  |
| 7. Reporting | Load Plan |  | VDR ECDIS [ECDIS/ECS] voyage log |  |  |  |
|  | Casualty Description |  |  |  |  |  |
|  | Ship Stability data |  |  |  |  |  |
|  | Voyage Plan |  |  |  |  |  |
|  | Statutory Reports, i.e.: - Report Hazard Matl - Port State control data - Local reporting schemes - SafeSeaNet reporting - Pre-arrival notice | Ship’s MSI# Position Arrival schedule (passage plan) | VHF/Voice Satcom AIS binary messages GNSS LRIT |  |  |  |
|  | Commercial reports, e.g. notice of readiness, engine monitoring, etc. |  | Satcom |  |  |  |
|  | Onboard logs - navigation sensors - ship systems, activity | Electronic Visual observations Measurements | VDR (Approved electronic logbook) Official log books (Rough logs, ISM checklist, GMDSS logbook, Oil record book Ballast log, etcetera) |  |  |  |
|  | Reporting dangers (ice, fixed and floating hazards, NavAid discrepancies ) | Visual observations Radar / AIS observations Echo sounder | GMDSS/voice radio Satcom (Email) GNSS |  |  |  |
|  | Weather reporting | METOC Position | GMDSS /radio Satcom (Email) Shipboard weather instruments Visual observation Published list of radio signals - ALRS, Sailing Directions |  |  |  |

6. Presentation/Display of e-Navigation Information

Shore-based

At present, there are no specific standards related to the presentation/display of shore-based information (e.g., at a VTS Centre).  Potentially, some of the standards that apply to the display of shipborne information may be suitable for shore-based displays as well.

Shipborne

There are a number of general and equipment-specific international standards that have been adopted by IMO, IHO, and IEC that contain “guidance” related to the presentation/display of shipborne navigation-related information.

General

- Performance Standards for the Presentation of Navigation-related Information on Shipborne Navigational Displays, IMO Res. MSC.191(79), 6 December 2004.

- Guidelines for the Presentation of Navigation-related Symbols, Terms and Abbreviations, IMO SN/Circ.243, 15 December 2004.

- Presentation of Navigation-related Information on Shipborne Navigational Displays – General Requirements, methods of testing, required test requirements. IEC 62288, Edition 1.0, July 2008.

Equipment Specific (only a partial list)

- Revised Performance Standards for ECDIS, IMO MSC.232(82), 2006.

- Specifications for Chart Content and Display Aspects of ECDIS, IHO S-52, Ed. 4.2, March 2004.  (also includes: IHO S-52, Appendix 2, Colour and Symbol Specifications for ECDIS, March 2004.)

- Performance Standards for Radar Equipment, IMO MSC.192(79), 2004.

- Performance Standards for a Universal Shipborne Automatic Identification System (AIS), IMO Res. MSC 74(69), Annex 3, 19 May 1998.

- Guidelines for the Onboard Operational Use of Shipborne Automatic Identification Systems (AIS), IMO Res. A.917(22), 25 January 2002.

- Display of AIS Target Information, IMO SN/Circ. 217.

- Guidance of the Application of AIS Binary Messages, IMO SNCirc.236, 28 may 2004.

- Performance Standards for an Integrated navigation System (INS), IMO Res. 86(70), Annex 3.

- Integrated Navigation Systems (INS) - Operational and performance requirements, methods of testing and required test results.  IEC 61294, Ed. 1,

- General requirements for shipborne radio equipment forming part of the GMDSS, and for electronic navigational aids, IMO A.694(17), 1991.

7. Harmonization Challenges and Opportunities

Currently, the performance standards and operation of most ashore and shipborne equipment/systems have been developed independently of one another.  As such, there is a need (or challenge) to describe how it will be possible to achieve the goal of e-Navigation:

“the harmonized collection, integration, exchange, presentation, and analysis of marine information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment.”

a) Shore-based considerations

The exchange of information required for berth-to-berth navigation would benefit from global standards and protocols. These standards and protocols should be in an open form so everybody can share and make use of them.  But, it is also important that they are appropriate and not subject to constant change. It is the increase in traffic density along with the diversity and enlargement of the shipping fleet that has increases the need for more reliable and up-to-date information. This information can still be presented with different symbols by different users, as long as the information content is exactly the same.  But, there is also a need to consider a mutual compatibility and interoperability of equipment, systems, symbolism and operational procedures. We need to articulate a viable and coherent framework to achieve a common and consistent format to avoid potential conflicts between users.

The first step will be to get consistent global standards and protocols.  The next challenge will be to decide on the best process for collection, integration, simplification, presentation and analysis of marine information onboard and ashore by electronic means.

b) Shipborne

Initially, there will significant challenges associated with trying to reconcile the existing shipborne equipment/system performance standards such that they conform to an e-Navigation Concept of Operation.   One way to start this harmonization process is to develop and agree upon some guiding principles.

Guiding Principles

Goal:  	Unless there is a compelling need not to do so, both shore-based and shipborne equipment/systems/services should provide information content to all users (ashore and shipborne) that is uniform or consistent in presentation/display.

General

a) There may be a need to develop new international and supra-national regulations and guidelines.

b) Potentially, there should be some “guiding principles” that are followed related to the information provided by shore/shipboard marine navigation systems.  These include:

- Consistency/uniformity -- yet flexible to allow for innovation.

- Presentation/display ashore and onboard vessels (i.e., is there a need to be different?)
- Ergonomics/human factors must be considered (e.g., too much information or cluttered display can be worse than too little information)

- Education and training of personnel – both shore-based and shipborne – will be crucial.

c) The proposal to the IMO work program on e-Navigation defines seven key components of a safe and comprehensive e-navigation policy.  These include:

1. Electronic chart and weather information

2. Electronic positioning signals

3. Electronic information on vessel route, course, maneuvering, etc.

4. Transmission of positional and navigational information

5. Display of information

6. Information reporting, prioritization and alert capability

7. Transmission of distress alert and maritime safety information

In order to be able to exchange and communicate maritime information between different systems, ashore and shipborne, new global standards may need to be developed.

There may also be a need to design an infrastructure that would enable authorized information to be transferred between shore authorities, and other parties with opportunities for improving the efficiency of transports, logistics and distributions.

8. Recommendations on Way Forward

Shore-based

Pursue a holistic approach of global standardization and protocol.  Specifically:

- Enhance the VTS through better provision, coordination, and exchange of comprehensive data in support of vessel safety and efficiency.

- Create a global combined integration of exchanging information within e-Navigation and therefore the VTM. The VTM concept must continue to facilitate global coverage, and continue to decrease the diversity of port services by co-ordination of allied services.

- Enhance the user-friendliness with the goal of reducing human error, while preventing distraction and overburdening by trying to reducing the range of reporting obligations on the ship-owner and ship master.

- Enhance and combine education and training for those authorities involved in the management of maritime transport.

- Continue to articulate a viable and coherent framework which sets out to achieve a common and consistent format for the use of electronic technologies.

- Keep the e-Navigation concept to be evolutionary and dynamic so that it will continue to develop as new user needs arise, and the emerging technological opportunities evolve together.

Shipborne

1. Review existing performance standards for shipborne equipment/systems/services that have been adopted by IMO, IEC, IHO, ISO, etc.  The focus should be on those that are e-Navigation-related.  Particular attention should be made to the recent IMO and IEC performance standards regarding the “Presentation of Navigation-related Information on Shipborne Displays”.

2. Compile a list of the various performance standards that pertain to e-Navigation.  This listing/review should include specific mention of any inconsistencies and/or conflicts noted that may inhibit the implementation and conduct of e-Navigation.

3. Develop a Concept of Operation/Best Practices for Shipborne e-Navigation.  The intent is to explain what needs to be done to harmonize the wide variety of equipment, systems, and services in order to achieve the goal of e-Navigation.

4. Provide recommendations to the IALA e-Navigation Committee IMO in regard to how e-Navigation can be implemented within the context of existing international performance standards.  Ultimately, this may include recommendations to IMO for potential new standards/guidance specific to achieving the eleven (11) Core Objectives of e-Navigation.

_____________________________

ANNEX A - Glossary of E-Navigation-related Equipment, Systems, and Services

Scope/goal: 	Describe the variety of shore-based and shipborne marine navigation systems that fall under the e-Navigation concept of operation.

Shore-based

Equipment - Navigational devices, sensors or equipment that are [or could be] used for navigation.  This includes both [analog] and electronic equipment?

GMDSS (Global Maritime Distress Safety System) is based on an international agreement regarding required types of equipment (carriage) and communication protocols used to increase safety and make search and rescue (SAR) rescuing easier.

Radar/ARPA (Automatic Radar Plotting Aid) is a computer assisted radar data processing system. The system creates tracks using radar contacts to calculate the objects course, speed and CPA to avoid danger of collisions.

AIS (Automatic Identification System) is a supplement to radar that automatically provides other ships and coastal authority with information about the ship’s dynamic (e.g., speed), static (e.g., identity), and voyage-related (e.g., destination). This information is broadcast via the VHF net.

ECDIS  (Electronic Chart Display and Information System) is a computer-based navigation information system which can be used in lieu of paper nautical chart. This shipborne navigational display provides continuously integrated position information from GPS, and other navigational sensors in addition to navigation-related information. All true ECDIS meets IMO`s requirement, and use vector chart with standardized content, structure and format.

CCTV (Closed-Circuit Television) is the use of video cameras to transmit a signal to a specific, limited set of monitors.

VHF Voice Radio - ?

[Others?]

System

LRIT (Long Range Identification and Tracking system). The most important distinction between LRIT and AIS is the range, and that LRIT only will be available to the recipients who are entitled to receive the information.

SSN (SafeSeaNet) is a reporting system for all ships approaching and departing Norwegian harbors.

GNSS (Global Navigation Satellite System) is the standard generic term for the global satellite navigation systems. The only fully functional GNSS in the world is NAVSTAR GPS (Global Position System), developed and operated by United States. To enhance the GPS we have DGPS (Differential GPS) which can provide the accuracy down to centimeters.

Navigational Warnings (such as unlit lights, drifting objects wrecks, etc.)

Service

VTS (Vessel Traffic Service) provides information service, navigational assistant’s service, and    traffic organization service. Immediately might the role of VTS change and expand if it should be incorporated in a concept on a global level, Vessel Traffic Management.

VTM (Vessel Traffic Management) is the structured framework of strategic, tactical and operational efforts to maximize the safety, security and efficiency of vessel traffic in interaction with its operating environment worldwide. Here we have VTS, pilotage, legislation, routing and reporting means.

MRCC (Maritime Rescue Coordinated Center) coordinates SAR operations.

Oil polluting response.

Computer systems such as Dynamic MaRA. Dynamic MaRA is used in specific areas with a high environmental vulnerability and difficult weather conditions.

Maritime Coastal Radio stations coordinate information between ship and shore.

Weather Service (e.g., meteorlogic/hydrographic information including wind, wave, current flow, visibility, ice, etc.).

Sea Ice (ice coverage, types of ice, icebergs, etc.)

Shipborne

Equipment

Definition - Navigational devices, sensors or equipment that are directly related to providing information that can be used to enhance navigation and decision-making onboard vessels.

Radar/ARPA – Nick Ward

Satellite-based (e.g., GNSS) – Nick Ward

Terrestrial (e.g., eLORAN) – Nick Ward

ECDIS/ECS – Lee Alexander and Michael Bergmann

AIS [check with AIS WG]

- e.g., AIS Binary Messages

System [need definition]

INS/IBS – [check with Florian Motz]

Automated Reporting Systems – [check with Michael Bergmann]

- e.g., SSN (SafeSeaNet) is a reporting system for all ships approaching and departing European harbors.

[others?]

Service	  [use what is listed in NAV54/WP.2, Annex 1, Section 8 (MIS)

Maritime Information Systems [& Services]:

8.1 SOLAS chapter V, regulation 13, requires Contracting Governments to provide navigational information to mariners. Regulation 13, paragraph 3 states: “Contracting Governments undertake to arrange for information relating to aids-to-navigation to be made available to all concerned. Changes in the transmissions [i.e., status report] of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.”

8.2 The promulgation of information on navigational safety is coordinated by means of the World-Wide Navigational Warning Service that was established jointly by IMO and IHO in 1977.

8.3 The World-Wide Navigational Warning Service is administered through 21 NAVAREAS5. Each NAVAREA has an Area Coordinator who is responsible for collecting information, analyzing it, and transmitting NAVAREA Warnings.

8.4 Lists of aids to navigation are produced by (or for) most national authorities as part of the navigational information made available to mariners in support of SOLAS chapter V, regulation 13.

8.5 The joint IMO/IHO/WMO Manual on Maritime Safety Information (IMO COMSAR/Circ.15) provides definitions of standard terms to describe particular events that should be used when composing navigational warnings. The terms that are relevant to the condition of aids to navigation are defined in the circular.

Processes

Marine Information Overlays (MIOs) - Lee Alexander

- static (	IHO S-57  IHO S-100)

- dynamic (AIS Binary Messages)

Types of e-Navigation Related Data and Information

[Add section describing how the (U)MDM could be used]

It is important to make a clear distinction between “information” and “data.”  “Information” includes voice, text, fax, graphical, geo-spatial (map or chart), etc.  If/when this information is converted into another format for digital transmission, it becomes “data” (e.g., NMEA 0183 or AIS binary message).  However, to be relevant to a mariner onboard a vessel this data must eventually be converted back into a presentation/display of “information” that can be used for situational awareness or decision-support.

Shipborne

Electronic chart data

vector (e.g., IHO S-57 ENCs, SENC Direct Distribution)

raster (e.g., UKHO Admiralty Raster Chart Service, or ARCS)

MIOs (Marine Information Overlays)

Static (e.g., IHO S-57)

Dynamic (e.g., AIS Binary Messages)

Working Paper  - IALA e-NAV7 Committee Meeting, 20-25 Sept, St. Germain en Laye, France, Version 1 – 28 August 2008, Version 2 – 18 September 2008, Version 3 - 13 March 2009, Version 4 – 25 March 2009, Version 5 – 23 September, 2010.

See “Proposed Strategy for the Development and Implementation of e-Navigation” (NAV 54/WP.2, 11 June 2008), Annex, Section 5, p.3-4)