Liaison Note to IMO Correspondence Group on e-Navigation

Maritime Information Systems (MIS) as a Component of

e-Navigation

# Introduction

This Paper addresses what constitutes “Maritime Information Systems” as a component of e-Navigation.  It describes a variety of shore-based and shipborne equipment, systems, services and processes that, collectively, can be considered to fall under the scope of e-Navigation. In particular, a series of tables are provided that lists the tasks/functions, information needed, shipborne and shore-based sources of information, the communication means to provide this information, and VTS-related information to be shown onboard.

# Objectives

The two main purposes of this document are to:

1. Contribute to the work of the IMO Correspondence Group on the Gap Analysis process.

2. Be considered by the IMO/IHO Harmonization Group on Data Model.

# Background

As stated in the IMO “Strategy for the Development and Implementation of e-Navigation” (MSC85/26, Annex 20), the general expectations for the onboard, onshore and communications elements of e-Navigation are:

- Onboard:

Navigation systems that benefit from the integration of own ship sensors, supporting information, a standard user interface, and a comprehensive system for managing guard zones and alerts. Core elements of such a system will include, actively engaging the mariner in the process of navigation while preventing distraction and overburdening,

- Ashore:

The management of vessel traffic and related services from ashore enhanced through better provision, co-ordination, and exchange of comprehensive data in formats that will be more easily understood and utilized by shore-based operators in support of vessel safety and efficiency, and

- Communications:

An infrastructure providing authorized seamless information transfer onboard ship, between ships, between ship and shore and between shore authorities and other parties with many related benefits, including a reduction of single person error.

Since onboard and ashore elements of e-Navigation are closely related, they will be considered together.  In particular, communications applies to both elements (ashore and shipborne) since it is the means to communicate information from shore-to-ship, ship-to-ship, and ship-to-shore (4S Communications).

Documents Supporting Maritime Information Systems [& Services]:

- SOLAS chapter V, regulation 13, requires Contracting Governments to provide navigational information to mariners. Regulation 13, paragraph 3 states: “Contracting Governments undertake to arrange for information relating to aids-to-navigation to be made available to all concerned. Changes in the transmissions [i.e., status report] of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.”

- The promulgation of information on navigational safety is coordinated by means of the World-Wide Navigational Warning Service that was established jointly by IMO and IHO in 1977.

- The World-Wide Navigational Warning Service is administered through 21 NAVAREAS. Each NAVAREA has an Area Coordinator who is responsible for collecting information, analyzing it, and transmitting NAVAREA Warnings.

- Lists of aids to navigation are produced by (or for) most national authorities as part of the navigational information made available to mariners in support of SOLAS chapter V, regulation 13.

- The joint IMO/IHO/WMO Manual on Maritime Safety Information (IMO COMSAR/Circ.15) provides definitions of standard terms to describe particular events that should be used when composing navigational warnings. The terms that are relevant to the condition of aids to navigation are defined in the circular.

# Shore-based Information Needs

As described in IMO Resolution A.857(20), there are three primary tasks of present and future VTS’s and related shore-based organizations:

- Information Service (INS).  An Information Service provides essential and timely information to assist the on-board decision-making process. An Information Service does not participate in on-board decision-making;

- Traffic Organisation Service (TOS).  A Traffic Organisation Service is a service to provide for the safe and efficient movement of traffic and to identify and manage potentially dangerous traffic situations.  A Traffic Organisation Service provides essential and timely information to assist the on-board decision-making process and may advise, instruct or exercise the authority to direct movements;

- Navigational Assistance Service (NAS).  A Navigational Assistance Service may be provided in addition to an Information Service and/or Traffic Organisation Service. It is a service to assist in the on-board navigational decision-making process and is provided at the request of a vessel, or when deemed necessary by the VTS. A Navigational Assistance Service provides essential and timely navigational information to assist the on-board decision-making process and may inform, advise and/or instruct vessels accordingly.

Supportive Services may additionally be provided by VTS to support other shore-based authorities, stakeholders or services, i.e.:

- Allied Services (Logistic)

  - includes nautical services (e.g., linesmen)

- Remedial services (SAR & Incident Management)

- Law enforcement

- Security

- and other services

Tables 1 to 4 provide a preliminary description of elements pertaining to each category of services.

# TABLE 1.  Information Service (INS) [reference  IMO Resolution A.857(20)]

| Task/Functions | Information Needed | Typical Source Shipborne | Typical Source Ashore | Communication means |
| --- | --- | --- | --- | --- |
| Maintaining a traffic image | IMO Ship identity: Ship’s name MMSI number Call sign IMO number | AIS, radio voice communications AIS, LRIT, radio voice communications AIS, Radio voice communications AIS, Radio voice communications | S-AIS, AIS base station or network, radio voice communications, electro-optical equipment S-AIS, AIS base station or network, LRIT, radio voice communications (DSC) S-AIS, AIS base station or network AIS base station or network, radio, databases In general, most ship’s IDs can be obtained from several sources, including other VTSs. | VHF/MF/HF SATCOM Broadband mobile phone S-AIS, AIS |
|  | Other ship identities: Ship’s name (native and international) MMSI number Call sign | AIS, radio voice communications AIS, LRIT, radio voice communications AIS, radio voice communications | S-AIS, AIS base station or network, radio voice communications, electro-optical equipment S-AIS, AIS base station or network, LRIT, radio voice communications (DSC) S-AIS, AIS base station, Radar In general, most ship’s IDs can be obtained from several sources, including other VTSs. | VHF/MF/HF SATCOM Broadband mobile phone S-AIS, AIS |
|  | Ship positions: - Time stamp - Time reference | GNSS (Inertial Navigation System – INS) AIS position(msg 17 available and INS no differentially correct) Geonavigation | S-AIS, AIS base station or network, LRIT AIS base station or network AIS base station or network Voice, other reports Other Radio-direction finder Radar Visual Electro-Optical | VHF/MF/HF SATCOM Broadband mobile phone S-AIS, AIS |
|  | Voyage plan related information (Mandatory reporting system) | Email Web service Radio voice communications AIS Fax | Email Web service Radio voice communications Agent (User interface needed for agents to update HAZMAT, contact details) AIS base station or network Fax | VHF/MF/HF SATCOM Broadband mobile phone S-AIS, AIS |
|  | Ship Navigational Status (Mandatory reporting system) | AIS Radio voice communications Visual signals (lights, shapes, flags) | S-AIS, AIS base station or network Radio voice communications Agent (User interface needed for agents to update HAZMAT, contact details) Visual observation, electro-optical equipment | S-AIS, AIS VHF/MF/HF SATCOM Broadband mobile phone Visual Telephone |
|  | Cargo information (Mandatory reporting system) | Radio voice communications AIS Visual signals (lights, flags) Email Web service Fax | Agent (User interface needed for agents to update HAZMAT, contact details) S-AIS, AIS base station or network Visual observation, electro-optical equipment Email Web service Fax | VHF/MF/HF SATCOM Broadband mobile phone Visual S-AIS, AIS |
|  | Number of persons on board (Mandatory reporting system) | Radio voice communications AIS Email Web service Fax Visual signals ( flags) | Agent Radio voice communications S-AIS, AIS base station or network Email Web service Fax Visual observation, electro-optical equipment | VHF/MF/HF SATCOM Broadband mobile phone S-AIS, AIS |
|  | Pilot Onboard | Radio voice communications AIS Email Web service Fax | Agent Radio voice communications S-AIS, AIS base station or network Email Web service Fax | VHF/MF/HF SATCOM Broadband mobile phone S-AIS, AIS |
|  | Ship data |  | Databases Web service Additional local service | Broadband |
|  | Chart data |  | Chart data and updates Notice to mariners | Broadband Paper format |
|  | Meteorological , oceanographic and hydrological conditions |  | hydro/meteo sensors Web service | Broadband AIS base station VHF mobile phone |
|  | Navigation safety warnings (eg. fairway information, obstructions, infrastructure out of order, AtoN off-station) | GMDSS AIS Radio voice communications | GMDSS WWNWS AIS base station or network Radio voice communications | Broadband Paper format VHF/MF/HF SATCOM S-AIS, AIS |
| Provision and Exchange of Information | VTS information to mariner: - METOC - Navigation hazard - Traffic situation - Ice report - Recommendation route - Special areas - Special operations - Piracy reports - Georeferenced data - Pilotage information - VTS track information - Etc. | Radio voice communications AIS Web service Fax | Radio voice communications AIS base station or network Web service Fax GMDSS | Broadband Paper format VHF/MF/HF SATCOM S-AIS, AIS |
|  | Request from VTS by Master: - METOC - Navigation hazard - Traffic situation - Ice report - Recommendation route - Special areas - Special operations - Piracy reports - Georeferenced data - Etc. | Radio voice communications AIS Web service Fax | Radio voice communications AIS base station or network Web service Fax GMDSS | Broadband Paper format VHF/MF/HF SATCOM S-AIS, AIS |
| Incident mitigation | In addition to traffic image infos: Safety incident report SAR incident report Security incident report Contingency plan Pollution incident report Alert Refugee place | AIS GMDSS Email Fax | AIS base station or network GMDSS Surveillance system Email Web service Operational data links Databases Fax | Broadband VHF/MF/HF SATCOM Paper format Mobile phone S-AIS, AIS |

# TABLE 2.  Traffic Organization Service (TOS), in addition to INS.

| Task | Information Needed | Typical Source Shipborne | Typical Source Ashore | Communication means |
| --- | --- | --- | --- | --- |
| Strategic planning | (long term) sailing plan |  | Agent | Ashore-ashore interfaces |
|  | ship database |  | e.g. Lloyds, VTS database | Ashore-ashore interfaces |
|  | berth planning |  | Port Administration | Ashore-ashore interfaces |
|  | route structure (traffic separation scheme) |  | Transport Administration | Ashore-ashore interfaces |
|  | Cargo information policy |  | Transport Administration | Ashore-ashore interfaces |
| Route adherence monitoring | Specific sailing plan |  | Radio voice communications | Ashore-ashore interfaces |
|  | Cargo information and risk profile |  | HAZMAT, Risk analyses (Check the completeness of IMO Guidelines) Review existing standards in this area | Ashore-ashore interfaces |
|  | traffic clearances (passing of waypoints) | VHF ECDIS/ENC Radar Visual electro-optical AIS | Radio voice communications Radar Visual electro-optical ENC AIS base station or network | VHF/MF/HF SATCOM Mobile phone S-AIS, AIS Radar bands |
| Traffic separation (operations) | Processing of information according to rules | VHF Visual electro-optical | Radio voice communications, switching of traffic lights | VHF/MF/HF Mobile phone |
| Coordination with allied services | Vessel database, Allied Services database, sailing plan | Agent, Allied Services, Lloyds |  | Ashore-ashore interfaces |

# TABLE 3.  Navigational Assistance Service (NAS), in addition to INS

Miscellaneous Issues

| Task | Information Needed | Source | Requirements |
| --- | --- | --- | --- |
| Vessels in VTS area to have adequate charts | Automated request from VTS Centre to Vessel for ENC chart version | Chart: Hydrographic office or commercial provider | Automated check that version of local ENC chart on board is adequate (future e-Nav) |
| Vessels in VTS area to have correct AIS data, if not, then report to responsible authority | AIS Static and voyage related data | Vessel AIS system |  |
| Navigation Safety Advisory | Tide info (static/dynamic, see environmental data) | AIS binary GMDSS |  |

# TABLE 4. Supportive Service

| Task/Functions | Information Needed | Source | Communication means | Requirements (in addition to V-128) |
| --- | --- | --- | --- | --- |
| Allied Services (Maintaining logistics) | Berth Availability | VTS Data | Ashore-ashore interfaces |  |
|  | Departure Time (ETD) | VTS Data | Ashore-ashore interfaces |  |
|  | Port Facilities | VTS Data | Ashore-ashore interfaces |  |
|  | ISPS | VTS Data | Ashore-ashore interfaces |  |
|  | Pilot | VTS Data | Ashore-ashore interfaces |  |
|  | Berth Availability | VTS Data | Ashore-ashore interfaces |  |
|  | Departure Time (ETD) | VTS Data | Ashore-ashore interfaces |  |
|  | Tug information | VTS Data | Ashore-ashore interfaces | Position & availability |
|  | Linesmen | VTS Data | Ashore-ashore interfaces |  |
|  | Regular Traffic | VTS Data | Ashore-ashore interfaces | Schedules services in the area such as ferries |
| Provide traffic image to allied services | Surface picture of vessels and their movements in the VTS area to be transmitted | VTS Data | Ashore-ashore interfaces | Internal storage |
| Archive and replay of traffic image & concurrent data (voice, CCTV, met) |  | VTS Data | Ashore-ashore interfaces |  |
| Coordination with Allied Services | SAR incident reports: - passenger/crew list; - passenger tracking systems data; - hazardous cargo; - evacuation plans; - ships plans; - number of lifeboats/liferafts/life saving appliances (Survival suits); - EPIRB/SART and AIS/SART data; - SAR- co-operation plan data for passenger ships; and - Ship's IMO number. - Information about vessels in a SAR area that informs shore stations on the ship's communications options available. - Information on a vessel's capabilities for supporting SAR operations. | VTS Data | Ashore-ashore interfaces | SAR authority, Coastguard |
|  | Pollution incident reports (Mechanism to escalate efficiently to responsible parties) | VTS Data | Ashore-ashore interfaces | Pollution Authority & Vessels |
|  | …many others not listed… | VTS Data | Ashore-ashore interfaces | All VTS equipment |

A summary of the information that should be part of the ship’s database is presented in Table 5. These information aims to identify or define the minimum fields required to deliver global consistency.

# TABLE 5. Ship database

| Description | Data Form |
| --- | --- |
| MMSI number | Std |
| Ship Name |  |
| Ship Native Name |  |
| IMO number |  |
| Flag State |  |
| Ship owner |  |
| Length |  |
| Beam |  |
| Draught |  |
| Air Draught |  |
| Type |  |
| Gross Tonnage |  |

| User requirement | Data Sources | Typical Source Ashore | Communication means | Comments |
| --- | --- | --- | --- | --- |
| Position | AIS, VHF, GMDSS | S-AIS, AIS, radar, visual, VHF, LRIT | VHF Radar bands Satcom | AIS could also be from space |
| Course | AIS, VHF, GMDSS | S-AIS, AIS, radar | VHF Radar bands Satcom |  |
| Speed | AIS, VHF, GMDSS | S-AIS, AIS, radar, VHF | VHF Radar bands Satcom |  |
| Heading | AIS, VHF, GMDSS | S-AIS, AIS,VHF | VHF Satcom |  |
| Draught | AIS, VHF, | S-AIS, AIS, radar, VHF, agent /email, pilot, reporting systems | VHF Radar bands Satcom Telephone, broadband |  |
| Air draught | AIS, VHF | S-AIS, AIS, VHF, agent, pilot | VHF Satcom Telephone, broadband |  |
| Flag | AIS, VHF | AIS, VHF, Databases | VHF Ashore-Ashore interfaces |  |
| Destination | AIS, VHF, Satcom/email | S-AIS, AIS, VHF, web/email reporting- and agent systems | VHF Telephone broadband |  |
| Arrival Time (ETA (new)) | AIS, VHF, Satcom/email | AIS, VHF, agent | VHF Telephone broadband |  |
| Ports of call | AIS, VHF, Satcom/email | AIS, VHF, agent | VHF Telephone broadband |  |
| ROT | AIS, | AIS | VHF |  |
| ISPS status | VHF, Satcom/email | VHF, web/email | VHF Telephone broadband |  |
| Ship Status | AIS, VHF, Satcom/email | S-AIS, AIS, VHF, web/email visual, electro-optical | VHF Telephone broadband |  |
| Passage Plan | VHF, Satcom/email | VHF, web | VHF Telephone broadband |  |
| HAZMAT | VHF, Satcom/email | S-AIS, AIS, VHF, web | VHF Telephone broadband |  |
| Marine Pollution | VHF, Satcom/email | S-AIS, AIS, VHF, web | VHF Telephone broadband |  |
| Pilot Status | VHF, mobile phone | VHF | VHF Telephone Mobile phone |  |
| Under-keel Clearance | AIS, VHF, Satcom/email | S-AIS, AIS,VHF | VHF Telephone broadband |  |
| CCTV | VHF, Satcom/email, | VHF, agent, web | VHF Telephone broadband |  |
| Radar Data | VHF, Satcom/email | VHF, agent/email, web | VHF Telephone broadband | From ship, shore &space |
| Rebroadcast AIS | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband | From ship |
| Statutory Information | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband |  |
| Paris MOU status | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband |  |
| MET/HYD & wind information | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband |  |
| NAVAREA (new) | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband | warnings (e.g., unlit lights, drifting objects, wrecks, etc.) |
| Limitations such as local regulations and legislations. (new) | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband |  |
| SAR (Search and Rescue) information (new) | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband |  |
| GEO(Group on Earth Observations)-special information such as sandbanks. (new) | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband |  |
| Oil polluting information (new) | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband |  |
| Dynamic MaRA information based on coastal vulnerability and weather conditions. (new) | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband |  |
| Voyage planning (VP) information such as fairway availability, obstruction and events following the traffic behavior. (new) | AIS, VHF, GMDSS | VHF, Telephone, web/email | VHF, Telephone broadband |  |
| # persons aboard | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband |  |
| Quarantine status | VHF, Satcom/email | VHF, Telephone, web/email | VHF, Telephone broadband |  |

Logistics

| User requirement | Typical Source Shipborne | Typical Source Ashore | Communication means | Comments |
| --- | --- | --- | --- | --- |
| Port Facilities | VHF, Satcom/email | Agent – email/phone | VHF, Telephone broadband | Ex. Bunker, crane, waste |
| ISPS | VHF, Satcom/email | Agent – email/phone | VHF, Telephone broadband |  |
| Pilot | VHF, Satcom/email | Agent – email/phone | VHF, Telephone broadband |  |

# Shipborne Information

The following identification is based on the primary tasks of present and future shipborne navigation systems in order to optimize ship operations in terms of safety and security, and protection of the environment.

- Route Planning/Grounding avoidance [ref. Res A.893(21) ]

- Route Execution

- Maintaining Situational Awareness

- Operational Mode Awareness

- Collision Avoidance

- Emergency & SAR [ ref MSC/Circ.1061 ]

SOLAS Conference 29.11.1995 ’Decision support system for masters on passenger ships (SOLAS/Conf.3/46, Annex, page 14, regulation 24.4). Operation of Integrated Navigation, Control and Communication systems should be considered in the following procedures:

- Blackout;

- Fire;

- Stranding;

- Collision;

- Man-over-board situations;

- Unlawful acts threatening the safety of the ship and the security of its passengers and crew;

- - Emergency assistance to other ships

- Reporting [ ref. Res A.916(22) ]

- Special conditions (berthing, anchoring)

Details related to each task are presented in table 6.

# Reference documents

IMO Res A.893(21) Guidelines For Voyage Planning

IMO Res A.916(22) Guidelines for the recording of events related to navigation.

IMO SOLAS V/19    Carriage requirements for shipborne navigational systems and equipment

IMO MSC Circ. 1061 Guidance for the Operational Use of IBS

IMO SN.1/Circ.288  Guideslines for Bridge Equipment and Systems, Their Arrangement and Integration (BES)

IMO SN.1 Circ.265 Guidelines on the Application of SOLAS Regulation V15to INS, IBS and Bridge Design

ISO 8468 Ship’s bridge layout and associated equipment. Requirements and guidelines

ISO 14612 Additional requirements and guidelines for centralized and integrated bridge functions

STCW 	International Convention on Standards of Training, Certification and Watchkeeping for Seafarers

ISM Code International Safety Management Code

# TABLE 6. Shipborne Information

| Task/Functions | Information Needed | UMDS Information Type |  | Typical Shipborne Source | Typical External Source (Shore or other Ship ) |
| --- | --- | --- | --- | --- | --- |
| 1. Route planning/Grounding avoidance [IMO Res A.893] | Route, Navigational hazards, Bathymetry, Traffic lanes, Traffic separation scheme AtoN’s, Ports, etc. Under keel clearance (draft) Vertical clearance (air draft) | Charted information |  | ECDIS [ECDIS/ECS] Appropriate Charts/Chart database Chart updating/ECDIS [ECDIS/ECS] chart database update Routing charts Company orders Port state control | Chart provider Chart update service Notice to mariners MSI (e.g, NAVTEX, SafetyNet, Radio) Proprietary Routing services (Weather, other) |
|  | Climate information Ephemeris Currents METOC forecast Ice forecast | Tide prediction Water level forecast Wind forecast Published information or databases Updates to published info Marine Information Overlays |  | Publications or databases - Climatology - Almanac - Tide Tables - Coastal Pilots & Sailing Directions - List of aids to navigation GMDSS | Notice to mariners MSI (e.g, NAVTEX, SafetyNet, Radio) |
|  | Radio service information | Published information or database |  | List of radio stations & services (nautical pubs) |  |
|  | Environmental protection | Published information or database Updates to published info Appropriate Charts/Chart databases Marine navigation overlays |  | GMDSS | Notice to mariners MIOs Nautical pubs AIS |
|  |  |  |  | ECDIS [ECDIS/ECS] AIS |  |
|  | Ship characteristics (conditions) - static - dynamic | Draft Air draft Maneuver characteristics Speed table Etc. |  | Onboard Ship design documentation |  |
|  | Ship stability condition | Visual observations Measurements Calculations |  | Onboard management | Load plan |
|  | Other voyage constraints | Passage schedule, shore service needs, special cargo characteristics, etc. |  | Company orders | Port State control Berthing availability Pilot availability |
|  | Security issues | Security conditions, Warnings, precautions, procedures, etc. |  | GMDSS | Flag State Ship owners MSI (e.g. NAVTEX, SafetyNet, Radio) |
| 2. Route execution & monitoring | Voyage Plan | Voyage plan Appropriate Charts/Chart database |  | Chart or ECDIS [ECDIS/ECS] |  |
|  | Vessel position | Position sensors Radar fix, obs Visual fix, obs Chart databases |  | GNSS (e.g., GPS/DGPS), inertial sensors Radar Compass, binoculars, pelorus, ATON’s ECDIS Chart databases | ATON’s ENC databases |
|  | Sensor integrity/validity | Course Speed Heading |  | GNSS or calculated. SDME Gyrocompass, Mag compass |  |
|  | Time of day (HH:MM) | Time |  | GNSS, radio time broadcast Ship’s time system |  |
|  | VTS instructions | Position fix Waypoints |  | GMDSS/voice radio |  |
|  | Vessel parameter (squat) | Ship’s documentation Depth under keel Speed log |  | Ship’s data Speed log Echo sounder | Contracted services |
|  | Ship stability update | Ship’s dynamic table |  | Ship’s trim Heel |  |
|  | Heading/rudder & propulsion orders |  |  | Heading control system (HSC) Track control system Rate of turn indicator Rudder indicator | --- |
|  | Under-keel clearance | water level salinity |  | Depth sounder Static draft Dynamic draft predictions ENC data Tide tables | Broadcast water level information |
|  | Updated Vertical clearance (air draft) | water level salinity |  | Depth sounder Static draft Dynamic draft predictions ENC data Tide tables | Broadcast water level information |
|  | Set & drift | Wind & current information |  | Sensors & instruments (GNSS, heading sensors, speed log, radar) Track analysis | Broadcast NAVTEX Websites |
|  | Sea state | Wind, Wave height & period |  | Visual observation s | Radio Broadcast AIS Broadcast NAVTEX SafetyNet MSI Web services |
| 3. Maintaining Situational Awareness | Route monitoring Collision Avoidance |  |  | [See #2 and #5 ] |  |
|  | Consider differences between various sensor outputs (e.g., comparison of multiple nav sensors) Actively engaging the mariner | Visual observation (traffic, AtoNs, etc.) Radar Observation (traffic AtoNs, etc.) Onship position and velocity ECDIS/ENC AIS Depth |  | Appropriate Charts/Chart database ECDIS [ECDIS/ECS] Radar AIS GNSS Compass/Pelorus Visual/Electro-Optical (night vision, HSC) Speed log Echo Sounder Geonavigation | Broadcast DGPS VTS track data |
| 4. Operational Mode Awareness | Integrated Navigation Systems (e.g., INS & IBS) |  |  | IMO PS Integrated Bridge System Guidelines for |  |
|  | System-level integrity | Indications alarms |  | Integrated Navigation System, Centralized Alert Management, Bridge equipment design & layout |  |
|  | Information authentication | Indications alarms |  | Navigation equipment |  |
|  | Personnel status | BNWAS status |  | Bridge navigation watch alarm (BNWAS) |  |
| 5. Collision avoidance | Relative bearing rate Range rate Relative bearing Relative position (Course of ship) Relative speed (STW) Heading of other Heading, ownship | Visual observation |  | Compass/pelorus Electro-optical (night vision, HSC) |  |
|  |  | Radar display |  | Radar, Speed Log (water track) | AIS |
|  |  |  |  | Compass, THD, TMHD |  |
|  |  |  |  | AIS |  |
|  | Visibility Condition | Visual/Radar |  | Radar Charts/ECDIS [ECDIS/ECS] Bridge team |  |
|  | Status of other vessel | Visual observation |  | Lights/day-shapes AIS Voice radio | AIS broadcast VHF |
|  | Exchange/Confirm navigational intentions (Deviations necessary from COLREGS) | Voice comm. |  | Voice VHF/ VHF-DSC AIS | AIS broadcast |
|  | COLREG, geographical position | Visual radar charted position |  | AIS Radar GNSS DGPS ECDIS [ECDIS/ECS] VHF radio | AIS DGPS VHF |
|  | Avoiding contact with floating objects (i.e., icebergs) | Radar Visual |  | Radar Speed Log (water track) Compass, THD, TMHD AIS GMDSS VHF radio | AIS broadcast MSI (e.g., NAVTEX, SafetyNet) |
| 6. Emergency & SAR Management | Distress, Type |  | GMDSS |  | MRCC |
|  | Position |  |  |  | GMDSS |
|  | Beacon |  |  |  |  |
|  | Piracy Event |  |  |  |  |
|  | Security Event |  | Ship Security Alert System (SSAS) |  |  |
| 7. Reporting | Load Plan |  | VDR ECDIS [ECDIS/ECS] voyage log |  |  |
|  | Casualty Description |  |  |  |  |
|  | Ship Stability data |  |  |  |  |
|  | Voyage Plan |  |  |  |  |
|  | Statutory Reports, i.e.: - Report Hazard Matl - Port State control data - Local reporting schemes - SafeSeaNet reporting - Pre-arrival notice | Ship’s MSI# Position Arrival schedule (passage plan) | VHF/Voice Satcom AIS GNSS LRIT |  |  |
|  | Commercial reports, e.g. notice of readiness, engine monitoring, etc. |  | Satcom |  |  |
|  | Onboard logs - navigation sensors - ship systems, activity | Electronic Visual observations Measurements | VDR (Approved electronic logbook) Official log books (Rough logs, ISM checklist, GMDSS logbook, Oil record book Ballast log, etcetera) |  |  |
|  | Reporting dangers (ice, fixed and floating hazards, NavAid discrepancies ) | Visual observations Radar / AIS observations Echo sounder | GMDSS/voice radio Satcom (Email) GNSS |  |  |
|  | Weather reporting | METOC Position | GMDSS /radio Satcom (Email) Shipboard weather instruments Visual observation Published list of radio signals - ALRS, Sailing Directions |  |  |