e-NAV9

Agenda item	08 (WG 2)

Task Number	01, 02 and 03

Author(s)	E. Engler, R. Ziebold, M. Hoppe, J. Winell, J.-H. Oltmann,

under consideration of notices provided by further Swedish colleagues

Development of Maritime Integrated PNT System

# Summary

The aim of the input paper is the initiation of a discussion about the development of the maritime Integrated PNT System in the framework of e-navigation.

- Position Fixing Systems such as GNSS/DGNSS support the determination of horizontal or three-dimensional position (x,y,z) and the provision of time.

- An Integrated PNT System is more than a Position Fixing System because of the additional provision of navigation data such as heading, rate of turn, speed and course over ground, and possibly three-dimensional attitude data.

Both systems – the Position Fixing System as well as the Integrated PNT System - are the overlay of satellite based, ashore and aboard components, whose integrated use can ensure the accurate and reliable provision of output data to applications during all phases of vessel navigation.

An overall indication of accuracy and reliability for all PNT data is an identified user need of e-navigation. Reliability can only be achieved by the implementation of redundancy. An efficient and integrated use of sensors and services should be considered for the provision of PNT data as well as for related accuracy assessment (integrity monitoring).

Therefore the development of an Integrated PNT System should be considered as one main objective of e-navigation. For this purpose terms describing user needs and technical requirements should be further developed in detail regarding their meaning and to achieve a common understanding. The results should support the identification of next IALA activities dealing with conceptual and architectural design of an Integrated PNT System, preparation of Performance Standards for components, and creation/update of assigned recommendations and guidelines.

## Action arising from this document

The e-NAV Committee is invited to consider the establishment of an Integrated PNT System as a superior objective of e-navigation. A discussion should be promoted to identify the role and activities of IALA for the analysis and development process of an Integrated PNT System. The discussion results should be used to address and prioritise the working program of IALA.

## Related documents

- Related Documents

| Short Name | Title of Document |
| --- | --- |
| [IALA WWRNP] | IALA World Wide Radio Navigation Plan, Edition 1, December 2009 |
| [IALA R-121] | IALA Recommendation R-121: On the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz, Edition 1.1, December 2004 |
| [IALA R-129] | IALA Recommendation R-129: On GNSS Vulnerability and Mitigation Measures, Edition 2, December 2008 |
| [IALA R-135] | IALA Recommendation R-135: On The Future of DGNSS, Edition 2, December 2008 |
| [IALA Guidline No. 160] | IALA Guideline No 1060: On Recapitalisation of DGNSS, Edition 1, December 2008 |
| …. |  |

# Background

From references such as [IMO NAV 54/25 Annex 12], [IMO A.915(22)], [IALA R-129] and [IALA WWRNP]:

- Position fixing systems are identified as one strategic key element of e-navigation. Existing and future Global Navigation Satellite Systems (GNSS) like GPS, GLONASS and GALILEO are fundamental infrastructures for global determination of PNT data. Technical requirements on future GNSS are described in terms of accuracy, integrity, continuity and availability of positioning.

- IALA has introduced the term “integrated PNT device” to describe the on-board part of a maritime Integrated PNT System. The integrated PNT device is considered as “a device using any available IMO recognised radio navigation systems simultaneously to provide the best electronic position fix for the ship”. Following this definition, the outlined objective is primary focussed on the provision of position data to different applications.

- Additionally, terrestrial services are used or considered as candidates to improve performance (augmentation services: e.g. IALA Beacon DGNSS, AIS DGNSS, RTK) or to ensure backup functionality (backup services: e.g. eLORAN, R-Mode) respectively to GNSS. Due to their interoperability and compatibility these systems can be used alternatively or complementary for determination of position, navigation and time data.

The basic version of integrated PNT device can be interpreted as a single GNSS receiver or a multi-system radio navigation receiver in this context.

From references such as [IMO MSC.112(73)], [IMO MSC.113(73)], [IMO MSC.114(73)], [IMO MSC.115(73)], and [IMO MSC.233(83)]:

- Several performance standards for ship borne GNSS and DGNSS receivers were developed and approved by IMO in the last decade: GPS, GLONASS, DGPS and DGLONASS, combined GPS/GLONASS, and GALILEO.

Therefore, a logical consequence of this standardisation process is the preparation of Performance Standards for multi-system shipborne radio navigation receivers. This is initiated and promoted by UK by preparation of a new work item proposal to IMO MSC89.

From references such as [IMO NAV 54/25 Annex 12], [IMO NAV 56/WP5], [IMO A.915(22)], [IALA R-129] and [IALA WWRNP]:

- A more general approach of PNT data provision (see Fig. 1) should be discussed aiming at the fulfilment of the e-navigation user needs “reliability” and “indication of reliability”.

- An Integrated PNT System should be considered as overlay of satellite based, shore-based and ship-board components, whose integrated use ensures the accurate and reliable provision of position, navigation and timing data to applications during all phases of berth-to-berth vessel navigation.

- More attention should be given to cost-effective multi-sensor, multi-system and multi-service approaches for improved PNT data provision and for reliable accuracy assessment of PNT data (integrity monitoring) in real time.

- The ship-board part of the Integrated PNT System (named as Integrated PNT Module) should be considered as an important module for the ship-board Integrated Navigation System (INS). The PNT Module enables cross-evaluation of sensors and services on the one hand. On the other hand the crossover from data validity and plausibility checks to reliable estimation of achieved accuracy can be possible in real time

- An overall specification of accuracy assessment functionalities (algorithm, software) and shared responsibilities (ship-board, shore-side) should be preferred towards a harmonised meaning and understanding of specific integrity monitoring results.

- Integrated PNT System (dark grey: components in practice; light grey: additional components)

In chapter 3 first discussion topics are outlined to explain the preferred approach for an Integrated PNT System.

# Discussion

## Positioning Fixing System or Position, Navigation and Timing System

GNSS is described as a system to provide worldwide position, velocity and time determination for multimodal use [IMO A.915(23)]. An operational requirement outlines that future GNSS should enable shipborne equipment to provide the user with information on position, time, course and speed over the ground. But in Appendix 2 and 3 of the same document minimum maritime user requirements are only given for horizontal position. Otherwise the use of the item Electronic Position Fixing System (EPFS) [IMO NAV 56/WP5] implies that the scope of GNSS is rather the provision of position data than the provision of PNT data.

- Therefore it should be clarified, whether the main objective resulting from user needs would be either the provision of position data (position fixing system with aboard multi-system radio navigation receiver) alone or the combined provision of position, navigation and timing data (Integrated PNT System with on-board PNT Module).

- With respect to [IALA WWRNP] the robust electronic Integrated PNT system (with redundancy) should be considered as the more desirable objective:

- Indication and improvement of reliability of PNT data provided depend on the degree of measurement redundancy introduced. The increase of measurement redundancy correlates with increasing equipment costs at service and vessel site. Dependencies between characteristics of PNT data and their creation should be exploited to achieve cost-efficient as well as robust technical solutions.

- “Mariners have expressed a desire for systems to automatically assess the accuracy and integrity of … position fixing data … and other ship sensors to return a graphical indication of assessment” [IMO NAV 56/WP5]. This clarifies that the assessment task should be concurrently applied on position as well as navigation and timing data. An integrated approach of PNT data provision to applications should be preferred to achieve an optimised utilisation of sensors and services.

- As illustrated in Fig. 1 the PNT data provision can be achieved with several signal and data processing chains which differ in the:

- use of GNSS and DGNSS signals;

- usage of the portfolio of available shore-based services;  and

- processing techniques applied.

- A non-ambiguous assessment of PNT data accuracy can only be achieved, if the complete process of generation of PNT data is taken into account: An improved and robust assessment of PNT data accuracy requires as many uncorrelated processing chains as possible. A joint consideration and optimisation of shore-based and ship-board integrity functionalities should be preferred.

- Primary aim of a single GNSS receiver or a multi-system radio navigation receiver is the provision of position and timing data. More than one multi-system radio navigation receiver is necessary for the provision of navigation data.

- The use of more than one radio navigation receiver is required to enable plausibility, validity and simple reliability checks and to minimise the risk of equipment failure [IMO-MSC.86(70); IMO-MSC.252(83)]. Their raw data should be exploited for robust PNT data provision (basis functionality) and accuracy assessment of PNT data (integrity functionality).

- Therefore the combined use of multi-system radio navigation receivers, related services and additional aon-board sensors should be preferred towards robust and reliable PNT data provision.

## Conceptual and architectural development of the Integrated PNT System

### Discussion of safety-relevant items

The enhancement of berth to berth navigation and related services [IMO NAV 54/25 Annex 12] requires the harmonised and integrated use of satellite based, shore-based and ship-board systems for robust PNT-data provision (basis functionality).

A wide spectrum of different technical and common requirements is outlined in several IMO and IALA documents considering the safety-critical nature of PNT-data. There is a demand to add precision to the safety-relevant requirements as follows:

- PNT-data should be provided with an accuracy which varies depending on the operation area and the planned manoeuvre (ocean, coast, and port; docking) [IMO A.915(22)]. An automatic assessment of the accuracy as an identified users need [IMO NAV 56/WP5] requires the implementation of integrity functionalities at each contributing system and a coordinated integrity management of the applied complete processing chain.

- One important objective of the integrity functionality is the estimation whether the accuracy requirements are fulfilled during run-time. Such an assessment requires measurement redundancy. The type of measurement redundancy implemented determines which error sources can be detected. Dublicated devices operating with the same measurement technique and under identical conditions will provide output data with comparable error behaviour. This redundancy allows primary the detection of equipment failure. The fulfilment of accuracy requirements can only be assessed based on a reliable determination of the provided output data’s error size under consideration of the complete applied PNT processing chain (integrity functionalities and joint integrity management).

- In the maritime community integrity is defined as the ability to provide users with warnings within a specified time when the system should not be used for navigation [IMO A.915(22), IALA R-121]. To achieve such a decision process requires the introduction of performance key identifiers and the specification of tolerable value ranges (thresholds). Procedures for the determination of performance key identifiers as well as for the utilisation inside the decision process should be specified to achieve comparable assessment results.

- The assessment of integrity is outlined as a further users need [IMO NAV 56/WP5]. The assessment of integrity means that failures of the integrity functionality should be detected. A failure of integrity functionality exists, if the system is flagged as unusable although the real error size is smaller than the allowed total error threshold. A more dramatic failure of integrity functionality occurs, if the system is signed as usable although the error which occurs exceeds the allowed total error threshold. It should be clarified, whether assessment of integrity means either the provision of integrity functionality or the monitoring of correctness of integrity functionality.

- Robustness is often used as a desired characteristic of a system and therefore this should be specified accordingly. Robustness could be interpreted as the ability of a system to provide the output data or signals in accordance with its specification under changing external conditions and in cases of external disturbances (interferences, jamming, atmospheric influences). The robustness should therefore be required both for the basic functionality (output data with required accuracy) and for the integrity functionality (assessed accuracy of output data).

- Redundancy is specified as “the existence of multiple equipment or means for accomplishing a given function in order to increase the reliability of the total system” [IMO A.915(22), IALA R-121]. Or in other words, “a redundant system provides the same functionality as the primary system, allowing a seamless transition with no change in procedures” [IALA R-129]. Existing and future GNSS like GPS, GLONASS and GALILEO are in principle redundant systems. A concept for their redundant utilisation should be developed in the framework of the Integrated PNT Unit. Special attention should be given to the correlation behaviour of error sources such as ionosphere or jamming in upper or lower L-band. Such a concept could be the necessary basis to enable the coordinated application of all PNT relevant components in the maritime sector. Consequently, a harmonised meaning and understanding of provided PNT and integrity data can be achieved. Additionally, it should be clarified, whether redundancy should be applied only to PNT data provision or additionally to integrity data provision.

- “A backup system ensures continuation of the navigation application, but not necessarily with the full functionality of the primary system and may necessitate some change in procedures by the user” [IALA R-129]. A backup system is identified as a complementary component of the Integrated PNT System to ensure PNT data provision in the event of GNSS system failures or during phases of increased GNSS vulnerability [IALA WWRNP]. A formal safety assessment study should be conducted as a suitable approach to clarify the residual risk of the redundantly designed Integrated PNT System and to determine the requirements of the foreseen backup system.

- The provision of the “best” position fixing result (or PNT data) is outlined in different IMO and IALA documents [e.g. IALA WWRNP]. First, it should be clarified, what “best” means:

- The best result could be interpreted as the result with the highest accuracy. Alternatively, the best result could be also the provision of output data with lower accuracy but ensured integrity. The provision of the best result requires parallel processing chains (shore-side and on-board) for the generation of output data and a superordinat assessment functionality implementing the decision process “what is the best”.

- Different operation areas (such as coast, port) and vessel manoeuvres (such as docking) result in different accuracy requirements for provided PNT data. Several services are applied or identified as candidates to enable the position determination with higher accuracy. This is realised by suitable provision of augmentation and correction data. The type of provided augmentation and correction data specifies, which technique can be applied for position determination and which nominal service level (degree of accuracy) can principally be achieved. The real achievable service level can be different from the nominal service level due to disturbances coming from signal propagation errors, reduced signal availability, equipment failures, inteferences, or jamming. The coverage areas of different terrestrial services result in changes of service availability and available nominal and real service level during berth to berth voyage (oceancoast, coastport). In consequence, the PNT Module should be able to select the service, which exhibits the highest degree of real service level. This can differ to the ranking of nominal service levels of the available services.

- Points 1 to 9 should be also applied on the provision of navigation and timing data. To that end, PNT performance requirements should additionally be specified for navigation and timing data.

### Shore-side part of Integrated PNT System: Maritime PNT Service Portfolio

The modernisation of maritime augmentation services and the provision of contributions to e-navigation are approved activities of IALA [IALA R-135, IALA R-129, and IALA Guidline No 160]. Following the objective of e-navigation the provision and enhancement of services for all phases of vessel navigation should be promoted considering safety-critical requirements.

- Standardised and optional PNT services for berth to berth navigation

- The modernisation of the IALA Beacon DGNSS and the technical enhancement by the application of the Virtual Reference Station concept should be taken into account as well as the increasing potential of GNSS measurement redundancy from independent systems and from different usable frequency bands. The development of redundancy and backup concepts in the framework of DGNSS enhancement for costal areas should be initiated. Special attention in this context should be given to available communication resources for augmentation signal provision and their optimised use.

- The provision of GNSS integrity messages by maritime augmentation systems is a further contribution to the optimised use of available communication capacities. The benefits of such integrity messages should be investigated in PNT relevant performance parameters like accuracy, integrity, continuity and availability.

- A part of the berth to berth navigation is the vessel’s navigation in ports and the realisation of docking manoeuvre. Therefore the standardisation of augmentation services enabling position accuracies better than 1 m or 1 dm and ensuring system integrity should be promoted.

- As described in chapter 3.2.1 (1), the procedures for accuracy assessment and coordinated integrity management with the on-board equipment (PNT Module including multi-system radio navigation receiver)) should be detailed to achieve reliable and comparable assessment results.

### Ship-board part of the Integrated PNT System: Integrated PNT “device” or PNT Module

The preparation of Performance Standards for multi-system radio navigation receivers should be considered as an intermediate step towards Performance Standards for the on-board integrated PNT Module.

The original item “device” should be discussed with respect to the modular and open architecture of the Integrated Navigation System (INS) on the one hand [IMO-MSC.86 (70); IMO-MSC.252 (83)]. On the other hand the use of all electronic means including other on-board sensors for the provision of PNT data (see Fig. 1) is in conflict to a self-contained device. An alternative to the term PNT “device” could be the integrated PNT Module (see Figure 1):

Objective of the PNT Module is the provision of position, navigation and timing data and assigned integrity data in accordance with specified performance requirements, which change during berth to berth navigation. Core of the PNT Module is the PNT (value added data processing) Unit using data of available radio navigation systems and services in combination with data of further on-board sensors for accurate and reliable PNT data provision.

The PNT Module is on the one hand the on-board part of the maritime Integrated PNT System. On the other hand the PNT module is an essential module for the on-board Integrated Navigation System.

The preparation of Performance Standards for the ship-board PNT Module has several advantages as follows:

- Development and introduction of hierarchical processing concepts ensuring that the “best” PNT data (accuracy and integrity) will be provided;

- Identification of complementary and alternatively usable processing chains for PNT data provision including redundancy and backup options;

- Multi-directional accuracy assessment (integrity) of available radio navigation systems and ship-board navigation sensors based on the application of integrated raw data processing and integrity monitoring techniques;

- Accuracy improvement of PNT data by application of data fusion algorithms and internal interactive assistance;

- Output data driven generation of warnings will be applicable and reduces the probability of navigation relevant warnings;

- Harmonised understanding and indication of accuracy and integrity of provided PNT data.

## Role of IALA

The development and establishment of an Integrated PNT System should be considered as a cooperative task of IMO, IALA and further organisations (such as IEC, ITU-R, RTCM). IALA has the expertise and background to promote the development process. This should be communicated by suitable liaison note(s).

The future fulfilment of users need such as “reliability” and “indication of reliability” requires a coordinated design and development of all components of the Integrated PNT system. This is essential for an overall safety assessment of PNT data provision, for the implementation of integrity and risk management, and for the optimisation of responsibilities and functionalities of shore-based and ship-board components towards their cost-effective implementation and operation. Consequently, IALA should also be involved in the development of the on-board PNT module of Integrated PNT System.

# References

| [IMO A.915(22)] | IMO Resolution A.915 (22): Revised maritime policy and requirements for a future Global Navigation Satellite System, November 2001 |
| --- | --- |
| [IMO MSC.86(70)] | IMO Resolution MSC.86(70): Adoption of new and amended performance standards for navigational equipment. |
| [IMO MSC.112(73)] | IMO Resolution MSC.112 (73): Adoption of the revised performance standard for shipborne Global Positioning System (GPS) receiver equipment. |
| [IMO MSC.113(73)] | IMO Resolution MSC.113 (73): Adoption of the revised performance standard for shipborne GLONASS receiver equipment. |
| [IMO MSC.114(73)] | IMO Resolution MSC.114 (73): Adoption of the revised performance standard for shipborne DGPS and DGLONASS maritime radio beacon receiver equipment. |
| [IMO MSC.115(73)] | IMO Resolution MSC.115 (73): Adoption of the revised performance standards for shipborne combined GPS/GLONASS receiver equipment. |
| [IMO MSC.233(83)] | IMO Resolution MSC.233 (83): Adoption of the revised performance standards for shipborne GALILEO receiver equipment. |
| [IMO MSC.252(83)] | IMO Resolution MSC.252(83), Annex 30: Adoption of the revised performance standards for Integrated Navigation Systems (INS). |
| [IMO NAV 54/25] | IMO MSC Report Annex 12: Draft strategy for the development and implementation of e-navigation. |
| [IMO NAV 56/25] | IMO MSC Report of the Working Group: Development of an e-navigation strategy implementation plan. |

# Action requested of the Committee

The Committee is requested to:

- consider the establishment of an Integrated PNT System as one of superior objectives of e-navigation.

- initiate or support the preparation of new IMO Performance Standards for a multi-system radio navigation receiver, a ship-board PNT Module including PNT Unit, and for augmentation services for port and docking.

- promote the establishment of an Integrated PNT System by IALA contributions to conceptual and architectural design as well as technical gap analysis.

- consider the enhancement of an Integrated PNT System as high-priority task of IALA working program  in co-operation with IMO and related organisations and communicate this by liaison note(s).

- setup a time plan for IALA contributions that indicates possible inputs regarding the time schedule of other relevant international organisations (IMO, ITU-R, IEC, RTCM, etc.).