e-NAV9	Input paper

Agenda item	9.3

Task Number	14

Author(s)	Nick Ward

The Committee is invited to note this information from the General Lighthouse Authorities, which is an update of that provided in e-NAV 8/9/18.

Risk Analysis for Virtual AtoNs

Introduction

This input paper summarises the results of a study carried out for the General Lighthouse Authorities of the UK and Ireland (GLA) by consultants Helios.  The work included a Failure Mode and Effect Analysis (FMEA) and Fault Tree Analysis (FTA) of a proposed AIS Virtual Aid to Navigation (Virtual AtoN) service to be provided by the GLAs. It is envisaged that this service will be provided in the UK through the AIS network of the Maritime and Coastguard Agency (MCA) and in Ireland via the network of the Irish Republic Coast Guard (IRCG).

Purpose

The aim of the study was to provide input for the GLAs and IALA to discussions on:

- the definition of a suitable service availability requirement for an AIS Virtual AtoN service; and

- recommendations for the design of an AIS Virtual AtoN system to meet the defined service availability requirements

Service Availability

The AIS Virtual AtoN service availability level should correspond to the appropriate IALA AtoN Category. The proposed AIS Virtual AtoN service definition is:

- The transmission of the following correct information (contained in message 21) relating to all Virtual AtoNs deployed:

- The type of AtoN.

- The name of the AtoN.

- A valid 2D position of the AtoN

- Time stamp.

- Virtual AtoN flag.

- RAIM flag.

- The AIS Virtual AtoN transmissions shall have a signal level of greater than or equal to -107dBm when measured at the air-antenna interface of the user’s receiver within the specified coverage area of the transmitter.

FMEA

The AIS Virtual AtoN FMEA has identified different failure modes and the effect they have on the service, as provided by the GLAs through the MCA network. In particular, these can be split into two types of failure effects:

- Failures that directly affect the provision of the AIS Virtual AtoN service.

- These failures are largely associated with the navigation department procedures, base station and the AIS VHF communication functions.

- Failures that do not directly affect the provision of the AIS Virtual AtoN service but which affect the ability to detect a failure of the service. These failures affect the time to detect a failure of the service and therefore the overall availability of the service.

- These failures are largely associated with the control and monitoring functions and the communications functions.

FTA

The Fault Tree Analysis (FTA) was carried out to calculate the probability of an AIS Virtual AtoN service unavailability event occurring based on AIS Virtual AtoN system component reliability data gathered. The FTA calculated that on average 0.39 service unavailability events per year would be expected for Virtual AtoN messages transmitted from 2 MCA base stations and the deployment (and cancellation) of 10 Virtual AtoNs during the course of a year.

It can be concluded that the redundancy in coverage of the MCA AIS base stations is important in reducing the probability of failure. This explains why the failure rate for providing a Virtual AtoN service using MCA base stations is lower than for providing the service using AIS AtoNs or AIS equipped ships.

It should be noted that no details were available on how the automatic software at the MCA Control Centre for processing Virtual AtoN configuration request emails would be implemented. The specific implementation of this function may therefore affect the likely error rates assumed for configuration of the Virtual AtoN transmission by email. Checks for errors and corruption should be carried out on messages requesting or specifying the configuration of AIS Virtual AtoN transmissions, to minimise the number of service unavailability events due to configuration errors. For example this might be a CRC or digital signature added to an email communication. This is particularly important because data corruption could lead to Virtual AtoN transmissions containing incorrect data, with a significant safety impact.

Calculation of theoretical AIS Virtual AtoN service availability

It should also be noted that while the FTA has focussed on the probability of an AIS Virtual AtoN service unavailability event, it has not calculated the availability (which includes detection and repair times) of the service. In order to calculate service availability the following information would be required:

- Probability of service unavailability event occurring for the entire AIS Virtual AtoN system (including all constituent base stations and AIS Virtual AtoNs).

- Probability of failure of the AIS Virtual AtoN control and monitoring function (required to calculate mean time to detection of an AIS Virtual AtoN failure).

- Calculation of time to detect and repair a failure of the AIS Virtual AtoN service or control and monitoring function.

- Calculation of the effect of interference from other transmissions (assuming correct operation of the AIS channel access protocols).

- Note: the full report on this study is now available on: www.gla-rrnav.org