IEC TC 80 WG 15

056 IEC-AIS-WG 15 MTG 3

26 January 2011

INTERNATIONAL ELECTROTECHNICAL COMMISSION

TECHNICAL COMMITTEE No 80: Maritime navigation and radiocommunication equipment and systems

Working Group 15

# Liaison note to IALA AIS TWG:

# Request for Technical Clarification to ITU 1371

# 1. Introduction

IEC TC 80 WG 15 thanks IALA e-Nav Committee for their Liaison note to IEC from September 2010 responding a request of WG 15 for technical clarification on the behaviour of Cl B SO.

IEC TC80 WG15  held a meeting in London, UK, during the week of January 24th to discuss the IEC62287-2 SOTDMA Class B AIS standard and CDV comments to IEC61993-2 Edition 2 (Class A AIS).

The following points were raised during the meeting:

1. Regarding use of GNSS SBAS by AIS equipment

Current AIS equipment standards provide a prioritised list of position sensor fallback conditions (for example Table 4 in 6.10.3.5 of IEC61993-2 Edition 2 FDIS). The table describes the priority of GNSS position sources when differentially corrected by AIS message #17 or a DGNSS beacon receiver.

The working group noted that the majority of current GPS receivers suitable for integration into AIS equipment provide support for SBAS (WAAS / EGNOS / MSAS etc.) corrections. If enabled, reception of SBAS corrections will result in the GPS receiver entering differential mode and outputting a differentially corrected position fix.

Current AIS equipment standards do not explicitly permit or exclude the use of a GNSS position source with SBAS differential corrections, nor do they give a relative priority for sensor fallback in this mode of operation.

Consensus in the working group was that a position source with SBAS corrections should be permitted with a priority below beacon corrections.

IALA is asked to provide clarification on the use of GNSS SBAS corrections in AIS equipment; specifically should this mode of operation be permitted, and if permitted, where should it be positioned in the sensor fallback priority list.

2. Regarding Tx power

Class B SO specification in Annex 2 in M. 1371 state that Class B SO shall have two power settings:

low = 1 Watt,

high = 12,5 W (default)

WG15 is of the opinion that a lower power level is more appropriate because:

  - Class B is intended to be a low-cost system for use mainly on pleasure and small vessels.

  - A high power setting of 2 Watts provides sufficient range and will provide compatibility with existing Class B CS designs.

  - Power settings above 2 Watts will result in a more expensive product.

  - 12,5 Watts increases the blocking problem when vessels are in close proximity which is more likely with small vessels.

  - 12,5 Watts increases the possibility of  negative impact to the VDL when Class B population increases in the future.

We ask IALA to consider these concerns and review the high power setting in Annex 2 in  M.1371-4

3. Regarding free slot versus garbled slots

The impacts of treating a garbled slot as something other than a free slot have not been investigated.

Simulations by the USCG to investigate the issue are still ongoing.

So far no amendments have been made to the CDV Class B SO regarding the consideration of garbled sots.

4. Regarding receiver and transmitter characteristics of Class B SO

ITU-R M.1371-4 defines in Annex 2 the receiver and transmitter characteristics for AIS using TDMA techniques and in Annex 7 the receiver and transmitter characteristics for Class B AIS using CSTDMA technology.

The receiver characteristics of Class B SO should be as defined in Annex 7 and the transmitter characteristics should be as defined in Annex 2.

The rational is to apply the more stringent requirements from Annex 2 to the transmitter to maintain a high level of VDL protection while the requirements for the receiver may be reduced to achieve a more economical solution.

This should be reflected in a technical clarification on ITU-R M.1371-4

5. Regarding message 25 and 26

It has been brought to the attention of the group that there is a byte boundary problem with the current definition of message 25 and 26. These issues should be addressed in ITU-R M.1371 before use if these messages become widespread.

Message 25 (in addressed mode) and Message 26 (in all configurations) are missing spare bits to fulfil the byte boundary rules of VDL messages and binary data payloads.

A 30 bit field is added for Destination ID if addressed mode is used in Message 25 and Message 26. In this case two spare bits should be inserted after Destination ID, to realign the byte boundaries and to be able to use byte sized data for payload as in Message 6 and Message 8.

In Message 26 an additional 4 bits are needed to make the complete message respect byte boundaries, regardless of addressing of not. The best place to insert the permanent 4 spare bits would be just before the CommState selector flag.

Until this issue has been corrected, we believe is it premature to make transmissions of message 25 and 26 mandatory for Class A units.

CommState in message 26 will be identified by decoding the last 20 bits of the message 26, assuming no spare bits has been added after the CommState, to adjust for byte boundaries of VDL messages.

Proposed solution to definition of message 25 and 26 respecting byte alignment below.

Message 25 respecting byte boundaries

| Parameter | Number of bits | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Message ID | 6 | Identifier for Message 25; always 25 |  |  |  |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |  |  |  |
| Source ID | 30 | MMSI number of source station |  |  |  |
| Destination indicator | 1 | 0 = Broadcast (no Destination ID field used) 1 = Addressed (Destination ID uses 30 data bits for MMSI) |  |  |  |
| Binary data flag | 1 | 0 = unstructured binary data (no Application Identifier bits used) 1 = binary data coded as defined by using the 16-bit Application identifier |  |  |  |
| Destination ID | 0 / 30 | Destination ID (if used) |  | If Destination indicator = 0 (Broadcast); no data bits are needed for the Destination ID If Destination indicator = 1; 32 bits are used for Destination ID and spare bits for byte alignment |  |
| Spare | 0/2 | Spare (if Destination ID used) |  |  |  |
| Binary data | Broadcast Maximum 128 Addressed Maximum 96 | Application identifier (if used) | 16 bits |  | Should be as described in § 2.1, Annex 5 |
|  |  | Application binary data | Broadcast Maximum 112 bits Addressed Maximum 80 bits |  | Application specific data |
| Maximum number of bits | Maximum 168 | Occupies up to 1 slot subject to the length of sub-field message content |  |  |  |

Message 26 respecting byte boundaries

| Parameter | Number of bits | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Message ID | 6 | Identifier for Message 26; always 26 |  |  |  |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated |  |  |  |
| Source ID | 30 | MMSI number of source station |  |  |  |
| Destination indicator | 1 | 0 = Broadcast (no Destination ID field used) 1 = Addressed (Destination ID uses 30 data bits for MMSI) |  |  |  |
| Binary data flag | 1 | 0 = unstructured binary data (no Application Identifier bits used) 1 = binary data coded as defined by using the 16-bit Application identifier |  |  |  |
| Destination ID | 0/30 | Destination ID (if used) |  | If Destination indicator = 0 (Broadcast); no data bits are needed for the Destination ID If Destination indicator = 1; 30 bits are used for the Destination ID and 2 spare bits for byte alignment |  |
| Spare bits | 0/2 | Spare (if Destination ID used) |  |  |  |
| Binary data | Broadcast Maximum 104 | Application identifier (if used) | 16 bits |  | Should be as described in § 2.1, Annex 5 |
|  | Addressed Maximum 72 | Application binary data | Broadcast Maximum 88 bits Addressed Maximum 56 bits |  | Application specific data |
| Binary data added by 2nd slot | 224 | Allows for 32 bits of bit-stuffing |  |  |  |
| Binary data added by 3rd slot | 224 | Allows for 32 bits of bit-stuffing |  |  |  |
| Binary data added by 4th slot | 224 | Allows for 32 bits of bit-stuffing |  |  |  |
| Binary data added by 5th slot | 224 | Allows for 32 bits of bit-stuffing |  |  |  |
| Spare | 4 | Needed for byte alignment |  |  |  |
| Communication state selector flag | 1 | 0 = SOTDMA communication state follows 1 = ITDMA communication state follows |  |  |  |
| Communication state | 19 | SOTDMA communication state (see § 3.3.7.2.1, Annex 2), if communication state selector flag is set to 0, or ITDMA communication state (§ 3.3.7.3.2, Annex 2), if communication state selector flag is set to 1 |  |  |  |
| Maximum number of bits | Maximum 1 064 | Occupies 1 to 5 slots subject to the length of sub-field message content |  |  |  |

Byte boundary references in 1371-4 below.

Annex 1, Section 5.2.1

Taking into account that safety related and binary messages should be used, it is of importance that the variable messages are set on byte boundaries.

Annex 2 Section 3.37

When data is output on the VHF data link it should be grouped in bytes of 8 bits from top to bottom of the table associated with each message in accordance with ISO/IEC 3309: 1993.

Annex 5, Section 4

All data fields of the FMs should observe byte boundaries. If needed to align with byte boundaries, spares should be inserted.)

3. Requested action

IALA is invited to consider drafting technical clarification as appropriate.

HÅKAN LINDLEY

Convenor IEC TC80 WG15

Swedish Transport Agency

Maritime Department

Box 653

SE-60115 NORRKÖPING

Mobile  +46 708 156800

E-mail     hakan.lindley@transportstyrelsen.com