SUB-COMMITTEE ON SAFETY OF	NAV 57/8/xx

NAVIGATION	25 April 2011

57th Session	Original: ENGLISH

Agenda item 8

DEVELOPMENT OF AN E-NAVIGATION STRATEGY IMPLEMENTATION PLAN

Resilient Position, Navigation & Timing (PNT)

Submitted by IALA

| SUMMARY |  |  |
| --- | --- | --- |
| Executive summary: |  | This document discusses the need for Resilient Position, Navigation and Timing within e-Navigation, and includes a summary study of options. |
| Strategic direction: |  | 5.2 |
| High-level action: |  | 5.2.4 |
| Planned output: |  | 5.2.4.4 |
| Action to be taken: |  | Paragraph 23 |
| Related documents: |  | A.953(23), A.915(22), MSC 86/26 |

Purpose of Document

1	The IMO Maritime Safety Committee has stated that “e-Navigation systems should be resilient and take into account issues of data validity, plausibility and integrity for the systems to be robust, reliable and dependable. Requirements for redundancy, particularly in relation to position fixing systems should be considered.” (IMO MSC 85/26 Add.1)

2	Discussions  at NAV 56 in July 2010 identified redundant positioning as an aspect of the Gap Analysis to be considered further at NAV 57. This document summarises the problems affecting position-fixing systems and condenses the results of a comprehensive study of alternative positioning solutions, carried out in the British Isles, which could be used to inform the discussions at NAV 57.

Vulnerability of GNSS

3	It is recognised that Global Navigation Satellite Systems (GNSS will be the primary PNT input for e-Navigation applications and services. However, GNSS is vulnerable to space-vehicle failures, solar disturbances, unintentional radio interference and, increasingly, deliberate jamming. The growing use of ECDIS and other integrated systems increases the reliance on positional data from GNSS and therefore the probability of a related accident.

4	Problems in GNSS reception are relatively common in a number of areas, such as the Golfo di Genova, the Golfo di Napoli and in the Golfe du Lion. The problems in the three named areas are known to be caused by interference from certain high power TV transmitters. Problems have also been reported in areas where high power military radars are in use, and pressure on the electromagnetic spectrum is ever increasing.

5	On 1 January 2004, the failure of an atomic clock in a satellite caused ships’ GPS receivers in European waters to display serious position errors. There was no warning before incorrect positions appeared that were plausible, misleading and dangerous. Later these errors built up to between 10km and 40km. They affected all vessels that did not take advantage of the integrity service provided by IALA DGPS beacons or other augmentation services. This type of major failure should also be effectively resolved by the inclusion of Receiver Autonomous Integrity Monitoring (RAIM), when fitted.

6	On 6 December 2006, a solar flare sent out radio noise so intense that many GPS receivers failed to give a valid PNT solution across the entire sunlit side of the earth. Such solar cycle events are expected to be more frequent for the next few years.

7	The principal vulnerabilities of GNSS result from the extremely weak signals from the satellites that reach the user’s receiver on earth. A satellite transmits no more power than a car headlight, yet must illuminate nearly half the earth’s surface from 20,000km away. Natural noise or man-made interference can easily swamp the received signal. There have been several documented examples of this type of accidental interference.

8	GPS is increasingly being jammed intentionally. Low-power transmitter devices are on retail sale for under $100. Criminals use such jammers to overcome GPS security systems and so steal vehicles and their loads. Devices in the hands of the public include GPS jammers with ranges of tens of kilometres. There is also a potential risk from spoofing, in which erroneous GNSS signals are broadcast with the intention of producing inaccurate but apparently valid GNSS outputs. Detection of jamming and spoofing events are a key part of a resilient system.

9	In 2007, vessels in the harbour of San Diego suddenly lost GPS service for two hours, when a military jammer used for naval trials was accidentally left on. In the city, more than 100 mobile telephone sites failed. The communications of first responders to the incident and other emergency services were affected.

10	In 2010 GPS was lost along 300 km of coastline, in an Asian country, for significant periods over several days, as a result of a jamming signal from an adjacent country. A similar, smaller event occurred in March 2011, affecting user equipment and telecommunications.

11	Recent UK trials with a jammer of less than one thousandth the power of a mobile phone, on board a ship, easily and frequently induced false GPS positions and velocities with no alarm indications. The ship’s position on the chart display would begin to depart from the truth, in ways that watchkeepers would possibly not notice. This is a very dangerous phenomenon. When the jamming power was increased slightly, multiple alarms sounded in various parts of the bridge, causing confusion. The following systems all failed concurrently: the main Electronic Chart Display and with it the ship’s autopilot; DSC and GMDSS. In addition, the principal navigation aids on which officers depend for navigation in the absence of GPS also showed alarms: the ship’s radar and gyrocompass; as did the AIS. For a small bridge team in poor visibility, safely maintaining navigation whilst dealing with multiple alarms, and continuing the voyage without these systems, would have been challenging if not impossible.

A Range of Approaches to Resilient PNT

12	Resilience can be defined as “…the ability to withstand or recover quickly from difficult conditions….” Systems may be made resilient by building-in, or otherwise providing, standby capacity or by switching to alternative means.

13	GNSS itself will become more resilient as multiple satellite constellations are deployed, each with its independent monitoring and control. Augmentation systems or RAIM can enhance integrity, ensuring that PNT information is correct or immediately warning the user when it is not. However, these approaches do not provide a solution when solar noise, or man-made interference affects all GNSS signals and the augmentations themselves may also be affected.

14	Alternative systems with dissimilar failure modes can provide integrity and protect against these hazards, although they may not provide the same performance as GNSS. Radar systems are not susceptible to solar activity or jamming at GNSS frequencies, although they can be degraded by multipath and clutter effects. Terrestrial PNT systems, with their higher signal levels and dissimilar frequencies, are much more resilient to interference and jamming. Like satellite systems, they may occasionally be affected by solar activity although not at the same time, since the causes are different. Candidate systems may include eLoran; other developments of LF technology; or ranging (R-mode) signals broadcast via existing maritime infrastructure. eLoran is the most developed of these alternatives at present.

15	Onboard inertial navigation systems provide independent measurements of a ship’s motion. Inertial systems with the long-term stability to act as a backup for a useful period are much too costly for civil use, though developments in solid-state sensors and digital processing will eventually reduce prices.  However, inertial navigation could contribute to GNSS back-up and PNT error detection. Developments in this technology should be monitored.

16	Resilience of PNT information within e-Navigation requires dissimilar systems, optimally combined.

17	An example study of options is given at Annex A.

Complement to GNSS to achieve resilient PNT

18	The ideal maritime complement to GNSS would:

- enable resilient PNT for use by critical infrastructure applications including maritime transport.

- be readily integrated with GNSS at chip-level.

- support GNSS interference detection and mitigation.

- maintain the user’s concept of operations with a seamless transition to a complementary service when GNSS is lost.

- have the potential to be deployed world-wide.

- support maritime general navigation applications.

- be independent of GNSS.

- be dissimilar in terms of failure modes.

- provide similar levels of performance to GNSS.

Conclusions

19	PNT is an essential component of e-Navigation. MSC has stated that “e-Navigation systems should be resilient and take into account issues of data validity, plausibility and integrity for the systems to be robust, reliable and dependable. Requirements for redundancy, particularly in relation to position fixing systems should be considered.”

20	PNT systems are vulnerable to interference effects, both natural and man-made. All GNSS have common failure modes, which can be mitigated by the use of dissimilar systems.

21	There are several options to improve the resilience of PNT. The best solution may be a combination of these options and different solutions may be used in different parts of the world. This approach would be facilitated by the standardisation of a maritime multi-system PNT receiver.

22	As improved PNT continues to increase the number of routeing options available to the mariner, it is recognised that changing traffic patterns could result in an increase in the number of short range aids to navigation required.

Action requested of the Sub-Committee

23	The Sub-Committee is invited to note the above observations.

Annex A: Example Study of Options identified

Options for the future of maritime Aids-to-Navigation (AtoN) within an e-Navigation environment have been developed for a study in the British Isles, through a series of workshops with prominent national experts in the fields of maritime systems and navigation. The methodology used for this study may also be applicable to developments in other areas.

Four options were identified, which were credible within the timescale for the introduction of e-Navigation:

- Option 1 – Do minimum;

- Option 2 – Maximise the current infrastructure;

- Option 3 – Hardening of GNSS;

- Option 4 – eLoran implementation.

The number of physical AtoN required to support each option was determined based on the navigational requirements. It was recognised that changing traffic patterns could result in an increase in the number of short range aids to navigation.

Option 1 ‘Do Minimum’

The first option represents a broad continuation of the current AtoN provision, using existing approaches and technologies to mark hazards, channels and traffic separation schemes. In this option IMO’s e-Navigation concept is assumed to be implemented as expected by 2018-2020 and is predicated upon existing technologies. It is expected that the majority of SOLAS vessels will be equipped with standards-compliant ECDIS by 2018 and will, over time, install NT radar systems and multi-constellation GNSS receivers, as their existing equipment needs replacing. This approach will provide minimal improvements to PNT resilience.

Option 2 ‘Maximise Current Infrastructure’

The second option recognises the current trends in maritime radar equipage and assumes an expansion or enhancement of the physical AtoN infrastructure to support a fallback mode based on radar positioning. It is assumed that e-Navigation is implemented by 2018-2020; all SOLAS vessels are equipped with ECDIS by 2018 and multi-constellation GNSS receivers over time. NT radar systems will be installed by 2018 and will be capable of supporting vessel absolute positioning in the coastal and harbour environment, providing position to the ECDIS for use in the event of a loss of GPS position. This would require some cooperative action on the part of radar manufacturers, possibly stimulated by regulatory changes.

Recognising the dependence of users upon GNSS-based navigation, some initiatives would be taken by shore-based authorities to mitigate the risk as the mariner navigates closer to danger due to misplaced confidence in vulnerable GNSS:

- The first would be to deploy an enhanced infrastructure of radar AtoN (corner reflectors or target enhancers) at appropriate charted points around the coastline, to support NT radar positioning as a key component of integrated navigation that provides resilient PNT for e-Navigation.

- The second activity would be to extend the introduction of new physical AtoN such as synchronised lights and to take measures to enhance the conspicuity of lights.

As well as additional marks for expanding offshore installations for renewable energy, marking of new channels, hazards and traffic separation schemes may be necessary. Position-fixing resilience may be improved by this approach, however timing remains vulnerable.

Option 3 ‘Hardened GNSS’

The third option represents a concerted effort to reduce the risks of complete dependence upon GNSS by the maritime community for the implementation of e-Navigation. Actions would be undertaken to protect the GNSS infrastructure against the effects of interference (so-called ‘hardening’). This would include installation of multi-constellation GNSS receivers, incorporating more robust testing for the effects of interference, with alerting when interference is detected. Anti-spoofing methods would also need to be developed.

Recognising this dependence on GNSS based navigation, the infrastructure would need upgrading, enhancing the capabilities of augmentation systems to support multiple GNSS systems/additional frequencies. There would also need to be support for the standardisation of more robust user GNSS receiver equipment and its safe integration into future bridge systems.

This option would require action from maritime authorities and telecommunications regulators to ensure adequate means to deter, detect and respond to GNSS interference events. Reversionary procedures would need to be developed by maritime authorities, VTS operators and shipping organisations, through IMO, for mariners in the event of potentially hazardous GNSS interference. PNT resilience would be significantly improved in the longer term.

Option 4 ‘eLoran implementation’

The fourth option represents the provision of an alternative position-fixing system by maritime authorities, complementing GNSS to ensure resilient PNT. This option assumes that the IMO’s e-Navigation concept would be implemented as expected by 2018-2020 predicated upon robust PNT sourced from GNSS and eLoran. It is also assumed that all SOLAS vessels will be equipped multi-system (integrated eLoran/GNSS) receivers by 2020.

eLoran implementation activities would lead to Full Operational Capability in 2018. This would ensure a resilient PNT service for harbour and coastal operations around all coastlines where ‘the volume of traffic justifies and the degree of risk requires’ (SOLAS Chapter V).

The study assumes that, as a consequence of the reliability of PNT and the resulting robustness of e-Navigation, Option 4 makes it possible for shore-side authorities to rationalise their lights infrastructure. Any lights selected for withdrawal would be based on sound navigational principles. Remaining lights would be replaced using low cost technology, independently mounted, or mounted on existing structures if a day mark is required. These changes could result in substantial income and savings.

Economic appraisal of options

The approach taken to the economic appraisal was to perform a discounted cash flow analysis of the costs and benefits of each individual option. Comparisons were then made to the ‘do minimum’ option (Option 1). This required the identification and valuation of all of the relevant costs and benefits that fall to all of the maritime stakeholders within each option. The economic appraisal includes costs for the upgrade and extension of the existing eLoran infrastructure in the British Isles.

A conservative approach was taken to evaluate the economic case so that the results were credible for all options. In this connection, the main economic benefits were assumed to be the possibility of reducing the provision of physical AtoN (lighthouses and buoys) as a result of confidence in the resiliency of electronic position-fixing (Option 4). Estimates of savings associated with the removal of lights incorporate the conservative assumption that the overall cost of provision of lights (capital, operational and overhead costs) would be subject to efficiency savings (2.5% p.a.) irrespective of the option pursued.

In some of the options, the primary benefits arise from avoided expenditure. These costs and benefits were valued over the transition to, and the minimum lifetime of, the eLoran service. The timing of the costs and benefits was established and the resulting figures discounted.

Subsequently the Net Present Value (NPV) of each option was ascertained. This enabled comparison with the ‘do minimum’ option. A number of parameters (costs, benefits and timing) were varied in order to understand the sensitivity of the results to specific assumptions.

General assumptions

It was assumed within a number of options that the maritime users equip with new capabilities in order to receive the benefits of e-Navigation. It was assumed that only those costs incurred specifically due to each option were to be valued. For example, ECDIS equipage was assumed across all scenarios (including the ‘do minimum’ option) and was therefore not valued.

Widespread fitting of enhanced equipment was assumed, in order to enable the full benefits of e-Navigation. In practice this would mean that most vessels transiting the waters in which e-Navigation is implemented as well as those making port calls, would have the required equipment. The appraisal has not attributed the costs of equipage of all international maritime users in this study, which was carried out for a limited region.

In addition, the full cost of new equipment was not attributed. Instead, the analysis used the cost delta between the basic capability that the vessel owner would ordinarily equip with and that of the more advanced receiver. The cost delta per receiver was estimated to be £200 for multi-constellation GNSS and £500 for eLoran/GNSS on SOLAS vessels and £100 and £250 respectively on non-SOLAS vessels. This figure was assumed to decrease over time until in 10 years’ time there would effectively be no cost delta as the new receiver becomes the de facto standard. This model is based on experience of the cost profile of GPS receivers as new capabilities are introduced.

Preferred option on economic grounds

The economic analysis of the four options developed within the business case process resulted in only one option (Option 4) that produced a positive NPV over the duration of the analysis. Furthermore, neither of the other options resulted in a better NPV than the ‘do minimum’ option.

The preferred option on purely economic grounds would be Option 4 (Maritime eLoran). The benefit-to-cost ratio of the preferred option was in excess of 2:1. Sensitivity analysis of Option 4 showed that the benefits are robust to significant changes in its underpinning assumptions. It is not particularly sensitive to variations in the cost of implementing eLoran, either the core system or the necessary vessel equipage. In addition, the likely extension of the system life beyond 10 years greatly enhances the overall benefits of the option.

Study Outcome

On the basis of the economic analysis, the net financial benefits and ongoing savings of implementing eLoran in the case of the British Isles would be substantial. It has been shown that when all the building blocks of e-Navigation are in place, with fully resilient PNT it may be possible to significantly reduce expenditure on conventional AtoN. Any changes to AtoN would need to be based on sound navigational principles, ensuring retention of those lights that safeguard the mariner’s local situational awareness.