e-NAV9

Agenda item	8.2

Task Number	6

Author(s)	Johann Winell, Michael Hoppe

Update of IALA Recommendation R-121 on the Performance and Monitoring of DGNSS Services

in the frequency band 283.5 – 325 kHz

Chapter 5 “Integrity”

# Summary

Aim of the input paper is the update of the IALA Recommendation R-121 on the Performance and Monitoring of DGNSS Services in the frequency band 283.5 – 325 kHz. The input paper focuses on the required changes of the Integrity Functionality (Chapter 5) of R-121. This results from the different replacement options described in IALA R-135 on the Future of DGNSS. Especially for the replacement option Virtual Reference Station (VRS) the document explains the integrity functionality based on the new Pre-Broadcast-Monitoring (PBM) method.

## Action arising from the input of the document

The e-NAV Committee is invited to consider the proposed changes and new structure of chapter 5 (Integrity) of IALA Recommendation R-121.

## Related documents

- Related Documents

| Short Name | Title of Document |
| --- | --- |
| [IALA R-121] | IALA Recommendation R-121: On the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz, Edition 1.1, December 2004 |
| [IALA R-135] | IALA Recommendation R-135: On The Future of DGNSS, Edition 2, December 2008 |
| [IALA Guidline No. 160] | IALA Guideline No 1060: On Recapitalisation of DGNSS, Edition 1, December 2008 |
| .. |  |

# Background

The DGPS service, according to the “IALA-Standard” (International Association of maritime aids to Navigation and Lighthouse Authorities) provides GPS corrections within the maritime radio beacon band (283.5 – 325.0 kHz). The GPS L1 C/A-Code corrections are normally generated from local on-site reference receivers. Furthermore the GPS corrections are monitored and integrity proofed to inform users in case of satellite failures or if the service is outside the defined performance specifications. Due to the fact that most of the hard- and software of the DGPS system was installed around 1995, the whole service is getting more and more difficult to run and maintain. In addition it could also be necessary to provide augmentation information for future new GNSS signals from GPS, Galileo, GLONASS or COMPASS. Therefore a re-engineering and further development of the existing maritime DGPS service towards a modernized DGNSS service is necessary to fulfill future maritime requirements.

# Discussion

## Replacement Options

According to IALA Recommendation R-135 on Future DGNSS the following options are identified to re-engineer the existing radio beacon DGPS:

- Hard- and software replacement

- Use of a modular software solution based on COTS (Commercial off-the-shelf) receiver technology

- SBAS integration (existing SBAS, such as WAAS or EGNOS could offer a low-cost solution to service providers)

- Use of virtual reference stations.

Because Integrity is nowadays a key feature of the maritime IALA DGNSS, it is important that all identified replacement options facilitate the same level of integrity functionality.

- The current version of IALA Recommendation R-121 describes the integrity functionality which is just performed within on-site integrity monitor receivers. To enable also other realization concepts (e.g. Pre-Broadcast-Monitoring or Integrity within SBAS), it is necessary to update Chapter 5 (Integrity) of IALA Recommendation R-121

## Proposed new structure of R-121 (Chapter 5 Integrity)

5 	Integrity

5.1 	Definition

“Integrity is the ability to provide users with warnings within a specified time when the system should not be used for navigation.” (Ref. 4)

  - On-Site Integrity Monitoring

The On-Site integrity monitoring or local monitoring is the preferred option for DGPS stations which provides GNSS corrections from local on-site reference receivers.  The On-Site integrity monitoring is performed at the reference station site within the integrity monitoring (IM) receiver. The IM sends flags (alarms) to the reference station (RS) receiver along with system feedback. The alarms can further be separated into the following categories:

- Position domain alarms
Position domain alarms are generated when the radial position error, computed by the IM, exceeds the DGNSS service provider protection limit for more than the pre-set period of time. When the RS receives a position domain flag, it sets the Station Health Field in the header of each broadcast message to indicate this "unhealthy" condition.

- Pseudorange domain alarms
Pseudorange domain alarms are generated when the computed residual for a given pseudorange exceeds the pre-set threshold for more than the pre-set period of time. Upon receipt of a pseudorange alarm the RS will set the PRC and RRC values for the satellite to the "do not use" values, as defined in RTCM SC 104.

- Integrity Monitor System Feedback
When a pseudorange and/or position domain alarm threshold is exceeded, the IM sends an immediate system feedback message to the RS.  Additionally, this message, a RTCM SC104 RSIM 20, is sent on a pre-set periodic basis, which confirms to the RS that it is actually being monitored. When the RS will not receive IM system feedback within the specified time period, it sets the Station Health Field in the header of each broadcast message to "unmonitored".

- MSK Radiobeacon Monitoring
Further some characteristics of the MSK radiobeacon signal like signal strength, signal to noise ratio, RTCM message throughput (WER) and correction age will be monitored at the IM. This kind of monitoring will be indicated as IM alarm or warnings to the DGNSS system provider and may cause a switch to standby equipment.

  - Pre-Broadcast-Monitoring for the use of Virtual Reference Stations (VRS)

For the realisation of the VRS replacement option the use of the Pre-Broadcast Monitoring (PBM) is recommended. The PBM algorithm can be integrated in the central VRS process (see figure 1).

The VRS process, normally running at a central server installation, has the following tasks:

- Receive input data streams from a network of GNSS receivers

- Perform the plausibility check of the GNSS raw data streams

- Generate GNSS corrections for virtual reference stations

- Perform the PBM functionality based on

  - the calculated PRC and RRC for multiple VRS positions and

  - Input data streams which are not used for the generation of GNSS corrections

Figure 1: Generation of GNSS corrections and Pre-Broadcast Monitoring based on Virtual 
 	     Reference Station Concept (VRS)

The PBM enables all relevant checks before the data will be transmitted. The PBM can be divided in the following two fields (figure 2):

Satellite related Monitoring

The PBM performs a detection of faulty satellites. A satellite is faulty, if

- the GPS system provider marks a satellite as „unhealthy“,

- the DGPS system provider marks a satellite as „unhealthy“ or

- the PRC or RRC of a satellite exceeds a user preset threshold.

As a result of the above checks the PBM algorithm will prevent the transmission of corrections or will mark faulty satellites with “do not use” within the RTCM messages (1 or 9-3).

Monitoring the quality of the generated VRS corrections

The main task of the PBM is the detection of faulty or inaccurate VRS corrections. The calculations are based on calculated position deviations according to a given position and a preset threshold. For this kind of integrity monitoring it is important that the corrections will use independent GNSS raw data streams which are not included in the VRS generation process. A VRS correction stream is faulty, if

- the calculated position error is higher than the preset threshold 
As a result the VRS reference station will be marked as “unhealthy”

- the monitoring source is not available or monitoring functionality could not performed 
As a result the VRS reference station will be marked as “unmonitored”

Figure 2: Pre Broadcast Monitoring (PBM) as part of the VRS-Server

Monitoring of the MSK Radio beacon throughput when using VRS

Since the VRS-corrections are calculated at a central server installation they have to be transferred to the transmitted site for transmission. On the site the data stream has to be interpreted and sent to the modulator for MSK modulation, and from there to the transmitter for transmission. This chain is essential for the DGNSS-service to reach the user, and must therefore be monitored in some way. There are xx different options of perform that monitoring:

- The preferred way to monitor that the transmission is reaching the user is to check the RTCM-throughput (WER) in the far-field, since the whole chain then is tested. When this option is chosen, the monitoring-chain also includes the far-field monitoring equipment and the transmission from the far-field monitor to the control segment. To avoid introducing an additional single point of failure (SPF) in the monitoring chain the radio beacon has to be monitored by two far-field monitors forming redundancy (preferable located in separated locations).



Figure 3:  Monitor MSK Beacon throughput using far-field monitoring

- Another option of monitoring the radio beacon throughput is to monitor it on the transmitting site. This can be monitored at different test points:

- After the interpretation of the VRS data stream, then relying on BIT and status indicators of the  
 MSK-modulator, the status indicators of the transmitter and the monitoring of the signal strength
 of the MF-signal.

- After the MSK-modulator, then relying on the status indicators of the transmitter and the 
 monitoring of the signal strength of the MF-signal

- Monitoring of the signal strength and (WER) of the MF-signal.

Studying the options above, it is first obvious that the c) option is the preferred way of doing the monitoring on site. However it introduces the need of a MF-receiver on site having the capability to determine the WER. Since this part becomes essential for this monitoring step it needs to be doubled as in the case of the far-field monitoring.

When monitoring on site the same transmission as fort the VRS-data steam can be used to connect the monitoring devices to the control segment.

Figure 4:  Monitor MSK Beacon throughput using on site monitoring

Importance of VRS raw-data sources

The raw-data sources are another essential part in the VRS-solution approach. The VRS-server can be configured to handle if a single or a couple of RAW-data sources becomes inaccessible, then the VRS-server can be operated with a reduced number of raw-data sources to some extent. A problem arises if all raw-data sources get unavailable at the same time. The designing of the connections to the raw-data sources is therefore of great importance. They are preferably designed with diversity, either by using different network operators to different subsets of raw-data sources, or having redundant connections to some raw-data sources. It is of importance to consider the possibility of a single point of failure (SPF) within a network operator’s backbone that might affect both wired and wireless access connections to the raw-data sources.

  - Pre-Broadcast-Integrity for the use of Software Reference Stations and Integrity Monitoring

[TBD]

  - Integrity of Space based Augmentation Systems (SBAS)

[TBD]

  - Far Field Integrity Monitoring

Some nations have also implemented far field monitoring facilities. In this case IM will be installed at remote locations. This kind of monitoring can provide better information of current coverage and availability of a DGNSS site.

  - General

The criteria used for identifying an integrity failure should be stated by the Administration in their publications. The time for broadcasting an integrity warning to the user, from the time the integrity problem is identified, should not be greater than 10 seconds.

The recommended thresholds for detecting failures are provided in Table 3 Section 12 – DGNSS STATION SETTINGS.

When it has been determined that the error condition no longer exists, there may be a delay, similar to that used for initially identifying the error, before the error warning is withdrawn.

## Role of IALA

The IALA Recommendation R-121 on the Performance and Monitoring of DGNSS Services in the frequency band 283.5 – 325 kHz is a important document for member states which provide a DGNSS service within the maritime area. Because current and future GNSS systems can provide position accuracy of less than 10 m, the integrity function has become a key functionality of the DGNSS.

# References

| e-NAV8/18/13 | Report from Swedish Maritime Administration about a VRS Workshop held in Malmö, Sweden on the 10th and 11th of February 2010 |
| --- | --- |
| e-NAV8/9/21 | Installation of a VRS Test Pilot for the transmission of GNSS corrections using MF radio beacons and AIS base stations |
| .. |  |

# Action requested of the Committee

The Committee is requested to:

- comment the utility of the draft paper,

- develop information concerning Pre-Broadcast-Integrity for the use of software reference stations and integrity of space based augmentation systems (SBAS),

- update IALA Recommendation R-121 with respect to different integrity options