e-NAV9	Information paper

Agenda item	8.2

Task Number	6

Author(s)	M.Hoppe/P.Douglas

Report of Working Group meeting 23-24 November 2010

# Summary

An inter-sessional meeting of the Technology for e-Navigation Working Group was held in Koblenz, Germany on 23-24 November 2010 to identify future developments in DGNSS technology and progress the WG Work Programme.

# BACKGROUND

2.1	Virtual Reference Station Demonstration & Workshop

The VRS-Demonstration/Workshop was held in Koblenz, Germany, on Tuesday, 23rd November 2010 with the aim to discuss the VRS solution as one option to recapitalise the existing IALA DGPS radiobeacon stations. Further the German Federal Waterways and Shipping Administration has implemented a VRS test pilot at the Traffic Technologies Centre in Koblenz, which was demonstrated to members of the Working Group.

### 2.1.1	Opening, general information

Mr. Rainer Strenge, head of the Traffic Technologies Centre (FVT), welcomed the group and gave a brief overview about the tasks of the FVT.

2.1.2	Briefing about current DGPS re-capitalisation status in each country

Each participant introduced himself and gave a short status report of the current recapitalisation process.

- Swedish Maritime Administration gave a report in form of a presentation (see below).

- General Lighthouse Authorities (UK and Ireland) have chosen the hardware replacement option.

- Rijkswaterstaat (NL) gave a report in form of a presentation.

- Norwegian Coastal Administration reported that an upgrade of the existing DGPS 
  network is planned, starting in 2011.

- Maritime Office Gdynia has chosen the hardware replacement option with L1 and L2  geodetic antenna with a possible RTK option in the future, and have replaced the transmitter and antenna system at the Dziwnow DGPS site.

- Finnish Transport Agency is preparing for VRS trials and there is an initiative to replace the existing MSK-Modulator to enable also the transmission of  precise and GNSS independent timing information.

- Danish Maritime Safety Administration reported that the existing DGPS sites were  already upgraded with respect to GPS receivers and new site computer components, and that enough spare parts are available for the next years.

- German Federal Waterways and Shipping Administration gave a report in 
  form of a presentation.

2.1.3	Presentations regarding VRS implementation

- General aspects regarding VRS (M.H.)

M.H. gave a general overview about the status of the present DGPS network and the motivation for recapitalisation. Further he provided information regarding the VRS concept and the VRS features.

- VRS test pilot installation (M.W.)

M.W. presented details of the implementation of the German VRS test pilot and first results regarding accuracy measurements, resulting latency and pre broadcast monitoring.

- Remote control concept of MF-transmitters when using VRS (J.W.)

J.W. stated that the SMA will start a VRS realisation project in 2011.  A three step approach (“layout 1 – 3”) is planned and will establish a joint contact with the National Survey of Sweden. In the last layout, the VRS will be operated completely by the National Land Survey of Sweden. The concept is based on two independent communication networks. J.W. also started a development of a prototype for command and control of the Amplidan Transmitter by presenting a list of parameters of interest, that are candidates to be get and controlled by this prototype.

- Preliminary Integrity Monitoring Concept for maritime Ground Based Augmentation System” (E.E)

E.E. gave a presentation about a preliminary integrity monitoring concept for

maritime ground based augmentation system based on RTK measurements. Such a system can be used to provide a RTK solution in a harbour region providing also integrity monitoring functionality. E.E presented the experimental M-GBAS installation in the research port of Rostock. A graphical user interface of the integrity monitoring for the different phases of navigation was also demonstrated.

- IALA radio beacons in the Netherlands: Current status and future plans (R.L.)
R.L. presented the current status of their three radio beacon reference stations in operation. NL is considering the possibilities of a VRS solution.

- Company presentation (Alberding GmbH, VRS-solutions)
J.A., the manufacture of the German VRS system did a company presentation.

- Visiting the VRS test pilot installation at the FVT
After lunch time the group visited the VRS test pilot installation at FVT and M.H. gave a live - demonstration of the VRS software. In the Lab an additional live demo was setup showing the transmission of AIS Messagetype 17 with corrections generated by the VRS system.

- Visiting the VRS test pilot installation at the DGPS site near Koblenz
The group visited the DGPS reference station near Koblenz where especially the required modifications to transmit VRS corrections were demonstrated.

2.1.4	Discussion and Closure

During the meeting the following topics were discussed:

- Development of a new PNT receiver concept

- Possible backup for GNSS to provide an GNSS independent PNT information for maritime applications

- The future need for DGNSS

- Possible future use of DGNSS radiobeacons (added value)

- 	the problem of time distribution was stressed with regard to future use of DGNSS radiobeacons .

It was decided to further discuss these topics, and to modify the work plan and terms of references for the WG accordingly.

The meeting closed at 18:00.

2.2	Follow-up meeting

The WG reconvened on 24 November onboard the MS Mainz.

The Chairman introduced the Agenda. He thanked the WSV for hosting the meeting and Michael Hoppe and Mario Walterfang for co-ordinating the arrangements.

The group initiated a review of IALA recommendation R-121 on the Performance and Monitoring of DGNSS, specifically to include VRS aspects, and reviewed the current work programme of the WG. It was agreed that:

- The impact of WiMax on S-Band radars would be discussed at e-Nav9 (PD to identify input paper.

- A drafting group would begin work on the requirements for a multi-modal e-Navigation receiver. The group would initially include WSV, DLR, GLAs and FMA and would be co-ordinated by MH. The receiver will follow on from the WWRNP and include work undertaken by the Gauss Initiative and the concept of R-Mode.

- PD would contact N.Ward to initiate the production of an input document to e-Nav9 on alternatives to GNSS and timing in particular.

- PD would contact Geneq for input to receiver work.

- The WG noted that any specification should be user-driven, by changes to IMO Recommendations if necessary.

- AG to identify input on radiobeacon coverage to inform discussions regarding polarregions.

- All members of the WG were requested to review their National inputs to the IALA List of Radionavigation Services for accuracy.

- IALA to be requested that the publication in Council paper 8.1.6A (Provisional meeting dates 2010-14) should include dates of relevant international meetings eg, IMO MSC and sub-committees, NAV, STCW, COMSAR and ITU-R, and preferably also latest delivery dates for documents for these meetings. Calendar should go to all committees/working groups.

- The review of alternatives to GNSS should be publicised at VTS Istanbul & eNav Seattle.

2.3	Technical Visit

At the invitation of WSV, the WG visited the Rheinwacht radar monitoring station at Oberwesel. Inland ENC integrated with AIS and shipborne radar was demonstrated during the river trip on MS Mainz.

ANNEX A: Participant List

| Country | Participant | Organisation | Email |
| --- | --- | --- | --- |
| Denmark | Ivan M. Carlsson | Danish Maritime Safety Administration P.O. Box 1919, DK-1098, CPH-K | imc@frv.dk |
| Finland | Kaisu Heikonen | Finnish Transport Agency P.O. Box 33, Fl-00521 Helsinki | kaisu.heikonen@fta.fi |
| Germany | Michael Hoppe | Traffic Technologies Centre P.O.Box 100420, D-56070 Koblenz | michael.hoppe@wsv.bund.de |
| Germany | Mario Walterfang | Traffic Technologies Centre P.O.Box 100420, D-56070 Koblenz | mario.walterfang@wsv.bund.de |
| Germany | Evelin Engler | German Aerospace Centre Kalkhorstweg 53, D-17235 Neustrelitz | evelin.engler@dlr.de |
| Germany | Thoralf Noack | German Aerospace Centre Kalkhorstweg 53, D-17235 Neustrelitz | thoralf.noack@dlr.de |
| Germany | Jürgen Alberding | Alberding GmbH Freiheitstraße 124, D-15745 Wildau | alberding@alberding.eu |
| Germany | Dirk Stöcker | Alberding GmbH Freiheitstraße 124, D-15745 Wildau | stoecker@alberding.eu |
| Netherlands | Pieter Paap | NL Ministry of Transport P.O.Box 20904, NL-2500 EX The Hague | pieter.L.paap@quicknet.nl |
| Netherlands | Rob Langen | Rijkswaterstaat Lange Kleiweg 34, NL-2280 GK Rijswijk | rob.langen@rws.nl |
| Norway | Harald Åsheim | Norwegian Coastal Administration P.O.Box 466 NO-5501 Haugesund | harald.aasheim@kystverket.no |
| Poland | Marek Dziewicki | Maritime Office Gdynia ul. Chrzanowskiego 10, PL-81-338 Gdynia | marekdz@umgdy.gov.pl |
| Sweden | Johan Winell | Swedish Maritime Administration 60178 Norrköping, Sweden | johan.winell@sjofartsverket.se |
| UK | Peter Douglas | Northern Lighthouse Board 84 Georg St, Edinburgh, Scotland | peterd@nlb.org.uk |
| UK | Alan Grant | General Lighthouse Authorities Trinity House, the Quay, Harwich, UK | alan.grant@gla-rrnav.org |

ANNEX B: WG2 Draft programme

| eNAV 9 March 2011 | - Draft paper on alternatives to GNSS for Positioning and timing (to become output)(NW to draft – circulate before meeting) - First draft of receiver specification (with approval of Council, info paper to NAV ?) - Input on WiMax & S Band radar - Finalise G1060 |
| --- | --- |
| eNAV 10 Sept 2011 | - Second draft of receiver specification - Update Rec.R-129 (second reading) - Investigate polar issues |
| eNAV 11 Feb 2012 | - Final draft of receiver specification - Rec.121 first revision - Investigate disaster recovery issues |
| eNAV 12 Sept 2012 | - First review of WWRNP - Update R-121 - Follow up RX spec with RTCM |
| eNAV 13 Feb 2013 | - Second Review of WWRNP - Rec. R-135 first revision |
| eNAV 14 Sept 2013 | - Output WWRNP issue2. - Update R-135 |