Input paper: ENAV21-11.16

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		□  ENG		□  PAP						  Input

  ENAV		□  VTS								  Information

Agenda item	(from agenda)			11

Workplan Task Number / Technical Domain	VDES Tele -communication

Working Group					WG3

Author(s) / Submitter(s)				ORBCOMM

# ORBCOMM VDE Demonstration and Cooperation

# Purpose of the document

ORBCOMM invites the Committee to consider the submission for information of an existing VHF data exchange service using satellites, as well as the offer from ORBCOMM to conduct a demonstration of such a scenario.

# Related documents

There are no specific documents that relate to this input from ORBCOMM, however there are a number of VDES documents that could be considered in the context of this paper for supplementing the VDES. Document title: ENAV21-11.3 (ENAV20-11.5) VDES FAQ, ENAV21-11.4 (ENAV20-14.2.6) ITU-R M.2092-0 Draft revision 20170310_WD_WG3_Item_112 _REC-M.2092, ENAV21-11.4.1 Draft ITU-R Rec-M.2092 Tech char for a VHF data exchange system in the VHF mar mob band 20170816-JCJ review, ENAV21-11.5 D1.10 Strategy for Future Digital Communications v2.0
Originator: EfficienSea2, ENAV21-11.12 Draft Guideline 1117 Ed.2 VDES Overview Dec 2017-Proposed amendments

# Background

This paper seeks to inform members of IALA eNavigation Committee of how ORBCOMM would like to assist in the development of the VDES project and could provide additional spectrum that may be valuable for the VHF Data Exchange, whilst also showing the willingness of ORBCOMM to contribute to VDES.

The paper also sets out the outline of a VDE demonstration activity that ORBCOMM would like to undertake with interested parties of IALA.

ORBCOMM notes that IALA have been pursuing Machine-to-Machine (M2M) VHF data exchange service designed to provide connectivity between vessels and other vessels, as well as shore services to be used for safety of navigation information, weather, port services, Vessel Traffic Management, voyage advisory services and general eNavigation information exchange.

# Discussion

To avoid confusion, while all VHF data exchange developed to meet eNavigation is in effect, M2M, the paper will endeavour to distinguish between the up and coming IALA VDES VDE, and ORBCOMM M2M VDE, which operates in the 137 to 150 megahertz range of the VHF spectrum. This paper is an outline offering which will be expanded on to suit the information required, expressing ORBCOMM’s interest in VDE and willingness to provide/participate in a demonstration activity.

ORBCOMM supports the intention of IALA to evolve AIS to VDES, to use both terrestrial and satellite components to provide a global service for two way data exchange, and to clarify the communications and recommendations from IALA to move forward. ORBCOMM would like to build on and share with IALA their own experience of ORBCOMM’s M2M VHF two-way data messaging services. ORBCOMM recognizes that although the IALA initiative is based on evolving and enhancing safety of navigation information, weather, port services, Vessel Traffic Management, Voyage advisory services and general eNavigation information exchange, it is likely that commercial services will also evolve over this same period to provide two-way data exchange information serving both SOLAS and NON SOLAS fleets. Accordingly, ORBCOMM also recognise that the total data throughput required to meet all the requirements sited within the IALA DRAFT Guideline on User Requirements for VDES is likely to require more spectrum than has currently been sought via ITU for reuse of the former maritime voice channels.

ORBCOMM, separate from the IALA initiative, is currently capable of providing both VDE type services on their own VHF frequencies along with AIS and recently introduced a new service combining their VHF two-way data service with AIS for small vessels.

## 4.1	VHF data exchange & ORBCOMM

ORBCOMM have been providing M2M data services over VHF for almost 20 years and currently operates a global constellation of M2M satellites that include AIS payloads, which utilize 16 Gateway Earth Stations (GES) located around the world.

In addition to extensive deployments in the transportation and heavy equipment markets of almost 1 million subscribers, the service has been used for a number of years in the fisheries VMS market, as well as other niche maritime applications. The ORBCOMM VHF spectrum used is free of interference and has global coverage (current optimal performance is between the mid latitudes and tropic zones between the poles).

M2M means machine-to-machine which is what VDES really is since no operator involvement is intended for the communication of information. M2M communications may deliver information to be used within a system, but equally could be used for portraying information to an operator. When one considers the use of VHF data exchange for safety of Navigation, meteorological services, eNavigation or other messaging requirements. One of the principles of eNavigation is that it should be able to connect and install information without intervention of an operator, as such eNavigation, and indeed VDES are effectively M2M.

## 4.2Advantages of ORBCOMM VDE / M2M

ORBCOMM, more than other AIS and VHF satellite data exchange service providers, is aware of the limitations of detecting and operating services within the former voice maritime bandwidth.

If vessels were able to use the spectrum operated by ORBCOMM, as well as that formerly used for Maritime Voice, this would have great advantages to the mariner and to the project of VDES. The ORBCOMM VHF spectrum is separated from the maritime spectrum, as such there are some advantages to using the ORBCOMM frequencies for VHF data exchange in addition to those currently being sought via ITU within the former maritime voice band;

- there is no limitation of use anywhere in the world,

  - the spectrum does not require masking as does the IALA VDES

- In VHF congested areas there is a higher probability of successful communication and detection of messages by satellite than the former maritime voice band that was originally intended for terrestrial use.

- The ORBCOMM VHF spectrum will not suffer interference from or to the current voice or AIS channels.

- Reduction of the amount of traffic on the current Maritime VHF data Link.

- No unwanted voice transmission on parts of the IALA sought spectrum by current voice users.

- The ORBCOMM spectrum is VHF and so can use existing ships’ antennae.

- New receivers will be required by ships to use VDES, as such the spectrum range could be built into these new ship sets.

Furthermore, over time novel protocols will enable terrestrial VDES use to expand into a much greater population of vessels than today. For example, peer grouping protocols will enable vast fleets of vessels to be monitored, but instead of every vessel needing to transmit via AIS, only their changes of status would be transmitted to satellite. Additionally, the range of terrestrial use can be easily extended using multi hop protocols. However, these techniques, though efficient, will need multiple terrestrial channels for local use. Also as voice fades from the maritime spectrum, there will likely be a huge increase in requirements of data channels. Therefore, separating some of the satellite component of VDES from the former maritime Voice band seems to be prudent.

ORBCOMM would be happy to enter into discussions with others considering satellite services with respect to opening up frequencies to maritime VDE (M2M) users.

## 4.3Receivers

Currently AIS transponder manufacturers are eagerly following the discussions at IALA and ITU to design future vessel VHF transceivers. Since ORBCOMM currently manufactures and sells their own VHF M2M transceivers and chipsets, in large commercial scale, there is no technical reason as to why future ship VDES-based systems could not also be designed to use the frequencies operated by ORBCOMM. .  This would minimize development and integrations costs as well as reduce schedule and risk.

# Demonstration

ORBCOMM would like to collaborate with IALA to demonstrate how safety of Navigation information can be broadcast as a data message in real time from a vessel to all other vessels within the regional area using VHF.

The demonstration would comprise of a vessel transmitting messages over both the AIS and ORBCOMM VHF frequencies which would be received by the ORBCOMM constellation, and then routed, via the ORBCOMM network, to other vessels equipped with transceivers operating on the ORBCOMM VHF M2M spectrum.

This demonstration would serve to show that the information that might have historically been broadcast by a vessel to all other vessels in line of site (VHF range), or be relayed by a coast station, could use satellite VDE to exchange messages with all vessels outside terrestrial range in the region that may be effected by the event due to their route, and as such would need that information. It would be intended that the message or messages sent would correspond to regional NAVAREAs, of which are numbered 1 to 21.

ORBCOMM is capable of broadcasting these messages and/or designating them for specific recipients and/or groups of vessels.  For example messages might be appropriate for vessels of a certain route, draft, length or type, but not for others

The promulgation of information via ORBCOMM for the demonstration could be anywhere in the world, and as such cover any sea areas and not require masking such as the frequencies of the IALA VDES.

The actual message that would be exchanged would be discussed between IALA and other interested parties. But examples could include the following 30 kinds of messages identified by Maritime service portfolios and considered by IALA use cases for VDES and listed below. It is proposed that the message should be from UC-2.

| Use Case | MSP Reference |
| --- | --- |
| UC-1 – SAR Communications | MSP 9 - Telemedical Maritime Assistance Service (TMAS) MSP 16 - Search and Rescue (SAR) Service |
| UC-2 – Maritime Safety Information | MSP 5 - Maritime Safety Information (MSI) service MSP 13 - ice navigation service MSP 14 - Meteorological information service MSP 15 - real-time hydrographic and environmental information services |
| UC-3 – Ship Reporting | MSP 8 vessel shore reporting |
| UC-4 – Vessel Traffic Services | MSP 1 - VTS Information Service (IS) MSP 2 - VTS Navigation Assistance Service (NAS) MSP 3 - VTS Traffic Organization Service (TOS); MSP 4 - Local Port Service (LPS) MSP 6 - pilotage service MSP 7 - tugs service |
| UC-5 – Charts and Publications | MSP 11 - nautical chart service MSP 12 - nautical publications service MSP 15 - real-time hydrographic and environmental information services |
| Use Case 6 – Route Exchange (UC-6) | MSP 1 - VTS Information Service (IS) MSP 2 - VTS Navigation Assistance Service (NAS) MSP 3 - VTS Traffic Organization Service (TOS); MSP 4 - Local Port Service (LPS) MSP 5 - Maritime Safety Information (MSI) service MSP 6 - pilotage service MSP 7 - tugs service MSP 8 - vessel shore reporting MSP 10 - Maritime Assistance Service (MAS) MSP 11 - nautical chart service MSP 12 - nautical publications service MSP 13 - ice navigation service MSP 14 - Meteorological information service MSP 15 - real-time hydrographic and environmental information services MSP 16 Search and Rescue (SAR) Service |
| Use Case 7 – Logistics (UC-7) | MSP 7 tugs service |

The safety of Navigation message would be formed by;

- Information detected by a sensor on a buoy, float, drone or vessel

- A message formed by an operator such as an application specific message by a vessel or coast station

- A message coded under the international code of signals regarding an event that vessels have a duty to promulgate.

The message ideally would be language independent in line with the user guidelines and for demonstration. Those of alphanumeric composition would be coded under the international code of signals, to ensure that the reception could be understood by operators of any language covered by the code. In the future, it would be expected that a data dictionary would accompany all Enavigation and VDES systems, enabling language independent exchange of information of graphical or alphanumeric kind, so enabling the static description of the element to be within the dictionary, and only the variable or dynamic element of the information exchanged. ORBCOMM would be very happy to cooperate with any party that has already available for demonstration such a data e-dictionary.

Though for demonstration purposes the information exchanged would be destined for a portrayal display on the bridge. In practice, the information could equally be destined to be read, or integrated within a system such as an ECDIS or other information management system.

## 5.1Data to be transmitted

The demonstration message would be formed of information regarding safety of navigation and protection of the environment.

This could include Maritime Safety Information (MSI) as defined in IMO SOLAS V, regulation 4 (navigational warnings), SOLAS V, regulation 5 (meteorological services and warnings) and SOLAS V, regulation 9 (hydrographic services) SOLAS V, regulation 31 (Danger Messages).

The requirement includes the ability to send information to a predetermined area (i.e. NAVAREA and METAREA) or an area of particular interest defined by an administration.

### Data - Meteorological Services and Warnings / Navigational Warnings (UC-2.1)

The message will be in the form of alphanumeric coded text, but could equally be a graphic.

Provide information to a specified vessel.

Provide information to a shore authority (ship to shore).

Provide weather information to ships in the area (ship to ship) based on region or route.

Interface – For demonstration purposes the message would be formed by an operator as a plain message, however if templates are available this could be considered as an option.

The message will be language independent, however as data libraries are not yet available, the International code of signals will be used to code and decode the message.

## Demonstration Technical System options

### ORBCOMM M2M Device

ORBCOMM would provide transceivers to 1 or more vessels that would be fitted as a temporary device. It is envisaged that a temporary antenna would be mounted on the monkey island and a cable run down to the bridge via an open window, or door during demonstration time, and so not require any cutting or drilling of the superstructure for access. ORBCOMM would also provide a simple antenna that could be connected to the ship rails on the Monkey Island.

It is envisaged that vessels would make several demonstration windows available. Ideally at least two vessels will participate, one to raise the message via AIS or VDE ship to satellite, and then the second to receive. The originator of the message could alternatively be a coastal station, but for reality of a demonstration a vessel would be preferred.

The participating vessels would inform, likely via their onboard Inmarsat system, their availability to perform a demonstration while en-route. The ORBCOMM equipment would be placed on board the vessel either at a suitable port, or during a transit of a canal such as the Panama Canal or at a point where the vessel is replenishing stores or personnel. This will be a vessel of opportunity.

Since this is a VHF demonstration, ORBCOMM would require the cooperation of the vessel operators to use an ORBCOMM M2M VHF transceiver or other device provided by ORBCOMM.

# References

None currently.

# Action requested of the Committee

The Committee is requested to: (Body text)

Consider how ORBCOMM can best provide input to WG3 on their experience of M2M VHF Data Exchange

Amend the current guidelines for VDES which currently states that VHF exchange is limited to the Former AIS frequencies so as to recognise the VHF services hat already are capable of providing VHF data exchange for vessels.

Discuss how ORBCOMM VDE, and indeed any other VHF satellite operators, and IALA VDE can work together as presently the IALA VDE does not recognise an existing service that exists today.

Discuss how novel terrestrial protocols can enhance the efficiency of VHF data exchange between vessels to ensure all vessel are monitored in a more efficient way than today using Peer group management and other network management techniques. This is also vital to make the satellite segment efficient as possible.

Discuss how information can use these techniques to provide information handover and alerts to ensure all vessels are aware of the existence of information, so that hey might use the most efficient method of retrieving it depending on their geographic location.

Make available or study a real estimate of the quantity of information that will be required to use VDE, based on codification using data libraries and alphanumeric messages provide the information required. This is necessary to ensure that the current satellite segment has the throughput potential to handle the traffic.