Recommendation ITU-R M.2092-0+ (09/2017)

Technical characteristics for a VHF data exchange system in the VHF maritime mobile band ##### WORKING VERSION – BE WARNED !!! CHEERS, IALA WG3

M Series Mobile, radiodetermination, amateur and related satellite services

REVISON FILE NAME: 20170921-1246_WD_item182_SPI_REC-M.2092.docx

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2015

ITU 2015

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  M.2092-0,

Technical characteristics for a VHF data exchange system 
in the VHF maritime mobile band

(2015)

Scope

This Recommendation provides the technical characteristics of a VHF data exchange system (VDES) which integrates the functions of VHF data exchange (VDE), application specific messages (ASM) and the automatic identification system (AIS) in the VHF maritime mobile band (156.025-162.025 MHz).

Keywords

Maritime, VHF, VDES, ASM, data, exchange

Abbreviations/Glossary

3GPP	Third generation partnership project

ACK	Acknowledgement

ADDC	Assigned data transfer

ACPR 	Adjacent channel power ratio

AIS	Automatic identification system

AOS	Acquisition-of-signal

APSK	Amplitude phase shift keying

ARQ	Automatic repeat request

ARSC	Announcement response channel

ASC	Announcement signalling channel

ASM	Application-specific messages

ATDMA	Allocated Time-Division Multiple Access

AWGN	Additive white Gaussian noise

BBSC 	Bulletin board signalling channel

BCH	Bose Chaudhuri Hocquenghem, an error-correcting-code

BER	Bit error rate

BPSK	Binary phase shift keying

BT	Bandwidth-time

CEPT	European conference of postal and telecommunications administrations

CDMA	Code division multiple access

CG 	Coding gain

CIR	Carrier to interference ratio

C/M 	Carrier to multipath

CNR	Carrier to noise ratio

COMSTATE	Communication state

Control Station	Coast base station, satellite or other VDES station that is transmitting bulletin board and provides routing services to the maritime cloud

CPM	Continuous phase modulation

CQI	Channel quality indicator

CR	Code rate

CRC	Cyclic redundancy check

CRL	Configuration revision level

CS	Carrier sense

CIRM	Comité international radio maritime

CSTDMA 	Carrier sense time division multiple access

CW	Continuous wave

DA	Doherty amplifier

DLS	Data link service

DPD	Digital pre-distortion

EDN 	End delivery notification

EDF	End delivery failure

EIRP	Equivalent isotropic radiated power (e.i.r.p.)

ERP	Effective radiated power (e.r.p.)

ET	Envelope tracking

FATDMA	Fixed access time-division multiple access

FCS 	Frame check sequence

FEC 	Forward error correction

FIFO	First-in first-out

GMSK	Gaussian-filtered minimum shift keying

GNSS	Global navigation satellite system

HS	Hexslots

IALA	International association of marine aids to navigation and lighthouse authorities

ICAO	International civil aviation organization

ID	Identification

IEC	International electrotechnical commission

IMO	International maritime organization

IP	Internet protocol

LC	Logical channels

LEO	Low-earth orbiting

LFSR	Linear feedback shift register

LME	Link management entity

LNA	Low noise amplifier

LOS	Loss-of-signal

LSB	Least significant bit

MEO	Medium-earth orbiting

MAC	Media access control

MCS 	Modulation and coding scheme

MDC 	Multicast data channel

MMSI	Maritime mobile service identity

MSB	Most significant bit

NF	Noise figure

NM	Nautical mile

NRZI	Non-return to zero inversion

OFDM 	Orthogonal frequency division multiplexing

OSI	Open systems interconnection

PAPR 	Peak to average power ratio

PC	Physical channels

PL	Physical layer

PFD	Power flux-density

ppm	parts per million

PSK	Phase shift keying

QAM	Quadrature amplitude modulation

QPSK	Quadrature phase shift keying

RADC	Random access short messaging channel

RATDMA 	Random access time-division multiple access

RAC 	Random access channel

RF 	Radio frequency

RSC	Recursive systematic convolutional

RQSC	Random access resource request

RR	Radio regulations

RSSI	Received signal strength indication

SCTDMA 	Slot carrier sense time division multiple access

SFTP	Secure file transfer protocol

SI	Selection interval

SMTP	Simple mail transfer protocol

SNMP	Simple network management protocol

SNR	Signal to noise ratio

SOLAS	Safety of life at sea convention

SOTDMA	Self-organized time division multiple access

SS	Spreading sequences

Sym	Symbol

SYNC	Synchronization

TBB	Terrestrial bulletin board

TBBSC 	Terrestrial bulletin board signalling channel

TDMA	Time division multiple access

UDC	Unicast data channel

UDP	User data protocol

UTC	Coordinated universal time

VDE	VHF data exchange

VDES	VHF data exchange system

VDE-SAT	VHF data exchange-satellite

VDL	VHF data link

VHF	Very high frequency

References

{RD-1}	ETSI EN 302 583 (V1.2.1) – Digital Video Broadcasting (DVB); Framing Structure, channel coding and modulation for Satellite Services to Handheld devices (SH) below 3 GHz.

{RD-2}	TM Synchronization and Channel Coding. Recommendation for Space Data System Standards, CCSDS 131.0-B-2. Blue Book. Issue 2. Washington, D.C.: CCSDS, August 2011.

{RD-3}	R. Mueller, On Random CDMA with Constant Envelope, ISIT 2011.

{RD-4}	Recommendation ITU-R P.372 – Radio Noise.

{RD-5}	Recommendation ITU-T V.42 (03/2002) – Series V: Data Communication over the Telephone Network – Error control – Error-correcting procedures for DCEs using asynchronous-synchronous conversions.

The ITU Radiocommunication Assembly,

considering

a)	that the International Maritime Organization (IMO) has a continuing requirement for a universal shipborne automatic identification system (AIS);

b)	that the use of a universal shipborne AIS allows efficient exchange of navigational data between ships and between ships and shore stations, thereby improving safety of navigation;

c)	that the VHF data exchange system (VDES) should use appropriate access schemes that ensure the protection of AIS while making efficient use of the spectrum and accommodate all users;

d)	that while AIS is used primarily for surveillance and safety of navigation purposes in ship-to-ship use, ship reporting and vessel traffic services applications, a growing need for other maritime safety related communications has developed;

e)	that the VDES shall give priority to AIS, and also accommodate future expansion in the number of users and diversification of data communications applications, including vessels which are not subject to IMO AIS carriage requirements, aids to navigation and search and rescue;

f)	that the VDES has data communications capacity and technical characteristics that support the harmonized collection, integration, exchange, presentation and analysis of marine information on board and ashore by electronic means to enhance berth to berth navigation and related services for safety and security at sea and protection of the marine environment,

recognizing

that the implementation of VDES must ensure that the functions of digital selective calling, AIS and voice distress, safety and calling communication (Channel 16), are not impaired,

noting

that Report ITU-R M.2371 describes the use cases and requirement for VDES,

recommends

1	that VDES should be designed in accordance with the operational characteristics given in Annex 1 and the technical characteristics and examples given in Annexes 2 to 7;

2	that applications of the VDES which make use of application specific messages (ASM) designed for AIS, as defined in Recommendation ITU-R M.1371 should also take into account the international application identifier branch, as specified in IMO SN.1/Circ. 289, maintained and published by IMO;

3	that the design and installation of VDES should also consider relevant technical requirements, recommendations and guidelines published by IMO, IEC and IALA.

Annex 1

Operational characteristics of a VHF data exchange system 
in the VHF maritime mobile band

# 1	General

1.1	The system should give its highest priority to the automatic identification system (AIS) position reporting and safety related information.

1.2	The system installation should be capable of receiving and processing the digital messages and interrogating calls specified by this Recommendation.

1.3	The system should be capable of transmitting additional safety information on request.

1.4	The system installation should be able to operate continuously while under way, moored or at anchor.

1.5	The system should use for the terrestrial links time-division multiple access (TDMA) techniques, access schemes and data transmission methods in a synchronized manner as specified in the Annexes.

1.6	The system should be capable of various modes of operation, including the autonomous, assigned and polled modes.

1.7	The system should provide flexibility for the users in order to prioritize some applications and, consequently, adapt some parameters of the transmission (robustness or capacity) while minimizing system complexity.

1.8	The system should address the use cases identified in Report ITU-R M.2371.

# 2	General description of the VDES

The VDES provides a variety of means for the exchange of data between maritime stations, ship-to-ship, ship-to-shore, shore-to ship, ship-to-satellite and satellite-to-ship. The VDES functionally includes the AIS, either by integration, by interface connection or by radio frequency connection.

# 3	VHF data exchange system functions and frequency usage

The VDES functions and frequency useage are illustrated pictorially in Figure A1-1.

Figure A1-1

VHF data exchange system functions and frequency usage

NOTE – SAT Up is receive-only by satellite.

## 3.1	VHF data exchange system channel usage in accordance with RR Appendix 18

### 3.1.1	VHF data exchange system: data exchange between terrestrial stations

–	AIS 1 (channel 2087) and AIS 2 (channel 2088) are AIS channels, in accordance with Recommendation ITU-R M.1371

–	ASM 1 (channel 2027) and ASM 2 (channel 2028) are the channels used for application specific messages (ASM)

–	VDE1-A lower legs (channels 1024, 1084, 1025, 1085) are ship-to-shore VDE

–	VDE1-B upper legs (channels 2024, 2084, 2025, 2085) are shore-to-ship and ship-to-ship VDE.

### 3.1.2	VHF data exchange system: data exchange between satellites and terrestrial stations

–	AIS 1 (channel 2087) and AIS 2 (channel 2088) are terrestrial AIS channels that are also used as uplinks for receiving AIS messages by satellite

–	Long Range AIS using channel 75 and channel 76 are specified channels to be used as uplinks for receiving AIS messages by satellite. SAT Up1 (channel 2027) and SAT Up 2 (channel 2028) are used for receiving ASM by satellite

–	SAT Up3 (channels 1024, 1084, 1025, 1085, 1026 and 1086) are used for ship-to-satellite VDE uplinks

–	SAT Downlink (channels 2024, 2084, 2025, 2085, 2026 and 2086) are used for satellite-to-ship VDE downlinks.

## 3.2	Identification

Identification and location of all active maritime stations is provided automatically by means of the AIS. All VDES stations should be uniquely identified. For the purpose of identification, a unique numerical identifier is used as defined by the following:.

If the unique identifier has a range which is less than 999999999, then this number is defined by the latest version of Recommendation ITU-R M.585.

If the unique identifier has a range which is greater than 999999999, then this is number is free form.

## 3.3	Presentation interface protocol

For VDES transceivers:

–	data may be input via the presentation interface to be transmitted by the VDES station;

–	data received by the VDES station should be output through the presentation interface.

### 3.4	Technical characteristics

#### 3.4.1	Shipborne VHF data exchange system receivers are protected

As in AIS, shipborne VDES receivers are on the upper legs of RR Appendix 18, 4.6 MHz above the lower legs, which facilitates protection by filtering from receiver blocking by ships VHF radios.

#### 3.4.2	SAT Downlink

The satellite downlink complies with the power flux-density (PFD) mask described in Table A4-1 to minimize interference to terrestrial services and to maximize reception by ship VDES stations.

#### 3.4.3	VDE1 uses both legs of the duplex channels

Channel capacity is utilized for the duplex channels in VDE1 by using the lower legs (VDE1-A) for ship-to-shore and the upper legs (VDE1-B) for shore-to-ship and ship-to-ship digital messaging.

Table A1-1 describes the RR Appendix 18 channels used for the various applications of VDES.

TABLE A1-1

RR Appendix 18 channels for VHF data exchange systems applications: Automatic identification system, application specific messages, VHF data exchange

| RR Appendix 18 channel number |  | Transmitting frequencies (MHz) |  |
| --- | --- | --- | --- |
|  |  | Ship stations (ship-to-shore) (long range AIS) Ship stations (ship-to-satellite) | Coast stations Ship stations (ship-to-ship) Satellite-to-ship |
| AIS 1 |  | 161.975 | 161.975 |
| AIS 2 |  | 162.025 | 162.025 |
| 75 (long range AIS) |  | 156.775 (ships are Tx only) | N/A |
| 76 (long range AIS) |  | 156.825 (ships are Tx only) | N/A |
| 2027 (ASM 1) |  | 161.950 (2027) | 161.950 (2027) |
| 2028 (ASM 2) |  | 162.000 (2028) | 162.000 (2028) |
| 24/84/25/85 (VDE 1) 24 84 25 85 | 24/84/25/85/26/86 (Ship-to-satellite, satellite-to-ship) 24 84 25 85 26 86 | 100/150 kHz channel (24/84/25/85, lower legs (VDE1-A) merged) Ship-to-shore (24/84/25/85/26/86) Ship-to-satellite | 100/150 kHz channel (24/84/25/85, upper legs (VDE1-B) merged) Ship-to-ship, Shore-to-ship (24/84/25/85/26/86) Satellite-to-ship |
|  |  | 157.200 (1024) | 161.800 (2024) |
|  |  | 157.225 (1084) | 161.825 (2084) |
|  |  | 157.250 (1025) | 161.850 (2025) |
|  |  | 157.275 (1085) | 161.875 (2085) |
|  |  | 157.300 (1026) | 161.900 (2026) |
|  |  | 157.325 (1086) | 161.925 (2086) |

3.4.4	VHF data exchange system functions and frequency usage engineer’s perspective

The VDES functions and frequency useage from an engineer’s perspective are illustrated pictorially in Figure A1-2.

|  |  | - | - |
| --- | --- | --- | --- |

Figure A1-2

VHF data exchange system functions and frequency usage engineer’s perspective

# 4	Functions of the VDES

The priority and timing of transmissions shall be in accordance with the following service priorities:

Highest 	Priority 1: AIS transmissions on AIS channels

Priority 2: specified and approved ASM transmissions on ASM channels

Priority 3: all other data exchange on VDE channels

The VDES receivers shall always be active. It is understood that transmissions by own equipment will impair reception by own equipment on own ship.

The VDES should support the following:

## 4.1	Automatic Identification System

The AIS will operate as defined by Recommendation ITU-R M.1371.

## 4.2	Application-Specific Messages – Annex 2

Annex 2 describes the characteristics of the ASM channel that will support applications specific messages in order to improve the efficiency of application-specific message transmissions and to protect the original function of the AIS.

## 4.3	VDE terrestrial – Annex 3

Annex 3 describes the characteristics of the VDE terrestrial channels providing an efficient terrestrial data transfer link enabling a wide variety of applications for the maritime community.

## 4.4	VDE satellite – Annex 4

Annex 4 describes the characteristics of a satellite downlink that will support multi-cast multi-package data transfers and shore originated unicast multi-package data transfers via satellite.

## 4.5	VDES sharing options – Annex 6

Annex 6 describes the characteristics necessary for each component of the VDES to share the available spectrum such that impact between services is minimized and AIS is respected.

## 4.6	VDES original design considerations – Annex 7

Annex 7 is an informative annex that provides additional information on the technical consideration of the VDES. It identifies aspects of both terrestrial and satellite VDE components, including access scheme options, antenna designs, and system sharing.

# 5	Common technical elements of the VDES

This section describes those elements of the VDES that may be common across the ASM and VDE channels.

## 5.1	Protocol layer overview

The VDES architecture should utilize the open systems interconnection layers 1 to 4 (physical layer, link layer, network layer, transport layer) as illustrated in Fig. A1-3.

Figure A1-3

Seven layer OSI model

Application layer

Presentation layer

Session layer

Transport layer

Network layer

Link layer

Physical layer

Responsibilities of the OSI layers for preparing VDES data for transmission:

## 5.2	Physical layer [changed SP to heading 2]

This layer provides transmission and reception of raw bit streams over a physical medium including signal modulation, filtering/shaping upon transmission, and amplification, filtering, time and frequency synchronization, demodulation, and decoding upon reception.

### 5.2.1	Transmission accuracy figures [changed SP to heading 3]

#### 5.2.1.1	Symbol timing accuracy (at the output)

The timing accuracy of the transmit signal should be better than 5 ppm.

#### 5.2.1.2	Transmitter timing jitter

The timing jitter should be better than 5% of the symbol interval (peak value).

#### 5.2.1.3	 Slot transmission accuracy at the output

The slot transmission accuracy should be better than 100 µs peak relative to UTC time reference for ship stations.

### 5.2.2	Frame Structure [changed SP to heading 3]

The system uses the Recommendation ITU-R M.1371 concept of a frame. A frame equals one (1) minute and is divided into 2 250 slots. Access to the data link is, by default, given at the start of a slot.  The VDES frame structure is identical and synchronized in time to UTC (as in AIS). The general slot formats are is shown in Fig. A1-4A and B. The slot structure for each service is shown in Fig. A1-4. Each element is described in Table A1-X in section 5.2.7. Table A1-2 shows the resulting net bitrates (Data bits of the slot structure).

Figure A1-4A ASM-TER and VDE-TER general packet format

Figure A1-4B ASM-SAT and VDE-SAT general packet format

Training sequence

27 sym

Ramp up

4 symbols

Link CFG

ID

16 sym

Data with CRC-32

192, 448, 704, 960 symbols

Ramp Down

4 symbols

Guard

13   symbols

ASM

Bandwidth 16 kHz

9.6 ksym/s

26.67 ms

256 symbols

Training sequence

27 sym

Ramp up

16 symbols

Link CFG

ID

16 sym

Data with CRC-32

896 symbols

Ramp Down

16 symbols

Guard

53 symbols

VDE

Bandwidth 50 kHz

38.4 ksym/s

1024 symbols

26.67 ms

Training sequence

27 sym

Ramp up

32 symbols

Link CFG

ID

16 sym

Data with CRC-32

1792 symbols

Ramp Down

32 symbols

Guard

[149] symbols

VDE

Bandwidth 100 kHz

76.8 ksym/s

2048 symbols

26.67 ms

Training sequence

27 sym

Ramp up

8 symbols

Link CFG

ID

16 sym

Data with CRC-32

432 symbols

Ramp Down

8 symbols

Guard

21 symbols

VDE

Bandwidth 25 kHz

19.2 ksym/s

512 symbols

26.67 ms

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

27 sym

Ramp up

4 symbols

Link CFG

ID

16 sym

Data with CRC-32

192, 448, 704, 960 symbols

Ramp Down

4 symbols

Guard

13   symbols

ASM

Bandwidth 16 kHz

9.6 ksym/s

26.67 ms

256 symbols

Training sequence

27 sym

Ramp up

16 symbols

Link CFG

ID

16 sym

Data with CRC-32

896 symbols

Ramp Down

16 symbols

Guard

53 symbols

VDE

Bandwidth 50 kHz

38.4 ksym/s

1024 symbols

26.67 ms

Training sequence

27 sym

Ramp up

32 symbols

Link CFG

ID

16 sym

Data with CRC-32

1792 symbols

Ramp Down

32 symbols

Guard

[149] symbols

VDE

Bandwidth 100 kHz

76.8 ksym/s

2048 symbols

26.67 ms

Training sequence

27 sym

Ramp up

8 symbols

Link CFG

ID

16 sym

Data with CRC-32

432 symbols

Ramp Down

8 symbols

Guard

21 symbols

VDE

Bandwidth 25 kHz

19.2 ksym/s

512 symbols

26.67 ms

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

#### 5.2.2.1	Frame hierarchy definition [changed SP to heading 4]

The frame hierarchy is shown in Fig. A1-5. The frame hierarchy definition is independent of the assigned bandwidth to the VDE channel

Figure A1-5

Frame Hierarchy for shared frequency

##### 5.2.2.1.1	Slot [changed SP to heading 5]

The slot is a time interval of approximately 26.667 ms (60 000 / 2 250 = 80/3 ≈ 26.667).

The slot should be cyclically numbered by a slot number ranging from 0 to 5.

##### 5.2.2.1.2	Hexslot [changed SP to heading 5]

Six slots should form a Hexslot (HS). The HS has duration of 160 ms.

The HS should be numbered cyclically from 0 to 4. The HS should be incremented after every 6 slots.

##### 5.2.2.1.3	Uberslot [changed SP to heading 5]

Five Hexslots should form a Uberslot (US). The US should have duration of 800 ms.

The US should be numbered by a US Number. The US should be cyclically numbered from 0 to 14. The US should be incremented whenever the Hexslot returns to 0.

##### 5.2.2.1.4	Sub frame  [changed SP to heading 5]

Fifteen US should form a sub frame. The sub frame should have a duration of 12 seconds. The sub frame should be numbered by a sub frame number. The PL-frame should be cyclically numbered from 0 to 4. The sub frame should be incremented whenever the US returns to 0.

### 5.2.3	Burst transmission structure  [changed SP to heading 3]

#### 5.2.3.1	Ramp up

The ramp up time from −50 dBc to −1.5 dBc of the power shall controlled rise time and occur in 416 µs. A gradual ramp-up period provides important spectral shaping to reduce energy spread outside the desired signal modulation bandwidth, and reduces interference to other users of the current and adjacent channel. The Ramp-up pattern shown in TABLE A1-1A is mapped like defined in section 5.2.3.3 Bit mapping for training sequence.

Table A1-1a. Ramp-up symbol patterns for VDES

| Link Config ID |  | Ramp-Up Time (ms) | Ramp-Up # of transmission symbols | Ramp-UP Pattern |
| --- | --- | --- | --- | --- |
| ASM-TER CH BW 16kHz | 1, 2, 3, 5, 6, 7, 8, 9, 10 | 0.41 | 4 | 0, 0, 1, 1 |
| VDE-TER CH BW 25kHz | 11, 12, 13 | 0.41 | 8 | 0, 0, 1, 1, 0, 0, 1, 1 |
| VDE-TER CH BW 50kHz | 14, 15, 16 | 0.41 | 16 | 0, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1 |
| VDE-TER CH BW 100kHz | 17, 18, 19 | 0.41 | 32 | 0, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1 |

#### 5.2.3.2	Training sequence [changed SP to heading 4]

Table A1-2 shows the syncwords used for VDES.

TABLE A1-2

SYNCWORDS for VDES

| Usage | Symbol size | Sequence | Type | Comment |
| --- | --- | --- | --- | --- |
| ASM-TER | 27 | 1 1111100110101 0000011001010 | 1+ Barker13+ inverted Barker13 | TC under consideration |
| VDE-TER |  |  |  |  |
| ASM-SAT | 27 | 010001010010010000000110011 | Best autocorrelation for differential detection | Based on search. Looses 0.3% of packets at Es/N0 of 5 dB |
| VDE-SAT |  |  |  |  |
| VDE-SAT | 48 | 000100011110011011000001010111011010110111101000 |  | Needs to work at Es/N0 of -2 dB |

The Double Barker sequence used for ASM-TER and VDE-TER allows for detection of the 2 correlation peaks and the 13 bit known noise inbetween. Furthermore, the correlation peak size indicates frequency offset.

#### 5.2.3.3	Bit mapping for training sequence and signal information

For training and signal information, following mapping applies:

–	1 maps to π/4 QPSK symbol 3 (1, 1) (see Figure A1-10

–	0 maps to π/4 QPSK symbol 0 (0, 0).

For π/4 QPSK bit mapping, see § 5.2.8.

#### 5.2.3.4	Link Configuration Id [changed SP to heading 4]

The Link Configuration Id defines the channel configurations.  The Link Configuration Id is used to index the table of channel configurations, see Table A1-6.

The Link Configuration Id follows the training sequence for transmissions, see Figure A1-4A and B.

The Link Configuration Id consists of 6 bits (D0, D1, D2, D3, D4, D5) encoded into a sequence of 32 bits using biorthogonal (32,6) code.

The link configuration id is not used by the SAT link.

#### 5.2.3.5	Data with CRC-32  [changed SP to heading 4]

The data payload with its appended CRC-32 is interleaved (refer to Table A1-3), encoded (refer to § 5.2.4.1), scrambled (refer § 5.2.6) and bit mapped.

Unused payload data is zero-filled.

Figure A1-6

Order of operations for TDMA; if CR=1 FEC is not applied

#### 5.2.3.6	Bit scrambling  [changed SP to heading 4]

Scrambling of the user data is required to avoid the power spectral density to be concentrated in the narrow band. Refer to § 5.2.6 for the detailed definition of the scrambler sequence.

#### 5.2.3.7	Guard time  [changed SP to heading 4]

The guard time consists of the ramp down time from full power to −50 dBc of less than or equal to 416 µs. The remaining time is for delay and jitter.

### 5.2.4	Forward Error Correction  [changed SP to heading 3]

#### 5.2.4.1	Encoder Structure

This paragraph defines the general structure of the forward error correction encoder to be used on the satellite and the terrestrial component of the VDES. The overall structure follows the specification in the ETSI EN 302 583 standard {RD-1}.

The general encoder structure is depicted in Fig. A1-7. The encoder consists of two recursive systematic convolutional (RSC) encoders concatenated in parallel. Each encoder produces 3 output bits per input bit. The first RSC encoder produces the bits X, Y0 and Y1, while the second encoder produces the bits X’, Y’0 and Y’1. The <pi> block in Fig. A1-7 represents the interleaving function as described in section 5.2.4.3.

The first encoder gets as input a word u of k bits, with k, as specified in § 5.2.4.3. The second encoder input is denoted by u’ and it is a permuted version of the vector u. The permutation is performed according to the definition provided in § 3.5.3. The input u is the data (including padding and CRC), with MSB of each byte first. For example if the data is 0x7F, 0xA5, … u will be 01111111 10100101 ….

Figure A1-7

Turbo encoder structure (high-level)

#### 5.2.4.2	Constituent codes  [changed SP to heading 4]

The constituent codes are specified by the transfer function

GD=1n0(D)d(D)n1(D)d(D)

where

n0D=1+D+D3

n1D=1+D+D2+D3

dD=1+D2+D3.

The constituted encoder definition is provided in Fig. A1-8. For the first k clocks the switch is in position (a), i.e. information is fed into the encoder. For the subsequent 6 clocks, the switch is moved to position (b) to handle the RSC trellis termination. In the first 3 termination clocks, only the RSC 1 (upper branch) is output, while in the subsequent 3 termination clocks, only the output of RSC 2 (lower branch) is provided. The termination is thus given by the sequence of 6 termination bits (X, Y0, Y1, X’, Y’0, Y’1) with X output first.

Figure A1-8

RSC code encoder

#### 5.2.4.3	Interleaver definition  [changed SP to heading 4]

The interleaver specification follows {RD-2}.

First factorize k = k1 k2, where the parameters k1 and k2 depend on the choice of the respective code, where k is the information block length. Then select prime numbers and puncturing parameters values as given in Table A1-3.

TABLE A1-3 – Interleaver and puncturing parameters for different information lengths/code rates

| Link Conf ID | Nominal code rate | Information length | k1¦k2 | p1¦p2¦p3¦ p4¦ p5¦ p6¦ p7¦ p8 | Puncturing ID | Tail ID |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 3/4 | 952 | 4¦238 | 113¦31¦59¦163¦29¦181¦101¦11 | 8 | 8a |
| 5* | 3/4 | 288 | 2¦144 | 31¦37¦43¦47¦53¦59¦61¦67 | 8 | 8 |
| 6* | 3/4 | 672 | 2¦336 | 31¦37¦43¦47¦53¦59¦61¦67 | 8 | 8 |
| 7* | 3/4 | 1056 | 2¦528 | 31¦37¦43¦47¦53¦59¦61¦67 | 8 | 8 |
| 8* | 1/2 | 192 | 2¦96 | 31¦37¦43¦47¦53¦59¦61¦67 | 6 | 6 |
| 9* | 1/2 | 448 | 2¦224 | 31¦37¦43¦47¦53¦59¦61¦67 | 6 | 6 |
| 10* | 1/2 | 704 | 2¦352 | 31¦37¦43¦47¦53¦59¦61¦67 | 6 | 6 |
| 11 | 1/2 | 432 | 2¦216 | 31¦37¦43¦47¦53¦59¦61¦67 | 6 | 6 |
| 12 | 3/4 | 972 | 2¦486 | 31¦37¦43¦47¦53¦59¦61¦67 | 8 | 8 |
| 13 | 3/4 | 1296 | 2¦648 | 31¦37¦43¦47¦53¦59¦61¦67 | 8 | 8 |
| 14 | 1/2 | 896 | 2¦448 | 31¦37¦43¦47¦53¦59¦61¦67 | 6 | 6 |
| 15 | 3/4 | 2016 | 4¦504 | 31¦37¦43¦47¦53¦59¦61¦67 | 8 | 8 |
| 16 | 3/4 | 2688 | 4¦672 | 31¦37¦43¦47¦53¦59¦61¦67 | 8 | 8 |
| 17 | 1/2 | 1792 | 4¦448 | 31¦37¦43¦47¦53¦59¦61¦67 | 6 | 6 |
| 18 | 3/4 | 4032 | 4¦1008 | 31¦37¦43¦47¦53¦59¦61¦67 | 8 | 8 |
| 19 | 3/4 | 5376 | 8¦672 | 31¦37¦43¦47¦53¦59¦61¦67 | 8 | 8 |
| 20 | 1/4 | 96 | 2¦48 | 37¦83¦211¦61¦107¦101¦149¦167 | 2 | 2a |
| 21 | 2/3 | 736 | 2¦368 | 139¦17¦241¦ 47¦109¦11¦29¦163 | 7a | 7a |
| 22 | 2/3 | 3120 | 8¦390 | 17¦31¦61¦67¦101¦103¦113¦131 | 7a | 7b |
| 23 | 2/3 | 4544 | 4¦1136 | 197¦157¦13¦191¦241¦101¦149¦109 | 7a | 7b |
| 24 | 5/6 | 3788*2 | 4¦947 | 83¦251¦ 7¦ 47¦ 79¦ 5¦227¦73 | 9 | 9 |

*) No previous definitions or simulations results available, but a default configuration suggested.

Table A1-3 will be extended as different information block lengths are defined.

This FEC will be calculated by first choosing prime numbers pq, q  (1, ..., 8) as given in the table A1-3.

The following operations shall be performed for s  (1, ..., k) to obtain the permutation numbers π(s):

m=s-1 mod 2

i=floor (s-1 /(2k2))

j=floor (s-1 /2))-ik2

t=19i+1 mod (k1/2)

q=t mod 8+1

c=pqj+21m mod k2

πs=2t+ck1/2+1-m

The permutation numbers shall be interpreted such that the sth bit read out after interleaving is the π(s)th bit of the input information block.

#### 5.2.4.4	Rate Adaptation

Rate adaptation is obtained by puncturing the encoder output as in § 5.3.1 of {RD-1}, as recalled in Table A1-4 for the first k clocks, and as in {RD-1}.

The puncturing table for the termination part is given in Table A1-5. The two last rows of Table A1-4 are not part of {RD-1}.

TABLE A1-4

Puncturing patterns for data bit periods

| Punc. Pattern ID | Code Rate | Punc. Pattern (X; Y0; Y1; X’; Y’0; Y’1 ¦ X; Y0; Y1; X’; Y’0; Y’1 ¦ …) |
| --- | --- | --- |
| 0 | 1/5 | 1;1;1;0;1;1 |
| 1 | 2/9 | 1;0;1;0;1;1 ¦ 1;1;1;0;1;1 ¦ 1;1;1;0;0;1 ¦ 1;1;1;0;1;1 |
| 2 | 1/4 | 1;1;1;0;0;1 ¦ 1;1;0;0;1;1 |
| 3 | 2/7 | 1;0;1;0;0;1 ¦ 1;0;1;0;1;1 ¦ 1;0;1;0;0;1 ¦ 1;1;1;0;0;1 |
| 4 | 1/3 | 1;1;0;0;1;0 |
| 5 | 2/5 | 1;0;0;0;0;0 ¦ 1;0;1;0;0;1 ¦ 0;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 0;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 0;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 1;0;1;0;0;1 ¦ 0;0;1;0;0;1 |
| 6 | 1/2 | 1;1;0;0;0;0 ¦ 1;0;0;0;1;0 |
| 7 | 2/3 | 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;1;0;0;1 |
| 7a | 2/3 | 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;1;0;0;1;0 |
| 8 | 3/4 | 1;0;1;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;1 |
| 9 | 5/6 | 1;0;0;0;0;0 ¦ 1;1;0;0;0;0 ¦ 1;0;0;0;1;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 |
| For each rate, the puncturing table shall be read first from left to right and then from top to bottom. |  |  |

Within a puncturing pattern, a ‘0’ means that the symbol shall be deleted and a ‘1’ means that a symbol shall be passed. A ‘2’ or a ‘3’ means that two or three copies of the symbol shall be passed. This is relevant for the termination periods. In particular

–	For the rate 1/5 turbo code (Punct_Pat_ID=0), the tail output symbols for each of the first three tail bit periods shall be XXXY0Y1, and the tail output symbols for each of the last three tail bit periods shall be X’X’X’Y’0Y’1.

–	For the rate 2/9 turbo code (Punct_Pat_ID=1), the tail output symbols for the first and the second output period shall be XXXY0Y1, for the third output period XXY0Y1, for the fourth and fifth output period X’X’Y’0Y’1, and for the sixth (last) output period X’X’X’Y’0Y’1.

–	For the rate 1/4 turbo code (Punct_Pat_ID=2), the tail output symbols for each of the first three tail bit periods shall be XXY0Y1, and the tail output symbols for each of the last three tail bit periods shall be X’X’ Y’0Y’1.

All other code rates shall be processed similar to the given examples above with the exact puncturing patterns to be derived from {RD-1}.

The puncturing table for the termination part is given in Table A1-5. The last rows of the table are introduced in this document to obtain higher rates and are not part of {RD-1}.

TABLE A1-5 - Puncturing and repetition patterns for tail bit periods (last 6 clocks) Punct. Pattern ID; Code Rate; Punct. / Rep. Pattern (X; Y0; Y1; X’; Y’0; Y’1 ¦ X; Y0; Y1; X’; Y’0; Y’1 ¦ …). 0; 1/5; 3;1;1;0;0;0 ¦ 3;1;1;0;0;0 ¦ 3;1;1;0;0;0 ¦ 0;0;0;3;1;1 ¦ 0;0;0;3;1;1 ¦ 0;0;0;3;1;1. 1; 2/9; 3;1;1;0;0;0 ¦ 3;1;1;0;0;0 ¦ 2;1;1;0;0;0 ¦ 0;0;0;2;1;1 ¦ 0;0;0;2;1;1 ¦ 0;0;0;3;1;1. 2; 1/4; 2;1;1;0;0;0 ¦ 2;1;1;0;0;0 ¦ 2;1;1;0;0;0 ¦ 0;0;0;2;1;1 ¦ 0;0;0;2;1;1 ¦ 0;0;0;2;1;1. 2a; 1/4; 1;1;1;0;0;0 ¦ 1;1;1;0;0;0 ¦ 1;1;1;0;0;0 ¦ 0;0;0;1;1;1 ¦ 0;0;0;1;1;1 ¦ 0;0;0;1;1;1. 3; 2/7; 1;1;1;0;0;0 ¦ 2;1;1;0;0;0 ¦ 2;1;1;0;0;0 ¦ 0;0;0;2;1;1 ¦ 0;0;0;1;1;1 ¦ 0;0;0;1;1;1. 4; 1/3; 2;1;0;0;0;0 ¦ 2;1;0;0;0;0 ¦ 2;1;0;0;0;0 ¦ 0;0;0;2;1;0 ¦ 0;0;0;2;1;0 ¦ 0;0;0;2;1;0. 5; 2/5; 1;1;1;0;0;0 ¦ 1;1;1;0;0;0 ¦ 1;0;1;0;0;0 ¦ 0;0;0;1;1;1 ¦ 0;0;0;1;1;1 ¦ 0;0;0;1;0;1. 6; 1/2; 1;1;0;0;0;0 ¦ 1;1;0;0;0;0 ¦ 1;1;0;0;0;0 ¦ 0;0;0;1;1;0 ¦ 0;0;0;1;1;0 ¦ 0;0;0;1;1;0. 7; 2/3; 1;0;0;0;0;0 ¦ 1;0;1;0;0;0 ¦ 1;0;1;0;0;0 ¦ 0;0;0;1;0;0 ¦ 0;0;0;1;0;1 ¦ 0;0;0;1;0;1. 7a; 2/3; 1;1;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 0;0;0;1;1;0 ¦ 0;0;0;1;0;0 ¦ 0;0;0;1;0;0. 7b; 2/3; 1;1;0;0;0;0 ¦ 1;1;0;0;0;0 ¦ 1;1;0;0;0;0 ¦ 0;0;0;1;1;0 ¦ 0;0;0;1;1;0 ¦ 0;0;0;1;1;0. 8; 3/4; 1;0;1;0;0;0 ¦ 1;0;1;0;0;0 ¦ 1;0;1;0;0;0 ¦ 0;0;0;1;0;1 ¦ 0;0;0;1;0;1 ¦ 0;0;0;1;0;1. 8a; 3/4; 1;0;1;0;0;0 ¦ 1;0;1;0;0;0 ¦ 1;0;1;0;0;0 ¦ 0;0;0;1;0;1 ¦ 0;0;0;1;0;1 ¦ 0;0;0;1;0;0. 9; 5/6; 1;1;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 1;0;0;0;0;0 ¦ 0;0;0;1;1;0 ¦ 0;0;0;1;0;0 ¦ 0;0;0;1;0;0. For each rate, the puncturing table shall be read first from left to right and then from top to bottom.

### 5.2.5	CRC

The 32 bit ITU-T V.42 {RD-5} polynomial 0x04C11DB7 CRC is appended to the last segment of the datagram. The CRC is calculated over all fragments of the datagram datagram (including any zero padding) with MSB of each byte processed first. The resulting 32 bit CRC is appended MSB first so the receiver can verify that the result is 0x00000000 when calculated over data+padding+CRC.

F(x) = x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1

Initial state: 0xFFFFFFFF

### 5.2.6	Bit scrambling

The bit scrambler shown Figure A1-9 uses the polynom:

F(x)=1 + x-14 + x-15

and the initialization sequence as indicated in the top of the figure. For each new encoding block, the bit scrambler is re-initialized.

FIGURE A1-9

Bit scrambling

### 5.2.7	Modulation Coding Schemes  [changed SP to heading 3]

All MCS formats are defined in the Link Configuration ID Tables A1-6 to A1-9 (refer to Figures A1-4A and A1-4B. The Channel Quality Indicator (CQI) value is used by the Adaptive Coding and Modulation (ACM) mechanism.

TABLE A1-6

ASM Link Configuration ID parameters

| PL format # | ASM-MCS-1.16-1 | ASM-MCS-1.16-2 | ASM-MCS-1.16-3 | ASM-MCS-1.16-4 |  | ASM-MCS-1.16-5 |  | ASM-MCS-1.16-6 | ASM-MCS-1.16-7 | ASM-MCS-1.16-8 |  | ASM-MCS-1.16-9 | ASM-MCS-1.16-10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Li nk Config ID | 1 | 2 | 3 | 4 (SAT) |  | 5 |  | 6 | 7 | 8 |  | 9 | 10 |  |
| Channel BW | 16 |  |  |  |  |  |  |  |  |  |  |  |  | kHz |
| Roll off filtering | 0,35 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Signal BW | 13,0 |  |  |  |  |  |  |  |  |  |  |  |  | kHz |
| Symbol rate | 9,6 |  |  |  |  |  |  |  |  |  |  |  |  | ksps |
| Burst size | 1 | 2 | 3 | 3 |  | 1 |  | 2 | 3 | 1 |  | 2 | 3 | slots |
| Guard time | 0,83 |  |  |  | 8 |  | 0,83 |  |  |  |  |  |  | ms |
| Burst duration | 25,8 | 52,5 | 79,2 | 72,0 |  | 25,8 |  | 52,5 | 79,2 | 25,8 |  | 52,5 | 79,2 | ms |
| Symbols/ burst | 248 | 504 | 760 | 691 |  | 248 |  | 504 | 760 | 248 |  | 504 | 760 | symbols |
| Ramp-up/down | 4/4 |  |  |  |  |  |  |  |  |  |  |  |  | symbols |
| Ramp-up/down | 0,41/0,41 |  |  |  |  |  |  |  |  |  |  |  |  | ms |
| Syncword size | 27 |  |  |  |  |  |  |  |  |  |  |  |  | symbols |
| Syncword modul. | PI/4 QPSK ( 00/11 only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Link Config ID symbols | 16 |  |  |  |  |  |  |  |  |  |  |  |  | symbols |
| Link Config ID modul. | Pi/4 QPSK |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Net symbols/ burst | 197 | 453 | 709 | 640 |  | 197 |  | 453 | 709 | 197 |  | 453 | 709 | symbols |
| Channel bits | 394 | 906 | 1418 | 1280 |  | 394 |  | 906 | 1418 | 394 |  | 906 | 1418 | bits |
| Padding/ flush bits | 10 |  |  |  | 11 |  | 10 |  |  |  |  |  |  | bits |
| FEC decoder input symbols | 192 | 448 | 704 | 634.5 |  | 192 |  | 448 | 704 | 192 |  | 448 | 704 | symbols |
| FEC decoder input bits | 384 | 896 | 1408 | 126 |  | 384 |  | 896 | 1408 | 384 |  | 896 | 1408 | bits |
| FEC output bits | 384 | 896 | 1408 | 952 |  | 288 |  | 672 | 1056 | 192 |  | 448 | 704 | bits |
| FEC output bytes | 48 | 112 | 176 | 119 |  | 36 |  | 84 | 132 | 24 |  | 56 | 88 | bytes |
| Modul. | PI/4 QPSK |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bits / symbol | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FEC rate | 1 |  |  |  | 3/4 |  |  |  |  |  | 1/2 |  |  |  |
| Es/N0 on AWGN | 11,0 | 11,0 | 11,0 | 4,5 |  | 5,3 |  | 5 | 4,8 | 3,6 |  | 3 | 2,8 | dB |
| C/(N0+I0) threshold | 50,8 | 50,8 | 50,8 | 44,3 |  | 45,1 |  | 44,8 | 44,6 | 43,4 |  | 42,8 | 42,6 | dBHz |
| Minimum CQI value | 50 | 51 | 53 | 43 |  | 45 |  | 45 | 45 | 43 |  | 43 | 43 |  |

TABLE A1-7

VDE-TER Link Configuration ID parameters

| PL format # | TER-MCS-1.25 | TER-MCS-3.25 | TER-MCS-5.25 | TER-MCS-1.50 | TER-MCS-3.50 | TER-MCS-5.50 | TER-MCS-1.100 | TER-MCS-3.100 | TER-MCS-5.100 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Link Config ID | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |  |
| Channel BW | 25 |  |  | 50 |  |  | 100 |  |  | kHz |
| Roll off filtering | 0,3 |  |  |  |  |  |  |  |  |  |
| Signal BW | 25,0 |  |  | 49,9 |  |  | 99,8 |  |  | kHz |
| Symbol rate | 19,2 |  |  | 38,4 |  |  | 76,8 |  |  | ksps |
| Burst size | 1 |  |  |  |  |  |  |  |  | slot |
| Guard time | 0,83 |  |  |  |  |  |  |  |  | ms |
| Burst duration | 25,8 |  |  |  |  |  |  |  |  | ms |
| Symbols/burst | 496 |  |  | 992 |  |  | 1984 |  |  | symbols |
| Ramp-up/down | 8/8 |  |  | 16/16 |  |  | 32/32 |  |  | symbols |
| Ramp-up/down | 0.41/0.41 |  |  |  |  |  |  |  |  | ms |
| Syncword size | 27 |  |  |  |  |  |  |  |  | symbols |
| Syncword modulation | PI/4 QPSK ( 00/11 only) |  |  |  |  |  |  |  |  |  |
| Link Config ID size | 16 (32,6 block code) |  |  |  |  |  |  |  |  | symbols |
| Link Config ID modulation | PI/4 QPSK |  |  |  |  |  |  |  |  |  |
| Net symbols/burst | 437 |  |  | 917 |  |  | 1877 |  |  | symbols |
| Channel bits | 874 | 1311 | 1748 | 1834 | 2751 | 3668 | 3754 | 5631 | 7508 | bits |
| Padding/flush | 10 | 15 | 20 | 42 | 63 | 84 | 170 | 255 | 340 | bits |
| FEC decoder input symbols | 432 |  |  | 896 |  |  | 1792 |  |  | symbols |
| FEC decoder input bits | 864 | 1296 | 1728 | 1792 | 2688 | 3584 | 3584 | 5376 | 7168 | bits |
| FEC output bits | 432 | 972 | 1296 | 896 | 2016 | 2688 | 1792 | 4032 | 5376 | bits |
| FEC output bytes | 54 | 122 | 162 | 112 | 252 | 336 | 224 | 504 | 672 | bytes |
| Modulation | PI/4 QPSK | 8PSK | 16 QAM | PI/4 QPSK | 8PSK | 16 QAM | PI/4 QPSK | 8PSK | 16 QAM |  |
| FEC rate | 1/2 | 3/4 | 3/4 | 1/2 | 3/4 | 3/4 | 1/2 | 3/4 | 3/4 |  |
| Es/N0 on AWGN | 1,0 | 7,9 | 10,2 | 1,0 | 7,9 | 10,2 | 1,0 | 7,9 | 10,2 | dB |
| C/(N0+I0) threshold | 43,8 | 50,7 | 53,0 | 46,8 | 53,7 | 56,0 | 49,9 | 56,8 | 59,1 | dBHz |
| Minimum CQI value | 42 | 53 | 56 | 46 | 57 | 59 | 49 | 60 | 62 |  |

TABLE A1-8

VDE-SAT Uplink Configuration ID parameters

| PL format | SAT-MCS-1.50-2 | SAT-MCS-1.50-3 | SAT-MCS-1.50-4 | SAT-MCS-3.50-2 | SAT-MCS-5.50 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Link Config ID | 20 | 21 | 22 | 23 | 24** |  |
| Channel bandwidth | 50 |  |  |  |  | kHz |
| Roll off filtering | 0,25 |  |  |  |  |  |
| Signal bandwidth | 42,0 |  |  |  |  | kHz |
| CDMA chiprate | 33,6 |  |  |  |  | kcps |
| Codelength | 16 |  |  |  |  | chips |
| Symbol rate | 2,1 | 33,6 |  |  |  | ksps |
| Burst size | 5 | 1 | 3 |  |  | slots |
| Guard time | 8 |  |  |  |  | ms |
| Burst duration | 125,3 | 18,7 | 72,0 |  |  | ms |
| Symbols/burst | 263 | 627 | 2419 |  |  | symbols |
| Ramp-up/down | 14/14* |  |  |  |  | symbols |
| Ramp-up/down | 0.41/0.41 |  |  |  |  | ms |
| Syncword size | 48 | 27 |  |  |  | symbols |
| Syncword modulation | QPSK/CDMA (00/11) | PI/4 QPSK (00/11) |  |  |  |  |
| Link Configuration ID size | 0 | 16 (32,6 block code) |  |  |  | symbols |
| Link Configuration ID modulation | NA | PI/4 QPSK |  |  |  |  |
| Pilot symbol distance | 17 | NA |  | 20 |  | symbols |
| Total pilot symbols | 12 | 0 |  | 71 |  | symbols |
| Net symbols/burst | 201 | 556 | 2348 | 2277 | 2277 | symbols |
| Channel bits | 352 | 1112 | 4696 | 6831 | 9108 | bits |
| Padding/flush bits | 18 | 8 | 16 | 15 | 8*2 | bits |
| FEC decoder input symbols | 192 | 552 | 2340 | 2272 | 2277 | symbols |
| FEC decoder input bits | 384 | 1104 | 4680 | 6816 | 4546*2 | bits |
| FEC output bits | 96 | 736 | 3120 | 4544 | 3788*2 | bits |
| FEC output bytes | 11 | 92 | 390 | 568 | 947 | bytes |
| Modulation | QPSK/ CDMA | PI/4 QPSK |  | 8PSK | 16QAM |  |
| FEC rate | 1/4 | 2/3 |  |  | 5/6 |  |
| Es/N0 on AWGN | -0,9 | 3,9 | 3,9 | 8,0 | 12,2 | dB |
| C/(N0+I0) threshold | 32,3 | 49,2 | 49,2 | 53,3 | 57,5 | dBHz |
| Minimum CQI value | 32 | 49 | 51 | 54 | 58 |  |

*) For spread sequence it is 14/14 chips.

**) FEC block is split into two sub-blocks in order to avoid very long FEC block.

TABLE A1-9

VDE-SAT Downlink Configuration ID parameters

| PL format | SAT-MCS-0.50-1 |  | SAT-MCS-1.50-1 | SAT-MCS-3.50-1 | SAT-MCS-0.100 | SAT-MCS-0.150 | SAT-MCS-0.300 | SAT-MCS-0.500 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Link Config ID | 25 |  | 26 | 27 | 28 | 29 | 30 | 31 |  |
| Channel BW | 50 |  |  |  | 100 | 150 | 300 | 500 | kHz |
| Roll off filtering | 0,25 |  |  |  |  |  |  |  |  |
| Signal BW | 42,0 |  |  |  | 90,0 | 141,0 | 291,0 | 492 | kHz |
| CDMA chiprate | 33,6 |  |  |  | 72,0 | 112,8 | 232,8 | 393,6 | kcps |
| Codelength | 8 |  |  |  | 4 |  |  |  | chips |
| Symbol rate | 4,2 |  | 33,6 |  | 18,0 | 28,2 | 58,2 | 98,4 | ksps |
| Burst size | 90 |  |  |  |  |  |  |  | slots |
| Guard time | 8 |  |  |  |  |  |  |  | ms |
| Burst duration | 2392,0 |  |  |  |  |  |  |  | ms |
| Symbols/burst | 10046 |  | 80371 |  | 43056 | 67454 | 139214 | 235372 | symbols |
| Ramp-up/down | 14/14 |  |  |  | 30/30 | 47/47 | 97/97 | 163/163 | symbols / chips |
| Ramp-up/down | 0.41/0.41 |  |  |  |  |  |  |  | ms |
| Syncword size | 48 |  | 27 |  | 48 |  |  |  | symbols |
| Syncword modulation | BPSK/ CDMA |  | PI/4 QPSK (00 /11) |  | BPSK/CDMA |  |  |  |  |
| Link Config ID | 16 (32,6 block code) |  |  |  |  |  |  |  | symbols |
| Link Config ID modulation | BPSK/ CDMA |  | PI/4 QPSK |  | BPSK/CDMA |  |  |  |  |
| Pilot distance | 20 |  |  |  |  |  |  |  | symbols |
| Total pilots symbols | 497 | 4015 |  |  | 2146 | 3364 | 6947 | 11749 | symbols |
| Net symbols/burst | 9457 | 80300 |  |  | 40786 | 67296 | 138956 | 234982 | symbols |
| Channel bits | 9457 | 160600 |  | 240900 | 40786 | 67296 | 138956 | 234982 | bits |
| Padding/flush | 1 | 24 |  | 4 | 2 | 0 | 12 | 22 | bits |
| FEC decoder input symbols | 9457 | 80300 |  | 80300 | 40786 | 67296 | 138956 | 234982 | symbols |
| FEC decoder input bits | 9456 | 160576 |  | 240896 | 40784 | 67296 | 138944 | 234960 | bits |
| FEC output bits | 4728 | 40144 |  | 120448 | 20392 | 33648 | 69472 | 117480 | bits |
| FEC output | 591 | 5018 |  | 15056 | 2549 | 4206 | 8684 | 14685 | bytes |
| Modulation | BPSK/ CDMA | PI/4 QPSK |  | 8PSK | BPSK/CDMA |  |  |  |  |
| FEC rate | 1/2 | 1/4 |  | 1/2 | 1/2 |  |  |  |  |
| Es/N0 on AWGN | -2,0 | -2,4 |  | 5,0 | -2,0 |  |  |  | dB |
| C/(N0+I0) thres | 34,2 | 42,9 |  | 50,3 | 40,6 | 42,5 | 45,6 | 47,9 | dBHz |
| Minimum CQI value | 33 | 44 |  | 52 | 40 | 44 | 47 | 49 |  |

### 5.2.8 	Bit mapping

The bit mappings used throughout the Annexes are shown in figures Figs. A1-10 to A1-13.

The first output from the bit scrambler is mapped to the MSB of the first symbol, the second bit to the next bit in the symbol, and so on until the LSB of the symbol has been filled, then mapping continues in the next symbol. If more bits are needed to complete the last symbol, 0 shall be used.

The initial state of the alternating π/4 QPSK bit mapping is defined such that the first symbol of the training sequence is mapped to the constellation defined by points {1+j/2, -1+j/2, -1-j/2, 1-j/2} (shown in green in Figs. A1-7); the next symbol is mapped to the constellation defined by points {1+0j, 0+j, -1+0j, 0-j} (shown in purple in Figs. A1-7); and so on.

Figure A1-10

bit mapping for π/4 qpsk

±1±j/2±1, ±j

Figure A1-11

8PSK symbol to bit mapping

Figure A1-12

Bit Mapping for 16QAM

Bit Mapping for 16QAM

Figure A1-13

16APSK bit to symbol mapping

The 16 APSK modulation constellation is composed of two concentric rings of uniformly spaced 4 and 12 PSK points, respectively in the inner ring of radius R1 and outer ring of radius R2.

The ratio of the outer circle radius to the inner circle radius (γ =R2/R1) shall be equal to 3. R1 shall be set to 1/√7, R2 shall be set to 3/√7 in order to have the average signal energy equal to 1.

Similar to AIS, when data is output on the VHF data link it should be grouped in bytes of 8 bits from top to bottom of the table associated with each message in accordance with ISO/IEC 13239:2002. Each byte should be output with least significant bit first.

### 5.2.9	Antenna gain for VDES ship stations  [changed SP to heading 3]

Existing ship antennas may be used for VDES. The maximum antenna gain for these antennas ranges from 2 dBi to 10 dBi. Representative antenna patterns are shown in Figure A1-14.

A ship antenna with a minimum gain at 0 degrees elevation of 2 dBi at the receiver input is required.

### 5.2.10	Noise and interference level

The noise floor is a function of many sources such as vessel electronics, other radio equipment, power supplies, etc., and sensitivity is also reduced by RF cabling losses and the LNA noise figure. Table A1-10 presents representative values for the receiver noise figure.

TABLE A1-10

Ship receiver noise figure calculations

| Antenna noise temperature* | 245.0 | K |
| --- | --- | --- |
| LNA noise figure | 6.0 | dB |
| LNA noise temperature | 813.8 | K |
| Feed loss noise temp at LNA | 0.0 | K |
| Antenna noise temp at LNA | 245.0 | K |
| System noise temp at LNA | 1058.8 | K |
| System noise temp at LNA | 30.2 | dBK |
| * The galactic background antenna noise temperature is 245 K at 160 MHz {RD-4}. |  |  |

### 5.2.11	Transmitter requirements for VDES  [changed SP to heading 3]

#### 5.2.11.1	Transmitter power

Except for Annex 2, Table A1-11 defines the requirements for VDE station transmitters.

TABLE A1-11

Transmitter parameters

| Transmitter parameters | Requirements | Condition |
| --- | --- | --- |
| Frequency error | 1.5 ppm | normal |
| Maximum transmit power capability | For ship stations: Transmit average power should be at least 1 watt and not exceed 25 watts at the transmitter output. For shore stations: Transmit average power should be at least 12.5 watt and not exceed 50 watts at the base of the antenna. ±1.5 dB normal, +2/−6 dB extreme | Conducted |
| Maximum adjacent power levels for 25 kHz channel | ∆fc < ±12.5 kHz: 0 dBc ±12.5 kHz < ∆fc < ±25 kHz: below the straight line between −25 dBc at ±12.5 kHz and –70 dBc at ±25 kHz ±25 kHz < ∆fc < ±62.5 kHz: −70 dBc |  |
| Maximum adjacent power levels for 50 kHz channel | ∆fc < ±25 kHz: 0 dBc ±25 kHz < ∆fc < ±37.5 kHz: below the straight line between −25 dBc at ±25 kHz and –70 dBc at ±37.5 kHz ±37.5 kHz < ∆fc < ±125 kHz: −70 dBc |  |
| Maximum adjacent power levels for 100 kHz channel | ∆fc < ±50 kHz: 0 dBc ±50 kHz < ∆fc < ±62.5 kHz: below the straight line between −25 dBc at ±50 kHz and –70 dBc at ±62.5 kHz ±62.5 kHz < ∆fc < ±250 kHz: −70 dBc |  |
| Spurious emissions | −36 dBm −30 dBm | 9 kHz to 1 GHz 1 GHz to 4 GHz |

FIGURE A1-13b Slotted transmission mask for VDE stations

#### 5.2.11.2	Ship e.i.r.p. vs. elevation angle  [changed SP to heading 4]

The minimum ship e.i.r.p. vs elevation angle is shown in Table A1-12. There are no minimum e.i.r.p. requirements above 80 degrees elevation. Table A1-12 is based on a linear transmitter meeting the maximum Adjacent Channel Interference levels defined in Table A1-11. For saturated operation the e.i.r.p. shall be 3 dB higher.

TABLE A1-12

Minimum ship e.i.r.p. vs. elevation angle

| Ship elevation angle | Ship antenna gain | Minimum ship e.i.r.p. with 6 W transmitter |
| --- | --- | --- |
| degrees | dBi | dBW |
| 0 | 3 | 10.8 |
| 10 | 3 | 10.8 |
| 20 | 2.5 | 10.3 |
| 30 | 1 | 8.8 |
| 40 | 0 | 7.8 |
| 50 | −1.5 | 6.3 |
| 60 | −3 | 4.8 |
| 70 | −4 | 3.8 |
| 80 | −10 | −2.2 |
| 90 | −20 | −12.2 |

#### 5.2.11.3	Shutdown procedure

An automatic transmitter hardware shutdown procedure and indication should be provided in case a transmitter continues to transmit for more than 2 s. This shutdown procedure should be independent of software control.

#### 5.2.11.4	Safety precautions

The VDES installation, when operating, should not be damaged by the effects of open circuited or short circuited antenna terminals.

## 5.3	Link layer

This layer ensures reliable transmission of data frames between ships, ship and shore, and ship and satellite. Both connection oriented and store and forward (e.g. satellite) transfer are supported. In the case of store and forward transfer, the packet protocols are terminated at both the ship and Control Station ends.

The link layer is divided into three sub-layers with the following tasks:

### 5.3.1	Link management entity

Assemble Unique Word, format header, Physical Layer Frame (PL-Frame) headers, pilot tones (satellite) and VDES message bits into packets.

### 5.3.2	Data link services

Calculates and adds CRC check sum and completes the PL-Frame/packet.

### 5.3.3 Resource Management

The connection between ship and shore is normally session oriented where a Logical Channel is reserved for a particular ship either for a certain amount of data transfer or time duration.

Ship originated short messages can be sent on the Random Access Channels without resource allocation.

During heavy network loading, the network control may introduce time dispersion for resource requests or only allow traffic with high priority levels.

### 5.3.4 Data Structures

Data is generally transferred as packets using fixed duration Logical Channel Frames, and consist of one or multiple data packets, zero padding and a 4 byte CRC.

Long data packets are fragmented and sent over multiple Logical Channel frames.

The two cases are shown in figure A1-14.

figure a1-14

single/multiple datagram, zero padding and crc structure

Note that the zero padding is defined as a separate sub-packet. The 4 byte CRC is always at the end of the frame. Preamble and FEC flushing bits are not shown.

### 5.3.5 Adaptive Coding and Modulation/Rate Adaption  [changed SP to heading 3]

The signal and interference environment is expected to change with time and location.  The Control Station may measure channel quality of the received ship signal and request the ship to adjust the Modulation and Coding Scheme to maximize throughput.

Similarly, a ship will report a Channel Quality Indicator (CQI) equivalent to received C/N0 (based on a Link Adaption Model) to the Control Station which then will adjust the Modulation and Coding Scheme to maximize throughput.

### 5.3.6 Signalling packets  [changed SP to heading 3]

Signalling packets are used for data management and control whilst data packets are used for information transfer.

### 5.3.6.1 To ship signalling packets

These are used for ship paging, resource allocation and data transfer handling.

All packets contain a 1 byte packet type and a variable length header defining the length, ship ID and various other parameters.

Table A1-13 defines the to ship signalling packets.

Table A1-13

To ship signalling packets

| Media Access Control |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 000 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length | 6: Total packet size in bytes, fixed at 6 bytes |
| 3 | 0-255 | 1 | Media Access priority level | 0: All accesses allowed 1: High priority only 2: Medium priority 3: Low priority 255: No accesses allowed |
| 4 | 0-255 | 1 | Random Access spreading interval | Randomly selected from RA Logical Channel group, randomly selected within LC group |
| 5 | 0-255 | 1 | System status | 0: Normal 10: Busy 20: Temporally out of service 30: Scheduled out of service |
| To ship Announcement/Paging |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 001 or 002 | 1 | Type | 001: Paging a specific ship for in-coming datagram 002: Paging a specific ship for mobility management update |
| 2 | 0 to 216-1 | 2 | Length | Total packet size in bytes, variable |
| 3 |  | 4 | Source ID | Source of the request |
| 4 | 0 to 232-1 | 4 | Ship ID #1 |  |
|  | ……… |  |  |  |
| Last | 0 to 232-1 | 4 | Ship ID #N |  |
| Multicast resource allocation announcement |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 003 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length | Total size in bytes, variable |
| 3 | 0-255 | 1 | Type of multicast | 1: Group multicast 2: Area multicast |
| 4 | 0-255 | 1 | Logical Channel no |  |
| 5 | 0-255 | 1 | Session ID | To support multiple sessions within a logical channel |
| 6 | 0 to 232-1 | 4 | Datagram size in bytes |  |
| Unicast to ship resource allocation announcement |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 004 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length | Total size in bytes, variable |
| 3 | 0 to 232-1 | 4 | Ship ID #1 |  |
| 4 | 0-255 | 1 | Logical Channel no |  |
| 5 | 0-255 | 1 | Session ID | Multi session support |
| 6 | 0-255 | 1 | ACK Logical Channel |  |
| 7 | 0-255 | 1 | ACK LC Subchannel | Set to 0 if not used |
| 8 | 0-255 | 1 | CQI |  |
|  | ……… |  |  |  |
|  | 0 to 232-1 | 4 | Ship ID #N |  |
|  | 0-255 | 1 | Logical Channel |  |
|  | 0-255 | 1 | Session ID |  |
|  | 0 to 232-1 | 4 | Datagram size in bytes |  |
|  | 0-255 | 1 | ACK Logical Channel |  |
|  | 0-255 | 1 | CQI |  |
| Last | 0-255 | 1 | ACK LC Subchannel |  |
| Unicast resource allocation announcement to ship originated requests |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 005 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length | Total size in bytes, variable |
| 3 | 0 to 232-1 | 4 | Ship ID #1 |  |
| 4 | 0-255 | 1 | Logical Channel no |  |
| 5 | 0-255 | 1 | Session ID | Multi stream support |
| 6 | 0-255 | 1 | Logical Channel Subchannel | 0: When full LC frame is allocated, 1-255 indicates the logical subframe number that will be used |
| 7 | 0-255 | 1 | CQI |  |
|  | ……… |  |  |  |
|  | 0 to 232-1 | 4 | Ship ID #N |  |
|  | 0-255 | 1 | Logical Channel |  |
|  | 0-255 | 1 | Session ID | Multi stream support |
|  | 0-255 | 1 | CQI |  |
| Last | 0-255 | 1 | Logical Channel Subchannel |  |
| To ship Access Denied/Resource de-allocation |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 006 | 1 | Type | This can be used both at system access and during a given session |
| 2 | 0 to 216-1 | 2 | Length | Total size in bytes, variable |
| 3 | 0 to 232-1 | 4 | Ship ID #1 |  |
| 4 | 0-255 | 1 | Reason for Reject/Clear | 0: Unrecognised ship ID 1: Unrecognised packet 2: Access denied by operator 3: Protocol error 4: System reset 5: All sessions cleared 6: Cleared for high priority traffic 7: Timeout 8: Shore address not recognised 9: Requested service not supported 10: Ship requested de-allocation 11: Called ship not reachable |
|  | ……… |  |  |  |
|  | 0 to 232-1 | 4 | Ship ID #N |  |
|  | 0-255 | 1 | Session ID |  |
| Last | 0-255 | 1 | Reason for Reject/Clear |  |
| Control station ACK/ACM |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 007 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length | Total size in bytes, variable |
| 3 | 0 to 232-1 | 4 | Ship ID #1 |  |
| 4 | 0-255 | 1 | Session ID |  |
| 5 | 0-255 | 1 | ACM or EDN | 0: Maintain MCS 1: Increment MCS (higher rate) 2: Decrease MCS 3: End Delivery Notification |
| 6 | 0 to 216-1 | 2 | ACK/NACK mask | Relevant bit set for failed fragments 1 to 16 |
| 7 | 0-255 | 1 | CQI |  |
| 8 | 0-255 | 1 | Power setting | 0: Maintain Power Level 1: Increase Power Level 2: Decrease Power Level |
|  | ……… |  |  |  |
|  | 0 to 232-1 | 4 | Ship ID #N |  |
|  | 0-255 | 1 | Session ID |  |
|  | 0-255 | 1 | ACM or EDN | 0: Maintain MCS 1: Increment MCS (higher rate) 2: Decrease MCS 3: End Delivery Notification |
|  | 0-255 | 1 | CQI |  |
|  | 0-255 | 1 | Power setting | 0: Maintain Power Level 1: Increase Power Level 2: Decrease Power Level |
| Last | 0 to 216-1 | 2 | NACK mask | Relevant bit set for failed fragments 1 to 16 |

### 5.3.6.2 From ship signalling packets

These are used for ship resource request and data transfer handling.  A special very short (11 byte) packet is used for unusual interference conditions and uses a 16 bit CRC.

Table A1-14 defines the from ship signalling packets.

Table A1-14.

From ship signalling packets

| Ship response to Announcement/Paging |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 010 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length | Fixed, 12 bytes |
| 3 | 0 to 232-1 | 4 | Ship ID |  |
| 4 | 0-255 | 1 | Session ID |  |
| 5 | 0 to 232-1 | 4 | Terminal capabilities | Update to reflect mask values |
| 6 | 0 to 224-1 | 3 | Coarse latitude, +/- 90 degrees N, 2s complement 1/10th minute resolution, relative the service area NW corner |  |
| 7 | 0 to 224-1 | 3 | Coarse longitude, +/- 180 degrees E, 2s complement, 1/10th minute resolution relative to the service area SE corner |  |
| 8 | 0-255 | 1 | CQI | Received C/N0 in dBHz |
| Very short response to Announcement (11 bytes) – satellite only |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 011 | 1 | Type |  |
| 2 | 0 to 232-1 | 4 | Ship ID |  |
| 3 | 0-255 | 1 | Session ID |  |
| 4 | 0-255 | 1 | Terminal capabilities | 1: Compliant 2092-1 2: Compliant 2092-2 |
| 5 | 0 to 216-1 | 2 | CRC-2 | 16 bit CRC used |
| Ship resource request |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function |  |
| 1 | 012 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length |  |
| 3 | 0 to 232-1 | 4 | Source ID |  |
| 4 | 0-255 | 1 | Session ID |  |
| 5 | 0 to 232-1 | 4 | Destination ID |  |
| 6 | 0 to 232-1 | 4 | Terminal capabilities | mask |
| 7 | 0 to 232-1 | 4 | Datagram size in bytes |  |
| 8 | 0-255 | 1 | CQI | Received C/N0 in dBHz |
| 9 | 0-255 | 1 | Priority | 0 Default |
| 10 | 0-255 | 1 | Communications mode | 0: Default 1: Direct ship to ship mode |
| Ship ACK/ACM |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 013 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length |  |
| 3 | 0 to 232-1 | 4 | Ship ID |  |
| 4 | 0-255 | 1 | Session ID |  |
| 5 | 0 to 216-1 | 2 | ACK/NACK mask | Relevant bit set for failed fragments 1 to 16 |
| 6 | 0-255 | 1 | CQI | Received C/N0 in dBHz |
| 7 | 0-255 | 1 | ACM or EDN | 0: Maintain MCS 1: Increment MCS (higher rate) 2: Decrease MCS 3: End Delivery Notification |
| 8 | 0-255 | 1 | Power setting | 0: Maintain Power Level 1: Increase Power Level 2: Decrease Power Level |
| Very short ACK/ACM (11 bytes) – satellite only |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 014 | 1 | Type |  |
| 2 | 0 to 232-1 | 4 | Ship ID |  |
| 3 | 0-255 | 1 | Session ID |  |
| 4 | 0 to 216-1 | 2 | ACK/NACK mask | Relevant bit set for failed fragments 1 to 16 |
| 5 | 0-255 | 1 | CQI | Received C/N0 in dBHz |
| 6 | 0 to 216-1 | 2 | CRC-2 | 16 bit CRC used |
| Ship de-allocation request |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 015 | 1 | Type | Release the allocated resources |
| 2 | 0 to 216-1 | 2 | Length |  |
| 3 | 0 to 232-1 | 4 | Ship ID |  |
| 4 | 0-255 | 1 | Session ID | Session ID 0 clear all ship connections |
| 5 | 0-255 | 1 | Reason for de-allocation | 1: Time-out, 2: Retry limit exceeded 3. End of session |
| Very short ship clear session (11 bytes) – satellite only |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 016 | 1 | Type |  |
| 2 | 0 to 232-1 | 4 | Ship ID |  |
| 3 | 0-255 | 1 | Session ID |  |
| 5 | 0-255 | 1 | Reason for disconnection | 1: Time-out, 2: Retry limit exceeded |
| 6 | 0 to 216-1 | 2 | CRC-2 | 16 bit CRC used |

### 5.3.6.3 Ship to ship signalling

These are used to set up connections with other ships in the area.  RATDMA is used to select a Logical Channel for the data transfer. All signalling takes place on Logical Channel 0 defined on the Bulletin Board.

Table A1-15 shows the signalling packet content.

Table A1-15

Ship to ship signalling outside Control Station service area

| Ship paging ship |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 90 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length | Total packet size in bytes, variable |
| 3 | 0 to 232-1 | 4 | Source Ship ID # |  |
| 4 | 0-255 | 1 | Priority | Default 0 |
| 5 | 0 to 232-1 | 4 | Destination Ship ID # |  |
| 6 | 0 to 232-1 | 4 | Terminal capabilities |  |
| Ship response to Paging |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function |  |
| 1 | 091 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length |  |
| 3 | 0 to 232-1 | 4 | Ship ID |  |
| 4 | 0-255 | 1 | Session ID |  |
| 5 | 0 to 232-1 | 4 | Terminal capabilities |  |
| 6 | 0 to 224-1 | 3 | Coarse latitude, +/- 90 degrees N, 2s complement 1/10th minute resolution, relative the service area NW corner |  |
| 7 | 0 to 224-1 | 3 | Coarse longitude, +/- 180 degrees E, 2s complement, 1/10th minute resolution relative to the service area SE corner |  |
| 8 | 0-255 | 1 | CQI | Received C/N0 in dBHz |
| 9 | 0-255 | 1 | Logical Channel no |  |
| 10 | 0-255 | 1 | Session ID | Multi stream support |
| 11 | 0-255 | 1 | Logical Channel Subchannel | 0: When full LC frame is allocated |

| Ship to ship short message |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field no | Value (DEC) | Size (bytes) | Function | Comment |
| 1 | 92 | 1 | Type |  |
| 2 | 216-1 | 2 | Length |  |
| 3 | 232-1 | 4 | Originating ship ID |  |
| 4 | 28-1 | 1 | Session ID |  |
| 5 | 232-1 | 4 | Destination ship ID |  |
| 6 | 28-1 | 1 | Retransmission no | Handles lost ACKs |
| 7 |  | Variable | Payload |  |
| Ship to ship short message ACK |  |  |  |  |
| Field no | Value (DEC) | Size (bytes) | Function | Comment |
| 1 | 92 | 1 | Type |  |
| 2 | 216-1 | 2 | Length |  |
| 3 | 232-1 | 4 | Originating ship ID |  |
| 4 | 28-1 | 1 | Session ID |  |
| 5 | 232-1 | 4 | Destination ship ID |  |
| 6 | 28-1 | 1 | ACK/NACK/Reject | 0: ACK 1: NACK 3: Communications refused |

### 5.3.7 Data transfer packets

[text to introduce section]

### 5.3.7.1 Shore originated data

Multicast data is transmitted on a shared Logical Channel, whilst Unicast date can be either time multiplexed or carried on an allocated LC.

Several short datagrams that fit within a LCF Frame are sent sequentially.  Zero padding packets are added at the end to fill the LCF.

Large datagrams that exceeds the capacity of a Logical Channel Frame is transferred as multiple fragments.  Each fragment is sequencely numbered 0 to 15, and a 2 byte bit map is used to request retransmission of one or multiple fragments.  Datagrams with more than 15 fragments use modulo 15 numbering.

The packet types are defined in Table A1-16.

Table A1-16

To ship data packet types

| Bulletin Board datagram |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Field no |  | Value (Dec) | Size (Bytes) |  | Function |  | Content |
| 1 |  | 020 | 1 |  | Type |  |  |
| 2 |  | 0 to 216-1 | 2 |  | Length |  | Total size in bytes, variable |
| 3 |  |  | Variable |  | Payload |  | Bulletin Board fixed fields |
| Zero padding variable length |  |  |  |  |  |  |  |
| Field no |  | Value (Dec) | Size (Bytes) |  | Function |  | Content |
| 1 |  | 021 | 1 |  | Type |  |  |
| 2 |  | 0-255 | 1 |  | Length |  | Total number of zero bytes |
| Zero padding single byte |  |  |  |  |  |  |  |
| Field no |  | Value (Dec) | Size (Bytes) |  | Function |  | Content |
| 1 |  | 022 | 1 |  | Type |  | 1 byte zero padding |
| Group Multicast start datagram |  |  |  |  |  |  |  |
| Field no |  | Value (Dec) | Size (Bytes) |  | Function |  | Content |
| 1 |  | 023 | 1 |  | Type |  |  |
| 2 |  | 0 to 216-1 | 2 |  | Length |  | Total size in bytes, variable |
| 3 |  | 0 to 232-1 | 4 |  | Destination |  | Group ID |
| 4 |  | 0 to 216-1 | 2 |  | Number of fragments |  |  |
| 5 |  |  | Variable |  | Payload |  | Multicast content |
| Area Multicast start datagram |  |  |  |  |  |  |  |
| Field no |  | Value (Dec) | Size (Bytes) |  | Function |  | Content |
| 1 |  | 024 | 1 |  | Type |  |  |
| 2 |  | 0 to 216-1 | 2 |  | Length |  | Total size in bytes, variable |
| 3 |  |  | 3 |  | Coarse latitude, +/- 90 degrees N, 2s complement 1/10th minute resolution, relative the service area NW corner |  | AIS format |
| 4 |  |  | 3 |  | Coarse longitude, +/- 180 degrees E, 2s complement, 1/10th minute resolution relative to the service area SE corne |  | AIS format |
| 5 |  | 0 to 216-1 | 2 |  | Number of fragments |  |  |
| 6 |  |  | Variable |  | Payload |  | Multicast content |
| Network operator text message to all ships |  |  |  |  |  |  |  |
| Field no |  | Value (Dec) | Size (Bytes) |  | Function |  | Content |
| 1 |  | 025 | 1 |  | Type |  |  |
| 2 |  | 0 to 216-1 | 2 |  | Length |  | Total size in bytes, variable |
| 3 |  |  | Variable |  | Payload |  | ASCII text message |
| Network operator binary message to all ships |  |  |  |  |  |  |  |
| Field no |  | Value (Dec) | Size (Bytes) |  | Function |  | Content |
| 1 |  | 029 | 1 |  | Type |  |  |
| 2 |  | 0 to 216-1 | 2 |  | Length |  | Total size in bytes, variable |
| 3 |  |  | Variable |  | Payload: Data format to be defined in the payload |  | Binary |
| To ship Start Fragment/Single Fragment |  |  |  |  |  |  |  |
| Field no |  | Value (Dec) | Size (Bytes) |  | Function |  | Content |
| 1 |  | 026 | 1 |  | Type |  |  |
| 2 |  | 0 to 216-1 | 2 |  | Length |  | Total size in bytes, variable |
| 3 |  | 0 to 216-1 | 2 |  | Number of fragments; for single fragment message / short message: number of fragments = 1 |  |  |
| 4 |  | 0 to 232-1 | 4 |  | Ship ID |  |  |
| 5 |  | 0-255 | 1 |  | Session ID |  |  |
| 6 |  |  | Variable |  | Payload |  |  |
| To ship Continuation Fragment |  |  |  |  |  |  |  |
| Field no |  | Value (Dec) | Size (Bytes) |  | Function |  | Content |
| 1 |  | 027 | 1 |  | Type |  |  |
| 2 |  | 0 to 216-1 | 2 |  | Length |  | Total size in bytes, variable |
| 3 |  | 0 to 232-1 | 4 |  | Ship ID |  | 0: For multicast |
| 4 |  | 0-255 | 1 |  | Session ID |  |  |
| 5 |  | 0-255 | 1 |  | Fragment no |  |  |
| 6 |  |  | Variable |  | Payload |  |  |
| To ship End Fragment |  |  |  |  |  |  |  |
| Field no |  | Value (Dec) | Size (Bytes) |  | Function |  | Content |
| 1 |  | 028 | 1 |  | Type |  |  |
| 2 |  | 0 to 216-1 | 2 |  | Length |  | Total size in bytes, variable |
| 3 |  | 0 to 232-1 | 4 |  | Ship ID |  |  |
| 4 |  | 0-255 | 1 |  | Session ID |  |  |
| 5 |  | 0-255 | 1 |  | Fragment no |  | Modulo 15, 1 to 15. |
| 6 |  |  | Variable |  | Payload |  |  |
| To ship short unicast message |  |  |  |  |  |  |  |
| Field no | Value (Dec) |  |  | Size (Bytes) | Function | Content |  |
| 1 | 30 |  |  | 1 | Type |  |  |
| 2 | 0 to 216-1 |  |  | 2 | Length | Total size in bytes, variable |  |
| 4 | 0 to 232-1 |  |  | 4 | Ship ID |  |  |
| 5 | 0-255 |  |  | 1 | Session ID |  |  |
| 6 |  |  |  | Variable | Payload |  |  |

### 5.3.7.2 Fron ship data transfer

These packet formats are similar to the shore originated formats and given in Table A1-17. Short messages can be transferred using the Random Access signalling channel. The very short (11 byte) packet is used for unusual interference conditions and uses a 16 bit CRC and predefined terrestrial addressing.

Table A1-17

From ship Data packet formats

| Short RA message from ship (with ACK) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 040 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length |  |
| 3 | 0 to 232-1 | 4 | Ship ID |  |
| 4 | 0-255 | 1 | Session ID |  |
| 5 | 0-255 | 1 | CQI | Received C/N0 in dBHz |
| 6 |  | 4 | Destination |  |
| 7 |  | Variable | Payload |  |
| Very short RA message from ship (11 bytes, with ACK) – satellite only |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 041-056 | 1 | Type | 16 types of content/ |
| 2 | 0 to 232-1 | 4 | Ship ID |  |
| 3 | 0-255 | 1 | Destination | Preset IPv 6 list |
| 4 |  | 3 | Payload |  |
| 5 | 0 to 216-1 | 2 | CRC-2 | 16 bit CRC used |
| Short RA message from ship (without ACK) |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 057 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length |  |
| 3 | 0 to 232-1 | 4 | Ship ID |  |
| 4 | 0-255 | 1 | Session ID |  |
| 5 |  | 4 | Destination ID | 6 |
| 6 |  | Variable | Payload |  |
| Very short RA message from ship (11 bytes, without ACK) – satellite only |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 058-073 | 1 | Type | 16 types of content |
| 2 | 0 to 232-1 | 4 | Ship ID |  |
| 3 | 0-255 | 1 | Destination | Preset IPv 6 list |
| 4 |  | 3 | Payload |  |
| 5 | 0 to 216-1 | 2 | CRC-2 | 16 bit CRC used |
| From ship Start Fragment/Single Fragment |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 074 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length | Total size in bytes, variable |
| 3 | 0 to 216-1 | 2 | Number of fragments / short message: number of fragments = 1 |  |
| 4 | 0 to 232-1 | 4 | Ship ID |  |
| 5 | 0-255 | 1 | Session ID |  |
| 6 |  | 4 | Destination ID |  |
| 7 |  | Variable | Payload |  |
| From ship Continuation Fragment |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 075 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length | Total size in bytes, variable |
| 3 | 0 to 232-1 | 4 | Ship ID |  |
| 4 | 0-255 | 1 | Session ID |  |
| 5 | 0-255 | 1 | Fragment no |  |
| 6 |  | Variable | Payload |  |
| From ship End Fragment |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 076 | 1 | Type |  |
| 2 | 0 to 216-1 | 2 | Length | Total size in bytes, variable |
| 3 | 0 to 232-1 | 4 | Ship ID |  |
| 4 | 0-255 | 1 | Session ID |  |
| 5 | 0-255 | 1 | Fragment no |  |
| 6 |  | Variable | Payload |  |
| Zero Padding Variable length |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 080 | 1 | Type |  |
| 2 | 0-255 | 1 | Length | Total number of zero bytes |
| Zero Padding Single byte |  |  |  |  |
| Field no | Value (Dec) | Size (Bytes) | Function | Content |
| 1 | 081 | 1 | Type | 1 byte zero padding |

### 5.3.8	Media Access Control

Provides methods for granting data transfer access.

### 5.3.9	Data encapsulation

The data segments of each PL-Frame contain multiple variable length encapsulated datagrams. Each datagram contains the following encapsulation fields:

–	Datagram type (1 byte)

–	Datagram size (3 bytes)

–	Ship ID (4 bytes)

–	Transaction ID (4 bytes, optional)

–	Datagram sequence number (2 bytes, for multisegment datagrams)

–	Source ID (8 bytes, optional)

–	Datagram payload (variable)

–	Data padding (variable, less than 8 bits)

–	CRC (4 bytes).

### 5.3.10	Cyclic redundancy check

Refer to § 3.6.

### 5.3.11	Automatic repeat request (ARQ)

Datagrams may or may not use ARQ, this is defined for each datagram type. An ARQ will request selective retransmission of a specific lost datagram segment.

### 5.3.12	Acknowledgement (ACK)

All datagrams without CRC errors are acknowledged over the satellite link.

### 5.3.13	End delivery notification (EDN)

All datagrams successfully delivered to the destination will be notified to the source.

### 5.3.14	End delivery failure (EDF)

All datagrams not successfully delivered within the timeout or retry limit will be notified to the source.

### 5.3.15	Physical and logical channels  [changed SP to heading 3]

VDES will operate under a wide range of conditions that vary with location and time. The capacity mix used for to/from ship signalling and data channels can therefore be reconfigured over the air to optimise performance. A network can use up to 13 Logical Channels (LC) each using a configurable number of timeslots at a particular frequency, a default Link Configuration ID (PL format) and a defined function (data/signalling) .

VDE-TER uses 7 configurable Logical Channels for signalling and 5 permanently allocated to user data.

VDE-SAT  uses 12 Logical Channels for signalling and data.

Logical Channels are used in signalling and for resource allocations and use a 4 bit field.

The Logical Channel Position uses a 7 bit field.

#### 5.3.15.1	Physical channels

The Physical channels parameters are defined in Tables xxx.  The channels can be 13 (ASM), 25, 50, 100, 150, 300 or 500 kHz wide. Adaptive Coding and Modulation (ACM) functionality is supported when CQI information is provided on the links.

ASM is defined as Random Access Channels using SOTDMA and ACK from a Control Station is sent on the general ASC. The ASM reception flag on the Bulletin Board is set when this functionality is provided. The ASM uplink frequencies are given in Table 3.

#### 5.3.15.2	Logical channels

A LC is defined by the following parameters:

- Logical Channel number (0-12 for 1/2 frequencies, 0-36 for 3/6 frequencies)

- Logical Channel Position (Satellite uplink only, 0-90)

- Function (Random Access, Announcement, Ship originated data, Ship terminated data and Bulletin Board)

- VDE-SAT may use combined functions (Bulletin Board + Assignments, Assignments+ Downlink Data)

- Centre frequency (up to 6)

- Size in slots (0, 1, 2, 3, 5, 30, 90 or 180 slots)

- Default Link Configuration ID (0-63)

The LCs are defined on a Bulletin Board retransmitted every minute starting at slot 0.  LC 0 defines the Bulletin Board.

The VDE-TER lowest frequency starts with LC 0.  The next frequency starts with LC13.  VDE-SAT LCs starts with the highest frequency.

The Terrestrial Bulletin Board is transmitted on the lowest frequency only and the Satellite Bulletin Board on the highest frequency only.

##### 5.3.15.2.1	Signalling logical channels

The following to ship signalling LC functionality/content is defined:

- Bulletin Board (Network ID, network configuration, system information, network operator message)

- Announcement channel (Bulletin Board version, ship paging, resource allocation (LC)/ LC Position no/access reject reason,  uplink minimum access priority level, random access randomizing channels, random access retry interval, received CQI, ACK/selective NACK, Message ACK, connection disconnect/reason, End Delivery Notifications)

The following from ship signalling LC functionality/content is defined:

- Random Access Data (short message, CQI, ACK supported flag)

- Random Access Resource request (priority level, CQI, datagram size, retry count)

- Random Access ACK (message ID, CQI, ACK/selective NACK, End Delivery Notification)

Assigned Signalling (message ID, CQI, ACK/selective NACK, End Delivery Notification)

##### 5.3.15.3 Bulletin Board

The Bulletin Board content is defined in Table A1-18

Table A1-18

VDES Bulletin Board construction (fixed fields)

| Name | Description | Total size (bytes) | Comment |
| --- | --- | --- | --- |
| Network (area of applicability) ID | The ID/type of the Network | 4 | The first 2 bits (MSB) define the Bulletin Board type. 00 is used for ship to shore 01 is used for shore to ship 10 is used for ship to ship 11 is used for satellite The remaining 30 bits are used to uniquely address up to 1.07 109 networks |
| Control Station ID | The Station number within a Network | 1 |  |
| Bulletin Board version | Version number of this Bulletin Board | 2 | All valid versions are stored in the ship terminal (includes Configuration Message) |
| Validity of this version | Lifetime of this version in number of 1 minute frames | 2 | Up to 45 days |
| Logical Channel definitions | Logical Channel, Link Configuration ID, slot size, function | TER: 42 SAT: 72 | See TABLE A1-15, A1-16, A1-17 Up to 37 Logical Channels using 6 frequencies defined. Additional Logical Channels can be defined in a Configuration Message, see TABLE A1-19 |
| Modulation, coding and protocol versions supported | Discrete combinations | 1 | Defines a mandatory base set and optional more capable versions. Network ID segmentation could be used to distinguish different network types. ASM reception flag one of the parameters for satellite. |
| Service Area Point 1 | Packet defining the Service Area North East corner | 7 | GNSS rectangle as defined in Recommondation ITU-R M.1371 |
| Service Area Point 2 | Packet defining the Service Area South West corner | 7 |  |
| System Configuration Message | Logical Channel number for the Configuration Message containing additional parameters/data | 1 | Used for additional data/information that does not fit within a standard Bulletin Board. Defined in Table 5. Set to 0 when not used. |
| Expansion | Future use | 9 |  |
| Authentication and integrity sequence |  | 32 | Optional. Filled with zeros if not used. |
| Padding |  | 0 | TBD |
|  | Fixed field size | 108/139=8 | TER/SAT Bytes |

The relationship between LC and Physical Layer for Ship to shore, Shore to ship, Ship to Ship and Satellite to/from Ship mappings are shown in Figures A1-15, A1-16, A1-17, and A1-18 [A, B, C and D].

###### 5.3.15.3.1 VDE-TER Default slot to LC mapping

VDE-TER uses hex slot numbering and all signalling takes place in Hex slot 0.  There are TDMA 5 user data channels, each use exclusively one Hex slot (1 to 5). The 5 User Data Channels corresponds to Logical Channels 8 to 12. Each data channel has a dedicated Assignment Channel used for signalling (ASC1 to ASC5).

The function, default Link Configuration ID and number of slots for Logical Channels 0 to 6 are set by the Control Station, and repeated until the end of the frame is reached. (slot 2249). The Control Station can optimise the capacity used for random access and signalling based on traffic loading.   Logical Channel 7 is not used in VDE-TER and has a size of 0 slots.

Figure A1-15

VDE-TER Ship to shore default slot to LC mapping (lower leg)

| Logical channel | 1 |  |  |  |  | 2 | 3 | 4 | 5 | 6 |  | 6 | Logical channel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | RACH |  |  |  |  | ASC1 | ASC2 | ASC3 | ASC4 | ASC5 | …. | ASC5 |  |
| 1 | User Data Channel 1 (UDCH1) associated with ASC1 |  |  |  |  |  |  |  |  |  | …. |  | 8 |
| 2 | User Data Channel 2 (UDCH2) associated with ASC2 |  |  |  |  |  |  |  |  |  | …. |  | 9 |
| 3 | User Data Channel 3 (UDCH3) associated with ASC3 |  |  |  |  |  |  |  |  |  | …. |  | 10 |
| 4 | User Data Channel 4 (UDCH4) associated with ASC4 |  |  |  |  |  |  |  |  |  | …. |  | 11 |
| 5 | User Data Channel 5 (UDCH5) associated with ASC5 |  |  |  |  |  |  |  |  |  | …. |  | 12 |
| TDMA channel number | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  | 374 | Logical channel |
|  | Hex slot |  |  |  |  |  |  |  |  |  |  |  |  |

Figure a1-16

VDE-TER Shore to ship default slot to LC mapping (upper leg)

| Logical channel | 0 |  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 |  | 5 | Logical channel |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | TBB |  |  |  |  | ASC 1 | ASC 2 | ASC 3 | ASC 4 | ASC 5 | RACH | …. | ASC 5 |  |
| 1 | User Data Channel 1 (UDCH1) associated with ASC1 |  |  |  |  |  |  |  |  |  |  | …. |  | 8 |
| 2 | User Data Channel 2 (UDCH2) associated with ASC2 |  |  |  |  |  |  |  |  |  |  | …. |  | 9 |
| 3 | User Data Channel 3 (UDCH3) associated with ASC3 |  |  |  |  |  |  |  |  |  |  | …. |  | 10 |
| 4 | User Data Channel 4 (UDCH4) associated with ASC4 |  |  |  |  |  |  |  |  |  |  | …. |  | 11 |
| 5 | User Data Channel 5 (UDCH5) associated with ASC5 |  |  |  |  |  |  |  |  |  |  | …. |  | 12 |
| TDMA channel number | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |  | 374 | Logical channel |  |
|  | Hex slot |  |  |  |  |  |  |  |  |  |  |  |  |  |

By monitoring the Terrestrial Bulletin Board, ships will determine if they are within a Control Station service area and not initiate ship-to-ship transmissions when interference to ship to shore communications is likely.

Figure a1-17

VDE-TER Ship to ship default slot to LC mapping (upper leg)

| Logical channel | 0 |  |  |  |  | 1 | 2 | 3 | 4 |  | 5 | 6 |  | 5 | Logical channel |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | TBB |  |  |  |  | ASC 1 | ASC 2 | ASC 3 | ASC 4 |  | ASC 5 | RACH | …. | ASC 5 |  |
| 1 |  | User Data Channel 1 (UDCH1) associated with ASC1 |  |  |  |  |  |  |  |  |  |  | …. |  | 8 |
| 2 |  | User Data Channel 2 (UDCH2) associated with ASC2 |  |  |  |  |  |  |  |  |  |  | …. |  | 9 |
| 3 |  | User Data Channel 3 (UDCH3) associated with ASC3 |  |  |  |  |  |  |  |  |  |  | …. |  | 10 |
| 4 |  | User Data Channel 4 (UDCH4) associated with ASC4 |  |  |  |  |  |  |  |  |  |  | …. |  | 11 |
| 5 |  | User Data Channel 5 (UDCH5) associated with ASC5 |  |  |  |  |  |  |  |  |  |  | …. |  | 12 |
| TDMA channel number | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  | 9 | 10 |  | 374 | Logical channel |  |
|  |  | Hex slot |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 5.3.15.3.2 VDE-SAT default Bulletin Board

Hex slot numbering is not used by VDE-SAT because physical layer formats with length 1, 3, 5 are used on uplink and 90 slots is used for downlink. Global slot numbering from 0 to 2249 is used.

Figure D shows the default half-duplex satellite LC channel mapping.

Figure A1-18 –

VDE-SAT half duplex default slot to LC mapping (upper and lower leg)

|  | SBB+ ASC | ASC+ Data Down | Data Down 1 | Data Down 2 |  | Data Down 3 |  | Data Down 4 |  | Data Up 1 |  | Data Up 2 |  | Data Up 3 |  | Data Up 4 |  | Data Up 5 |  | Data Up 6 | RACH |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Global start slot | 0 | 90 | 180 | 270 |  | 360 |  | 450 |  | 540 |  | 570 |  | 600 |  | 630 |  | 660 |  | 690 | 720 |  |
| Size in slots | 90 | 90 | 90 | 90 |  | 90 |  | 90 |  | 30 |  | 30 |  | 30 |  | 30 |  | 30 |  | 30 | 90 |  |
| Logical channel | 0 | 1 | 2 | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 | 12 |  |
| Repeat 1 |  | ASC+ Data Down | Data Down 1 | Data Down 2 |  | Data Down 3 |  | Data Down 4 |  | Data Up 1 |  | Data Up 2 |  | Data Up 3 |  | Data Up 4 |  | Data Up 5 |  | Data Up 6 | RACH |  |
| Global start slot |  | 810 | 900 | 990 |  | 1080 |  | 1170 |  | 1260 |  | 1290 |  | 1320 |  | 1350 |  | 1380 |  | 1410 | 1440 |  |
| Size in slots |  | 90 | 90 | 90 |  | 90 |  | 90 |  | 30 |  | 30 |  | 30 |  | 30 |  | 30 |  | 30 | 90 |  |
| Logical channel |  | 1 | 2 | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 | 12 |  |
| Repeat 2 |  | ASC+ Data Down | Data Down 1 | Data Down 2 | Data Down 3 |  | Data Down 4 |  | Data Up 1 |  | Data Up 2 |  | Data Up 3 |  | Data Up 4 |  | Data Up 5 |  | Data Up 6 |  |  | RACH |
| Global start slot |  | 1530 | 1620 | 1710 | 1800 |  | 1890 |  | 1980 |  | 2010 |  | 2040 |  | 2070 |  | 2100 |  | 2130 |  |  | 2160 |
| Size in slots |  | 90 | 90 | 90 | 90 |  | 90 |  | 30 |  | 30 |  | 30 |  | 30 |  | 30 |  | 30 |  |  | 90 |
| Logical Channel |  | 1 | 2 | 3 | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  |  | 12 |

Downlink slots are transmitted by the satellite 2 ms before UTC epoch, and uplink slots are received 2 to 10 ms after UTC epoch. This could cause the satellite to 4 to 12 ms of a packet when switching from receive to transmit. The last receive slot before a transmit slot is therefore not used. For the default configuration the last RACH slot is not used.

The VDE-SAT and ASM-SAT uplink formats use slots lengths 1, 3 and 5 and these are repeated until all allocated/assigned slots in a Logical Channel are used.

In the default Bulletin Board all uplink data Logical Channels are 30 slots long and may contain 30, 10 or 6 assigned transmissions for burst lengths 1,3 or 5 slots. These slots are assigned using the LC number and a LC Position (LCP). The LC Position number can range from 1 to 90 (7 bit field).

For the default Bulletin Board, a ship can only transmit 3 times during a 1 minute frame. There are also three periods of 12 s when the ship is not transmitting, thereby providing quiet periods when ships with 10s AIS reporting interval can be detected.

Table A1-19 shows the Logical Channel parameters for a frequency pair. For VDE-TER the lowest frequency pair starts with LC 0, the 2nd start with 13. Up to 127 LCs can be supported. For VDE-SAT the highest frequency starts with LC 0.

The ship equipment shall prevent any transmission on channels 16 and 70 for all Bulletin Board settings.

Table A1-19

Logical Channel definition

| Name | Value | Field size (bits) | Comment |
| --- | --- | --- | --- |
| Frequency 1: Ship rx Channel Frequency Index (CFI) | 156-162.0375 MHz, 12.5 kHz step Channels 0-482 Centre frequency= 156+0.0125*CFI | 9 | VDES upper leg: 465: 161.8125 MHz (TER default) 469: 161.8625 MHz 473: 161.9125 MHz (SAT default) |
| Frequency 2: Ship tx CFI | 156-162.0375 MHz, 12.5 kHz step Channels 0-482 | 9 | VDES lower leg: 97: 157.2125 MHz (TER default) 101: 157.2625 MHz 105: 157.3125 MHz (SAT default) ASM: 476: 161.950 MHz 480: 162.000 MHz |
| BB Link Configuration ID Logical channel 0 | 0-63 | 6 | The symbol rate, bandwidth, burst duration, modulation and coding are defined in Table 4. |
| BB size in slots | TER: 0, 1, 2, 3 or 5 SAT: 0, 90 or 180 | 3 | Defaults: TER: 5 (011) SAT: 90 (001) |
| BB data flag | 0 or 1 | 1 | Set to 1 if signalling is transmitted on the Bulletin Board Logical Channel. Default 0. |
| ASM ACK flag | 0 or 1 | 1 | Set to 1 if ASM messages either channel is acknowledged. Default 0. |
| Logical Channel 1 |  |  |  |
| Tx flag | 0 or 1 | 1 | 0 for ship receive, 1 for transmit, default 0 |
| Link Configuration ID | 0-63 | 6 |  |
| Size in slots | 0, 1, 2, 3, 5, 30, 90 or 180 | 3 | Logical channel not used: 0 (000) |
| Function | TER: 1: RACH 2: ASC 1 3: ASC 2 4: ASC 3 5: ASC 4 6: ASC 5 SAT: 1: RACH 2: ASC 3: ASC/Data down 4: Data Down 5: Data Up | 3 | Default: TER: Fig xx SAT: Fig yy |
| …………………. |  |  | Frequency + BB= 29 bits Each Logical Channel=13 bits |
| Logical Channel N TER: N=6 SAT: N=12 |  |  |  |
| Tx flag | 0 or 1 | 1 | 0 for ship receive, 1 for transmit, default 0 |
| Link Configuration ID | 0-63 | 6 |  |
| Size in slots | 0, 1, 2, 3, 5, 30, 90, 180 | 3 | Default: 90 (111) Not used: 0 (000) |
| Function | TER: 0: Bulletin Board 1: RACH 2: ASC general 3: ASC 1 4: ASC 2 5: ASC 3 6: ASC 4 7: ASC 5 SAT: 0: Bulletin Board 1: RACH 2: ASC 3: ASC/Data Down 4: Data Down 5: Data Up | 3 | Default: TER: Fig xx SAT: Fig yy |
|  | Total/Channel | TER: 14 Bytes SAT: 24 Bytes | TER: 29+ 6*13+ 5 spare=112 bits=14 B SAT: 29+12*13+7 spare=192 bits=24 B |
|  | Total 3 channels | TER: 42 B SAT: 72 B |  |

###### 5.3.15.3.3  Configuration Message

The Configuration Message, Table A1-20, has a variable length data format and is used to handle additional information that is not defined or does not fit within the Bulletin Board.

Table A1-20

Configuration Message construction (variable fields)

| Name | Description | Total size (bytes) | Comment |
| --- | --- | --- | --- |
| Channel Formats | Additional frequencies/slot usage | 42/72 | TER/SAT new channel |
| Free text message | Containing up to 254 ASCII characters | 256 | Network operator message to all ships, information only. Includes 8 bit packet type and 8 bit message length |
| Padding | A zero filling sequence with no content. | 3 | Includes 8 bit packet type and 16 bit length |

###### 5.3.15.3.4 Digital Signature of Bulletin Board {DRAFT, requires further work}

It is assumed that a Public Key Infrastructure (PKI) is established with primarily IMO as Certificate Authority (CA), and that ITU-T X.509 (10/2016) is used for public key certificates and the PKI implementation. The PKI will serve several systems and among these VDES. For VDES the primary purpose is to attach a digital signature to the Bulletin Board (BB) issued by a VDES control station to authenticate the control station transmitting the BB.

Ships will need to retrofit a dedicated PKI unit to their bridge system, or build the functionality into the VDES equipment. This unit provides cryptographic services to both general and bridge network applications. The unit will utilize a smartcard for tamper-proof storage of the security credentials.

In case the verification of the signature fails on the VDES mobile station this shall be flagged to the user {Rephrase, we have no display}. The system shall continue its operation as if the signature was verified.

Cryptographic algorithm for the end-entities digital signatures is the Elliptic Curve Digital Signature Algorithm (ECDSA). The Elliptic Curve Cryptography (ECC) public key shall therefore be 256 bit. With this key size, the recommendations from RFC 5480 states that the minimum bits of security should be 128, the message digest algorithm SHA-256, and the curve secp256r1. The lifetime of the selected key material is 3 years. [Paragraph TBD based on available space in BB. Refer to ch 3.5.3 in d2.2 from CySiMS].

###### Communication with separate PKI unit shall be based on network protocol. [Standard for message exchange TBD] [Ref to IEC 61162-450 and -460?]

## 5.4	Network layer

This layer is responsible for the management of priority assignments of messages, distribution of transmission packets between channels and data link congestion resolution.

### 5.4.1	Data transfer protocols

The following downlink protocols shall be supported:

- Shore originated multicast

- Shore originated unicast

- Ship originated short message

- Ship to ship unicast inside/outside controlled areas (note for Hans , new drawing)

- Ship to ship short message

- Shore originated short message

### 5.4.1.1	Shore originated Multicast without ACK

The timing diagram for shore originated multicast without ACK is shown in figure A1-19.  The transfer starts with a resource allocation. The diagram shows a large multi-fragmented datagram.  Both predefined groups and geographical areas are supported.

The datagram payload has source, destination and format encapsulated for routing and presentation purposes.

figure a1-19

multicast timing diagram

### 5.4.1.2	Shore originated Unicast

The timing diagram for shore originated unicast is shown in figure A1-20.  The transfer starts with a ship/ ship paging to verify that it is within coverage.  The ships response packet contains terminal capabilities and received signal quality. At the Control Station this is used to allocate an exclusive Logical Channel for the transfer. The diagram shows a large multi-fragmented datagram.  Normally up to 16 fragments are sent before the ship sends a selective NACK indicating which fragments have to be resent. The Logical Channel is kept allocated until all fragments have been received by the ship and a ACK has been received or a retry limit has been exceeded.

The datagram payload has source, destination and format encapsulated for routing and presentation purposes.

figure A1-20

To ship Unicast protocol timing diagram

### 5.4.1.3	Ship orginated short message

The timing diagram for ship originated short message is shown in figure A1-21.  This protocol is used for short messages that fit within a single transmission burst.  A random slot in the randomizing interval given in the MAC signalling is used for the transmission. Zero padding is added as required.  16 byte IPv6 addressing is used.

When maximum robustness is needed a special fixed 11 byte packet is used. 16 different packet types define the content. All types can select from 255 pre-assigned terrestrial addresses.  A 16 bit CRC is used for these packets.

The Control Station sends and ACK when the message is received correctly, otherwise the ship may automatically retry until the retry limit is reached.

figureA1-21

Short message protocol timing diagram

### 5.4.1.4	Ship orginated data transfer

The timing diagram for ship originated data transfer is shown in figure A1-22.  The transfer starts with the ship requesting Control Station resources. The Control Station allocate an exclusive Logical Channel for the transfer. The diagram shows a large multi-fragmented datagram.  Normally up to 16 fragments are sent before the Control Station sends a selective NACK indicating which fragments have to be resent. The Logical Channel is kept allocated until all fragments have been received by the Control Station and the final ACK has been received or the retry limit has been exceeded.

The End Delivery Notification is optional and defined in the datagram payload, it is mainly used in store-and-forward systems.

figure A1-22

Ship originated datagram

### 5.4.1.5 Ship to ship short message

Resource and mobility management

Ships outside the Control Station range may communicate directly. In this case AIS receptions may be used to determine if a ship is within range, combined AIS/VDES transceivers will set a bit in their periodic AIS report that indicates VDES support.

To minimize transmissions to ships outside communications range, sending short messages or paging other ships shall not use more than 1% of the signaling channel (LC 0) slots when the signaling channel is loaded less than 1% averaged over 1 minute. During heavier loading, the activity rate shall be reduced to 0,01%.

The protocol timing for ship to ship short message transfer is shown in FIGUREA1-23.

figure a1-23 ship to ship short message transfer diagram

### 5.4.1.6 Ship to ship communications

Ship to ship communications uses a modified RATDMA protocol where up to 6 Logical Channels are defined. Logical Channel 0 is used for signaling. The called ship selects the most quite channel.

figure a1-24

Ship to ship protocol timing diagram

### 5.4.1.7 Ship to ship short message

The Control Station service area is defined on its Bulletin Board. Within this service area the Control Station manages the VDES resources to minimize interference.

AIS receptions may be used to determine if a ship is within range, combined AIS/VDES transceivers will set a bit in their periodic AIS reports that indicates VDES support.

The resource is de-allocated by the Control Station if the called ship is unreachable.

Field 4 in packet type #6  is set to 11 if the destination ship is not reachable.

See the protocal timing diagram in FIGURE A1-23.

FIGURE A1-23

Ship to ship short message via control station diagram

## 5.5	Transport layer

This layer ensures reliable transmission of the data segments between ships, ship and shore, and ship and satellite, including segmentation, acknowledgement and multiplexing.

### 5.5.1	Presentation interface protocol

Data, which is to be transmitted by the station, may be input via the presentation interface; data, which is received by the station, should be output through the presentation interface. The formats and protocol used for this data stream are defined by IEC 61162 series.

### 5.5.2	End to end protocols

Existing Internet protocols such as UDP, SNMP, secure file transfer protocol (SFTP), simple mail transfer protocol (SMTP) as shown in Figs. A4-10 to A4-15 are used.

Terrestrial IP protocols are assumed to be terminated at the Control Station gateway.

### 5.5.	3 Ship, application and device physical addressing

Most commercial ships use a 7-digit IMO number of which the last is a checksum, thus the IMO system can address 1 million ships. The 4 byte VDES physical addressing field has 4.3  109 unique IDs. The Control Station or the access network will therefore include a Network Address Translator that converts either a 16 byte IPv6 address or a 7 digit IMO number to this 4 byte ship ID.

The number of networked applicatioins / devices on ships is growing fast and the datagrams will, where needed, include sub addresses.

### 5.5.4 Terrestrial network addressing

IPv6 with 16 byte addressing is used.

To support multiple connections/applications from a ship, up to 255 sessions with unique IDs can be supported, these can be routed to different IPv6 addresses.

5.5.5 Mobility management

Last transaction coarse position or AIS location data stored in the Control Station database is used to route paging of ships.

### 5.5.6	Shore addressing of ships, gateways and devices

VDES will be accessed from shore using Internet, and it is desirable to use standard protocols such as email.

A database at the gateway will allow shore users to define their own meaningful ship, gateway and device names.

Training sequence

27 sym

Ramp up

4 symbols

Link CFG

ID

16 sym

Data with CRC-32

192, 448, 704, 960 symbols

Ramp Down

4 symbols

Guard

13   symbols

ASM

Bandwidth 16 kHz

9.6 ksym/s

26.67 ms

256 symbols

Training sequence

27 sym

Ramp up

16 symbols

Link CFG

ID

16 sym

Data with CRC-32

896 symbols

Ramp Down

16 symbols

Guard

53 symbols

VDE

Bandwidth 50 kHz

38.4 ksym/s

1024 symbols

26.67 ms

Training sequence

27 sym

Ramp up

32 symbols

Link CFG

ID

16 sym

Data with CRC-32

1792 symbols

Ramp Down

32 symbols

Guard

[149] symbols

VDE

Bandwidth 100 kHz

76.8 ksym/s

2048 symbols

26.67 ms

Training sequence

27 sym

Ramp up

8 symbols

Link CFG

ID

16 sym

Data with CRC-32

432 symbols

Ramp Down

8 symbols

Guard

21 symbols

VDE

Bandwidth 25 kHz

19.2 ksym/s

512 symbols

26.67 ms

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

27 sym

Ramp up

4 symbols

Link CFG

ID

16 sym

Data with CRC-32

192, 448, 704, 960 symbols

Ramp Down

4 symbols

Guard

13   symbols

ASM

Bandwidth 16 kHz

9.6 ksym/s

26.67 ms

256 symbols

Training sequence

27 sym

Ramp up

16 symbols

Link CFG

ID

16 sym

Data with CRC-32

896 symbols

Ramp Down

16 symbols

Guard

53 symbols

VDE

Bandwidth 50 kHz

38.4 ksym/s

1024 symbols

26.67 ms

Training sequence

27 sym

Ramp up

32 symbols

Link CFG

ID

16 sym

Data with CRC-32

1792 symbols

Ramp Down

32 symbols

Guard

[149] symbols

VDE

Bandwidth 100 kHz

76.8 ksym/s

2048 symbols

26.67 ms

Training sequence

27 sym

Ramp up

8 symbols

Link CFG

ID

16 sym

Data with CRC-32

432 symbols

Ramp Down

8 symbols

Guard

21 symbols

VDE

Bandwidth 25 kHz

19.2 ksym/s

512 symbols

26.67 ms

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

Training sequence

14 sym

Ramp up

[xx] symbols

Pilot

[xx] sym

Data with CRC-32

[xx] symbols for 1 block burst

Ramp Down

[xx] symbols

Guard

[xx]   symbols

SAT

Bandwidth 50 kHz

[xx.x] ksym/s

26.67 ms

[xxxx] symbols

GD=1n0(D)d(D)n1(D)d(D)

n0D=1+D+D3

n1D=1+D+D2+D3

dD=1+D2+D3.

m=s-1 mod 2

i=floor (s-1 /(2k2))

j=floor (s-1 /2))-ik2

t=19i+1 mod (k1/2)

q=t mod 8+1

c=pqj+21m mod k2

πs=2t+ck1/2+1-m

Annex 2

Technical characteristics of the Application Specific Message (ASM) channels for the VDES in the VHF maritime mobile band

1	Introduction

This section describes those elements of the ASM that are unique to ASM operation. It contains a description of the different protocols according to the OSI layer model and recommends implementation details for each layer. For those elements that are common, the cross reference into Annex 1 is provided.

The system should use TDMA techniques in a synchronized manner.

This Annex describes the characteristics of the TDMA access schemes which include random access TDMA (RATDMA), Multiple Incremental TDMA (MITDMA), fixed access TDMA (FATDMA), techniques. Slot carrier sense TDMA (SCTDMA) may be implemented as a option.  The behavior should be in accordance with Recommendation ITU-R M.1371-5 Annex 7.

## 1.1	OSI layers

Refer to Annex 1.

### 1.1.1	Physical layer

Convert digital transmission packet to π/4 Quadrature Phase-Shift Keying (QPSK) signal to modulate transmitter.

### 1.1.2	Link layer

The link layer is divided into three sub-layers with the following tasks.

#### 1.1.2.1	Link management entity

This sub layer has the following functions:

–	Assemble ASM message bits

–	Order the ASM message bits into 8-bit byte for assembly of transmission packet.

#### 1.1.2.2	Data link services

This sub layer has the following functions:

–	Calculate frame check sequence (FCS) of the ASM message bits (see § 3.2.2.3).

–	Append FCS to ASM message to complete creation of transmission packet contents.

–	Complete assembly of transmission packet.

#### 1.1.2.3	Media access control

Media access control provides a method for granting access to the data transfer to the VHF data link (VDL). The method used is a TDMA scheme using a common time reference.

### 1.1.3	Network layer

The network layer is responsible for the management of priority assignments of messages, distribution of transmission packets between channels, and data link congestion resolution.

### 1.1.4	Transport layer

The transport layer is responsible for converting data into transmission packets of correct size and sequencing of data packets.

# 2	Physical layer

## 2.1	Parameters

### 2.1.1	General

The physical layer is responsible for the transfer of a bit-stream from an originator, out on to the data link. The performance requirements for the physical layer are summarized in Tables A2-1 to A2-3.

The low setting and the high setting for each parameter is independent of the other parameters.

TABLE A2-1

Minimum required time division multiple access transmitter characteristics

| Parameter name | Units | Low setting | High setting |
| --- | --- | --- | --- |
| Channel spacing (encoded according to RR Appendix 18 with footnotes)(1) | kHz | 25 | 25 |
| ASM 1 (2027)(1) | MHz | 161.950 | 161.950 |
| ASM 2 (2028)(1) | MHz | 162.000 | 162.000 |
| Transmit output power | W | 1 | 12.5 |
| (1) See Recommendation ITU-R M.1084, Annex 4. |  |  |  |

### 2.1.2	Transmission media

Data transmissions are made in the VHF maritime mobile band. Data transmissions should use ASM 1 and/or ASM 2 channels.

### 2.1.3	Multi-channel operation

The ASM station should be capable of receiving on two parallel channels and transmitting on two independent channels. Two separate TDMA receiving processes should be used to simultaneously receive on two independent frequency channels. One TDMA transmitter may be used to enable TDMA transmissions on one or two independent frequency channels.

ASM transmission should alternate between the two ASM channels

MITDMA linked transmissions should be on the same channel.

## 2.2	Transceiver characteristics

The transceiver should perform in accordance with the characteristics set forth herein, see TABLE A2-2 and FIGURE A2-1a. The resolution bandwidth for the mask measurement is 300Hz.

TABLE A2-2

Minimum required time division multiple access transmitter characteristics

| Transmitter parameters | Requirements |
| --- | --- |
| Carrier power error | ±1.5 dB |
| Carrier frequency error | ± 500 Hz |
| Slotted modulation mask | ∆fc < ±8 kHz: 0 dBc ±8 kHz < ∆fc < ±16 kHz: below the straight line between −25 dBc at ±8 kHz and –60 dBc at ±16 kHz ±16 kHz < ∆fc < ±25 kHz: below the straight line between −60 dBc at ±16 kHz and –70 dBc at ±25 kHz ±25 kHz < ∆fc < ±62.5 kHz: −70 dBc |
| Spurious emissions | −36 dBm: 9 kHz ... 1 GHz −30 dBm: 1 GHz ... 4 GHz |

FIGURE A2-1a ASM Slotted modulation mask

TABLE A2-3

Minimum required time division multiple access receiver characteristics without FEC

| Receiver parameters | Requirements |
| --- | --- |
| Sensitivity | 20% PER @ −107 dBm |
| Error behaviour at high input levels | 1% PER @ −77 dBm 1% PER @ −7 dBm |
| Adjacent channel selectivity | 20% PER @ 70 dB |
| Spurious response rejection | 20% PER @ 70 dB |
| Intermodulation response rejection | 20% PER @ 74 dB |
| Spurious emissions | −57 dBm (9 kHz to 1 GHz) −47 dBm (1 GHz to 4 GHz) |
| Blocking | 20% PER @ 86 dB |

## 2.3	Modulation scheme

The base modulation is defined by the Link Configuration Id, see Table [x].

For the modulation bit mapping, see Annex 1.

### 2.4	Frequency stability

The frequency stability of the VHF radio transmitter/receiver should be ±500 Hz or better.

## 2.5	Data transmission bit rate

The transmission bit rate should be 19.2 kbit/s ±10 ppm for π/4 QPSK.

## 2.6	Frame structure

For the generic definition of the frame structure, see Annex 1.

## 2.7	Signal information

Signal information selects the modulation scheme and coding according to the Link Configuration Id defined in Table A1-6

## 2.8	Forward error correction and Bit Scrambling

When forward error correction is used, it will be used as defined in Annex 1. Interleaving and bit scrambling are used, as defined by the FEC designated in the signal information. In the event of no FEC, bit scrambling according to Annex 1 shall be implemented.

## 2.9	Transmitter transient response

The time taken to switch from transmit to receive conditions, and receive to transmit conditions, should not exceed the transmit ramp up and ramp down , see Annex 1; 5.2.3.1. It should be possible to receive a message from the slot directly after or before own transmission.

The equipment should not be able to transmit during channel switching operation.

## 2.10	Transmitter power

The power level is determined by the link management entity (LME) of the link layer.

2.10.1	Provision should be made for two levels of nominal power (high power, low power) as required by some applications. The default operation of the ASM station should be on the high nominal power level.

2.10.2	The nominal levels for the two power settings should be 1 W (PEP) and 12.5 W (average power); tolerance should be within ±1.5 dB.

# 3	Link layer

The link layer specifies how data is packaged in order to apply error detection and correction to the data transfer. The link layer is divided into three sub-layers.

## 3.1	Sub-layer 1 – medium access control

The medium access control (MAC) sub layer provides a method for granting access to the data transfer medium, i.e. the VHF data link. The access scheme is TDMA using a common time reference.

### 3.1.1	TDMA synchronization

TDMA synchronization is achieved using an algorithm based on a synchronization state as described below. The sync state flag within the  MITDMA communication state indicates the synchronization state of a station.

The TDMA receiving process should not depend on slot boundaries.

Synchronization other than UTC direct may be provided by the AIS system.

#### 3.1.1.1	Coordinated universal time direct

A station, which has direct access to coordinated universal time (UTC) timing, with the required accuracy, should indicate this by setting its synchronization state to UTC direct.

#### 3.1.1.2	Coordinated universal time indirect

A station, which is unable to get direct access to UTC, but has access to the AIS system, may get its synchronization from the AIS system. It should then change its synchronization state to indicate the type of synchronization which is being provided by the AIS system.

### 3.1.2	Time division

The slot and frame are as defined in Annex 1. Access to the data link is, by default, given at the start of a slot. The frame start and stop coincide with the UTC minute, when UTC is unavailable the AIS system may provide the frame synchronization.

### 3.1.3	Slot phase and frame synchronization

Slot phase synchronization and frame synchronization is done by using information from UTC or from the AIS system.

#### 3.1.3.1	Slot phase synchronization

Slot phase synchronization is the method whereby the slot boundary is synchronized with a high level of synchronization stability, thereby ensuring no message boundary overlapping or corruption of messages.

#### 3.1.3.2	Frame synchronization

Frame synchronization is the method whereby the current slot number for the frame is known.

### 3.1.4	Slot identification

Each slot is identified by its index (0-2249). Slot zero (0) should be defined as the start of the frame.

### 3.1.5	Slot access

The transmitter should begin transmission by turning on the RF power at slot start.

The transmitter should be turned off after the last bit of the transmission packet has left the transmitting unit. This event must occur within the slots allocated for own transmission. The slot access is performed as described in Annex 1  § -5.2.2.

### 3.1.6	Slot state

Each slot on an ASM channel can be in one of the following states:

–	Free: meaning that the slot is unused on the channel within the receiving range of the own station for AIS/ASM/VDE

–	Internal allocation allocated by own station for the purpose of AIS/ASM/VDE/long range transmission

–

-	 AIS Available –externally allocated by an AIS which may be considered as a candidate slot

–	AIS Unavailable –externally allocated by an AIS which should not be considered as a candidate slot for the following reasons:

SOTDMA slot timeout = 0

FATDMA allocated slot

–	ASM Unavailable – Externally allocated by an ASM station within the receiving range of the own station

–	VDE Unavailable – slots allocated to own station for VDE reception

## 3.2	Sub layer 2 – data link service

The data link service (DLS) sub layer provides methods for:

–	data link activation and release;

–	data transfer; or

–	error detection, correction and control.

### 3.2.1	Data link activation and release

Based on the MAC sub layer the DLS will listen, activate or release the data link. A slot, marked as free or externally allocated, indicates that own equipment should be in receive mode and listen for other data link users.

### 3.2.2	Data transfer

Data transfer should use a bit-oriented protocol and should be in accordance with this standard.

#### 3.2.2.1	Packet format

Data is transferred using the generic transmission packet as defined in Annex 1 Fig. A1-4:

The packet should be sent from left to right. The training sequence is be used to synchronize the VDES receiver.

#### 3.2.2.2	Summary of the transmission packet

The data packet is defined in Table A2-4.

The ASM channel configurations are defined by Link Configuration Id table, see Table [x].

TABLE A2-4

Packet symbol structure for π/4 QPSK modulation scheme

|  |  | Description |
| --- | --- | --- |
| Ramp up | 4 symbols |  |
| Training sequence | 27 symbols | Necessary for synchronization |
| Link configuration id | 16 symbols | Decoded from (32,6) biorthogonal code ASM channel configurations as defined in Link Configuration Id table |
| Data | 1 slot: 176 2 slot: 432 3 slot: 688 symbols | The symbol count and the information bits varies according to coding rate as defined by the link configuration id field |
| CRC | 16 symbols | The CRC only includes the data field |
| Ramp Down | 4 symbols | Distance delay and jitter |
| Guard Time | 13 symbols | Distance delay and jitter |
| Total | 1 slot: 256 2 slot: 512 3 slot: 768 symbols |  |

#### 3.2.2.3	Transmission timing

The modulation may be applied during the ramp up period, but it shall not be considered as part of the training sequence

#### 3.2.2.4	Long transmission packets

A station may occupy a maximum of 3 consecutive slots, as defined by the Link Configuration id, for one (1) continuous transmission. Only a single application of the overhead (ramp up, training sequence, flags, FCS, guard time) is required for a long transmission packet.

### 3.2.3	Error detection and control

Error detection is accomplished using a CRC polynomial as described in Annex 1.

### 3.2.4	Forward Error correction

Forward error correction should be handled as described in Annex 1, and specified by the Link Configuration Id see Table A1-6.

## 3.3	Sub layer 3 – link management entity

The LME controls the operation of the DLS, MAC and the physical layer.

### 3.3.1	Access to the data link

There should be different access schemes for controlling access to the data transfer medium. The application and mode of operation determine the access scheme to be used.

The access schemes are  MITDMA, RATDMA, and FATDMA.

#### 3.3.1.1	Cooperation on the data link

The access schemes operate continuously, and in parallel, on the same physical data link. They all conform to the rules set up by the TDMA. The ASM system must give priority to the AIS system when accessing the physical data link.

#### 3.3.1.2	Candidate slots

Slots, used for transmission, are selected from candidate slots in the selection interval (SI) which is defined as 300 slots.

The selection process uses received data from AIS, ASM and VDE.

There should be, at minimum, a set of eight candidate slots to choose from.

The candidate slots are primarily selected from slots that are free on AIS, ASM and VDE.

The available AIS slots are as defined in Recommendation ITU-R M.1371 and must only be taken from the most distant station(s) within the SI.

If the candidate slot set contains less than eight slots, additional candidate slots can be obtained by using the following rules and order (rule 1 followed by rule 2):

Rule 1:	available slot on one AIS channel and free on the other AIS and  free on the ASM and VDE channels

Rule 2:	available slot on both AIS channels (AIS1 and AIS2) and free on all ASM and  VDE channels.

When selecting candidates for messages longer than one (1) slot, a candidate slot should be the first slot in a consecutive block of slots that conform to the selection criteria stated above.

If the station cannot find sufficient number of candidate slots, the station should not transmit and should re-schedule the transmission.

The candidate slot selection process also has to consider time periods reserved for the reception of the bulletin board.

The purpose of maintaining a minimum of eight candidate slots within the same probability of being used for transmission is to provide high probability of access to the link.

Note that the AIS and VDE channels need only to be considered when ASM and the other system share the same antenna. Or when there is not [sufficient isolation] to support independent operations such that the AIS station will still meet its receiver performance requirements.

### 3.3.2	Modes of operation

There should be two modes of operation, autonomous and assigned. The default mode should be autonomous.

#### 3.3.2.1	Autonomous

A station operating autonomously should determine its own schedule for transmission. The station should automatically resolve scheduling conflicts with other stations.

#### 3.3.2.2	Assigned

A station operating in the assigned mode takes into account the transmission schedule of the assigning message when determining when it should transmit.

### 3.3.3	Channel access schemes

The access schemes, as defined below, should coexist and operate simultaneously on the TDMA channel. The access scheme FATDMA is as defined in Recommendation ITU-R M.1371.

3.3.3.1	Multiple incremental time division multiple access (MITDMA)

The MITDMA access schemes allows a station to pre-announce transmission slots that the station will use in the future.  A single MITDMA transmission may be used to schedule up to three future transmissions with each transmission occupying up to 3 slots.

3.3.3.1.1 Multiple incremental time division multiple access algorithm

The first transmission within a MITDMA chain will be a single slot transmission using RATDMA access. Prior to the first transmission the station will randomly select up to three additional transmission slots.  The offset from the first transmission to the first future transmission is calculated and provided in the MITDMA communication state.  If a second future transmission is desired, then the offset from the first future transmission to the second future transmission is provided in the MITDMA communication states.  If a third future allocation is desired, then the offset from the second future transmission to the third future transmission is provided in the MITDMA communication state.

Receiving stations should mark these slot allocations as unavailable.

MITDMA may chain up to 15 transmissions together in a single frame.

3.3.3.2 Random access time division multiple access (RATDMA)

RATDMA is used when a station needs to allocate a slot, which has not been pre-announced. This is generally done for the first transmission slot during MITDMA chain, or for messages of a non-repeatable character.

3.3.3.2.1 Random access time division multiple access algorithm

The RATDMA access scheme should use a probability persistent (p-persistent) algorithm as described in this paragraph (see Table A2-4).

When a candidate slot is selected, the station randomly select a probability value (LME.RTP1) between 0 and 100. This value should be compared with the current probability for transmission (LME.RTP2). If LME.RTP1 is equal to, or less than LME.RTP2, transmission should occur in the candidate slot. If not, LME.RTP2 should be incremented with a probability increment (LME.RTPI) and the station should wait for the next candidate slot in the frame.

The SI for RATDMA should be 300 time slots, which is equivalent to 8 s. The candidate slot set should be chosen within the SI, so that the transmission occurs within 8 s.

Each time that a candidate slot is entered, the p-persistent algorithm is applied. If the algorithm determines that a transmission shall be inhibited, then the parameter LME.RTCSC is decremented by one and LME.RTA is incremented by one.

LME.RTCSC can also be decremented as a result of another station allocating a slot in the candidate set. If LME.RTCSC + LME.RTA < 8 then the candidate set shall be complemented with a new slot within the range of the current slot and LME.RTES following the slot selection criteria.

3.3.3.2.2 Random access time division multiple access parameters

The following parameter are used to control the RATDMA scheduling:

TABLE A2-4

RATDMA Paramenters

| Symbol | Name | Description | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| RTCSC | Candidate slot counter | The number of slots currently available in the candidate set. NOTE 1 – The initial value is always 8 or more (see § 3.3.1.2). However, during the cycle of the p-persistent algorithm the value may be reduced below 8 | 1 | 300 |
| RTES | End Slot | Defined as the slot number of the last slot in the initial SI, which is 150 slots ahead | 0 | 2249 |
| RTPS | Start probability | Each time a new message is due for transmission, LME.RTP2 should be set equal to LME.RTPS. LME.RTPS shall be equal to 100/LME.RTCSC. NOTE 2 – LME.RTCSC is set to 8 or more initially. Therefore LME.RTPS has a maximum value of −25 (100/8) | 0 | 25 |
| RTP1 | Derived probability | Calculated probability for transmission in the next candidate slot. It should be less than or equal to LME.RTP2 for transmission to occur, and it should be randomly selected for each transmission attempt | 0 | 100 |
| RTP2 | Current probability | The current probability that a transmission will occur in the next candidate slot | RTPS | 100 |
| RTA | Number of attempts | Initial value set to 0. This value is incremented by one each time the p-persistent algorithm determines that a transmission shall not occur | 0 | 149 |
| RTPI | Probability Increment | Each time the algorithm determines that transmission should not occur, LME.RTP2 should be incremented with LME.RTPI. LME.RTPI shall be equal to(100 − LME.RTP2)/LME.RTCSC | 1 | 25 |

#### 3.3.3.3	Network access and entry of a new data stream

At power on, a station should monitor the TDMA channels for one (1) min to determine channel activity, other participating member IDs, current slot assignments and reported positions of other users, and possible existence of base stations, as shown in Figure A2-1. During this time, a dynamic directory of all members operating in the system should be established. A frame map should be constructed, which reflects TDMA channel activity. After one (1) minute has elapsed, the station may be available to transmit ASM messages according to its own schedule.

figure A2-1

Network Access for  MITDMA and RATDMA

#### 3.3.3.4 RATDMA channel access

When the ASM station needs to transmit a single ASM message which is not repeated periodicly, it should use RATDMA access.

#### 3.3.3.5 MITDMA channel access

When the ASM station needs to transmit a block of ASM messages, or if it needs to transmit ASM message perodically, it should use MITDMA access.

### 3.3.4	Message structure

The messages should have the following structure shown in Figure A2-2 inside the data portion of a data packet.

Figure A2-2

Message Structure

Message Structure

Training sequence

MSG

ID

Link CFG

ID

Data

CRC

Training sequence

MSG

ID

Link CFG

ID

Data

CRC

Each message is described using a table with parameter fields listed from top to bottom. Each parameter field is defined with the most significant bit first.

Parameter fields containing sub-fields (e.g. communication state) are defined in separate tables with sub-fields listed top to bottom, with the most significant bit first within each sub-field.

#### 3.3.4.1	Message identification

The message ID should be 4 bits long and should have a range of 0 – 16. The message ID should identify the message type.

3.3.4.2.	User identification

The user ID should be a unique identifier and is 30 bits long. All ASM messages will contain the user identifier to identify the source of the transmission.

#### 3.3.4.3	Incremental time division multiple access message structure

The MITDMA message structure is shown in Figure A2-3.

figure A2-3

ATDMA Message Structure

Training sequence

MSG

ID

Link CFG

ID

Data

CRC

User

ID

Communication State

Training sequence

MSG

ID

Link CFG

ID

Data

CRC

User

ID

Communication State

##### 3.3.4.3.1 Multiple 	Incremental time division multiple access communication state

The communication state provides the

information used by the slot allocation algorithm in the   MITDMA concept.

The MITDMA communication state is structured as shown in Table A2-5.

TABLE A2-5

MITDMA communication state parameters

| Parameter | Number of bits | Description | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| Transmit block counter | 4 | A decrementing counter used to indicate how many transmissions are left to transmit within the chain A value of 1 indicates this is the last transmission within the chain A value of 0 indicates a recurring transmission. | 0 | 15 |
| Block Identifier | 4 | This identifier uniquely identifies the block of data within the transmit chain. This identifier also maps to the acknowledgment for addressed messages. | 0 | 15 |
| Slot Increment 1 | 8 | Offset to the next slot to be used, referenced to the current transmission start slot. A value of 0 indicates no additional slot allocations | 20 | 255 |
| Number of Slots 1 | 2 | Indicates the number of consecutive slots, which are allocated, starting at the slot increment A value of 0 indicates the 8 bits from Slot Increment 1 become the MSB for the Slot Increment 2 | 0 | 3 |
| Slot Increment 2 | 8 | Offset to the next slot to be used, referenced to the slot specified by slot increment 1 (or current transmission slot if the Number of Slots 1 is set to 0) A value of 0 indicates no additional slot allocations | 20 | 255 45001 |
| Number of Slots 2 | 2 | Indicates the number of consecutive slots, which are allocated, starting at the slot increment | 1 | 3 |
| Slot Increment 3 | 8 | Offset to the next slot to be used, referenced to the slot specified by Slot Increment 2 | 20 | 255 |
| Number of Slots 3 | 2 | Indicates the number of consecutive slots, which are allocated, starting at the slot increment | 1 | 3 |
| Total bits | 37 |  |  |  |
| Note 1: When combining Slot Increment 1 and Slot Increment 2 as a 16 bit field. This value should not exceed 2 frames. |  |  |  |  |

# 4	Network layer

The network layer should be used for:

–	Establishing and maintaining channel connections

–	Management and priority assignments of messages

–	Distribution of transmission packets between channels

–	Data link congestion resolution.

# 4.1	Multi-channel operations

Two frequencies have been designated in RR Appendix 18 for ASM transmissions.  These frequencies are:

–	ASM1 (Channel 2027, 161.950 MHz)

–	ASM2 (Channel 2028, 162.000 MHz)

Channel access is performed independently on each of the two channels.  Generally, ASM transmission should alternate between the two channels when available.

Terrestrial transmissions of acknowledgements to addressed messages should be done on the channel as the initial message was received.

Chained transmissions using MITDMA shall all be done on the same channel.

# 4.2	Management of priority assignment for messages

ASM messages support message priority.  The priority of the message is determined by the PI interface.  The messages are serviced in order of priority.  Messages with the same priority are dealt with in a FIFO order.

# 4.3	Data link congestion resolution

As the data link becomes loaded, the availability of transmission slots will reduce. When the data link is loaded to such a level as reception of ASM message`s is jeopardized, measures should be taken to reduce the loading.

ASM channel loading shall be measured independently per channel over a window of the past 2250 slots (1 Minute).

The amount of ASM transmissions on a specific channel shall be adopted to the channel loading on that channel.

The maximum number of slots allocated by one station on one channel shall not exceed 50 slots over a period of one minute. (2.2% duty cycle)

# 4.3.1	Mandatory quiet times

After the completion of a singular Non-MITDMA ASM channel transmission or a complete MITDMA transmission block chain, the ASM station shall wait for a specific time before additional transmission can be scheduled. This time is referred to as Quiet Time. The Selection Interval for finding candidate transmission slots starts after the Quiet Time.

For a singular transmission, Quiet Time shall per default be one second per timeslot.

For an MITDMA linked transmission chain, the Quiet Time is a function of the number of transmission slots within that chain. The Quiet Time shall be increased by one second per time slot used in the transmission chain.

The Quiet Time shall be increased with a multiplier, depending on channel load (Table A2-6)

Table A2-6

Quiet Time Multiplier

| Channel load | <10% | 10%-30% | 30%< |
| --- | --- | --- | --- |
| Multiplier | 1 | 2 | 3 |
| Quiet Time [seconds] = Transmission slots * Multiplier |  |  |  |

Quiete Time [seconds] = Transmission slots  *  Multiplier

# 5	Transport layer

The transport layer is responsible for:

–	converting data into transmission packets of correct size;

–	sequencing of data packets;

–	interfacing protocol to upper layers.

## 5.1	Definition of transmission packet

A transmission packet is an internal representation of some information which can ultimately be communicated to external systems. The transmission packet is dimensioned so that it conforms to the rules of data transfer. Transmission packets are fixed block sizes on slot boundaries with a maximum of 3 consecutive slots.  When data does complety fill the block, then padding bits with the value of 0 should be added to completely required block size.

## 5.2	ASM identifier

Addressed and broadcast binary messages should contain a 16-bit application identifier (Table A2-7)

TABLE A2-7

ASM identifier parameters

| Bit | Description |
| --- | --- |
| 15-6 | Designated area code (DAC). This code is based on the maritime identification digits (MID). Exceptions are 0 (test) and 1 (international). Although the length is 10 bits, the DAC codes equal to or above 1 000 are reserved for future use |
| 5-0 | Function identifier. The meaning should be determined by the authority which is responsible for the area given in the designated area code |

Whereas the application identifier allows for regional applications, the application identifier should have the following special values for international compatibility.

## 5.3	Transmission packets

### 5.3.1	Addressed Messages

Addressed messages are point to point communications between VDES stations. Addressed messages may require an acknowledgment. When an acknowledgement is required and not received, the VDES stations may retransmit the message.

### 5.3.2	Broadcast messages

A broadcast message lacks a destination identifier ID. Therefore, receiving stations should not acknowledge a broadcast message up to 3 times.

### 5.3.3	Conversion to presentation interface messages

Each received transmission packet should be converted to a corresponding presentation interface message and presented in the order they were received regardless of message category. Applications utilizing the presentation interface should be responsible for their own sequencing numbering scheme, as required. For a mobile station, addressed messages should not be output to the presentation interface, if Destination ID (unique identifier) is different to the ID of own station (own unique identifier).

### 5.3.4	Conversion of data into transmission packets

The transport layer should convert data, received from the presentation interface into transmission packets. If the data exceeds the maximum limit, then a negative acknowledgement should be returned on the PI

## 5.4	MITDMA access

When the length of the data requires more than 3 consecutive slots, then the data should be divided up into sub-groups of 3 slot packets and MITDMA should be used to chain the transmissions together.  A total of 15 MITDMA transmissions may be chained together.  If the data provided by the PI exceeds this limit, a negative acknowledgment should be provided on the PI.

If data transmissions are repetitive in nature, and have a transmit interval less than 2 frames (4 500 slots), then MITDMA should be used to maintain the link.

If multiple messages are queued for transmission, then MITDMA should be used to allocate slots for the additional messages.

When using MITDMA for addressed messages, the MITDMA will provide the return slot for the message acknowledgment as specified in Slot Increment 3 during block identifier 2, 1 or 0.

## 5.4.1	MITDMA access example

The first transmission (TX 1) of a MITDMA chain is always a single slot transmission.

Determine the candidate slots for the TX 1. Apply the RATDMA algorithm until transmit criteria is meet.

Before transmitting at TX 1, determine the candidate slots for up to three additional tranmissions.  Randomly select the transmit slots from the candidate slot lists.  Calculate the offsets for these future tranmissions.  This information is provided in the MITDMA communication state. Slot Increment 1 reserves Tx 2, Slot Increment 2 reserves Tx 3, and Slot Increment 3 reserves Tx 4.

Before transmitting at Tx 2, determine the candidate slots for the next transmission, e.g. Tx 5.  Randomly select a transmit slot from the canadiate slot list.  This information is provided in the MITDMA communication state.  Slot Increment 1 reserves Tx 3, Slot Increment 2 reserves Tx 4, and Slot Increment 3 reserves Tx 5.

If this is a broadcast message, then starting at Tx n-2, the unused Slot Inrements are set to 0.  If this is an addressed message, then the following process happens.

At Tx n-2, determine the candidate slots for the acknowledgment message. Randomly select the acknowledgment slot from the candidate slot list.  Calculate the offset  for the ACK Slot. This information is provided in the MITDMA communication state. Slot Increment 1 reserves the Tx n-1, Slot Increment 2 reserves the Tx n, and Slot Increment 3 reserves the ACK Slot.

At Tx n-1 a new offset is calculated for the ACK Slot.  This information is provided in the MITDMA communication state.  Slot Increment 1 reserves the Tx n, Slot Increment 2 reserves the ACK Slot, and Slot Increment 3 is set to 0.

At Tx n a new offset is calculated for the ACK Slot. This information is provided in the MITDMA communication state.  Slot Increment 1 reserves the ACK Slot, Slot Increment 2 and 3 are set to 0.

At the ACK Slot, the receiving station transmits the acknowledgment message.

Figure A2-4

[figure title?]

# 6	Packet structure

The ASM transmission packets are used to transport data from one ASM station to another.  There are multiple types of packet definitions which use different address modes and channel access schemes.  The packet structures are defined by the message identifier.

# 6.1 	General packet structure

The generic data packet is defined in Table A2-8.

TABLE A2-8

General packet structure

| Parameter | Number of bits | Descriptions |
| --- | --- | --- |
| Ramp up | 8 bits | 417 us |
| Training sequence | 54 bits | Necessary for synchronization |
| Link configuration id | 32 bits | Six information bits decoded from (32,6) biorthogonal code ASM channel configurations as defined in Link Configuration Id, see Table A1-6 Note that the Link Configuration Id will identifiy how many slots make up the message. |
| Message | 1 slot: 352 2 slot: 864 3 slot: 1376 SAT : 1236 | The symbol count and the information bits varies according to coding rate as defined by the link configuration id field |
| CRC | 32 bits | The CRC only includes the data field |
| FEC termination bits | 12 bits | Set to zero when not used |
| Ramp Down | 8 bits | 417 us |
| Guard Time | 14 bits TER 154 bits SAT | Distance delay TER 729 us Distance delay SAT 8.02 ms |
| Total | 1 slot: 512 2 slot: 1024 3 slot: 1536 |  |

# 6.2	Message summary

The defined message types are summarized Table A2-9.

TABLE A2-9

Message summary

| Message ID | Name | Description | Access scheme | Communication State |
| --- | --- | --- | --- | --- |
| 1 | Scheduled Broadcast Message | Broadcast data using communication state | FATDMA RATDMA MITDMA | MITDMA |
| 2 | Broadcast Message | Broadcast data with no communication state | FATDMA RATDMA | none |
| 3 | Scheduled Individual Addressed Message | Individual addressed data with communication state. Requires acknowledgement | FATDMA RATDMA MITDMA | MITDMA |
| 4 | Individual Addressed Message | Individual addressed data with no communication state. Requires acknowledgement | FATDMA RATDMA | none |
| 5 | Acknowledgment Message | This message is used to provide and acknowledgment for one or more addressed messages | FATMDA RATDMA MITDMA | none |
| 6 | Geographical Multicast Message | Addressed to a group of stations defined by their geographical location with no communication state. No acknowledgment required. | FATDMA RATDMA | none |
| 7 | Group Multicast Message | Address to a group of stations defined by the group address identifier | FATDMA RATDMA | none |

# 6.3 	Message 1: Scheduled broadcast message

This ASM message is used to broadcast data to all targets, and utilizes MITDMA communication state.  Multiple messages, or periodically broadcasted messages may be chained together using the MITDMA communication state.  The first transmission in the chain will use RATDMA to access the link, and all additional transmission will use slots allocated by the MITDMA communication state.  Scheduled broadcast message is defined in Table A2-10.

TABLE A2-10

Scheduled broadcast message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 4 | 1 - Broadcast message with MITDMA communication state |
| Retransmit flag | 1 | Indicates that this is a retransmission of data |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 - 3; 0 = default; 3 = do not repeat any more |
| Communication State | 37 | MITDMA communication state as described in section 3.3.3.1 |
| Source ID | 32 | The Unique Identifier of the transmitting station as described in Annex 1 section 3.2 |
| ASM identifier | 16 | Application identifier and described in section 5.2 |
| Data Count | 11 | 1 – Max data count |
| Binary Data | 1 slot – 245 2 slot – 757 3 slot – 1269 SAT – 1129 | Application data as specified by the ASM Identifier. The available length of the binary data is specified by the LinkConfigId. Unused payload data is zero-filled |

# 6.4 	Message 2: Broadcast message

This ASM message is used to broadcast data to all targets, and does not contain a communication state.  These broadcast messages are used for non-periodic transmission of data, and access the link using RATDMA.  Broadcast message is defined in Table A2-11.

TABLE A2-11

Broadcast Message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 4 | 2 - Broadcast message with no communication state |
| Retransmit flag | 1 | Indicates that this is a retransmission of data |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 - 3; 0 = default; 3 = do not repeat any more |
| Source ID | 32 | The Unique Identifier of the transmitting station as described in Annex 1 section 3.2 |
| ASM identifier | 16 | Application identifier and described in section 5.2 |
| Data Count | 11 | 1 – Max data count |
| Binary Data | 1 slot – 286 2 slot – 798 3 slot – 1310 SAT – 1170 | Application data as specified by the ASM Identifier. The available length of the binary data is specified by the LinkConfigId. |

# 6.5 	Message 3: Scheduled addressed message

This ASM message is used to send data to an individual target, and utilizes MITDMA communication state.  Multiple transmission of messages, or periodically transmissions of messages may be chained together using the MITDMA communication state.  The first transmission in the chain will use RATDMA access the link, and all additional transmission will use slots allocated by the MITDMA communication state.

These transmissions require the destination station to return a message acknowledgment (Message 5). This addressed message supplies the return slot for the message acknowledgment.  Scheduled addressed message is defined in Table A2-12.

TABLE A2-12

Scheduled addressed message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 4 | 3 – Individually addressed message with MITDMA communication state |
| Retransmit flag | 1 | Indicates that this is a retransmission of data |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 - 3; 0 = default; 3 = do not repeat any more |
| Communication State | 37 | MITDMA communication state as described in section 3.3.3.1 |
| Source ID | 32 | The Unique Identifier of the transmitting station as described in Annex 1 section 3.2 |
| Destination ID | 32 | The Unique Identifier of the transmitting station as described in Annex 1 section 3.2 |
| ASM identifier | 16 | Application identifier and described in section 5.2 |
| Data Count | 11 | 1 – Max data count |
| Binary Data | 1 slot – 213 2 slot – 725 3 slot – 1237 SAT – 1097 | Application data as specified by the ASM Identifier. The available length of the binary data is specified by the LinkConfigId. |

# 6.6 	Message 4: Addressed message

This ASM message is used to send data to an individual target, and does not contain a communication state.  This message is used for non-periodic transmission of data, and access the link using RATDMA.

These transmissions require the destination station to return a message acknowledgment (Message 5). The destination station will used RATDMA to send the message acknowledgment.  Addressed message is defined in Table A2-13.

TABLE A2-13

Addressed message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 4 | 4 – Individually addressed message with no communication state |
| Retransmit flag | 1 | Indicates that this is a retransmission of data |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 - 3; 0 = default; 3 = do not repeat any more |
| Source ID | 32 | The Unique Identifier of the transmitting station as described in Annex 1 section 3.2 |
| Destination ID | 32 | The Unique Identifier of the transmitting station as described in Annex 1 section 3.2 |
| ASM identifier | 16 | Application identifier and described in section 5.2 |
| Data Count | 11 | 1 – Max data count |
| Binary Data | 1 slot – 254 2 slot – 766 3 slot – 1278 SAT – 1138 | Application data as specified by the ASM Identifier. The available length of the binary data is specified by the LinkConfigId. |

# 6.7 	Message 5: Acknowledgment message

This ASM message is used to return message acknowledgments to one or more addressed messages. Note that this message should always use Link Configuration Id of 8 (1/2 coding rate).  Acknowledgement message is defined in Table A2-14.

TABLE A2-14

Acknowledgment message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 4 | 5 – Multiple acknowledgment message with no communication state |
| Retransmit flag | 1 | Indicates that this is a retransmission of data |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 - 3; 0 = default; 3 = do not repeat any more |
| Source ID | 32 | The Unique Identifier of the transmitting station as described in Annex 1 section 3.2 |
| Destination ID | 32 | The Unique Identifier of the transmitting station as described in Annex 1 section 3.2 |
| ACK/NACK Mask | 16 | Specifies which MITDMA block ids failed. Bit map field with the LSB representing Block id 1, the MSB representing Block Id 16. “1” indicates a packet failed “0” indicates the packet was received ok |
| Coding rate adaption request | 2 | 0: Maintain MCS 1: Increment MCS (higher rate) 2: Decrease MCS 3: Reserved for future uses |
| Signal Quality Indicator | 8 | Received C/N0 in dBHz |
| Zero padding | As required | Padding bits are added as require to complete the block size. |

# 6.8 	Message 6: Geographical multicast message

This ASM message is used to broadcast data to a group of stations as defined by the specified geographical area.  The broadcast message does not contain a communication state.  These broadcast messages are used for non-periodic transmission of data, and access the link using RATDMA.  Geographical multicast message is defined in Table A2-15.

TABLE A2-15

Geographical multicast message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 4 | 6 – Geographical addressed message with no communication state |
| Retransmit flag | 1 | Indicates that this is a retransmission of data |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 - 3; 0 = default; 3 = do not repeat any more |
| Source ID | 32 | The Unique Identifier of the transmitting station as described in Annex 1 section 3.2 |
| Longitude 1 | 18 | Longitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (180, East = positive, West = negative) |
| Latitude 1 | 17 | Latitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (90, North = positive, South = negative) |
| Longitude 2 | 18 | Longitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (180, East = positive, West = negative) |
| Latitude 2 | 17 | Latitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (90, North = positive, South = negative) |
| ASM identifier | 16 | Application identifier and described in section 5.2 |
| Data Count | 11 | 1 – Max data count |
| Binary Data | 1 slot – 216 2 slot – 728 3 slot – 1240 SAT – 1100 | Application data as specified by the ASM Identifier. The available length of the binary data is specified by the LinkConfigId. |

# 6.9 	Message 7: Group multicast message

This ASM message is used to broadcast data to a group of stations as defined by the specified geographical area.  The broadcast message does not contain a communication state.  These broadcast messages are used for non-periodic transmission of data, and access the link using RATDMA.  Group multicast message is defined in Table A2-16.

TABLE A2-16

Group multicast message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 4 | 7 – Group addressed message with no communication state |
| Retransmit flag | 1 | Indicates that this is a retransmission of data |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 - 3; 0 = default; 3 = do not repeat any more |
| Source ID | 32 | The Unique Identifier of the transmitting station as described in Annex 1 section 3.2 |
| Group Destination ID | 32 | The Unique Identifier of a group of receiving |
| ASM identifier | 16 | Application identifier and described in section 5.2 |
| Data Count | 11 | 1 – Max data count |
| Binary Data | 1 slot – 254 2 slot – 766 3 slot – 1278 SAT – 1138 | Application data as specified by the ASM Identifier. The available length of the binary data is specified by the LinkConfigId. |

Annex 3

Technical characteristics of VDE-terrestrial 
in the maritime mobile band

# 1	Introduction

This section describes those elements of the VDE-TER that are unique to VDE-TER operation.  For those elements that are common, the cross reference into Annex 1 is provided. It contains a description of the different protocols according to the OSI layer model and recommends implementation details for each layer.

Data transmission is made in the VHF maritime mobile band. Data transmissions are made within the spectrum allocated for the VDE1-A and VDE1- B. The spectrum may be used as 25 kHz, 50 kHz or 100 kHz channels.

The system should use TDMA techniques in a synchronized manner.

# 2	OSI layer

Refer to Annex 1.

# 3	Physical layer

## 3.1	Range

The communication range of terrestrial VDE is typically 20−50 NM.

## 3.2	Transmitter Parameter settings

Refer to Annex 1 for transmitter parameter settings for mobile stations.

## 3.3	Antenna

Terrestrial VDE may share the same antenna(s) with the other subsystems AIS, ASM, VDE-SAT.

Refer to Annex 1.

## 3.4	Modulation

### 3.4.1	Waveforms

The waveforms are defined in Annex 1, Modulation and Coding Schemes.

### 3.4.2	Bit Mapping

For bit mappings, see Annex 1.

## 3.5	Sensitivity and Interference

VDE uses adaptive modulation and coding to maximise spectral efficiency and throughput. Sensitivity and interference levels for the supported modulation methods are given in Table A3-1.

TABLE A3-1

Sensitivity and Carrier to Interference Ratios

| Modulation Coding Scheme | 25 kHz |  | 50 kHz |  | 100 kHz |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Sensitivity (dBm) | CIR (dB) | Sensitivity (dBm) | CIR (dB) | Sensitivity (dBm) | CIR (dB) |
| MCS-1* | −110 | 8 | −107 | 8 | −104 | 8 |
| MCS-3* | −104 | 14 | −101 | 14 | −98 | 14 |
| MCS-5* | −102 | 16 | −99 | 16 | −96 | 16 |
| * Modulation Coding Schemes, see Table A1-6. This table assumes a BER of 1e-6. |  |  |  |  |  |  |

TBD: add missing receiver parameters (look at Table A2-1).

## 3.6	Symbol timing accuracy

See Annex 1.

## 3.7	Transmitter timing jitter

See Annex 1.

## 3.8	Slot transmission accuracy at the output

See Annex 1.

## 3.9	Frame structure

See Annex 1.

# 4	Link layer

## 4.1	Access Schemes

The VDE terrestrial system should support the following TDMA access schemes:

–	FATDMA;

- ??	;

–	ATDMA.

## 4.2	Data encapsulation

Refer to Annex 1.

## 4.3	Cyclic redundancy check

Refer to Annex 1.

## 4.4	Automatic repeat request (ARQ)

Refer to Annex 1.

## 4.5	Acknowledgement (ACK)

Refer to Annex 1.

## 4.6	End delivery notification (EDN)

Refer to Annex 1.

## 4.7	End delivery failure (EDF)

Refer to Annex 1.

## 4.8	Frame hierarchy

Refer to Annex 1.

## 4.9	Channel resource and media access control for VDE terrestrial

VDE spectrum (and logical channel) resources are allocated by the reception of shore (or satellite-based) media access control messaging (for example, shore stations perform this operation using the bulletin board service). In the event of vessel reception of, both satellite and shore-based stations, the shore-based resource allocations should take precedence.

Duplexing modes are a function of the resource allocation above, as well as the operational mode.

### 4.9.1	Simplex modes

–	Ship-to-ship

Same channel resource is used for both directions of communications.

### 4.9.2	Duplex modes

–	Shore-to-ship

–	Ship-to-shore

Upper and lower spectrum are used for ship to shore or shore to ship respectively, only one side transmits at one time.

Refer to Annex 1 for details of frequency allocations.

## 4.10	Physical and logical channels

VDES uses several channels to carry data. These channels are separated into Physical and Logical channels. A bulletin board may prescribe this channel usage.

Lacking bulletin board information, ship borne stations should employ a default channel configuration of 100 kHz channels on the terrestrial VDES (channel 2024, 2084, 2025 and 2085 combined) operating in a simplex ad-hoc access scheme. The simplex ad-hoc access scheme for ship-to-ship communications should be ATDMA

## 4.11	Physical channels

The physical channels (PC) are determined by the centre frequency and bandwidth.

## 4.12	Logical channels

The logical channels (LC) are divided into signalling and traffic channels. These are described below. Logical channel definitions can be defined based on the physical channel and message time information (frame hierarchy, start time, etc.).

Signalling channels:

–	Terrestrial Bulletin Board (TBB), see § 4.12.1

–	Announcement, see § 4.12.2

–	Random access, see § 4.12.5.

All signalling channels use the most robust modulation and coding scheme.

Traffic channels:

–	Multicast, see § 4.12.3

–	Unicast, see § 4.12.4

–	Random access, see § 4.12.5.

Traffic channels may use a combination of robust and higher bitrate modulation and coding schemes.

### 4.12.1	Terrestrial bulletin board (TBB) signalling channel

Each VDE shore station should employ a fixed logical channel for the TBB. All TBB logical channels will be based on one of a number of predefined structures of the frame hierarchy 50 kHz shore to ship physical channel (2024 and 2084 combined). These are defined to occupy only a portion of the frame (60 seconds, 2 250 slots) to permit possible spectrum and temporal sharing with satellites, see Annex 6.

The TBB defines the network configuration parameters such as signalling channels (control channels) and data channel(s), protocol versions and future network configuration. The TBB takes precedence in the allocation of spectrum (logical channel) resources. This may be co-ordinated with the satellite bulletin board signalling channel to facilitate sharing of mutual spectrum resources.

The logical channels are normally repeated based on the VDES frame hierarchy.

The VDE terrestrial channel usage for the service area of VDE shore station is defined by the TBB, see Annex 1.

The TBB information includes the area of applicability. The TBB does not change often and should be transmitted in regular intervals.

### 4.12.2	Announcement signalling channel (ASC)

This channel(s) will normally carry announcements, MAC information, VDE forward and return resource allocation, CQIs, ARQs, and ACKs. Announcements also include the co-ordination of uni-cast and multi-cast (broadcast) datagrams.

The MAC information includes changes to network version, congestion control (randomization interval (hold-off) and minimum priority level). Some of these parameters will be reflected in the TBB on periodic basis.

The ASC logical channels will be assigned in the TBB and consist of a number of defined structures of the frame hierarchy 50 kHz shore to ship physical channel (2024 and 2084 combined). These are defined to occupy only a portion of the frame (60 seconds, 2 250 slots) to permit possible spectrum and temporal sharing with satellites, see Annex 6.

The ASC defines the physical channel usage (logical channel, i.e. frequency and slot) to an individual ship following a resource request. The VDE shore station uses CQI information from the ship terminal to select the highest throughput format with adequate link margin.

### 4.12.3	Multicast data channel (MDC)

This traffic channel(s) is utilized to send messages to be received by a large number of ships. By default multicast messages are addressed to all stations (i.e. broadcast).

### 4.12.4	Unicast data channel (UDC)

This traffic channel is allocated a specific ship for the duration of a unicast datagram.

This channel is set up after a ship responds to an announcement, and the response includes received channel quality information (CQI) allowing the shore station to maximise throughput.

### 4.12.5	Random access channel (RAC)

This channel has the characteristics of a slotted Aloha channel, uses a random access scheme and will be selected from a predefined list of logical channels.

#### 4.12.5.1	For ship-to-shore, and shore-to-ship communications

A ship station uses this channel to access the network or send a short message.

#### 4.12.5.2	For ship-to-ship when ships are within control area of a VDE shore station

A ship station uses this channel to communicate directly with other ships. This logical channel is allocated by shore station via TBB or ASC.

#### 4.12.5.3	For ship-to-ship when outside the control area of a shore VDE station

A ship station uses these channels to communicate with other ship stations directly via short message, and will also use these random access channels to co-ordinate communication with other ships for larger messages. These logical channels will be based on a number of predefined structures of the frame hierarchy of the ship-to-ship physical channels (2024, 2084, 2025 and 2085 combined). Ship-to-ship random access channels should have fixed physical channel assignments and use the most robust modulation and coding scheme. These logic channels are distinct from the TBB logical channels.

# 5	Network layer

Refer to Annex 1, assuming “Control Station” refers to a shore station.

# 6	Transport layer

Refer to Annex 1.

Annex 4

Technical characteristics of VDE-SAT service in the VHF maritime mobile band

# 1	Introduction

This section describes those elements of the VDE-SAT that are unique to VDE-SAT operation.  For those elements that are common, the cross reference into Annex 1 is provided. In this context, the following types of functionality are envisaged:

–	VDE-SAT allows two-way communications and transmit-only:

–	Shore initiated polling of information from ships (satellite-ship-satellite)

–	Ship initiated enquiry for information from shore (ship-satellite-ship)

–	Ship initiated data transfer to shore (ship-satellite)

–	Collection of information from transmit-only VDES terminals (ship-satellite). This could be either event driven or periodic. The slot and frequency band for this service should be assigned by the bulletin board and announcement signalling channels.

- Downlink multicast multi-packet data transfer (satellite-ship)

- Shore originated unicast multi-packet data transfer via satellite (satellite ship)

In this annex low earth orbit (LEO) satellites with 600 km altitude are considered to present typical examples of VDE satellite solutions. It should be noted that other orbital selections are also possible according to the overall system design consideration.

The focus of this annex is to describe the physical layer of the OSI model as defined in Annex 1. The overall description of the link, network and the transport layers is provided in Annex 1.

# 2	VDE-SAT physical layer

## 2.1	VDE-SAT uplink key parameters

This section outlines assumptions regarding the VDE-SAT system parameters that are used as representative examples in this annex.

### 2.1.1	Satellite to surface distance range

The orbit height determines the satellite range variations. For example, for a 600 km LEO the maximum range is 2 830 km. For timing purposes a maximum range of 3 000 km will be used.

The minimum range is equal to the orbit height. For a LEO satellite at 600 km altitude the minimum range will be 600 km. This value is used to determine the minimum propagation delay time. Considering these exemplary values for the minimum and maximum ranges, the path delay will vary from 2 ms to 10 ms, a variation of 8 ms as shown in Figure A4-1 and Figure A4-2.

For the VDE-SAT downlink, in addition to the relative delays between signal receptions at a vessel from different satellites, there could be absolute delay due to other sources such as signal processing delay. The satellite service provider should pre-compensate for absolute delay.

Figure A4-1

VDE-SAT downlink timing

Figure A4-2

VDE-SAT Uplink timing

### 2.1.2	Satellite transmission carrier frequency error

The satellite transmission carrier frequency error is the sum of the satellite transmission frequency error and Doppler, and the frequency uncertainty at the receiver. The transmit frequency error at the satellite shall be less than 1 ppm, i.e. ±160 Hz.

A LEO satellite will move at a speed of about 8 km/s and this will cause a maximum Doppler of ±4 kHz at VHF.

### 2.1.3	Ship station transmitter requirements

For ship station transmitter requirements, see Annex 1.

### 2.1.4	Ship station antenna gain

For ship station antenna gain, see Annex 1.

### 2.1.5	Ship station noise plus interference level

For noise plus interference level of ship station, see Annex 1.

### 2.2	Link budget analysis

The link C/N0 is determined by the transmitted e.i.r.p., path losses, propagation losses, receiver sensitivity/figure of merit and local interference levels.

### 2.2.1	VDE-SAT downlink link budget

Examples of link budgets for the VDE-SAT downlink are provided in the following sections.

### 2.2.1.1	Satellite downlink e.i.r.p.

The e.i.r.p. can be derived from PFD mask given in Table A4-1.

TABLE A4-1

Proposed power spectral and PFD mask

θ°=earth-satellite elevation angle

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                        0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

Table A4-2 shows the theoretical maximum satellite e.i.r.p. as a function of elevation angles for this mask.

TABLE A4-2

Satellite maximum e.i.r.p. vs. elevation angle

| Ship Elevation angle | Powerflux density on ground | Satellite range | Maximum downlink satellite e.i.r.p. |
| --- | --- | --- | --- |
| (degrees) | (dBW/m2/4 kHz) | (km) | (dBW in 25 kHz) |
| 0 | −149.0 | 2 831 | −1.0 |
| 10 | −147.4 | 1 932 | −2.7 |
| 20 | −145.8 | 1 392 | −4.0 |
| 30 | −144.2 | 1 075 | −4.6 |
| 40 | −142.6 | 882 | −4.7 |
| 50 | −139.4 | 761 | −2.8 |
| 60 | −134.0 | 683 | 1.6 |
| 70 | −133.0 | 635 | 2.0 |
| 80 | −132.0 | 608 | 2.6 |
| 90 | −131.0 | 600 | 3.5 |

### 2.2.1.2	Satellite e.i.r.p. vs. elevation

Most of the satellite coverage area and visibility time will be at low elevation angles, and high elevation angle coverage may be sacrificed without significant system capacity loss.

The following two satellite antennas have been analysed and are acceptable.

1)	Yagi Antenna: For this antenna the link budget is optimised for 0 degrees ship elevation angle using a three element Yagi antenna with the satellite pointed at the horizon is given in Table A4-3. Assuming a peak antenna gain of 8 dBi, a transmit RF power of −12.4 dBW in 25 kHz will ensure compliance with the PFD limit. Satellite e.i.r.p. vs. ship elevation is shown in Table A4-3.

TABLE A4-3

Satellite e.i.r.p. vs. elevation using a Yagi antenna

| Ship elevation angle | Nadir offset angle | Boresight offset | Satellite antenna gain | Satellite e.i.r.p. in circular polarization | Satellite range | PFD | Table A4-1 PFD limit | PFD margin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| degrees | degrees | degrees | dBi | dBW | km | dBW/m2/4 kHz | dBW/m2/ 4 kHz | dB |
| 0 | 66.1 | 0 | 8 | −4.4 | 2 830 | −152.4 | −149.0 | 3.4 |
| 10 | 64.2 | 1.9 | 8 | −4.4 | 1 932 | −149.1 | −147.4 | 1.7 |
| 20 | 59.2 | 6.9 | 8 | −4.4 | 1 392 | −146.2 | −145.8 | 0.4 |
| 30 | 52.3 | 13.8 | 7.8 | −4.6 | 1 075 | −144.2 | −144.2 | 0.0 |
| 40 | 44.4 | 21.7 | 6.9 | −5.5 | 882 | −143.4 | −142.6 | 0.8 |
| 50 | 36 | 30.1 | 5.5 | −6.9 | 761 | −143.5 | −139.4 | 4.1 |
| 60 | 27.2 | 38.9 | 3.6 | −8.8 | 683 | −144.5 | −134.0 | 10.5 |
| 70 | 18.2 | 47.9 | 0.7 | −11.7 | 635 | −146.7 | −133.0 | 13.7 |
| 80 | 9.1 | 57 | −2.2 | −14.6 | 608 | −149.2 | −132.0 | 17.2 |
| 90 | 0 | 66.1 | −5.5 | −17.9 | 600 | −152.4 | −131.0 | 21.4 |

2)	Isoflux antenna: This antenna is designed to point at the nadir direction providing a symmetric radiation pattern around the pointing direction. Assuming a peak antenna gain of 2 dBi, a transmit RF power of −5 dBW in 25 kHz will ensure compliance with the PFD limit. Satellite e.i.r.p. vs. ship elevation is shown in Table A4-4.

TABLE A4-4

Satellite e.i.r.p vs. elevation using an isoflux antenna

| Ship elevation angle | Nadir offset angle | Boresight offset | Satellite antenna gain | Satellite e.i.r.p. in circular polarization | Satellite range | PFD | Table A4-1 PFD limit | PFD margin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| degrees | degrees | degrees | dBi | dBW | km | dBW/m2/4 kHz | dBW/m2/4 kHz | dB |
| 0 | 66.1 | 0 | 2 | −3.0 | 2 830 | −151.0 | −149.0 | 2.0 |
| 10 | 64.2 | 1.9 | 1.5 | −3.5 | 1 932 | −148.2 | −147.4 | 0.8 |
| 20 | 59.2 | 6.9 | 1 | −4.0 | 1 392 | −145.8 | −145.8 | 0.0 |
| 30 | 52.3 | 13.8 | −0.5 | −5.5 | 1 075 | −145.1 | −144.2 | 0.9 |
| 40 | 44.4 | 21.7 | −2 | −7.0 | 882 | −144.9 | −142.6 | 2.3 |
| 50 | 36 | 30.1 | −4 | −9.0 | 761 | −145.6 | −139.4 | 6.2 |
| 60 | 27.2 | 38.9 | −5 | −10.0 | 683 | −145.7 | −134.0 | 11.7 |
| 70 | 18.2 | 47.9 | −7 | −12.0 | 635 | −147.0 | −133.0 | 14.0 |
| 80 | 9.1 | 57 | −8 | −13.0 | 608 | −147.6 | −132.0 | 15.6 |
| 90 | 0 | 66.1 | −8.5 | −13.5 | 600 | −148.0 | −131.0 | 17.0 |

### 2.2.1.3	Link C/(N0+I0)

The nominal signal level and C/(N0+I0) vs. elevation for a 25 kHz channel are provided in Table A4-3 and Table A4-4 for Yagi and Isoflux on-board antennas. The assumed ship antenna gain is 3 dBi and the system noise temperature is 30.2 dBK as shown in Table A1-10 of Annex 1.

Because the downlink is PFD limited, increasing the channel bandwidth to 50 kHz or 100 kHz will increase the signal level and C/(N0+I0) by 3 and 6 dB respectively. Limiting the service area to ship elevation angles between 10 and 55 degrees also improves the link margin by 3 dB.

The Isoflux antenna improves the link budget at low elevation angles and provides a wider symmetrical coverage area, but requires a 5 times larger transmitter power on the satellite.

The link budget results with a satellite Yagi antenna is shown in Table A4-5. Isoflux antenna is shown in Table A4-6.

It should be noted that the analyses based on single satellite visibility.

TABLE A4-5

Link budget with satellite Yagi antenna (transmit RF power = −12.4 dBW/25 kHz)

| Ship elevation angle | Satellite EIRP in circular polarization | Satellite range | Path loss | Polarization loss | Ship antenna gain | Antenna signal level | C/N0 | Noise level in 25 kHz BW | C/(N0+I0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (degrees) | (dBW) | (km) | (dB) | (dB) | (dBi) | (dBm) | (dBHz) | (dBm) | (dBHz) |
| 0 | −4.4 | 2 830 | 145.6 | 3 | 3 | −120.0 | 48.4 | −116 | 40.0 |
| 10 | −4.4 | 1 932 | 142.2 | 3 | 3 | −116.7 | 51.7 | −116 | 43.3 |
| 20 | −4.4 | 1 392 | 139.4 | 3 | 2.5 | −114.3 | 54.1 | −116 | 45.7 |
| 30 | −4.6 | 1 075 | 137.2 | 3 | 1 | −113.8 | 54.6 | −116 | 46.2 |
| 40 | −5.5 | 882 | 135.4 | 3 | 0 | −114.0 | 54.4 | −116 | 46.0 |
| 50 | −6.9 | 761 | 134.2 | 3 | −1.5 | −115.6 | 52.8 | −116 | 44.4 |
| 60 | −8.8 | 683 | 133.2 | 3 | −3 | −118.0 | 50.4 | −116 | 41.9 |
| 70 | −11.7 | 635 | 132.6 | 3 | −4 | −121.3 | 47.1 | −116 | 38.7 |
| 80 | −14.6 | 608 | 132.2 | 3 | −10 | −129.8 | 38.6 | −116 | 30.2 |
| 90 | −17.9 | 600 | 132.1 | 3 | −20 | −143.0 | 25.4 | −116 | 17.0 |

TABLE A4-6

Link budget using Isoflux antenna (transmit RF power = −5.0 dBW/25 kHz)

| Ship elevation angle | Sat. EIRP | Path loss | Pol. loss | Ship antenna gain | Ship G/T | C/N0 no interference | Antenna level | Noise level in 25 kHz | C/(N0+I0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| deg | dBW | dB | dB | dBi | dB/K | dBHz | dBm | dBm | dBHz |
| 0 | −3.0 | 145.6 | 3 | 3 | −27.2 | 49.8 | −118.6 | −116 | 41.4 |
| 10 | −3.5 | 142.2 | 3 | 3 | −27.2 | 52.7 | −115.7 | −116 | 44.2 |
| 20 | −4.0 | 139.4 | 3 | 2.5 | −27.7 | 54.5 | −113.9 | −116 | 46.1 |
| 30 | −5.5 | 137.2 | 3 | 1 | −29.2 | 53.7 | −114.7 | −116 | 45.3 |
| 40 | −7.0 | 135.4 | 3 | 0 | −30.2 | 53.0 | −115.4 | −116 | 44.5 |
| 50 | −9.0 | 134.2 | 3 | −1.5 | −31.7 | 50.7 | −117.7 | −116 | 42.3 |
| 60 | −10.0 | 133.2 | 3 | −3 | −33.2 | 49.2 | −119.2 | −116 | 40.8 |
| 70 | −12.0 | 132.6 | 3 | −4 | −34.2 | 46.8 | −121.6 | −116 | 38.4 |
| 80 | −13.0 | 132.2 | 3 | −10 | −40.2 | 40.2 | −128.2 | −116 | 31.8 |
| 90 | −13.5 | 132.1 | 3 | −20 | −50.2 | 29.8 | −138.6 | −116 | 21.4 |

### 2.2.2	VDE-SAT uplink link budget

Examples of link budgets for the VDE-SAT uplink are provided in the following sections.

### 2.2.2.1	Ship station e.i.r.p. vs. elevation angle

For ship station e.i.r.p vs elevation angle, see Annex 1.

### 2.2.2.2	Satellite antenna gain

Table A4-7 presents the gain of a 3-element Yagi satellite antenna with a peak gain of 8 dBi as a functon of elevation angle.

TABLE A4-7

Satellite antenna gain vs. ship elevation angle

| Ship elevation angle | Nadir offset angle | Boresight offset angle | Satellite antenna gain |
| --- | --- | --- | --- |
| deg. | deg. | deg. | dBi |
| 0 | 66.1 | 0 | 8 |
| 10 | 64.2 | 1.9 | 8 |
| 20 | 59.2 | 6.9 | 8 |
| 30 | 52.3 | 13.8 | 7.8 |
| 40 | 44.4 | 21.7 | 6.9 |
| 50 | 36 | 30.1 | 5.5 |
| 60 | 27.2 | 38.9 | 3.6 |
| 70 | 18.2 | 47.9 | 0.7 |
| 80 | 9.1 | 57 | −2.2 |
| 90 | 0 | 66.1 | −5.5 |

### 2.2.2.3	Satellite system noise temperature

The satellite noise level at the receiver input is shown in Table A4-8. Without external interference the system noise temperature is 25.7 dBK.

TABLE A4-8

Satellite receiver system noise temperature

| Antenna noise temperature | 200.0 | K |
| --- | --- | --- |
| Feed losses | 1.0 | dB |
| LNA noise figure | 2.0 | dB |
| LNA noise temperature | 159.7 | K |
| Feedloss noise temp. at LNA | 56.1 | K |
| Antenna noise temp. at LNA | 158.9 | K |
| System noise temp. at LNA | 374.7 | K |
| System noise temp. at LNA | 25.7 | dBK |

### 2.2.2.4	Uplink C/N0

The baseline uplink link budget is given in Table A4-10. It is optimised for 0 degree ship elevation angles.

It can be seen from Table A4-9 that the C/N0 is better than 74 dBHz for ship elevation angles between 0 and 65 degrees.

TABLE A4-9

VDE-SAT Uplink link budget, 6 W ship transmit power

| Ship elevation angle |  | Ship antenna gain | Ship e.i.r.p. | Polarization loss | Range | Path loss | Satellite antenna gain | Satellite G/T | C/N0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| deg |  | dBi | dBW | dB | km | dB | dBi | dB/K | dBHz |
| 0 | 3 |  | 10.8 | 3 | 2 830 | 145.56 | 8 | −17.6 | 73.2 |
| 10 | 3 |  | 10.8 | 3 | 1 932 | 142.25 | 8 | −17.6 | 76.5 |
| 20 | 2.5 |  | 10.3 | 3 | 1 392 | 139.40 | 8 | −17.6 | 78.9 |
| 30 | 1 |  | 8.8 | 3 | 1 075 | 137.16 | 7.8 | −17.8 | 79.4 |
| 40 | 0 |  | 7.8 | 3 | 882 | 135.44 | 6.9 | −18.7 | 79.2 |
| 50 | −1.5 |  | 6.3 | 3 | 761 | 134.16 | 5.5 | −20.1 | 77.6 |
| 60 | −3 |  | 4.8 | 3 | 683 | 133.22 | 3.6 | −22 | 75.2 |
| 70 | −4 |  | 3.8 | 3 | 635 | 132.58 | 0.7 | −24.9 | 71.9 |
| 80 | −10 |  | −2.2 | 3 | 608 | 132.21 | -2.2 | −27.8 | 63.4 |
| 90 | −20 |  | −12.2 | 3 | 600 | 132.09 | -5.5 | −31.1 | 50.2 |

### 2.2.3	Propagation effects

The received signal level on-board a ship will vary due to a number of factors as shown in Table A4-10. A Rice distribution with a carrier to multipath (C/M) ratio of 10 dB and fading bandwidth of 3 Hz is assumed (see Figure A4-3), however the system shall be adaptable to handle significantly worse and better propagation conditions. Mid-latitude fade depths due to ionospheric scintillation are shown in Table A4-11.

TABLE A4-10

Ionospheric effects for elevation angles of about 30° one-way traversal 
(derived from Recommendation ITU-R P.531)

| Effect | Frequency dependence | 0.1 GHz | 0.25 GHz | 1 GHz |
| --- | --- | --- | --- | --- |
| Faraday rotation | 1/f 2 | 30 rotations | 4.8 rotations | 108 |
| Propagation delay | 1/f 2 | 25 µs | 4 µs | 0.25 µs |
| Refraction | 1/f 2 | 1 | 0.16 | 0.6 |
| Variation in the direction of arrival (r.m.s.) | 1/f 2 | 20 | 3.2 | 12 |
| Absorption (auroral and/or polar cap) | ≈1/f 2 | 5 dB | 0.8 dB | 0.05 dB |
| Absorption (mid-latitude) | 1/f 2 | 1 dB | 0.16 dB | 0.01 dB |
| Dispersion | 1/f 3 | 0.4 ps/Hz | 0.026 ps/Hz | 0.0004 ps/Hz |
| Scintillation (1) | See Rec. ITU-R.P.531 | See Rec. ITU-R P.531 | See Rec. ITU-R P.531 | 20 dB peak-to-peak |
| * This estimate is based on a TEC of 1 018 electrons/m2, which is a high value of TEC encountered at low latitudes in day-time with high solar activity. (1) Values observed near the geomagnetic equator during the early night-time hours (local time) at equinox under conditions of high sunspot number. |  |  |  |  |

TABLE A4-11

Mid-latitude fade depths due to ionospheric scintillation (dB)

| Percentage of time (%) | Frequency (GHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0.1 | 0.2 | 0.5 | 1 |
| 1.0 | 5.9 | 1.5 | 0.2 | 0.1 |
| 0.5 | 9.3 | 2.3 | 0.4 | 0.1 |
| 0.2 | 16.6 | 4.2 | 0.7 | 0.2 |
| 0.1 | 25.0 | 6.2 | 1.0 | 0.3 |

Figure A4-3

Ricean fade depth probability

### 2.3	Protection of the radio astronomy service in the 150.05-153 MHz band from harmful interference from the VDE-SAT downlink

An appropriate protection limit for Radio Astronomy service in the 150.05-153.0 MHz band would be −238 dBW/m2 in a 2.95 MHz bandwidth centred around 152 MHz. Accordingly the maximum VDE-SAT downlink emission in the 150.05-153 MHz band should be below values shown in Table A4-12.

TABLE A4-12

Maximum satellite unwanted emissions in the 150.05-153 MHz band

| Ship elevation angle (deg) | RAS limit (W/m2/ 2.95 MHz) | Range (km) | Sat. max. interference e.i.r.p. |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | (W) | (dBW) | (dBW/Hz) |
| 0 | 1.58E-24 | 2830 | 1.60E-10 | −97.97 | −162.67 |
| 10 | 1.58E-24 | 1932 | 7.43E-11 | −101.29 | −165.99 |
| 20 | 1.58E-24 | 1392 | 3.86E-11 | −104.14 | −168.83 |
| 30 | 1.58E-24 | 1075 | 2.30E-11 | −106.38 | −171.08 |
| 40 | 1.58E-24 | 882 | 1.55E-11 | −108.10 | −172.80 |
| 50 | 1.58E-24 | 761 | 1.15E-11 | −109.38 | −174.08 |
| 60 | 1.58E-24 | 683 | 9.29E-12 | −110.32 | −175.02 |
| 70 | 1.58E-24 | 635 | 8.03E-12 | −110.95 | −175.65 |
| 80 | 1.58E-24 | 608 | 7.36E-12 | −111.33 | −176.03 |
| 90 | 1.58E-24 | 600 | 7.17E-12 | −111.44 | −176.14 |

## 2.4	Bit mapping

For bit mappings, see Annex 1.

## 2.5	Spread spectrum with constant envelope

Direct sequence spreading with constant envelope can be implemented according to the spreading strategy {RD-3}. This provides a way to generate constant envelope signals whilst allowing the use of linear modulations (i.e. BPSK, or QPSK for data modulation). In this approach the CPM spreading sequences are selected such that the spread symbols maintain quasi continuous phase even at the transition from one symbol to the next. The CPM spreading principle is provided in Figure A4-4.

Figure A4-4 –

CPM Spreading Principle

In order to avoid phase discontinuity at the data symbol transitions, the proposed solution is to adapt the spreading sequence to the modulation data. In other words, the CPM spreading sequence at the edge of each symbol is adapted according to the new input modulation symbol value to avoid any phase discontinuity. Such a solution produces a small loss at the receiver as the receiver does not know the edge symbol part of the used CPM spreading sequence. For a spreading factor of 16 or higher, the resulting correlation loss experienced by the receiver due to this issue is less than 0.25 dB. Performance losses with respect to conventional spreading is thus quite negligible provided that SF = 16 or larger is used.

The CPM spreading sequences are computed and optimized off-line and then stored in the memory of the terminals and receivers. A single spreading code is sufficient for all the users in the system. There is thus no need for storing multiple spreading sequences but just a single spreading sequence.

The stored spreading sequence is then applied starting from the preamble and continuing in the data part (as shown in Figure A4-5). It should be noted that the actual spreading sequence is actually partly dependent on the modulation symbols in order to ensure continuity of the signal phase when the modulation symbol changes (Figure A4-4). The spread samples are computed on the basis of the current modulation symbol and previous modulation symbol. For QPSK modulation there are 4 possible values for the phase difference of these two symbols. An index from 0 to 3 can point to the possible phase differences and is used to address which of 4 possible spreading sequences is actually used for computing the output signal. Figure A4-6 illustrates the power spectral properties of the proposed modulation scheme (with spreading factor 16). Due to its constant envelope properties, this modulation scheme can operate with a transmit power amplifier operating close to saturation while maintaining a low power leakage to adjacent channels.

Figure A4-5

Proposed Spreading in the CPM

Figure A4-6 –

Power spectral properties of spread spectrum with constant envelope

## 2.6	Baseband shaping and quadrature modulation

For baseband shaping of symbols, see Annex 1.

## 2.7	Transmission timing accuracy

For transmission accuracy figures, see Annex 1.

## 2.8	Half duplex and full duplex satellites

The system can be configured for both half and full duplex satellites as shown in Figure A4-7.

Figure A4-7

Half-duplex and full duplex satellite operation

## 2.9	Frame structure

For frame and transmission burst structure, see Annex 1.

## 2.10	Forward Error Correction and interleaving

For forward error correction and interleaving, see Annex 1.

## 2.11	VDE-SAT link configuration formats

For link configurations available for the VDE-SAT uplink and downlink, see Annex 1.

## 2.12 	VDE-SAT Downlink Block Channel Interleaver

A block channel interleaver is considered on the VDE-SAT downlink in order to reduce the impact of the channel short blockage (for example due to the AIS transmission from the vessel or fast fading events). The channel interleaver is applied to the code-words at the output of the encoder.

The interleaver can be applied on data blocks by column permutation (as long as the number of columns can be made as an integer power of 2). The interleaver memory in this case (from the point of view of the transmitter) is written by row and read by columns after having applied an inter-column permutation. The proposed column permutation is resulting from reading the column index in the reverse order  (bit shuffling), i.e. the column with index i5 i4, i3 i2, i1, i0  become the column i0, i1,i2, i3, i4, i5, where i0, i1 i2 i3 i4 and i5 are the bits representing a given number.

In more general cases (where the number of columns is not an integer power of 2), the interleaver index can be made available as table-lookup.

## 3	VDE-SAT link layer

For description of the link layer, see Annex 1.

## 4	Network layer

For description of the network layer, see Annex 1.

## 5	Transport layer

For description of the transport layer, see Annex 1.

[Where is Annex 5?]

Annex 6

Resource sharing method for VDES Terrestrial and Satellite Services

# 1	Introduction

This Annex describes how resource sharing (i.e. in time and frequency) for utilizing the VHF spectrum available between different VDES services and stations should be accomplished.

A ship may be within range of multiple controlling shore stations.  This annex describes a method for coordinating time and frequency resources between multiple controlling shore stations, particularly the use of bulletin boards and announcement signalling channels, as defined in Annex 1 and 3.

The VDE-SAT is an effective means to extend the VDES to areas outside of coastal VHF coverage. However, due to the large footprint of satellite, the VDE-SAT downlink signal may interfere with VDE-TER in the coastal areas when satellite is in visibility. Similarly, the terrestrial ship-to-shore VDE signals can interfere with the satellite reception of VDE-SAT uplink when a VDE Satellite is in the field of view. The method described in this Annex for resource sharing is derived based on the characteristics of VDE-TER and VDE-SAT, particularly the use of bulletin board and announcement signalling channels, as defined in Annexes 1, 3 and 4.

The channels 24, 84, 25, 85, 26 and 86 are allocated for VDE, with the lower leg frequencies used for ship-to-shore and the upper leg frequencies used for shore-to-ship and ship-to-ship. The channels 2027 (ASM 1) and 2028 (ASM 2) are allocated for ASM. Currently, 2 alternative frequency utilization plans for VDES are under consideration. They describe how resources are allocated and shared between VDE-TER, VDE-SAT and ASM. These 2 alternative frequency utilization plans are illustrated in Figure A6-1, and described further in this section.

Figure A6-1

RR APPENDIX 18 and VDES frequency utilization plans

## Frequency plan alternative 1

Frequency plan alternative 1 allow for utilization of the channels 24, 84, 25, 85, 26 and 86 in a shared manner between VDE-TER and VDE-SAT.

–	The four channels 1024, 1084, 1025 and 1085 are shared between ship-to-shore and ship-to-satellite (VDE-SAT uplink) services

–	The two channels 1026 and 1086 are exclusively reserved for ship-to-satellite (VDE-SAT uplink) services

–	The four channels 2024, 2084, 2025 and 2085 are shared among shore-to-ship, ship-to-ship and satellite-to-ship (VDE-SAT downlink) services

–	The two channels 2026 and 2086 are exclusively reserved for satellite-to-ship (VDE-SAT downlink) services.

–	Two channels 2027(ASM 1) and 2028 (ASM 2) are shared between ship-to-shore, ship-to-ship, shore-to-ship and ship-to-satellite services

## Frequency plan alternative 2

Frequency plan alternative 2 allow for utilization of channels 24, 84, 25 and 85 primarily for VDE-TER, while channels 26 and 86 exclusively reserved for VDE-SAT uplink. VDE-SAT uplink is also possible in channels 24, 84, 25 and 85, but the VDE-SAT uplink in these channels do not impose constraints on VDE-TER. Frequencies are exclusively reserved for VDE-SAT downlink within the frequency range 160.9625 MHz to 161.4875 MHz, which is not channelized in RR Appendix 18.

–	The four channels 1024, 1084, 1025 and 1085 are reserved for ship-to-shore services, but ship-to-satellite (VDE-SAT uplink) services are possible without imposing constraints on ship-to-shore services

–	The four channels 2024, 2084, 2025 and 2085 are reserved for shore-to-ship and ship-to-ship services, but ship-to-satellite (VDE-SAT uplink) services are possible without imposing constraints on shore-to-ship and ship-to-ship services

–	The four channels 1026, 1086, 2026 and 2086 are exclusively reserved for ship-to-satellite (VDE-SAT uplink) services.

–	Frequencies are exclusively reserved for satellite-to-ship (VDE-SAT downlink) services within the frequency range 160.9625 MHz to 161.4875 MHz, which is not channelized in RR Appendix 18

–	Two channels 2027(ASM 1) and 2028 (ASM 2) are shared between ship-to-shore, ship-to-ship, shore-to-ship and ship-to-satellite services

# 2	VDES resource sharing principles

## 2.1	AIS priority

Understanding that when transmissions occurs a VDES mobile station with a single antenna will suffer decreased receiver sensitivity, care must be taken to respect the AIS transmission and reception as the highest priority.

## 2.2	Coordination between ASM and VDE

VDE ship transmissions should be coordinated with transmissions on the ASM channels to ensure that ASM messages with new safety and navigational related information can be received.

## 2.3	Shore station VDES control area

The VDES resource assignments in the proximity of a shore station is monitored and controlled by a shore station. Shore stations utilize terrestrial bulletin board (TBB) and announcement signalling channels (ASC) to coordinate the resource assignment within the control area. The shore station may incorporate information regarding VDE satellite communications within the TBB and ASC. The shore station may acquire the VDE satellite information directly from the VDE-Satellite downlink (the satellite bulletin board and ASC) or in coordination with the satellite service providers.

There are dedicated slots and frequency bands for TBB and ASC that are reserved to communicate the required information to each vessel in the control area of a shore station. The default (or initial) assignment are described in § 4 of this Annex.

## 2.4	VDE-SAT resource assignment

Each satellite should use bulletin board and announcement channels (as defined in Annex 1 and 4) to communicate the VDE-SAT resource assignments (both downlink and uplink) to vessels in the coverage area.

There are dedicated slots and frequency bands for the satellite bulletin board and announcement channels that are reserved to communicate the required information to each vessel in the field of view of a satellite.

Since the satellite coverage may include several shore station control areas, the VDE-SAT resource assignment should respect all requirements of shore control areas that are within the field of view at any given time. Within each satellite orbit the information regarding the resource assignment should be updated according to the shore station control areas in the satellite field of view.

A default (or initial) VDE-SAT resource allocation is defined in § 4 below to serve as the starting point for the resource sharing.

# 3	VDE-TER Resource sharing between multiple controlling shore stations

The allocation of frequency and time slots used for the bulletin board announcement must be coordinated between controlling stations.  Other resource assignments are managed based on the content of the bulletin board and announcement signalling channels. The assignment may change dynamically (according to temporal demands).

There are dedicated resources in channel 2024 and 2084 that are assigned to the terrestrial bulletin board and announcement channels, as described in Annex 1 and 3.

Channels 2024, 2084, 2025 and 2085 are shared between multiple controlling stations. The resource sharing must be coordinated between shore station operators. This coordination can be done either directly between the operators or rely on the bulletin board and announcement channels of the shore stations, depending on the shore control areas, the resource assignment may vary. As an initial configuration for resource sharing, the controlling shore stations should adopt a static assignment in time and frequency.

# 4	VDE-TER and VDE-SAT downlink resource sharing

## 4.1	Resource sharing with frequency plan alternative 1

With frequency plan alternative 1, the channels 2026 and 2086 are dedicated to VDE-SAT downlink. Within these exclusive VDE-SAT channels, there are dedicated time slots that are assigned to the satellite bulletin board and announcement signalling channels as described in Annex 1 and 4. Other slot assignments in the exclusive VDE-SAT frequency bands are managed based on the content of the bulletin board and announcement signalling channels. The assignment may change dynamically (according to the satellite coverage or temporal demands).

Channels 2024, 2084, 2025 and 2085 are shared between VDE-SAT Downlink and VDE-TER. Depending on the satellite coverage area and the shore control areas, the resource assignment may vary.

There are dedicated time slots in channel 2024 and 2084 that are assigned to the terrestrial signalling channel and terrestrial bulletin board, as described in Annex 1 and 3. These slots should not be used by VDE-SAT downlink when a VDE shore station is within the satellite coverage area.

A shore station may assign the full resources of channels 2024, 2084, 2025 and 2085 for terrestrial services when there is no transmitting VDE satellite in the field of view.

When a transmitting VDE satellite is in the field of view the resource sharing between VDE-SAT downlink and VDE shore-to-ship and ship-to-ship must be coordinated between the shore operator and the satellite operator. This coordination can be done either directly between the operators or rely on the bulletin board and announcement channels of the satellite and shore stations. As an initial configuration for resource sharing, a static assignment in time and frequency should be adopted by the terrestrial and satellite entities.

–	Channels 2024 and 2084 are exclusively used for terrestrial VDE, maintaining the original signalling assignment that was described above

–	Channels 2026 and 2086 are exclusively used for VDE-SAT downlink, maintaining the original signalling assignment that was described above

–	Channels 2025 and 2085 are time-shared between VDE-SAT downlink and VDE terrestrial services. The time sharing is based on time intervals of 2.4 s (90 slots) that are assigned periodically to VDE-SAT and VDE terrestrial services

As the starting point of VDES resource sharing or in the absence of coordination between the shore and satellite operation, this resource sharing method should be used.

Coordination of resource sharing between VDE ship-to-ship and VDE-SAT downlink for areas not controlled by a VDE shore station is managed by the VDE-SAT bulletin board, as described in Annex 4. As a starting point for this resource sharing or in the absence of any VDE-SAT bulletin board, the resource sharing method described above should be used.

## 4.2	Resource sharing with frequency plan alternative 2

With frequency plan alternative 2, the frequency band from 160.9625 MHz to 161.4875 MHz is dedicated to VDE-SAT downlink. The frequencies in this band are not channelized in RR APPENDIX 18. Within this exclusive VDE-SAT band, there are dedicated channels and time slots that are assigned to the satellite bulletin board and announcement signalling channels as described in Annex 1 and 4. Other slot assignments in this exclusive VDE-SAT frequency band are managed based on the content of the bulletin board and announcement signalling channels. The assignment may change dynamically (according to the satellite coverage or temporal demands).

# 5	Sharing between different VDE satellite systems

The sharing between two or more satellite systems is coordinated between the satellite operators and organized through the bulletin board, delivered by satellites in VDE-SAT downlink band, as described in Annex 1 and 4. Ships use the satellite bulletin boards for channel and resource configuration.

The waveform used for the bulletin board should allow for detection of overlapping signals received from multiple satellites. The use of direct sequence spreading as defined in Annex 4 allows for detection of up to 8 overlapping satellite signals.

# 6	VDE-TER and VDE-SAT uplink resource sharing

## 6.1	Resource sharing with frequency plan alternative 1

With frequency plan alternative 1, the lower frequency bands, channel 1026 and 1086 are dedicated to VDE-SAT uplink while channels 1024, 1084, 1025 and 1085 are shared between VDE-TER and VDE-SAT.

The exclusive VDE-SAT uplink channels may be used for dedicated (demand assigned) or random access to satellite. Since there is no VDE terrestrial interference on these two channels, these channels should be used for higher priority message (safety, distress, acknowledgement, etc.).

Through the bulletin board, a shore station may assign the full resources of channels 1024, 1084, 1025 and 1085 for terrestrial services when there is no receiving VDE satellite in the field of view.

When a transmitting VDE satellite is in the field of view the resource sharing between VDE-SAT uplink and VDE-TER ship-to-shore must be coordinated between the shore operator and the satellite operator. This coordination can be done either directly between the operators or rely on the bulletin board and announcement channels of the satellite and shore stations. As an initial configuration for resource sharing, a static assignment in time and frequency should be adopted by the terrestrial and satellite entities.

–	Channels 1024 and 1084 are exclusively used for VDE-TER ship-to-shore

–	Channels 1026 and 1086 are exclusively used for VDE-SAT uplink (ship-to-satellite)

–	Channels 1025 and 1085 are time-shared between the VDE-SAT uplink and VDE-TER services. The time-sharing is based on time intervals of 1 hexslot (6 slots) that are assigned alternately to VDE-SAT and VDE-TER services

As the starting point of VDES resource sharing or in the absence of coordination between the shore and satellite operation, this resource sharing method should be used.

## 6.2	Resource sharing with frequency plan alternative 2

With frequency plan alternative 2, the utilization of channels 24, 84, 25 and 85 is primarily for VDE-TER. VDE-SAT uplink is also possible in channels 24, 84, 25 and 85, but the VDE-SAT uplink in these channels do not impose constraints on VDE-TER and should only use resources not reserved by VDE-TER.

Channels 26 and 86 exclusively reserved for VDE-SAT uplink. Therefore, on these channels no resources are shared and no sharing scheme is required.

θ°=earth-satellite elevation angle

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                        0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

-

|  |  |  |  |  |  |  | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  |  | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  |  | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

-

-

-

-

-

-

### --

-

-

-

-

Antenna gain dBi

Elevation in degrees

Gain in dBi

Antenna gain in dBi

Antenna gain dBi

Elevation in degrees

Gain in dBi

Antenna gain in dBi

-

θ°=earth-satellite elevation angle

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                        0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

--

| Known: R = Rearth + height = 6.92 106 m Mearth = 5.98 1024 kg G = 6.673 10−11 N m2/kg2; Unknown: v a T. |  |
| --- | --- |

-

-

+29.1 dB reference

−103 dBm

+29.1 dB reference

−103 dBm

85 km (46 NM) ship-to-shore Coverage range for −103 dBm shore antenna height = 75 m

85 km (46 NM) ship-to-shore Coverage range for −103 dBm shore antenna height = 75 m

85 km (46 NM) shore-to-ship Coverage range for −98 dBm shore antenna height = 75 m

85 km (46 NM) shore-to-ship Coverage range for −98 dBm shore antenna height = 75 m

+28.3 dB reference −107 dBm

+28.3 dB reference −107 dBm

--

Elevation in degrees

Elevation in degrees

Delay in seconds

Delay in seconds

Time

Time

PFDθ° (dBW/(m2*4 kHz))=-149+0.16*θ°                         0°≤θ<45°;-142+0.53*θ°-45°      45°≤θ<60°;-134+0.1*θ°-60°         60°≤θ≤90°.

Elevation in degrees

Elevation in degrees

Power in dBm

Power in dBm

Elevation in degrees

Elevation in degrees

Power in dB

Power in dB

Antenna gain dBi

Elevation in degrees

Gain in dBi

Antenna gain in dBi

Antenna gain dBi

Elevation in degrees

Gain in dBi

Antenna gain in dBi

Elevation in degrees

Elevation in degrees

Carrier at Rx input (with antenna gain)

Antenna gain dBi

Carrier at Rx input (with antenna gain)

Antenna gain dBi

Gain in dBi

Gain in dBi

Signal level in dBm

Signal level in dBm

Eb/N0 in dB

Eb/N0 in dB

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

Antenna 1

Antenna 2

Antenna 3

Antenna 4

-

Es/N0 (dB)

Es/N0 (dB)

The use of some frequencies in the band 156-164 MHz, contained in this Recommendation, do not comply with the RR currently in force. This Recommendation therefore should not be seen as prejudging the decisions of WRC-15. ITU-R Study Group 5 is invited to review this Recommendation taking into account the decisions made by WRC-15.

Note by the BR Secretariat – The figures in this Recommendation are available in English only. The other languages will be prepared in due course.

The baseband shall employ a root raised cosine filter.