Input paper:   ENAV21-11.22

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		□  ENG		□  PAP						□ Input

x  ENAV	              □  VTS							               x  Information

Agenda item 	(from agenda)			11

Workplan Task Number / Technical Domain 2	…………………………………

Working Group					WG3

Author(s) / Submitter(s)				Hongxing Li

Developing Progress of VDES Experimental Satellite

# Summary

VHF Data Exchange System (VDES) has been recognized as the one of the core techniques of E-Navigation and GMDSS modernization. On the ENAV committee meetings and VDES workshop meetings, VDES has been studied and discussed by government organizations, institutes, companies etc. from different country. In China, an VDES experimental satellite has been built, which was initiated by Shanghai Spaceflight Institute of TT&C and Telecommunication and Zhejiang Space-Iching Science and Technology Co., Ltd.. This document gives an introduction of the satellite.

## Purpose of the document

The document is an information paper submitted to WG3 of ENAV21.

## Related documents

none

# Background

The payload of VDES experimental satellite was researched and designed under the technical framework of recommendation ITU-R M.2092-0. Although most of the technical issues have been discussed many times, they are needed to be tested and verified via satellite in a real operating environment. The mission of this And then, experimental data will be submitted to committee meeting to be discussed.

# Discussion

## VDES Payload

The experimental VDES payload consists of VHF antenna and VDES signal transceiver，all of which are operated in VHF maritime band.

### VDES Antenna

The VDES antenna has two folded dipole Yagi antenna which is used together to form one left hand circle polarization beam of which the maximum gain is 12dBi (Fig. 1 and Fig. 2) . Each Yagi antenna has three elements in crossed configuration. At the launch stage, two Yagi antennas are stowed by nylon thread (Fig. 3) onto the satellite platform(Fig. 4) in order to minimize the launch volume of the satellite. The stowed geometry size of each antenna does not exceed 150mm×240mm×588mm. The total mass of two Yagi antennas is baout 3.5kg . When the satellite is on-orbit, the pre-stowed VDES antenna will be released automatically after nylon threads burnout by heating resistance (Fig. 5).

Fig. 1  VDES Antenna Pattern (156 MHz)                    Fig. 2  VDES Antenna Pattern (162 MHz)

Fig. 3  VDES Antenna Stowed by nylon thread

In order to verify that the antennas would be released reliably, VDES antenna deployment testing were  conducted both in normal and space environment.  Initially, the testing work was conducted in normal atmosphere, while further deployment testing in the vacuum was conducted throughout -90℃ to +90℃ at 1 ℃ intervals . The testing results show that the antenna can be deployed reliably.

- Fig.4  VDES Satellite Stowed Configuration

- Fig. 5  VDES Satellite deployed Configuration

### VDES signal transceiver

The mission of the VDES signal transceiver is processing the radio signal, such as AIS, LAIS, ASM, and VDE from the VDES antennas, and then transforming the radio signal to digital signal to be demodulated and decoded. On the other hand, VDE data is be transmitted after modulation, conding, framing, and other processing. The transceiver is a software-defined radio (SDR) hardware platform, which is based on high-performance FPGA , DSP , and ADC chips. It also has two reception and transmitting channels. So the tranceiver hardware can be simplified.

As far as signal processing software is concerned, five kinds of physical layer( PL ) frames proposed in recommendation ITU-R M.2092-0 were chosen to be realized in FPGA. These PL-Frames include PL1 , PL3 , and PL4 of uplink PL-Frames and PL1 and  PL3 of downlink PL-Frame. Accordingly, the modulation schemes spread-QPSK, OQPSK, 16APSK, spread-BPSK and 8PSK were adopted. Simultaneously, Turbo coding and decoding with 1/3, 1/2, and 3/4 code rate were used in both uplink and downlink.

According to the suggestion of ITU-R M.2092-0, transmitting power of the transceiver is less than 4W. The other parameters of the transceiver are shown below.

- Sensitivity: AIS/ASM -112dBm@PER=15% and VDE down to -106dBm@PER≤1%

- Mass: 2.0kg

- Dimensions: 190mm*170mm*49mm

- Power consumption: ≤13W in the long-term and 19W increment in the short-term

- Anti-burnout ability: ≤20dBm

- Operational temperature: -15℃ ~ +50℃

- Supply voltage:6V~13.5V

In order to testing the function of link layer, network layer, and transport layer (all these three layers can be called protocol layer) of VDES protocol, an upload channel was preset in the transceiver. The mission of upload channel is to upload the optimized and up to date protocol layer software in order to obtain the most significant performance or VDES-SAT. The upload channel was operated in VHF band, using VDES-SAT uplink frequency. The upload data was modulated by OQPSK scheme.

## Satellite platform

The experiment satellite is a 45 kg, 2417mm（X）×2001mm（Y）×1400mm（Z）satellite. Three-axis stability of zero momentum controlling is used to obtain steady direction to ground. The designed on-orbit lifetime is 12 months. The satellite was planned to be launched at the 2nd quarter 2018.

## On-orbit Verification Scheme

Since the VDES integrates AIS, ASM, and VDE, the basic function of the VDES payload is receiving and processing AIS signals all day time. Simultaneously, ASM signal receiving and bidirectional VDE communication testing and verification campaign will be demonstrated while the satellite flights over head. The objective of the mission is to demonstrate the feasibility and assess the performance of satellite link of the VHF data exchange via satellite in a real operating environment. Based on these results, recommendations regarding the uplink and downlink physical layer will be given. Thus the assessing results can enable standardisation of a suitable set of waveforms and corresponding parameters.

Two experiment scenarios have been considered, which are point to point communication and broadcasting. For the first scenario, two testing equipments will be placed respectively at Shanghai and Shenzhen, China.  Two remotely placed test equipments transmit predeterminate information to each other. The objective of this testing is to verify the maximum data rate of uplink and downlink, modulation efficiency and PL frame feasibility. For the second scenario, the testing equipment placed at Shanghai, China, will transmits predeterminate signal to the satellite. The satellite receives and processes the signal, then remodulate and recode the received information, ultimately, broadcast it to the receivers at  Zhoushan and Shenzhen, China.

# References

1)Recommendation  ITU-R  M.2092-0, Technical characteristics for a VHF data exchange system in the VHF maritime mobile band.

Input document number, to be assigned by the Committee Secretary

Input papers should be assigned to a work task as listed in the Committee work plan which is available in  input papers. Leave open if uncertain but consider how the paper is to be processed if not relevant to a work task