Input paper:   ENAV21-11.21

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		□  ENG		□  PAP						   Input

ENAV	□  VTS								□  Information

Agenda item 	(from agenda)			11

Workplan Task Number / Technical Domain 2	2.2.1

Working Group					WG 3

Author(s) / Submitter(s)				Qing Hu, Jiang Yi, Bo Hu

Proposal for the Revision on the Recommendation ITU-R M.1371-5

# Summary

During updating of the automatic identification system (AIS) technique recommendation ITU-R M.1371-5, the BeiDou Satellite Navigation System (BDS) as a recognized component of the World Wide Radio Navigation System (WWRNS) should be considered as one of electronic position fixing system (EPFS) in AIS.

It is proposed that the next revised version of AIS technique recommendation accept BDS as an AIS applied EPFS and update related AIS messages parameters on ANNEX8 of ITU-R M.1371-5.

## Purpose of the document

To propose to revise the Recommendation ITU-R M.1371-5.

## Purpose of the document

Recommendation ITU-R M.1371-5.

# Background

The automatic identification system (AIS) has been widely applied in the safety of maritime navigation. The electronic position fixing system (EPFS) plays an important role in AIS, and EPFS device information should be contained in several AIS messages, including AIS message 4, message 5, Message 11, message 19, message 21 and message 24B. Nowadays, the BeiDou Satellite Navigation System (BDS) is one of EPFS, which has been recognized by the International Maritime Organization (IMO) as a component of the World Wide Radio Navigation System (WWRNS).

In May 2016, ITU-R WP5B received the proposal that proposed to revise the AIS recommendation ITU-R M.1371-5 for accommodating to the requirement of appending BDS as one option of the type of EPFS device of AIS.

# Discussion

## 3.1 Description of the EPFS Information in AIS Messages

In the Recommendation ITU-R M.1371-5 Annex 8, the information of EPFS device type is included in several AIS messages, including the base station report (message 4), the static and voyage related data (message 5), the coordinated universal time and date response (message 11), the extended class B equipment position report (message 19), the Aids-to-navigation report (message 21) and the static data report (message 24B).

The parameters of the EPFS device are described and ordered as following:

0 = Undefined (default)

1 = GPS

2 = GLONASS

3 = Combined GPS/GLONASS

4 = Loran-C

5 = Chayka

6 = Integrated Navigation System

7 = surveyed. For fixed AtoN and virtual AtoN, the charted position should be used. The accurate position enhances its function as a radar reference target

8 = Galileo

9-14 = not used

15 = internal GNSS

Considered BDS as an option of the AIS PEFS type, accordingly, in the revised version of the Recommendation ITU-R M.1371, the parameters of EPFS device might be described as following:

0 = undefined (default)

1 = global positioning system (GPS)

2 = GNSS (GLONASS)

3 = combined GPS/GLONASS/BDS

4 = Loran-C

5 = Chayka

6 = integrated navigation system

7 = surveyed

8 = Galileo

9 = BeiDou Navigation Satellite System (BDS)

10 -14 = not used

15 = internal GNSS

# References

- ITU-R M.1371-5;

- IMO, Resolution A.1046(27) WWRNS.

# Action requested of the Committee

The Committee is requested to:

- 1	To propose to revise the Recommendation ITU-R M.1371-5 appending BDS as one option of EPFS in the related AIS Messages as annex1.

| MB/NC1 | Line number (e.g. 17) | Clause/ Subclause (e.g. 3.1) | Paragraph/ Figure/ Table/ (e.g. Table 1) | Type of comment2 | Comments |  | Proposed change | Observations of the secretariat |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CN | 1 | Abbreviations/Glossary | Paragraph.6 | Ge |  |  | Appending ” BDS BeiDou navigation satellite system” |  |
| CN | 2 | Annex 8 §3.2 | Table 51 | Ge | Message 4& Message11: | Parameter field “Type of electronic position fixing device” Use of differential corrections is defined by field position accuracy above: 0 = undefined (default) 1 = global positioning system (GPS) 2 = GNSS (GLONASS) 3 = combined GPS/GLONASS 4 = Loran-C 5 = Chayka 6 = integrated navigation system 7 = surveyed 8 = Galileo 9 -14 = not used 15 = internal GNSS | Use of differential corrections is defined by field position accuracy above: 0 = undefined (default) 1 = global positioning system (GPS) 2 = GNSS (GLONASS) 3 = combined GPS/GLONASS/BDS 4 = Loran-C 5 = Chayka 6 = integrated navigation system 7 = surveyed 8 = Galileo 9 = BeiDou Navigation Satellite System (BDS) 10 -14 = not used 15 = internal GNSS |  |
| CN | 3 | Annex 8 §3.3 | Table 52 | Ge | Message 5: |  |  |  |
| CN | 4 | Annex 8 §3.17 | Table 71 | Ge | Message 19: |  |  |  |
| CN | 5 | Annex 8 §3.19 | Table 73 | Ge | Message 21: |  |  |  |
| CN | 6 | Annex 8 §3.22 | Table 79 | Ge | Message 24 part B: |  |  |  |

Input document number, to be assigned by the Committee Secretary

Input papers should be assigned to a work task as listed in the Committee work plan which is available in  input papers. Leave open if uncertain but consider how the paper is to be processed if not relevant to a work task