Input paper:   ENAV21-11.23

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		□  ENG		□  PAP						x  Input

x  ENAV	              □  VTS							               □  Information

Agenda item 	(from agenda)				11

Workplan Task Number / Technical Domain 2	…………………………………

Working Group					WG3

Author(s) / Submitter(s)				Wenquan Wu

VDE-SAT Downlink Frequency Division Analysis and Liaison to ITU-R WP 5B Chinese Representative

# Summary

This file mainly analyzes the legitimacy of VDE-SAT downlink frequency in China Radio frequency division regulation, and explains the preliminary coordination with ITU-R WP 5B China representative. Technical suggestions are put forward for the interference within the frequency band pointed out in the ITU-R WP 5B report submitted by IALA.

## Purpose of the document

This file is submitted as one of the input files for the ENAV21 conference WG3.

## Related documents

1)IALA submit ITU-R WP 5B Report in May 2017: R15-WP5B-C-0305!N24!MSW-E

2)Regulation of frequency division in People's Republic of China(2014 Edition)

# Background

The ITU-R WP 5B from 22nd May to 2nd June 2017 was the third meeting in the study cycle. IALA was represented by Stefan Bober. by kind permission of the Federal Waterways and Shipping Administration of Germany. At the last ITU-R WP 5B meeting in May the Working Document towards a Preliminary Draft New Report ITU-R M.[VDES-SAT] was updated to include contributions from multiple administrations and IALA. This document is as R15-WP5B-C-0305!N24!MSW-E.  However, there is one contentious issue remaining, it is the sharing compatibility between the VDE-SAT and the land mobile service. Russia and China questions if the protection criteria as defined in ITU-R M.2092-0 are sufficient.

All WG3 members are invited to contact their administration’s ITU representative attending WP 5B and urge them to support frequency allocations for VDE-SAT at WRC-19 and the contribution from IALA at the next WP 5B meeting.

# Discussion

## VDE-SAT downlink frequency division legitimacy analysis

### Frequency alternative 1

VDE-SAT downlink frequency division alternative 1, channels 2024, 2084, 2025, 2085, 2026, 2086, frequency range is 161.800~161.925MHz. In the China Radio frequency division regulation (2014 Edition), 161.475-161.9875MHz is the preferred frequency band for water mobile services (based on 5.226 footnotes). But it still faces coordination with land mobile, air mobile (5.228E footnotes) and satellite mobile (earth to space, 5.228F footnotes) operations.

### Frequency alternative 2

VDE-SAT downlink frequency division alternative 2 of 160.9625-161.4875MHz, in China's regulation, 160.975-161.475MHz is assigned to land fixed services and mobile services. Therefore, alternative 2, as an alternative frequency of VDE-SAT downlink frequency, is infeasible in china.

### About band 162-164MHz

The provisions of the "China radio frequency division" (2014 Edition), 162.05-167MHz (ground) can be used for space operation, but the need to coordinate in accordance with RR Nos.  9.21. This section also has land fixed, mobile and radio positioning services available, so it needs to be coordinated with the China Radio administration.

## Preliminary coordination with the ITU-R WP 5B China representative

### Frequency alternative 1

After preliminary coordination with the ITU-R WP 5B China representative, although 161.800~161.925MHz is a water priority band, it still needs to be coordinated with other available services.

### Frequency alternative 2

After preliminary coordination with the ITU-R WP 5B China representative, 160.9625-161.4875MHz is not currently available for water mobile services in china.

### About band 162-164MHz

After preliminary coordination with the ITU-R WP 5B China representative, 162-164MHz is not currently available for water mobile services in china.

### Next step work

Shanghai Spaceflight Institute of TT&C and Telecommunication is striving to become a industry member of IALA, and will be further coordinated with the ITU-R WP 5B China representative and the China Radio administration

## Technical suggestions

### About in-band interference

According to the 6.1.2.4 conclusions of R15-WP5B-C-0305's N24 MSW-E report, the VDES satellite component emissions in downlink can cause unacceptable interference to the systems in the mobile service. Therefore sharing of VDES satellite component in downlink with the stations of the mobile service is unfeasible in the considered frequency bands.

For future VDES satellites, possible technical solutions are available:

1) through high-performance VDES satellite antenna, to reduce satellite coverage, antenna formation technique can also be used to avoid interference;

2) it can be coordinated on time according to the time of satellite transit;

3) the VDES satellite closes down the downlink when  moves on the ground mobile system service area.

# Conclusion

The VDE-SAT downlink frequency alternative 1(161.800~161.925MHz) is available in China but needs coordination with land mobile services. Frequency alternative 2(160.9625-161.4875MHz) and 162-164MHz are not currently available for water mobile services in china.

Some technical measures can be adopted to mitigate the interference of satellite downlink transmission to other services within the frequency band.

It is suggested that the report on the submission of ITU WP5B after the ENAV21 meeting will be fully communicated with the Chinese representatives ahead of November. So that the November coordination meeting can make progress.

Input document number, to be assigned by the Committee Secretary

Input papers should be assigned to a work task as listed in the Committee work plan which is available in  input papers. Leave open if uncertain but consider how the paper is to be processed if not relevant to a work task