Input paper: 	     ENG5-9.26

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□X ENG		□  PAP					□X  Input

□  ENAV	□  VTS							□  Information

Agenda item 			9

Technical Domain / Task Number 2			TD#1 - Light and vision physics, Visual Signalling / 26

Author(s) / Submitter(s)			Pärtel Keskküla

# Russian guidelines on leading lines

# Summary

This is my translation of more interesting (i.e not covered in IALA or Finnish guidelines) parts and concepts concerning design of leading lines from Russian guidelines on design of AtoNs (Инструксия по навигационному оборудованию (ИНО-2000). Санкт-Петербург: Министерство Обороны Российской Федерации Главное Управление Навигаций и Океанографий, 2001), and are not covered in IALA guidelines.

# Action requested of the Committee

The Committee is requested to:

1.	Review the input paper and consider if any of parts or concepts from it could be included in the updated IALA Guidelines on Design of Leading Lines.

parts concerning design of leading lines from Russian guidelines on design of AtoNs

# Инструксия по навигационному оборудованию (ИНО-2000)

# Санкт-Петербург: Министерство Обороны Российской Федерации Главное Управление Навигаций и Океанографий, 2001

Морские навигационные створы

# Contents

Russian guidelines on leading lines	1

1	Summary	1

2	Action requested of the Committee	1

ANNEX A	parts concerning design of leading lines from Russian guidelines on design of AtoNs	2

Инструксия по навигационному оборудованию (ИНО-2000)	3

Санкт-Петербург: Министерство Обороны Российской Федерации Главное Управление Навигаций и Океанографий, 2001	3

Contents	4

List of Figures	4

List of Tables	5

1.	elements of leading line	6

2.	acceptable off-axis distance	6

3.	Calculation of distance between leading marks	8

4.	defining dimensions of leading marks	9

5.	calculation of vertical angle of leading line	9

6.	Shape of dayboards	11

7.	setting up of leading lines on site	11

8.	checking of leading line	12

9.	using binoculars for sailing by leading line	13

10.	order of calculation and some recommendations on calculation and installation of leading lines	14

# List of Figures

Figure 1. Zone of leading line	5

Figure 2.		6

Figure 3. Checking of a leading line	11

Figure 4. The location of the light zoomed in. Location of the light is shown with red circle.	12

# List of Tables

Table 1.		7

Table 2.		9

Table 3.		9

Table 4.		10

# elements of leading line

Sailing along a leading line takes place in the zone where the mariner considers himself being on the centre line of the leading line and only at the edges of that zone perceives deviation of the lights or the centre lines of the marks from one vertical line (figure 1).

Curves that limit the zone of leading line are called visor curves. Both curves consist of geometric locations of points where one can perceive the deviation of the marks or lights from one vertical line. Distance from the centre line to the curve is called off-axis distance PK.

Figure 1. Zone of leading line

For using leading lines types (appearances) of leading lines must be distinguished:

- day leading line – leading line with marks;

- night leading line – leading line with lights.

A day leading line can be point leading line if at the end of the leading line shape of the daymarks is not detectable and the marks are visible as dots or points, or leading line with marks with finite dimensions.

# acceptable off-axis distance

* Off-axis distance (in Russian literally “lateral deviation” - Pärtel) on the whole length and, especially off-axis distance in the end of the useful segment of a leading line PK must not exceed the acceptable off-axis distance, i.e

PK ≤ Pacc

In practice, for achieving most favourable conditions for using a leading line with its optimal sensitivity value of off-axis distance at the end of useful segment is calculated in the following order:

- for artificial channels and fairways with the width at the bottom 2B ≤ 200 m

Pacc = B – 2q						(25)

where

q – width of the vessel (m)

At the edges of the channels floating AtoN have to be placed.

- for fairways with width from 200  to 4000 m

Pacc = KB						(26)

Coefficient K is taken depending on the width of the fairway 2B:

0,6 	for fairways with width from 	200 to 400 m

0,5 	for fairways with width from 	400 to 2000 m

0,4 	for fairways with width from 	1000 to 2000 m

0,3 	for fairways with width from 	2000 to 3000 m

0,2 	for fairways with width from 	3000 to 4000 m

The given coefficients are not rigid, they guarantee necessary reserve of “free water” behind the limits of the zone of leading line according to extent of possible deviation practically for all types of vessels. If necessary, these they can be changed, up or down. For example, for leading lines for fairways with width of 2000 to 3000 m coefficient K can be increased, with the cost of some decrease of sensitivity, up to the level where Pacc is in the limits of 400 to 800 m.

…

* Acceptable off-axis distances for a leading line serving navigation between dangers to navigation that are positioned symmetrically in relation to the axis of the leading line is calculated with the formulae (25) or (26). 2B is meant to be the width of the safe passage between the dangers.

* Acceptable off-axis distances for a leading line serving navigation between dangers to navigation that are not positioned symmetrically in relation to the axis of the leading line is calculated with the formulae (25) or (26) for each of the dangers nearest to the axis of the leading line.

Figure 2.

In this case B is meant to be the distances B1, B2, B3 (figure 2) from the axis of the leading line perpendicularly to the points at the dangers 1, 2, 3 nearest to the leading line. The distances B1, B2, B3, but also the distances from the front leading mark are taken from a survey sheet or chart with the largest possible scale. The results are put into the following table:

| Number of danger | Distance B, m | Pacc, m | Distance D, km |
| --- | --- | --- | --- |

Table 1.

The most distant of these data is used for calculating the distance between the leading marks.

# Calculation of distance between leading marks

* The distance d between leading marks is calculated with the formula:

d = DK2 / [(3,44/) Pacc – DK]					(27)

where

DK – distance from the front mark to the end of useful segment, km

Pacc -  acceptable off-axis distance, m

 – horizontal angle at which deviation is detected, ‘ (minute of arc)

IALA considers 1:10 the most favourable ratio for the distance between the leading marks and the distance from the front mark to the end of useful segment (in Russian literally “working distance of the leading line”). However, acceptable deviation from this ratio is from not more than 1:2 to not less than 1:20.

* Value of horizontal angle  depends on the type of the leading line (point or with the daymarks with finite dimensions) and on the vertical angle , which can be defined only by following calculations of the heights of the leading marks. Therefore the calculation of the distance between the leading marks is done in two steps: initial and final.

* Initial calculations of the distance between the marks d for a day leading line is done with the following values of :

1.5’ – for a point leading line;

1.0’ – for a leading line with marks with finite dimensions.

These values are used for calculations if the form of the leading line (calculated with the formulae 37 – 40) is not already known.

The distance DF (km) at which the form of the front leading mark is detectable is calculated with the formula:

DF = 1,72 × 2b

where

2b – width of the leading mark, m

A leading line for which DF is more or equal to the working distance DK is considered a leading line with daymarks with finite dimensions, for which it is less, is a point leading line.

* Initial calculation of the distance between the marks for a night leading line is made with the value of horizontal angle   = 1.5’.

* Final calculation of the distance d is made after defining the heights of the marks, determining the vertical angles and control calculations of the off-axis distances in the whole length of the useful segment.

If the calculated off-axis distances are less than acceptable distance, the results of the initial calculations can be considered final. If calculated off-axis distances are bigger than acceptable distance the elements of the leading line must be recalculated.

…

# defining dimensions of leading marks

...

* To ensure confident detection and identification of the front leading mark from the end of the useful segment the vertical base angle  (vertical subtense of the front mark - Pärtel) must be taken not less than 4’ (1.16 mrad).

In isolated cases with significantly high coast where the leading marks are projected on the background of the sky when observed from the useful segment it is acceptable to take  = 2’ (0.58 mrad).

When installing leading marks in ports and other places where they are located among different buildings and constructions height of the front leading mark is determined based on the condition of clear visibility of it among surrounding buildings and constructions.

In any case height of the front mark from its base cannot be lower than 10 m in such places.

…

* Width of the daymarks is calculated with the formulae:

- day point leading line

2b1 = 0.29DK							(37)*

2b2 = 0.29(DK + d)						(38)**

- for day leading line with marks with finite dimensions

2b1 = 0.58DK							(39)

2b2 = 0.58(DK + d)						(40)

where

2b1 – width of the front daymark, m

2b2 – width of the rear daymark, m

In practice of leading marks the ratio of height and width of daymarks is mostly 3:1, 4:1, 5:1 and is determined by the selected design of the mark.

…

(* gives 1’ horizontal angle)

(** gives 2’ horizontal angle)

# calculation of vertical angle of leading line

* …

The vertical angle is calculated also for the distance Dmax. The distance Dmax where the vertical angle has the largest value is calculated with the formula:

Dmax = h-eH-e- h-ed						(43)

where

h – height of the front mark, m

H  - height of the rear mark, m

e – height of the observer, m

d – distance between the marks, km

If the point at the distance Dmax is out of the useful segment of the leading line, it is not necessary to be calculate the of value max. In this case it can be taken that values of vertical angles in useful segment are between the values calculated for D0 and DK. If Dmax is in the useful segment calculation of max is necessary.

The distance where D=0 is calculated with the formula:

D = h-eh-hd							(44)

* The values of vertical angle  are calculated for day leading lines and night leading lines. For day leading lines  is calculated as the angle between the upper edges of the marks (daymarks) or upper edges of top marks.

If there is at least 2’ visible vertical gap between the marks the value  is defined as angle between the upper edge of the front and lower edge of the rear mark. In this case instead of H, height of the base of the rear mark or height of the lower edge of the rear dayboard must be used in the formula.

On night leading lines  is defined as angle between the centres of the leading lights. For gas discharge light sources with height up to 1,0 m  is defined as height between upper edge of front and lower edge of rear light source. When using gas discharge tubes forming vertical lines on the whole height of the daymarks  is measured between the upper ends of the tubes of front and rear mark.

* Minimal acceptable vertical angle  for a day leading line  = 2’ (0.58 mrad).

3’ (0.87 mrad) <  < 14’ (4.1 mrad) can be considered most favourable values of vertical angle  in the useful segment of a day leading line.

Minimal acceptable vertical angle for a night leading line  = 4.12’ (1.2 mrad).

Minimal acceptable vertical angle depends on illumination in the observer’s eye E… (IALA formulae)

…

* If vertical angles are less than calculated (in the lALA formulae), horizontal angles will be different from the values used in initial calculations of the distance between the leading marks and the following calculation of the height of the rear mark.

In these cases it may be necessary to recalculate the elements of the leading line with the new value of  that correspond to the values of .

The value of horizontal angle in the new calculations depending on the type (appearance) of the leading line and the vertical angles  is taken:

- for a day leading line with daymarks with finite dimensions

| , ‘ | 2 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 50 | 60 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| , ‘ | 1.0 | 1.0 | 1.0 | 1.0 | 1.2 | 1.4 | 1.6 | 1.8 | 2.0 | 2.3 | 2.6 |

Table 2.

- for a point day leading line

| , ‘ | 2 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 50 | 60 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| , ‘ | 1.5 | 1.5 | 1.5 | 1.5 | 1.7 | 1.9 | 2.0 | 2.2 | 2.4 | 2.7 | 3.0 |

Table 3.

If the vertical angle of a day leading line is less than 2’ (0.58 mrad), then the horizontal angle has to be taken 3,0’ (0.87 mrad).

For night leading lines serving channels and fairways with width 2B ≤ 200 m horizontal angle depending on the vertical angle is calculated with the formulae: … (the IALA formulae)

…

In any case vertical angle for a night leading line calculated by the heights of lights must not less than min calculated by the illumination in the observer’s eye.

If vertical angle of a night leading line < 4’ and there is no possibility to increase it, as an exemption, the calculated horizontal angle  has to be taken from the table below, but discernibility of the lights and off-axis distances have to be sailing checked in different meteorological conditions and approved in case of satisfactory results.

| , ‘ | 3 | 2 | 1 |
| --- | --- | --- | --- |
| , ‘ | 2.8 | 3.6 | 4.0 |

Table 4.

# Shape of dayboards

Rectangular of cylindrical shape is considered the best for leading marks as they provide sharp contour of silhouette and the latest moment when the mark is visible as a point. For leading lines with short working distance dayboards with trapezoidal of triangular shape can be used. The moment when such dayboards become visible as points come more quickly than for dayboards with rectangular shape. However, in case of observing the leading marks at the background of city buildings trapezoidal or triangular shape are better recognizable. The worst shape for daymarks is square.

As a rule the shape of front and rear marks is taken the same. In exceptional cases, when conditions at the location of placement prescribe, use of dayboards with different shapes is allowed.

Topmarks with the shape of a rhomb, circle, triangle or other geometrical shape can be placed at top of both front and rear marks to provide better recognition of the leading line.

Selection of colour of leading marks is done according to p 51 – 55.

It is recommended to paint a vertical stripe in the whole height of the dayboard. The width of the vertical stripe is taken 0.25 times the width of the dayboard. On white dayboards the stripe is black or white, on black and red dayboards the stripe is white or yellow.

To provide best illumination conditions the planks of the dayboard are placed at the angle of 10º - 20º backwards from the direction of the useful segment of the leading line.

Using marks with cylindrical dayboards provides even illumination of the dayboard by sun during most of the daytime.

# setting up of leading lines on site

Initial data for setting up a leading line are:

-	cartesian coordinates of one of the marks

-	direction angle of the axis of the leading line

-	heights of the locations of the front and rear marks

Placing of leading marks is realized in the following order:

-	one of the marks is placed on its location according to the given coordinates using one of geodetic means with obligatory control;

-	based on opposite angles (?, примычные углы) (at least two) the axis of the leading line is marked down. Determining the direction of a leading line by coordinates of the leading marks with calculation of reverse geodetic problem is not allowed;

-	using the direction of the leading line and the distance between the marks the location of the other mark is determined and the placement of it is done.

During the process of placement of leading marks the actual location of the axis of the leading line in relation to edges of the channel, limits of the fairway or location of the dangers has to be checked. One of methods of checking is placing buoys or spar buoys in the control points in the centre line of the channel, fairway or safe passage between the shallows and check the position of the axis of the designed leading line in relation to these points.

The final locations for erection of the leading marks are fixed with the main centre points and additional (offset) centre points at the distance of 20-40 m from the place of erection of the leading marks. The offset centre points have to stay reliably in their position during all the construction period and be used for checking when the new marks are taken in use. Types of the centre points are selected according to local conditions.

All the marks are connected to geodetic network but for offset centre points opposite angles (?, примычные углы) and distances to the main centre points are measured for latter checking of placement of the marks and the direction of the axis of the leading line.

With placement of each of the marks a scheme is compiled with both main centre and the offset centres shown.

# checking of leading line

After finishing construction works obligatory check of the leading line takes place. Control survey and sweeping/trawling of the useful segment, but also determining of final coordinates and the direction of the leading line are carried out. Final direction of the leading line is calculated taking into account the convergence of meridians for the middle of the useful segment of the leading line.

Actual off-axis distances and position of the visor curves in relation to the dangers to navigation are checked. Checking is carried out at sea at both day time and night time.

The idea of checking is in sailing across the zone of leading the line repeatedly and fixing the position of the vessel at the moments when shifting of the centres of the lights or vertical axes of the daymarks off the line is observed.

Survey lines are made perpendicular to the axis of the useful segment of the leading line with spacing of 1-2 km with obligatory checking of off-axis distance PK at the end of the useful segment DK. If the useful segment is shorter than 2 km, the lines are made with spacing of 0.25 km.

Figure 3. Checking of a leading line

Figure 4. The location of the light zoomed in. Location of the light is shown with red circle.

Mean root square of positioning of the vessel must be 10-12 times less than off-axis distances.

The survey lines are drawn on a plan with centre line of the leading line, edges of the channel or dangers to navigation limiting the room. The locations of the moments of observing opening of the line are drawn on the plan, averaged and connected with a smooth curve that is the visor curve of the day leading line. Similarly by the locations of opening of the lights visor curve for night leading line is constructed.

Actual off axis distances are determined as perpendicular distances from the centre line of the leading line to the visor curves.

Results of the checking are presented as a graph of vertical and horizontal angles and off-axis distances for day and night leading line (figure 3).

These graphs are attached to report and are is used for estimating the quality of the leading line.

# order of calculation and some recommendations on calculation and installation of leading lines

* Calculation and installation of a leading line include the following works:

- collecting cartographic and written materials on the area of the location of the leading line (charts, survey and topographic sheets, pilot books etc.);

- initial calculations of the elements of the leading line;

- reconnaissance and sailing through of the area of the location of the leading line;

- additional works, if necessary (survey, sweeping, levelling);

- final calculations of the elements of the leading line and selecting the lanterns;

- layout of the marks on the locations;

- installation of temporary marks and checking of the leading line;

- construction of the leading marks and installation of the lanterns;

- checking of the leading line;

- compiling of the documents for report, preparation of data for Notices to Mariners. Employing the leading line.

..

* Leading marks are recommended to be located in open places so that they won’t be hidden behind irregularities of the landscape like forest, hills etc. The best observing conditions are achieved when the marks are designed on the background of the sky, which is especially important for the leading lines that are located on coast and observed significant time of the day against the sun.

Background of terrain at a significant distance can also be considered a favourable background.

…

* Where conditions in the area allow it is advisable to design leading marks at both ends of the line, especially if the length of the useful segment is longer than 3 NM.

…

* If due to local conditions vertical angle of the leading line becomes significantly large (more than 60’), it is recommended to install a third, intermediate leading mark between the front and the rear marks.

Off-axis distances of such a leading line are calculated by the distance between the extreme marks and by the horizontal angle that corresponds to the new decreased vertical angle.

* Height of the marks, and accordingly the vertical angles and off-axis distances of a leading line that are designed for use by vessels with different bridge heights (height of the eye of an observer) must be calculated for 5 m height of the observer and checked for the real heights.

If checking shows that the off-axis distances calculated for the real heights of the observer are larger than accepted, elements of the leading line have to be recalculated (heights of the marks or distance between the marks changed).

* To avoid building large and expensive leading marks for ships with low bridge height, as an exception, it is acceptable to use leading lines with vertical angle less than 2’ (0,58 mrad) on a small length of the useful segment. Decisive factor that determines possibility of using such a leading line is value of off-axis distances P calculated for the section of the leading line where vertical angle is out of normal limits.

* Calculation of off-axis distances for such sections of useful segment is made:

- for day leading line – on the condition of observing daymarks separated (one side of one daymark on the vertical line with the other side of the other daymark - Pärtel) for the section where the vertical angle ≤2’ (0.58 mrad), up to the near end of the useful segment; length of such section is taken from the graph;

- for night leading lines – on the condition that horizontal angle for detection is 3 – 4’.

Besides that, usability use such a leading line is decided case by case based on the location and length of the section where vertical angles are out of normal limits, intensity of traffic using the leading line and expediency of building large and expensive marks.

Final decision of acceptable vertical and horizontal angles of such a leading line must be done after sailing check of the leading line.

* For leading lines with small vertical angles (less than 4.5’ (1.31 mrad)) and lights with the same colour synchronising of the lights is not recommended. Synchronising such lights significantly hampers detecting side of deviation from the axis of the leading line.

* Satisfying the condition that upper edge of front daymark must be projected on the background of rear daymark for the whole length of the useful segment is not necessary. Small vertical gap between the daymarks does not degrade sensitivity of the leading line.

* Day leading line calculated with the given formulae and tables 2 and 3 are more sensitive than night leading line (calculated with IALA formulae), therefore first the night leading line has to be calculated and then checked if it corresponds to the day conditions.

* Calculation of a lighted day leading lines is the same as for a night leading line.

Sensitivity of a lighted day leading line is near to the sensitivity of a night leading line.

…

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain