A number of queries and suggestions arose when this Guideline was being reformatted to the new IALA image, as shown in the comments throughout the document. The ENG Committee is requested to respond to these queries and suggestions and submit to the Secretariat to enable completion of the reformatting. ENG5-11.2.13 Suggestions are entered at ENG5 by WG1, awaiting for “Accepting changes” IALA Guideline

1073

Conspicuity of AtoN lights at night

Edition 1.0

June 2011

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

1.	INTRODUCTION	6

2.	PURPOSE	6

3.	SCOPE	7

4.	DEFINITION OF THE VISUAL TASK	7

4.1.	Detection, Recognition and Identification	7

5.	DISCUSSION OF CONSPICUITY FACTORS	7

5.1.	Introduction to Conspicuity Factors	7

5.2.	The Observer	8

5.2.1.	Adaptation (Eye State)	8

5.2.2.	Knowledge/Experience	9

5.2.3.	Colour	9

5.2.4.	Colour Contrast	9

5.2.5.	Illuminance Ratio	11

5.2.6.	Illuminance (Int/Dist2)	11

5.2.7.	Angular Subtense	11

5.3.	The Atmosphere	12

5.3.1.	Distance	13

5.4.	The AtoN Light	14

5.4.1.	Spectral Properties	14

5.4.2.	Intensity	14

5.4.3.	Rhythmic Character and Flash Profile	15

5.4.4.	Size of Light Source	16

5.4.5.	Shape of Light Source	17

5.4.6.	Floodlighting	18

5.4.7.	Contour Lighting	19

5.4.8.	Movement and Apparent Movement (including sequencing)	19

5.4.9.	Synchronisation	19

5.5.	The Background	20

5.5.1.	General Background (Luminance)	20

5.5.2.	Rival Lights (Intensity)	20

5.5.3.	Spectral Properties of Background Lighting	21

6.	A SUMMARY OF METHODS FOR IMPROVING THE CONSPICUITY OF AtoN LIGHTS	21

6.1.	Current Methods	21

6.2.	Designing an AtoN Light using Traditional Methods	22

6.3.	Methods for Improving the Conspicuity of AtoN Lights	23

6.3.1.	Changing Colours to Improve Colour Contrast	23

6.3.2.	Rapidly Alternating Colours	24

6.3.3.	Increasing the intensity to improve conspicuity	24

6.3.4.	Light Source Shape	24

6.3.5.	Flash Profile and Rhythmic Character	24

6.3.6.	Synchronisation	24

6.3.7.	Sequencing	24

7.	WAY FORWARD	25

8.	ACRONYMS AND DEFINITIONS	25

8.1.	Acronyms	25

8.2.	Definitions	26

9.	REFERENCES	27

ANNEX A	PRESENTATIONS MADE TO THE JAPAN COAST GUARD EXPERT MEETING (NOVEMBER 2008)	28

ANNEX B	CONSPICUITY OF AtoN – EXTENDED LIGHT SOURCES	31

ANNEX C	REPORT OF PWM FLICKERING LIGHT EXPERIMENT IN ACTUAL SEAS	46

List of Tables

Table 1	Detection and thresholds background for signal lights	34

Table 2	Conspicuity Index for extended sources. (Detection Probability = 0.75)	41

Table 3	Conspicuity Index Least Square Fits	42

Table 4	Angular Size of extended sources (Minutes of Arc)	43

Table 5	Physical size of extended sources (Feet)	43

Table 6	Total data on visibility (discrimination)	48

Table 7	Total data on unpleasantness	49

Table 8	Results and categories on comments	50

List of Figures

Figure 1	Conspicuity factors	9

Figure 2	Daytime (Photopic) and Dark Adapted (Scotopic) Eye Response	10

Figure 3	CIE 1931 Chromaticity Diagram	11

Figure 4	Diagram explaining Angular Subtense	12

Figure 5	Required Intensity Wavelength Dependent, according to Ångström, for Photopic Vision	13

Figure 6	Required Intensity Wavelength Dependent, according to Ångström, for Scotopic Vision	14

Figure 7	Comparison of Red, Green and White Flickering Lights at Japan Coast Guard	16

Figure 8	Viewing an object from different angles	17

Figure 9	Pictures of the arc-pointer device installed in the Port of Barcelona	19

Figure 10	Lighthouse Tower Floodlit in different Colours	20

Figure 11	Background luminance due to town lighting	21

Figure 12	Rival lights caused by individual point sources	21

Figure 13	Flow Chart to help the design of AtoN lights	24

Figure 14	Observer view of display in lab experiment	33

Figure 15	Schematic of TIR lamp	35

Figure 16	View of extended source in New London Harbour at a distance of 1.0 nautical miles	35

Figure 17	Average reaction times for fixed lights.	36

Figure 18	Average reaction times of fixed and flashing lights.	37

Figure 19	Effect of background lights on fixed and flashing lights	38

Figure 20	Cumulative per cent detected for fixed lights	38

Figure 21	Cumulative per cent detected for flashing lights	39

Figure 22	Per cent detected for field experiment	39

Figure 23	Conspicuity indexes for fixed and flashing lights.	41

Figure 24	Lighthouse locations and surrounding areas	47

Figure 25	Evaluation of visibility (discrimination)	48

Figure 26	Visibility (discrimination) per distance	48

Figure 27	Evaluation of unpleasantness of the light	49

Figure 28	Evaluation of conspicuity	50

Figure 29	Conspicuity per distance	50

List of Photographs

Photograph 1	Funadamari Breakwater Light	47

Photograph 2	View of the Dock Light from around Yokohama Kita Suitei harbour light	52

Photograph 3	View of the Dock Light from around Yokohama Kita Suitei harbour light - Taken at intervals between flashes	52

Photograph 4	View of the Dock Light from Izuta Cho offshore - Taken during flash	53

Photograph 5	View from the direction of DaikokuOhashiiew from the direction of DaikokuOhashi - Taken at intervals between flashes)	53

Photograph 6	View from a distance of 0.4 nautical miles - Taken during flash	54

Photograph 7	View of the Dock Light from a distance of 0.6 nautical miles - Taken at intervals between flashes)	54

Photograph 8	Bay Bridge -Taken during flash	55

Photograph 9	Dock Light from a distance of 1.2 nautical miles - Taken at intervals between flashes	55

# INTRODUCTION

In the past, marine Aid to Navigation (AtoN) lights were designed largely as landfall lights for mariners and intended to be viewed at a distance against a dark background and so were observed at the threshold of vision.  Consequently, older IALA documentation tended to concentrate on the luminous range at the threshold of vision, and on the geographical range.  As a result, most AtoN lights today are used as visual confirmation of position at ranges much less than the published nominal range. Furthermore, the background against which an AtoN light is viewed is more likely to be illuminated with artificial lighting.

With the proliferation of built up shorelines and consequent increase in light pollution, the mariner often has difficulty in detecting and identifying AtoN lights against a background scene of general lighting and individual bright light sources.  Features such as town or street lighting, harbour area floodlighting, architectural lighting and lit signage may look enchanting but can cause serious problems for a mariner trying to identify an important AtoN light.

This guideline looks at the factors affecting the usefulness of a marine AtoN light and ways to improve its effectiveness by increasing conspicuity.

Conspicuity is a complex subject.  Considering the many different types of AtoN signals, and the even higher number of possible backgrounds against which the AtoN signal may be viewed, the number of signal and background possibilities is huge.  There are a limited number of conspicuity studies that have examined the impact of changing one signal characteristic while the signal is viewed against a simple background.  However, considering the huge number of possible AtoN signal/background combinations, these studies barely scratch the surface.  Some of the information in this Guideline is qualitative and/or anecdotal.  It documents our current, incomplete understanding of conspicuity factors.  Quantitative information is desired but for the most part, does not exist.  Conspicuity research that can be used to develop quantitative models is encouraged.  The long-term goal is the development of a comprehensive model that can be used to quantify the conspicuity of any AtoN signal against any background.

# PURPOSE

This Guideline describes factors affecting conspicuity and methods of increasing the conspicuity of a marine AtoN light, or lights, when viewed against a background of general lighting or rival lights.  At this time, it is not possible to give models, or design parameters, for designing an AtoN to have a sufficient conspicuity under defined conditions.  This is because the state of scientific knowledge concerning conspicuity and the human eye is not complete today and the number of variables precludes quick development of models based on experiment.  Indeed, there is no recognised scientific measure or scale for conspicuity.

Consequently, this Guideline is a review of the current state of knowledge, with guidance and rules-of-thumb where possible, based on empirical evidence.  A number of studies have been conducted by IALA members, and these can be found on the IALA Wiki (see references).

# SCOPE

The scope of this Guideline is AtoN lights viewed at night against a background scene of background interfering lighting.  It addresses the conspicuity of a marine signal light when viewed against a background of general lighting or rival lights.

The Guideline does not consider AtoN lights viewed at threshold of perception; this is covered in IALA Recommendation E-200-2 [6].  It does not address daytime lights or daymarks.  For guidance on daytime lights refer to IALA Recommendation E-200-2 [6] and for guidance on Daymarks refer to IALA Guideline 1094 [ADD REFERENCE].

The subjects of geographical range and obstructions that may prevent the mariner from seeing an AtoN light, for example high waves, are not dealt with in this guideline and neither are lights moving as a consequence of being placed on a floating platform.  The IALA NAVGUIDE [22] and the IALA Guideline No. 1065 on Vertical Divergence [25] give further guidance on such topics.

# DEFINITION OF THE VISUAL TASK

There are three concepts to consider when judging conspicuity; these are detection, recognition and identification. Typically, an AtoN light may be detected very quickly, after a further period it will be recognised as an AtoN, but not until its rhythmic character has been fully understood will the AtoN be identified.

The visual task of the mariner is to reliably identify an AtoN light.  The time taken to undertake this task should be as short as possible.  By increasing the conspicuity of an AtoN, the time taken to complete the visual task will be reduced.

The following examples illustrate examples of these concepts:

A green lateral mark or buoy light – the green light may be quickly detected against a background of yellow street lighting, the fact that it is flashing green enables it to be recognised as an AtoN, but not until the rhythmic character of the mark has been viewed more than once will the exact AtoN be identified.

A bright strobe light exhibited by an AtoN light at the beginning of its rhythmic character – this will make the light conspicuous for detection.  In other words, it will draw the observer’s eye to where the AtoN character may be viewed and, in time, recognised and identified.

A unique lighthouse tower, prominent on the shoreline, floodlit with a magenta light, this colour also being unique in the area – once detected, the observer will very quickly recognise the object in view and identify it as a particular lighthouse.

# A SUMMARY OF METHODS FOR IMPROVING THE CONSPICUITY OF AtoN LIGHTS

## Current Methods

The conspicuity of a marine AtoN light is usually only improved after complaints from the mariner and it is done on a case by case basis.  The usual scenario is that a mariner complains that he or she has difficulty in detecting or identifying a particular light.  The AtoN provider then needs to investigate the complaint and this is done by communicating with the person lodging the complaint and asking where the problem occurred, whether it frequently occurs, if so at what position.  The AtoN provider should also ascertain what the exact nature of the problem is and whether local conditions (e.g. weather, height of observer or background) affect the conspicuity.

If the complaint is deemed justified, the provider would usually prompt an inspection or review of the AtoN light concerned to ensure that it was performing satisfactorily or whether there was a fault with the AtoN light apparatus.  During the inspection, causes other than the performance of the AtoN can be investigated but it is often difficult to pinpoint a problem when positioned at the AtoN itself.  Following an inspection of the AtoN, if the problem is still unresolved, the provider should carry out a viewing trial of the AtoN light in question from the area where the difficulty occurred.  If possible, it is a good idea to invite the person that prompted the complaint along to the viewing trial.

Once the exact nature of the conspicuity problem has been identified, the AtoN provider may look at the list of options given in this and other relevant IALA documents in order to improve conspicuity.  This should be done whilst taking the navigational and operational requirements into account.

Many conspicuity problems arise, not from new AtoN installations, but from shoreline development.  A new housing estate, retail park or industrial complex usually brings with it a plethora of lighting that can interfere with an existing AtoN.  Quite often, liaison with the planners, builders, owners or occupiers of such sites can be fruitful before, during and after development.  A slight change in the direction, position or luminaire design of a few lights can make a significant difference in the performance of an AtoN light from the viewpoint of the mariner.

Further information on improving AtoN light conspicuity can also be found in IALA Guideline No. 1051 on the Provision of Aids to Navigation in Built-up Areas, which, for example, was used to solve problems of conspicuity at Kinnaird Head in Scotland [18].  The heart of the problem lay with a new shopping development and associated car park lights, which were placed on high poles in line with the existing AtoN light.  After discussions with the local authority, the poles were reduced in height and the conspicuity of the AtoN light restored.

## Designing an AtoN Light using STANDARD Methods

The IALA E-200 Series of Recommendations E-200[3] gives design guidance for marine AtoN lights.  When designing AtoN lights, the starting point is usually the navigational requirement, which can be viewed as the requirements of the user.  This will specify such parameters as colour, nominal range, rhythmic character etc., and these are usually relatively inflexible.

There may well be additional concerns at some locations about factors such as prevailing visibility or background lighting.  These are addressed in IALA Recommendation E-200-2[6] using standard models such as Allard’s Law and a three-step background light model: no background light; minor background light; and substantial background light.  IALA Recommendation E-200-2[6] may be used therefore to define the operational or system requirements.

A flowchart is provided in IALA Recommendation E-200-0[4] to help the design of AtoN lights.

However, the IALA E-200 Series of Recommendations does not give guidance on many conspicuity issues such as rival lights and colour contrast.  It gives details on how range is affected by visibility but not how the spectral distribution of the light is affected by the atmosphere.  It also gives guidance on the effects of background luminance on range but assumes AtoN light and background are the same colour.  Therefore, its use is somewhat limited when trying to solve conspicuity problems.

## Methods for Improving the Conspicuity of AtoN Lights

If it is not possible to avoid rival lights or background illumination, the following methods to improve conspicuity may be utilised.

### Changing Colours to Improve Colour Contrast

If possible, the colour of the AtoN light should be made different to the colour of the background lighting.  For example, changing an incandescent filament lamp to a high colour temperature discharge lamp would improve the contrast between the AtoN and a background of sodium street lighting.

See Section 6.2.4 for more details.

### Rapidly Alternating Colours

To make a light conspicuous at short range, rapidly alternating colours have proved to be effective.  An example of this application is the emergency wreck-marking buoy (EWMB), which has an alternating blue and yellow light.

See Section 6.4.1 for more details.

### Increasing the intensity to improve conspicuity

#### For General Background Luminance

E200-2 recommends tenfold increase in AtoN light intensity for minor background lighting and a hundredfold increase if the background lighting is significant.

See Section 6.5.1 for more details.

#### For Rival Lights

Allard’s Law may be used to calculate the illuminance at the eye of the observer for any given light, over a given distance, for a given atmospheric transmissivity.  When comparing AtoN lights to rival lights of the same colour, the minimum illuminance ratio required for an observer to see the AtoN light against a rival light is approximately 4:1.

See Section 6.5.2 for more details.

### Light Source Shape

For short range navigation, illuminated areas, illuminated contours and extended light sources can be used to provide an identifiable shape.  The shape may be used as a standalone AtoN, or used to point towards an AtoN [10].

Arrangements of point light sources can be used in a similar way.  The use of sequenced lights within an arrangement or shape is particularly effective as it can provide apparent movement and/or give direction.

See Sections 6.4.4 and 6.4.5 for more information.

### Flash Profile and Rhythmic Character

At high levels of observer illuminance, quickly repeated short flashes are generally more conspicuous than longer flashes with longer eclipse times between them.

A rectangular flash shape, i.e. one with a fast rise and fall time, is generally more conspicuous than a flash shape with a slow rise and fall time (e.g. Gaussian) [12].

At relatively short ranges, where background lighting and rival light interference are high, flickering the AtoN light within the flash profile at a frequency of around 10 Hz is effective [1].  This flickering technique yields better results for red and white AtoN lights than for green.

It is possible to improve conspicuity by utilising a low-level fixed light during the eclipse period of a character.

See Section 6.4.3 for more details.

### Synchronisation

Synchronising several AtoN lights can increase conspicuity considerably, particularly when viewed from a distance.  This technique is particularly suitable for leading lights.

See Section 6.4.8 for more details.

### Sequencing

Apart from sequencing lights in an arrangement or shape that are viewed together, sequencing several AtoN lights over a wider area can also increase conspicuity as it provides apparent movement and indicates direction as well as relative distance.  However, for sequencing to work satisfactorily several lights need to be in the view of the observer and this may mean increasing the number of lights in a given area for sequencing to be effective.  Further information can be obtained from the IALA Guideline 1116 on Selection of Rhythmic Character and Synchronisation of Lights for Aids-to-Navigation [14].

# DISCUSSION OF CONSPICUITY FACTORS

## Introduction to Conspicuity Factors

When considering the conspicuity of AtoN lights within the scope of this document, there are a number of factors which need to be taken into consideration.  Many of these factors cannot be considered in isolation because changes in one factor may affect the conditions of another factor.

Each of these ‘Conspicuity Factors’ are shown in Figure 1 and are grouped into types (Observer, Atmosphere, AtoN Light and Background).

The following paragraphs will describe each of these factors, or combination of factors, where they affect one another.

| The Observer | The Atmosphere | The AtoN Light | The Background |
| --- | --- | --- | --- |
| Adaptation Knowledge/Experience Colour Colour Contrast Illuminance Ratio Illuminance (Int/Dist2) Angular Subtense | Visibility (Atmospheric Scatter) Distance | Spectral Properties Intensity Rhythmic Character and Flash Profile Size of Light Source Shape of Light Source Floodlighting Contour Lighting Movement and Apparent Movement Synchronisation | General Background (Luminance) Rival Lights (Intensity) Spectral Properties |

Conspicuity factors

## The Observer

### Adaptation (Eye State)

It is known that the performance of the eye varies with the ambient light level to which it is exposed.  In full daylight the normal observer has full colour vision in direct view (photopic vision) and is not very sensitive to movement (or apparent movement) in peripheral view.  After twenty minutes or so in full darkness, the eye adapts to the dark and becomes very sensitive.  In this very sensitive state at low light levels, the colour sensing elements of direct vision are not active and objects appear grey.  The most sensitive region of the eye in this (scotopic) state is the periphery, about 10 degrees away from direct vision.  In scotopic vision, the peak spectral response of the eye shifts towards shorter wavelengths.  This ‘blue-shift’ causes blue-white lights to be more pronounced at night when compared to yellow-white lights.  Therefore, changing an AtoN light to a higher colour temperature will increase its conspicuity under scotopic or mesopic conditions, even though its luminous intensity remains the same [12].

Daytime (Photopic) and Dark Adapted (Scotopic) Eye Response

At low levels of light (e.g. twilight), the eye reaches a state midway between photopic and scotopic that exhibits elements of both states (mesopic).  The mariner at night on the low-lit bridge of a ship is probably in mesopic vision, where there is limited direct colour vision and enhanced sensitivity in peripheral vision.  At low levels of incident light, when the observer strains to see an AtoN, the observer’s visual response is much slower than at higher levels of light.  There is also much more chance of colour confusion, especially for flashing lights, and especially between green, blue and white.  A longer flash duration will reduce colour confusion at these low levels  [12].

### Knowledge/Experience

When a mariner has prior knowledge or experience of a particular port entry or navigational channel, he or she will be able to discern the relevant AtoN lights more easily.  AtoN that appear conspicuous and obvious to an experienced mariner can often appear indistinct or be missed altogether by an inexperienced one.  This is due to the expected field of view. An experienced mariner will have a narrower field of view where he expects to see the AtoN which therefore takes less effort and time to detect, recognise and react to an AtoN.  Whereas an inexperienced mariner will have a much larger initial field of view within which an AtoN will be more difficult to detect, recognise and identify.

### Colour

Colour perception is a function of the human visual system and not a property of the light.  The spectrum emitted by a light source, seen by the eye, is translated into colour by the tristimulus function of the eye and brain.  The standard human colour observer is usually represented by a two-dimensional colour chart such as that developed by CIE in 1931 as shown in Figure 3.

Despite the science of colour measurement and representation, the human colour observer can be easily fooled. Lights that are dim, small in size, flashing or seen against another background colour can be easily mistaken for a light of a different colour.  This is less true of red than any other colour.  Generally, the purer (less saturated) the light colour, the less chance there is of confusion.

### Colour Contrast

The ability of the human observer to detect slight differences in colour is well known.  For surface colour, a slight difference in hue or chroma can be easily detected by the average person.  However, when it comes to discerning differences in emitted light, knowledge is less well documented.

At very low levels of illuminance, when an observer can barely detect a light, the colour of the light and the duration of a flash of light are difficult to discern.  As illuminance levels at the eye of the observer increase, colour recognition and the duration of a flash become clearer.  But, the appearance of a point source of light is more difficult to predict than that of an area of light.

When an AtoN light is seen against a background luminance, it is more easily discernible when the colours of background light and AtoN light are different.  The greater the difference, as shown on the chromaticity diagram, the easier it is to detect one against the other.

CIE 1931 Chromaticity Diagram

#### Changing Colours to Improve Colour Contrast

Colour change is often limited with AtoN lights because navigational requirements dictate the colour of the AtoN light but most background and rival lights are white.  The usual problem is therefore trying to see a white light against a white background.  Since many street lights are sodium, they tend to be yellowy-white.  The change of AtoN light source from incandescent filament lamp to a light source of high colour-temperature (e.g. metal halide or white LED) often provides a significant colour contrast to the background whilst still maintaining the correct AtoN colour.

There are two exceptions to this:

when the background is for example a marshalling yard or football stadium where high colour temperature floodlights are used;

when the AtoN light is yellow (for example, a special mark) and the background is low-pressure sodium light.

#### Differences between observers

When a signal light colour is chosen to stand out against the background, the increase in conspicuity experienced by normal observers may not be experienced by colour deficient observers.  As an example, for approximately 5% of male observers a red light will appear no different than a yellow light.  So choosing a red AtoN light for use against a background of yellow lights will not achieve the desired conspicuity results for this portion of the population.  It is important to note that people in command of commercial vessels at sea undergo regular health checks including colour vision tests.  However, amateur sailors may go to sea without undergoing any such screening.

For information on colour deficient observers, including lines of colour confusion, see IALA Recommendation E-200-1.  Blue-yellow confusion is very rare (1:63000), reinforcing the advantage of blue-white lights against a yellowy sodium background.

Age affects the performance of an observer’s vision.  Older observers have a reduced sensitivity to blue because of degradation of the cornea.  Typically, a 50-year-old observer has between 10% and 20% of sensitivity to blue compared with a 20-year-old observer.  This clearly has implications on the conspicuity of blue lights.  In observers of all age groups, tiredness adversely affects the performance of the visual system.

### Illuminance Ratio

When an AtoN light is brighter than a neighbouring light, it is more conspicuous.  The ratio required for an observer to notice a difference in eye illuminance between two light sources is roughly 4:1 [13].  However, when dealing with a point source of light to be just discernible against a background luminance, the relationship is approximately to the power of two [13].  For example, a three-fold increase in background luminance should be matched with a nine-fold increase in AtoN light intensity.

### Illuminance

The amount of illuminance an AtoN light casts upon the eye of the observer depends upon its intensity, the distance of AtoN from the observer and the state of the atmosphere. The minimum discernible illuminance at the eye of a photopic observer is often quoted as 0.05 microlux (for a dark-adapted observer in scotopic vision however, it can be as low as 0.0015microlux).  At these levels, i.e. at the threshold of visual perception, the chances of seeing a light are little better than 50%.  The minimum recommended illuminance level for an AtoN light in darkness is 0.2 microlux, based on international agreement in 1933 [16].  This is the value from which the nominal range figure is calculated.  At this level, the colour and rhythmic character of an AtoN can be recognised with confidence.

It should however be noted that with high levels of background lighting, a higher value of illuminance is required in order to see the AtoN light.  At these higher levels, the human visual system behaves quite differently than at low illuminance levels.  Phenomena such as short flashes (strobes), fast repeating flashes and flicker become more conspicuous as illuminance levels increase.

### Angular Subtense

The subtense angle of the target is not considered a human factor but a property of the geometry of size and distance.  However, the visual acuity of the eye determines the degree of detection and recognition of an object that has a particular subtense angle.

Viewed from far away, most AtoN lights are point sources and therefore have no discernible size.  As the observer gets closer to the AtoN light, the size increases so that the size and shape of the light source become noticeable.  If the angle subtending the eye, in other words the angle at the eye from one side of the viewed object to the other side, is less than one minute of arc, it can be said to be a point source.

Diagram explaining Angular Subtense

As a rule of thumb, the angular subtense required to recognise a simple shape is three minutes of arc.  In order to recognise complex shapes, such as letters, ten minutes of arc is the minimum angular subtense required.  These minimum subtense angles may vary with luminance contrast and colour contrast between the AtoN light and the background.  Illuminated signs and objects may also cause glare when the illuminance at the eye of the observer is high.  This may cause the shape of the lit area to become indistinct [24].

## The Atmosphere

Visibility is usually thought of as meteorological visibility, which is defined as:

the greatest distance at which a black object of suitable dimensions can be seen and recognized by day against the horizon sky [17].

For night time viewing:

The greatest distance at which a black object of suitable dimensions could be seen and recognized if the general illumination were raised to daylight level [17].

The state of the atmosphere will determine the greatest distance at which a light of given intensity can be seen at night.  This distance is known as the ‘luminous range’ of the light.  The method by which luminous range is determined is Allard’s law, which takes into account the intensity of the light, the distance of the observer from that light and the visibility.

Another way of defining the state of the atmosphere is by its ‘transmissivity’, which can be thought of as the throughput of light for one nautical mile of atmosphere.  Transmissivity is usually given as a factor, e.g. 0.74, which means that light exiting one nautical mile of atmosphere will be 74% of the light input after the inverse square law has been applied [6].  Both meteorological visibility and transmissivity assume that the atmosphere is spectrally neutral.  In other words, all wavelengths of visible light are affected equally.  Unfortunately, the impact of the atmosphere is somewhat more complicated than this and will often affect wavelengths within the visible spectrum unequally.

The two phenomena traditionally used to describe the way the atmosphere scatters light are Rayleigh scatter and Mie scatter.  These are traditionally used to describe how sunlight is affected.  A model that more completely describes how light at sea level is affected is that of Ångström.  This model includes the effects of Rayleigh and Mie scattering as well as the effect of aerosol particulates such as salt.

Generally, Rayleigh scattering occurs when visibility is moderate to good where light is scattered preferentially by wavelength to the fourth power of the wavelength.  Therefore, a blue light will be scattered more, and will have less luminous range, than a red light of the same intensity.  Mie scattering occurs during conditions of poor visibility when water droplets within the atmosphere are larger.  Mie scattering is spectrally neutral.  The Ångström model combines both Mie and Rayleigh and includes other factors for aerosols present in the atmosphere close to the sea.

The chart below shows how required intensity for a given luminous range can vary with wavelength.  It is valid for a photopic (daytime) observer (see Section 5.2.1).  This chart can be considered during the design stage of implementing an AtoN light, however, luminous range calculations should be carried out using IALA Recommendation E-200-2 [6].

Required Intensity Wavelength Dependent, according to Ångström, for Photopic Vision

For an observer in scotopic vision, which has a different spectral response to photopic.  These required intensity values will change, as seen below in Figure 6.

Required Intensity Wavelength Dependent, according to Ångström, for Scotopic Vision

As blue light passes though the atmosphere it tends to be selectively scattered and, as a result, blue signal lights consisting of a filament lamp and filter may appear purple or red when viewed at a distance.  So choosing a blue light, because blue is more conspicuous against a given background, may not be satisfactory if the blue light is intended to be viewed at a distance.

### Distance

The single most important factor that affects a mariner’s ability to detect an AtoN light signal against a background of general lighting and individual light sources is the observer’s distance from the AtoN light signal.  However, when analysing conspicuity it is not distance, per se, that is analysed but rather the impact of distance on other factors that in turn influence conspicuity.  Because distance ultimately affects conspicuity via so many other conspicuity factors, one must first look at the impact of distance on other conspicuity factors and then consider the impact of these other conspicuity factors on the observer’s ability to detect an AtoN signal.  Consider the following examples:

- Increasing the distance from an AtoN light signal decreases the signal’s illuminance upon the eye of the observer.  The illuminance is used to analyse conspicuity.

Increasing the distance from an AtoN signal increases the length of atmosphere that the light passes through before being viewed by the observer.  Because different wavelengths experience different atmospheric absorption and scattering, the distance affects the spectral distribution of the signal received by the observer.  The spectral distribution of the received signal is used to analyse conspicuity.

Increasing the distance from an AtoN signal decreases the signal’s angular subtense.  This, in turn, affects the observer’s ability to distinguish the shape of an extended light source or a floodlit object.  The target’s angular subtense and shape are used to analyse conspicuity.

Increasing the distance from an AtoN signal decreases the ability to detect the movement – or apparent movement – of the light source.  The movement – or apparent movement – is used to analyse conspicuity.

Decreasing the distance from an AtoN signal increases the illuminance contrast between the AtoN signal and the background lighting.  The illuminance contrast is used to analyse conspicuity.

In each of these examples, the identified factor – rather than distance – is used to analyse conspicuity.  Observation distance influences far too many factors to allow a direct link from distance to conspicuity.

## The AtoN Light

### Spectral Properties

The spectral properties of a light source are translated by the human visual system to colour; however, it is normal to associate a light source with a given colour and, for simplicity, this shall be the convention in this section.

AtoN lights are often restricted by their navigational requirements.  For example, a starboard hand buoy in Region B must exhibit a green light and this may limit the ability to enhance conspicuity by changing spectral distribution or colour.  However, improving the purity of the colour, for example, by using a light source with a narrow spectral distribution, will often improve conspicuity.

For white lights, changing the colour temperature of an AtoN light will often improve the conspicuity against a background of different colour temperature white lights.  An example is when a high colour temperature (blue-white) AtoN light, such as an LED buoy light, is viewed against a background of low colour temperature (yellow-white) sodium street lighting (see 6.2.4 Colour Contrast).

To make a light conspicuous at short range, rapidly changing colours have proved to be effective.  An example of this application is the emergency wreck-marking buoy (EWMB), which has an alternating blue and yellow light.

### Intensity

The intensity of an AtoN light determines its nominal range and its luminous range for a given visibility, as described in IALA Recommendation E-200-2 [6]. However, to make a light more conspicuous by increasing the intensity, that increase usually needs to be substantial, typically tenfold.  This is because the intensity required of a light to be perceived against a background luminance is proportional to the square of the background luminance (from Langmuir & Westendorp 1931 [13]). Guidance is given in IALA Recommendation E-200-2 [6] for required intensities at different levels of background lighting.

When considering a flashing light, effective intensity is the relevant parameter to use at threshold illuminance levels.  However, when considering short flashes at higher levels of illuminance, and this is usually the case when viewed against background luminance, the value of effective intensity is not proportionate to conspicuity. It has been found that the eye responds quicker to flashes with a higher level of illuminance of the eye, and more details can be found in IALA Recommendation E-200-4.

Excess intensity may make a signal light conspicuous but it can lead to glare at short ranges.  Glare can restrict the ability of the mariner to see other salient features or signals.  The whole zone of utilisation of the AtoN should be considered when deciding the intensity of a given signal light.

#### Increasing the intensity to improve conspicuity

The traditional method of dealing with poor conspicuity of an AtoN light is to increase its intensity.  Information on such methods, including the need to consider the effective intensity of a flash of light, can be found in IALA E-200 Series of Recommendations [3].  However, IALA Recommendation E-200-4 [8] also recognises the inappropriateness of the effective intensity concept when dealing with flashes above threshold of perception (e.g. when viewed against background lighting).

Typically, intensity needs to be increased by orders of magnitude to achieve any improvement in conspicuity against background lighting.  IALA Recommendation E-200-2 [6] recommends tenfold increase for minor background lighting and a hundredfold increase if the background lighting is substantial. The nominal range of the light may be increased significantly by such actions but the useful range improvement may be much less.  The relationship between intensity and range can be found in IALA Recommendation E-200-2 [6].

### Rhythmic Character and Flash Profile

The rhythmic character of an AtoN light is what distinguishes it from its surroundings and from other AtoN lights.  Generally, the rhythmic character of signal lights, such as Cardinal Marks, cannot be altered for operational reasons.  However, it may be possible to alter the timing of the rhythm, for example to increase the repetition rate of flashes within a quick flash to improve conspicuity.  At high levels of illuminance, quickly repeated short flashes are generally more conspicuous than longer flashes with longer eclipse times between them.  Care should be taken to ensure that the character is still suitable in conditions of poor visibility, when illuminance levels at the eye of the observer may be low and the visual response slows down as a result (see section 6.2.1 Adaptation).

The shape of the flash, or flash profile, is also important to conspicuity.  A rectangular flash shape, i.e. one with a fast rise and fall time, is more conspicuous at high illuminance levels than, say, a Gaussian flash shape (such as observed from a rotating lens system) [12].

Flickering the AtoN light within the flash profile at a frequency of around 10 Hz, is a technique that has had some success in Japan [1].  In situations where background luminance and rival light interference are high, flickering the AtoN light can greatly increase its conspicuity. However, this technique is only effective when illuminance levels at the eye of the observer are high and the frequency of the flicker is below the flicker fusion frequency.  At low levels of observer illuminance, the advantage of the flicker effect is lost and some observers fail to notice any flicker at all.

Japan Coast Guard experiments [19] with flickering light showed that red, green and white flickering light enhanced conspicuity and the lower the duty ratio within the flicker frequency, the higher the conspicuity becomes.  However, the degree of the conspicuity varies with colour.  The following figure shows the comparison of red, green and white flickering light when flicker frequency is fixed at 10 Hz.  The graph compares the duty cycle of the coloured lights flickering at 10 Hz with a relative conspicuity value.

Comparison of Red, Green and White Flickering Lights at Japan Coast Guard

A conclusion from these experiments was that at the duty ratio of 50% or less, the conspicuity of green flickering light is obviously lower than red and white flickering light.  Therefore, given other factors such as the Ångstrøm effect, the design of flickering light as aids to navigation required careful consideration.

An expert meeting of Japan Coast Guard in November 2008 [1] concluded:

‘That a flickering LED light is conspicuous and therefore has the possibility of becoming a new lighting method for marine aids to navigation however the flicker range is shorter than the nominal luminous range of the flickering light under certain conditions and thus the designing the flickering light should be done carefully.  Further study and research are needed for its practical application.’

A report on various trials undertaken by Japan Coast Guard is given on the IALA Wiki.

#### Choosing a Suitable Flash Profile

The following points may be of some assistance in improving the conspicuity of a flashing light.

Simple flash characteristics (e.g. a group flash four, Fl(4), light on an AtoN is more conspicuous against a background of fixed rival lights than an AtoN with a fixed light characteristic).

A flash profile with a sharp rise time (and fall-time) is generally more conspicuous than one with a slow rise-time.

The use of a flickering light may increase conspicuity for an AtoN against a high level of background lighting but only at high levels of observer illuminance (typically close range).  There is no advantage at lower levels of illuminance.  Flicker at certain frequencies and duty cycles provides a conspicuity gain for white and red signal lights but the gain for green is less noticeable at lower duty cycles. There is uncertainty on improvement of conspicuity by implementing flickering light on yellow and blue lights.

It is possible to improve conspicuity by utilising a low-level fixed light during the eclipse period of a character. This technique enables the user to confirm the position of the AtoN by providing a visible fixed point of reference. See IALA Recommendation E-110 [REFERENCE] and IALA Guideline 1116 [REFERENCE] for more information.

Some work has been done on how the repetition rate of a flashing light affects its conspicuity but this is almost certainly dependent on the illuminance at the eye of the observer.

### Shape of Light Source

The shape of a light source is irrelevant when it is viewed as a point source. However, if it is intended to be viewed as a lit area or extended light source, a given shape can be instantly recognisable to an observer. A lit shape can provide a very conspicuous marker, used either as a pointer to an AtoN or as an AtoN in its own right.

Light source shapes are recognised more quickly and effectively than a flashing point source.  Japan Coast Guard has done some work on illuminated numbers on buoys.  This has the advantage, not only of fast recognition but also enables the mariner to gauge his/her distance from the buoy more effectively.  This has reportedly reduced the number of collisions with buoys.

Such things as extended light sources or floodlit areas are generally designed to given shape information [10] [20] [21].  Therefore, the extremities of such lights need to correspond to an angular subtense of 3 minutes of arc or greater to be useful as AtoN (see section 6.2.7).  The physical size of such a light source would therefore determine its useful range.

### Size of Light Source

The size of an AtoN light will determine its horizontal/vertical subtense for an observer at a given distance. For an extended light source, it is important to make the size large enough so that it may be recognised at the maximum operational distance.  Furthermore, if the light source is to be viewed obliquely within its arc of utilisation, the width of the light source should be wide enough to account for the oblique view (see Figure 9).  The cosine of the angle of view may be used to calculate the required increase in depth/height.

Viewing an object from different angles

Because it is not viewed as a point source, the luminance of the extended light source, rather than its intensity, is relevant to the observer.  Therefore, as a rule of thumb, the luminance of the extended source should be at least four times greater than the background luminance for the eye to detect a difference.

In a paper presented to the 12th IALA Conference in Holland in 1990, Marc Mandler of the U.S. Coast Guard gave details of experiments carried out on extended light sources [21].  The paper is available on the IALA Wiki.  However, one important feature of the USCG experiment was the introduction of a conspicuity index based on the reciprocal of the time taken for observers to recognise a light signal.  Such a metric is considered a valid way of modelling conspicuity and could form the basis of a conspicuity model.

Among the conclusions of the experiment was that:

Conspicuity increased with size of extended sources;

Conspicuity decreased with higher luminance of background lighting;

Higher luminance backgrounds caused less reduction in conspicuity of large signals than small signals;

Flashing enhanced conspicuity of small targets more than large targets.

In the paper, methods are provided for aiding design engineers in the sizing and selection of extended sources.

The size of a light source has an effect on its performance at distance when scintillation occurs.  When observing the difference between a large light source and a small light source of similar intensity at a distance, the difference in scintillation is quite marked.  On a clear night, when distant lights scintillate or flicker, a large light source, for instance a large lens panel, scintillate much less than a small one.  Trying to find a flashing light among twinkling lights can be difficult.  A large light source, even when it is flashing, scintillate less and stands out from its surroundings as a result.

### Floodlighting

Floodlighting can be considered as an extended light source.  At close range, an illuminated structure can provide a conspicuous mark for an AtoN light.  If the structure is uniquely recognisable, it can provide instant and reliable recognition to the mariner.  However, floodlighting usually requires high-powered lamps and may not be practicable for AtoN with low-energy systems.

At distances of a few miles, the effects of floodlighting usually start to fade as the angular subtense of the area illuminated becomes smaller and the recognition of the object becomes more difficult.  Therefore, floodlighting is recommended for improving conspicuity at relatively short distances, although this will depend on the illuminated area and the degree of illumination.  Further information can be obtained from the IALA Guideline No. 1061 on Lighting Applications on the Illumination of Structures [10].

Since an illuminated structure is a luminous area being viewed rather than a point source, as with ‘light-pipes’ or contour lights, the advantages are instant recognition and ability to gauge distance.

However, the effectiveness of floodlighting depends upon the surface being lit and its ability to reflect the light illuminating it.  Different surfaces reflect light (and different colours of light) differently.  The angle of illumination is also important, as is the area of structure illuminated, which in turn depends upon the beam angle of the floodlight and its distance from the structure. Consideration should be given not to confuse the user with different colour lights, particular due to colour mixing of the AtoN light. All efforts should be made to ensure that floodlighting does not impair the conspicuity of the AtoN light.

Further factors are the change of surface reflectance due to deposits of water (e.g. rain), dirt or growth and the way the floodlit structure is viewed against the background.  If possible, a compromise between acceptable floodlight colour, the structure’s reflectance of that colour and contrast with the background illuminance should be sought.

Further information can be obtained from the IALA Guideline No. 1061 on Lighting Applications on the Illumination of Structures [10].

Lighthouse Tower Floodlit in different Colours

### Contour Lighting

Highlighting the outline shape of a structure with additional low luminance strips of light can be useful for two reasons: it provides a recognisable shape and it gives an impression of size and distance.  Further information can be obtained from the IALA Guideline No. 1061 on Lighting Applications on the Illumination of Structures [10].

### Synchronisation

Synchronising several AtoN lights can increase conspicuity considerably.  This technique is very effective for several buoys marking a channel, for port and starboard port entry lights or for leading light.  When viewed at a distance, the effect of several lights all coming on together over a wide area is highly conspicuous.  However, if all AtoN lights in the synchronised group have the same rhythmic character, there can be problems in identifying individual AtoN within the group.  This is particularly relevant in poor visibility, when perhaps only one of the AtoN can be seen at any one time.

It is possible to retain individual characters within a group of AtoN and synchronise them at the start of rhythmic character with the longest period.  For this to be practicable, the AtoN characters need to have a common period or exact divisions of the longest period (or exact multiples of the shortest period).  Further information can be obtained from the IALA Guideline 1116 on Selection of Rhythmic Characters and Synchronisation of Lights for Aids-to-Navigation [14].

## The Background

### General Background (Luminance)

A general area or large patch of light of roughly uniform brightness, directly behind an AtoN light, can impair the mariner’s view of an AtoN light of interest.  The luminance of the background lighting can be measured in candelas per square metre.

When a large mass of shore lighting forms a backdrop to an AtoN light and the individual light sources within the background are indistinguishable, it may be taken as a homogenous background luminance.

Background luminance due to town lighting

To the observer, shore lights tend to merge at a distance, for instance, when the mariner is some way offshore.  Therefore, the AtoN lights most affected by background luminance are usually long-range lights.

When closer to shore, background lighting may usually be considered as a patch of individual point sources with a given density.  This is a much more complex picture than a patch of uniform luminance and the density of the point sources will vary with the number background lights in view, their proximity to each other and the observer's distance from them.

### Rival Lights (Intensity)

Rival lights can be considered as individual point sources of unwanted light that mask or obscure an AtoN light and prevent easy detection or recognition of the AtoN.  Sometimes a slight change in position of the mariner within a channel, or the height of observation can make the difference between seeing an AtoN or not.

Rival lights caused by individual point sources

Background scenes are often comprised of a mixture of general background lighting and rival lights.

Another example of rival lights is when there are too many AtoN lights in an area, thereby causing confusion.  An example is when a large number of fish farms, each with its own flashing light, surround a buoy with its own flashing light.  The buoy is therefore difficult to detect or recognise amongst many more flashing lights, even though the rhythmic characters in view are different.

#### Avoidance of Rival Lights and Background Illumination

Where possible, lighting around existing AtoN should be controlled such that any new lighting does not significantly affect the operation of the AtoN light.  This may be carried out by legislation or by existing planning arrangements.

It may also be possible to relocate, shade, or redirect rival lights [15].  This usually needs the consent of the owner or user of the rival lights in question but, quite often, a small adjustment in the position, direction or luminaire of a rival light can result in a major difference in conspicuity of the AtoN being interfered with.

If no other course of action is available, it may be possible to move the AtoN away from rival lights, or for a new installation to locate the AtoN away from a rival light or lights (discussed in IALA Guideline No. 1051 [11]), so that they are not in the same direction of view.  A small vertical shift can often separate the AtoN light from the background or rival light from the observer's viewpoint.  However, any relocation should not affect the primary navigational significance of the AtoN.  Conflict between rival lights and AtoN can often occur in specific locations within the AtoN's zone of utilisation.  The performance of the whole zone of an AtoN light should be considered over a range of observation heights.

### Spectral Properties of Background Lighting

Since most background luminance is white or near-white, the AtoN most susceptible to masking or interference are white lights.  However, low-pressure sodium lights are yellow and could easily mask a yellow 'special mark' AtoN light.

The colour of general background lighting is a mixture of all the lights in view but usually takes on the colour of the dominant light source.  For instance, if the background scene is comprised mainly of high pressure sodium, the overall colour seen from a distance will be yellow-white with a hint of orange.  If the background is a busy floodlit container port however, the background lighting could be predominantly metal-halide lamps giving a blue-white hue.

A rival light, on the other hand, could consist of an individual light source of any colour.  It may be a single street lamp, road traffic signal or navigation lights on-board other vessels.

# WAY FORWARD

As mentioned previously, conspicuity of AtoN signals is a complex subject.  This guideline is merely a review of current state of knowledge, and clearly there is a need for further work on this subject.

A future way of dealing with conspicuity for both the initial planning and design of AtoN lights, as well as subsequent reviews, is to take a more holistic approach.  The intention is to develop a conspicuity model that can be used to quantify conspicuity by inputting relevant information in order to define the operational requirement [23].  A probabilistic approach for such a model should be considered.

However, there are many factors that affect conspicuity and their impact and interrelation is barely understood at present.  Therefore, much in-depth scientific work is needed for such a development.  The results could provide not only a model but perhaps simulators based on that model.  This would enable preliminary trials to take place in simulation thereby reducing costs of AtoN provision and increasing the chances of providing a useful AtoN.

# ACRONYMS AND DEFINITIONS

## Acronyms

CIE	Commission Internationale de l'Eclairage (International Commission on Illumination)

eILV	Electronic International Light Vocabulary

## Definitions

| Detection of an AtoN light | The observer is aware of a light | ADD TO IALA DICTIONARY |
| --- | --- | --- |
| Recognition of an AtoN light | The observer is aware that the light is an AtoN light | ADD TO IALA DICTIONARY |
| Identification of an AtoN light | The observer is aware of the exact AtoN to which the light belongs | ADD TO IALA DICTIONARY |
| Conspicuity | Quality of an object or a light source to appear prominent in the surroundings – eILV 845-11-30 | [2] |
| Background Luminance | A general area or large patch of light of roughly uniform brightness, directly behind an AtoN light, that impairs the mariner’s view of the AtoN light of interest | ADD TO IALA DICTIONARY |
| Rival light | Rival lights can be considered as individual point sources of light that mask or obscure an AtoN light and prevent easy detection or recognition of the AtoN | ADD TO IALA DICTIONARY |
| Background Lighting | Encompasses ambient lighting either directly behind or adjacent to the AtoN having regard to the range of perspectives or directions of intended viewing | [11] ADD TO IALA DICTIONARY |
| Luminous Intensity | IALA Dictionary 2-1-035 | [17] |
| Illuminance | IALA Dictionary 2-1-055 | [17] |
| Luminance | IALA Dictionary 2-1-045 | [17] |
| Glare | IALA Dictionary 2-1-420 to 2-1-430 | [17] |
| Visibility | IALA Dictionary 2-1-275 | [17] |

# REFERENCES

Japan Coast Guard / Ocean Police Research Foundation, ‘Report of the Expert Meeting on Standardization of New Lighting Method for Marine Aids to Navigation’, Tokyo (November 2008).

CIE Publication 17.4 International Lighting Vocabulary (1987).

IALA E-200 Series of Recommendations.

IALA Recommendation E-200-0 Overview

IALA Recommendation E-200-1 Colours

IALA Recommendation E-200-2 Calculation, definition and notation of luminous range

IALA Recommendation E-200-3 Measurement

IALA Recommendation E-200-4 Determination and Calculation of Effective Intensity

IALA Recommendation E-200-5 Estimation of the Performance of Optical Apparatus

IALA Guideline No. 1061 ‘Lighting Applications on the Illumination of Structures’.

IALA Guideline No. 1051 ‘On the Provision of Aids to Navigation in Built-up Areas’.

GLA R&RNAV Technical Report RPT-28-MN-04 ‘Conspicuity - Colour Observed and measured comparison of a White LED Beacon versus a White Halogen Beacon’ (2004).

‘Some observations in connection with the note on the perceptibility of lights, presented by Messrs. Hampton and Holland to the International Lighthouse Conference Paris 1933’, P. van Braam van Vloten, Scheveningen (10th May 1933).

IALA Guideline No.1069 ‘On Synchronisation of Lights’.

GLA Development Department Report No. 5/IT/1999 ‘Lowestoft Background Lighting Investigation’ (19 April 1999).

‘Minimum figure of the perception of light’, Jean REY, International Lighthouse Conference (1933).

IALA Dictionary

GLA R&RNAV Technical Report RPT-14-MN-IT-06 ‘Kinnaird Head Viewing Trial’ (20th November 2006).

‘Japan Coast Guard Field Experiment on Conspicuity of Flickering Lights’, Kujukuri beach, Sanmu city, CHIBA (October 2008).

‘Technical Note on Pointer Devices for Improving the Conspicuity of the Visual AtoN’, Juan F. Rebollo, Puertos del Estado- Spain (2009).

U.S. Coast Guard Research and Development Center, ‘Conspicuity of Aids to Navigation: Extended Light Sources’, Marc B. Mandler (1990)

IALA NAVGUIDE.

GLA R&RNAV Technical Report RPT-10-MN-10 ‘A First Generation Conspicuity Model’ (1st June 2010).

Farbgebung und Tagessichbarkeit von Tonnen – Frank Hermann, German Federal Waterways Administration – Traffic Technologies Centre (Koblenz).

IALA Guideline No. 1065 on Vertical Divergence.

PRESENTATIONS MADE TO THE JAPAN COAST GUARD EXPERT MEETING (NOVEMBER 2008)

MR. LARRY JAEGER, USCG, USA

After an introduction by the Master of Ceremonies, Mr Jaeger started by thanking the JCG and the OPRF for inviting him to the meeting.  He said that his presentation had three main topics, effective intensity, apparent intensity and conspicuity.  He then described the effective intensity of a flashing light as a concept that has meaning only at threshold.

He mentioned that effective intensity is typically not a useful concept for the mariner because, at threshold, a light can barely be detected and neither its colour nor its flash rhythm can be discerned.  He then explained that apparent intensity is a supra-threshold concept that relates to a light’s perceived brightness.  The apparent intensity of a flashing is the intensity of a steady light that would appear equally bright.  He showed a famous Broca & Sulzer model (1902) that indicates that a flashing light with the same intensity of a steady light may appear brighter than the steady light.  However, he added that there have been a limited number of apparent intensity studies and that a comprehensive apparent intensity model will require more research.

Finally, he indicated that conspicuity relates to how easy it is to locate a light-of-interest against a background of rival lights.  He showed several examples that demonstrated how colour and flash rhythm can increase conspicuity.  He noted that conspicuity is a complex concept and that there are many factors that will influence conspicuity including the intensity, colour, flash rhythm, shape, and movement of both the light-of-interest and the rival lights.  He indicated that he was not aware of any single measure of conspicuity, but that in many studies the measure of conspicuity is the time it takes to locate a light-of-interest against a background of rival lights.

After the presentation, he was asked how to describe the conspicuity of a light to mariners.  He answered that aids-to-navigation authorities need to adequately describe the nature of a light (colour, temporal pattern [including flicker], and intensity [expressed as nominal range]), but that he did not foresee the need to publish a separate conspicuity measure.  He was also asked if there was a concrete maximum amount of time for locating an Aid to Navigation and he replied that there was a no such standard.

DR. YOSHI OHNO, NIST, USA

At the beginning of his presentation, he extended his appreciation to the JCG and the OPRF for the invitation to this interesting meeting.  Then he introduced the National Institute of Standards and Technology (NIST) that maintains and disseminates national standards for optical radiation and added that the NIST is the only national lab that provides calibration service for flashing lights.

Next he described the effective intensity that was a luminous intensity of a steady light which had the same luminous range of a flashing light under the identical condition such as threshold of detection, white light, dark background, foveal vision, point source.  He showed the three methods of the effective intensity;

- Blondel-Rey(-Douglas) which is good for a rectangular pulse.

Form Factor Method which is good for non-rectangular pulse but not for train of pulse.

Allard method which is based on a vision model considering the eye as a low pass filter.

He said that the Allard method seemed to work well for all pulses but there was a deviation from the Blondel-Rey.

He then mentioned that he and his colleague therefore developed the Modified Allard Method to solve the problem by modifying the visual impulse response function q(t) of Allard.  He said that the Modified Allard matched the results with Blondel-Rey for rectangular pulses; the calculation of effective intensity of all pulses seemed accurate but an experimental verification was missing at that time.  But he added that after a long search by CIE TC2-49, the Committee finally found the 1986 USCG paper reporting vision experiments on the effective intensity of multiple-flick flashtube signals and confirmed that the calculation by Modified Allard matched the experimental result of the paper surprisingly well.  This was considered as experimental verification for Modified Allard method.

Finally, he stated that in 2007 the CIE TC2-49 adopted the Modified Allard method for the current draft, which will be finalized and to be published as CIE recommendation for effective intensity.

MR. OMAR FRITS ERIKSSON, DANISH MARITIME SAFETY ADMINISTRATION, DENMARK

After thanking the JCG and the OPRF for the invitation, Mr Eriksson started his presentation by mentioning that, since the shortcomings of existing models became obvious with the advent of the LED based light sources, IALA has been searching for a better model for calculating the sensation of flashing and flickering light.  He added that IALA is seeking an all-encompassing model for flashing light of any pulse shape and spectral composition.

He then said that under certain conditions flickering light is perceived brighter than a steady light of the same luminous intensity.  This may be due to the so-called BrocaSulzer effect or other similar effects.  He also mentioned that the brightness of flickering light is perceived differently at different observation distances.  Therefore, he and his colleague at IALABATT/IALALITE and EEP committee had conducted a test using a lantern that emitted two types of flash profiles.  While one was a steady flash, the other had a flickering part at the beginning of the flash.  The result of test was that the perceived brightness and conspicuity of the two flash profiles were not so different at a distance of 1.5 nautical miles, however at shorter ranges, the flickering flash was more conspicuous.

He then said that it was concluded that flickering light could be useful for providing easy identification of certain Aids to Navigation at close range.  He added that Mr Kergadallan of the French lighthouse authority also had conducted a study of perception of flickering light with distance and he would be reporting the findings on behalf of Mr Kergadallan.

Mr Eriksson then talked about conspicuity and proposed an analogy between measuring conspicuity and measuring signal to noise ratio of a radio signal.  He explained that the concept of signal demodulation was similar to the concept of recognizing an Aids to Navigation signal, however the model of the radio signal was a simple one compared to the complexity of visual perception.  He said that the human visual system is complex and that the eye is constantly changing its state.  Therefore, there is a need to define of one or more standard observers with well-defined visual properties.

He made the point that due to the shortcomings of existing models which are relatively simple, there is a need for more detailed modelling of the visual system, a model capable of capturing both complex temporal and spectral characteristics of the stimulus at any eye adaptation level.  He also said that the IALA EEP Committee now has flickering light on their work program and that IALA also has a small group of light experts looking into the issue.  In conclusion Mr Eriksson expressed that flickering light should be only be applied a few aids in any one region in order to harvest conspicuity.  Finally, he urged everyone to continue to conduct visual experiments and to support IALA in its search for the all-encompassing model for calculating the sensation of any flash profile.

MR. XAVIER KERGADALLAN, CETMEF, FRANCE, SUBSTITUTED BY MR. ERIKSSON

Mr Eriksson kindly made the presentation on the behalf of Mr KERGADALLAN.  Mr Eriksson explained that Mr Kergadallan had conducted a visual experiment on the perception of flickering light at various distances, in both direct (foveal) and peripheral (at an angle of 45 degrees) vision.

He then described how the experiment had been conducted, and the apparatus used.  The illumination level at the eye of the 5 observers, placed 12.5 meters away from the light source, had been varied using a neutral density filter to simulate an observation distance of 235, 715, 2035 and 5035 meters.  The nominal luminous range of the light source had been 8149 meters.  The light was then flickered at the rate of 5, 10 and 20 Hz with a duty cycle of 20%.  Mr Eriksson then explained that the experiment had yielded some interesting results.  Firstly, the data suggests that the flickering effect disappears as the observation distance is increased - and thus the level of illumination at the eye of the observer is decreased.  Secondly the data suggest that the flicker can be observed at larger distances when flickered at a lower frequency.  Thirdly the observers detected flicker at greater distances using direct vision rather than peripheral vision.

In conclusion, Mr Eriksson said that Mr Kergadallan is of the opinion that it is better to use low flicker frequencies and that there is a need to define the useful range of flickering lights.  Thus, in addition to the nominal luminous range of a light, a flickering range for direct vision and a flickering range for peripheral vision should be stated.  Finally, Mr Eriksson conveyed from Mr Kergadallan his regrets for not being present and greetings to the all participants of the symposium.

After the presentation Mr Eriksson was asked if the energy saving effect of the flicker had been considered.  He answered that the flicker light had such effect, but only under certain conditions.  He then was asked if a standard LED light source should be defined and he replied that the question was related to the measurement and Dr OHNO was more suited to answer the question.  Dr OHNO answered that developing such a standard light source is difficult, but for example, with Modified Allard method, the ratio of effective intensity to physical quantity (candela second) can indicate energy efficiency of the lights.  Finally there was a question on the border between a flash and a flicker.  Mr Jaeger answered that there appears to be an overlap near 5 Hz: an IALA ultra-quick flashing light flashes at 5 Hz, but 5 Hz is also being tested as a flicker frequency.

MR. YOSHIYASU FUKUSUMI, JCG RESEARCH CENTER, JAPAN

At first, he explained that the origin of his research was to save energy of aids to navigation using flicker light.  Then he summarized the result of the flicker experiment of a red LED lantern until 2007 that although the most conspicuous flicker is frequency of 5 Hz and duty ratio of 20%, it was decided to use higher frequency because 5 Hz is already used as ultra-quick light of IALA character.  He added that considering a cost performance of lantern, the flicker of 10 Hz and 30% is finally decided to be the most appropriate to use.  During his explanation, he also demonstrated various combinations of flicker light using special equipment.

Then he explained the experiment in 2008 that studied green and white LED lantern by comparing combinations of frequency and duty ratio.  He stated that this experiment was conducted at a beach and 52 observers attended.  He said that fixing frequency at 10 Hz and comparing duty ration, the result shows the lower duty ratio is more conspicuous.  He added that fixing duty ratio at 30%, the lower frequency is more conspicuous and the Modified Allard Method almost matches the result of the experiment.

After the presentation, he was asked how he estimated if there is only one pulse in three seconds and he answered such lighting has more conspicuity and the Modified Allard Method probably shows the same result.

CONSPICUITY OF AtoN – EXTENDED LIGHT SOURCES

This is an electronic version of the paper presented to 12th IALA Conference in Holland in 1990.

CONSPICUITY OF AIDS TO NAVIGATION:

EXTENDED LIGHT SOURCES

by

Marc B. Mandler

U.S. Coast Guard Research and Development Center

Avery Point Groton, Connecticut 06340-6096

United States of America

SUMMARY

Conspicuity of point and extended sources was measured in field and laboratory experiments to assess the effects of signal size and background lighting density on a mariner's ability to find a navigation light.  In the laboratory experiment, observers searched a computer display for a signal embedded in a background of point sources.

The time required for observers to correctly locate the signal light was used as the measure of conspicuity.  In the field experiment, observers searched the shoreline of New London Harbour in Connecticut for vertically oriented extended sources that had been installed for the experiment.

The proportion of time each light was located in a fixed time period was used as the measure of conspicuity.  Conspicuity increased with size of extended sources and decreased with density of background lighting.  High density backgrounds caused less reduction in conspicuity of large signals than small signals.  Flashing enhanced conspicuity of small targets more than large targets.  Methods are provided for aiding design engineers in the sizing and selection of extended sources.

INTRODUCTION

Visual signals used for navigation must be distinctive to ensure that the signals can be located and interpreted rapidly.  Road signs, channel lights or runway lights must be easy to distinguish from the background clutter, since time spent searching for a navigation signal may interfere with the demands of safely operating an automobile, ship or plane.

Conspicuity is a measure of the attention-getting quality of a navigation signal.  High conspicuity signals, or signals that attract the attention of an observer, are most desirable so that the likelihood of missing a critical signal is small.

Currently no standard methods exist for rating conspicuity of different signals.  Designers of visual signals must rely on the traditional measure of intensity as a measure of signal effectiveness.  High intensities are considered more effective since they can be detected at greater distances than low intensities.  The intensity of a signal, however, does not reflect how well it can be located within a background.  An intense, white navigation light, for example, may be quite difficult to find in a background of bright, white lights, while a dim red light may highly conspicuous.

A signal does not have an inherent conspicuity in the absence of a background, and thus any measure of conspicuity must take into account the background.  High intensity signals may be effective when the background has low intensity lights.  Red signals are conspicuous when the background has a high density of green lights.  Flashing lights are conspicuous against backgrounds of fixed lights.

For mariners approaching harbours night-time backgrounds generally consist of many point sources of light of different colour and intensity (Worthey, 1988) [2].  Typical navigation lights are red, green and white point sources that flash in some characteristic way.  Mariners often report great difficulty in determining which lights are relevant signal lights and which lights are the irrelevant background lights.  In many situations the conspicuity of the signal lights is inadequate.

One approach for improving the conspicuity of navigation signals is to use signals that look physically different from the point sources that comprise the background.  Extended sources, large lights that appear to have physical dimension at a distance, are one class of signal light that may have a higher conspicuity than point sources.  A mariner searching a harbour for a navigation light can rapidly reject as not being a navigation light any light that is obviously a point source.

This report, the first in a series to deal with the issue of conspicuity, is an evaluation of the concept of using extended sources for aids to navigation.  This report provides data of a laboratory and field experiment designed to understand the relationships between conspicuity, signal size and background lighting.  The laboratory experiment used computer simulated backgrounds to study conspicuity while the field experiment involved the task of locating real extended sources in an industrial harbour.

This report provides extended source design guidelines for the engineer.  The report addresses the conspicuity advantage expected from extended sources, describes the effects of signal size and background lighting on conspicuity, and provides guidelines to be used for developing extended source hardware.

BACKGROUND

Conspicuity of aids to navigation has become a problem in the past 50 or 60 years with the advent of cheap electricity and the development of the shoreline.  A Committee on Vision Working Group of the National Academy of Sciences - National Research Council studied the conspicuity problem for the Coast Guard in 1971 and reported that background lighting was a severe problem (NAS-NRC, 1971) [1].  The Working Group recommended that a permanent solution be obtained through regulation of lighting along the shoreline.  The Group observed that many of the developments for aids to navigation, such as flashing/rotating lamps and flashtubes have been copied by bars and restaurants along the shoreline, adding to the Coast Guards difficulty in improving conspicuity.

In 1983 the Signal Effectiveness Project was begun to investigate methods of improving navigation safety through improvements in aids to navigation.  The idea of using extended sources became realistic with the development of a light from TIR Systems Limited that required only a single, standard lamp to illuminate a large area.

METHODS

Laboratory Experiment

Apparatus and Stimuli

Signal and background lights were generated on the face of a high resolution (1024 x 768 pixels) colour display controlled by a Hewlett-Packard Series 9000 Model 320 computer.  Each pixel of the display subtended approximately 1.75 min arc at the 61 cm viewing distance.  Signals were all horizontally oriented extended sources with a width of 1.75 min arc and a length that varied between 5.25 and 28 min arc.  Some signals flashed with a flash rate of 1.0 flash/sec and a duty cycle of 0.3.  Background lights were square and varied in size from 1.75 min arc per side to 3.5 min arc per side.  Background lights were always fixed on.

Observers searched the display for a single signal light that was placed randomly within a 35 cm wide x 9 cm, high (30 x 8 degree) search area.  The signal light was always larger than the background lights.  When the signal light was found observers pressed a response button.  The computer recorded the time required to locate the signal.

The display was divided into five equal-size areas.  Observers reported the area in which the signal was found by pressing one of five response buttons.  The computer delivered a tone when an incorrect button was pressed.

Background lights were randomly placed on the screen such that each light was at least 10.5 min arc (6 pixels) from its nearest neighbour.  Signal lights were also randomly placed with the constraint that they be no closer than 10.5 min arc to any background light.  In the absence of signal and background lights the screen had a luminance of approximately 0.01 ft-L, equivalent to a clear night with full moon.  A drawing of the stimulus display is provided as Figure 14.

Observer view of display in lab experiment

If signals were all of equal luminance, the larger signals would tend to appear brighter than the smaller signals.  Conspicuity would then be a function of both signal size and brightness.  To ensure that the measure of size conspicuity was not confounded with the brightness of the signal, all signals were adjusted to be a constant multiplier above threshold.  Thresholds, minimum luminance required to just detect a light, were measured for all sizes of lights.  Average thresholds of eight observers, shown in Table 1, were used as the base luminances.  All signals were adjusted to be 15 times their respective thresholds. Luminance of background lights was randomly selected for each light to be between 10 and 20 times threshold.

Detection and thresholds background for signal lights

| Size (min arc) | Threshold (ft-L) |
| --- | --- |
| 1.8X1.8 | 0.32 |
| 3.5 x1.8 | 0.22 |
| 5.3 x1.8 | 0.19 |
| 7.0 x1.8 | 0.17 |
| 8.8 x1.8 | 0.16 |
| 10.5 x1.8 | 0.16 |
| 14.0 x1.8 | 0.14 |
| 21.0 x1.8 | 0.14 |
| 28.0 x1.8 | 0.13 |
| 3.5 x3.5 | 0.16 |

Procedure

A session was composed of 300 trials plus a brief practice session.  Thirty different experimental conditions (signal size, background density, flash/fixed) comprised a session with ten presentations of each experimental condition.  Instructions for the experiment were provided on the computer display as observers adapted to the background luminance level.  Thirty practice trials were provided to familiarize observers with the task.

A tone sounded to signal the start of each trial.  The background lights and signal were illuminated simultaneously, and the observer began searching for the signal.  When the signal was found, the observer pressed one of five buttons corresponding to the area in which it was found. If an incorrect button was pressed, the computer provided a tone to inform the observer and the experimental trial was discarded.  If the signal was not found in 20 seconds, the trial was terminated and reaction time was recorded as 20 seconds.

An experimental session took between 35 and 60 minutes depending on the reaction times and length of rest periods taken by each observer.

Observers

Twenty people, all employees of the Research and Development Center and between the ages of 22 and 60, served as observers.  A vision exam on a Titmus Vision Tester was used to screen all observers had vision corrected to 20/30 or better.

Field Experiment

Materials

Seven extended sources were installed along the waterfront in New London Harbour. Two of the sources were light pipes purchased from TIR Systems Ltd. of Burnaby, British Columbia, Canada.  A schematic of this light source is shown in Figure 15.  A 75 Watt flood lamp installed at the base of the light shines toward the mirror at the top of the light.  Through the property of total internal reflection, the light is diffused uniformly across the surface of the light by reflecting from the plastic prismatic material.  The prismatic material allows a portion of the light to ‘leak’ out, providing a 50 ft-L, IO foot long light source.

Five other extended sources were constructed at the Research and Development Center of off-the-shelf materials, including white, vinyl rain gutters, 60 1-Watt Christmas-type lamps equally spaced along the length of the rain gutter, and clear plastic diffusing material like that used for indoor lighting to diffuse fluorescent lamps.  These lights, constructed in 10-foot sections, had a luminance of 40 ft-L.

Schematic of TIR lamp

Each light was outfitted with a servo motor attached to the power switch of the light source so the lights could be remotely controlled by radio signal.  Ten-foot light sections were installed at four sites, and 20-ft sections (two stacked 10-ft sections) were installed at three sites, Figure 16 shows a photograph of a 10-foot tall extended source viewed from a distance of approximately one nautical mile.

View of extended source in New London Harbour at a distance of 1.0 nautical miles

Procedure

The experiment was conducted on five clear nights with calm seas in mid-September, 1988.  The River Queen, a harbour tour boat with a capacity of 100 people, was rented for the experiment.  Observers were taken to three different observation points within New London Harbour and instructed to locate all the extended sources that were illuminated.  At any one time two to five lights were illuminated.  Observers were given either 30 or 45 seconds to search for the lights (depending on the number illuminated) and note the number found, but were not informed as to how many lights were illuminated.  At the end of the search period they informed the experimenter of how many lights were found and pointed to the locations of the lights.

Observers

Fifty observers between the ages of 21 and 60 participated in the experiment.  Most participants were Research and Development Center employees or spouses.  All were assumed to have normal or near-normal vision.  No vision screening was performed.

RESULTS

Laboratory Experiment

Figure 17 shows average log reaction times as a function of number of background lights for seven signal sizes.  Each symbol represents an average of 200 observations.  Signal size is noted by each curve.  The vertical bars about each symbol show plus and minus two standard errors of the mean.

Reaction times increase with increasing numbers of background lights; the amount of increase being dependent on signal site.  In the absence of background lights reaction times for all signals are about 0.76 sec (-0.12 log sec).  With a modest increase of 25 background lights reaction times for the 5.3 and 7.0 min arc signals increased to 3.5 sec (0.55 log sec) and 2.1 sec (0.33 log sec), respectively.  Reaction times for small signals (5.3 min arc) were affected more by background lighting than large signals (28 min arc).

A repeated measures analysis of variance (size x number of background lights) revealed highly significant effects of signal size and number of background lights, as well as a significant interaction between size and number of background lights,

The effect of flashing the signal can be seen in Figure 18a and Figure 18b.  These figures compare log reaction times of fixed and flashing lights as a function of signal size for backgrounds of 100 lights (Figure 18a) and 400 lights (Figure 18b).

Average reaction times for fixed lights.

Signal size is shown as the parameter of each curve.  Vertical bars are +/-2 standard errors of the mean.

Figure 18b

Figure 18a

Average reaction times of fixed and flashing lights.

Figure 18a shows data for 100 background lights and Figure 18b 400 background lights.  Vertical bars are +/-2 standard errors of the mean.

When 100 background lights are present, flashing increases conspicuity (reduces reaction time) for small signals, but decreases conspicuity (increases reaction times) for large signals.  When 400 background lights are present, flashing reduces reaction times for all signals measured.

It is interesting to note that the effect of background lighting on reaction time is much smaller when signals are flashing than when they are fixed.  Figure 19a and Figure 19b re-plot the data of Figure 18 to show this result.  For the fixed lights (Figure 19a) a change from 100 to 400 background lights causes approximately a 50% increase in reaction time, while for the flashing lights (Figure 19b) the change in number of background lights has little effect,

A more complete representation of the data is shown in Figure 20 and Figure 21.  These figures show cumulative per cent detected as a function of log reaction Lime for different signal sizes against various backgrounds.  A single set of data represents 200 observations.  Each symbol plots the percentage of times the signal was found within the time specified on the abscissa.

Note that all data sets appear reasonably linear over the range of 20% to 80%.  Regression fines fit to each data set are shown as solid fines, Figure 20 shows data for fixed lights in the presence of 100 background lights (Figure 20a) and 400 background lights (Figure 20b).  Figure 21 shows data for flashing lights in the presence of 100 background lights (Figure 21a) and 400 background lights (Figure 21b).

These plots are instructive in showing how:

reaction time varies for a criterion detection probability;  or

detection probability varies for a criterion reaction time.

The point at which a horizontal line at some criterion detection percentage, say 75%, intersects each regression line shows the time required to locate a signal 75% of the time.  Similarly, the point at which a vertical dashed line at, say 0.3 log seconds, intersects each regression line indicates the probability that a light of the given size will be located in 2 sec.

Figure 19b

Figure 19a

Effect of background lights on fixed and flashing lights

Figure 19a is for fixed lights and Figure 19b for flashing lights.  Number of background lights is given as the parameter of each curve.  Vertical bars are +/-2 standard errors of the mean.

Figure 20b

Figure 20a

Cumulative per cent detected for fixed lights

Figure 20a is for 100 background lights and Figure 20b for 400 background lights.  Signal size is given as the parameter of each curve.  Lines are least-square fits to the data between 20% and 80%.

Figure 21a

Figure 21b

Cumulative per cent detected for flashing lights

Figure 21a is for 100 background lights and Figure 21b for 400 background lights.  Signal size is given as the parameter of each curve.  Lines are least-square fits to the data between 20% and 80%.

Per cent detected for field experiment

Data divided into two background lighting conditions shown on the figure.  Open symbols are from field experiment, filled symbols from lab.  Experiment, lines are least-square fits to the logarithm of signal size.

Field Experiment

Figure 22 shows the percentage of observers who detected a particular light as a function of the signal size of the light.  The data are categorized by background lighting present when observations were taken.  The open circles are for lights viewed against an area of low density background lighting.  The open triangles are for lights in a background of many lights, or near a glare source.  The filled symbols are data from the laboratory experiment that will be discussed below.  On occasion the background for a particular light varied from night to night due to factors beyond our control.  For example, one night a ship tied up near one of the extended sources.  The bright lighting on the deck of the ship caused sufficient glare to obscure the signal light.  The data were sorted to reflect these variations in background lighting; Least-square fats to the logarithm of signal size are shown on linear axes for the two sets of data.

As signal size increased, the probability that the light was detected also increased.  Moreover, lights viewed against low density backgrounds were detected more often than lights viewed against high density backgrounds.  The largest signal, 14 min arc viewed against a high density background was detected only 65% of the time, compared to an 8 min arc signal that was detected 75% of the time in the presence of minor background lighting.

Comparison of Field and Laboratory Data

Though the laboratory and field experiments used different methodologies and were conducted under different experimental conditions, an attempt was made to find some commonality between the two sets of data.  If the data from these two experiments accurately measured the relationship between detection, signal size and background lighting, it should be possible to superimpose the two data sets under the appropriate conditions.

The filled symbols in Figure 22 represent the percentage of signals detected within 3 seconds.

These data were obtained from the curves in Figure 20 by taking a vertical ‘slice’ through the data at a reaction time at 0.48 log sec (3.0 sec) and reading the percentages off the fitted curves.  Three seconds detection time was chosen as it provided a good fit, by eye, to the field data.  The minor and major background lighting data were taken from the data for 50 and 400 background lights, respectively.

DISCUSSION

Conspicuity Index

One of the goals of these experiments is to provide a method for rating conspicuity of different signals.  A metric must be established that reflects the ease with which a signal can be found.  Such a metric will be useful for selecting appropriate-size signals for a specific application.

Reaction time forms the basis of the conspicuity measure.  A simple metric using the inverse of reaction time yields large numbers for signals found quickly and small numbers for signals that are difficult to find.

What reaction time should be used?  We could use average reaction time, or a criterion like the reaction time at which 75% or 90% of lights are found.  A criterion of 75° has the advantage that it provides a metric that varies between 0.0 and approximately 1.0.  Table 2 shows this conspicuity metric (1/reaction time) for the 30 conditions of the laboratory experiment.  Values in parentheses are for flashing lights.

Conspicuity Index for extended sources. (Detection Probability = 0.75)

| Signal size | Number of Background Lights |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (min arc) | 0 | 25 | 50 | 100 | 200 | 400 |
| 5.3 | 0.98 | 0.15 | 0.11 | 0.07 | 0.04 | 0.01 |
|  |  |  |  | (0.18) |  |  |
| 7.0 | 1.08 | 0.31 | 0.24 | 0.15 |  | 0.07 |
|  |  |  |  | (0.25) |  | (0.22) |
| 8.8 |  |  | 0.38 | 0.26 | 0.18 | 0.09 |
|  |  |  |  |  |  | (0.27) |
| 10.5 |  |  |  | 0.37 |  | 0.21 |
|  |  |  |  |  |  | (0.30) |
| 14.0 | 1.14 |  | 0.64 | 0.53 |  | 0.30 |
|  |  |  |  | (0.41) |  | (0.39) |
| 21.0 |  |  |  |  |  | 0.57 |
| 28.0 |  |  |  |  |  | 0.72 |

The most conspicuous signals, with conspicuity indexes near 1.0, are those signals that are viewed in the absence of a background.  The 5.3 min arc flashing light, the signal most similar to a quick flash Aid to Navigation, has a conspicuity of 0.18 in a background of 100 lights.  By increasing the size of the signal to 14.0 min arc the conspicuity increases by a factor of 2.27 to 0.41.  This means that the larger signal can be found 2.27 times faster in the background than the smaller signal.

Figure 23b

Figure 23a

Conspicuity indexes for fixed and flashing lights.

The number of background Lights is shown as the parameter of each curve.  Figure 23a is for fixed lights and Figure 23b for flashing lights.  Lines are the least-square fit to the data.

Figure 23 shows how the conspicuity index varies with target size and background lighting for fixed (Figure 23a) and flashing (Figure 23b) lights.  Lines are least-square fits to the data.  The conspicuity indexes are well-fit by straight lines.  The slopes and intercepts of the fitted fines are shown in Table 3.

Conspicuity Index Least Square Fits

|  |  | Slope | Intercept |
| --- | --- | --- | --- |
| Fixed | 50 Background Lights | 0,060 | -0.212 |
|  | 100 Background Lights | 0.054 | -0.218 |
|  | 400 Background Lights | 0.030 | -0.105 |
| Flashing | 100 Background Lights | 0.025 | 0.057 |
|  | 400 Background Lights | 0.024 | 0.054 |

As density of background lights increases the slope of the fitted fine decreases.  This means that conspicuity increases more slowly with size against high density backgrounds than against low density backgrounds.  For flashing lights, the slope of the best fitting fine is less than for fixed lights.

Fixed v Flashing

An interesting result of this experiment is that flashing enhances conspicuity for some targets but reduces conspicuity for others.  Flashing signals have higher conspicuity then fixed signals when they are small (less than approximately 7 min arc) or when they are viewed against the densest background lighting conditions (400 lights).  For signals greater than 7 min arc viewed against a moderate density of background lights, flashing reduces conspicuity.

One explanation for this is that when a light flashes it is unavailable to the observer for a significant portion of the time.  The flash rate used in the laboratory experiment was that of a quick flash characteristic (0.3 sec on, 0.7 sec off).  Seventy per cent of the time the signal was not illuminated and thus unavailable for detection.  When the conspicuity provided purely by the size of the signal is much greater than that provided by the flashing, the impact of the 70% off time of the light will be to reduce conspicuity.  When the contribution of flashing to conspicuity is greater than the contribution of size, flashing will enhance conspicuity.  In a future experiment the effect of flash rate on conspicuity will be investigated.

Size of Extended Sources

The data show that conspicuity increases with the angular size of a signal.  In constructing an extended source for field use, the angular size, a (in minutes of arc), of the signal is computed with the equation:

a = 3438 (s / d)

where s is the size of the signal in feet and d is the distance from the signal in feet.  Table 4 shows how angular size varies with signal size and distance from the signal.  A 20-ft long signal viewed from 1.0 nautical mile has an angular size of 11.3 min arc.

Angular Size of extended sources (Minutes of Arc)

| Source Size | Distance (nautical mile) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| (feet) | 0.5 | 1.0 | 2.0 | 3.0 | 4.0 |
| 4 | 4.5 | 2.3 | 1.1 | 0.8 | 0.6 |
| 8 | 9.0 | 4.5 | 2.3 | 1.5 | 1.1 |
| 12 | 13.6 | 6.8 | 3.4 | 2.3 | 1.7 |
| 16 | 18.1 | 9.0 | 4.5 | 3.0 | 2.3 |
| 20 | 22.6 | 11.3 | 5.7 | 3.8 | 2.8 |
| 24 | 27.1 | 13.6 | 6.8 | 4.5 | 3.4 |
| 28 | 31.7 | 15.8 | 7.9 | 5.3 | 4.0 |
| 32 | 36.2 | 18.1 | 9.0 | 6.0 | 4.5 |
| 36 | 40.7 | 20.4 | 10.2 | 6,8 | 5.1 |
| 40 | 45.2 | 22.6 | 11.3 | 7.5 | 5.7 |

Table 5 shows similar computations for determining signal size from angular size and distance.  These tables are useful in showing how large signals have to be in order to have a particular angular size, and thus a particular conspicuity.

Physical size of extended sources (Feet)

| Angular Size | Distance (nautical mile) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| (min arc) | 0.5 | 1.0 | 2.0 | 3.0 | 4.0 |
| 5 | 4.4 | 8.8 | 17.7 | 26.5 | 35.4 |
| 6 | 5.3 | 10.6 | 21.2 | 31.8 | 42.4 |
| 7 | 6.2 | 12.4 | 24.8 | 37.1 | 49.5 |
| 8 | 7.1 | 14.1 | 28.3 | 42.4 | 56.6 |
| 9 | 8.0 | 15.9 | 31.8 | 47.7 | 616 |
| 10 | 8.8 | 17.7 | 35.4 | 53.1 | 70.7 |
| 12 | 10.6 | 21.2 | 42.4 | 63.7 | 84.9 |
| 14 | 12.4 | 24.8 | 49.5 | 74.3 | 99.0 |
| 16 | 14.1 | 28.3 | 56.6 | 84.9 | 113.2 |
| 18 | 15.9 | 31.8 | 63.7 | 95.5 | 127.3 |
| 20 | 17.7 | 35.4 | 70.7 | 106.1 | 141.4 |
| 25 | 22.1 | 44.2 | 88.4 | 132.6 | 176.8 |
| 30 | 26.5 | 53.1 | 106.1 | 159.2 | 212.2 |

Ideally one would like to provide signals with maximum conspicuity.  The largest signals in the laboratory experiment, 21.0 and 28.0 min arc, which showed the best conspicuity, may not be practically achieved in the field.  Table 5 shows a signal subtending 25 min arc at 2.0 nautical miles must have a physical size of more than 88 feet.

Table 4 and Table 5 can be used in the design of new aids to navigation.  As an example, assume that we wish to improve the conspicuity of an important aid.  The light is a quick flash viewed against a moderate background.  From Figure 23, or the slopes and intercepts of Table 3, a 5 min arc (nearly point source) flashing light has a conspicuity of 0.19.  To improve the conspicuity by a factor of two to 0.38, Figure 23 or Table 3, show that a 13 min arc signal is required. Given a viewing distance of 1.0 miles, Table 5 shows that the signal would have to be about 23 feet tall.  For a viewing distance of 2.0 miles, the signal would have to be about 46 feet tall.

Application of Extended Sources

Extended sources can provide a conspicuity advantage over point sources in most applications where background lighting is present.  Extended sources may see their greatest benefit as range lights, where it is necessary to acquire and reacquire a light several times during a single transit.  The conspicuity advantage of extended sources means that less lime must be spent in searching for the signal.  In the case of range lights, this may result in a real improvement in navigation safety by enabling mariners to spend more lime piloting their vessel than in searching for a navigation light, especially in a meeting traffic situation.

Generality of Backgrounds

The laboratory experiment used simulated backgrounds for measuring times to locate various sized signals.  The generality of the data reported here depends on whether or not the simulated backgrounds can be considered a valid representation of real backgrounds.  Worthey (1988) [2] measured the background lights of New York Harbour from three vantage points to provide histograms of background light density and illuminance.  Measurements were made from Sandy Hook looking north through the Verrazano Narrows, as well as from Governor's Island looking both north toward Manhattan and south toward New Jersey and Staten Island.

Over the areas that were measured, Worthey reported that densities of lights varied between 1.8 to 2.4 lights per square degree.  In the present experiments, the number of background lights was limited to 400 randomly placed in a 240 square degree area to give a density of 1.7 lights per square degree.  This density is comparable to that of New York Harbour.

Extended Source Orientation

In a background of only point sources, extended sources oriented horizontally, vertically or obliquely should have the same conspicuity.  In harbours all background lights are not point sources. Lights in windows of buildings, reflections off light posts, and reflections off calm water, to name a few, are typically oriented either horizontally or vertically.  During one evening of the field experiment we found that the vertically oriented reflections off the calm water were occasionally mistaken for light signals.

White orientation of the signal lights was not an experimental variable, the impression was formed that extended sources oriented obliquely would be better than those oriented vertically or horizontally because there would be less confusion with some of the background lights.

CONCLUSIONS

Extended sources provide a substantial improvement in conspicuity over that of point sources.  Signal conspicuity depends on angular size of the signal and the density of background lighting against which the signal is viewed.  As signal size increases, conspicuity also increases.  As the density of background lighting increases, conspicuity decreases.  The density of background lighting reduced the conspicuity of small signals more than that of large signals.  Flashing enhanced the conspicuity of small targets more than large targets,

Extended sources will improve the conspicuity of Aids to Navigation in situations where background lighting is a problem.  The practical application of extended sources depends on the distances over which a conspicuity improvement is required.  Extended sources provide adequate conspicuity improvement when they exceed about 8 min arc in angular subtense.  This corresponds to an extended source length of more than 14 feet when viewed from one nautical mile or 28 feet when viewed from two nautical miles.  As distance increases the physical size of extended sources must increase to obtain a given angular size.

Extended sources will be most effective as range lights where mariners must acquire and reacquire the light many times to make a successful transit.

REFERENCES

- NAS/NRC Working Group.  The effects of background lighting on the usefulness of navigation lights.  Coast Guard Office of Research and Development Report, October 1971.

Worthey, J. A. Lights of New York Harbour.  National Bureau of Standards Internal Report, NBSIR 88-3807, June 1988.

REPORT OF PWM FLICKERING LIGHT EXPERIMENT IN ACTUAL SEAS

Japan Coast Guard

OBJECTIVE

20 years have passed already since LED lanterns, which have more visibility and conspicuity than existing lanterns, were adopted as lights for aids to navigation.  Meanwhile, the illuminating power of light at night has increased in the metropolitan area as a centre of activity.  And given these background lights, more advanced measures were expected for certainty of visibility of lights for aids to navigation.  But simply increasing the illuminating power of light is not an option because the power to be used for aids to navigation is limited.

But an LED light source, with advanced light emission response, can be configured freely with a light emission pattern (the light modulated as to its flashing time, circle and flashing width of pulse emission- PWM flickering light), according to a report about how conspicuity can be improved with lights formed by a group of pulses without consuming more electricity.  (‘Research and Study of How LED Lamps Look’, 2006 to 2008.)

Preparation of this report was based on the evaluation of visibility and troublesomeness of seeing lights for vessels in general navigating in actual seas, and the information was collected through questionnaires and analysed.  In addition, the results of observation by preparatory personnel of this report is reported as well in order to clarify the relationship between aids to navigation, which were used for the experiment, and lights in the background or visual obstacles.

PERIOD

Period during which surveys by questionnaire were conducted.

Between December 22nd (Tue.) 2009 and February 1st (Mon.) 2010.

SUBJECT

Yokohama Daikoku Futo Funadamari Breakwater Light (aids to navigation under jurisdiction of Yokohama Coast Guard Office).

Light characteristics: Fl 4.0 sec, red (see Figure 24 and Photograph 1).

METHOD

The surveys were conducted for vessels, which use Yokohama Daikoku Futo Funadamari Breakwater Light (hereinafter called ‘Funadamari Breakwater Light’) or which pass the area where the light is visible.300 or more questionnaire forms were mailed to 29 organizations including government and municipal offices, and 113 responses (108 were valid) from about 20 organizations were received by February 12th either via mail or fax.  The contents of the questionnaire included:

- Date of seeing the light.

Evaluation of visibility of the light (discrimination).

Unpleasantness of seeing the light.

Evaluation for conspicuity of the light.

These were the questions and the questionnaire also asked for comments and opinions.

MISCELLANEOUS

Besides the use of the questionnaire survey, the flickering light installed in the Funadamari Breakwater Light, the area surrounding the Funadamari Breakwater Light and the conditions of the various light sources in the background were observed and studied from ‘Hakuko’, a lighthouse patrol vessel of Yokohama Coast Guard Office, between 16:30 and 18:00 on January 21st (Thur.), 2010.

Lighthouse locations and surrounding areas

Funadamari Breakwater Light

The photograph on the right is the lantern using flickering light

Funadamari Breakwater Light

Color/light: Red

Light characteristics: Fl 4.0 sec.

Height: Approx. 9.8 m

RESULTS OF THE QUESTIONNAIRE

Question 2

Results on the visibility of the light (discrimination) are shown in Figure 25, Figure 26 and Table 6.

Evaluation of visibility (discrimination)

Visibility (discrimination) per distance

Total data on visibility (discrimination)

| Q 2 -② Did you spot it sooner than before? |  |  |
| --- | --- | --- |
|  | No. of people | % |
| 1 Yes | 52 | 92.9 |
| 2 No | 4 | 7.1 |

Positive responses regarding the effectiveness of flickering light are categorized in 1, 2 and 3 in Figure 25 as 33.9%, 14.8% and 16.5%, respectively.  Total is 65.2%.  But category 4 is 26.1% who see the effectiveness of the flickering but expressed insufficiency.  Category 5 is 8.7% who see no effectiveness of the flickering and expressed no change from existing lantern.  People in categories 1 and 2 showed the distance from the light (Figure 26).  The majority of the distance was from 1.0 to 1.5 nautical miles, which is 37.7%, and from 0.5 to 1.0 nautical miles, which is 30.2%. 1.5 nautical miles or more is 30.2% including 2.0 nautical miles or more.  Another question was if they could spot it sooner than before at that time; 92.9% had a positive response (Table 6).

Question 3

Results on unpleasantness of the light shown in Figure 27 and Table 7.

Evaluation of unpleasantness of the light

Total data on unpleasantness

| Q3 - ① The distance |  |  | Q3 - ② How unpleasant |  |
| --- | --- | --- | --- | --- |
| Nautical mile | No. of people |  |  | No. of people |
| Up to 0.5 | 1 |  | 1 | 1 |
| 0.5 to 1.0 | 4 |  | 2 | 0 |
|  |  |  | 3 | 4 |
| Table 7a |  |  | Table 7b |  |

*1 among 6 who expressed unpleasantness had no response to 1 and 2

As shown in 0, 5.6% expressed unpleasantness of the light of the Funadamari Breakwater Light, and the other 94.4% expressed no unpleasantness.  The details for those who expressed unpleasantness are shown in Table 7a and Table 7b.  One of them expressed ‘troublesome’ of the light, and the other 4 expressed strangeness of the new pattern of the light.  None expressed sick feeling.  The distance was 0.5 nautical miles for those who expressed ‘troublesome’, and 500 m, 0.5 nautical miles and 1.0 nautical mile for the other 4, which shows no relations between unpleasantness/strangeness and distance.

Question 4

Results on conspicuity as shown in Figure 28 & Figure 29.

Evaluation of conspicuity

Conspicuity per distance

Categories 1 and 2 are positive responses to the flickering lights in Figure 28 as well as Q2.  The totals are 53.8%, 37.7% and 16.1%, respectively.  But category 3 is 31.1% who see the effectiveness of the flickering but expressed insufficiency.  Category 4 is 15.1% who see no effectiveness of the flickering and see no change from the existing nearby lights.  Categories 1, 2 and 3 indicated the distance in Q4, and categories 1 and 2 are separated from category 3 (Figure 29). From 1.0 to 1.5 nautical miles (35.1%) is the majority for categories 1 and 2 as well as Q2 with similarity in the distributions.  From 0.5 to 1.0 nautical mile is the majority in category 3.  The distance tends to be shorter but response was negative.

Summary of Comments

Positive responses and negative responses are mixed in the evaluations and descriptions in category 2 of Table 8.  The overall responses came out positive as 61.5% and negative as 38.5%, if the positive and the negative in category 2 are to be 7.7% and 7.7%, respectively, and calculated.

Results and categories on comments

| Comments | No. of people | % |  |
| --- | --- | --- | --- |
| 1 Positive | 21 | 53.8 | 61.5 |
| 2 Mix of positive and negative | 6 | 15.4 |  |
| 3 Negative | 12 | 30.8 | 38.5 |
| 4 Others | (2) | - | - |

Basis of category

- Positive: The answer was 1, 2 or 3 in Q2 or 1 or 2 in Q4 (one way or the other) and had mainly positive comments or requests.

Mix of positive and negative: The answer was 1, 2 or 3 in Q2 and 1 or 2 in Q4 but had negative comments.  The answer was 4 in Q2 and 3 in Q3 but had no negative comments.

Negative: The answer was 5 in Q2.  The answer was 4 in Q2 and 3 in Q3 but had negative comments.

Others: Comments without evaluations (not included in calculations)

Analysis of results of the questionnaire

- ’Visibility (discrimination)’ in Q2 and ‘Conspicuity’ in Q4 were asked to determine how much users acknowledge the effectiveness of the flickering.  The overall result was that an average of about 60% of users saw the effectiveness and about 30% recognized ‘blinking’ of flickering light but expressed its insufficiency and about 10% saw no effects.

Most users recognized the Funadamari Breakwater Light at a distance of 1.0 to 1.5 nautical miles.  This may be because the maximum distance is only about 1.5 nautical miles from the Funadamari Breakwater Light to vessels on the W side of Bay Bridge in Yokohama port, thus most users may have seen the light within this range and been able to make an evaluation.

Users who evaluated at 1.5 nautical miles or more may have seen the light from the direction of the entrance to the Yokohama passage on the east side of Bay Bridge.

The light can be seen only between the Bay Bridge piers and the wharf from this direction, and the other lights, such as berth lights, make it difficult to spot the light.  But some users saw the light at a distance of 2 or more nautical miles, and that indicates the possibility of long distance recognition.

The majority of the distance range is from 0.5 to 1.0 nautical mile for users who answered ‘3’ in Q4.  This indicates that the brightness of the background lights is different depending on the direction of the view.

Among those who expressed ‘unpleasantness’ in Q3, one user expressed ‘troublesome’ and four expressed ‘strangeness’, but few expressed ‘unpleasantness’ on the actual seas.

61.5% had positive comments and opinions which show similar results of 65.5% in Q2 and 53.8% in Q4.

RESULTS OF VISIBILITY STUDY AT NIGHT

Routes of study: Started from Yokohama Coast Guard Office, ① Rin-ko Park offshore (about 1.3 nautical miles of the Dock Light), Yokohama kita Suitei harbor Light, ② Izuta town offshore (about 1 nautical mile of the Dock Light) North side of Mizuho Warf, ③ North side of Daikoku Bridge (about 0.8 nautical miles of the Dock Light), ④ East side of Yokohama Bay Bridge (about 1.2 nautical miles of the Dock Light) and back to Yokohama Coast Guard Office (see Figure 24).

Conditions of visibility of the flickering light

From Rin-ko Park

Photograph 2 and Photograph 3 show the view of the Dock Light from around Yokohama Kita Suitei harbor Light.  The lights around the warehouses at Daikoku Warf, Daikoku JCT of Metropolitan Expressway and road/bridge lamps of Daikoku Ohashi are conspicuous.  The flicker light blinks very differently from others and is easy to spot.  The glare of berth lights in the background may cause difficulty in distinguishing the flickering light depending on the direction.

The berth lights of Daikoku Warf 	Yokohama Bay Bridge

View of the Dock Light from around Yokohama Kita Suitei harbour light

(Dock Light in the red circle.  The same shall apply hereinafter.  Taken at intervals between flashes)

Road/bridge lamps of Daikoku Ohashi

View of the Dock Light from around Yokohama Kita Suitei harbour light - Taken at intervals between flashes

Conditions of the view from the direction of Izuta Town

Photograph 4 shows the view of the Dock Light from Izuta Cho offshore.  It was relatively easy to spot the Dock Light from the direction where the photo was taken.  But there is a large area and a wide range of views, in which the glare of the berth lights is always in view and causes difficulty in distinguishing the light.

Berth lights of Daikoku WarfBerth	Bay Bridge	lights of Shin Yamashita Warf and Honmoku Warf

View of the Dock Light from Izuta Cho offshore - Taken during flash

Conditions of the view from the direction of Daikoku Ohashi

Photograph 5 shows the view of the Dock Light from Daikoku Ohashi at a distance of 0.2 nautical miles.  It was easy to spot the Dock Light since it was close but also close to the berth lights of Honmoku Warf which may be confused with the Dock Light.

Bay Bridge	Berth lights of Shin Yamashita Warf and Honmoku Warf

View from the direction of Daikoku Ohashi - Taken at intervals between flashes)

Photograph 6 shows the view from a distance of 0.4 nautical miles. Berth lights stayed out of sight and the lights of downtown Yokohama became visible in the background.  Flickering light was very visible.

Downtown Yokohama lights	Landmark tower

View from a distance of 0.4 nautical miles - Taken during flash

Photograph 7 shows the view of the Dock Light from a distance of 0.6 nautical miles at the pier of Daikoku Ohashi after passing through it.  Downtown Yokohama lights completely overlapped with the Dock Light. Bridge lamps in front stood out very much, but the flicker light was easy to spot.  But the Dock Light moved behind the warehouse from the view about 0.8 nautical miles away.

When entering Yokohama Ko from this direction, the Dock Light, which is a relatively short distance away, can be used after passing through.

Dakoku Ohashi and bridge lamps	Downtown Yokohama lights

View of the Dock Light from a distance of 0.6 nautical miles - Taken at intervals between flashes)

Conditions of the view from the direction of Yokohama Passage

Photograph 8 shows the view of the Dock Light from the direction of 0.5 nautical miles right under the Yokohama Bay Bridge.  No other background lights make it possible for the light to be spotted.

Bay Bridge

Bay Bridge -Taken during flash

Photograph 9 shows the view of the Dock Light from a distance of 1.2 nautical miles.  Berth lights in front came into view. It is difficult to distinguish the Dock Light without blocking the glare.

Bay Bridge Berth lights	Daikoku Warf

Dock Light from a distance of 1.2 nautical miles - Taken at intervals between flashes

Results of visibility study at night

The Dock Light lies between the pier of Bay Bridge and Daikoku Warf from the view from the direction of the entrance to Yokohama Passage, and glare caused by bridge lamps and berth lights may interfere with distinguishing the Dock Light.

Berth lights of Honmoku Warf and Daikoku Warf sometimes overlap the Dock Light in the view from Izuta Town offshore or around Daikoku Ohashi, and reduce the effectiveness of flickering.

The berth lights look like a belt of lights without dark spots. It takes some amount of time to distinguish the Dock Light, if it gets lost in the belt of lights.  These berth lights could be the big disincentive for discriminating aids to navigation lights in Yokohama Port.

Road lamps and bridge lamps are strong background lights as well. But the Dock Light can be found in no time even if it gets lost, because these lamps are lined up in order and have some dark spaces in between.

Downtown lights are mostly white with some red and blue, etc. but they are mostly fixed lights.  The flicker light is a totally different object, in a sense among others, and it was relatively easy to distinguish it.

The Dock Light didn’t quite stand out among other nearby aids to navigation lights, but overall it was more noticeable than other lights at a distance of 1.0 to 1.5 nautical miles.

When passing near the Dock Light (about 300 m), it didn’t cause unpleasantness by looking at the flicker light directly.

SUMMARY

- According to the results of the questionnaire about the flickering light, the ratios of ‘Effective’, ‘Insufficient’ and ‘Not effective’ are about 60%, 30% and 10% respectively.

Many of the users may have evaluated from the light on the W side of Bay Bridge within the port that its distance is about 1.0 to 1.5 nautical miles, according to the answers to the questionnaire.  The users who have evaluated the light at a distance of 1.5 or more nautical miles may seem to be on the E side of Bay Bridge.  But the Dock Light can be seen between the pier of Bay Bridge and Daikoku Warf from this direction, and there are many existing obstacles such as bridge lamps, berth lights, etc.

Many of the users who concluded as ‘Insufficient’ or ‘Not effective’ are speculated to have evaluated from this direction, and the users who concluded as ‘Effective, at 1.5 nautical miles or more’, remains only 30 to 35%, which is understandable.  Considering that it was conducted during operations and within regularly used sea areas, these numbers are not quite negative.

We really appreciate that these users, who conscientiously evaluated from these directions, cannot see the Dock Light well.

From the direction of downtown lights in the background, the flicker light can be seen as a totally different kind of light, and is effective in the visibility study at night. But from the direction of the berth lights in the background, it was confirmed that the glare of the lights could be a factor that decreases the evaluation significantly.

If the main reason the Dock Light does not ‘stand out’ as effectively as expected from the short distance was the glare of other lights, then according to the results of the questionnaire, ‘Effective, Insufficient, Not effective’, which were 60%, 30% and 10%, respectively, were quite reasonable as results of a test using a 2 nautical mile model lamp.

There was no user who felt sick from looking at the flickering light directly.  As far as the 2 nautical mile model lamp (Fl 4.0 sec) is concerned, the flicker light is not likely to influence human health, etc. and few users even come close enough to be influenced (the minimum distance was 400 m as reported).

The following should be considered to have been based on the results of the experiment analysis:

Installing the flicker light in the aids to navigation at the entrance to the passage in order to focus on the direction to see the object and easily spot it from a distance (e.g. Yokohama Passage No. 1 or No. 2 Offshore fixed lights at Yokohama Passage).

Installing in aids to navigation, which are closer to downtown, in order to make it is easy to see under strong background lights (e.g. Yokohama Kita Suitei Light, Yokohama Higashi Suitei Lighted Buoy).

Study the effectiveness of several flickering lights within the same sea area (at the entrance to the passage and in the inner port in the case of the above mentioned aids to navigation).

Scintillation is caused by a temperature difference between the air and the sea. Its effect is experienced as a random flickering of a distant light.

The separation between lights minimized the effects of spatial summation.  Spatial summation is the process whereby nearby lights sum their brightness to create an area brighter than that of the individual lights.  Spatial summation at luminances similar to that of this experiment occurs over 3-4 min arc.  The separations chosen for this experiment ensure that no areas of high brightness compete for the attention of the observer.

A Logarithm (base 10) transformation makes the reaction time distributions closer to normal distributions, reducing the skewness and making variances homogeneous.  Normal distribution and homogeneous variance are prerequisites for performing parametric statistics.

There were many instances where the signal was not found in the 20 second search tune.  When the signal was not found, reaction time was recorded as 20 sec even though it had to be greater than 20 sec.  This has the effect of overestimating conspicuity of the signals that were most difficult to find. The greatest impact was on signals with conspicuities less than 0.1.  Therefore conspicuity indexes less than 0.1 are not included in the fits.