Draft IALA S-247

Product Specification

Working Draft 0.0.1 – September 2018

IALA S-247 Differential eLoran Reference Station Almanac Product Specification

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

1 Overview	6

1.1 Introduction	6

2 References	6

2.1 Normative references	6

2.2 Informative references	6

3 Terms, definitions and abbreviations	6

3.1 Terms and definitions	6

3.2 Abbreviations	6

4 Product specification metadata	7

4.1 IALA Product Specification Maintenance	7

4.1.1 Introduction	7

4.1.2 New Edition	7

4.1.3 Revisions	7

4.1.4 Clarification	8

4.1.5 Version Numbers	8

5 Specification Scope	8

5.1 Metadata content	9

5.2 Dataset Identification	9

6 Data Content and structure	9

6.1 Introduction	9

6.2 Application Schema	9

6.2.1 Meta features application schema	18

6.3 Feature Catalogue	19

6.4 Feature Types 	20

6.5 Data Product Types	20

6.5.1 Dataset rules
	20

6.6 Geometry 	21

6.6.1 Co-ordinate Reference Systems (CRS)	21

6.6.1.1 Introduction 	21

6.6.1.2 Horizontal Geodetic Datum 	22

6.6.1.3 Vertical Datum 	22

6.6.1.4 Units of Measure	22

7 Data Quality	22

7.1 Introduction	22

7.2 Data Compliance and Integrity	22

7.3 Uncertainty and lineage information	22

7.3.1 Positional accuracy	22

7.3.2 Lineage	23

8 Data Capture and Classification	23

9 Data Maintenance
	23

9.1 Introduction	23

9.2 Maintenance and Update Frequency	23

10 Data Product format (encoding)	23

10.1 Introduction	23

10.2 Encoding of Latitude and Longitude 	23

10.2.1 Encoding of coordinates as decimals	24

10.3 Numeric Attribute Encoding	24

10.4 Text Attribute Values 	24

10.5 Mandatory Attribute Values 	24

10.6 Unknown Attribute Values 	24

10.7 Structure of dataset files	25

10.7.1 Sequence of objects	25

10.8 Object identifiers	25

10.9 Dataset validation	25

10.10 Location of Data Product Format schema Files	25

10.11 Detailed documentation of  schema	26

11 Data Product Delivery 	26

11.1 Exchange set	26

11.2 Dataset	28

11.2.1 Datasets 	28

11.2.2 Dataset size	28

11.2.3 Dataset file naming 	28

11.2.4 New Editions and re-issues	28

11.2.5 Cancellations	29

11.3 Support Files	29

11.3.1 Support File Naming	29

11.3.2 Support file management	30

11.4 Exchange Catalogue	30

12 Metadata	30

12.1 Introduction	30

12.2 Metadata classes	32

12.2.1 S100_ExchangeSet	34

12.2.2 S100_ExchangeCatalogue	34

12.2.3 S100_CatalogueIdentifier	34

12.2.4 S100_CataloguePointofContact	34

12.2.5 S100_Dataset	34

12.2.6 S100_DatasetDiscoveryMetadata	34

13 Language	37

# Overview

## Introduction

Enhanced Loran is an internationally standardized positioning, navigation, and timing (PNT) service for use by many modes of transport and in other applications. It is the latest in the long-standing and proven series of low-frequency, LOng-RAnge Navigation (LORAN) systems, one that takes full advantage of 21st century technology.

eLoran meets the accuracy, availability, integrity, and continuity performance requirements for aviation non-precision instrument approaches, maritime harbor entrance and approach maneuvers, land-mobile vehicle navigation, and location-based services, and is a precise source of time and frequency for applications such as telecommunications.

eLoran is an independent, dissimilar, complement to Global Navigation Satellite Systems (GNSS). It allows GNSS users to retain the safety, security, and economic benefits of GNSS, even when their satellite services are disrupted.

The Product Specification can be used to exchange Differential eLoran Reference Station in a consistent form between stakeholders.

# References

## Normative references

IHO S.100 IHO Universal Hydrographic Data Model, Edition 2.0.0, January 2015

IHO S.63 Data Protection Scheme, Edition 1.2.0, January 2015

ISO 8601:2004 Data elements and interchange formats _ Information interchange _ Representation of dates and times

ISO/TS 19103:2005 Geographic information - Conceptual schema language

ISO 19111:2003 Geographic information - Spatial referencing by coordinates

ISO 19115:2006 Geographic information – Metada	ta (Tech Corr. 1, 2006)

ISO 19115-2:2009 Geographic information - Metadata: Extensions for imagery and gridded data

ISO 19131:2007 Geographic information - Data product specifications

ISO/IEC 19505-1:2012, Information technology — Open Distributed Processing - Unified Modelling Language Version 2.4.1

International Loran Association: 2007 Enhanced Loran (eLoran) Definition Document

RTCM SC 127 Minimum Performance Standards for Marine eLoran Receiving Equipment

## Informative references

ISO, 2006. ISO 19109 Geographic Information - Rules for Application Schema.

ISO, 2007. ISO 19135 Geographic Information – Procedures for Item Registration.

ISO, 2009. ISO 19136 Geographic Information - Geography Markup Language (GML).

IMO, 2008. Safety of Navigation Circular SN/Circ.243

IALA, 2012. Guideline 1088 on an Introduction to Preparing S-100 Product Specifications

# Terms, definitions and abbreviations

## Terms and definitions

Terms and definitions have been taken from the normative references cited in clause 1.1.

## Abbreviations

AtoN	Aid to navigation

CRS		Coordinate Reference System

ECDIS		Electronic Chart Display and Information System

eLoran	Enhanced Loran

e-Navigation	Enhanced Navigation (maritime)

EPSG		European Petroleum Survey Group

ENC		Electronic Navigational Chart

FTA	Finnish Transport Administration

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

ICAO	International Civil Aviation Organization

IHO		International Hydrographic Organization

IMO		International Maritime Organization

ISO		International Organization for Standardization

Loran	LOng-RAnge Navigation

PNT	Positioning, Navigation, and Timing

WWRNS	World Wide Radionavigation System

XML	Extensible Markup Language

# Product specification metadata

| Title | Differential eLoran Reference Station Almanac |
| --- | --- |
| S-247 version | 0.0.1 |
| S-100 version | 3.0.0 |
| Date | 28 September 2018 |
| Language | English |
| Identifier | S-247 |
| Classification: | 001 - unclassified |
| Contact: | IALA-AISM 10, rue des Gaudines 78100 Saint Germain en Laye, France Telephone: +33 1 34 51 70 01 Fax: +33 1 34 51 82 05 |
| URL: | iala-aism.org |
| Maintenance: | The product specification is maintained by IALA-AISM and amendments are performed on a needs base, up to maximum one new release per calendar year. |

## IALA Product Specification Maintenance

This chapter is for clarification only on PS Maintenance.

### Introduction

Changes to a product specification will be released by IALA-AISM as a new edition, revision, or clarification.

### New Edition

New editions of a product specification introduce significant changes. New editions enable new concepts, such as the ability to support new functions or applications, or the introduction of new constructs or data types.

### Revisions

Revisions are defined as substantive semantic changes to a product specification. Typically, revisions will change a product specification to correct factual errors; introduce necessary changes that have become evident as a result of practical experience or changing circumstances.  A revision must not be classified as a clarification.  Revisions could have an impact on either existing users or future users of a product specification.  All cumulative clarifications must be included with the release of approved corrections.

Changes in a revision are minor and ensure backward compatibility with the previous versions within the same edition.  Newer revisions, for example, introduce new features and attributes.  Within the same edition, a data product of one version could always be processed with a later version of the feature and portrayal catalogues.

### Clarification

Clarifications are non-substantive changes to a product specification. Typically, clarifications: remove ambiguity; correct grammatical and spelling errors; amend or update cross references; insert improved graphics, spelling, punctuation and grammar.  A clarification must not cause any substantive semantic change to a product specification.

Changes in a clarification are minor and ensure backward compatibility with the previous versions within the same edition.  Within the same edition, a data product of one clarification version could always be processed with a later version of the feature and portrayal catalogues, and a portrayal catalogue can always rely on earlier versions of the feature catalogues.

### Version Numbers

The associated version control numbering to identify changes (n) to a product specification must be as follows:

New editions denoted as n.0.0

Revisions denoted as n.n.0

Clarifications denoted as n.n.n

# Specification Scope

This product specification describes one data product and therefore requires only one scope which is described below:

Scope ID:  		Differential eLoran Reference Station Almanac datasets.

Hierarchical level: 		MD_ScopeCode - 005

Hierarchical level name: 	dataset.

Level description:		information applies to the dataset

Extent:	EX_Extent.description: Global coverage of maritime areas

## Metadata content

Mandatory elements from S-100 3.0.0 11-6:

geographicDescription, spatialResolution, purpose (ex specificPurpose), language)

Optional elements: useLimitation, classification, spatialRepresentationType, point of contact.

## Dataset Identification

| title | Differential eLoran Reference Station Almanac |
| --- | --- |
| abstract | An Differential eLoran Reference Station is a vector dataset containing all relevant information regarding the eLoran Reference Station within a defined geographical area. |
| acronym | Differential eLoran Reference Station Almanac |
| content | Differential eLoran Reference Station information, such as characteristics, status, location, maintenance of the Differential eLoran Reference station. |
| spatialExtent | Description: Global East Bounding Longitude: -180 West Bounding Longitude: 180 North Bounding Latitude: 90 South Bounding Latitude: -90 |
| temporalExtent | Datasets are valid till a new version is released. |
| specificPurpose | Differential eLoran Reference Station Almanac are produced to allow the producer to exchange Differential eLoran Reference Station information with interested stakeholders. |

# Data Content and structure

## Introduction

An Differential eLoran Reference Station dataset is a feature-based product.  The following sub-sections contain the product application schema expressed in UML and an associated feature catalogue.  The feature catalogue provides a full description of each feature type including its attributes and attribute values in the data product.

## Application Schema

The application schema conforms to the modelling conventions of UML as constrained in S-100 Part 1, and conforms to the General Feature Model described in Part 3.

An overview of the application schema is provided in the following figure (Figure 1). The subsequent figures provide details for feature types and their relationships.

- The feature type DLoranStation is a type which is used to indicate DLoran station.

- Reference-referenceby associations are modelled by the association labelled ReferenceReferenceby, between classes DLoranStation and DLoranStationAlmanac in Figure 1 below.

- The information type DLoranStationAlmanac is a type which is used to indicate a characteristics in the DLoran Reference station.

Figure 1. Application schema of Differential eLoran Reference Station Almanac

## Feature Catalogue

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  It also assigns the geometric primitives. The S-247 Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema and can be downloaded from the IALA website (iala-aism.org).  A printed version of the feature catalogue is provided in Annex X.

Name: Differential eLoran Reference Station Almanac

Scope: Ocean, Coastal, Ports, Harbors and Inland waters

Version Number: 0.0.1

Version Date: 2018-09-28

Producer:

IALA-AISM

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01    Fax: +33 1 34 51 82 05

URL : https://iala-aism.org

Language: English

## Feature Types

Feature types contain descriptive attributes and do not contain any geometry (i.e. information about the shape and position of a real world entity). Features have two aspects – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

S-247 makes use of the following feature types:

Geographic (Geo) feature type – carries the descriptive characteristics of a real world entity.

Relationship feature type – A feature relationship links instances of one feature type with instances of the same or a different feature type.

Meta features – Meta features contain information about other features within a dataset. Information defined by meta features overrides the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

## Data Product Types

A Dataset is a grouping of features, attributes, geometry and metadata which comprises a specific coverage. eLoran Station Almanac data is scale independent, but may use the scaleMinimum and scaleMaximum attributes of S100_GF_SpatialAttributeType to declutter the data in certain viewing scales. The use of scaleMinimum and scaleMaximum is at the discretion of the data producer.

## Geometry

The specifications for S-247 geometry are the same as for S-101 geometry except that 3-D geometry is not permitted even for sounding features since these features are not part of the S-247 application schema. The relevant (modified) extract from the S-101 product specification is reproduced below.

The underlying geometry of an S-247 dataset is constrained to level 3a which supports 0, 1 and 2 dimensional features (points, curves and surfaces) as defined by S-100 Part 7 – Spatial Schema.

Level 3a is described by the following constraints:

- Each curve must reference a start and end point (they may be the same).

- Curves must not self intersect.

- Areas are represented by a closed loop of curves beginning and ending at a common point.

- In the case of areas with holes, all internal boundaries must be completely contained within the external boundary and the internal boundaries must not intersect each other or the external boundary. Internal boundaries may touch other internal boundaries or the external boundary tangentially (i.e. at one point).

- The outer boundary of a surface must be in a clockwise direction (surface to the right of the curve) and the curve orientation positive. The inner boundary of a surface must be in a counter-clockwise direction (surface to the right of the curve) and the curve orientation negative.

S-247 further constrains Level 3a with the following:

- Coincident linear geometry must be avoided when there is a dependency between features.

- The interpolation of GM_CurveSegment must be loxodromic.

- Linear geometry is defined by curves which are made of curve segments. Each curve segments contains the geographic coordinates as control points and defines an interpolation method between them. The distance between two consecutive control points must not exceed 0.3 mm at a display scale of 1:10000.

The following exception applies to S-247:

- The use of coordinates is restricted to two dimensions.

- Soundings features which use GM_Point or GM_Multipoint with three dimensional coordinates are not currently included in S-247.

### Co-ordinate Reference Systems (CRS)

#### Introduction

The single coordinate reference system is separated into the horizontal and vertical components.

#### Horizontal Geodetic Datum

WGS84 (World Geodetic System of 1984) must be used for the horizontal reference system for spatial data. WGS84 must be used as the reference ellipsoid.

#### Vertical Datum

Vertical datum shall be selected from the list in verticalDatum enumeration. If the local datum value is used, additional information must be provided using the comment attribute in the metadata, or information attribute on the feature instance. Positive value is above vertical datum and negative is below vertical datum.

# Data Quality

## Introduction

Data Quality is considered to be meta information and for S-247 it is divided into two parts:

- Compliance and integrity

- Uncertainty and lineage

## Data Compliance and Integrity

S-247 datasets must be validated using the conformance checks that are listed in S-58. The data quality elements listed in S-100 Part 4C that are applicable to S-247 are indicated in the table of conformance checks listed in S-58.

Datasets must not be published unless they pass all the compliance checks designated as “Critical”.

The detailed results of applying compliance checks listed in S-58 are not required to be reported as part of the exchange set. They may be conveyed as support files or by separate arrangement.

## Uncertainty and lineage information

### Positional accuracy

S-247 positional accuracy data quality indicators do not use the model of data quality data quality elements defined in S-100 Part 4c.

However, positional accuracy must be evaluated and must be indicated in dataset metadata or spatial attribute metadata as provided in the application schema. This specification does not prescribe a specific required level of positional accuracy.

### Lineage

Lineage and process step information elements are not required to be present in S-247 datasets distributed to end user. They may be included as extra metadata in exchange sets distributed to vendors and distributors.

Required source information about S-247 datasets is limited to the source and distribution information contained in discovery metadata described in S-100 Part 4a and Section 12 of this product specification.

# Data Capture and Classification

The Data Capture and Classification (DCEG) is found in Annex X

# Data Maintenance

## Introduction

Datasets are replaced by new editions that must be considered a whole replacement of the previous version. Delta change datasets are not permitted. Each edition of a dataset shall have an edition number that is greater than the previous version by one. First edition of a dataset shall have edition number set to 1.

## Maintenance and Update Frequency

Datasets are maintained as needed in accordance with data producer policy.

# Data Product format (encoding)

## Introduction

The GML encoding of S-247 datasets is based on the S-100 profile of GML 3.2.1. This is described in S-100 Edition 3.0.0 Part 10b.

Detailed documentation of the S-247 encoding schema is provided in Annex X of this document.

Format Name: GML, Specification: S-100 profile of GML Part 10b.

File Structure: S-247 (IALA Differential eLoran Reference Station Almanac Product Specification), Annex X.

## Encoding of Latitude and Longitude

Values of latitude and longitude must be expressed with a precision of 9 decimal places. Coordinates must be encoded as decimals in the format described below. The encoding is indicated by multiplication factor fields defined in the dataset identification record by the S-100 GML schemas.

### Encoding of coordinates as decimals

Values should be coded as decimal numbers with 7 or fewer digits after the decimal. The normative encoding is in degrees, with an accuracy of 10-7 degrees, i.e., 7 digits after the decimal point.

The decimal point must be indicated by the “.” character.

Trailing zeroes after the decimal point (and the decimal point itself if appropriate) may be omitted at producer discretion, but the accuracy must still be as indicated (e.g., 10-7 degrees for coordinates of default accuracy).

Latitude and longitude multiplication factors held in the Dataset Structure Information field under [coordMultFactorX] and [coordMultFactorY] must be set to a value corresponding to the encoding, i.e., {1} for coordinates encoded in decimal degrees.

EXAMPLE 1	A longitude = 42.0000 is converted into X = longitude * coordMultFactorX = 42.0000 * 1 = 	42.0000000.

## Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

## Text Attribute Values

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

## Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue and summarised in Annex X – Data Classification and Encoding Guide.

## Unknown Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

## Structure of dataset files

### Sequence of objects

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

S-247 Collection objects

## Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

## Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

## Location of Data Product Format schema Files

The schema files are available from the locations given below.

| File | Description | Version | Location |
| --- | --- | --- | --- |
| S100_gmlProfile (XSD file) | S-100 GML profile schema | 1.0 | TBD (IHO site) |
| S100_gmlProfileLevels (XSD file) | Compliance levels schema for the S-100 GML 3.2.1 Profile | 1.0 | TBD (IHO site) |
| S100_gmlBase (XSD file) | IHO TSMAD S-100 common components schema for S-100 GML data format | 1.0 | TBD (IHO site) |
| S100_gmlProfileExt (XSD file) | S-247 extensions to the IHO GML profile | 1.0 | TBD (IALA-AISM web site) |
| S100_gmlBaseExt (XSD file) | S-247 extensions to the IHO GML common components | 1.0 | TBD (IALA-AISM web site) |
| S247 (XSD file) | S100 XML/GML Schema for S-247 (IALA Aids To Navigation) | 0.1 | TBD (IALA-AISM web site) |
| (Schematron files for dataset validation) | [To be developed] | TBD | TBD (IALA-AISM web site) |
| S247-1.gml (XML/GML file) | S-247 example | 0.1 | TBD (IALA-AISM web site) |
| OGC GML 3.2.1 schemas | OGC schemas for GML 3.2.1 | 3.2.1 | http://schemas.opengis.net/gml/3.2.1/ |

## Detailed documentation of schema

The detailed documentation of the S-100 Profile of GML Part 10b schema is in Annex B (currently a separate document enclosed with this file).

# Data Product Delivery

This clause specifies the encoding and delivery mechanisms for an S-247 dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

## Exchange set

S-247 datasets are grouped into exchange sets. Each exchange set consists of one or more Differential eLoran Reference Station datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:					Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- Differential eLoran Reference Station Almanac datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the S-247 exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

- Portrayal Catalogue - If it is necessary to deliver the latest portrayal catalogue to the end user it may be done using the S-247 exchange set mechanism for datasets datasets – i.e., include the updated feature catalogue in an exchange set.

## Dataset

### Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces.  A new edition can also be eLoran Station Almanac data that has previously been produced for this area. The encoding structure is located in Annex X.

- Cancellation:  The dataset is cancelled and is deleted from the SYSTEM.  The structure for a cancellation is described in clause 12.2.5.

### Dataset size

Datasets must not exceed 20 MB.

### Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory) in accordance with IHO S-62..

- the third to tenth characters are optional and may  be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- the maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### Cancellations

In order to cancel a dataset, a cancellation dataset is created for which the edition number must be set to 0. Dataset edition number is a field in exchange set metadata, class S100_DatasetDiscoveryMetadata (clause Metadata classes). The cancellation dataset file may contain no data objects (features, information types, etc.) and any data objects present it in are ignored. The cancellation dataset may be part of an exchange set which contains other datasets, e.g., a new edition of a different dataset. This method is only used to cancel a dataset. Where a dataset is cancelled and its name is reused at a later date, the issue date must be greater than the issue date of the cancelled dataset.  When the dataset is cancelled it must be removed from the system.

## Support Files

Dataset support files offer supplementary information that can be included in an eLoran Station Almanac exchange set.

Text files must contain only general text as defined by this standard. (Extensible mark-up language (XML) supports UTF-8 character encoding).Text files may be (TXT), (XML), (HTM), JPEG files.

Support files may be in any of the support file formats defined in the S-100 enumeration S100_SupportFileFormat (defined in Part 4a-D-2.12) except that “Other” may not be used.

### Support File Naming

All support files must have unique universal file identifiers. The file identifier of support information should not be used to describe the physical content of the file. The support file metadata that accompanies the file will inform the user of the name and purpose of the file (i.e. new, replacement and deletion).

In this encoding the support files are named according to the specifications given below:

CCXXXXXXXX.EEE

The main part forms an identifier where:

- the first two characters identify the issuing agency (mandatory)

- the third to tenth characters can be used in any way by the producer to provide the unique file name. The following characters are allowed in the support file name, A to Z, 0 to 9 and the special character _ (underscore)

- .EEE – support file extension. (Must conform to the file format.)

- the maximum number of characters is ten

### Support file management

When a support file is created or a subsequent version is issued it must carry its own issue date and be supported with a digital signature which authenticates it against the producer’s public key included in the exchange set metadata.

The type of support file is indicated in the “purpose” field of the discovery metadata.  Support files carrying the “deletion” flag must be removed from the system.  When a feature pointing to a text, picture or application file is deleted or updated so that it no longer references the file, the system software must check to see whether any other feature referenced the same file, before that file is deleted.

Each support file must be used only once in the exchange set.

Support files should be stored in a separate folder within the exchange set.

## Exchange Catalogue

The exchange catalogue acts as the table of contents for the exchange set. The catalogue file of the exchange set must be named S247_1_0_CAT.XML. No other file in the exchange set may be named S247_1_0_CAT.XML.  The contents of the exchange catalogue are described in Clause 13.

XML schemas for the exchange catalogue are located at http://registry.iho.int/

# Metadata

## Introduction

The metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

The realization of exchange set classes, the exchange set catalogue, and the structure of exchange sets are the same as in S-100. The relevant UML diagrams from S-100 are therefore reproduced here for reference.

Figure 1. Realization of the Exchange Set Classes (from S-100)

Figure 2. Exchange set catalogue and metadata (from S-100)

Since the class S100_DatasetDiscoveryMetadata aggregates ISO 19115 metadata (class S100_19115DatasetMetadata), metadata conforming to ISO 19115 is required to be included. Metadata for each support file is required if the exchange set contains support files.

Figure 3. Exchange set including metadata (From S-100)

ISO 19115 metadata conforms to ISO 19115 with the additional constraints in S-100 Part 4A Appendix 4A-B.1 (Table B.1) which make the fileIdentifier element mandatory.

## Metadata classes

Details for the S-100 metadata classes are depicted in the figure below.

Figure 4. S-100 and S-247 metadata class details

[Refer to S-100 Edition 3.0.0 Part 4A-D.2 for detailed documentation of S-100 metadata. If S-247 differs, then the S-100 classes should be specialized by S-247 classes if possible and the differences indicated (e.g., figure 4a-2 in S-100 which depicts how S-100 constrains the fileIdentifier attribute of MD_Metadata to be mandatory).]

### S100_ExchangeSet

### S100_ExchangeCatalogue

### S100_CatalogueIdentifier

### S100_CataloguePointofContact

### S100_Dataset

### S100_DatasetDiscoveryMetadata

The class S100_DatasetDiscoveryMetadata is not directly used in S-247 metadata. Instead, its specialization S247_DatasetDiscoveryMetadata (clause Error: Reference source not found) is used.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_DatasetDiscoveryMetadata | Metadata about the individual datasets in the exchange catalogue | - | - | S-100 metadata class |
| Attribute | fileName | Dataset file name | 1 | CharacterString |  |
| Attribute | filePath | Full path from the exchange set root directory | 1 | CharacterString | Path relative to the root directory of the exchange set. The location of the file after the exchange set is unpacked into directory <EXCH_ROOT> will be <EXCH_ROOT>/<filePath>/<filename> |
| Attribute | description | Short description giving the area or location covered by the dataset | 1 | CharacterString | E.g. a harbour or port name, between two named locations etc. |
| Attribute | dataProtection | Indicates if the data is encrypted | 0..1 | Boolean | 0 indicates an unencrypted dataset 1 indicates an encrypted dataset |
| Attribute | protectionScheme | specification or method used for data protection | 0..1 | CharacterString | Eg S-63 |
| Attribute | purpose | The purpose for which the dataset has been issued | 1 | MD_Identification>purpose CharacterString | E.g. new, re-issue, new edition, update etc. |
| Attribute | specificUsage | The use for which the dataset is intended | 1 | CharacterString | E.g. in the case of ENCs this would be a navigation purpose classification. |
| Attribute | editionNumber | The edition number of the dataset | 1 | CharacterString | when a data set is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. Edition number remains the same for a re-issue. |
| Attribute | updateNumber | Update number assigned to the dataset and increased by one for each subsequent update | 1 | CharacterString | Update number 0 is assigned to a new dataset. |
| Attribute | updateApplicationDate | this date is only used for the base cell files (i.e. new data sets, re-issue and new edition), not update cell files. All updates dated on or before this date must have been applied by the producer | 0..1 | Date |  |
| Attribute | issueDate | date on which the data was made available by the data producer | 1 | Date |  |
| Attribute | productSpecification | The product specification used to create this dataset | 1 | S100_ProductSpecification |  |
| Attribute | producingAgency | Agency responsible for producing the data | 1 | CI_ResponsibleParty |  |
| Attribute | optimumDisplayScale | The scale with which the data is optimally displayed | 0..1 | Integer | Example: A scale |
| Attribute | maximumDisplayScale | The maximum scale with which the data is displayed | 0..1 | Integer |  |
| Attribute | minimumDisplayScale | The minimum scale with which the data is displayed | 0..1 | Integer |  |
| Attribute | horizontalDatumReference | Reference to the register from which the horizontal datum value is taken | 1 | characterString | EPSG |
| Attribute | horizontalDatumValue | Horizontal Datum of the entire dataset | 1 | Integer | 4326 |
| Attribute | verticalDatum | Vertical Datum of the entire dataset | 1 | S100_VerticalAndSoundingDatum |  |
| Attribute | soundingDatum | Sounding Datum of the entire dataset | 1 | S100_VerticalAndSoundingDatum |  |
| Attribute | dataType | The encoding format of the dataset | 1 | S100_DataFormat |  |
| Attribute | otherDataTypeDescription | Encoding format other than those listed. | 0..1 | CharacterString |  |
| Attribute | dataTypeVersion | The version number of the dataType. | 1 | CharacterString |  |
| Attribute | boundingBox | The extent of the cell limits | 0..1 | EX_GeographicBoundingBox |  |
| Attribute | boundingPolygon | A polygon which defines the actual data limit | 0..* | EX_BoundingPolygon |  |
| Attribute | comment | any additional information | 0..1 | CharacterString |  |

# Language

The exchange language must be English. Other languages may be used as a supplementary option. National geographic names can be left in their original national language using the complex attribute Feature Name.

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8). A BOM (byte order mark) must not be used.

- Data Classification and Encoding Guide

[This is a placeholder. The detailed documentation of the S-247 XML schema is currently in a separate document enclosed with this file.]

- Data Product Format (Encoding)

[This is a placeholder. The detailed documentation of the S-247 XML schema is currently in a separate document enclosed with this file.]

- NORMATIVE IMPLEMENTATION GUIDANCE

IALA to determine if this annex is needed.

- FEATURE CATALOGUE

To be completed later.

- PORTRAYAL CATALOGUE

To be completed later.

- Application Schema Documentation Tables

- S-247 dataset validation rules

To be completed later.