Navguide: Chapter 7 - Power supplies

7.1 Types

A wide range of power systems and energy sources have been used or contemplated for operating lighthouses and floating aids.

Everything from clockwork to radio-active isotopes have been used.

Some of the more common types are listed in Table 35.

| Electric Energy Sources & Storage | Non-Electric Energy Sources |
| --- | --- |
| Utility generated electricity | Acetylene |
| Photovoltaic solar modules | Propane |
| Diesel and petrol engine driven generators | Butane |
| Primary & secondary battery cells | Kerosene |
| Wind generators |  |
| Wave activated generators |  |
| Fuel cells using alcohol or hydrogen |  |
| Thermo nuclear devices |  |

Table 35 - Power Sources for Operating Lighted Aids to Navigation.

The most popular method of powering AtoN is by utility generated electricity if it is available, if it is not available then by solar powered photovoltaic cells charging secondary battery cells.

ByThe

IALA has created a series of documents to assist in the selection of electrical power systems for aids to navigation.

Refer to IALA publications:

- Guideline 1067-0 on Selection of Power Systems for Aids to Navigation and Associated Equipment;

- Guideline 1067-1 on the Total Electrical Loads of Aids to Navigation;

- Guideline 1067-2 on Power Sources;

- Guideline 1067-3 on Electrical Energy Storage for Aids to Navigation.

- Guideline 1039 on Designing solar power systems for AtoN (The Solar Power sizing tool)

Refer to:

•	Applicable national standards for the safe handling of gases.

| Navguide: Chapter 7 - Power supplies | 128 |
| --- | --- |

7.2 Electric - Renewable Energy Sources

7.2.1 Solar Power (Photovoltaic cell)

Solar power is an ideal power source for many aids to navigation applications. It offers:

- a power source with no moving parts;

- no maintenance requirements other than being cleaned;

- slight deterioration in power output over its life;

- low life-cycle costs;

- enhanced electrical safety on extra low voltage systems.

When used to power a light, the battery recharging process is separated from the operation of the light source so that the recharge voltage can be optimized without detriment to the light’s operation.

Potential difficulties associated with solar power are:

- finding ways to minimise bird fouling;

- ensuring power remains reliable during periods of poor weather;

- solar array footprint can be large at high latitudes;

- protecting solar modules from:

  - wave damage on buoys and exposed lighthouses;

  - vandalism and theft; and,

  - Lightning

Photo Courtesy of the Australian Maritime Safety

Authority

Aids to navigation exposed to icing conditions are perhaps the only applications unsuited to the use of solar modules.

Types

The three common technologies employed in the manufacture of silicon based solar modules are listed in Table 36.

| Technology | Comments |
| --- | --- |
| Monocrystaline | Made from a thin slice cut from a single large crystal of silicon, usually produced as a circular section rod. Generally have the |
| Cells | highest efficiency of the three technologies. If circular wafers of silicon are used, the module fill factor is significantly less than |
|  | with polycrystalline cells. It is now usual for the cells to be trimmed to approximate a square. |
| Polycrystaline | Made from a thin slice cut from a large cast billet of silicon comprising many crystals. Are slightly less efficient than the mono |
| Cells | crystalline cell but they can be shaped to completely fill the module. |
| Thin Film | Made by depositing thin films of silicon directly onto a glass or stainless steel substrate a thin slice cut from a single large |
| Technology | crystal of silicon. The cell has a lower efficiency than either of other technologies but can be multi-layered for enhanced |
|  | performance. Problems have been found with lifetime of these cells. |

Table 36 - Silicon Solar Cell Technology

In addition to the silicon cell technologies, there are two optional module configurations based on the numbers of series connected cells. The standard module normally has 36 cells in series to give an open circuit voltage of around 20 volts. For all battery charging applications, a voltage (charge) regulator is considered essential.

Modern developments in electronics have allowed voltage (charge) regulators to be developed that use maximum power point tracking (MPPT). This ensures that they operate the solar module at a level to obtain the maximum power, for any given level of irradiance. This operating level is independent to the battery charge voltage level. This technology can lead to up to 30% more output than would be achieved with conventional voltage regulators and can ensure effective solar charging in locations where high ambient temperature exists. It should be

| Navguide: Chapter 7 - Power supplies | 129 |
| --- | --- |

noted that better output is achieved when the panel voltage is at least twice that of the nominal battery voltage. Module or Array Orientation

In the northern hemisphere, solar modules are normally installed facing south and inclined at an angle to the horizontal that is related to the latitude of the site such that they can maximise output during the period of the year when irradiance is least, and vice versa for the southern hemisphere. The inclination angle for solar modules is often optimised for the particular site as part of the sizing calculations.

One of the main problems experienced with solar powered aids to navigation has been bird fouling. Numerous, innovative solutions have been trialled, generally with mixed results. Generally solar modules mounted at an angle or vertically benefit from self washing from rain.

The cost of additional solar modules needed for a vertical installation may be largely off-set by the savings that result from simplifying the mounting arrangements or framework.

Refer to IALA publications:

- Guideline 1012 on Protection of Lighthouses and other AtoN against damage from Lightning

- NEW GUIDELINE due 2021 on What constitutes a good Marine Solar Panel

- Guideline 1091 on Bird deterrents and bird fouling solutions

7.2.2 Wind Energy

Aids to Navigation Applications

Wind generators (or wind turbines) have been used by a number of IALA Members to power their aids to navigation.

The most popular type are horizontal axis machines with a two or more bladed (propeller type) turbine.

The maintenance requirements arising from the number of moving parts of a wind generator and susceptibility to storm damage, has limited the wide use of wind generators and they are not considered as reliable as photovoltaic cells but can provide a good back up power supply where solar irradiance is poor due to latitude, fog or high power requirements.

Installations

Wind generator installations at aids to navigation sites pose a number of problems:

- wind generators tend to require a lot of maintenance if operated in turbulent air flows;

- if the wind generator is installed on a separate mast some distance from the aid to navigation, consideration has to be given to the inherent cable voltage drop;

- operation of wind generators to power aids to navigation needs to take into account the impact it may have on any environmental factors associated with the location, such as flora and fauna.

7.2.3 Wave Energy

The wave activated generator (WAG) was developed in Japan and has been successfully used to power lighted buoys. The interaction between the buoy and wave motions acts as a simple air pump that is used to drive an air turbine and electricity generator. The WAG is mounted on an extension of a hollow tail tube that passes through the buoy hull. With wave heights of 0.5 metres, the power output can be as much as 100 watts. WAGs have limited life and current systems suffer from excessive wear.

Site conditions will determine the rate at which the tail tube of the buoy accumulates weed and other forms of fouling. These aspects need to be taken into consideration when developing the maintenance regime for the WAG. WAGs can also be very susceptible to storm damage.

| Navguide: Chapter 7 - Power supplies | 130 |
| --- | --- |

7.3 Rechargeable Batteries

7.3.1 Principal Types

There are various types of storage battery technologies applied to aids to navigation – lead acid Lithium-ion and nickel cadmium. The lead acid type is generally preferred because of its lower cost,good energy exchange efficiency (95% vs. 80%) and general reliability and availability. . Lithium-ion is becoming much more popular with its higher power density though is more expensive to purchase. The nickel cadmium battery can operate in low temperatures and for a greater number of deep discharge cycles.

Recently, new secondary battery technologies have appeared, including nickelmetal-hydride (Ni-MH) batteries, lithium-iron phosphate (LiFePO4) batteries, and lead crystal batteries. These batteries offer good weight and a greater cycle life for a given capacity but come at a premium cost.

Battery technology is a rapidly developing field with power density to size and weight improving all the time, this technology is being driven by the automotive sector and will undoubtedly migrate into AtoN applications.

Lead Acid

The basic form of this battery uses a lead dioxide positive plate and a pure lead negative plate immersed in an electrolyte of dilute sulphuric acid.

Lead acid batteries are available in two main designs, flooded lead acid and valve regulated (VRLA). The VRLA comes in two types, absorbed glass-mat (that use a micro glass separator system to absorb the electrolyte), and gel batteries, that use a jellified electrolyte and polymeric separators to prevent short circuits between the positive and negative plates.

Nickel Alkaline Battery

These batteries use compounds of nickel and, generally, cadmium with a solution of potassium hydroxide as the electrolyte.

Nickel-cadmium cells use perforated steel plates that hold the active material, mainly a nickel hydroxide in the positive plate and a cadmium compound in the negative plate. The construction is generally referred to as a “pocket plate” cell.

A range of valve regulated nickel-cadmium batteries that use a recombination process now complements the traditional flooded cell design. Under normal float charging conditions any gas produced is recombined within the battery and water loss is negligible. However if the battery is overcharged it will vent but water can be added if necessary.

Lithium ion Battery.

- The Lithium ion battery is a fast developing technology with many established variants on the market offering proven performance, their general advantages are a high energy density with potential for yet higher capacities, they do not need conditioning when new to acquire maximum capacity, they have a relatively low self-discharge and do not require a periodic discharge to maintain performance. They are also suitable for low and high power applications.

- The disadvantages of the Lithium ion battery are that they require a protection circuit to maintain voltage and current within safe limits and are subject to transportation restrictions due to safety concerns. They are expensive to manufacture and there is emerging debate about the social impact of the raw materials required to make them.

Battery Disposal

A number of countries now have standards and regulations relating to the safe and environmentally acceptable methods of disposing or recycling of batteries as well as the environmental and social impacts of acquiring battery raw materials. This should be a key factor when selecting a suitable battery for an AtoN application.

7.3.2 Primary Battery Cells

Primary battery cells provide electrical energy by a non-reversible chemical process. They were used in large numbers up until the 1980s to operate buoys and automatic beacon lights. The usage of primary cells has declined sharply since commercial solar power (photovoltaic) modules have become available. A related issue that hastened the decline of primary cells was the tightening environmental standards in a number of countries that required cells to be recovered from site for disposal in an approved manner. Disposal compliance costs, and occupational health and safety aspects of the frequent replacement of primary cells have worked in favour of converting to renewable energy sources. They remain a useful power source where solar irradiance is low or where theft is prevalent as they are less attractive to steal.

Zinc-Air Cell

The zinc-air primary cell was a common energy source for operating buoy and beacon applications. The cell uses a porous carbon block to supply oxygen from the air through an alkaline electrolyte to oxidize a zinc anode. Individual

| Navguide: Chapter 7 - Power supplies | 131 |
| --- | --- |

primary cells have an open circuit voltage of about 1.2 volts and can supply 1000 to 2000 Ah at a maximum rate of about 1 ampere.

Sealed Alkaline Battery

This type is commonly used in some countries, and has the benefits of good low temperature performance.

7.3.3 Internal Combustion Engine/Generators

Diesel Generators

Diesel engine driven generators are often used as the primary source of electrical power where the location of an aid to navigation is too remote to be supplied from utility generated electricity and the power demand is high. Diesel generators are also used to provide emergency or backup power.

The generator capacity to support the operational and domestic loads of a standard lighthouse is in the region of 10kW. Smaller generators in the range of 2 to 5kW, combined with batteries and inverter-charger systems are now available to meet this variable load. This arrangement can be more suitable and flexible if the load is likely to be light for extended periods, with short periods of heavy loads.

The requirement for diesel generators in lighthouses is decreasing as a result of:

- reduction in electrical load;

- improved efficiency of renewable energy sources.

Petrol Engine Generators

Petrol engine generators are a useful source of power for maintenance work, but are less common in permanent installations due to:

- fuel storage and transportation safety issues;

- maintenance requirements on the spark-ignition system;

- the petrol engine generally being regarded as less durable than a diesel.

Fuel Cell

This is a solid-state device that uses a catalytic process to oxidise fuel to generate an electrical current. A common fuel is hydrogen, or hydrogen rich fuels such as Methanol. It can be thought of as a continuously fed battery, ideally preferring a constant load.

The fuel cell is now commercially available, although the technology is still being further developed. Fuel cells offer a reliable and environmentally friendly energy source for supplementing AtoN power supplies.

Fuel cells do present an environmentally suitable solution, as methanol can be manufactured from sustainable sources and the by-products of the generation of electrical energy is heat and water. There is some interest in the use of fuel cells in hybrid power systems with wind energy or solar energy.

| Navguide: Chapter 7 - Power supplies | 132 |
| --- | --- |

7.4 Electrical Loads and Lightning Protection

7.4.1 Electrical Loads

IALA has prepared a standard methodology for calculating and defining the load profile of electric aids.

Some of the loads that this methodology covers are:

- lights;

- RACONs;

- AIS AtoN;

- audible warning signals;

- visibility detectors;

- monitoring and telemetry systems;

- charge controllers.

Refer to IALA publication:

•	Guideline 1067-1 Total Electrical Loads of Aids to Navigation.

7.4.2 Lightning Protection

To ensure reliable operation of aids to navigation during electrical storm events, both physical lightning protection and zoned surge protection should be considered. IALA has produced guidelines to describe practical methods for the design, installation, inspection and testing of lightning protection systems. The information covers lightning protection for aids to navigation structures, equipment and systems.

Refer to IALA publication:

•	Guideline 1012 on the Protection of Lighthouses and Aids to Navigation Against Damage from Lightning.

7.5 Non-Electric Energy Sources

Historically, non-electric energy sources were frequently used in aids to navigation, however, the use of electric energy sources is currently the norm and is the recommended practice for new installations. There are various non-electric power supplies, the main types used in aids to navigation are acetylene and propane.

Acetylene

Acetylene (C2H2) has been used to operate lights on buoys and unattended aids to navigation for many years. Acetylene can explode if compressed directly, but can be safely contained under low pressure in special cylinders when dissolved in acetone. The manufacture of acetylene, standards for the cylinders and the filling process are usually controlled by government regulations.

Acetylene has been a convenient and reliable energy source for aids to navigation. However appropriate attention should be given to:

- safe handling of cylinders;

- the broad range of explosive mixtures with air (between 3 and 82% acetylene);

- the purity of the gas;

- minimizing leaks in pipe work and fittings.

Propane

Propane gas (C3H8) has been used as an alternative fuel to acetylene, particularly in buoys. Although propane has to be consumed in an incandescent mantle burner to provide a white light, it has several advantages over acetylene:

•	it is a by-product in oil refining processes;

| Navguide: Chapter 7 - Power supplies | 133 |
| --- | --- |

- its abundance and low cost;

- propane liquefies at a pressure of 6 atmospheres at 17ºC, and can be transported in low weight and low cost gas bottles;

- propane will maintain a positive pressure down to -40ºC and will not freeze in conditions likely to be encountered at sea;

- placing the bottles in pockets in the buoy or by filling it directly into the body of an buoy, or pressure vessel;

- the comparable containers are the 20 kg propane bottle with gross weight of 48 kg and the 7,000 litre acetylene cylinder, weighing 105 kg;

- furthermore the cost of the propane bottle is only about one third of that of a acetylene cylinder;

- propane is a particularly safe gas, as only some 6% of all its possible mixtures with air are explosive against a figure of 80% for acetylene;

- burns cleanly without the risk of sooting that can occur with a poorly adjusted acetylene burner.

Refer to:

•	Applicable national standards for the safe handling of gases.

Notes

Navguide: Chapter 8 - Provision, design and management

8.1 International Criteria

The International Convention for the Safety of Life at Sea, 1974 (as amended), or SOLAS is one of the oldest international conventions and originates from a conference held in London in 1914 to address aspects of safety at sea following the sinking of the White Star liner Titanic in 1912. Since then, there have been four other SOLAS Conventions, the latest being the 1974 version that came into force in 1980.

The SOLAS Convention is administered by the United Nations through the International Maritime Organisation (IMO). The 1974 Convention (as amended) is divided into twelve chapters and within these are a series of regulations. The contents[1] are outlined in Table 37.

|  | Chapter | Contents |  |
| --- | --- | --- | --- |
|  | Chapter I | General Provisions |  |
|  | Chapter II-1 | Construction - Structure, subdivision and stability, machinery and electrical installations |  |
|  | Chapter II-2 | Construction - Fire protection, fire detection and fire extinction |  |
|  | Chapter III | Life-saving appliances and arrangements |  |
|  | Chapter IV | Radiocommunications |  |
|  | Chapter V | Safety of navigation |  |
|  | Chapter VI | Carriage of cargoes and oil fuels |  |
|  | Chapter VII | Carriage of dangerous goods |  |
|  | Chapter VIII | Nuclear ships |  |
|  | Chapter IX | Management for the safe operation of ships |  |
|  | Chapter X | Safety measures for high-speed craft |  |

| Navguide: Chapter 8 - Provision, design and management | 134 |
| --- | --- |

Chapter XI-1	Special measures to enhance maritime safety

Chapter XI-2	Special measures to enhance maritime security

Chapter XII	Additional safety measures for bulk carriers

Appendix	Certificates

Table 37 - Contents of SOLAS Convention

SOLAS Chapter V

SOLAS Chapter V, and Regulations 12[2] and 13 in particular, define the obligations on Contracting Governments to provide vessel traffic services and aids to navigation and related information. These Regulations define the primary roles of IALA National Members.

In December 2000, the 73rd session of the IMO Maritime Safety Committee (MSC) adopted a completely revised SOLAS Chapter V on Safety of Navigation that came into force on 1 July 2002.

In October 2005, IMO adopted IMO Resolution A.973(24) and A.974(24), outlining the IMO Member State Voluntary Audit Scheme which includes all aspects of SOLAS, including Chapter V, Regulations 12 and 13.

SOLAS Chapter V, Regulation 13 - Establishment and operation of aids to navigation states:

1 Each Contracting Government undertakes to provide, as it deems practical and necessary either individually or in co-operation with other Contracting Governments, such aids to navigation as the volume of traffic justifies and the degree of risk requires.

2 In order to obtain the greatest possible uniformity in aids to navigation, Contracting Governments undertake to take into account the international recommendations and guidelines (Reference is made to IALA) when establishing such aids.

3 Contracting Governments undertake to arrange for information relating to aids to navigation to be made available to all concerned. Changes in the transmissions of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.

To satisfy the obligations of Regulation 13, the Contracting Government has to make assessments on:

- whether or not to provide particular types of aids to navigation;

- the type, number and location of aids to navigation;

- what information services are necessary to adequately inform all concerned - principally mariners.

8.2 Level of Service

Level of Service (LOS) is the commitment of service by the Competent Authority to mariners who are navigating or operating in an area, as well as clients and/or governments responsible for funding the provision of the relevant service.

Level of service can be articulated through a Level of Service Statement that should be clear, easy to understand and available to all concerned.

8.2.1 Benefits

An established level of service is integral to efficient planning and delivery and provides users with a clear understanding of the expected services. Moreover, it ensures that services are delivered in a nationally consistent, integrated, predictable, measurable and fair manner.

| Navguide: Chapter 8 - Provision, design and management | 135 |
| --- | --- |

8.2.2 Components

A level of service statement should include, at minimum, the following components:

Type

Should describe what the Competent Authority will provide. It is a description of the service provided, such as, visual aids to navigation, radionavigation systems, or Vessel Traffic Services.

Extent

Should describe where and why a service will be provided by the Competent Authority. Most Competent Authorities are bound by the International Convention on the Safety of Life at Sea, 1974 as amended (SOLAS) Chapter 5, Regulation 13, which states that Each Contracting Government undertakes to provide, as it deems practical and necessary either individually or in cooperation with other Contracting Governments, such aids to navigation as the volume of traffic justifies [where] and the degree of risk requires [why]. The extent of service provided may also vary by Competent Authorities for specific areas, category of users, or due to national obligations.

Quality

Should address to what level the Competent Authority will provide a service. It is a minimum standard at which clients can expect a service to be performed, also known as a performance standard. A performance standard is a benchmark against which actual performance of a service can be measured. It may be expressed in the form of a target such as percentage of availability of a service or service response times.

8.2.3 Layers of Service

A summary of available aids to navigation systems and obtainable accuracies is provided in Table 4.

The various type of AtoN have advantages and disadvantages for the user as well as for the provider as indicated in Table 38.

| System |  | Users | Providers |  |
| --- | --- | --- | --- | --- |
|  | Advantages | Disadvantages | Advantages | Disadvantages |
| Visual | Can be used to position | Range depends on site, height, colour, | For hazard warning, | Maintenance expensive |
|  | Convey immediate | background | traffic regulation, | Planning for maintenance |
|  | information | Limited by visibility | guidance, etc. | depends on weather |
|  | Can be used without a | Position of floating aids not always accurate | Placement flexible | conditions |
|  | chart if user has a good |  | Maintenance requires | Logistic system required |
|  | local knowledge |  | little training | Training maintenance |
|  |  |  |  | personnel |
| Radar | Identification with racon | Onboard equipment needed | Can replace visual aids | Radar reflectors needed |
|  | possible in reduced | Racons may interfere if not placed in an | Warnings of dangers | Some vessels do not have |
|  | visibility conditions | appropriate configuration, aids equipped with | (New dangers) | radar |
|  | With a racon identification | radar reflector are difficult to identify |  | Racon investment |
|  | of low coastline |  |  | expensive |
|  | Only one aid is required |  |  | Training for maintenance |
|  | Rapid deployment |  |  | of racons |
| Radionavigation | Wide scale coverage | On board equipment needed | Reduced maintenance | May not be under Aids to |
|  | All weather use |  | Automatic monitioring | Navigation Authority's |
|  | Automatic navigation |  | Reduction of visual | control |
|  | Precision possible |  | aids possible | Monitoring requirement |
|  |  |  |  | Training maintenance |
|  |  |  |  | personnel |
|  |  |  |  | Large investment |

Table 38 - Comparison of Different Types of Aids to Navigation

Refer to IALA publications:

| Navguide: Chapter 8 - Provision, design and management | 136 |
| --- | --- |

- Guideline 1004 on Levels of Service;

- Guideline 1079 on Establishing and Conducting User Consultancy by Aids to Navigation Authorities.

8.3 Risk Management

Dealing with “risk” is an intrinsic aspect of human existence. The establishment of the early lighthouses represented a tangible way of addressing some of the problems that arose when humans decided to venture out to sea, and then into global trade and the mass transport of people by ships.

The traditional definition of risk is the probability of an unwanted event occurring, multiplied by the impact or

consequence of that event.

R=P*C

Unwanted events include deprival, loss or injury to persons, property or the environment.

Risk management is a term applied to a structured (logical and systematic) process illustrated further below.

The correct, efficient and useful result of hazard identification, assessment of risk and establishment of risk control measures, can be affected by Human Factors. The concept of Human Factors and references to relevant models is included in the IALA Guideline 1018 on Risk Management. It is recommended that organisations and persons involved in a risk assessment process have suitable knowledge in the application of Human Factors disciplines.

With the advances of e-Navigation, the mariner will be provided with additional real time information to assist with navigation. The positive impact on ship control and navigation needs to be incorporated into the formal risk assessment process. For risk control options, the continuous development of e-Navigation and man-machine interfaces may provide new possibilities. However, physical AtoN risk control measures will remain important to address the needs of all user groups.

The risk management approach works equally well for identifying the risks at a detailed or broad level. It can also address the risks from different perspectives.

Refer to IALA publication:

- Recommendation R1002 Risk Management for Marine Aids to Navigation

- Guideline G1018 Risk Management

For example, if the issue is the automation and destaffing of a lighthouse, there are likely to be different sets of risk for:

- service providers (aids to navigation authority, lightkeepers, etc.);

- service users (mariners);

- external groups (politicians, local community, conservation groups, etc.).

8.3.1 IALA Risk Management Tools

IALA continously develops its Risk Management Toolbox which includes tols that are capable of:

- assessing the risk in ports or waterways, compared with the risk level considered by Authorities and stakeholders to be acceptable. The elements that can be taken into consideration include those relating to vessel conditions, traffic conditions, navigational conditions, waterway conditions, immediate consequences and subsequent consequences;

- identifying appropriate risk control options to decrease the risk to the level considered to be acceptable. The risk control options available include improved co-ordination and planning; training; rules and procedures including enforcement; navigational, meteorological and hydrographical information; radio communications; active traffic management and waterway changes; pilotage; and,

- quantifying the effect on the risk level of an existing port or waterway that may result from a change or reduction of any of the risk control options in use.

| Navguide: Chapter 8 - Provision, design and management | 137 |
| --- | --- |

Risk management tools can also assist in assessing the risk level of existing ports and waterways as well as determining the probable risk level of proposed new ports and waterways or if substantial changes to existing ports and waterways are being planned. The IALA Risk Management Toolbox includes three different approaches:

- PAWSA Mk II (Port And Waterway Safety Assessment) which is a Qualitative Risk Assessment approach;

- IWRAP Mk II (IALA Waterway Risk Assessment Programme) which is a Quantitative Risk Assessment approach;

- SIRA (Simplified Risk Assessment) which utilizes a basic risk matrix approach.

The three approaches can be used individually, or in combination, sequentially or in parallel. The IALA Risk Management Toolbox is now being used for risk assessments in conjuntion with submissions to the IMO. In addition to these three basic IALA tools, various simulation tools are nowadays also widely used for assessing risk in ports and waterways.

The IALA World-Wide Academy provides training on the use of the Risk Management Toolbox through regular training courses. Background information on the elements of the toolbox is provided on a wiki accessible through the IALA website. If further guidance or assistance is required, please contact the IALA World-Wide Academy.

Authorities are encouraged to provide copies of risk assessments made with the IALA Risk Management Toolbox to the IALA Secretariat.

Refer to IALA publication:

- Guideline G1123 The Use of IALA Waterway Risk Assessment Pprogramme (IWRAP Mk II)

- Guideline G1124 The Use of Ports and Waterways Safety Assessmen (PAWSA Mk II) Tool

- Guideline G1138 The Use of the Simplified IALA Risk Assessment Method (SIRA)

8.3.2 Risk Management Decision Process

The Risk Management process described in the IALA Guideline 1018 comprises five steps that follow a standardized management or systems analysis approach:

- Identify hazards;

- Assess risks;

- Specify risk control options;

- Make a decision;

- Take action.

The diagram in Figure 35 provides a guide to the steps involved in the IALA Risk Assessment and Risk Management process.

Hazard – an unwanted event or occurrence, a source of potential harm, or a situation with a potential for causing harm, in terms of human injury; damage to health, property, the environment, and other things of value; or some combination of these.

Risk – the Risk is a measure of the likelihood that an undesirable event will occur together with a measure of the resulting consequence within a specified time i.e. the combination of the frequency and the severity of the consequence. This can be either a quantitative or qualitative measure.

| Navguide: Chapter 8 - Provision, design and management |  | 138 |  |
| --- | --- | --- | --- |

Figure 35 - Risk Assessment and Risk Management Process

The central part of the Figure 35 illustrates the five steps in the risk management process. In addition the figure suggests a consultation and reporting element throughout the process.

Stakeholders including practitioners and users shall be consulted and receive feed back continuously to ensure the best possible input to the decision makers, to validate decisions and to ensure ownership of the results and actions taken. The monitoring and review part in the right side of the model is vital to ensure a verification of the decisions, to check if initial conditions have changed and to constantly monitor if control measures are implemented effectively.

| Navguide: Chapter 8 - Provision, design and management | 139 |
| --- | --- |

8.3.3 Risk Level and Acceptability

Once the possible unwanted risk scenarios have been identified and described, they must be ranked in terms of their probability and associated consequences. This yields a risk value, enabling prioritization by placing each risk scenario in a matrix similar to the one below. This allows resources to be assigned to mitigate the more serious risks first.

Figure 36 - Risk Value Matrix

The SIRA method provides suitable scoring scales for probability (frequency of occurrance) and consequences/impact. It should be kept in mind that the consequences can include both short- and long-term impacts.

Risks with a low risk value (green) may be fully acceptable and require no action, while risks with a high value (red) need urgent attention. The intermediate risk values (yellow and amber) will need to be adressed at some stage, the essens being that all risk values should be As Low As Reasonably Practicable (ALARP) considering the cost and effectiveness of the identified risk control options.

8.4 Availability Objectives

The measurement of ‘Availability’ provides a quantitative measure of performance or service to the mariner.

‘Availability’ is a useful indicator of the level of service provided by individual or defined groups of aids to navigation because it is representative of all the considerations, within the control of the Authority, that have gone into providing and maintaining the facility.

These include:

- quality assurance procedures;

- design and systems engineering;

- procurement;

- installation and commissioning;

- maintenance procedures;

- failure response;

- logistics.

| Navguide: Chapter 8 - Provision, design and management | 140 |
| --- | --- |

To obtain a true representation of Availability, it is necessary to measure the long-term performance of an aid to navigation. To achieve this it is recommended that the calculations should use a time interval greater than 2 years.

8.4.1 Calculation of Availability

The availability of an aid to navigation may be calculated using one of the following equations, and is usually expressed as a percentage:

MTBF = Mean Time Between Failures

MTTR = Mean Time To Repair

8.4.2 Definition and Comments on Terms

Reliability

Ability of a device or system to satisfy the requirements of its intended use within defined limits, and for a stated period of time.

Availability

The percentage of time that an aid, or system of aids, is performing a required function under stated conditions. Non-availability can be caused by scheduled and/or unscheduled interruptions.It is also defined within IMO resolution A.1046(27) for WWRNS as “The system is considered to be available when it provides the required integrity for the given accuracy level”. IALA generally uses the term as a historical measure of the percentage of time that an aid to navigation was performing its specified function. The non-availability can be caused by scheduled and/or unscheduled interruptions.

Continuity

The probability that, assuming a fault-free receiver, a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation within a limited part of the coverage area.

For example, if a DGNSS station is functioning correctly when a vessel is about to make its approach into a port, the continuity factor is the probability that the DGNSS service will not be interrupted in the time it takes the vessel to reach its berth. As for GNSS systems, IALA has proposed that the time interval for continuity calculations be based on a 15 minute time frame in accordance with IMO A.1046(27) for WWRNS.

Redundancy

The existence of more than one means for accomplishing a given function. The various means of accomplishing the function need not necessarily be identical. The concept is such that a complete failure can occur only when all means have failed.

Integrity

This is the ability to provide users with warnings within a specified time when the system should not be used for navigation. IMO Resolution A.1046(27) for WWRNS, stares that this time to alarm should be within 10 seconds.

Failure

This is the (unintentional) termination of the ability of a system or part of a system to perform its required function.

Mean Time Between Failures (MTBF)

This is the average time between successive failures of a system or part of a system. It is a measure of reliability. For components, such as lamps, it is usual to determine the MTBF (or life) statistically by testing a representative sample of components to destruction. As for a system such as a DGNSS station, the MTBF is determined from the number of failures that have occurred within a given interval. For example; if four failures occur over a two year interval, the MTBF would be 4380 hours (ie. =24*365*2/4).

Mean Time to Repair (MTTR)

| Navguide: Chapter 8 - Provision, design and management | 141 |
| --- | --- |

This is a measure of an Authority’s administrative arrangements, resources and technical capability to rectify a failure. For a small port, the MTTR times might only be several hours. Meanwhile, an Authority with a more distributed network of aids to navigation may have MTTR times equivalent to several days because of the distances and transport mobilisation limitations.

Failure Response Time

This is a sub-set of the MTTR and relates to the time it takes to be notified of a failure, to confirm the details and mobilise personnel to depart for the aid to navigation.

Refer to IALA publications:

- Recommendation O-130 on Categorisation and Availability Objectives for Short Range Aids to Navigation;

- Guideline 1035 on Availability and Reliability of Aids to Navigation.

8.4.3 IALA Categories for Traditional Aids to Navigation

IALA provides a method to categorise and calculate aids to navigation availabilities for both individual aids to navigation and systems of aids to navigation as shown in Table 39. IALA Recommendation O-130 does not consider other aids to navigation considered in the mix of aids to navigation such as radionavigation systems or Vessel Traffic Services (VTS). It does provide guidance on suitable and realistic levels of operational performance for competent authorities to adopt.

| Category | Objective (%) | Calculation |
| --- | --- | --- |
| 1 | 99.8 | Availability Objectives are calculated over a three-year continuous period, unless otherwise specified. |
| 2 | 99.0 |  |
| 3 | 97.0 |  |

Table 39 - Availability Objectives by Category

Category 1

An Aid to Navigation (AtoN) or system of AtoN that is considered by the Competent Authority to be of vital navigational significance. For example, lighted aids to navigation and RACONs that are considered essential for marking landfalls, primary routes, channels, waterways or new dangers or the protection of the marine environment.

Category 2

An AtoN or system of AtoN that is considered by the Competent Authority to be of important navigational significance. For example, it may include any lighted aids to navigation and RACONs that mark secondary routes and those used to supplement the marking of primary routes.

Category 3

An AtoN or system of AtoN that is considered by the Competent Authority to be of necessary navigational significance.

The Recommendation also states that the absolute minimum level of availability of an individual aid to navigation should be set at 95%.

| Navguide: Chapter 8 - Provision, design and management | 142 |
| --- | --- |

8.4.4 Availability and Continuity of Radionavigation Services

The availability objectives for Radionavigation services have been handled somewhat differently from traditional aids to navigation. This reflects the broader policy formulation process that includes IMO Resolution A.1046(27) on a World Wide Radionavigation System and IALA Recommendation R-121.

Refer to IALA publications:

- Recommendation R-121 on the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz

Recommendation R-121 retains the original definition of availability, but adds a statement about “nonavailability”.

Non-availability is equivalent to “down time” but as proposed includes both scheduled and/or unscheduled interruptions (ie. preventative and corrective maintenance). The revised equation becomes:

MTBO = Mean time between outages; based on a 2 year averaging period MTSR = Mean time to service restoration; based on a 2 year averaging period

IMO uses a more elaborate definition of Continuity than that given in Section 8.4.2. It states that: Continuity is the probability that, assuming a fault free receiver, a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation within a limited part of the coverage area. This is the same definition of as “mission reliability”.

If the service is available at the beginning of the operation, then the probability "P" that it is still available at a time "t" later is:

This is the standard expression for reliability and excludes scheduled outages. It uses MTBF and assumes that planned outages will be notified.

The Continuity, or probability that the service will be available after a continuity time interval (CTI), is then:

If MTBF is much greater than CTI, the equation approximates to:

Where:

MTBF = Mean time between failures based on a 2 year averaging period.

CTI = Continuity Time Interval – in the case of Radionavigation calculations, this is equal to 15 minutes (from A1046(27)).

There is no need to include the availability at the beginning of the time period of the operation because if there is no service, then the operation will not commence.

Example 1: Using the figures in the previous example for a system with a 2 year MTBF, the continuity over a 15 minute period is 1-(15/1,051,200), or 99.9986%.

Example 2: Using the figures in the previous example for a system with a 1000 hour MTBF, the continuity over a 15 minute period is 1-(15/60,000), or 99.9750%.

| Navguide: Chapter 8 - Provision, design and management | 143 |
| --- | --- |

8.4.5 Over and Under Achievement

The actual availability achieved by an individual aid to navigation is a reflection of the quality of the logistical processes, the maintenance regime and the skill of personnel involved. There is a cost associated with prescribing a higher level of availability for a system such as an aid to navigation. This is irrespective of whether or not the increased availability is required by the mariner. There is also a cost associated with the maintenance of unreliable systems. The interrelationship is complex, but the objective is to find the minimum cost solution as illustrated in Figure 37.

Figure 37 - The Cost of Reliability

Over-Engineering vs. Unreliability

For a lighthouse in a remote location, the cost of time and transport to rectify equipment failures can be very high.

From this perspective:

- the one-off cost of over-engineering is generally not as expensive in the long term as the ongoing cost of attending to un-reliable equipment and/or poor system designs;

- a conservative design approach has its merits.

If the aid is not achieving its availability objective, the Authority should ascertain the reasons for this and implement actions that remedy the situation. IALA has recommended that if a facility cannot achieve an availability of 95% (ie. 50 days out per 1000 days) after reasonable endeavours, consideration should be given to withdrawing the facility (as an aid to navigation).

If a single aid within a group is performing above its availability objective, it could be due to either technical or environmental reasons. If the performance difference occurs between sites using similar equipment, and this trend has been established for some time,  the Authority should investigate the reasons for the difference.

| Navguide: Chapter 8 - Provision, design and management | 144 |
| --- | --- |

If a group of aids is found to be over performing for a relatively long period of time, there is an opportunity to review the maintenance practices with a view to determining the reasons, and possibly to consider extending the maintenance intervals or reducing the maintenance effort. This could lead to lower operating costs and issues relating to surplus maintenance capacity.

8.5 Reviews and Planning

8.5.1 Reviews

In many countries, the network of aids to navigation has been built up over a considerable time, in some cases, centuries.

It should be recognised that the nature of shipping is continually changing and this means that the aids to navigation infrastructure should be reviewed periodically.

The rate of change varies from place to place, but it would be reasonable to adopt a review process using one of the change management tools that provides:

- a Strategic Plan with a suggested minimum 10 year outlook;

- an Operational Plan with a suggested rolling 5 year work program.

The increasing availability of AIS-derived ship data (type, position, speed, cargo etc.) is proving to be a very useful tool in reviewing the relevance of existing aids to navigation and identifying new requirements.

Effective use of AIS data requires a data management strategy and appropriate technology to efficiently store and manipulate very large amounts of data and be able to be integrated with other electronic data, for example electronic nautical charts to display shipping patterns.

8.5.2 Strategic Plans

A Strategic Plan is the result of an informed and consultative process that sets the long term goals and objectives for an organization.

For a Competent Authority it would include:

- the role of the authority, for example: -- to promote a high standard of maritime safety; -- to provide infrastructure and information services to support the safety of navigation in a particular area.

- how the authority will go about discharging its responsibilities, for example: -- outline of the corporate values of the authority; -- corporate governance arrangements; -- funding arrangements; -- reviews of industry trends;

- an understanding of the users and navigation requirements.

Because of its importance and its effect on the mariners, any strategic plan should be developed as much as possible in full consultation with the mariners and other stakeholders.

8.5.3 Operational Plans

The Operational Plan might cover:

- the implementation of the strategic plan, and may include statements on current policy issues such as: -- maintenance; -- current and new technology; -- the design life of new infrastructure -- remote monitoring and control; -- historic lighthouses; -- environmental culture and safety; -- the program for aids to navigation reviews; -- contract services (core and non-core); -- transport services; -- sources of revenue; -- external relationships[3]; -- information, communication and consultation management.

- a list of changes to individual aids to navigation, including any new facilities.

The list would reflect: -- decisions resulting from user and stakeholder consultations; -- reviews, including those that

use: risk analysis, risk management procedures (see section 8.3), or; a level of service process, (see section 3.2), or;

| Navguide: Chapter 8 - Provision, design and management | 145 |
| --- | --- |

the authority’s quality management procedures (see section 8.7) or; the authority’s technical and maintenance policies etc.

- project schedules that reflect known priorities, such as: -- government policies; -- user requirements; -- available resources; -- budget (revenue) forecasts and constraints.

8.5.4 Use of Geographic Information Systems in AtoN Planning

The use of Geographic Information Systems (GIS) may assist in effective AtoN planning, including evaluation and validation; ensuring that money is invested wisely in new technology.

Coastal waterways are becoming increasingly congested with vessel traffic and developments such as offshore wind farms, tidal turbines and aquaculture sites, which may be required to be marked.

In addition, light pollution through coastal development, the advent of larger and faster ships and the continued growth in small craft usage means that designing suitable AtoN systems becomes ever more complex.

Using GIS, accurate design and provision of AtoN systems as well as suitable simulation can prove very useful and may reduce the chance of costly mistakes being made.

AtoNs are distinctly linked to physical locations and their use by mariners invariably involves the use of more than one AtoN at a time, that is, AtoN networks or systems.

These single and interdependent linkages between AtoNs and their physical locations mean that GIS technology can provide AtoN authorities with enhancements in many areas of their business, which may ultimately lead to benefits for mariners.

A GIS captures, displays, stores, analyses and manages spatially referenced data. A key feature of GIS is its analytical functionality, which allows a user to interact with spatial data to determine relationships between different types of data and to produce qualitative (diagrammatic/graphical) and quantitative (numeric/tabular) results.

Refer to IALA publications:

- Recommendation O-138 on the use of Geographic Information Systems (GIS) and Simulation by Aids to Navigation Authorities;

- Guideline 1057 on the use of Geographic Information Systems (GIS) by Aids to Navigation Authorities;

- Guideline 1058 on the use of Simulation as a Tool for Waterway Design and AtoN Planning.

8.7 Quality Management

Quality Management Systems have been developed and introduced by numerous businesses, but increasingly are being based on the ISO9000 series of standards. These standards provide a broadly accepted framework for implementing a quality management system. A generic quality management system is process focused and defines procedures for how things are to be done and what resources are necessary.

It addresses:

- who does what?

- what skills and qualifications are necessary?

- what processes have to be followed to get consistent outcomes?

- what resources are necessary to do the work efficiently? The equipment in aids to navigation systems can be divided into two aspects: the specific AtoN aspect, and the more generic aspect.

Each aspect must comply with applicable standards and regulations. IALA Recommendations and Guidelines provide a basis for the AtoN specific aspect, while international, national or regional regulations apply to the more generic aspects.

Refer to IALA publications:

| Navguide: Chapter 8 - Provision, design and management | 146 |
| --- | --- |

- Recommendation O-132 on Quality Management for Aids to Navigation Authorities;

- Guideline 1034 on product certification procedures;

- Guideline 1052 on Quality Management in Aids to Navigation Service Delivery,

- Guideline 1054 on Preparing for a Voluntary IMO Audit on Aids to Navigation Service Delivery

8.7.1 Performance Measurement

Performance measurement is the process of collecting, analyzing and/or reporting information regarding the performance of an individual, group, organization, system or component. It is very important to establish an continuous performance measurement as background for the quality management.

The information obtained can be used to:

- show accountability to government and stake holders;

- demonstrate the efficiency and effectiveness of the service being provided;

- monitor and improve occupational health and safety performance;

- compare the performance of: -- similar systems or equipment in different locations; -- contract and internally provided services[4];

- amend: -- system designs; -- procurement decisions; -- equipment choices; -- maintenance procedures and practices;

- increase or reduce maintenance effort;

- adjust maintenance intervals.

Refer to IALA publications:

- Guideline 1037 on Data Collection for Aids to Navigation Performance Calculation;

- Guideline 1035 to Availability and Reliability of Aids to Navigation Theorie and Examples;

8.7.2 International Standards

ISO 9000 Series

The 1994 quality standard series of ISO 9001, 9002 and 9003 have been jointly revised and amalgamated into ISO 9001-2000.

The new series of standards designated as ISO 9000 comprises:

- ISO 9000 Quality management systems - Fundamentals and vocabulary.

- ISO 9001 Quality management systems - Requirements.

- ISO 9004 Quality management systems - Guidance for Performance Improvement.

ISO 9001 - 2015

ISO 9001 specifies requirements for a quality management system that can be used for internal application by organizations, or for certification, or for contractual purposes. It focuses on the effectiveness of the quality management system in meeting customer requirements. See Figure 38.

ISO 9004 - 2009

ISO 9004 gives guidance on a wider range of objectives of a quality management system than does ISO 9001, particularly for the continual improvement of an organization’s overall performance and efficiency, as well as its effectiveness. ISO 9004 is recommended as a guide for organizations whose top management wishes to move beyond the requirements of ISO 9001, in pursuit of continual improvement of performance. However, it is not intended for certification or for contractual purposes.

ISO 14000 Series

This is a collection of voluntary consensus standards that have been developed to assist organizations to achieve environmental and economic gains through the implementation of effective environmental management systems.

| Navguide: Chapter 8 - Provision, design and management | 147 |
| --- | --- |

There are three standards that deal with Environmental Management Systems (EMS). These are ISO 14001, 14002 and 14004. ISO 14001 is the only standard intended for third party accreditation. The other standards are for guidance.

Figure 38 - ISO 9001 Diagram on the Emphasis on Satisfying Customer Requirements

ISO 14001

ISO 14001 specifies the requirements for an environmental management system, to enable an organization to:

- formulate a policy and objectives taking into account legislative requirements and information about significant environmental impacts;

- apply the requirements to those environmental aspects that the organization can control and over which it can be expected to have an influence;

- demonstrate to itself, and to other interested parties, conformance with its stated environmental policy;

- seek certification/registration of its environmental management system by an external organization;

- manage and measure a program of continual improvement.

ISO 14001 does not itself state specific environmental performance criteria.

Refer to IALA publications:

- Recommendation R1004 on Sustainability in the provision of Marine Aids to Navigation

- Guideline 1036 on Environmental Management in AtoN

- Guideline 1137 on AtoN Management in protected areas.

| Navguide: Chapter 8 - Provision, design and management | 148 |
| --- | --- |

8.8 Maintenance

8.8.1 Guiding Principles for Maintenance

Maintenance is required to ensure that AtoN equipment and systems continue to perform at the levels required by mariners to safely navigate the World’s waterways.

A maintenance system should be adopted to ensure that AtoN assets deliver the desired performance while minimizing Total Cost of Ownership. This performance is normally defined as the level of availability required. Depending on the criticality or category of the AtoN, the same AtoN type might require different maintenance approaches to deliver the required availability outcome in a given location.

The following guiding principles may assist Authorities in developing their overall AtoN maintenance strategy.

Minimise the Cost of Ownership

AtoN service providers are accountable to their stakeholders for the provision of a reliable AtoN network that meets international standards for a reasonable cost. Maintenance strategies adopted by authorities should seek to reduce the total cost of ownership of their AtoN.

Design for Maintenance

The majority of maintenance costs are determined by the design of the equipment itself. Maintenance costs are usually the most significant component of the total ownership cost of the equipment or system; therefore, it is crucial to account for long-term maintenance and logistics support early on in the design process.

The goal should be to reduce the need for maintenance, extend the time interval between maintenance periods, enable maintenance upon the evidence of need (condition-based maintenance), facilitate the maintenance task by the servicing personnel, and reduce the “logistics footprint” required for maintenance and support.

All of these factors will contribute to reducing the total ownership cost over the entire life cycle of the equipment or system.

8.8.2 Improving Efficiency

Authorities have been able to achieve significant cost savings by a number of means:

Automation

Automation can reduce the work load of light-keepers or allow for de-staffing altogether. This reduces:

- staff costs;

- power consumption;

- the frequency of stores replenishment;

- commitments on infrastructure such as houses or accommodation facilities, water and fuel storage and in some cases jetties and cargo handling equipment;

- the requirements for station vehicles and equipment.

Equipment

It may be possible to use more reliable equipment, better system designs, with “fail safe” or “fail by stages” features coupled with:

- longer intervals between maintenance visits;

- a review of maintenance management procedures.

In addition, it may be possible to use standardised equipment to simplify spare part management.

This could also:

- benefit the purchasing power of the organisation;

- reduce the range of skills required by maintenance staff (and thus the training burden);

| Navguide: Chapter 8 - Provision, design and management | 149 |
| --- | --- |

- give more flexibility in the choice of basic qualification when recruiting maintenance staff;

- provide more opportunity to understand the inherent deficiencies in particular pieces of equipment and for remedial actions to be implemented.

Fixed vs Floating

Depending on location, it may be possible to replace floating aids with fixed structures in waterways of moderate depth; particularly if it also allows a dedicated buoy tender to be replaced by some other means of transport such as smaller vessel or launch.

A whole-of-life cost benefit analysis should be carried out to assist in any such decisions.

Materials

By introducing low maintenance materials such as high density polyethylene, Glass Reinforced Plastic (GRP), stainless steel, etc., it may be possible to reduce maintenance requirements and time on site. This may also decrease the number of ship-day requirements and reduce the need for construction (or structural maintenance) skills.

Remote Control and Monitoring

Remote control and monitoring (RCMS)of distant or isolated aids to navigation can save on the cost of responding to what is later found to be a false outage report. It can also allow for analysis of aids to navigation systems using risk analysis / risk management techniques that may produce cost savings from a rearrangement and or reduction of the aids to navigation within a nominated area. Correctly applied, the monitoring will enable staff to travel to site with the required spares to repair an issue without a return visit. The provision of a control function enables a remote reset which may avoid the need for a site visit.

Refer to IALA publications:

- Guideline 1008 on Remote control and monitoring of AtoN

8.8.3 Trends

IALA Conference papers, IALA Bulletin articles and feedback from IALA Members demonstrate an increasing trend in the extension of maintenance intervals for aids to navigation sites. The ongoing striving for greater efficiency in the delivery of aids to navigation reflected in measures such as the automation of lighthouses has seen alteration of the maintenance intervals from a daily activity to significantly less frequent regimes.

The optimal maintenance interval for aids to navigation is determined from a consideration of national priorities and the Authority’s administrative, technical and environmental constraints.

Where cost efficiency and effectiveness is the driving issue, Authorities are:

- using automation, alternative structural materials, more durable coatings and renewable power supplies to contain or reduce costs;

- addressing the potential for new technology to:

  - reduce acquisition and operating costs;

  - extend maintenance intervals;

- remote maintenance by design and remote control to avoid additional visits;

- reviewing transport service options.

Extension of maintenance intervals at sites exposed to more extreme weather conditions may result in more extensive maintenance works at each visit which may offset some of the cost savings achieved through extension of service intervals.

| Navguide: Chapter 8 - Provision, design and management | 150 |
| --- | --- |

8.8.4 Maintenance Intervals

The maintenance intervals for aids to navigation vary from daily in the case of a manned lighthouse to perhaps five years for a lighted buoy.

It is difficult to establish a clear view of the appropriate maintenance interval as this will vary from authority to authority.

- This will vary on the quality of the installation, the prevailing environmental conditions and the performance of the AtoN.

Advances in self-contained beacons, lamps, solar power supplies and remote monitoring make it relatively easy for a well-designed system on a fixed structure to achieve annual or biannual servicing intervals. Systems that can be maintained in multiples of a year can be set up to take advantage of the times of the year that minimise the weather risk on work schedules and disturbance to flora and fauna.

However, a balance has to be found since longer maintenance intervals affect the authority’s knowledge of storm damage, general deterioration to aids to navigation, and control over vegetation growth and dirty glazing that could increase the risk of obscuration and fire damage etc. There may also be a detrimental effect on the detailed level of knowledge held by maintenance personnel.

Refer to IALA publications:

- Recommendation R1018 on The responsible design, operation and maintenance in the provision of marine Aids to Navigation

- Guideline 1077 on Maintenance of Aids to Navigation;

- Guideline 1076 on Building Conditioning of Lighthouses.

8.9 Service Delivery

Authorities responsible for the provision of aids to navigation are generally at a state or national level.

They are usually the sole national regulator of marine aids to navigation infrastructure and services, but are not necessarily the sole provider of these services.

In some countries there is a division of responsibility between the authority representing the national government and other organisations that include:

- state and territorial authorities;

- local government organisations;

- port, harbour or waterway authorities;

- local private organisations.

| Navguide: Chapter 8 - Provision, design and management |  | 151 |  |
| --- | --- | --- | --- |

Photo Courtesy of CEREMA

8.9.1 Service Delivery Requirements

Where more than one local authority provides aids to navigation services, the Contracting Government has the ultimate responsibility to comply with the obligations under the SOLAS Convention as listed in Section 8.1.

8.9.2 Contracting Out

For a number of decades, national authorities have utilized service providers for the delivery and maintenance of Marine Aids to Navigation (AtoN) services.

Every organisation considering contracting AtoN services should have a clear understanding of what is to be achieved by delivering the service through alternative contracted means as opposed to delivering the service directly by the National Authority. The best practices, advantages, disadvantages, and keys to successfully managing a contracted AtoN servicing program are covered in detail in IALA Guideline 1005, Contracting Out Aids to Navigation Services. It is recommended that National Authorities review this Guideline if they are considering the contracting out their AtoN servicing responsibilities and be mindful of retaining in house expertise to understand the technologies and requirements of AtoN Management and delivery.

Refer to IALA publication:

•	Guideline 1005 on Contracting Out Aids to Navigation Services.

New Guideline on Quality Control of 3rd Party AtoN service Providers

| Navguide: Chapter 8 - Provision, design and management | 152 |
| --- | --- |

8.10 Environment

IALA encourages all members to deliver AtoN services in environmentally responsible manner in line with its motto "Successful Voyages, Sustainable Planet"

Aids to navigation play a critical role in protecting the environment by preventing maritime disasters that could have potentially catastrophic ecological consequences at sea and on shore. However, the aids to navigation equipment and activities themselves can create significant environmental damage through pollution, waste generation, and the disruption of ecosystems. It is essential to minimise these negative impacts so that the benefits of aids to navigation are not outweighed by unintended harm to the environment, and to eliminate the potential for pollution and waste of the Earth’s limited resources.

International standard ISO 14001 provides a framework for environmentally responsible service delivery.

IALA is committed to align its activities and that of its members with the 17 United Nations Sustainability Goals

Refer to IALA publication:

Recommendation R1004 on Sustainability in the provision of marine AtoN

- Guideline 1036 on Environmental Management.

8.10.1 Hazardous Materials

Mercury

A number of historic lighthouses still utilise rotating glass lenses and mercury float pedestals. This was a clever method for providing a heavy lens with an almost frictionless bearing so that it could be turned by a clockwork mechanism. However, given the toxic and corrosive properties of mercury, the following information may assist Competent Authorities to implement appropriate safety procedures.

The mercury float pedestal for a first-order rotating lens[5]contains about 13 litres of mercury. Quantities of mercury may also be found in the electrical slip-ring units in rotating lamp array lighting equipment, some tilt-action switches, high current contact breakers, manometers and thermometers. It is being phased out by some Authorities now due to the tighter legislative restrictions about its use and transport.

Physical Properties

Mercury is a heavy metal that has the unusual property of remaining liquid at normal temperatures (above – 38 degrees celcius).

Spill Risk

The mercury in a lighthouse optic system does not present a significant hazard, unless personnel come into contact with “uncontained” mercury as a result of accidental spills. Such events are usually the result of a mishap during maintenance work, or as a result of a natural disaster such as an earth tremor that displaces mercury from its containment bath.

If spilt, the mercury can enter cracks in floors, and is readily absorbed into porous surfaces such as concrete, masonry and timber. When broken into small globules or droplets, the surface area and vaporisation rate rises rapidly. Minute droplets will adhere readily to dust and can form particles that can be inhaled.

Mercury is a corrosive substance if it comes into contact with metals such as zinc and aluminium.

Occupational Hazard

The occupational hazard associated with mercury relates to:

Vapour Inhalation: Some vaporization from a free mercury surface will occur at normal room temperature and this is the most likely first contact that lighthouse personnel will have with mercury. Unless the mercury vapour levels have been measured, personnel are unlikely to be aware of the hazard. If the work-space around lighthouse equipment containing mercury is not well ventilated, the concentration levels can rise above recommended limits and there is potential for mercury poisoning. Mercury vapour is heavier than air and in still air will tend to concentrate in low parts of the work-space. Well designed ventilation will allow such concentrations to disperse.

Ingestion: This can lead to acute mercurial poisoning.

| Navguide: Chapter 8 - Provision, design and management | 153 |
| --- | --- |

Skin Absorption: Mercury is not easily absorbed through the skin.

Precautions It is essential for the Authority to have detailed and strictly managed working procedures for all personnel working with, or in close proximity to mercury.

Staff must be trained in these work procedures and regularly medically monitored to ensure that they do not become contaminated with mercury.

The working procedures must follow national health and safety regulations and should be written by an expert in this field.

For work on optics the procedure will cover emptying, cleaning and re-filling the optic bath. Clean-up procedures will detail methods to recover all visible particles of mercury and the use of chemicals to neutralise smaller spills.

Personal protective equipment must be supplied that is specifically designed for use with mercury. This will include overalls, gloves, overshoes, respirator and eye protection. Procedures for the safe storage and disposal of this equipment must be in place.

A mercury vapour meter must be available to monitor the working environment and procedures in place for regular testing and calibration.

Consignment

Mercury is a hazardous substance and the relevant national and international regulations must be followed with regard to the type of container to be used, the packaging of this container for transport and the marking of this packaging.

Both IMO and the International Air Transport Association (IATA) have regulations covering the transportation of mercury.

Paints

Aids to Navigation authorities use a significant quantity and variety of paints and related surfacing materials. There is potential for hazardous situations to arise and for environmental pollution. For example:

- storage of inflammable paints and solvents;

- during surface preparation and removal of paint prior to repainting;

- contact with vapours and solvents during application;

- clean-up and waste disposal.

Lead and Isocyanate

Lead and Isocyanate based paints have been widely used in the past, but are now restricted or prohibited in some countries. Authorities maintaining older lighthouses are likely to be faced, at some stage, with having to remove lead based paint and disposing of the waste.

Members are encouraged to assess the risks and to adopt appropriate measures to safeguard maintenance personnel and the environment.

Antifouling Coatings

Antifouling paints contain biocides and are applied to vessels and floating aids to navigation to reduce the accumulation of marine organisms. For service vessels the antifouling paint assists to minimise fuel consumption.

On buoys and lightvessels the build-up of marine growth is not particularly detrimental. In view of the concentration of these types of aids to navigation in port approaches and internal waterways, less toxic paint systems may be preferred to minimise environmental pollution.

A particular group of antifouling paints using Tributyltin (TBT) has been banned from use. For further information, consult the International Convention on the Control of Harmful Antifouling Systems on Ships, published by the International Maritime Organization (IMO).

Asbestos

Asbestos is a carcinogenic substance that was (and in some areas, still is) used in the construction of buildings. Asbestos has excellent properties and with its resistance to fire, mould, rot and electrical insulation made it a material of choice through the early 20th century and many Authorities now have a regulatory duty to manage the safe retention or disposal of all asbestos in their estate. Asbestos is highly dangerous to anyone breathing in the microscopic fibres and it can take many years for the symptoms to become evident to the exposed person in the form of Mesothelioma which is generally fatal. Asbestos must be treated with care and relevant regulatory guidance on handling and disposing of it must be followed.

| Navguide: Chapter 8 - Provision, design and management | 154 |
| --- | --- |

8.11 Preservation of Historic Aids to Navigation

The IALA Advisory Panel on the Preservation of Lighthouses, Aids to Navigation, and Related Equipment of Historic Interest (PHL) was established by the IALA Council in 1996 in response to membership interest in the heritage value of lighthouses. In 2002, this Panel became part of the IALA Committee on Engineering, Environment, and Preservation (EEP) now AtoN Engineering and Sustainability (ENG) Committee. This work was supported further by the Incheon Declaration signed at the IALA General Assembly in Incheon, RoK in 2018

Objectives of the Incheon Declaration are to:

- Raise awareness of the need for the conservation and sustainable management of historical lighthouses as cultural heritage;

- Recognize that the significance of historical lighthouses extends beyond the navigational and architectural value to include maritime culture and history, social history, environmental aspects and that there is great value in documenting, researching and interpreting these for the benefit and inspiration of future generations;

- Conserve and develop historical lighthouse sites and associated maritime cultural spaces in order that people of all ages and backgrounds can appreciate and enjoy their respective nations’ maritime contribution and culture;

- Support IALA’s efforts to further sustainable lighthouse management through knowledge sharing, training and international cooperation projects; which will strengthen the capacity of coastal nations to manage their lighthouses in a coordinated manner and will encourage good conservation practices and foster interest in lighthouse heritage globally;

- Note the launch of the Incheon Project designed for IALA and its members to provide assistance for developing nations through education, training, technology transfer and capacity building as well as contributing to the advancement of global aids to navigation by means of various programmes and management in efficient and coordinated manner as an important legacy of the 19th IALA Conference in Incheon, Korea in 2018.

Examples of work accomplished to date:

- the creation of the format for an IALA database for recording details of historic lighthouses;

- The development of The Lighthouse of the Year award, based on international nominations and awarded on World AtoN day each year.

- Planning of a Heritage Workshop to share ideas and experiences on Lighthouse heritage.

- The approval of Recommendation R1005 on Conserving the build heritage of Lighthouses and other AtoN

- Publication of the Complimantary Lighthouse use Manual.

- ;

| Navguide: Chapter 8 - Provision, design and management |  | 155 |  |
| --- | --- | --- | --- |

Restoring a Heritage Lighthouse - Photo courtesy of Instituto Hidrografico (Portugal)

Lighthouse Interpretative Display - Photo Courtesy of IALA

Refer to IALA publications:

- Lighthouse Conservation Manual;

- Recommendation R1005 on Conserving the build heritage of Lighthouses and other AtoN

- Guideline 1093 on Management of surplus Lighthouse property

- Guideline 1049 on the Use of Modern Light Sources in Traditional Lighthouse Optics;

- Guideline 1063 on Agreements for Complementary Use of Lighthouse Property;

- Guideline 1074 on Branding and Marketing of Historic Lighthouses;

- Guideline 1075 on a Business Plan for Complementary Use of a Historic Lighthouse;

- Guideline 1076 on Building Conditioning of Lighthouses;

- Guideline 1080 on the Selection and Display of Heritage Artefacts;

8.11.1 Lens Size and Terminology

Information on terminology for historical glass lens systems and the typical amount of mercury held in mercury float pedestals (for rotating lens systems) is provided in Table 40.

| Navguide: Chapter 8 - Provision, design and management |  |  |  |  | 156 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Description | Focal distance | Typical Quantity of Mercury for Mercury Float Pedestals |  |  |  |
|  |  | mm |  | kilograms | litres |  |
|  | Hyper-radial | 1330 |  |  |  |  |
|  | Meso-radial | 1125 |  |  |  |  |
|  | First Order | 920 |  | 175 | 12.9 |  |
|  | Second Order | 700 |  | 126 | 9.3 |  |
|  | Third Order | 500 |  | 105 | 7.7 |  |
|  | Small Third Order | 375 |  | 96 | 7.0 |  |
|  | Fourth Order | 250 |  |  |  |  |
|  | Fifth Order | 187.5 |  |  |  |  |
|  | Sixth Order | 150 |  |  |  |  |

Table 40 - Terminology for Historical Glass Lens Systems and Associated Quantities of Mercury

8.11.2 Third Party Access to Aids To Navigation Sites

IALA acknowledges that Authorities face an increased demand to share aids to navigation sites with “third parties”. While it is important to ensure that the integrity and security of aids to navigation are maintained, the presence of a third party may be beneficial:

- in reducing the risk of vandalism;

- as a source of revenue or sharing of operational costs (e.g. power, road maintenance, etc);

- as a means of monitoring the operation of the aid.

If an Authority receives a request for a third party installation, it should first establish whether such involvement is permitted in the Authority’s legislation. If there are no impediments, the Authority may consider negotiating an agreement with the potential third party to clearly establish the responsibilities and liabilities of each party. The agreement may also address:

- conditions to apply to the third party installation and operation to ensure that the equipment does not compromise the integrity and security of the aids to navigation and other property owned by the Authority;

- access to electrical power. At sites with main power, it may be advisable for the Authority to require separate metering of the third party supply so that electricity costs can be recovered;

- if no main power is available, it is reasonable to require that the third party provide its own power supply;

- where practical, the installation of the third party equipment should take into consideration and preserve the heritage value of the aid to navigation.

Authorities should reserve the right to cancel any third party agreement if continued use jeopardizes the performance or functionality of the aid to navigation.

| Navguide: Chapter 8 - Provision, design and management | 157 |
| --- | --- |

8.12 Human Resources Challenges

One of the aims of IALA is to foster the safe and efficient movement of vessels through the harmonization of aids to navigation services worldwide.

SOLAS (2004 edition) Chapter V, Regulation 13, states that, in order to obtain the greatest possible uniformity in aids to navigation, Contracting Governments undertake to take into account the international recommendations and guidelines when establishing aids to navigation’.

Recommendations and Guidelines produced by IALA clearly identify the role that IALA has to play in ensuring harmonized delivery of AtoN services.

In addition, Resolution 10 of the Standards of Training & Certification for Watchkeepers (STCW) code states that the contribution of vessel traffic service personnel contributes to the safety of life and property at sea and the protection of the marine environment. IALA addresses this aim in several ways, one of which is to recommend that Aids to Navigation and VTS Authorities ensure their staff receive a high standard of training. To assist with this approach, IALA Recommendation V-103 and E-141, together with associated model courses and supporting Guidelines, were developed. This approach provides a means to ensure VTS Personnel are trained to an agreed, minimum, level.

In addition, both the ARM and ENG Committees are currently developing the training requirements for AtoN Management and Engineering Personnel through the World Wide Academy (WWA).

8.12.1 Source of Skills

Competent Authorities should ensure that all employees have the knowledge, skills and training to perform their duties effectively, and with safety. The term ‘employees’ includes newly hired, part time and temporary employees.

The ISO 9001 Quality Management standard places considerable emphasis on competence, awareness and training.

(See Section 8.7.1).

Refer to IALA publication:

•	Recommendation R0141 - Recommendation on Standards for Training and Certification of AtoN Personnel.

Notes

- Reference SOLAS Consolidated edition 2014.

- For VTS issues, please refer to Chapter 5 of the NAVGUIDE and the IALA VTS Manual.

- For example with national, state, territory, and local governmental bodies and international organisations.

- Only where the opportunity arises and where both are engaged in substantially similar work.

- The quantity of mercury used in higher order optics is shown in section 8.11.1.