IALA Guideline

Maritime Service Portfolios:
digitising maritime services

Edition 1.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

1.	INTRODUCTION	10

1.1.	General Description	10

1.2.	Purpose	10

1.3.	Implementation	10

1.3.1.	Change process	10

1.4.	Technical Issues	12

2.	Governing body, SERVICE PROVIDERS & STAKEHOLDERS	16

2.1.	Definitions	16

2.2.	Responsible service providers [to be decided later]	16

3.	Defined sea areas for MSP's	17

4.MARITIME SERVICES	18

4.1.	MS 1 VTS Information Service (INS)	18

4.1.1.	Submitting Organisation	18

4.1.2.	Description of the Digital Maritime Service	18

4.1.3.	Purpose	19

4.1.4.	Operational Approach	19

4.1.5.	User Needs	19

4.1.6.	Information to be Provided	20

4.1.7.	Associated Technical Services	20

4.1.8.	Relationship To Other DMSs	20

4.2.	MS 2 Navigational Assistance Service (NAS)	22

4.2.1.	Submitting Organisation	22

4.2.2.	Description of the Digital Maritime Service	22

4.2.3.	Purpose	22

4.2.4.	Operational Approach	23

4.2.5.	User Needs	23

4.2.6.	Information to be provided	24

4.2.7.	Associated Technical Services	25

4.2.8.	Relation to other DMSs	25

4.3.	MS 3 Traffic Organization Service (TOS)	25

4.3.1.	Submitting Organisation	25

4.3.2.	Description of the Digital Maritime Service	25

4.3.3.	Purpose	27

4.3.4.	Operational Approach	27

4.3.5.	User Needs	27

4.3.6.	Information to be provided	28

4.3.7.	Associated Technical Services	28

4.3.8.	Relation to other MSPs	28

4.4.	MS 4 Local Port Service (LPS)	29

4.4.1.	Submitting Organisation	29

4.4.2.	Description of Digital Maritime Service	29

4.4.3.	Purpose	29

4.4.4.	Operational Approach	29

4.4.5.	User Needs	29

4.4.6.	Information to be Provided	29

4.4.7.	Associated Technical Services	29

4.4.8.	Relation to other Digital Maritime Services	30

4.5.	MS 5 Maritime Safety Information service (MSI)	31

4.5.1.	Submitting Organisation	31

4.5.2.	Description of Digital Maritime Service	31

4.5.3.	Purpose	31

4.5.4.	Operational Approach	32

4.5.5.	User Needs	34

4.5.6.	Information to be Provided	34

4.5.7.	Associated Technical Services	34

4.5.8.	Relation to other Digital Maritime Services	34

4.6.	MS6 Pilotage service [IMPA]	35

4.6.1.	Submitting Organisation	35

4.6.2.	Description of Digital Maritime Service	35

4.6.3.	Purpose	35

4.6.4.	Operational Approach	35

4.6.5.	User Needs	35

4.6.6.	Information to be Provided	35

4.6.7.	Associated Technical Services	35

4.6.8.	Relation to other Digital Maritime Services	35

4.7.	MS 7 Tugs service	36

4.7.1.	Submitting Organisation	36

4.7.2.	Description of Digital Maritime Service	36

4.7.3.	Purpose	36

4.7.4.	Operational Approach	36

4.7.5.	User Needs	36

4.7.6.	Information to be Provided	36

4.7.7.	Associated Technical Services	36

4.7.8.	Relation to other Digital Maritime Services	36

4.8.	MS 8 Vessel shore reporting	37

4.8.1.	Submitting Organisation	37

4.8.2.	Description of Digital Maritime Service	37

4.8.3.	Purpose	37

4.8.4.	Operational Approach	37

4.8.5.	User Needs	37

4.8.6.	Information to be Provided	38

4.8.7.	Associated Technical Services	38

4.8.8.	Relation to other Digital Maritime Services	38

4.9.	MS 9 Telemedical Assistance Service (TMAS)h	38

4.9.1.	Submitting Organisation	38

4.9.2.	Description of Digital Maritime Service	38

4.9.3.	Purpose	38

4.9.4.	Operational Approach	38

4.9.5.	User Needs	39

4.9.6.	Information to be Provided	39

4.9.7.	Associated Technical Services	39

4.9.8.	Relation to other Digital Maritime Services	39

4.10.	MS 10 Maritime Assistance Service (MAS)	40

4.10.1.	Submitting Organisation	40

4.10.2.	Description of Digital Maritime Service	40

4.10.3.	Purpose	40

4.10.4.	Operational Approach	40

4.10.5.	User Needs	40

4.10.6.	Information to be Provided	40

4.10.7.	Associated Technical Services	40

4.10.8.	Relation to other Digital Maritime Services	40

4.11.	MS 11 Nautical Chart Service	41

4.11.1.	Submitting Organisation	41

4.11.2.	Description of Digital Maritime Service	41

4.11.3.	Purpose	41

4.11.4.	Operational Approach	42

4.11.5.	User Needs	42

4.11.6.	Information to be Provided	43

4.11.7.	Associated Technical Services	43

4.11.8.	Relation to other Digital Maritime Services	44

4.12.	MS 12 Nautical publications service	45

4.12.1.	Submitting Organisation	45

4.12.2.	Description of Digital Maritime Service	45

4.12.3.	Purpose	45

4.12.4.	Scope	46

4.12.5.	Objective	46

4.12.6.	Operational Approach	47

4.12.7.	User Needs	47

4.12.8.	Information to be Provided	48

4.12.9.	Associated Technical Services	48

4.12.10.	Relation to other Digital Maritime Services	48

4.13.	MS 13 Ice navigation service WMO	49

4.13.1.	Submitting Organisation	49

4.13.2.	Description of Digital Maritime Service	49

4.13.3.	Purpose	49

4.13.4.	Operational Approach	49

4.13.5.	User Needs	50

4.13.6.	Information to be Provided	54

4.13.7.	Associated Technical Services	54

4.13.8.	Relation to other Digital Maritime Services	54

4.14.	MS 14 Meteorological information service [WMO]	55

4.14.1.	Submitting Organisation	55

4.14.2.	Description of Digital Maritime Service	55

4.14.3.	Purpose	55

4.14.4.	Operational Approach	56

4.14.5.	User Needs	57

4.14.6.	Information to be Provided	59

4.14.7.	Associated Technical Services	59

4.14.8.	Relation to other Digital Maritime Services	59

4.15.	MS 15 Real-time hydrographic and environmental information services	60

4.15.1.	Submitting Organisation	60

4.15.2.	Description of Digital Maritime Service	60

4.15.3.	Purpose	60

4.15.4.	Operational Approach	61

4.15.5.	User Needs	61

4.15.6.	Information to be Provided	61

4.15.7.	Associated Technical Services	61

4.15.8.	Relation to other Digital Maritime Services	61

4.16.	MS 16 Search and Rescue (SAR) Service	62

4.16.1.	Submitting Organisation	62

4.16.2.	Description of Digital Maritime Service	62

4.16.3.	Purpose	62

4.16.4.	Operational Approach	62

4.16.5.	User Needs	62

4.16.6.	Information to be Provided	62

4.16.7.	Associated Technical Services	62

4.16.8.	Relation to other Digital Maritime Services	62

4.17.	MS 17 Aids to Navigation services (AtoN)	64

4.17.1.	Submitting Organisation	64

4.17.2.	Description of Digital Maritime Service	64

4.17.3.	Purpose	64

4.17.4.	Operational Approach	64

4.17.5.	User Needs	64

4.17.6.	Information to be Provided	64

4.17.7.	Associated Technical Services	64

4.17.8.	Relation to other Digital Maritime Services	64

4.18.	MS19 PNT and augmentation services	64

4.18.1.	Submitting Organisation	64

4.18.2.	Description of Digital Maritime Service	64

4.18.3.	Purpose	64

4.18.4.	Operational Approach	64

4.18.5.	User Needs	64

4.18.6.	Information to be Provided	64

4.18.7.	Associated Technical Services	65

4.18.8.	Relation to other Digital Maritime Services	65

6.	RELEVANT ASSOCIATED IMO GUIDELINES	72

6.1.	Guidelines on SQA and HCD	72

6.2.	Guidelines on Display of navigation information from communications	72

6.3.	Guidelines on test beds reporting	72

7.	LIST OF PUBLICATIONS THAT CAN BE DIGITAL	72

8.	ACRONYMS To be checked	72

ANNEX A	MS 1 INFORMATION SERVICE Template	74

8.1.	74

List of Tables

Table 1	Responsible Authorities	7

# INTRODUCTION

## General Description

When developing the International Maritime Organization (IMO) e-Navigation strategy to improve safety and efficiency of maritime transportation, it became clear that information services provided from coastal states to ships would be an essential part of this initiative. In order to adequately describe, structure and implement those services, IMO introduced the concept of “Marine Service Portfolios” (MSPs).

A “Maritime Service Portfolio (MSP)” defines and describes a set of operational and technical Maritime Services and the required level of service provided by a stakeholder in a given sea area, waterway or port, as appropriate. (NAV 57/6 para 23)

IMO has identified a preliminary list of sixteen Maritime Services (MS). Essential services under the mandate of IALA such as Aids to Navigation (AtoNs) and Positioning, Navigation, and Timing (PNT) were not included in the IMO’s preliminary list but, have been added to these guidelines.

In parallel to the development of the MS content, the IMO is working on Guidelines for harmonized display of navigation information received via communication equipment. These Guidelines aim to develop standards and specifications to allow display of MS information received from a recognized and official means of communication on shipboard equipment. This interoperability will allow a national authority to select, among e-Navigation recognized means of communication, those adapted to its own situation and capacity.

## Purpose

These Guidelines describe the operational aspects regarding the IMO e-navigation solution on maritime information services provided from shore to ship and displayed on a graphical display onboard the ship.

International organizations have provided the information that belongs to their respective domains and this cooperation constitutes the basic foundation for further global technical harmonization.

These Guidelines are mainly for providers of Maritime Services as defined in MSPs and aim to clarify what is expected by the maritime community if a dedicated provider of such services is declaring the availability of an MS in their jurisdiction. It presents briefly the content of an MS, its purpose and operational requirements and the links to other MSs when applicable.

This overall approach will help maritime service providers to better understand the steps required to migrate from analogue communication systems to digital services. Technical specifications necessary for the harmonized implementation of those MSs are defined elsewhere through the respective competent bodies, and are only referenced in this document for easy access.

The maritime services described in these Guidelines are not limited to IALA’s domain, but also include other responsible international organizations. This coordination and common understanding will facilitate the implementation of maritime services.

## Implementation

Implementation of Maritime Services will be achieved according to a coastal state capacity. Therefore, one should expect a gradual transition towards digital communication services.

### Change process

To be successful, the transformation of existing services into digital services will require proper and realistic planning and will include several different processes (Figure 1).

The first step is an assessment of the current situation and communication capacities as well as a clear definition of what the future digital situation may require. This assessment includes a review of the organization processes, systems, operational procedures and outputs for both situations: Current and Future.

In order for this assessment to serve as the basis for further analysis, it would need to consider different categories of information:

- technical

- procedural

- existing experience, and

- guidelines.

The analysis phase will document all relevant information in accordance with the initial assessment of the expected outcomes. Both a Change Process Plan as well as a Change Process Timeline will have to be created. Once these two plans are available, the execution can start in accordance to the plans.

Figure 1

For example: If a certain Competent Authority decides to change or develop a service to digital information (implementing an MS), it should take into account the following steps:

Assess the services being delivered by traditional means and the actual delivery platforms (e.g. VHF network);

Identify the services under own responsibility and the eventual need for new delivery platforms.

Identify the services to be delivered in the new format (MS) and the timeline for the implementation

Identify the providers of information for the selected MS

Establish a common workable platform between all providers. This platform must allow interoperability between the different providers and systems. A working protocol must also be established to guarantee the responsibility levels and the flow of information.

Coordinate the intended work with MS users and organizations creating tools for using MS outputs, to ensure usability by intended end users.

Implement the onshore structures (infrastructures, hardware and software) necessary for the delivery of MS portfolio.

Guarantee a testing phase to assess the resilience of the system.  When the Competent Authority determines that the service is reliable, it can be made fully operational and used as primary means for the delivery of that information to mariners. However, due to the existence of different levels of equipment onboard vessels, both analogue and digital services must be maintained as long as required.

During the transformation phase, it is necessary to continuously evaluate each process  to improve and optimize the service, and also to identify gaps and implement the required changes. This continuous process must be complemented by a permanent monitoring of the system to detect any communication failures, as well as the capacity to collect feedback from mariners and other stakeholders.

Figure 2

## Technical Issues

According to the situation, information provided electronically (digital format) could complement and/or replace voice communication. However, critical and urgent situations would continue to be best addressed through radio and voice communication, which ensures that the distress call has been received and that a proper response is triggered. For non-critical situations, information could be provided in a digital format but care should be taken to ensure that all ships have the capacity to properly receive and display the information.

Given that neither all shore-based authorities nor all ships will have modernized their equipment to the digital format level at the same time, a transition period from the current provision of Information Services to the Digital Service format must be envisaged. This transition period will also allow both ship and shore authorities to become familiar with the new technology and have confidence in it.

It is also important to consider that the provision of Maritime Services in many areas is not limited to SOLAS ships but also includes other type of vessels (domestic, fishing, pleasure craft, etc.). These vessels will need a similar period of transition, which may be longer according to the financial investment that might be required. Thus, the transition period will require that both methods of communication, analogue and digital, be maintained in parallel to ensure safety of navigation, until the digital format proves to be efficient and reliable for all situations (figure x).

Figure 3 – Illustration of the provision of information services over time.

Each existing MS and future new MS should make use of the methodology in Annex A for mapping and identifying what would be digital information in the future. This form will also identify existing product specifications in the s-100 registry and the GAPs that has to developed in addition.

Also, many services are produced by different national authorities (see figure 4 below) and the conversion of information such as ice, meteorological and hydrographic data to the digital format will gradually change the role of communication centres, such as VTS centres, with respect to this data. In some cases, the digitalization process will allow transmission of the information directly to mariners without passing through a VTS centre. However, these situations will require that VTS centres have constant access to this data in order to monitor their status and provide the information to ships experiencing electronic problems.

Currently, there are situations where the exchange of information between shore authorities and ships is done electronically.

For example:

Provision of VTS services such as INS can be done electronically without radio voice communication;

Meteorological, hydrographical and other types of environmental data can be broadcasted directly to ships through AIS Application Specific Message (AIS ASM), without passing through a VTS centre; and

The content of VHF communications can be transmitted electronically and be displayed as a text in parallel to voice communications.

Figure 4: Illustration of the historical communication process between VTS onshore and ship.

Over the last several decades, the common practice has been to “push” information to ships. This allowed VTS to have a knowledge of the status of this information and to trigger a notification procedure whenever necessary. Digitalization of information introduces new ways to communicate with ships and although it will still be possible to push it through communication systems such as AIS and VDES, the capacity of ships  to “pull” it directly from the internet through web services will also exist. In both cases, the assistance of VTS may not be required. This may decrease the situational awareness  of VTS as they may not know whether or not information has been received by ships. In order to keep pace with the digital transformation of information service, VTS will need to have access and monitor the status of information transmitted directly to ships. Hence, if there is ever a technological failure and information services cannot be digitally communicated to ships, VTS will be in a position to notify mariners as well as the service producer, or to provide the information directly to the ship.

Even though information could be made accessible directly to ships in the near future, VTS will remain the primary contact with ships for urgent and important messages, including as a back-up for electronic failure. Figure 5 illustrates some of the current and future possibilities with respect to shore  ship communications.

Figure 5: Illustration of some current and future communication processes between shore and ship.

Implementation of MS1 by a coastal state will require  strong coordination with all other MS producers to clarify the means of communication that wil be used to transmit their own service information to mariners, and to determine how VTS will be kept informed. Development of national procedures detailing how shore  ship communications will proceed will be essential to maintain safety to navigation.

# Governing body, SERVICE PROVIDERS & STAKEHOLDERS

## Definitions

- MSP governing body, which defines and maintains the overall architecture of the MSPs, endorses the definition and scope of individual Maritime Services, ensures interoperability and consistency, etc. (the IMO/IHO HGDM could be the initial basis for defining further that structure);

- Service definition owner which proposes the definition to the governing body and then implements the agreed definition through technical specifications;

- Service provider responsible for delivering an operational service according to the relevant specifications;

- Service producer refers to a national authority responsible for the collection or creation of an information or data.

- Users refer to all type of mariners who will receive the information transmitted by the service provider. In some cases (i.e. MSI service), there may be a need to distinguish between the provider of the information content (i.e. a NAVAREA coordinator) and the provider of the communication infrastructure/service (i.e. SafetyNET).

- Technical service specification owner  refers to the body responsible for developing and maintaining the technical specification(s) of a service, based on the corresponding service definitions [by way of example: for VTS Information Service, technical service specification owners could be IALA and the IHO]

## Responsible service providers

In each country there will be authorities responsible for providing information services.  The table below offers examples of authorities responsible for each maritime service., This example may differ according to a country’s internal organisation .

Responsible authorities may require service providers to deliver the operational service.

Responsible Authorities

| MS No | Identified Services | Identified Responsible Service Provider |
| --- | --- | --- |
| 1 | VTS Information Service (INS) | VTS Authority |
| 2 | Navigational Assistance Service (NAS) | VTS Authority |
| 3 | Traffic Organisation Service (TOS) | VTS Authority |
| 4 | Local Port Service (LPS) | Local Port/Harbour Authority |
| 5 | Maritime Safety Information (MSI) Service | National Competent Authority |
| 6 | Pilotage service | Pilotage Authority/Pilot Organization |
| 7 | Tug Service | National Competent Authority; Local Port/Harbour Authority |
| 8 | Vessel Shore Reporting | National Competent Authority and appointed service providers |
| 9 | Telemedical Assistance Service (TMAS) | National health organization / dedicated health organization |
| 10 | Maritime Assistance Service (MAS) | Coastal/Port Authority / Organization |
| 11 | Nautical Chart Service | National Hydrographic Authority / Organization |
| 12 | Nautical Publications service | National Hydrographic Authority / Organization |
| 13 | Ice Navigation Service | National Competent Authority Organization |
| 14 | Meteorological Information service | National Meteorological Authority Public Institutions |
| 15 | Real time hydrographic and environmental information service | National Hydrographic and Meteorological Authorities |
| 16 | Search and Rescue Service | SAR Authorities |

# Defined areas for A PORTOFOLIO OF MS’s

Maritime Services will be provided depending on the service type and the mandate that a service provider has for its provision in different geographic areas.  Each service provider should ensure that the service area is clearly defined and publicised.

# Flow chart of developing a new Maritime Service

The schedule of the flow chart below reflects a recommended process of steps to be     followed, with the following clarification

These descriptions of maritime services, using the templates, should contain references to relevant international Standards, Recommendations and Guidelines, which may contain criteria for the eventual implementation of these maritime services as well as identified user requirements for data and information;

The domain coordinating bodies will submit the descriptions of maritime services, using the template in Annex xx  to the Organization as the recipient (3), using the IMO Secretariat as a mailbox for automated registration and forwarding to the right IMO body.

The assigned IMO forum (eg HGDM) will study and evaluate the submitted descriptions of maritime services, using the templates, and identify potential harmonized data models (5) and finally report to NCSR (6) with a request for consideration (7) to incorporate, where appropriate, an approved description of a maritime service, using the template, into a relevant IMO instrument (e.g. Resolution, Guideline or Circular) (8);

Once the IMO instrument has been finalized and consequently, in accordance with IMO procedures, be approved by MSC, the Organization can provide guidance to the member states and the domain organizers (relevant NGO's and IGO's) to their members (9) on how to implement maritime services.

# MARITIME SERVICES

## MS 1 VTS Information Service (INS)

### Submitting Organisation

IALA VTS Committee

### Description of the Maritime Service

A VTS Contracting Government or Governments or the competent authority should establish and set the objectives for the types of services, areas and categories of vessels required or expected to participate.

The VTS Information Service is defined by IMO as “a service to ensure that essential information becomes available in time for on-board navigational decision-making” (Res. A857(20)).

The information service is provided by broadcasting information at fixed times and intervals, when deemed necessary by the VTS or at the request of a vessel. It may include reports on the position, identity and intentions of traffic; waterway conditions; weather; hazards; or any other factors that may influence the vessel's transit (IMO Res A.857(20)).

In some VTS areas the provision of VTS Information Service is not limited to SOLAS vessels and it should include all participating vessels. Other means to promulgate critical safety information to vessels unable to receive digital information should be considered.

IALA Guideline No. 1089 defines the “Provision of Vessel Traffic Services”(INS, TOS & NAS).

Examples of the types of information that may be provided by the VTS operating an Information Service

| Information related to: | Examples: |
| --- | --- |
| Navigational situations (including traffic and route information) | Position, identity, destination of vessels and the intention of other traffic; Amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points; the mandatory reporting of movements; Limited maneuverability that may impose restrictions on the navigation of other vessels, or any other potential hindrances; Suspension or change of routes; etc. |
| Navigational warnings | Dangerous wrecks, obstacles not otherwise promulgated, diving operations, vessels not under command, etc. |
| Meteorology | Information including the speed and direction of the prevailing wind, direction and height of the waves, visibility, atmospheric pressure, the formation of ice, etc. |
| Meteorological warnings | Gale, storm, tsunami, restricted visibility, etc. |
| Hydrography | Information such as the stability of the seabed, sea depth, the accuracy of surveys, tidal ranges, tidal streams, prevailing currents and swell, etc. |
| Electronic navigational aids | The availability of electronic navigational aids such as: GNSS, Loran, LRIT, DGPS, AIS, RACON etc. |
| Other information | Port information, pilot or tug request, cargo information, health condition, PSC, ISPS, etc. |

### Purpose

A VTS Information Service provides relevant information at appropriate times and on request for the promulgated VTS area. An Information Service involves maintaining a traffic image and allows interaction with traffic and response to developing traffic situations. (IALA Guideline No.1089 defines the " "Provision of Vessel Traffic Services" (INS, TOS & NAS)).

The purpose in providing VTS Information Service (INS) digitally is to reduce administrative burdens and information overload, reduce miscommunication due to external interference, simplify work procedures, and increase navigational safety.

Information provided digitally (non-verbal means) could complement and/or replace verbal/voice communication. The steps to achieve this transition to digital information exchange will vary in different areas and for different types of vessels. Depending on the categories of the Information Service in Annex A, (MS 1, Information Service Template) and its discoverability, new equipment may be required both on board and on shore side. Details about information exchange available should be published by the VTS authority

### Operational Approach

The provision of information digitally will diversify the means of communication between shore authorities and vessels and will likely change the VTS role and procedures. As digital information is transmitted directly to ships’ equipment without any intermediary, VTS may have to play a new role with respect to monitoring the availability of information for users and coordination with other service providers.

, VTS must remain the primary contact with vessels for urgent and important messages, and also as a back-up for electronic failure.

### User Needs

In IMO resolution A.857(20) are examples of information that can be provided to vessels. The user case below is based on the information from table 3

#### User Case – Vessel Arrival

Prior to the vessel arrival in the VTS area and according with reporting requirements provided in a digital form (e.g. S-127), a data collecting system on board will send all details regarding the arrival via relevant infrastructure to the VTS. The VTS collects the vessel's data directly into its system, and automatically updates the vessel's pre-registered data. Both the vessel and VTS use chart systems as a graphic interface to present details that are of interest to the voyage, such as reporting point lines and VHF channels.

Upon arrival in the VTS area, the vessel uses a data collecting system on board, and sends Actual Time of Arrival via relevant infrastructure to VTS.

This example is generic, and simplified through an example for description purposes only. Actions and template categories may differ for different countries. Content in the column named "Template Info (technical)"is pending submissions from relevant stakeholders.

The categories of services and the associated details are listed in annex 1, DMS 1 Information Service template.

| Time | Vessel Action | VTS Action | Template Info (category) | Template Info (technical) |
| --- | --- | --- | --- | --- |
| 01:00 | Provides pre-arrival info | Replying with information on weather | Environmental | Annex 1 |
| 02:00 | Enters VTS Area, provides sailing route | Traffic information to vessels | Traffic and Route information |  |
| 02:30 | Passes reporting point line | Provides information on current, wave height, etc. | Hydrographical information |  |
| 03:00 | Requires port information | Provides quay details | Traffic and Route information |  |
| 03:30 | Passes second reporting point | Provides operational information on AtoNs | Navigation Hazards |  |
| 04:00 | Vessel along side | Gives information on wind speeds, visibility | Environmental |  |

### Information to be Provided

See Annex 1, MS 1 Information Service template

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| Voyage Information Service | urn:mrn:stm:service:specification:sma:vis | The service supports exchange of voyage plans, text messages and area messages |  | IEC? |
| Weather Service |  | S-412 |  | WMO |
| Routing information |  | S-127 |  | IHO |
| Currents Service |  | S-101 |  | IHO |
| Wave height service |  |  |  |  |
| Port information (harbour charting information) |  | S-101 |  | IHO |
| Port information (harbour services information) |  |  |  | IHMA |
| AtoN Information |  |  |  | IALA |

[to be filled in later]

Note: Annex 1 could be complemented with required information regarding this table

### Relationship To Other MSs

MS1 has relationships with other MSs where it affects the VTS

Examples may be different depending on the coastal state arrangements.

| Description | Examples of data that could be used in MS 1 |
| --- | --- |
| MS 2 VTS NAS | See Annex 2 MS 2 Navigation Assistance Service Template |
| MS3 VTS TOS | See Annex 3 MS 3 Traffic Organisation Template |
| MS 4 Local Port Service | Delays, obstruction, cargo operations, port availability and anchorage areas in the port, ISPS state, Marsec level |
| MS 5 Maritime Safety Information | All information depending on the structure of MSI |
| MS 6 Pilotage Service | Pilot orders and updates |
| MS 7 Tug Service | Tug order and updates |
| MS 8 Vessel Shore reporting | Notification of arrival, dangerous cargo etc. |
| MS 9 Telemedical | Delays |
| MS 10 Maritime Assistance Service | Notifications, routing, places of refuge |
| MS 11 Nautical Chart Service | Local area updates, chart updates |
| MS 12. Nautical Publications Service | Updates to publications |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | VTS area weather |
| MS 15 Real Time Hydro and Information Service | Horizontal and vertical Ttidal information in VTS area, available water column |
| MS 16 Search and Rescue Service | Search pattern and vessel of opportunity |

## MS 2 Navigational Assistance Service (NAS)

### Submitting Organisation

IALA VTS Committee

### Description of the Maritime Service

A VTS Contracting Government or Governments or the competent authority should establish and set the objectives for the types of services, areas and categories of vessels required or expected to participate.

The VTS Navigational Assistance Service is defined by IMO as “a service to assist on-board navigational decision-making and to monitor its effects (IMO Res.A857(20)).

The navigational assistance service is especially important in difficult navigational, meteorological circumstances, or in case of defects or deficiencies. This service is normally rendered at the request of a vessel or by the VTS when deemed necessary (IMO Res.A857(20)).

IALA guideline 1089 provides additional details on Navigational Assistance Service (NAS).

Examples of the types of information that may be provided by a VTS operating a Navigational Assistance Service

| Information related to NAS | Examples |
| --- | --- |
| Request and identification | availability of NAS, start and end of NAS; request for ship identification such as position, course made good and speed over the ground; status of ship's equipment; etc. |
| Navigational information (including position and course information) | examples provided to an individual vessel: provide range and bearing from fixed objects, fairway/channel or way‐points; proximity to navigational hazards, etc. provide information related to navigating in a channel/fairway/lane (i.e. track is parallel/diverging/converging with/from/to reference line); etc. |
| Advice (or instruction) | advise (or instruct) a ship to alter the course, speed; advise (or instruct) to keep clear from area/position, close up/drop back on/from vessels; etc. |
| Warning | Diverging from the recommended track towards dangerous wrecks, obstacles not otherwise promulgated; diving operations; vessels not under command; etc. |

### Purpose

The purpose is to simplify the exchange of information with a ship requiring assistance. In non-time critical situations, exchange of information digitally can help clarify the procedures to follow and actions to take. This can prove to be useful in situations where voice communications are particularly difficult.

.

In time critical situations, information provided digitally could serve as a complement to voice communications  . For example, geographical information could be transmitted digitally (e.g. via AIS), and reference made to this information through voice communication

The steps to achieve this transition to digital information exchange will vary in different areas and for different types of vessels. Details about digital information exchange available should be published by the VTS authority.

### Operational Approach

VTS remains the primary contact for the Navigational Assistance Service and should be kept involved all along the assistance procedure.

Once informed about the nature and urgency of the assistance required by a vessel, the VTS operator will determine the best way to communicate with the ship officer. For example, in a time critical situation such as the risk of grounding, the VTS operator can opt to send the recommended route electronically in addition to voice communications. In a non-time critical situation such as assisting a vessel to an anchoring position, the VTS operator may provide the recommended route electronically without using voice communications at all.

The identity of the vessel receiving Navigational Assistance Service should be assured. Also, other items listed in the IALA Guideline 1089 On Provision of Vessel Traffic Service, annex B should be taken into consideration for digital transmission of information.

All information related to this service should be displayed in real time. Measures should be taken to ensure that the information is received and acknowledged.

### User Needs

The user cases below are based on the information from Table X.h

Each user case is generic and simplified through example for description purposes only. Actions and template categories may differ for different countries. Content in the column named "Template Info (technical)" is pending submissions from relevant stakeholders.

For example:

Pre-arrival reporting can be done digitally, without voice communication, for updates of routes and voyage plans, in order to avoid collisions, groundings and assist in safe navigation.

The content of the voice communication can be provided digitally and be displayed as text in parallel / in addition to voice communication.

#### User Case - vessel approaching VTS area

Vessel approaches VTS area according to received voyage plan. The VTS application observes and analyses the situation. It alerts the VTS operator, who then confirms that the vessel has deviated from its route. For example, based on statistical data/deep learning, the application then provides decision support to the VTS operator on the next course of action. The VTS operator validates and acknowledges the advice, and informs, warns and if necessary instructs / advises the vessel to change course via voice communication. The proposed solution is also presented on the vessel's own navigation system. The VTS operator confirms that the vessel has changed course according to the solution. The VTS application continues to monitor the vessel's voyage. It will alert the VTS operator if new deviation occurs.

The categories of services and the associated details are listed in annex 2, MS 2 Navigational Assistance Service template.

| Time | Vessel Action | VTS Action | Template Info (category) | Template Info (technical) |
| --- | --- | --- | --- | --- |
| 01:00 | Approaches VTS area | Receive voyage plan and monitor vessels progress | Traffic and Route Information |  |
| 01:30 | Deviates from her route | Informs, warns and advises/instructs the vessel to change course/speed | Navigational advice |  |
| 01:35 | Changes Course | Confirms that vessel has changed course and is no longer in danger |  |  |

#### User Case – vessel approaches shallow waters

The VTS operator concludes that the vessel wanted to make a lee for the pilot boat. The course given by the pilot boat would put the vessel in an even worse situation regarding shallow waters. VTS interrupted the boarding operation, gave instruction and guided the ship into a safe area for the embarkation of the pilot.

The VTS requests that the ship states its intention in the VTS area. In response, the ship provides its sailing route. In the template, this action is categorised as Navigational information.

The VTS gives a warning when the ship has run into shallow waters, upon which the ship confirms that the information has been received. In the template, this action is categorised as Navigational warning.

The VTS advises the ship to change course. The ship confirms. In the template, this action is categorised as Navigational advice.

The VTS instructs the ship to alter its course, upon which the ship confirms. In the template, this action is categorised as Navigational instruction.

| Time | VTS Action | Ship Action | Template Info (category) | Template Info (technical) |
| --- | --- | --- | --- | --- |
| 00:00 | Question: What is your intention? | Provides sailing route | Navigation Information |  |
| 02:00 | Warning: You are running into danger - shallow waters... | Confirms | Navigational warning |  |
| 02:10 | Advice: Recommended course to make good... | Confirms | Navigational advice |  |
| 02:20 | Instruction: Alter course to south-west... | Confirms | Navigational advice |  |

The VTS application indicates to the VTS operator the situation and suggests an appropriate decision. The VTS operator selects the correct decision from the VTS application and provides it digitally to the vessel. The vessel confirms the decision and takes appropriate action.

### Information to be provided

Information may be provided digitally, in general, but some situations will be time-critical.  Such time-critical information may be best provided by voice. On a case by case, ad hoc and time critical information may be provided using agreed-upon methods.  For details, please see  Annex 2, MS 2 Navigation Service Template.

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| Voyage Information Service | Urn:mrn:stm:service:specification:sma:vis | The service supports exchange of voyage plans, text messages and area messages |  | IEC ? |
| Route information |  | S-421 |  | IEC |

### Relation to other MSs

MS 2 has relationships with other MSs where it affects the VTS:

Examples may be different depending on the coastal state arrangements.

| Description | Examples of data that could be used in MS 2 |
| --- | --- |
| MS 1 VTS IS | See Annex 1 MS 2 Information Service Template |
| MS3 VTS TOS | See Annex 3 MS 3 Traffic Organisation Template |
| MS 4 Local Port Service | Delays, obstruction, cargo operations, port availability and anchorage areas in the port, ISPS state, Marsec level |
| MS 5 Maritime Safety Information | All information depending on the structure of MSI |
| MS 6 Pilotage Service | Pilot orders and Updates |
| MS 7 Tug Service | Tug order and updates |
| MS 8 Vessel Shore reporting | Notification of arrival, dangerous cargo etc. |
| MS 9 Telemedical | Delays |
| MS 10 Maritime Assistance Service | Notifications, routing, places of refuge |
| MS 11 Nautical Chart Service | Local area updates, chart updates |
| MS 12. Nautical Publications Service | Updates to publications |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | VTS area weather |
| MS 15 Real Time Hydro and Information Service | Horizontal and vertical tidal information in VTS area, available water column |
| MS 16 Search and Rescue Service | Search patterns and vessels of opportunity |

## MS 3 Traffic Organization Service (TOS)

### Submitting Organisation

IALA VTS Committee

### Description of the Maritime Service

A VTS Contracting Government or Governments or the competent authority should establish and set the objectives for the types of services, areas and categories of vessels required or expected to participate.

Traffic Organization Service (TOS) is defined by IMO as “a service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS area” (IMO Res.A857(20)).

The traffic organisation service concerns the operational management of traffic and the forward planning of vessels movements to prevent congestion and dangerous situations. It is particularly relevant in times of high traffic density or when the movement of special transports may affect the flow of other traffic. The service may also include establishing and operating a system of traffic clearances or VTS sailing plans, or both. This system may look at factors such as priority of movements, allocation of space, mandatory reporting of movements in the VTS area, routes to be followed, speed limits to be observed or other appropriate measures considered necessary by the VTS authority (IMO Res. A.857(20)).

IALA Guideline No. 1089 defines the Provision of Vessel Traffic Services (INS, TOS & NAS).

Examples of types of information that may be provided by the VTS within a Traffic Organization Service:

| Information related to: | Examples |
| --- | --- |
| Traffic clearance | Give authorization under conditional circumstances to a vessel: - prior to or when entering a VTS area; departing from a berth or an anchorage position within a VTS area; entering a fairway within a VTS area; or prior to commencing a manoeuvre that may be detrimental to safe navigation. Examples of conditions: a VTS sailing plan before entering a VTS area; lock and bridge passage planning; report position at determined reporting point/line/pilot station; use a second fairway in case of bad visibility/weather; use a tug boat in case of strong wind; dredging or compass swing in confined waterway. |
| Anchorage | Examples of anchorage situations: organizing the movements to/from an anchorage position/area; assignment of an anchorage position; assisting vessels into anchorage position. |
| Enforcement | Examples of enforcement: speed limits; adherence to rules regarding traffic routing measures; pilotage requirements; other traffic regulations and possibly local by‐laws. |
| Waterway (sea, channels and fairway) management | Examples of management measures: the use of one‐way traffic as an alternative to two way traffic, depending on the dimensions of ship or the weather conditions; organizing other traffic when a vessel has passed the point of no return; slot management to allocate ships in a time window; organizing the traffic concerning vessel dimensions in comparison to fairway restrictions; instruct vessels when overtaking is not permitted; establish and organise ship safety zones in case of particular operations; establish and organise exclusion zones; instruct vessels to keep clear from special areas/positions; organizing the traffic with regards to meteorological, hydrographical or other restrictions such as visibility, wind speed, current, sea state and under keel clearance. |

### Purpose

The purpose of the Traffic Organisation Service (TOS) is to provide a clear traffic picture in a waterway to enhance fluidity of ship’s transits and avoid incidents. To that end, the TOS collects information on all types of vessels, including fishing vessels and pleasure craft, and communicates the relevant information to all surrounding ships.

Given that AIS is not mandatory for all types of ships and that waterways’ configurations may include curves that prevent radar detection, the provision of TOS in a digital format may be challenging in some situations. For these reasons, it is recommended to initially implement digital communication to low-risk elements of the TOS and to move gradually to more complex situations.

All information provided digitally can complement and/or replace verbal/voice communication and would be distributed in real time to the vessel

The steps to achieve this transition to digital information exchange will vary in different areas and for different types of vessels. Depending on the categories of the Traffic Organisation Service in annex 3, (MS 3, Traffic Organisation Service template) and its discoverability, new equipment may be required both on board and on shore side. Details about digital information exchange available should be published by the VTS authority.

### Operational Approach

A Traffic Organization Service should be responsible for separating traffic in the interest of safety. This separation could be defined in space, time and/or distance.

Enforcement may also be carried out within a Traffic Organization Service where the VTS should monitor adherence to applicable rules and regulations and take appropriate action where required and within the authority of the VTS. (IALA Guideline 1089 On Provision of Vessel Traffic Service )

Transition to a digital TOS would be gradual and would not initially include all types of services. For that reason, it is recommended to start implementing this service in areas with relatively low traffic density to maintain safety of navigation and avoid confusion on the mariners’ side.

Also, it would be important to maintain the transmission of information for a determinate period of time in both conventional and digital formats. This would allow mariners and VTS operators to evaluate the capacity of ships’ equipment to display it, get familiar with the technology and assess the reliability and accuracy of digital data.

In that regard, many AIS Application Specific Messages (IMO SN. 1/Circ. 289) may be used to provide the TOS such as:

- Clearance time to enter port

- Berthing data

- Area notice

- Route information

- Meteorological and hydrographical information

- Air draught

- Etc.

### User Needs

The user case below is based on the information from Table 5.

The user case is generic and simplified through example for description purposes only. Actions and template categories may differ for different countries. Content in the column named "Template Info (technical)" is pending submissions from relevant stakeholders.

#### User case – vessel leaves quay

Vessel sends planned ETD digitally to VTS where it is presented in the VTS application. The VTS operator takes action and instructs the vessel digitally to remain alongside for five more minutes. The instructions are graphically displayed in the application, acknowledged and provided to other traffic via digital and/or verbal means for non-MSP-ready vessels. The application alerts the VTS operator on upcoming traffic conflicts and advises a solution, which is acknowledged and transmitted to vessels.

Detailed information can be found in the annex 3, MS 3, Traffic Organisation Service Template.

| Time | Ship Action | VTS Action | Template Info (category) | Template Info (technical) |
| --- | --- | --- | --- | --- |
| 00:00 | Requests permission to leave quay | Instruction: Remain alongside. Permission to leave in five minutes | Traffic clearance |  |
| 02:00 | Vessel passing reporting point before entering fairway A | Instruction: Provides sequence slot due to other traffic | Waterway management |  |
| 02:10 | Vessel had exceeded speed limit in the fairway | Instruction: speed limit in fairway 8 knots, keep safe speed | Enforcement |  |
| 02:20 | Requests anchorage | Provides position for anchorage | Waterway management |  |

### Information to be provided

See Annex 3, MS 3, Traffic Organisation Service template

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other MS

MS3 has relationships with other MSs where it affects the VTS:

This may be different depending on the coastal state arrangements.

| Description | Examples of data that could be used in MS 2 |
| --- | --- |
| MS 1 VTS IS | See Annex 1 MS 2 Information Service Template |
| MS2 VTS NAS | See Annex 2 MS 3 Navigation Assistance Service Template |
| MS 4 Local Port Service | Delays, obstruction, cargo operations, port availability and anchorage areas in the port, ISPS state, Marsec level |
| MS 5 Maritime Safety Information | All information depending on the structure of MSI |
| MS 6 Pilotage Service | Pilot orders and Updates |
| MS 7 Tug Service | Tug order and updates |
| MS 8 Vessel Shore reporting | Notification of arrival, dangerous cargo etc. |
| MS 9 Telemedical | Delays |
| MS 10 Maritime Assistance Service | Notifications, routing, places of refuge |
| MS 11 Nautical Chart Service | Local area updates, chart updates |
| MS 12. Nautical Publications Service | Updates to publications |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | VTS area weather |
| MS 15 Real Time Hydro and Information Service | Horizontal and vertical tidal information in VTS area, available water column |
| MS 16 Search and Rescue Service | Search pattern and vessels of opportunity |

## MS 4 Local Port Service (LPS)

### Submitting Organisation

International Harbour Master Association (IHMA)

### Description of the Maritime Service

A port authority is a hub from which several types of maritime and terrestrial services are provided. A port is generally not the final destination of goods, and so it needs to link to other modes of transportation (rail, truck, smaller ships). This ensures the fluid movement of goods to market. Port infrastructure and facilities are important links in the chain of intermodal transportation. To be capable of offering timely services, a port needs information on a ship’s transit from departure until time of arrival.

The size of a port and its status (local, regional, national or international) will influence the type and amount of services provided. These services include both maritime and shore sectors.

Table X.X - Examples of port services

| Maritime Services | Shore Services |
| --- | --- |
| Notification/Documentation/Reporting | Berthage |
| Navigation (anchorage, pilotage, tug, etc.) | Cargo Operations |
| Safety & Security | Fresh Water/Electricity/Waste/Repairs/Medical facilities/Shipping Agent/etc. |
| Nautical information | Bunkering |
| Communication | Linesmen |
| Weather | Intermodal transportation |

The combined services contribute to safe navigation, protection of the environment and the efficient movement of goods and people.

Figure x. Brief summary of services that may be provided by a port.

### Purpose

The Local Port Service aims to facilitate the exchange of information and coordination among key stakeholders during each phase of a ship’s transit. To promote optimal coordination and efficient operations all along the transportation chain, the exchange of information must be done in a timely manner. This allows all actors involved in LPS to take specific actions at the right moment either to respond or to adapt to new situations, such as delay, premature departure/arrival, incident, etc.

All actors involved in a ship’s transit will benefit from a common information platform. Access to timely information at each phase of a transit will improve:

- Just in time arrival and departure

- Reduction in waiting time for berthing (anchorage)

- Reduction of fuel consumption and greenhouse gases

- Predictability of cargo delivery to market

- Work schedules of services

A common communication platform will facilitate the fluid movement of cargo and persons, and contribute to a better and more transparent decision-making process.

### Operational Approach

An efficient LPS must allow all key stakeholders to access information they need during a ship transit. Sharing a common operational picture will optimize the decision-making process of each actor and facilitate adjustment whenever a situation changes. A basic operational approach for LPS would be the development and utilization of a common electronic information platform.

This platform will require each actor to enter its own relevant data in a timely manner for the benefit of other actors (see Figure xx). For sensitive and/or confidential data, restrictions can be set to provide access only to actors that require the information.

Figure xx. Examples of actors and data exchange that can contribute to a common LPS platform. Status of actions taken by each actor is illustrated by the lights (green, yellow and red).

Apart from sensitive information, all other data should be accessible to all participants involved in the transportation chain to create a common operational picture. Given that input from one actor might be critical for others’ decisions and influence their inputs, a management process would need to keep the platform updated in real-time. This would inform actors of status, or give notification, if required data is omitted. The port authority, or another organization, could assume responsibility for monitoring the platform and the status of data entries.

Figure 6 Terms for Vertical Measurement of vessels, depths and elevations. Identified in the IHMA and UKHO port information project.

### User Needs

- Provision of LPS is designed to improve coordination and exchange of information among stakeholders involved in a ship’s transit. The nature of this information is variable and has several purposesincluding safety, efficiency, protection of environment, fluidity of goods to market, etc. As such, user needs cover both maritime and shore information from departure to the time of arrival.

- Table X.X – Examples of types of information that LPS can cover

| Information related to: | Examples |
| --- | --- |
| Arrival and Berthing | Examples of Arrival and Berthing information: - Port allocation - ETA / ATA - Time slot, with slot management to allocate ships in a time window; - Mooring information - Draft and limitations - Safety and limitations of traffic concerning vessel dimensions in comparison to fairway restrictions - Local warnings and restrictions |
| Port information | Examples of port information: - ISPS information - Water supply - Crane and other cargo handling arrangement - Waste handling - Fuel arrangement - Immigration - Custom clearance - Pilot information - VTS information - Reporting formalities |
| Anchorage | Examples of anchorage situations: - organizing the movements to/from an anchorage position/area; - assignment of an anchorage position; - assisting vessels into anchorage position. |
| Enforcement | Examples of enforcement: - speed limits; - adherence to rules regarding traffic routeing measures; - pilotage requirements; - other traffic regulations and possibly local by‐laws |

### Information to be Provided

See Annex 4, MS 4 Local Port Service template

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| Port CDM |  | S-211, Port Collaborative Decision Making (Port CDM) services will make port calls more efficient for all stakeholders through improved information sharing, situational awareness, optimised processes, and collaborative decision making during port calls |  | IALA |

### Relation to other Maritime Services

This may be different depending on the coastal state arrangements.

| Description | Examples of data that could be used in MS 4 |
| --- | --- |
| MS 1 VTS IS | VTS area, type of VTS service, contact to VTS, information from sensors, radar, AIS, met, CCTV |
| MS2 VTS NAS | Information about vessel undertaking NAS |
| MS3 VTS TOS | Information about timeslots / ETA, route advisories |
| MS 5 Maritime Safety Information | All safety-related information |
| MS 6 Pilotage Service | Local pilot boarding place and stations, pilot regulations, information from pilot booking system |
| MS 7 Tug Service | Tug operation, available tug capacity |
| MS 8 Vessel Shore reporting | ETA/ATA, cargo information, incidents reports, dangerous goods, waste, number of persons onboard |
| MS 9 Telemedical | Local address of medical centre, communication and capacity |
| MS 10 Maritime Assistance Service | Information about vessels requesting relevant MAS,incidents , notifications, places of refuge |
| MS 11 Nautical Chart Service | Chart update information, hinge resolution charts for the port, Notice to Mariners, accuracy coverage characteristics, contour lines |
| MS 12. Nautical Publications Service | Local descriptions and publications. |
| MS 13 Ice Navigation Service | Ice forecast, ice charts, icebreaker services |
| MS 14 Meteorological Service | All meteorological information |
| MS 15 Real Time Hydro and Information Service | Sea state, current, wave, moving sandbanks |
| MS 16 Search and Rescue Service | SAR situations and operations in local areas, search and rescue capacity, SAR communication capability |

## MS 5 Maritime Safety Information service (MSI)

### Submitting Organisation

Local Met/Hydro organisation

### Description of Maritime Service

The Global Maritime Distress and Safety System (GMDSS), as described in SOLAS Chapter IV, defines the seventh functional requirement as: 'Every ship, while at sea, shall be capable of transmitting and receiving maritime safety information'.

### Purpose

The MSI service is an internationally co-ordinated network of broadcasts of Maritime Safety Information from official information providers, such as:

National Coastal Administration or shipping authority, for navigational warnings;

National Meteorological Offices, for marine weather warnings and forecasts;

Rescue Coordination Centres (RCCs), for shore-to-ship distress alerts;

The International Ice Patrol, for oceanic ice hazards.

- SOLAS Chapter V, regulations 4 through 7, govern the contracting government’s responsibilities with regards to providing MSI information.

Examples of Maritime Safety Information Service are listed in Table x.

Maritime Safety Information Service

| Information related to: | Examples: |
| --- | --- |
| Impediments to shipping and areas to avoid | Dangerous wrecks, obstacles not otherwise promulgated, diving operations, cable-laying operations, vessels not under command, etc. The routes of large unwieldy tows; Drifting hazards (including derelict vessels, ice, mines, containers, other large items, etc.); The establishment of offshore structures in or near shipping lanes and the positioning of rigs; Areas where search and rescue (SAR) and anti-pollution operations are being carried out (for avoidance of such areas) Military practice areas. |
| Status of navigation aids | Discrepancies with lights, fog signals, buoys and other aids to navigation affecting main shipping lanes; Establishment of major new aids to navigation or significant changes to existing ones. |
| Other urgent safety-related information | Unexpected alteration or suspension of established routes; Acts of piracy and armed robbery against ships; Tsunamis and other natural phenomena, such as abnormal changes to sea level; New or amended mandatory ship reporting systems or maritime regulations affecting ships at sea; Significant malfunctions or changes to maritime communications systems. World Health Organization (WHO) advisory information; Security-related requirements. |
| Marine weather warnings and forecasts | Information that will include the speed and direction of the prevailing wind, direction and height of the waves, visibility, atmospheric pressure, the formation of ice, etc. Gale, storm, tsunami, restricted visibility, etc. |

### Operational Approach

The joint IHO/IMO/WMO Publication S-53 states that ‘the Maritime Safety Information Service of the GMDSS is the internationally and nationally coordinated network of broadcasts containing information which is necessary for safe navigation, received on ships by equipment which automatically monitors the appropriate transmissions, displays information which is relevant to the ship and provides a print capability’.  This concept is illustrated in figure 4.5-1.

Figure 4.5-1 The maritime safety information service of the Global Maritime Distress and Safety System  (Source: S-53)

*Note that search and rescue information is outside the scope of this MS.

Additionally, local and regional governments may provide MSI information in other formats such as a website, push e-mail service or social media.

Within GMDSS, navigational warnings promulgation is done in defined areas that are managed by area coordinators, as illustrated on figure 4.5-2.

Figure 4.5-2 NAVAREAs for coordinating and promulgating navigational warnings under the World-Wide Navigational Warning Service (Source: S-53)

Marine meteorological warnings are, within GMDSS, promulgated in defined areas which are under the coordination of defined area coordinators. Within other distribution channels, other areas and operators are possible.

Figure 4.5-3 METAREAs for coordinating and promulgating meteorological warnings and forecasts within the GMDSS (Source: S-53)

### User Needs

Two principal methods are used for broadcasting maritime safety information in the areas covered by these methods, in accordance with the provisions of the International Convention for the Safety of Life at Sea, 1974, as amended::

NAVTEX: broadcasts to coastal waters; and

SafetyNET: broadcasts which cover all the waters of the globe except for Sea Area A4, as defined by IMO resolution A.801(19), annex 3, as amended.

Additionally, HF NBDP may be used to promulgate maritime safety information in areas outside Inmarsat or NAVTEX coverage (SOLAS regulation IV/7.1.5).

Ships are required to be capable of receiving maritime safety information broadcasts for the area in which they operate, in accordance with the provisions of the International Convention for the Safety of Life at Sea, 1974, as amended. The NAVTEX receiver should operate in accordance with the technical specifications set out in Recommendation ITU-R M.540, as amended. The SafetyNET receiver should conform to the Maritime Design and Installation Guidelines (DIGs), Annex B, Issue 6 of April 2008, published by Inmarsat. In Sea Area A4, outside of the coverage of NAVTEX, where MSI is received using HF NBDP, the HF NBDP receiver should operate in accordance with the technical specifications set out in Recommendation ITU-R M.688, as amended, and should meet the performance standards adopted by IMO resolution A.700(17), as amended.

Future MSI services should provide information in formats and by means that allow it to be better integrated with other systems on board, especially ECDIS. IHO is developing the S-124 standard which is expected to be the next generation Navigational Warning exchange standard. In addition, WMO is developing the S-411 and S-412 which are expected to be the next generation of Ice Warning and Weather Warning exchange standards. It is envisioned that this standard will interact in various forms with the ENC within the ECDIS.

Figure x.x.x : MSI can be portrayed on graphical display by using common standards such as S-124 and S-411

### Information to be Provided

See Annex 5, MS 5 Maritime Safety Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| Navigational Warning Service |  | Service that provides Navigational Warnings to mariners and related support services. |  | IHO |
| Ice Warning Service |  | Service that provides Ice Warnings to mariners and related support services. |  | WMO |
| Weather Alert Service |  | Service that provides Weather Alerts to mariners and related support services. |  | WMO |

### Relation to other Digital Maritime Services

Information related to ENC data, such as updates to the status of navigation aids, will supplement information that is part of MS 11. Meteorological information such as forecasts will overlap information that is part of MS 14 and MS 15, and again will potentially be extracts or summaries of information that is provided in more detail as part of those services.

## MS6 Pilotage service [IMPA]

### Submitting Organisation

International Maritime Pilots Association / Local Pilots

### Description of Digital Maritime Service

Seafarers seeking pilotage should have easy access to information regarding the pilot service provided in a specific area. Information such as local regulations, contact, support, boarding area, limitations, pilot booking systems, and guidance can be easily accessible by electronic standardized means, in a user-friendly format.   Efficient pilotage depends, among other things, upon the effectiveness of the communications and information exchanges between the pilot and bridge personnel, and upon the mutual understanding each has of the functions and duties of the other.

Establishment of effective co-ordination between the pilot, the master and the bridge personnel, taking due account of the ship's systems and equipment available to the pilot, will aid a safe and expeditious passage.

### Purpose

The purpose is to share all necessary information related to the pilot service when planning an operation, before the pilot embarks and during the operation, by using modern technology and common standards.

Efficient pilot operations depend on, among other things, the effectiveness of the communications and information exchanges between relevant stakeholders.

### Operational Approach

Pilot organizations providing pilot services in an area should provide information to vessels in a digital and easily accessible way. The information can be portrayed as a layer on ECDIS or on a graphical display.

Examples of information can be:

| Information related to: | Examples |
| --- | --- |
| General information | Examples of information: - Pilot services required in the area - Regulations - Limitations - Response time - Pilot booking system - Contact information for pilot organization/ operator - Mandatory needs for tug assistance - Safety procedures and requirements - Mooring information - Pilot boarding station - Information about pilot fee |
| Operational information | Examples of tug information: - Pilot identity - ETA / ETD - Contact for pilot boat - Position of vessels or station - Assisting methods - Required manoeuvring space - Speed limits - Communication - Exchange of PPU information |

### User Needs

Some user needs include: position exchange, pilot request, boarding stations, special regulations, special manoeuvring, time, cost and fees, tug information, a pilot application (My Pilot Service), exchange of passage plan, route exchange, pilot identity and any exemptions from pilotage requirements.

### Information to be Provided

See Annex 6, MS 6 Pilotage Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

MS 6 has relationships with other MSs where it affects the Pilot operation and contributes to safe and efficient operations.

Examples may be different depending on the local conditions and type of vessel / operation.

| Description | Examples of data that could be used in MS 7 |
| --- | --- |
| MS 1 VTS IS | VTS area, type of VTS service, contact for VTS, places of refuge, local regulations, limitation, visibility, information regarding traffic in the area |
| MS2 VTS NAS | Pilot needs information if NAS is conducted |
| MS3 VTS TOS | Traffic clearance, time slots |
| MS4 Local Port Service | Berthing information, timeslots, security, local regulations, supply, assistance, port contact information |
| MS 5 Maritime Safety Information | Warnings, no-go areas, diving and dredging operations, exercises |
| MS 7 Tug service | Tug allocation and direction, capacity |
| MS 8 Vessel Shore reporting | Vessel information, cargo information, persons onboard |
| MS 9 Telemedical | Not relevant |
| MS 10 Maritime Assistance Service | Contact information, places of refuge |
| MS 11 Nautical Chart Service | Local area updates, chart updates |
| MS 12. Nautical Publications Service | Digital information from nautical publications relevant for the operations at hand, including pilot contact and scheduling information. |
| MS 13 Ice Navigation Service | Ice chart, ice conditions, information regarding icebreaker service / assistance, ice routes |
| MS 14 Meteorological Service | All information regarding weather in the area |
| MS 15 Real Time Hydro and Information Service | All information from real time sensors |
| MS 16 Search and Rescue Service | Salvage information, drift patterns, SAR areas, rescue capabilities in the area. |

## MS 7 Tugs service

### Submitting Organisation

TBA

### Description of the Maritime Service

Tug services range from small vessels with limited capacity and service in ports and rivers to oceangoing vessels built for complex operations and salvage. This service contributes to the safety of navigation, protection of the environment, and efficiency of marine transportation by conducting different type of operations such as:

- transportation (personnel and staff from port to anchorage);

- ship assistance (ex: mooring);

- salvage (grounded ships or structures);

- shore;

- towage (harbour/ocean);

- escort;

- oil spill response.

The need of tug services differ from port to port, type of vessel and cargo. In some cases, information about a tug service capacity and/or availability may be difficult to obtain due to communication deficiencies. This maritime service is dedicated to improve information availability of tug services.

Tug service would encompass all kinds of tugs, such as

- conventional;

- azimuth stern drive;

- tractor;

- rotor.

| Information related to: | Examples information shared in a Tug service |
| --- | --- |
| Deep sea information | Examples of information: - Contact information for tug vessel / operator - Safety procedures and regulations - Available resources - Working hours |
| Local port or river information | Examples of information: - Contact information for tug vessel / operator - Mooring and berthing information - Available resources - Working hours |
| Tug information | Examples of tug information: - Type of tug - Capacity - Size - Assistance services - Response time - Contact information - Working hours |

Table xx – Examples of information that can be shared in a Tug service

### Purpose

Tug service aims to facilitate access to all necessary information required by ships heading to port, in order to optimize transit times and promote efficient movement of goods and persons by using modern technology and common standards.

Effective communications and exchange of information between relevant stakeholders would contribute to efficient tug service. Exchange of information electronically would significantly contribute to the improvement of this service.  For example, notifying a ship officer in advance about tug availability in port could lead the ship to adapt its speed accordingly.  In some cases this may prevent a requirement to anchor the ship. The types of information which can be exchanged include:

- ETA (request);

- confirmation requests;

- updates on transit status and tug availability;

- updates among stakeholders;

- standardized messages to overcome language barriers.

### Operational Approach

Tug service is a key element of the marine transportation chain and well-coordinated procedures and communication means should be in place to ensure fluid movement of ships.

Like the Local Port Service, utilization of a common platform to exchange information electronically and keep users updated on a regular basis about the status of operations would significantly improve this service, for both the ships’ operator and the tug owners. The tug service aims mainly to improve the communications involved in a ship request, rather than altering current operational procedures. Some of these communications may include:

- Ship’s size

- Number of tugs required

- Time the service is required

- Time the tug may be on site

- Estimated duration of operations

- End of operations.

- Access to this information electronically would enhance the awareness of a ship’s time-stamp.

- Figure xx. Example of an electronic communication platform for all actors involved in the tug service.

- Increased connectivity, through sharing of harmonized digital information regarding tug services in ports, rivers or deep-sea, will enhance efficiency through just-in-time services. It will also reduce human factor errors, such as language barriers or outdated information in analog publications, enhancing efficiency and access to information in a fast and easy-to-use-manner.

### User Needs

### Easy and timely access to tug service information is critical to ensure fluidity in the transportation chain. The main information required from this service are:

- Capacity

- Availability

- Time of response

- Status of operations

- Duration of operations

### In return, the tug services should be regularly updated on the ship’s ETA/ATA to plan its operations accordingly. In the event of an unanticipated change, the tug service and ship officers must be able to communicate easily with each other to keep both parties informed about the evolving situation and allow for proper decision-making. An easy communication link must be part of the user needs and this communication link would also benefit all other actors.

### Information to be Provided

See Annex 7, MS 7 Tug Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| S-211 Port Collaborative Decision Making (PortCDM) |  | Standard for information exchange between actors in ports, including tugs. |  | IALA |

### Relation to other Maritime Services

| Description | Examples of data that could be used in MS 7 |
| --- | --- |
| MS 1 VTS IS | VTS area, types of VTS service, VTS contact information, places of refuge, local regulations, limitation, visibility, information regarding traffic in the area |
| MS2 VTS NAS | Not relevant |
| MS3 VTS TOS | Traffic clearance, time slots |
| MS4 Local Port Service | Berthing information, time slots, security, local regulations, supply, assistance, port contact information |
| MS 5 Maritime Safety Information | Navigational warnings, meteorological information and other urgent safety-related information. |
| MS 6 Pilotage Service | Pilot regulations, contact information, request procedures |
| MS 8 Vessel Shore reporting | Vessel information, cargo information, crew information |
| MS 9 Telemedical | Not relevant |
| MS 10 Maritime Assistance Service | Contact information, places of refuge |
| MS 11 Nautical Chart Service | Charting information, chart updates |
| MS 12. Nautical Publications Service | Digital information from nautical publications that is relevant for the operations at hand |
| MS 13 Ice Navigation Service | Ice chart, ice conditions, information regarding icebreaker service / assistance, ice routes |
| MS 14 Meteorological Service | Information regarding weather in the area |
| MS 15 Real Time Hydro and Information Service | Information from real time sensors |
| MS 16 Search and Rescue Service | Salvage information, drifting parts, SAR areas, and rescue capabilities in the area |

## MS 8 Vessel shore reporting

### Submitting Organisation

### Description of Maritime Service

This service provides information from shore to ship about two different reporting regimes.

The first regime is Vessel Shore Reporting (VSR), implemented for pre-arrival reporting. This service can cover information and guidelines related to reporting formalities and instructions (when, what and how) for reporting to a specific port. In addition, this service can be extended to the full exchange of information required in a Single Window ship reporting system. The Vessel Shore Reporting (VSR) regime may contain the following elements:

- Marine security regulations

- Vessel Traffic Services zones regulations

- Customs and immigration regulations

- Port state regulations

- Health and veterinary regulations

- Environmental regulations

Many of these elements will be addressed by the IMO Facilitation Committee, which is developing the specifications for the Single Window Reporting System. This is linked to e-Navigation Strategy Implementation Plan Solution 2 - Means for standardized and automated reporting -.

IMO e-navigation solution 2, regarding automated ship reporting, is one of the most important solutions to reduce the mariners’ workload (amount of time spent on preparing and submitting reports to shore-based authorities). To achieve this, reports should be automatically generated as much as possible from on-board systems.

The second regime is linked to a transit in an area where a Ship Reporting System (SRS) is established.

In SRS areas, shore authorities can provide more automated and efficient reporting systems by using technology such as AIS and VDES, in combination with common data structures and product specifications.

In a ship reporting system (SRS), MSC 43(64) recalls that communication between a shore-based authority and a participating ship should be limited to information essential to achieve the objectives of the ship reporting system (SRS). The initial report required from a ship entering the system should generally be limited to:

- ship's name

- call sign

- IMO identification number if applicable, and

- position.

The existing resolution is based on Regulation 11 of SOLAS Chapter V as follows:

“Ship reporting systems contribute to safety of life at sea, safety and efficiency of navigation and/or protection of the marine environment. A ship reporting system, when adopted and implemented in accordance with the guidelines and criteria developed by the Organization pursuant to this regulation, shall be used by all ships, or certain categories of ships or ships carrying certain cargoes in accordance with the provisions of each system so adopted.

The Organization is recognized as the only international body for developing guidelines, criteria and regulations on an international level for ship reporting systems. Contracting Governments shall refer proposals for the adoption of ship reporting systems to the Organization. The Organization will collate and disseminate to Contracting Governments all relevant information with regard to any adopted ship reporting system.”

Although the regulation refers to safety of life at sea, safety and efficiency of navigation and/or protection of the marine environment, resolution MSC.43(64) states that other supplementary information may also be requested in the initial report, if justified to ensure the effective operation of the ship (MSC.43(64) Annex para 2.2.1.4).

This information may include the intended movement of the ship through the area covered by the reporting system and any operational defects or difficulties affecting the ship , as well as the general categories of any hazardous cargoes on board (MSC.43(64) Annex para 2.2.1.4)

A Vessel Shore Reporting (VSR) system normally includes a web solution and an onshore representative, or agency, for the vessel operator or owner.  The agency acts on behalf of the master or owner and provides information to relevant authorities.

The VSR as envisaged by e-navigation solution 2 aims to reduce time-consuming paperwork onboard.

Information required to be transmitted as part of a mandatory SRS is generally transmitted to the VTS centre, or  the relevant authority, via VHF voice communication. Some SRSs also accept reports transmitted through AIS, internet-based reporting systems, e-mail, fax, SATcom, mobile phone, or a combination of these communication means.

Figure XX – Examples of reporting points from Bay of Fundy

### Purpose

The purpose of the vessel shore reporting service is to:

- Secure submission and distribution of reports required by shore-based authorities in the harmonized standard and in the required timeframe.

- Reduce the administrative burden on board the ship and ashore.

- Reduce the number of human errors and missing information by automating the reporting processes as much as possible.  .  And,

- Provide real-time access to information to relevant stakeholders in a secure manner.

MSC 43(64) states that the objectives of a ship reporting system (SRS) should be based upon:

“the improvement of the safety of life at sea, the safety and efficiency of navigation and/or to increase the protection of the marine environment. They may or may not be operated as part of a vessel traffic service.”

As such, the VSR service contributes to the traffic situational awareness of a coastal administration either by being informed of vessels heading into its waters or the ones already in transit. Based on the information collected, a coastal administration can initiate an intervention plan, if required.

### Operational Approach

Electronic systems for reporting should use the same protocols and product specifications, in a Single Window solution, to send digital pre-arrival information such as the FAL forms and other regional/national requirements, as defined in SOLAS V Regulation 11.2 for ship reporting systems. This will ensure a common harmonized platform for all ship reporting systems.

National Competent Authorities should provide information about reporting formalities and ensure that all information regarding reporting is easy to understand, accessible and even automated for the master or operator.

This service should provide ICT tools for shipboard and shore-based personnel to streamline the processes and procedures associated with the generation and distribution of required reports, including retrieval of information from other ship systems (Ballast Management, Waste Management System, Emission Control System, Navigation System, etc) and from shore-based sources (cargo and passenger booking offices, crewing agents, stevedores, etc).

Examples of information to provide can be:

| Information related to: | Examples |
| --- | --- |
| Reporting regulations | - What to report. The pre-arrival information may consist of ship particulars, arrival notice, crew and passenger lists, crew and passenger effects declarations, stores list, IMDG information, waste declaration, ship’s certificates, ports of call list, dangerous cargo declaration and manifests, vaccination list, narcotic list, ship’s money declaration, etc. - When / What to report (e.g. 24h, 48h, 72h, 96h before arrival) - To who (e.g. immigrations, police, port master, etc.) |
| Reporting tools | - Web, App, etc. - Communication - Guidelines |
| Shore receivers And support | - Contact information - Support information - Information about local reporting aid / support such as coastal radio stations and agencies |

The type of information required and reporting periods may differ from country to country and create some confusion on the ships’ side, if the information requested is not clearly stated. Also, the reporting periods in some situations start as far in advance as 96 h before a ship enters a coastal administration’s waters. This may pose challenges with respect to the communication means available to provide the required information all along the transit.

Currently, the following communication systems are normally used to report information:

- AIS

- internet-based reporting systems

- e-mail

- fax

- SATCOM

- mobile phone, or

- a combination of these systems

As the scope, transmission capacity, and data format of each of these systems differ, the possibility of developing standardized protocols with their product specifications to automate the collection of data on-board and communicate it to shore-based authorities might constitute a real issue. There is a possibility of packaging the reporting information with a route exchange format (e.g. S-412), but the security of the means of communication would need to be carefully evaluated.

Figure XX – Example of current communication systems used to report information

Some of these reporting systems are not using a digital format, which prevents provision of an automated service. The first step in an operational approach would be the use of a communication system capable of transmitting digital data. Secondly, and given reporting requirements differ among coastal states, a library containing information required by each administration should be developed which complies with the standardized product specifications principle. Specific requirements requested by an administration would be part of the form pertaining to this country.

Figure XX – Example of electronic communication systems using a standardized product specifications library

Basic information such as the ship name, call sign, IMO number and MMSI, etc., can be automatically collected from the ship’s AIS system and transmitted through terrestrial or satellite AIS.

### User Needs

Ships’ masters are facing considerable administrative burdens to comply with different mandatory Vessel Shore Reporting requirements and transmission of  information. Different procedures, data formats, documentary requirements and formalities are applied depending on the country, ship reporting system or port of destination. One of the main challenges is that the ship has to communicate in different manners through different systems.

This, in turn, requires building and maintaining a library of required reports that are uniquely identified and characterized by their requirements for format, deadline, content, etc.  The vessel reporting system also requires developing and maintaining an S-100 Product Specification for CMDS that can be used to generate all required reports in the library.  Lastly it requires that ships’ systems that generate reporting information be certified to be compliant with an international machine-to-machine interface standard or ship network standards such as IEC 61162 series.

Figure x.x.x BAREP SRS between the Russian federation and Norway

### Information to be Provided

See Annex 8, MS 8 Vessel Shore Reporting

### Associated Technical Services

To be capable of generating this information automatically and transmitting it automatically, a realistic operational implementation of the VSR service would require the involvement of both coastal administrations and shipowners. Coastal administrations would need to develop an international library of ship reports that are uniquely identified and characterized by their requirements for format, deadline, content, etc. This library would be part of the S 10x register and standardized product specifications (e.g. S-127 and S-421) would be developed and maintained in order to generate the required reports.

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| S-127 Maritime Traffic Management |  | Marine Traffic Management (MTM) datasets describe the availability and reliability of vessel traffic services, pilotage, routing measures, and ship reporting systems. This includes their service areas, services offered and instructions for contacting or utilizing these services. |  | IHO |
| S-421 Route Plan based on S-100 |  | S-421 specifies the content, structure, and metadata needed for creating a fully S-100 compliant Route Plan Information and its portrayal within an S-100-based Application. |  | IEC |

### Relation to other Maritime Services

Digital information from the ship reporting system using common standards is beneficial for several stakeholders and actors involved.

In addition to authorities (e.g. border police, immigration, defence, coast guard, customs, etc.) claiming the information, the list below gives example of services that can benefit from access to ship reporting information.

| Description | Examples of data that could be used from MS 10 |
| --- | --- |
| MS 1 VTS IS | Type of vessel, nationality, MMSI, IMO no, contact information |
| MS2 VTS NAS | Draft, cargo |
| MS3 VTS TOS | ETA / ATA, ISPS information, purpose for arrival |
| MS4 Local Port Service | The majority of the information in a ship reporting system is useful and can be re-used an effective and transparent port operation |
| MS 5 Maritime Safety Information | Inform about changes to reporting requirements |
| MS 6 Pilotage Service | Ship reporting systems can exchange information with the pilot system. Easy access to information can be important for the pilot. A digital pilot requesting / booking system connected to the ship reporting system will increase efficiency. |
| MS 10 Maritime Assistance Service | Information about cargo, dangerous goods and persons onboard can reduce time before assistance and contribute to the allocation of the appropriate resources for the actual situation. |
| MS 11 Nautical Chart Service | Contain charted information about areas where reporting is required |
| MS 12 Nautical Publications Service | Contain detail information about reporting requirements, such as who must report, when reports are due, and to whom the reports must be submitted. |
| MS 13 Ice Navigation Service | Information from the ship reporting system can contribute to a more tailor-made and effective icebreaker service. |
| MS 16 Search and Rescue Service | Salvage information, drifting patterns, SAR areas, rescue capabilities in the area. |

## MS 9 Telemedical Assistance Service (TMAS)

### Submitting Organisation

### Description of the Maritime Service

According to IMO/ILO resolution 164, the TMAS centre should be able to provide medical advice for seafarers 24 h/day, 365 days/year. TMAS should be permanently staffed by physicians qualified in conducting remote consultations and who are well-versed in the particular nature of treatment on board ship.

Within maritime medicine, the prevailing view has for a long time been that a standardization of the TMAS services is both necessary and wanted. This would enhance the quality of the medical practice. A standardization of reporting and registering of medical events will also make a much better basis for advancement (MSC.1/Circ.1218; MSC/Circ.960).

### Purpose

TMAS can be delivered in all sea areas but differences in bandwidth, depending on available types of transmission techniques, will result in different levels of service.

Seafarers have some basic, but limited, training and skills in medical treatment. Sharing of digital information for diagnosis will increase the rate of appropriate treatment.

It is essential to provide seafarers with easy and fast access to a single point of contact with the TMAS centre,   as well as to sharee information between the TMAS centre and medically responsible personal onboard during treatment.

The purpose of TMAS is to give decision support and advice to the seafarer on board responsible for medical care whenever the provision of treatment cannot wait. This is today mostly done by voice communication using VHF, MF or short wave radio, or mail. In a future e-navigation scenario, the quality of remote diagnostics and even treatment can be expected to improve.

In addition to prevailing TMAS provisions, real time monitoring of the patient’s current health status will be important. The parameters monitored may differ in different systems, but may include heart rate, blood oxygen saturation and pressure. As an example, see the telemetric medical information screen below.

An example of a typical telemetry screen: top (green) curve is heart rate (Beats Per Minute from two redundant connectors), below (in red) is Ambulatory Blood Pressure (ABP) in mmHg, the third curve is Peripheral Capillary Oxygen Saturation (SPO2), then oxygen saturation of the blood (normal is 95-100 %), at the bottom of the screen are Noninvasive Blood Pressure (NBP) and the patient’s temperature in degrees Celsius.

Further needs might involve real-time video chat between the remote specialist doctor and the patient, as well as examinations with remotely controlled cameras. In addition,  extensive, detailed, cardiovascular (heart) data for a period of time should be able to be collected and sent as compressed files for further monitoring by the specialist doctor.

### Operational Approach

One may envision that in the not too distant future, what today is very expensive medical equipment could be installed on (at least some) ships, e.g. large cruise vessels. Examples of such equipment could be[short version] ultrasonography, computer tomography (CT) and surgical operation robots. Such equipment will then be remotely operated from land given reliable radio connections.

Diagnostic ultrasonography
is used to see internal body structures such as tendons, muscles, joints, vessels and internal organs. Its aim is often to find a source of a disease or to exclude any pathology. The practice of examining pregnant women using ultrasound is called obstetric ultrasound, and is widely used.

The European Space Agency has already tested a long-distance robotic ultrasound system for remote operation by distant specialists. An assistant on board simply holds the device against the patient and the ultrasound expert can move the probe as if present in the examination room, rather than thousands of kilometres away. They control the device in real time using a joystick, based on ultrasound imagery relayed back.

CAT
Computed tomography, more commonly known as a CT or CAT scan, is a diagnostic medical test that, like traditional x-rays, produces multiple images or pictures of the inside of the body.

Remote surgery (also known as tele-surgery)
Remote surgery allows the surgeon to remotely perform surgery either using direct tele-manipulator or through computer control. The surgeon does not have to be present, but can be anywhere in the world, leading to the possibility for remote surgery.

### User Needs

Today

A digital health emergency monitoring system allowing logged text interchange between a ship and medical specialist at a shore hospital including:

- Voice communication not only with the medical officer onboard, but also with the patient in the medical bay

- Real-time video for visual examination by the remote specialists using video

- Real-time transmission at length of medical telemetry

- Transmission of compressed data packages of e.g. EEG data

- Up to date digital georeferenced lists of available telemedical assistance services that can be offered by a coastal state. Comparison of TMAS with ship capabilities would facilitate the request of the most adapted service to respond to the situation on board.

In the future (examples)

- Transmission of data and remote control of ultrasonography

- Transmission of data and remote control of computer tomography and the like imaging techniques

- Transmission of data and remote control of robotic surgery

Figure x.x.x remote control of robotic surgery exist onshore already

https://www.welchallyn.com/content/dam/welchallyn/documents/upload-docs/Product-Literature/Brochure/MC11587%20Telemetry%20MedSurg.pdf

http://www.esa.int/Our_Activities/Space_Engineering_Technology/Long-distance_ultrasound_exams_controlled_by_joystick

### Information to be Provided

See Annex 9, MS 9 Telemedical Assistance Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Digital Maritime Services

| Description | Examples of data that could be used from MS 9 |
| --- | --- |
| MS 1 VTS IS | Communication with ship |
| MS 12. Nautical Publications Service | Provide up to date information on radio station and earth stations offering TMAS. |
| MS 16 Search and Rescue Service | Medical information and constraints for SAR procedures. |

## MS 10 Maritime Assistance Service (MAS)

### Submitting Organisation

### Description of the Maritime Service

IMO Resolution A.950(23) on Maritime Assistance Services (MAS) specifies that the circumstances of a ship’s operation that involve a MAS are not those requiring rescue of persons. Three situations can arise:

- the ship is involved in an incident (e.g., loss of cargo, accidental discharge of oil, etc.) that does not impair its seakeeping ability but nevertheless has to be reported;

- the ship, according to its master’s assessment, is in need of assistance but not in a distress situation (about to sink, fire developing, etc.) that requires the rescue of those on board; and

- the ship is found to be in a distress situation and those on board have already been rescued, with the possible exception of those who have remained aboard or have been placed on board to attempt to deal with the ship’s situation.

If, however, in an evolving situation, the persons on board find themselves in distress, the involvement of the MRCC and not the MAS will have priority. As such, the MAS is responsible only for receiving and transmitting communications and monitoring the situation.

The establishment of a MAS should not necessarily entail the setting up of a new organization. The functions of the MAS could, at the discretion of the Administration, be discharged by an existing organization, preferably an MRCC, or alternatively a harbour master’s office, a coast guard operations centre (if one exists) or another body.

Coastal States are requested to notify IMO of the existence and details (call numbers, call signs, etc.) of their MAS, in accordance with the predetermined format (A 23/Res.950, appendix to annex 2). The IMO Secretariat will periodically publish the collected particulars in a circular. National organizations that disseminate nautical information are invited to publish such particulars.

### Purpose

The primary mission of the MAS is to manage communications between the coastal state, ship's officers requiring assistance, and other responsible maritime organizations: fleet owners, salvage companies, port authorities, brokers, etc. As an intermediary, the main purposes of the MAS are:

•	 receiving the reports, consultations and notifications required by the IMO instruments referred to in Annex 1 to the present resolution;

•	monitoring the ship situation if a report, as referred to in paragraph (a) above, discloses an incident that may cause the ship to be in need of assistance;

•	serving as the point of contact between the master and the coastal state concerned, if the ship’s situation requires exchanges of information between the ship and the coastal state but is not a distress situation that could lead to a search and rescue operation;

•	serving as the point of contact between those involved in a marine salvage operation undertaken by private facilities at the request of parties having a legitimate interest in the ship and the coastal state, if the coastal state concerned decides that it should monitor all phases of the operation.

### Operational Approach

A 23/Res.950 indicates that a MAS should be operational on a 24-hour basis and that it should be possible for the English language to be used in exchanges between a ship in need of assistance and a MAS. Also, MASs should be authorized by their respective governments to exchange information concerning reports received and situations involving ships which may be in need of assistance with each other.

The most common events requiring MAS are (IMO A23/Res. 949):

- fire

- explosion

- damage to the ship, including mechanical and/or structural failure

- collision

- pollution

- impaired vessel stability

- grounding.

Development of scenarios including their potential consequences  with regard to safety of persons and pollution, fire, toxic and explosion risks.

The development of a database containing the key services contact information and availability of their respective resources would help move towards the digitalization of the MAS. Also, the use of templates listing the types of incidents that commonly occurring and their level of severity could facilitate ship – shore based communication.  The exchange of information on key services, their contact details, capabilities and geographic service areas could be developed as an S-100-based product specification. On the ship side, digital nautical publications in the S-12x-series of product specifications, such as S-123 (Marine Radio Service), could contain MAS information.

Figure xx illustrates briefly the different functions of a MAS.

Figure xx. Illustration of some functions provided by a MAS

### User needs

- Table X.X – Examples of types of information provision that MAS can cover

| Information related to: | Examples |
| --- | --- |
| General information | - competent MAS - roles and responsibilities of authorities and responders (fire fighting capability) - Responsible SAR coordination centre - Contact information to MRCC / operator - Ongoing operations in an area |
| MAS information | - MAS geographical coordinates, contact information, capacity, etc. - Information about available resources for lightering - Information about available resources for pollution combating and recovery - Information about available resources for towage - Information about available resources for stowage - Information about available resources for salvage - Information about available resources for storage. - Contingency planning - Evacuation facilities - Type of operation - What and when - Vessels involved - Communication |

In some of these situations, the ship’s assistance may require the designation of a place of refuge in order to limit the navigational impacts, protect the environment or facilitate repairing the ship. In that case, an assessment of risks related to the identified event should be done taking into account:

1.	Environmental and social factors:

- safety of those on board

- threat to public safety

- pollution caused by the ship

- sensitive habitats and species

- facilities available

- etc.

2.	Natural conditions:

- Prevailing winds and sea conditions

- Tides and tidal currents

- Bathymetry

- Etc.

### Information to be Provided

See Annex 10, MS 10 Maritime Assistance Service

### Associated Technical Services

Regarding communication facilities, IMO Res. 950 recommends the  use of COMSAR/Circ.18, Guidance on minimum communication needs of maritime rescue co-ordination centres (MRCCs), as a basis. This Circular identifies the following communication systems:

- Telephone links

- Fax links

- Telex link

- VHF and MF with DSC

- Ordinary VHF

- Special networks : examine the possibility of linking with existing networks (Administration, armed forces telephone networks), in particular the aeronautical fixed telecommunication network (AFTN), which provides a link with the aeronautical rescue organization

- Inmarsat-C ship earth station or other satellite service providers

- Etc.

S-123 (Marine Radio Service) data products contain contact details for MRCCs.

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other the Maritime Services

| Description | Examples of data that could be used in MS 10 |
| --- | --- |
| MS 1 VTS IS | Local sensor information such as CCTV, Radar, AIS. Regulations. Other traffic. |
| MS 2 VTS NAS | Exchange of routes, directions, navigation advices and assistance |
| MS3 VTS TOS | Information regarding regulations and other traffic |
| MS 4 Local Port Service | Port availability and anchorage are in the port, services related to the MAS situation |
| MS 5 Maritime Safety Information | All information depending on the structure of MSI |
| MS 6 Pilotage Service | Contact information for pilotage, pilot assistance, pilot request |
| MS 7 Tug Service | Tug capacity, contact information, tug order and updates |
| MS 8 Vessel Shore reporting | Information about the reporting formalities, local regulations, contact information, notification about dangerous cargo, number of persons onboard, arrival and departure information etc. |
| MS 9 Telemedical | Contact Information |
| MS 11 Nautical Chart Service | Local area updates, chart updates, Notice to Mariners |
| MS 12. Nautical Publications Service | Tidal tables, Notice to Mariners, List of Lights, information updates to publications |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | Weather information, visibility, wave information |
| MS 15 Real Time Hydro and Information Service | Horizontal and vertical tidal information in an area, real time current, real time wave form sensors in an area |
| MS 16 Search and Rescue Service | Search and Rescue contact information, SAR capacity. SAR areas of operations. |

## MS 11 Nautical Chart Service

### Submitting Organisation

Hydrographic Office

### Description of the Maritime Service

The aim of the nautical chart service is to safeguard navigation at sea by providing information, such as the nature and form of the coast, water depth, tides table, obstructions and other dangers to navigation, location and type of aids to navigation.  Most often, a nautical chart service would be signatories of the IMO SOLAS Convention where the responsibilities of a hydrographic service is defined

### Purpose

Contracting Governments to SOLAS V undertake to arrange for the collection and compilation of hydrographic data and the publication, dissemination and keeping up to date of all nautical information necessary for safe navigation. In particular, Contracting Governments undertake to co-operate in carrying out, as far as possible, the following nautical and hydrographic services, in the manner most suitable for the purpose of aiding navigation:

- to ensure that hydrographic surveying is carried out and adequate, as far as possible, to the requirements of safe navigation;

- to prepare and issue nautical charts, sailing directions, lists of lights, tide tables and other nautical publications, where applicable, satisfying the needs of safe navigation;

- to promulgate notices to mariners in order that nautical charts and publications are kept, as far as possible, up to date; and

- to provide data management arrangements to support these services.

Contracting Governments undertake to ensure the greatest possible uniformity in charts and nautical publications and to take into account, whenever possible, relevant international resolutions and recommendations, which means the appropriate resolutions and recommendations adopted by the International Hydrographic Organization (IHO).

IHO facilitates co-ordination of the hydrographic office activities to the greatest possible degree in order to ensure that hydrographic and nautical information is made available on a world-wide scale as timely, reliably, and unambiguously as possible.

Raster charts services also continue to be available and there is expected to be a continued demand for this service  at least during the transition period.

Examples of Nautical Chart Service is listed in Table x.

Table x – Nautical Chart Service

| Information related to: | Examples: |
| --- | --- |
| Navigational charts | Paper charts Raster Navigational Charts (RNC) Electronic Navigation Charts (ENC) |
| Chart catalogue | Catalogue of available products |
| Bathymetric charts | Bathymetric Information with greater details than normally given in a navigational chart |
| Notice to Mariners | Regular updating service for products issued by the individual Nautical Chart Service. |
| Hydrography | Information that will include factors such as the stability of the seabed, sea depth, the accuracy of surveys, tidal ranges, tidal streams, prevailing currents and swell, etc. |

### Operational Approach

A hydrographic service produces and distributes paper charts, ENCs and other products to safeguard navigation at sea. These services are carried out as defined in the IHO publications M-3 and, for ENCs, the WEND (Worldwide Electronic Navigational Chart Database) principles (M-3, Resolutions of the IHO – Resolution 1/1997 (as amended)) which note that:

“The purpose of WEND is to ensure a world-wide consistent level of high-quality, updated official ENCs through integrated services that support chart carriage requirements of SOLAS Chapter V, and the requirements of the IMO Performance Standards for ECDIS.”

The IHO S-65 Standard notes that hydrographic offices are responsible for:

- The preparation and provision of digital data and its subsequent updating for waters of national jurisdiction.

- Ensuring that, mariners, anywhere in the world, can obtain fully updated ENCs for all shipping routes and ports across the world and that their ENC data are available to users through integrated services.

- Assuring the high quality of its ENC services through the use of a Quality Management System,certified by a relevant body as conforming to a suitable recognised standard.  Typically this will be ISO 9001.

- Ensuring compliance with all relevant IHO and IMO standards and criteria (including IHO S-57, IHO S-52, or their replacements).

- Providing timely updates to the ENC for the mariner. These should be at least as frequent and timely as those provided by the Contracting Government for the correction of paper charts.

Similar arrangements and principles apply to nautical charts and other nautical products (e.g. S-102 high definition bathymetry) made available by hydrographic offices.

### User Needs

Nautical chart services should offer both paper charts and digital charts in raster and vector formats for use in ECDIS or ECS, depending on vessel classification. Nautical charts should be made available as widely as possible, using various distribution channels.  This may include direct distribution, agents and other service providers. ENC may be distributed via a RENC, which distributes them onward with the distributor network.

Figure 4.11-1 - General principle of ENC distribution (Source: S-65)

A chart catalogue service should be included as part of a Nautical Chart service to give users a quick reference guide regarding the products offered.

IHO is developing the next version of ENC, called S-101. It should be expected that a new ECDIS or ECS system may be needed to utilize these next generation ENC. For future nautical chart services, the commercial model of distribution is expected to continue. Chart vendors provide for licensing and payment on a when-needed basis even now, and this will continue to be a requirement. The technical specifications of this service must support licensing, payment (both subscription-based and spot payments), and data protection.

Requirements for future services include faster turnaround to process mariner reports to agencies and the issuance of chart updates, making it easier to include more types of relevant information with reports such as information in mixed media, and reporting methods that are better integrated with ENCs. For example, initial generation of a hydrographic report on an ECDIS followed by automatic transfer of the initial report to a back-of-bridge system and thence to the appropriate hydrographic office.

Provision should be made for value-added data (VAD) and value-added services (VAS) supplied by vendors (this means that VAD is inherent to chart data and not an overlay ), including SENC distribution.

Raster chart services also continue to be available and there is expected to be a continued demand during the transition period at least.

### Information to be Provided

See Annex 11, MS 11 Nautical Chart Service.

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

MS 12 and DMS 13 will supplement MS 11 data for navigation purposes. MS 5 will supplement it with up-to-date information on the status of navigation aids, obstructions to shipping and other events that impact navigational safety.

| Description | Examples of data that could be used in MS 11 |
| --- | --- |
| MS 5 Maritime Safety Information | Provides supplemental up-to-date information on the status of navigation aids, obstructions to shipping and other events that impact navigational safety. |
| MS 12 Nautical Publications Service | Provides supplemental navigational information |
| MS 13 Ice Navigation Service | Provides supplemental navigational information |
| MS 15 Real Time Hydro and Information Service | Provides supplemental navigational information |

## MS 12 Nautical publications service

### Submitting Organisation

### Description of the Maritime Service

The aim of the nautical publication service is to promote navigational awareness and safety to navigation. Nautical publications include tidal currents, aids to navigation system, buoys and fog signals, radio aids to marine navigation, chart symbols, terms and abbreviations, sailing directions and notices to mariners and local notices. Nautical publications often include information necessary for navigation that cannot be included in a chart service, for example blocks of text or information that are useful to mariners but not essential to the route monitoring. When the S-100 based product specifications are operational, the difference between ENC and many nautical publications will be based on content rather than format, since both will be available as feature-based datasets.

### Purpose

A nautical publication service is normally provided by a national hydrographic organization, most often signatory to tthe IMO SOLAS Convention. Responsibilities of a hydrographic service are defined as follows

Contracting Governments to SOLAS V undertake to arrange for the collection and compilation of hydrographic data and the publication, dissemination and keeping up to date of all nautical information necessary for safe navigation. In particular, Contracting Governments undertake to co-operate in carrying out, as far as possible, the following nautical and hydrographic services, in the manner most suitable for the purpose of aiding navigation:

- to ensure that hydrographic surveying is carried out, as far as possible, with respect to the requirements of safe navigation;

- to prepare and issue nautical charts, sailing directions, lists of lights, tide tables and other nautical publications, where applicable, satisfying the needs of safe navigation;

- to promulgate notices to mariners so that nautical charts and publications are kept, as far as possible, up to date; and

- to provide data management arrangements to support these services.

Contracting Governments undertake to ensure the greatest possible uniformity in charts and nautical publications and to take into account, whenever possible, relevant international resolutions and recommendations, which means the appropriate resolutions and recommendations adopted by the International Hydrographic Organization (IHO).

IHO facilitates co-ordination of the hydrographic office activities to the greatest possible degree to ensure that hydrographic and nautical information is made available on a world-wide scale as timely, reliably, and unambiguously as possible. Moreover, the IHO publication M-3 contains technical resolutions and recommendations that define how nautical publications shall be produced to be compliant with SOLAS (NAUTICAL PUBLICATIONS AND THE SOLAS CONVENTION, 3/2002 as amended).

While they have been largely book-based until now, IHO is actively working on making several of their publications more ECDIS-friendly by making them available as feature-based datasets.

Nautical publications are provided the majority of the time as printed paper and digital formatpublications and distributed via commercial channels. Updates and corrections on the status of navigation aids and lights are often distributed via agency web sites as well as through the distribution channels for the originals.

Examples of Nautical Publications Service are listed in Table x.h

Table x – Nautical Publications Service

| Information related to: | Examples: |
| --- | --- |
| Transits and routing | Routes in constricted shipping lanes or Routing measures, traffic separation schemes, and shipping lanes |
| Summary information about port facilities | Depth alongside berths, and quay lengths Cargo handling facilities at specified terminals and berths |
| Variations from charted information | Tendencies toward silting at river mouths, shifting sandbanks, etc., that may degrade the accuracy of charted information |
| Marine radio services | Geographic availability of services Frequencies and channels used and broadcast schedules Type of traffic supported – Weather forecasts, radiofax, telemedical assistance, etc. |
| Protected area information | Locations of marine protected areas Restrictions and regulations applicable within specific areas |
| Prevailing natural conditions | Seasonal hazardous conditions Periodic (e.g., tide-related) or irregular hazardous conditions |
| Regulatory information | Laws and regulations applicable in specific locations. Laws and regulations applying to vessels of specific dimensions or carrying specified cargo Local rules regarding use of specific pilot boarding places by vessels exceeding specified dimensions or carrying hazardous cargo |
| Services | Waste disposal, repair, bunkering, collection of ship pollutants such as oily wastes Pilot services contact information and notice times |
| Navigation aids | List of Lights |
| Tide information and forecasts | Tide tables, tide stream atlases |
| Planning | Routing guides |
| Controlled areas | VTS contact information |

### Operational Approach

A hydrographic service produces and distributes nautical publications to promote navigational awareness and safety to navigation. These services are carried out as defined in the IHO publication M-3. Technical resolution 1/2002 gives the following exhaustive list of nautical publications, but note that other publications may also be added.

Distance Tables

List of Buoys and Beacons

List of Lights

List of Radio Signals

List of Symbols, Abbreviations and Terms used on Charts

Mariners’ Handbooks

Notices to Mariners

Routeing Guides

Sailing Directions

Tidal Stream Atlases

Tide Tables

Moreover, M 3 states that nautical publications may be in printed form or digital form (Technical resolution 2/2002 as amended). The digital form may be a digital representation of the printed version, such as is often the case with PDF files, and it may be in other forms such as XML. M-3 classifies the different types of nautical publications in the following manner;

NP1 – Printed paper publications

NP2 – Digital publications based upon existing paper publications

NP3 – Digital dataset(s) fully compatible with ECDIS that serve the purpose otherwise provided by NP1 or NP2.

It should be noted that Data Specifications for NP3 have yet to be finalised and IHO is working on developing S-100 based product specifications that are functionally equivalent to the paper chart versions. These are often referred to as the S-12x series of standards.

Such a service might accept ports of call for a voyage, vessel and cargo characteristics, and return a catalogue of required publications and their latest revision information. , to This could be be compared by on- board software aid to digital publications currently installed, resulting in generation of a compliance report for review and action by the navigation officer.

NP2 nautical publications may be used back of bridge or even on a tablet computer. NP3 is expected to interact in various degrees with the next generation ENC within the ECDIS or ECS system, though some will not be included in an ECDIS at all – e.g., routeing guides.

Short update cycles are preferable – but must be consistent with the nature of the content and the needs of the vessel. For example, port information for a destination on the current voyage is higher priority than Chart No. 1 or the List of lights – and information about changes to the status of a navaid are higher priority than most other information.

Updates should be in a form that can be applied automatically to on- board datasets.

Automated aids for reporting discrepancies should be provided.

Consumers of the service should have the ability to obtain up-to-date information pertaining to a planned voyage, not just in the present area.

The services should be capable of working within multiple levels of bandwidth limitations, which will vary depending on whether the vessel is on the high seas, approaching a harbour, engaged in loading/unloading operations in port, etc. Performance should degrade gracefully with reductions in bandwidth, e.g., for low-bandwidth channels transfer of large graphics files may be deferred or replaced with lower-quality files.

Pictorial information needs to be transferred, but consistent with bandwidth limitations, if any, e.g., deferred updates for non-essential graphics, or transmission of lower-quality graphics.

Services should be compatible with WFS and WCS (Web Coverage Service) standards for providing up-to-date information about individual features, features within specified spatial constraints, and coverage data within specified spatial constraints. This will need to be integrated with licensing and subscription-based access at least, probably data protection as well.

In cases where information exists in narrative form, the information will need to be summarized or condensed by a producer before it is disseminated.

Some information (such as text from sailing directions) will have to be converted from narrative to geographic form .

1.1.1.	INFORMATION TO BE PROVIDED

### User Needs

Users should be trained in understanding the use of nautical publications.

SOLAS V, Regulation 27 requires that nautical publications must be adequate for the intended journey, and always be up-to-date. This update should be done using the means provided by the issuing hydrographic office or approved service provider. A shore-based Internet service that is kept up-to-date with carriage requirements would help mariners validate that they have the latest version on board  .

See Annex 12, MS 12 Nautical Publications Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

| Description | Examples of data that could be used in MS 12 |
| --- | --- |
| MS 1 VTS IS | Input information for nautical publications which act as an overview of VTS information. |
| MS 2 VTS NAS | Input information for nautical publications which act as an overview of VTS information. |
| MS 3 VTS TOS | Input information for nautical publications which act as an overview of VTS information. |
| MS 4 Local Port Service | Input information for nautical publications which act as an overview of service information in ports. |
| MS 5 Maritime Safety Information | Provides supplemental up-to-date information on the status of navigation aids, obstructions to shipping and other events that impact navigational safety. |
| MS 6 Pilotage Service | Input information for nautical publications which act as an overview of pilot service information. |
| MS 8 Vessel Shore reporting | Input information for nautical publications which act as an overview of ship reporting information. |
| MS 9 Telemedical | Input information for nautical publications which act as an overview of radio station information. |
| MS 10 Maritime Assistance Service | Input information for nautical publications which act as an overview of MAS information. |
| MS 11 Nautical Chart Service | Provides supplemental navigational information |
| MS 13 Ice Navigation Service | Provides supplemental up-to-date information on the status of ice navigational information |
| MS 14 Meteorological Service | Provides supplemental up-to-date information on the status of ice meteorological information |
| MS 15 Real Time Hydro and Information Service | Provides supplemental navigational information |
| MS 16 Search and Rescue Service | Input information for nautical publications which act as an overview of contact procedures for SRS. |

## MS 13 Ice navigation service

### Submitting Organisation

World Meteorological Organisation

### Description of the Maritime Service

To provide ice navigation information digitally to ships

### Purpose

- The World Meteorological Organization’s Manual on Marine Meteorology (WMO No. 558) defines the procedures for marine meteorological information text bulletins involving ice-related components in High Seas areas, Coastal, Offshore and Local Waters.

- SOLAS V Regulation 5 outlines obligations for the provision of weather information suitable for shipping with forecasts including ice conditions and hazards.

- The IMO Polar Code (MSC, MEPC) outlines the environmental information requirements for ships operating in polar waters.  The IMO Polar Code reinforces operating guidelines for the hazard of Ice Accretion and Ice Waters (sea-ice conditions), and introduces new operating guidelines for hazards related to Low Air Temperature, and defines the Polar Service Temperature for equipment performance.

SOLAS V, regulation 6 governs the ice patrol service in the North Atlantic stating that “Ice Patrol contributes to safety of life at sea, safety and efficiency of navigation and protection of the marine environment in the North Atlantic. Ships transiting the region of icebergs guarded by the Ice Patrol during the ice season are required to make use of the services provided by the Ice Patrol.” This service is currently carried out jointly by USA and Canada.

- The IMO Assembly Resolution A.1051(27) outlines the functions of the Worldwide Met-Ocean Information and Warning Service. The Worldwide Met-Ocean Information and Warning Service (WWMIWS) provides met-ocean Maritime Safety Information (MSI) including sea-ice conditions and hazards to mariners.

- The standards for the portrayal of ice conditions on Electronic Navigational Chart systems are documented within S-411.

WMO publication 574 (Sea Ice Information Services of the World) has comprehensive information on National Ice Services.

### Operational Approach

Examples of Ice Navigation Services are listed in Table x.

Table x – Ice Navigation Service

| Information related to: | Examples: |
| --- | --- |
| Ice conditions | Near real-time and forecasts of: - Sea ice concentrations - Sea ice stage of development (type or thickness) - Iceberg location and drift - Limit of all known sea ice or icebergs |
| Ice reports and bulletins | - Text summaries of ice conditions |
| Routeing aid | - Recommended routes - Icebreaker assistance |

The most important features of sea ice which affect marine operations are:

(a)	the amount of ice present, I.e. concentration usually measured as tenths of the sea surface covered by ice;

(b)	ice thickness, referred to as stage of development which is related to ice age;

(c)	form of ice, i.e. whether it is fast ice or pack ice, floe size, and the amount of ridging; and

(d)	ice movement.

The position of icebergs at specified times are required with information about their estimated size; and speed and direction of movement.

Warnings of ice accretion highlight areas where the accumulation of ice on the superstructure and deck equipment of vessels may seriously affect safety and operational efficiency.

Information about extremely low air temperatures is important for the safety of workers, while historical information about cold air temperatures enables planning and ship design based on the Polar Service Temperature guidelines.

#### Ice charting

Some 20 nations around the world offer an ice information service. Services may provide ice forecasts once a day for a period of 24 to 144 hours. These are tactical forecasts and may provide advice on difficult ice conditions forming or dissipating, the general motion of the pack, opening and closing of leads, etc. They are strongly influenced by meteorological prediction and should always be used in concert with the weather forecast.

Other longer-range predictions – those covering periods from 7–10 days to 30 days and seasonal predictions – are  based on climatological,  analog or statistical methods.

#### Vessel Escort and Ice breaking

Icebreaking and support services may be available to ships transiting ice-covered waters. Coast Guards or other national agencies may operate Ice Operations Centres. These Centres generally provide up-to-date ice information, suggest routes for ships to follow through or around ice, and co-ordinate icebreaker assistance to shipping. Ice Operations Centres are generally in contact with icebreakers at all times and monitor progress of shipping within their area of responsibility. Ice Operations Centres may also provide Recommended Ice Routing services, such as routing maps.

### User Needs

Activity based information requirements

| Information related to | Examples |
| --- | --- |
| En-route or at sea | Broad, area-based forecasts Higher detail in complex waterways Increased interest in synoptic features and movement Longer forecast lead-time essential |
| Entering, transiting and exiting a port | Point (small area) based forecasts, High spatial and temporal detail Real-time observations Focus on short-term lead-times |
| At berth | Forecasts of changes to ice conditions |
| Planning a trip | Focus on short-term timeframes, as well as longer forecast lead-times Forecasts and warnings Specific details on timing of wind changes or hazardous weather leading to changes in ice conditions Focus on forecast details for specific areas or routes |
| Vessel and equipment design | Historical values of low air temperatures and water temperatures Focus on ocean and sea routes |

Formats

Ice information may be provided in a number of formats to meet user requirements.  The following descriptions outline some of the benefits and constraints for each format:

- Map

  - Map displays provide highly detailed information across defined spatial domains, and if provided as a time sequence, a user can study the evolution of the weather, ice and ocean elements during specific time periods.  Colours may be used to highlight hazards or important conditions to operations. Maps may be produced in an image format, or provided as a web map service, gridded or S-100 compatible file for display by on-screen software.

- Text

  - Text products provide short summaries and broader detail for a defined area and time period.  These text products may be simpler to interpret for most users, and can be used for marine radio broadcasts.  Text products generally have a small file size for internet dissemination to mariners at sea.

- Voice

  - Voice products may be transmitted as audio or by video accompanied by other formats.

- Table

  - Information displayed as a table is usually for a specific location, so a user will get the benefit of detailed information over a period of time for that location but may lose context of what is happening over nearby areas.

- Grids

  - Gridded information may be integrated into decision support systems and situational awareness tools, or interrogated to output customized information for the particular marine activity or operating risk threshold.

#### Examples

Figure xx: Example of iceberg analysis.

Figure xx Example of a Recommended Ice Route in the Gulf of St. Lawrence (source ”Ice Navigation in Canadian Waters” – Canadian Coast Guard )

Figure xx - Example of Sea Ice satellite service (www.polarview.aq)

Figure xx – A MODIS satellite image taken on March 17, 2015 of the Gulf of St. Lawrence. The image highlights compacted first-year ice along Anticosti Island as well as young ice and grey-white ice off the coast. (Image credit: NASA)

### Information to be Provided

See Annex 13, MS 13 Ice Navigation Service Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

| Description | Examples of data that could be used in MS 13 |
| --- | --- |
| MS 5 Maritime Safety Information | Provides supplemental up-to-date information on the status of ice dangers. |
| MS 11 Nautical Chart Service | Provides supplemental navigational information |
| MS 12 Nautical Publication Service | Provides supplemental navigational information |
| MS 14 Meteorological Service | Provides supplemental navigational information |
| MS 15 Real Time Hydro and Information Service | Provides supplemental navigational information |

## MS 14 Meteorological information service

### Submitting Organisation

World Meteorological Organization

### Description of the Maritime Service

To provide meteorological information digitally to ships

### Purpose

- The World Meteorological Organization’s Manual on Marine Meteorology (WMO No. 558) defines two types of marine meteorological information:

- Forecasts and warnings for the High Seas; and

- Forecasts and warnings for coastal, offshore and local areas (including ports and harbours).

- SOLAS V Regulation 5 obligates contracting parties to produce and distribute to shipping warnings about severe weather such as gales, storms and tropical cyclones, and to produce and provide other weather information suitable for shipping consisting of data, analyses, warnings and forecasts of weather, waves and ice.

- The IMO Assembly Resolution A.1051(27) outlines the functions of the Worldwide Met-Ocean Information and Warning Service. The Worldwide Met-Ocean Information and Warning Service (WWMIWS) provides Maritime Safety Information (MSI) to mariners in the form of marine forecast and warning products. The WWMIWS is coordinated across the worlds’ oceans through 21 defined areas, called METAREAs. Ships receive the MSI products via marine communication systems such as SafetyNet and NAVTEX, which form part of the Global Maritime Distress and Safety System (GMDSS).

- The IMO Polar Code (MSC, MEPC) outlines the environmental information requirements for ships operating in polar waters.  The IMO Polar Code reinforces operating guidelines for the hazard of Ice Accretion and Ice Waters (sea-ice conditions), and introduces new operating guidelines for hazards related to Low Air Temperature, and defines the Polar Service Temperature for equipment performance.

- Weather routing services are provided in accordance with SOLAS Chapter V, Regulation 34, and IMO Resolution A.893 and IMO MSC/Circular 1063 – Minimum Standards for Provision of Weather Routeing Services, outlines the minimum characteristics for a service.  The SOLAS Chapter V, Regulation 5 states that met-ocean services shall be issued by the National Meteorological Service, and this would imply that WMO and its Members should oversee weather routing services and standards as well.

- The standards for the portrayal of met-ocean conditions on Electronic Navigational Chart systems are documented within S-412.

- Details of service availability, broadcast times and radio frequencies for services provided to vessels at sea are maintained in the WMO publication: WMO No. 9, Volume D, Information for Shipping.

Examples of Meteorological Information Service is listed in Table x.

Table x – Meteorological Information Service

| Information related to: | Examples: |
| --- | --- |
| Wind | Wind speed, direction, gust information Real-time values from instruments or satellite |
| Waves | Forecast wave height, direction, period Real-time values from buoys or satellite |
| Atmospheric conditions | Forecast temperature, squalls, cloud, rainfall Real-time values from instruments or satellite |
| Ocean | Forecast surface temperature, currents, salinity Forecast sub-surface temperature and currents Real-time values from instruments and satellite |
| Weather systems | Mean Sea Level Pressure contours System features such as cold fronts, tropical cyclones, low pressure centres, high pressure centres Satellite images |
| Dangerous weather | Warnings about location, strength, and movement of storms Warnings about fog or phenomena causing reduced visibility, ice accretion, cold air temperature, squalls |
| Bulletins and forecasts | Surface weather analysis, synoptic features with barometric pressure Forecasts of wind, waves, weather |
| Polar Service Temperature | Historical values for ocean and port areas |
| Low Air Temperature | Forecasts of hazard areas Historical values for ocean and port areas |
| Ship observations | Receipt of reports from ships in the Voluntary Observation System Transmission of information extracted from received ship reports to shipping |

MS14 can be delivered in all sea areas (1-6).

### Operational Approach

In general, marine meteorological services have two functions:

- To serve international shipping, fishing and other marine activities on the high seas; and,

- To serve the various activities which take place in coastal and offshore areas, ports and on the coast.

SOLAS convention (Chapter V, Regulation 34, and within Annex A.24 Voyage Planning) describes how vessels should prepare for their trip and route and therefore their information requirements.  The Annex specifically outlines to small vessels the importance of:

- checking the weather forecast for the journey;

- knowing the tides;

- knowing the vessel limitations for the expected weather and wave conditions.

SOLAS V Regulation 5 describes the underlying obligations for weather services, i.e., conveying warnings about severe weather and other weather information useful for shipping, and facilitating weather reports by ships and their distribution as needed for the safety of navigation.

In general, the impact which could result from a meteorological condition depends on its severity and on the sensitivity of a particular activity or operation to that condition. Similarly, meteorological phenomena can make recreational activities and the work of fishing and shipping fleets much more difficult or hazardous.

Marine operations are sensitive to environmental conditions. Generally, extreme values of waves, wind and obstructions to visibility increase the risk to the safety of the vessel or sea structure and to the persons involved in the operation. Less extreme values, even if safety is not threatened, will affect the efficiency, effectiveness or comfort of the operation. The usefulness of a warning or a forecast depends on the accuracy of the prediction, the format and communication platform through which the information is delivered, its timeliness, i.e. the number of hours or days in advance of the event that the forecast can be provided, and the ability of the user to react to the information.

### User Needs

#### Activity based information requirements

| Information related to | Examples |
| --- | --- |
| En-route or at sea | Broad, area-based forecasts Higher detail in complex waterways Increased interest in synoptic features and movement Longer forecast lead-time essential |
| Entering, transiting and exiting a port | Point (small area) based forecasts, High spatial and temporal detail Real-time observations Warnings of reduced visibility, squalls Focus on short-term lead-times |
| At berth | Warnings of squalls, thunderstorms Forecasts of general weather conditions |
| Planning a trip | Focus on short-term timeframes, as well as longer forecast lead-times Increased interest in synoptic features and movement Forecasts and warnings Specific details on timing of wind changes or hazardous weather Focus on forecast details for specific areas or routes |

#### Formats

Weather information may be provided in a number of formats to meet user requirements.  The following descriptions outline some of the benefits and constraints for each format:

- Map

  - Map displays provide highly detailed information across defined spatial domains, and if provided as a time sequence, a user can study the evolution of the weather and ocean elements during specific time periods.  Maps may be produced in an image format, or provided as a web map service, gridded or S-100 compatible file for display by on-screen software.

- Text

  - Text products provide short summaries and broader detail for a defined area and time period.  These text products may be simpler to interpret for most users, and can be used for marine radio broadcasts.  Text products generally have a small file size for internet dissemination to mariners at sea.

- Voice

  - Voice products may be transmitted as audio or by video accompanied by other formats.  There may be time limits imposed for radio broadcasts, and consideration should also be given to the reception quality on board vessels and impact on a mariner’s ability to interpret the information whilst doing other duties if the broadcast is too long.  These constraints have an impact on the information provided in the text product that it is based on.

- Table

  - Information displayed as a table is usually for a specific location, so a user will get the benefit of detailed information over a period of time for that location but may lose context of what is happening over nearby areas.

- Grids

  - Gridded information may be integrated into decision support systems and situational awareness tools, or interrogated to output customized information for the particular marine activity or operating risk threshold.

#### Examples

- Examples of products include surface analysis, wind and wave forecasts and analyses.

Figure 4.14-1. Surface analysis, West Atlantic. (NOAA)

Figure 4.14-2. Wind and wave forecast. (NOAA)

Figure 4.14-3. Wind and wave analysis (NOAA)

### Information to be Provided

See Annex 14, MS 14 Meteorological information service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Digital Maritime Services

| Description | Examples of data that could be used in MS 14 |
| --- | --- |
| MS 5 Maritime Safety Information | Provides supplemental up-to-date information on the status of extreme weather. |
| MS 11 Nautical Chart Service | Provides supplemental navigational information |
| MS 12 Nautical Publication Service | Provides supplemental navigational information |
| MS 13 Ice Navigation Service | Provides supplemental navigational information |
| MS 15 Real Time Hydro and Information Service | Provides supplemental navigational information |

## MS 15 Real-time hydrographic and environmental information services

### Submitting Organisation

TBA

### Description of the Maritime Service

The real time hydrographic and environmental information service is essential to safeguard navigation at sea and protect the environment.  The services provided include:

current speed and direction;

wave height;

marine habitat and bathymetry;

- Sensor networks such as ODAS, wave radar, water level gauges, anemometers, current gauges, etc., are widespread and utilized in providing data for these services. These data are often made available on web sites, web mapping services, and other appropriate internet locations such as major ports or VTS. However, the data format appears to be variable.

Examples of Real-time Hydrographic and Environmental Information Service are listed in Table x.

Table x – Real-time Hydrographic and Environmental Information Service

| Information related to: | Examples: |
| --- | --- |
| Current speed and direction | buoy information via AIS Surface current snapshots |
| Surface conditions | Wave heights via radar |
| Environmental conditions | Temperature, pressure, tendencies (rising/falling) Wind speed and direction Visibility |
| Water column | Depths, salinity, temperatures |

In general MSP 15 services can be delivered in all sea areas (1-6). Specific services may be limited to areas where the type of hydrographic and environmental information conveyed by the service is collected.

### Purpose

The main objective of this service is similar to MS 14, and consists of transmitting hydrographic and environmental information to ships wherever it is needed. IHO M-3 2/1962 as amended recommends that the results of oceanographic observations be communicated for the maximum utilization by all marine scientific and hydrographic users.

Another objective is the transmission of environmental conditions by ships and the distribution of information gathered from weather reports to shipping.

Local and regional services may transmit hydrographic and environmental information with AIS-ASM, specifically messages 6 and 8.

IHO is developing a product specification for surface currents named S-111, which is intended for use in ECDIS.

### Operational Approach

### User Needs

Real-time hydrographic and environmental information is distributed in a variety of ways, including radio, AIS-ASM, and internet.

For accurate on board display of AIS messages, receiving vessels must be equipped with a proper Class A AIS device.

Internet access and possibly a user account is required to access real-time hydrographic and environmental information via internet.

### Information to be Provided

See Annex 15, DMS 15 Real time hydrographic and environmental information services

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardization Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

| Description | Examples of data that could be used in MS 15 |
| --- | --- |
| MS 1 VTS IS | MS1 for communication |
| MS 5 Maritime Safety Information | Communicates warnings about extreme cases |
| MS 11 Nautical Chart Service | Provides supplemental navigational information |
| MS 12 Nautical Publication Service | Provides climatic data to supplement real time data |
| MS 13 Ice Navigation Service | Provides supplemental navigational information |
| MS 14 Meteorological Service | Provides meteorological information that may impact real time information, especially when related to same event |

## MS 16 Search and Rescue (SAR) Service

### Submitting Organisation

### Description of the Maritime Service

The IMO’s 1979 SAR Convention was aimed at developing an international SAR plan to ensure that everywhere in the world, the rescue of persons in distress at sea would be co-ordinated by a responsible SAR organization or by co-operation between neighbouring SAR organizations.

Following the adoption of the 1979 SAR Convention, IMO's Maritime Safety Committee divided the world's oceans into thirteen search and rescue areas, with provisional SAR plans in place for each of these areas.  In each area, the countries concerned have search and rescue regions for which they are responsible.

Parties to the Convention must ensure that arrangements are made for the provision of adequate SAR services in their coastal waters. Parties are encouraged to enter into SAR agreements with neighbouring states involving the establishment of SAR regions, the pooling of facilities, establishment of common procedures, training and liaison visits. The Convention also states that Parties should take measures to expedite entry into its territorial waters of rescue units from other Parties.

The Convention then goes on to establish preparatory measures which should be taken, including the establishment of Rescue Co-ordination Centres (RCCs) and Sub-Centres. It establishes a common glossary to be used by all Parties and outlines operating procedures to be followed in the event of emergencies/alerts and during SAR operations. This includes the designation of an on-scene commander and their duties.

The IMO and International Civil Aviation Organization (ICAO) jointly publish the three-volume IAMSAR Manual. This manual provides international guidelines for a common aviation and maritime approach to organizing and providing SAR services. The three volumes are divided as follows:

- Volume 1, Organization and Management

- Volume 2, Mission Co-ordination

- Volume 3, Mobile Facilities

### Purpose

The RCC responsible for the Search and Rescue Region (SRR) is the primary contact in a SAR incident and ultimately responsible for incident management and decision-making.

In maintaining a state of full readiness, the service is responsible for a number of search and rescue functions, including:

detection and coordination of maritime incidents

control and conduct of maritime search and rescue operations.

These functions may include activities such as:

assisting  the crew and passengers of vessels in distress

assisting victims of maritime and aircraft accidents or incidents

coordinating the medical evaluation of seriously injured or ill persons from a vessel at sea, and their subsequent evacuation, if deemed necessary, to a suitable medical facility

monitoring towing operations;

monitoring and evaluating levels of risk from Maritime Safety Information (MSI) broadcasts to ensure an immediate response to potential life threatening situations;

monitoring vessels not under command

monitoring pollution reports and vessels aground

assisting other emergency response organizations when they require additional resources to prevent loss of life

liasing with other organizations assisting with search and rescue operations

information collection, distribution, and coordination.

### Operational Approach

Time, resources and effective communication will always be challenges when it comes to search and rescue cases involving human lives.  Effective use of time and allocation of rescue resources to the person(s) in distress are of primary importance.

Several actors are normally involved in a SAR operation. Manual exchange of information can be easily misunderstood in the form of wrong positions, SAR areas and other significant search information, and even a short delay can be the difference between life and death.  Lack of language skills has been identified as a major challenge in SAR communications.

Digital exchange and sharing of information could significantly reduce the human errors in SAR case prosecution and also result in huge time savings by allowing RCCs more timely access to accurate and relevant information.

Examples of information for SAR coordination communications that could be shared electronically to mitigate these challenges include:

- last known position (LKP)

- search areas

- search patterns

- search resources

- drift predictions

- last known position (LKP)

- datum

- radio frequencies

- NOTSHIPs

- known routing and planning information for search object (in the case of overdue aircraft or vessels)

- EPIRB hex codes and associated information

- SLDMB information

- satellite imaging

- live stream and images from search resources

- images of search objects

- images of search area

- SAR Briefing Report (with information for vessels engaged in search)

Credit photo: JRCC Halifax.

Figure xx. Simulation of drift prediction

Credit photo: JRCC Halifax.

Figure xx. Illustration of search areas and additional information that may be used during a SAR operation

Credit photo: JRCC Halifax.

Figure xx. Illustration of a SAR Briefing Report

The RCC could then deliver this important information about the SAR situation to a graphical display  shared with the on-scene commander, search resources and all partners involved, improving communications and increasing situational awareness for everyone. This would make it easier for all units to get current information on the search operation underway.

Information from several services assist the SAR Coordinators in making timely decisions and appropriate use of resources. Some examples would include:

- vessel information such as position, course and speed,

- medical and firefighting capabilities of nearby resources,

- other emergency organizations in the area and their rescue capabilities

- meteorological information (including tides and currents, ice information)

- Maritime Safety Information

- ship reporting information such as persons on board, capacity, cargo information,

### User Needs

Coordination, communication and effective exchange of information are important elements of the SAR service. The coordinating RCC must be able to share and receive information quickly and easily with a number of different resources:

- Other RCCs

- Vessels assisting in search and transiting search area

- VTS services

- Ground search and rescue

- Search target

- Other emergency organizations (for example, police, fire, ambulance)

- Port authorities

| Information related to: | Examples |
| --- | --- |
| General information | - Responsible RCC - Other emergency organizations assisting |
| SAR information | - Type of operation - What and when - Last Known Position - Drift plot - Number persons missing/in distress - Dangers such as dangerous substances - In raft, lifeboat, persons in water etc - Datum - SAR area - SAR resources in area - Vessels of opportunity within and near search area - Vessels and aircraft involved in search - Search areas assigned and completed - on-scene commander - Communication - Meteorological information |
| Assistance | - Position of vessels or stations assisting - Towing vessels available and positions - Capacity, Bollard pull (BP), - Other salvage and oil recovery capacities - Response time - Connection information - Intended route / towing plan - speed limits - maneuvering limits - Communication |

Figure x-x-x: Example of common sharing of digital search and rescue information

### Information to be Provided

See Annex 16, MS 16 Search and Rescue Service.

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Digital Maritime Services

| Description | Examples of data that could be used in MS 16 |
| --- | --- |
| MS 1 VTS IS | Information on VTS areas, places of refuge, vessel traffic in the area, vessel identification information (type of vessel, nationality, MMSI, IMO number, contact information), information from sensors (meteorological, hydrographical, visibility, etc.), radar, AIS, CCTV, communication with ship |
| MS2 VTS NAS | Information about vessels undertaking NAS, exchange of routes, navigation advice and assistance |
| MS3 VTS TOS | Traffic clearance information, time slots, ETAs, route advisories |
| MS4 Local Port Service | Port availability and anchorage areas, berthing information, cargo operations, security, supply, available assistance, port contact information |
| MS 5 Maritime Safety Information | Warnings, no-go areas, diving operations, exercises |
| MS 6 Pilotage Service | Pilot boarding stations, Pilot vessel positions and orders, contact information, request procedures |
| MS 7 Tug service | Tug operations, available tug capacity |
| MS 8 Vessel Shore reporting | ETAs, Notification of arrival, vessel information, cargo information, number of persons on board |
| MS 9 Telemedical | Local address of medical centres and communication and capacity, medical advice |
| MS 10 Maritime Assistance Service | Contact information, places of refuge, routing, information about vessels requesting relevant MAS, incidents , notifications |
| MS 11 Nautical Chart Service | Local area updates, chart updates, notice to mariners |
| MS 12. Nautical Publications Service | Local descriptions and publications, digital information from nautical publications relevant for the operation |
| MS 13 Ice Navigation Service | Ice forecast, ice chart, ice conditions, information regarding icebreaker service / assistance, ice routes |
| MS 14 Meteorological Service | Weather in SAR area |
| MS 15 Real Time Hydro and Information Service | Tidal and current information in SAR area, all information from real time sensors |

## MS 17 Aids to Navigation services (AtoN)

### Submitting Organisation

### Description of the Maritime Service

### Purpose

### Operational Approach

### User Needs

### Information to be Provided

Annex 17, MS 17 Aids to Navigation Services (AtoN)

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

## MS19 PNT and augmentation services [UK GLA]

### Submitting Organisation

### Description of Maritime Service

### Purpose

### Operational Approach

### User Needs

### Information to be Provided

Annex 18, MS 18 PNT and Augmentation Services

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services