IALA Guideline

Navigating Officer Awareness of VTS

Edition 2.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| month/year approved by Council |  |  |

1.	INSTROCTION	8

1.1.	BACKGROUND	8

1.2.	APPLICATION	8

2.	AIMS AND OBJECTIVES	8

2.1.	AIMS	8

2.2.	OBJECTIVES	9

3.	VTS REVIEW（WHAT TO TEACH?）	9

3.1.	TERMS USED IN VTS	9

3.2.	AIMs, GOALs, ROLEs AND FUNCTIONs OF VTS	9

3.2.1.	AIMS	9

3.2.2.	GOALS	9

3.2.3.	ROLES	9

3.2.4.	FUNCTIONS	10

3.3.	WHAT CAN VTS DO FOR N-O	10

3.4.	COVERAGE	10

3.5.	TYPES OF VTS	10

3.5.1.	COASTAL VTS	10

3.5.2.	PORT /HARBOR VTS	11

3.5.3.	VTS IN INLAND WATERS	11

3.5.4.	INTEGRATED VTS	12

3.6.	DIFFERENCES AND CONNECTIONS BETWEEN VTS AND TRAFFIC SEPERATION &VESSEL REPORTING SYSTEM	12

3.6.1.	DIFFERENCES	12

3.6.2.	CONNECTIONS	12

3.7.	TYPES OF SERVICE PROVIDED BY VTS	12

3.7.1.	INFORMATION SERVICE	12

3.7.2.	TRAFFIC ORGANIZATION SERVICE	14

3.7.3.	NAVIGATIONAL ASSISTANCE SERVICE	15

3.7.4.	ALLIED SERVICES AND OTHER SERVICES	18

3.8.	APPLICATION OF VTS	19

3.8.1.	MANDATORY VTS	19

3.8.2.	VOLUNTARY VTS	19

3.9.	VTS PERSONNEL	19

3.9.1.	VTS OPERATOR	19

3.9.2.	VTS SUPERVISOR	20

3.9.3.	VTS MANAGER	20

4.	THE BASIC WORK OF VTS ----MONITORING THE MOVEMENTS OF VESSELS	20

4.1.	THE SPECIFIC WORK OF MONITORING THE MOVEMENTS OF VESSELS	20

4.1.1.	INFORMATION COLLECTION	20

4.1.2.	INFORMATION ASSESSMENT	20

4.1.3.	KEEP STANDING BY WORKING CHANNEL	21

4.1.4.	RESPOND	21

4.1.5.	VESSEL MARKING AND DYNAMIC RECORDS	21

4.2.	IMPLEMENTATION PROCESS	21

4.2.1.	ESTIMATED TIME OF ARRIVAL/DEPARTURE OF VESSEL	21

4.2.2.	VESSEL ENTERING VTS AREA	21

4.2.3.	VESSEL IS UNDERWAY	21

4.2.4.	VESSEL AT BERTH	21

4.2.5.	VESSEL AT ANCHOR	22

4.2.6.	VESSEL DEPARTURE FORM VTS AREA	22

5.	COMMUNICATION BETWEEN THE NAVIGATIONAL OFFICER AND THE VTS CENTRE	22

5.1.	TOOLS	22

5.2.	LANGUAGES	22

5.3.	REQUIREMENTS OF COMMUNICATIONS	22

5.4.	SMCP AND MESSAGE MARKS	23

5.5.	COMMUNICATION PROCEDURES	24

5.6.	COMMUNICATION SKILLS AND METHODS	24

5.6.1.	EFFICTIVE LISTENING SKILLS	24

5.6.2.	Read back	25

6.	UTILIZATION OF RELATED EQUIPMENT	25

6.1.	VHF WIRELESS TELEPHONE	25

6.2.	PROCESSES OF VHF COMMUNICATION	25

6.3.	USE OF AIS	27

6.4.	USE OF ECDIS	27

7.	ACQUISITION OF VTS INFORMATION	27

7.1.	INTRODUCTION OF THE VTS USERS GUIDE	28

7.2.	APPROACHES OF ACQUIRING THE VTS USERS GUIDE	28

8.	HOW TO RAISE THE VTS AWARENESS OF NAVIGATIONAL OFFICER	29

8.1.	ADDING VTS RELATED CONTENT SUBJECTS TO THE BRIDGE RESOURCE MANAGEMENT (BRM) TRAINING	29

8.1.1.	VTS KNOWLEDGE IN THEORY TRAINING	29

8.1.2.	ADDING VTS CONTENTS IN PRACTICAL TRAINING	29

8.2.	ADDING THE VTS CONTENTS IN NAVIGATIONAL EQUIPMENT TRAINING	30

8.2.1.	PRACTICAL TRAINING OF VHF	30

8.2.2.	AIS PRACTICAL TRAINING	30

8.2.3.	ECDIS PRACTICAL TRAINING	31

8.3.	CONSIDERATION OF VTS FACTORS IN PRACTICAL TRAINING of  VOYAGE PLANNING AND PREPARATION	31

8.4.	ADDING VTS CONTENTS TO NAVIGATION SIMULATOR TRAINING	32

8.5.	COMMUNICATION ACTIVITY BETWEEN NAVIGATIONAL OFFICERS AND VTS CENTRE	33

8.6.	STRENGTHENing PILOT'S FAMILIARITY WITH VTS	33

9.	ACRONYMS & DEFINITIONS	33

9.1.	ACRONYMS	33

9.2.	DEFINITIONS	35

10.	REFERENCES	35

ANNEX A	OPERATING GUIDE of VESSELs IN VTS AREA	36

ANNEX B	EXAMPLE OF COMMUNICATION BETWEEN N-O AND VTS	41

ANNEX C	BASIC WORDS	42

ANNEX D	SPELLING OF LETTERS AND NUMBERS	43

List of Tables

Table 1	Examples of the types of information that may be provided by the VTS operating an INS	11

Table 2	Examples of types of information that may be provided by the VTS within a TOS	13

Table 3	Examples of the types of information that may be provided by a VTS operating NAS	15

Table 4	Message marks used by VTS operator	21

Table 5	Message marks used by Navigational officers	22

Table 6	List of headings – guide to type of information	26

Table 7	Examples of practical training projects in BRM	27

Table 8	Examples of a communication training project using vhf and the VTS Centre	28

Table 9	Examples of VTS factors that can be considered in AIS practical training	29

Table 10	Examples of VTS factors that can be considered in ECDIS practical training	29

Table 11	Examples of VTS factors that can be considered in voyage planning and preparation practical training	30

Table 12	Examples of VTS training in Marine Simulator training	31

Table 13	Examples of reporting to VTS centre	35

Table 14	Example of Navigational officer operating in navigational assistance service	36

Table 15	Example of Navigational officer operating in traffic organization service	37

Table 16	Examples of Navigational officer operating in SAR	37

Table 17	Example of Navigational officer operating in other emergency situations	38

Table 18	Apply for weather information	40

Table 19	Apply for traffic information	40

Table 20	Table 20 Basic words and it’s meaning	41

Table 21	Spelling of letters	42

Table 22	Number Spelling Pronunciation	42

Table 23	Table 23 Example of Number Pronunciation	43

List of Figures

Figure 1	The implemention process of INS	12

Figure 2	The implementation  process of TOS	14

Figure 3	The implementation process of NAS	16

Figure 4	Vessel calling process	24

Figure 5	Vessel responding process	25

# INSTROCTION

## BACKGROUND

There is a consensus with in the industry that VTS is now a mature and reliable partner to improve the safety and efficiency of navigation, the safety of life at sea and to provide a central risk reduction measure regarding oil spill and environmental damage.

IMO res. A.857 (20), SOLAS Chapter 5 Regulation 12 Vessel Traffic Services, MSC/Circ. 1065, STCW 95 resolution 10 mention VTS and stress the importance of it.

The efficiency of VTS  depends upon the effectiveness of the communications involved. The effectiveness of the information exchanges between the VTS, the master or the bridge team depends upon each party's understanding of their respective and mutual functions and responsibilities. It is useful to establish effective coordination between them for enforcing the safe and efficient navigation. The VTS centre can be considered as an extension of the bridge resource management team. When the master and the bridge team have more knowledge about a VTS it would contribute to better teamwork between VTS and vessels to achieve safe navigation.

However, there is another consensus with in the industry that Navigation Officers’ knowledge of VTS is insufficient. Indeed, the lack of knowledge of masters and the bridge personnel concerning VTS capabilities limits the vessels to use services provided by VTS, resulting in confusion, hazardous situations and significantly weakening situational awareness. Meanwhile, there is little requirement for VTS training within STCW and it is only mentioned briefly in Table A-II/1 “Maintain a safe navigational watch” and the importance of VTS is not reflected in STCW.

It is necessary to develop a guideline to help navigational officer better grasp VTS functions, responsibilities and procedures, and to instruct the training institutions to conduct VTS awareness training; it also helps mariners to be on duty as required by the STCW Convention, thereby promoting the safety of navigation.

## APPLICATION

This Guideline applies to Maritime Training Centres, shipping companies, harbours and Pilot stations, flag State certification administrations and other relevant stake holders who have a role or assist in improving awareness of the navigating officers and their understandings of VTS.

# AIMS AND OBJECTIVES

## AIMS

This guideline presents guidance to what information should be made available to Navigating Officers as part of their basic Navigational training, whether this training is provided through Maritime Training Centres or any other stake holder.

## OBJECTIVES

The objective is to raise the awareness of the Navigation Officers about VTS through VTS training and to facilitate effective communications and exchange of information between the bridge team and VTS Operators through a greater mutual understanding between the two parties of their respective functions and responsibilities.

# VTS REVIEW（WHAT TO TEACH?）

## TERMS USED IN VTS

.1.Vessel traffic service (VTS) - a service implemented by a Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area.

.2.Competent authority - the authority made responsible, in whole or in part, by the Government for safety, including environmental safety, and efficiency of vessel traffic and the protection of the environment.

.3.VTS authority - the authority with responsibility for the management, operation and co-ordination of the VTS, interaction with participating vessels and the safe and effective provision of the service.

.4. VTS area - the delineated, formally declared service area of the VTS. A VTS area may be subdivided in sub-areas or sectors.

.5. VTS reporting line - an electronic virtual line established by the competent authority to report to the VTS centre as required by the specified procedures and requirements when passing through the line.

.6. VTS centre - the centre from which the VTS is operated. Each sub-area of the VTS may have its own sub-centre.

.7. VTS voyage plan - a plan which is mutually agreed between a VTS Authority and the master of a vessel concerning the movement of the vessel in a VTS area.

## AIMs, GOALs, ROLEs AND FUNCTIONs OF VTS

### AIMS

The aim of vessel traffic management system is to improve the safety and efficiency of navigation and the safety of life at sea, strengthen the protection of Marine environment and improve the possible adverse effects of Marine traffic on coastal, construction site and offshore installations.

### GOALS

The goals of implementing VTS are as follows:

- Aid the Navigational officer in the safe and efficient use of navigable waterways;

- Afford unhindered access to pursue commercial and leisure activities, subject to any restrictions that may exist;

- Contribute to keeping the seas and adjacent environment free from pollution.

### roles

As an auxiliary means for navigation, VTS is essentially a navigation aid. Because of the initiative characteristics of its services, VTS is also a kind of navigational aid that provides services actively.

### FUNCTIONS

VTS functions can be subdivided into internal and external functions. Internal functions are the preparatory activities that have to be performed to enable a VTS to operate. These include data collection, data evaluation and decision-making. External functions are activities executed with the purpose of influencing the traffic characteristics. They relate to the primary traffic management functions of rule-making, allocation of space, routine control of vessels and manoeuvres to avoid collisions, as well as to other management functions such as enforcement, remedial and ancillary activities.

Amongst the most important functions that a VTS may carry out are those related to, contributing to and thereby enhancing:

- Safety of life at sea;

- Safety of navigation;

- Search and Rescue;

- Efficiency of vessel traffic movement;

- Protection of the marine environment;

- Supporting maritime security;

- Supporting law enforcement; and

- Protection of adjacent communities and infrastructure.

## WHAT CAN VTS DO FOR N-O

As an active navigation aid, VTS has the following benefits for N-O:

- VTS has real-time knowledge of coastal, port and inland waterway infrastructure and its key and vulnerable locations, providing information services to Navigational officers and improving maritime traffic efficiency.

- VTS has real-time knowledge of vessel activities, including navigation plan, vessel position and intention, etc., to help Navigational officers improve the awareness of information between vessels.

- To provide navigational assistance service to Navigational officers in difficult navigation conditions to ensure the safety of vessels.

- With real-time knowledge of conventional port operations, including vessel route and port interface activities, VTS could reduce the pressure of Navigational officers in route planning and help Navigational officers fully understand the navigation environment in complex traffic situations.

- VTS can prevent traffic accidents or pollution accidents or prevent them from developing into disasters. Communication coordination and medical assistance can be provided to Navigational officers in emergency situations.

## COVERAGE

VTS will benefit N-O, shipowners, pilotage, schools, Protect & Indemnity Clubs, and other relevant institutions.

## TYPES OF VTS

### COASTAL VTS

The coastal VTS mainly involves vessel traffic through the VTS area. A coastal VTS can assist the safe and expeditious passage of shipping through coastal waters. It may also be involved in providing assistance to vessels before embarkation or disembarkation of a pilot. The types of service provided by a coastal VTS will depend on its legal basis but is likely to include more than just an information service.

### PORT /HARBOR VTS

Port/Harbor VTS are mainly related to vessel traffic in ports, wharves or harbors. In order to ensure safety and efficiency, the movement of vessels within port areas may demand specific traffic management which not only minimizes incidents but also promotes economic results. A Port/Harbor VTS help prevent unnecessary delays and reduce the number of close quarter situations between vessels that may jeopardize the ports integrity.

Port VTS around the world ensure the economic profitability of ports, the safety of human life and the protection of the marine environment. The techniques used in port VTS may be different from a coastal VTS, because the navigation is in itself different. In port areas both the separation between vessels and the required under-keel clearances may be reduced. These two factors directly affect the navigation of vessels and therefore port VTS procedures.

A Port/Harbor VTS may promulgate port entry requirements and may issue instructions.  Examples of such requirements/instructions are:

- Clearances for entering/exiting VTS areas;

- The obligation of taking pilot;

- The obligation of taking tugs;

- Maximum speed in the area;

- Separation distance between vessels;

- Assignments of anchor positions;

- Overtaking permissions in narrow channel.

These functions are normally rendered under a traffic organization service (TOS). In a port area, VTS should also provide a navigation assistance service (NAS), where vessels are assisted in complex navigational situations.

### VTS IN INLAND WATERS

The purpose of VTS in inland waters is to improve the safety and efficiency of navigation, safety of life and the protection of the environment and/or the adjoining waterway banks, nearby residents and enterprises from possible adverse effects of vessel traffic.

A part of the objectives of a VTS in inland waters may include the support of efficient transport and the collection of data and information that, as a consequence, may be required.

The benefits of implementing a VTS in inland waters are that it allows identification and monitoring of vessels, the strategic, tactical and operational planning of vessel movements, managing traffic by allocation of space, as well as the provision of navigational information and assistance.

A VTS in inland waters may provide an information service (INS), a navigational assistance service (NAS) and a traffic organization service (TOS). Further information may be found in IALA Recommendation V-120, - ‘Vessel Traffic Services in Inland Waters’.

### INTEGRATED VTS

Port/Harbor VTS are mainly related to vessel traffic in ports, wharves or harbors. While coastal VTS mainly involve vessel traffic through the VTS area. VTS can also be a combination of these two types, called integrated VTS.

## DIFFERENCES AND CONNECTIONS BETWEEN VTS AND TRAFFIC SEPERATION &VESSEL REPORTING SYSTEM

### DIFFERENCES

In the VTS area, VTS operators provide Vessel Traffic Services to vessels through real-time monitoring of vessels. So VTS is proactive.

Vessel routeing system and vessel reporting system are passive in that vessel traffic management is conducted through predetermined rules.

### CONNECTIONS

- VTS, Traffic Separation Scheme and Vessel Reporting System can be used independently or in combination.

- Vessel reporting system is the main data source for VTS traffic images.

- Vessel traffic service is mandatory only within the territorial sea. When a vessel is required to participate in VTS in international waters or the channel, it can be achieved by setting up the Vessel Reporting System in the relevant area or by Traffic Separation Scheme adopted by the IMO.

## TYPES OF SERVICE PROVIDED BY VTS

### INFORMATION SERVICE

#### Definition

An Information Service provides relevant information at appropriate time and on request for the promulgated VTS area.

#### General Requirements

- Information Service objects are vessels within the VTS area;

- The information is conducive to the safe navigation of the vessel;

- VHF working channel is the preferred communication method;

- The VTS centre provides the broadcast  at fixed time and interval.

#### Examples of Information Services

- Updated information about vessel traffic and routes contributing to safety;

- To remind navigation warnings relating to the safety of vessels;

- Meteorological and hydrological information affecting navigation safety of vessels;

- Failure of navigation equipment affecting the navigation safety of vessels;

- Other information relating to the safe navigation of the vessel.

#### 3.Occasion to Provide Information service

- Information broadcasted at fixed time or intervals;

- Deemed necessary by the VTS;

- At the request by vessels.

#### Information Service Content

- Examples of the types of information that may be provided by the VTS operating an INS

| Information related to | Examples |
| --- | --- |
| Navigational situations (including traffic and route information) | Position, identity, destination of vessels and the intention of other traffic; Amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points; the mandatory reporting of movements; Limited manoeuvrability that may impose restrictions on the navigation of other vessels, or any other potential hindrances; Suspension or change of routes; etc. |
| Navigational warnings | Dangerous wrecks, obstacles not otherwise promulgated, diving operations, vessels not under command, etc. |
| Meteorology | Information that will include the speed and direction of the prevailing wind, direction and height of the waves, visibility, atmospheric pressure, the formation of ice, etc. |
| Meteorological warnings | Gale, storm, tsunami, restricted visibility, etc. |
| Hydrography | Information that will include factors such as the stability of the seabed, sea depth, the accuracy of surveys, tidal ranges, tidal streams, prevailing currents and swell, etc. |
| Electronic navigational aids | The availability of electronic navigational aids such as: GNSS, Loran, LRIT, DGPS, AIS, RACON etc. |
| Other information | Port information, pilot or tug request, cargo information, health condition, PSC, ISPS, etc. |

#### Implementation Process

The VTS operators provide information services on their own initiative

Vessel requests information service

VTS broadcasts information related to navigation safety of vessels

Vessel response

To determine whether VTS has the information requested by the vessel

YES, information is provided in time

Vessel response

NO, state clearly that the information is unable to be provided or inform the vessel of possible access to get information

- The implemention process of INS

### TRAFFIC ORGANIZATION SERVICE

#### Definition

A Traffic Organization Service (TOS) is a service to prevent the dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the declared VTS area.

#### General Requirements

A Traffic Organization Service may be provided in part, or all, of the declared VTS area.

While VHF may be the primary means of providing a Traffic Organization Service, any available communication method may be used.

#### Examples of Traffic Organization Service provided

A Traffic Organization Service should be provided when the VTS is authorized to provide services, such as when:

- vessel movements need to be planned or prioritised to prevent congestion or dangerous situations;

- special transports or vessels with hazardous or polluting cargos that may affect the flow of traffic and need to be organised;

- an operating system of traffic clearances or sailing plans, or both, have been established;

- the allocation of space needs to be organised;

- mandatory reporting of movements in the VTS area has been established;

- special routes should be followed;

- speed limits should be observed;

- the VTS observes a developing situation and deems it necessary to interact and coordinate vessel traffic;

- nautical activities (e.g. sailing regattas) or marine works in‐progress (such as dredging or submarine cable‐ laying) may interfere with the flow of vessel movement.

#### Traffic Organization Service Content

- Examples of types of information that may be provided by the VTS within a TOS

| Information related to | Examples |
| --- | --- |
| Traffic clearance | Empower authorization under conditional circumstances to a vessel when: prior to or entering a VTS area; - departing from a berth or an anchorage position within a VTS area; - entering into a fairway within a VTS area; or - prior to commencing a manoeuver that may be detrimental to safe navigation. Examples of conditions: - a VTS sailing plan before entering a VTS area; - lock and bridge passage planning; - report position at determined reporting point/line/pilot station; - use a second fairway in case of bad visibility/weather; - use a tug boat in case of strong wind; - dredging or compass swing in confined waterway. |
| Anchorage | Examples of anchorage situations: - organizing the movements to/from an anchorage position/area; - assignment of an anchorage position; - assisting vessels into anchorage position. |
| Enforcement | Examples of enforcement: - speed limits; - adherence to rules regarding traffic routeing measures; - pilotage requirements; - other traffic regulations and possibly local by‐laws |
| Waterway (sea, channels and fairway) management | Examples of management measures: - the use of one‐way traffic as an alternative of two way traffic, depending on the dimensions of vessel or the weather conditions; - organizing other traffic when a vessel has passed point of no return; - slot management to allocate vessels in a time window; - organizing the traffic concerning vessel dimensions in comparison to fairway restrictions; - instruct vessels when overtaking is not permitted; - establish and organise vessel safety zones in case of particular operations; - establish and organise exclusion zones; - instruct vessels to keep clear from special areas/positions; - organizing the traffic as regards to meteorological, hydrographical or other restrictions such as visibility, wind speed, current, sea state and under keel clearance. |

#### Implementation Process

VTS collects information and focuses on traffic movements

VTS analysis the necessity of implementing traffic organization service

VTS determines the specific measures of traffic organization

VTS establishes contact with vessels

VTS provides information to relevant vessels

The vessel receives information and responds

- The implementation  process of TOS

### NAVIGATIONAL ASSISTANCE SERVICE

#### Definition

A Navigational Assistance Service is a service that provides essential and timely navigational information to assist in the on board navigational decision-making process and to monitor its effects. It may also involve the provision of navigational advice and/or instruction.

#### General Requirements

- Navigational Assistance Service requires positive identification and continuous communication throughout the process. If time permits, checks should usually be made prior to commencement of the provision of Navigational Assistance Service to assess the capability of the vessel to respond to the guidance given.

- Navigational Assistance Service should be provided within the declared VTS area and/or in specified areas or sectors within the VTS area.

- It is recommended as best practice that message markers are always used when delivering Navigational Assistance Service irrespective of the language ability of the recipient.

#### Examples of Navigational Assistance Service provided

- risk of grounding;

- vessel deviating from the VTS sailing/passage plan;

- vessel unsure of its position or unable to determine its position;

- vessel unsure of the route to its destination;

- assistance to a vessel to an anchoring position;

- vessel defects or deficiencies, such as navigation or manoeuvring equipment failure;

- severe meteorological conditions (e.g. low visibility, strong winds);

- risk of collision between vessels;

- risk of collision with a fixed or floating object;

- Assistance to a vessel to support the unexpected incapacity of a key member of the bridge team.

#### Occasions to Provide Navigational Assistance Services

- Deemed necessary by the VTS;

- The vessel has requested information.

#### Contents of Navigational Assistance Services

- Examples of the types of information that may be provided by a VTS operating NAS

| Information related to NAS | Examples |
| --- | --- |
| Request and identification | - availability of NAS, start and end of NAS; - request for vessel identification such as position, course made good and speed over the ground; - status of vessel's equipment; etc. |
| Navigational information (including position and course information) | Examples provided to an individual vessel: - provide range and bearing from fixed objects, fairway/channel or way‐points; proximity to navigational hazards, etc. - provide information related to navigating into a channel/fairway/lane (i.e. track is parallel/diverging/converging with/from/to reference line); etc. |
| Advice (or instruction) | - advise (or instruct) a vessel to alter the course, speed; - advise (or instruct) to keep clear from area/position, close up/drop back on/from vessels; etc. |
| Warning | Diverging from the recommended track towards dangerous wrecks, obstacles not otherwise promulgated; diving operations; vessels not under command; etc. |

#### Implementation Process

Elements of assessment

Whether belongs to the category of the NAS

Whether there are other options for alternative NAS such as pilots, tugs, patrol boats, etc.

VTS equipment, vessel navigation equipment and mechanical equipment status

The captain's knowledge of the local waters and NAS; the barriers of language communication between the captain and the VTS operator

Environmental factors (wind, day/night, visibility, high tide, strength of tidal currents), traffic conditions in water areas, and vessel loading capacity

The ability of a vessel to navigate safely with the aids of NAS

Emergency plan during Navigational Assistance Services

The VTS centre decides whether to provide NAS to vessels.

Select communication channel.

Choose a working language.

The master understands that the NAS does not relieve his responsibility for the safe navigation and agrees to continue the navigation under the conditions of NAS

The two sides confirm the start of NAS and maintain communication;

The VTS centre provides necessary information, advice, instructions or warnings to vessels

Continuous assessment of the vessel's ability to navigate safely and possible adverse effects

The VTS centre monitors the effects of actions taken by vessels

Requirements raised and confirmed by vessels

The two sides agree that the NAS meets expectations and jointly confirm the end of NAS

Decision making

Provide NAS

- The implementation process of NAS

### ALLIED SERVICES AND OTHER SERVICES

#### Definition

Allied services are services actively involved in the safe and efficient passage of the vessel through the VTS area. It is supported by ports, pilotage, salvage, customs, military, etc. to increase the safety and efficiency of transportation, protect the environment, and improve the effectiveness of VTS.

Other Services refer to services other than the allied services, which may use VTS data to more effectively undertake their work (e.g. ensuring local security or preventing illegal imports).

#### Inter-action with Allied Services and Other Services

- Search and Rescue

Procedures may be established to deal with incidents such as collision, capsize, sinking, grounding, fire on vessel, MOB, which may include the following actions:

- Send alert to rescue co-ordination centre;

- Inform relevant regulatory authorities;

- Support on-scene coordination;

- Consider back-up VTS personnel;

- Promulgate incident information to vessels in VTS area;

- Restrict traffic in the area;

- Inform pilot boats, tugboats and other support units;

- Providing shelter for vessels in need of assistance.

- Marine emergency events

Assist in pollution emergency:

- Alert or Inform relevant regulatory authorities;

- Control the pollution and take further actions for pollution response;

- Promulgate incident information to vessels in VTS area;

- Restrict traffic in the area.

Natural Disaster:

- Promulgate information to vessels in the VTS area;

- Act on local call-out procedures;

- Inform rescue co-ordination centre.

- Security

- VTS in-depth knowledge of coastal, port and inland waterway infrastructure and its critical and vulnerable locations;

- VTS real-time knowledge of vessel activity, including scheduled movements, vessel locations and intentions;

- VTS-capability to interact with law enforcement authorities;

- VTS in-depth knowledge of regular port operations, including vessel routes and vessel-port interface activities.

- The VTS maintains regular and continuous communications with vessels in case a vessel is denied access to a port or restricted area for security reasons.

- The VTS may determine a safe anchorage, holding area or designated / alternative route.

- Other Aspects

To conduct medical assistance, e.g. inform MRCC rescue co-ordination centre and coastal radio station.

When vessel is not under command, actions include delivering information to vessels in VTS area; maintaining communication; obtaining information on vessels not under command; assessing the approaching danger of surrounding vessels.

For more details, see annex C to IALA Guideline 1102 (VTS Interaction with Allied or Other Services)

## APPLICATION OF VTS

### MANDATORY VTS

Within the territorial sea, VTS implemented and operated by coastal States may be mandatory. Whether the VTS is mandatory depends on the governing rules and regulations of the coastal State to which it belongs. In the mandatory VTS area, VTS are applicable to all vessels, including general merchant vessels, passenger vessels, yachts, ferries, fishing vessels, official vessels, military vessels, etc.

### VOLUNTARY VTS

VTS is not mandatory in international waters or straits, but it can be realized by setting up a ship reporting system in the relevant area or adopting the vessel routeing system approved by IMO. For non-mandatory VTS, the vessel may participate voluntarily. The voluntary VTS only applys to participating vessels.

## VTS PERSONNEL

VTS Personnel include VTS Operator, VTS Supervisor and VTS Manager.

### VTS OPERATOR

VTS operator is a qualified person performing one or more VTS services.

The key person in any VTS operation is the VTS Operator. To specially carry out the main duties in VTS, main responsibilities are as follows:

- Maintain situational awareness and monitor the vessel traffic image with all available sensors within the area of responsibility;

- Maintain communication with vessels as appropriate to the service type provided by the VTS using all available communication facilities;

- Operate equipment for communications, data collection, data analysis and establishment of a vessel traffic image;

- In an Information Service (INS), provide relevant information at appropriate times;

- In a Traffic Organization Service (TOS), organize and plan the vessel traffic movements within a waterway to prevent congestion, groundings, collisions and other dangerous situations;

- In a Navigational Assistance Service (NAS), assist and provide such information as may be required to aid a vessel in difficult navigational or meteorological circumstances or in case of defects or deficiencies. NAS may be given on request by a vessel or when deemed necessary by the VTS;

- Communicate with allied services and other agencies as appropriate;

- Ensure that all adopted standard operating procedures and relevant waterway regulations are adhered to;

- Take appropriate actions in emergency situations and other special circumstances defined for the VTS area. Where appropriate, co-ordinate communications for such situations and/or circumstances; and maintain a log of all incidents/accidents and all other relevant events occurring within the area of responsibility.

### VTS SUPERVISOR

VTS supervisor is the leader of VTS duty team, responsible for coordinating, managing and coordinating the operation activities of VTS operators.

### VTS MANAGER

VTS manager is the director of VTS centre, which is responsible for the management and coordination of the VTS centre for VTS on behalf of the VTS Authority.

# THE BASIC WORK OF VTS ----MONITORING THE MOVEMENTS OF VESSELS

Monitoring the movements of vessels means that VTS operator monitors vessel movements in waterway, navigational route, precaution area, recommended routes, TSS, anchorage and other important waters within VTS area and takes responsibility during the duty period. The work of monitoring of traffic includes but not limited to the following aspects:

## THE SPECIFIC WORK OF MONITORING THE MOVEMENTS OF VESSELS

### INFORMATION COLLECTION

- Vessel particulars

- Movement of the vessel

- Sailing plan and implementation

- Condition of navigational route and anchorage

- Meteorological and hydrological conditions

- Malfunction of aids to navigation

- System alarm

- Information of vessels restricted from movements by detention, investigation, seizure, malfunction, construction, etc.

- Influence of surface and underwater operation on regional traffic

- Other information relating to the safety navigation

### INFORMATION ASSESSMENT

- Vessel traffic situation and interaction

- Risk of collision

- Risk of grounding

- The traffic condition of fairways, sluice gate and bridge passage

- Anchorage conditions

- The influence of hydrological and meteorological condition, surface and underwater activities and emergency

- Vessel comply with laws, regulations and special provisions

- Malfunction of vessel

### KEEP STANDING BY WORKING CHANNEL

The VTS Operators should keep standing by working channel.

### RESPOND

- The VTS operators should respond to a vessel's call in a timely manner.

### VESSEL MARKING AND DYNAMIC RECORDS

The VTS operators should operate in accordance with the requirements of the VTS Centre for the dynamic identification and recording of vessels.

## IMPLEMENTATION PROCESS

### ESTIMATED TIME OF ARRIVAL/DEPARTURE OF VESSEL

The VTS Centre should obtain basic information on the entry or departure of the VTS area as needed, including sailing plan, estimated time of arrival / departure time and draft.

### VESSEL ENTERING VTS AREA

- Receiving and responding to vessel reports;

- Marking of vessels;

- Information assessment, provide INF when deemed necessary or at the request of the vessel;

- If the vessel is unable to enter the harbour because of navigational restrictions, the reasons should be informed to the vessels.

### VESSEL IS UNDERWAY

- he following information should be collected to determine the vessel's navigational hazards:

- Navigation conditions;

- Traffic situation;

- Vessel conditions;

- Traffic control;

- Special provisions for vessels carrying harmful, dangerous or polluting cargo;

- Pilotage requirements.

- Provision of services when necessary or at the request of the vessel.

### VESSEL AT BERTH

VTS operator may provide information that includes but not limited to the following;

- Reporting requirements for completion of berthing;

- Security requirements and the level;

- Special requirements for standing by on certain VHF channel;

- Restrictions on other passing-by vessels;

- Reporting requirements before vessel departure.

### VESSEL AT ANCHOR

- Receive anchor reporting and respond in time;

- According to the function of anchorage, capacity, density of vessel, depth of water, etc., to determine whether to permit the anchor application;

- Tell the reason clearly when vessel is not permitted to anchor;

- Provide services to anchoring vessels when necessary or at the reasonable request of the vessels.

### VESSEL DEPARTURE FORM VTS AREA

- Assessment of traffic situation in VTS area;

- Provision of services when necessary or at the request of the vessel;

- If the sailing conditions are not satisfied, the reasons should be clearly informed.

# COMMUNICATION BETWEEN THE NAVIGATIONAL OFFICER AND THE VTS CENTRE

## TOOLS

VHF wireless phones are the main Communication tools, and it is generally performed at the frequency specified by the VTS centre.

## LANGUAGES

Communication usually uses the common language between navigational officers and the VTS centre or English.

## REQUIREMENTS OF COMMUNICATIONS

The following requirements should be paid attention to when Navigational Officers are communicating with the VTS centre:

- The vessel should be kept on duty on VTS working channel in VTS area.

- Broadcasting information irrelevant to navigation and safety on the VTS working channel are prohibited.

- When VTS working channel is not available, other channels including VHF16 channel can be used temporarily, but it must be confirmed that the channel is not used for distress and emergency calls.

- Any information sent by the VTS centre should be result-oriented, and the specific implementation should be made by Navigational officers.

- Any information issued by the VTS centre does not affect the statutory responsibilities and duties of Navigational officers.

- Communication language should be concise and unambiguous.

- Using SMCP correctly.

- Using message marks.

## SMCP AND MESSAGE MARKS

To ensure the communication efficiency, Navigational officers should have a good grasp of SMCP and use them correctly. 8 message marks are defined in SMCP. In the process of communication, the appropriate message mark should be used so as to increase the probability of correctly understanding the message.

- Message marks used by VTS operator

| message mark | Significance | Efforts to guide | The response of the Navigational officer |
| --- | --- | --- | --- |
| INSTRUCTION | An indication made according to a rule or a guide file | Strong | Without affecting the safety of the vessel or the safety of navigation, the vessel shall carry out and comply with the content of the instructions. If there is an impact, it should report and explain the situation to the VTS centre. |
| ADVICE | Indicating the action or measure that can be taken by the called subject | medium | The vessel should consider the proposal carefully and decide whether to carry out it. If it is not implemented, it should report to the VTS centre and explain it. |
| WARNING | Indicating the existence or imminent danger or serious consequence, which requires the attention of Navigational officers. | medium | The vessel should pay great attention to it and respond immediately and take measures to deal with the dangers. |
| INFORMATION | Indicating the provision of information related to navigation | general | The vessel should pay attention to the information and make sure that the information has an impact on navigational safety. |
| QUESTION | Indicating the need to get the information you required | / | The vessel should clarify the other's questions and the contents of the required information. |
| ANSWER | Indicating that the responder provides the corresponding information according to the request of the calling body. | / | The vessel shall provide the corresponding information and data according to the questions. |
| REQUEST | Expressing a request, demand, or need for assistance | / | The vessel should clarify the type and content of the information required . |

- Message marks used by Navigational officers

| Message Mark | Significance | The time for Navigational officers to use |
| --- | --- | --- |
| INFORMATION | Indicating the provision of information related to navigation | Providing information to the VTS Centre |
| QUESTION | Representing the required of information | Sending question to the VTS Centre |
| ANSWER | Indicating that the responder provides the corresponding information according to the request. | Answering the inquiries from the VTS Centre |
| REQUEST | Representing the required information | Sending information to a VTS Centre for requests, needs or assistance. |
| INTENTINON | Action to be taken by the vessel | Delivering the ship’s upcoming action to other ships |

## COMMUNICATION PROCEDURES

Communications between Navigational officers and VTS centre should be carried out in the following procedures:

- Communicating contents should be considered clearly before VHF communication, to avoid unnecessary interruption and delay.

- Before communication, it should make sure that the channel is not occupied.

- When there is difficulty in establishing communication with VTS centre, sufficient time should be given before repeating the call to avoid unnecessary occupation of channel resources.

- Responding the VTS centre in time.

- If the VHF signal is poor, it can be changed to another channel after confirming with VTS centre.

- The communication process should clearly indicate the identity of the vessel and the VTS centre.

- The information should be repeated if it is incomplete, inaccurate or not comprehend during the communication process.

## COMMUNICATION SKILLS AND METHODS

### EFFICTIVE LISTENING SKILLS

Effective listening skills are used to actively understand information provided by the speaker and it can be categorized into the following steps:

- Hearing

Hearing involves the reception of sounds from the sender by:

-  Avoiding interruptions.

-  Clear one's mind of distractions.

-  Focus on the speaker.

- Clarity

The sender and receiver both have a responsibility to ensure that what is said is understood:

-  Ask open questions to probe for further detail if required.

-  Avoid asking leading questions.

-  Avoid coming to conclusions before the sender finishes.

-  Be aware of the sender’s choice and application of words.

-  Encourage feedback through questioning.

- Interpretation

Interpretation not only requires verification of what the sender has said, but also the understanding of the information given.

Steps to ensure understanding are:

- Communicate your interpretation and verify its accuracy.

- Identify the main issues.

- Do not assume what the sender will say, particularly when receiving routine communications.

### Read back

Read back could be considered a powerful feedback tool.

There are two main principles for reading back communications. The first principle is to benefit other Navigational officers and the second is to ensure that the message is received correctly.

When the message markers ‘INSTRUCTION, ADVICE or WARNING’ are used by the VTSO，the VTSO may use the message: ‘Read back’. Navigational officer should then read back the message as it was sent providing the VTS with confirmation that the previous sent message has been received correctly.

# UTILIZATION OF RELATED EQUIPMENT

## VHF WIRELESS TELEPHONE

Navigational officers should read the manual of VHF equipment carefully, be familiar with operating methods, communication procedures and matters for attention, and test and maintain the equipment based on the requirements in the manual. During the operation of distress alarm, every step should be operated strictly according to the specifications, so as to ensure reliable transmission of distress alarm. Vessels which under way must keep watch on the designated channels according to chapter SOLAS74 Chapter IV.

All communications for distress calls or messages have absolute priority. When a distress call or message is received, all other transmissions should be stopped and it has to keep  standing by this channel. When the distress information is received nearby, it should be immediately confirmed; if not nearby there should be a short time interval before confirmation of receipt so as to allow the vessel closer to the distress to confirm receipt.  Any distress call or message should be recorded on the logbook.

## PROCESSES OF VHF COMMUNICATION

Vessel calling process:

- Vessel calling process

Vessel responding process:

- Vessel responding process

## USE OF AIS

Navigational officers should ensure that AIS equipment is in the normal working state, the AIS related information is correctly inputted and regularly checked. If the captain thinks that the continuous work of AIS may endanger the ship, the AIS equipment can be shut down until the danger disappeared. During voyage in port, the operation of AIS should comply with the regulations of the port.

Navigational officers should not rely on AIS as the sole information system, but should use all available safety related information. The use of AIS on board does not have any special effect on the arrangement of navigation duty. This work should be carried out continuously based on the STCW convention.

## USE OF ECDIS

ECDIS is helpful to the vessel understanding the surrounding navigational information intuitively in complicated navigational conditions such as coastal waters and narrow channels, thereby facilitating communication and coordination with other vessels. Meanwhile, there are several means (i.e. calculating the CPA and TCPA, setting up the warning ring and reporting lines) to ensure that the vessel navigate under safe conditions. Setting the appropriate anchor warning ring can effectively monitor dragging. During search and rescue operations, the location of the distress vessel can be quickly identified so as to dispatch rescue forces to the scene at the first time, determine the scope of search and rescue, and improve the efficiency of rescue.

# ACQUISITION OF VTS INFORMATION

VTS information is promulgated for navigational officers by the VTS authorities that including the requirements to be met and full details of procedures to be complied in VTS area. This information should include the categories of vessels required or expected to participate; radio frequencies to be used for reporting; areas of applicability; the times and geographical positions for submitting reports; the format and content of the required reports; the VTS Authority responsible for the operation of the service; any information, advice or instructions to be provided to participating vessels; and the types and level of services available.

- List of headings – guide to type of information

| VTS Authority | The name of the VTS Authority and contact details. |
| --- | --- |
| Title of VTS | The name of the VTS or VTS area. |
| Description | Brief over view of the services and whether participation is voluntary or mandatory and to which vessels it applies. |
| VTS area | Define the area boundaries. |
| VTS sectors | Where an area is sub-divided into separate operational sectors provide details of boundaries. Define location(s) and details of VTS centre(s) |
| VTS centre | (Location, Telephone, Fax and e-mail identities). |
| Communication | VHF channels to be used. Other communicating channels. Mother tongue and languages that may be used. Circumstances when SMCP is required. |
| Report | Details of reports required and when these should be made. |
| Reporting points | Identify geographical locations at which reports are to be made. |
| Call signs | Call signs to be used where this differs from the VTS name. Sector or local area call signs. |
| Hours of operation | Where services are not provided on a continuous basis specify hours of operation. |
| Traffic Image Generation | Key equipment used to generate the traffic image, coverage and, if appropriate, availability when less than‘continuous’. |
| Types of Service | Types of service provided (INS, NAS or TOS). |
| VTS Procedures | Procedures for vessels entering, transiting and departing VTS areas and/or sectors. |
| Accident Procedures | Procedures for vessels involved in specific accidents or emergencies. |
| VHF Frequencies | Details of working frequencies and authority/allied service using them. |
| Information Broadcasts | Details of the schedule, content and purpose of local broadcasts to vessels. |
| Allied Services | Availability of allied services and/or availability of VTS information to allied services. |
| Amplifying Notes | Local regulations and practice, and other relevant items not included above. |
| Diagrams | Including diagrams to indicate key features of the VTS area(s), anchorages and berths, With particular reference to dangers and areas to avoid. |

## INTRODUCTION OF THE VTS USERS GUIDE

The VTS Users Guide(hereafter referred to the Guide) should be written by every VTS centre to promulgate its VTS information. It is issued by the VTS authority in charge of setting up the VTS system which is convenient for vessels to join and to use the VTS system. Navigational officers can obtain information about a VTS centre via the guide.

## APPROACHES OF ACQUIRING THE VTS USERS GUIDE

When a vessel enters or passes through a VTS area, the Navigational officer should obtain the Guide published by the VTS centre in advance to acquire the details and procedures. The approach of achieving the Guide are:

- Referring to the Admiralty list of radio signals, ALRS Velune7，NP287（1&2）.

- Referring to the other navigational publication, e.g. The Guide to Entry or the Pilot or Sailing Direction.

- Additionally, it is recommended that other, more immediate approaches should be considered: such as websites, Master's Guide, e-mail or other text transfer media.

# HOW TO RAISE THE VTS AWARENESS OF NAVIGATIONAL OFFICER

## ADDING VTS RELATED CONTENT SUBJECTS TO THE BRIDGE RESOURCE MANAGEMENT (BRM) TRAINING

As an important external resource in the VTS area, VTS should be effectively utilized by the offices’ team. In the meanwhile, effective communication and cooperation between the bridge team and VTS would be more conducive to navigational safety and efficiency. Therefore, VTS should be included in the BRM training for navigational officers, and be reflected in the examination and assessment.

### VTS KNOWLEDGE IN THEORY TRAINING

VTS training in the BRM theory training course includes the following contents:

- Introduction of VTS, (i.e. types, functions, responsibilities, procedures and requirements of VTS);

- General requirements of transiting through the VTS area

- Benefits of VTS to BRM;

- Communicating methods and requirements between the bridge and VTS.

### ADDING VTS CONTENTS IN PRACTICAL TRAINING

Once a vessel enters a certain VTS area, it would be affected by VTS to some extent. The introduction of VTS in practical training is helpful for Navigational officers to be familiar with the operation of vessels in the VTS area.

- Examples of practical training projects in BRM

| Training project | specific items | Evaluation elements about VTS | Evaluation requirements |
| --- | --- | --- | --- |
| Sailing plan | 1)Develop a sailing plan to pass through the VTS area | (1) Identify the corresponding VTS tasks; (2) Obtain the necessary VTS information; (3) Utilization and arrangement of VTS resources; (4) Organization and arrangement of correspondence personnel with VTS Centre. | VTS influencing factors should be fully considered and reflected in the formulation of the sailing plan. |
|  | 2)Foreseeing accidental events and Countermeasures | (1) Reasonably foresee possible emergency events in the VTS waters and draw up corresponding measures (2) Identify possible contingencies (3) Procedures of responding to occasional incidents, including reporting and receiving information from the VTS Centre; (4) Response to incidents including a request for help from the VTS Centre; (5) Organization and arrangements for incidents, including coordination with the emergency response of the VTS Centre. |  |
| Actual operation through a certain VTS area | Resources and management | (1) Identify available VTS resources; (2) Coordination and use of VTS Resources. | In the VTS area, vessels should report to VTS as required, receive service, seek help from VTS centre, and cooperate with VTS centre to deal with the occasional incident. |
|  | Bridge team work | (1) Keep smooth communication with VTS Centre; (2) Maintain a high level of VTS awareness. |  |
|  | Contingency and emergency | (1) Emergency responding procedures include reporting to the VTS Centre; (2) VTS Centre participates in the decision-making of contingencies.; (3) VTS Centre participates in the process of handling accidental events; (4) Processing results are reported to the VTS Centre. |  |

## ADDING THE VTS CONTENTS IN NAVIGATIONAL EQUIPMENT TRAINING

### PRACTICAL TRAINING OF VHF

VHF is the main tool for communication between bridge and VTS centre. The simulating training of communication with VTS centre during VHF practical training is helpful to the communication and coordination between vessels and VTS centre in VTS area.

- Examples of a communication training project using VHF with the VTS Centre

| Training project | Evaluation elements |
| --- | --- |
| Communication language | 1) Common language used with the VTS centre; 2) English. |
| Communication methods and skills | 1) Active listening skills, including inquiries, feedback and rewording. 2) Transpositional consideration |
| Communication requirements | 1) Clear, concise, accurate, timely and meaningful communication; 2) Reword the information received 3) Appropriate use of message markers 4) Note that any VTS information is results-oriented. |
| Reporting on vessel intention | 1) Report vessel dynamic information; 2) Report vessel voyage information; 3) Answering queries from the VTS Centre. |
| Applying for INS | 1) Clarify the types and requirements of the information required; 2) Call the VTS Centre and apply for Information Services; 3) Receive relevant information; 4) End communication. |
| Applying for NAS | 1) Clarify circumstances of the required NAS; 2) Call VTS Centre and apply for NAS; 3) Confirmation of commencement of NAS; 4) Maintain continuous communication with the VTS centre; 5) Confirm termination of NAS. |

### AIS PRACTICAL TRAINING

Marine AIS is an important means of monitoring vessels in VTS Centre. VTS Centres obtain vessel information via AIS.

- Examples of VTS factors that can be considered in AIS practical training

| Training project | Evaluation elements |
| --- | --- |
| Reading vessel static information | Report static information to VTS Centre, such as vessel particulars |
| Reading vessel dynamic information | Report the course, speed, position and other dynamic information to VTS Centre |
| Reading security information | Receive and read security information that may come from a VTS centre |
| Setting voyage information | 1) Set vessel voyage information; 2) Report vessel voyage information to VTS Centre. |

### ECDIS PRACTICAL TRAINING

The use of ECDIS makes it easy to identify VTS coverage, and location of the VTS reporting line / point.

- Examples of VTS factors that can be considered in ECDIS practical training

| Training project | Evaluation elements |
| --- | --- |
| Route design through the VTS area | 1)Clarify coverage of VTS and location of VTS reporting lines / reporting points; 2) Passage planning should meet the requirements of the VTS area; 3) Proper use of VTS as a navigation aid. |
| Navigational Monitoring in VTS area | 1) Annotate VTS area and VTS reporting line; 2) Accept the monitoring of the VTS Center; 3) Navigational decision based on the Information provided by VTS Centre. |

## CONSIDERATION OF VTS FACTORS IN PRACTICAL TRAINING of  VOYAGE PLANNING AND PREPARATION

In the voyage plan and voyage preparation training, the ship may be set to sail through a VTS area, and the VTS factor should be fully considered during the training process.

- Examples of VTS factors that can be considered in voyage planning and preparation practical training

| Training project | Evaluation elements |
| --- | --- |
| Obtain VTS Information via navigational publications | 1) Look up VTS related navigational publication appropriately. 2) Obtain full details of requirements to be met and procedures to be followed in the VTS area. |
| Estimate the time entering and leaving VTS area | 1) The time of entering into VTS area is in compliance with the relevant regulations; 2) Report and requirements of vessels entering or leaving VTS area; |
| Passage planning in VTS area | 1) Routes should meet navigational requirements in VTS area. 2) Sailing should take into account the services provided by the VTS Centre and its impact |
| Develop the VTS sailing plan | Develop and determine VTS sailing plan with VTS authorities of the VTS area intended to pass through. |
| Considerations for ships in the VTS area | 1）Estimated arrival / departure from VTS area，noting the format, contents, time, and manner of the estimated arrival / departure report; 2) Entering VTS area，Noting the establishment of contacts with the VTS Centre, initial information exchange requirements, communications and reporting requirements, and dynamic planning arrangements; 3）When vessels passing through VTS area，it is necessary to note the mandatory requirements for ships in the VTS area, types of services provided by the VTS, communication and reporting requirements; 4) Anchoring in VTS area, noting the designated anchorage, communications and reporting requirements; 5) Berthing / unberthing in VTS area, noting the arrival berth report, maintenance of communication watch, restrictions on other passing-by vessel , and the report before unberthing ; 6) Departure from VTS area, taking note of the pre-departure report; 7) Traffic control in VTS area, paying attention to requirements of traffic control under adverse weather conditions in VTS area, Such as poor visibility, strong currents or tidal currents, strong winds and underwater activities, etc., which may affect navigation safety, and noting restrictions on vessel movements, prohibitions or additional requirements of reporting. |

## ADDING VTS CONTENTS TO NAVIGATION SIMULATOR TRAINING

Marine simulator training is a comprehensive practical training for Navigational officers. VTS training in it can simulate navigating, mooring, berthing, unberthing and responding to emergencies in the VTS area.

- Examples of VTS training in Marine Simulator training

| Training project | specific items | Evaluation elements |
| --- | --- | --- |
| Sailing | 1) Vessel estimated arrival / departure from VTS area | Submission of estimated arrival / departure reports in accordance with required format, content, time, manner, etc. |
|  | 2) Entering VTS area | (1) Establish contact with the VTS centre; (2) Exchange Initial information; (3) Communications and reporting; (4) Comply with dynamic planning arrangements. |
|  | 3) Passing through VTS area | (1) Receive vessel traffic services from the VTS Centre; (2) Communications and reporting. |
|  | 4) Departure from the VTS area | (1) reporting before departure; (2) Develop VTS sailing plan. |
|  | 5) Traffic control in VTS area | (1) Comply with traffic organization requirements; (2) Restrictions on vessel dynamic movements; (3) Communications and reporting. |
| Anchoring | Anchoring in VTS area | (1) designated Anchorage by VTS Centre; (2) Communications and reporting. |
| Berthing/ unberthing | berthing / unberthing in VTS area | (1) Arrival berth report; (2) Stand-by on VTS working VHF channel; (3) Report before unberthing. |
| Responding to emergencies | 1)SAR MOB | (1) Timely reporting to VTS Centre, including the basic information of the vessel, the time and position of the person falling into the water, the current environment, the hydrometeorological conditions, the rescue measures and the assistance needed, etc. (2) Issue a navigational warning; (3) Take full consideration of instructions from VTS Centre; (4) Take SAR actions with the VTS centre (5) Maintain communications and reporting. |
|  | 2) Encountering poor visibility | (1) Report visibility around the vessel to VTS Centre timely; (2) Apply for INS as appropriate (3) Apply for NAS as appropriate (4) Comply with the traffic organization (5) Maintain communications and reporting. |
|  | 3) Steering gear out of order | (1) Timely reporting to VTS Centre, including the time steering gear out of order, vessel position, reasons and assistance in need. (2) Apply for INS as appropriate (3) Apply for NAS as appropriate (4) Comply with the traffic organization (5) Take necessary measures to cooperate with VTS Centre (6) Maintain communications and reporting. |
|  | 4) Main engine out of order | (1) Timely reporting to VTS Centre, including Main engine out of order time, vessel position, cause and needed assistance. (2) Apply for INS as appropriate (3) Apply for NAS as appropriate (4) Comply with the traffic organization (5) Take necessary measures to cooperate with VTS Centre (6) Maintain of communications and reporting. |
|  | 5) Multi-ship encountering | (1) Establish communication with VTS centre (2) Apply for INS as appropriate (3) Apply for NAS as appropriate (4) Comply with the traffic organization (5) Maintenance of communications and reporting. |
|  | 6)collision | (1) Timely reporting to VTS Centre, including the time of the collision, the position of the vessel, the casualties, the damage to the vessel, the condition of the cargo, the name of colliding vessel, and the assistance required. (2) Apply for INS as appropriate (3) Apply for NAS as appropriate (4) Cooperate with VTS Centre to carry out rescue operation (5) Maintain communications and reporting. |
|  | 7) Grounding | (1) Timely reporting to VTS Centre, including the time of grounding, vessel position, casualties, damage to the vessel, cargo situation, draft in fore, aft and middle ship , the part which the vessel ran aground, the geology of the bottom of grounding, and the assistant required. (2) Apply for INS as appropriate (3) Cooperate with VTS Centre to carry out rescue operation (4) Maintain communications and reporting. |

## COMMUNICATION ACTIVITY BETWEEN NAVIGATIONAL OFFICERS AND VTS CENTRE

Strengthening the communication between Navigational officers and VTS centres is helpful to understand the VTS centre and VTS business knowledge, thereby to enhance their VTS awareness.

Benefits of communication activities：

- VTS Centre provides information directly;

- Navigational officers can give feedback directly to VTS personnel;

- Deepen the understanding of Navigational officers of VTS duties and responsibilities.

The main means of communication include visiting to the VTS Centre and its facilities. In addition, visiting the website of the VTS authorities, and pay attention to the public subscription of the VTS centre or the VTS authorities, etc.

## STRENGTHENing PILOT'S FAMILIARITY WITH VTS

Pilot is one of the users of VTS, its advantage lies in the familiarity with port and channel. At the same time, the pilot is also a voyager in the process of guiding the ship. Therefore, proper VTS training for pilots, strengthening their familiarity with VTS and cooperation with VTS will be beneficial to enhance the safety of navigation.

# ACRONYMS & DEFINITIONS

## ACRONYMS

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities – AISM

IMO	International Maritime Organization

VTS	Vessel Traffic Service

VTSO	Vessel Traffic Service Operator

INS 	Information Service

TOS	Traffic Organization Service

NAS	Navigational Assistance Service

N-O	Navigating Officers

ISPS	International Vessel And Port Facility Security (Code)

PSC	Port State Control

BRM	Bridge Resource Management

VHF	Very High Frequency

AIS	Automatic Identification System

ECDIS	Electronic Chart Display And Information System

SMCP	Standard Marine Communication Phrases

CPA	Closest Point Of Approach

DCPA	Distance To Point Of Approach

SAR	Search And Rescue

MOB	Man OverBoard

## DEFINITIONS

The definition of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at http://www.iala-aism.org/wiki/dictionary.Acronyms.

# REFERENCES

- IALA VTS MANUAL 2016

- IMO RESOLUTION A.857(20) GUIDELINES FOR VESSEL TRAFFIC SERVICES

- ITU RADIO REGULATIONS, VOLUME IVE, RECOMMENDATION ITU.R M.1171.0 AND SUBSEQUENT CHAPTERS

- IMO RESOLUTION A.918(22) IMO STANDARD MARINE COMMUNICATION PHRASES (SMCP)

- SOLAS 1974- INTERNATIONAL CONVENTION FOR THE SAFETY OF LIFE AT SEA CHAPTER V, REGULATION 12 VESSEL TRAFFIC SERVICES.

- STCW 1978 - (11) - 2010 MANILA AMENDMENTS TO THE STCW  CONVENTION AND CODE

- IMO RESOLUTION A.857(20) GUIDELINES FOR VESSEL TRAFFIC SERVICES;

- IALA GUIDELINE 1089 PROVISION OF VTS SERVICES (INS,TOS&NAS);

- IALA GUIDELINE 1071 ESTABLISHMENT OF A VESSEL TRAFFIC SERVICE BEYOND TERRITORIALSEAS;

- IALA GUIDELINE 1102 VTS INTERACTION WITH ALLIED OR OTHER SERVICES;

- IALA RECOMMENDATION V-120 VESSEL TRAFFIC SERVICES IN INLAND WATERS (EDITION2);

- IALA RECOMMENDATION V-127 VESSEL TRAFFIC SERVICES IN INLAND WATERS (EDITION2);

- ADMIRALTY LISTOF RADIO SIGNALS,ALRS

- INTERNATIONAL CONVENTION ON MARITIME SEARCH AND RESCUE,1979

- 1132-ED.1-VTS-VHF-VOICE-COMMUNICATION

- STW 43-9 - VTS TRAINING FOR NAVIGATING OFFICERS (IALA)

- 1131-ED.1-SETTING-AND-MEASURING-VTS-OBJECTIVES

- OPERATING GUIDE of VESSELs IN VTS AREA

- REPORTING AND ACCEPTING ENQUIRY AS REQUIRED

  - REPORTING TO VTS CENTRE

  - Examples of reporting to VTS centre

| Type of Report | Content of Report |
| --- | --- |
| Arrival/Departure Report | Sailing Plan, ETA/ETD Vessel’s Length/Draft, Cargo’s name, etc. |
| Passing Reporting Line Report | Vessel’s particular, such as LOA, Breadth, GT, etc. Voyage data, such as Draft, Last/Next Port, Cargo’s Name, etc. |
| Berthing/Unberthing Report | Sailing Plan, ETA/ETD, Vessel’s Length/Draft, Last/Next port of call, etc. |
| Dropping/Heaving up Anchor Report | Time of dropping/heaving up anchor, anchor position, length of shackles released, distance to other vessel, meteorological and hydrological conditions, etc. |
| Other report | The reporting content depends on the specific circumstances, such as occurring emergency situations, finding aids to navigation abnormal, etc. |

  - ACCEPTING ENQUIRY FROM VTS CENTRE

When the vessel navigating、anchoring or operating in the VTS area, the Navigational officers should keep a continuous watch on the designated radio frequencies channel, accept enquiry from VTS centre at any time, and report concerning information to VTS centre.

- ACCEPTING/REQUESTING INFORMATION SERVICE

  - ACCEPTING INFORMATION SERVICE

The vessel can receive the broadcasting information at fixed time and intervals from VTS centre, accept the point-to-point information service such as intention of vessels, meteorological and hydrological conditions, navigational warning or any safety information for navigating by information、warnings、advice or instructions.

  - REQUESTING INFORMATION SERVICE

Examples of Navigational officers requesting information services from VTS centre:

- The Navigational officers should confirm the content and types of information they need;

- Connecting with VTS centre and requesting information services;

- Accepting the information from VTS centre;

- Applying for the information to assist the on board decision-making process.

- ACCEPTING/REQUESTING NAVIGATIONAL ASSISTANCE SERVICE

A Navigational Assistance Service is a service that provides essential and timely navigational information to assist in the on board navigational decision-making process and to monitor its effects at the request of the vessel or when deemed necessary by the VTS. Navigational Assistance Service must be agreed by the vessel and VTS centre. When deemed necessary by the VTS, it can provide the navigational assistance service initiatively, the vessel should confirm with VTS centre whether accepting the navigational assistance service according to prevailing circumstances and conditions. The vessel can also request the navigational assistance service if it necessary. VTS centre can provide the navigational assistance service after getting agreed with the vessel.

- Examples of Navigational officer operating in navigational assistance service

| Operating Items | Content and Procedure |
| --- | --- |
| Request and identification | Availability of NAS, assure the time of starting and ending NAS. Report to VTS centre for vessel identification such as position, course made good and speed over ground Report to the VTS centre the status of vessel’s equipment Ensure continuous watch on the designated radio frequencies channel |
| Receiving navigational information (including position and course information) | Keeping a continuous communicating with VTS centre, navigating based on the information provided by VTS centre , such as the range and bearing from fixed objects, fairway/channel or way-points、the information concerning danger and a channel/fairway/ lane. |
| Receiving advice (or instructions) | Follow the advice or instructions from VTS centre to alter the course、speed. Accepting the advice、instructions、warning information issued by VTS centre and applying for the information to assist in the on board navigational decision-making process . Following the advice or instructions to keep clear from hazards、close up /drop back on/from vessels . |
| Continuance and end the operation | Feeding back efficiently and timely and reporting the problems occurred in the whole process to VTS centre. Confirming the time of end with VTS centre |

- ACCEPTING TRAFFIC ORGANIZATION SERVICE

- Examples of Navigational officer operating in traffic organization service

| Type of traffic organization service | Content and Procedure |
| --- | --- |
| Traffic Clearance | Request entering or departing VTS area; Request departing from a berth or an anchorage position within VTS area; Request entering into a fairway within VTS area; Report VTS sailing plan, sluice gate and bridge passing plan to VTS centre before entering VTS area; Report position at determined reporting point/line/pilot station ; Using a backup fairway in case of bad visibility/weather. |
| Anchorage | Report the movements arriving/leaving an anchorage position/area; Proceed to the anchor position assigned by VTS centre; Request assisting vessels in entering anchor position from VTS centre. |
| Waterway (sea, channels and fairway) management and enforcement | Comply with the speed limits; Comply with the rules regarding traffic routeing measures; Comply with the pilotage requirements; Comply with other traffic regulations and relevant local rules; Keep clear of the vessel safety zones which established and organized for keeping clear of particular operation; Keep clear of the special area/position; Do not overtake in the area where overtaking is prohibited. |

- COOPERATION WITH VTS CENTRE IN ALLIED SERVICE

  - COOPERATION WITH SEARCH AND RESCUe ACTIONs

  - Examples of Navigational officer operating in SAR

| Type of operating | Contents and procedure |
| --- | --- |
| Search and rescue the crew of your own vessel when encountering accidents. | (1) Report MOB to VTS centre, including Time, Vessel’s position, Wind speed and direction, etc. Broadcasting MOB information on VHF in order to get assistance from other vessel around the waters. (2) Keep in touch with VTS centre and report the latest search and rescue information to VTS centre in time. |
| Search and rescue the crew of other vessel when the vessel encounters accidents. | (1) The master should be informed timely when receiving the MOB information. The master shall actively cooperate with VTS centre to participate in the search and rescue operations without serious danger to the vessel, crewmember, and passengers. (2) Report the scene situation to the VTS centre including own vessel’s name, call sign and position. Following the arrangements and instructions issued by on-scene coordinator, and following the mission assigned by VTS centre. (3) Keep in touch with VTS centre and on-scene coordinator. Report the latest search and rescue situation to VTS centre in time. (4) When VTS centre or on-scene coordinator issues an order of terminating the search and rescue operation or the vessel is no longer required to take part in the actions, the vessel can carry out the original sailing plan. |

  - COOPERATION IN OTHER EMERGENCY SITUATIONS

  - Examples of Navigational officer operating in other emergency situations

| Type of emergency | Content and procedure |
| --- | --- |
| Collision | (1)The Navigational officer should report the time, position, casualties, cargos on board, vessel damage condition, risk of sinking and assistance needed to VTS centre. (2) If there is a risk of sinking, it should prepare to abandon the vessel based on muster list. If there is no risk of sinking, the damage of the vessel, the cargo holds, ballast tank and oil tank should be checked. And the information should be reported to the VTS centre timely. (3) Broadcasting the navigational warning, requesting the vessels around stay away or seeking assistance. (4) Keeping in touch with VTS centre and cooperating with VTS centre to carry out rescue actions. |
| grounding | (1) The Navigational officer should report the time, position, casualties, part of aground, damage of vessel, cargo on board, any oil spill pollution and assistance needed to the VTS centre. (2) Sounding the vessel’s six-sided draft and the depth of surrounding area. Checking whether flooding, damage of vessel, part of flooding, amount of flooding water and reporting to VTS centre timely. (3) Keeping in touch with VTS centre and cooperating with VTS centre to carry out rescue action. |
| Fire | (1)The Navigational officer should report the time, position, casualties, cargo on board, risk of sinking and assistance needed to the VTS centre. (2) Taking action as per muster list and reporting the on-scene fighting situation to VTS centre. (3) Keeping in touch with VTS centre and cooperating with VTS centre to carry out rescue action. |
| Pollution | (1) The Navigational officer should report the time, position, casualties, cargo on board, risk of sinking and assistance needed to the VTS centre. (2) Taking action as per muster list and reporting the on-scene situation to VTS centre. (3) Keeping in touch with VTS centre continuously and cooperating with VTS centre to carry out rescue actions. (4) Proceed to the designated area according to the VTS centre. |

  - COOPERATION IN VESSEL SECURITY INCIDENT

When the vessel security incidents occur in the VTS area, the Navigational officer should take action as the following procedure:

- Reporting the number of casualties and basic situation;

- Reporting the number of pirates or robbers, the type, number, size, speed of vessel they using and what weapons, explosives they using.

- Reporting the measures taken by the vessel and assistance needed.

- Reporting the damage of the machine, the condition of equipment and the ability to maneuver.

- Taking appropriate actions under the guidance of the VTS centre.

- Keeping in touch with VTS centre.

- EXAMPLE OF COMMUNICATION BETWEEN N-O AND VTS

- Apply for weather information

| 1. Establish contact | 1.1(name) VTS |  |
| --- | --- | --- |
|  | 1.2 THIS IS …. (Name of vessel/call sign) |  |
| 2. Exchange information | 2.1 WEATHER + INFORMATION | Pro‐word + Message Marker |
|  | 2.2 Weather Condition Statement |  |
|  | 2.3 (Message Marker) Request | Additional measures if relevant |
| 3. End of message/conversation | OVER |  |

- Apply for traffic information

| 1. Establish contact | 1.1(name) VTS |  |
| --- | --- | --- |
|  | 1.2 THIS IS …. (Name of vessel/call sign) |  |
| 2. Exchange information | 2.1 TRAFFIC + INFORMATION | Pro‐word + Message Marker |
|  | 2.2 Name of vessel/s we are referring | Who |
|  | 2.3 Position of vessel/s using cardinal point/ bearing and distance | Where |
|  | 2.4 Destination of vessel/s | Intentions |
|  | 2.5 QUESTION | Additional measures if relevant |
|  | 2.6 Content of marker |  |
| 3. End of message/ conversation | OVER |  |

- BASIC WORDS

The proper use of the basic words improves unambiguous communication from shore to vessel and vice versa.

- Table 20 Basic words and it’s meaning

| Basic Word | Meaning |
| --- | --- |
| Approved | Permission for proposed action granted |
| Check | Examine (something) in order to determine its accuracy, quality, or condition, or to detect the presence of something |
| Clearance | Authorised to proceed under the conditions specified |
| Confirm | I request verification of: (clearance, instruction, action, information) |
| Contact | Establish communications with… (your details have been passed) |
| Correction | A change that rectifies an error or inaccuracy |
| Disregard | Ignore last message |
| Incorrect | That is not correct |
| I repeat | I will state my message again |
| I spell | Phonetic spelling follows |
| Maintain | Continue in accordance with the condition(s) specified or in its literal sense, e.g.‘’Maintain your course’’ |
| Out | End of transmission. No answer is required or expected |
| Over | End of transmission. An answer is expected |
| Read back | Read back to me the message as you received it |
| Report | Pass requested information |
| Say again | A request to retransmit all or a portion of a transmission |
| Standby | Wait and I will call you |

- SPELLING OF LETTERS AND NUMBERS

Phonetic alphabets are used to distinguish between letters, which sound similar when transmitted over the radio. They are commonly used when transmitting call signs and in cases where a single letter is used to designate something.

For example:

Contact MS Liwai, I SPELL Lima India Whisky Alpha India, Liwai, on your starboard side.

- Spelling of letters

| Letter | Code | Letter | Code | Letter | Code |
| --- | --- | --- | --- | --- | --- |
| A | Alfa | J | Juliet | S | Sierra |
| B | Bravo | K | Kilo | T | Tango |
| C | Charlie | L | Lima | U | Uniform |
| D | Delta | M | Mike | V | Victor |
| E | Echo | N | November | W | Whisky |
| F | Foxtrot | O | Oscar | X | X-ray |
| G | Golf | P | Papa | Y | Yankee |
| H | Hotel | Q | Quebec | Z | Zulu |
| I | India | R | Romeo |  |  |

A few digits and numbers have a modified pronunciation compared to general English:

- Number Spelling Pronunciation

| Number | Spelling | Pronunciation | Number | Spelling | Pronunciation |
| --- | --- | --- | --- | --- | --- |
| 0 | Zero | ZEERO | 6 | Six | SIX |
| 1 | One | WUN | 7 | Seven | SEVEN |
| 2 | Two | TOO | 8 | Eight | AIT |
| 3 | Three | TREE | 9 | Nine | NINER |
| 4 | Four | FOWER | 1000 | thousand | TOUSAND |
| 5 | five | FIFE |  |  |  |

- Table 23 Example of Number Pronunciation

| Number | Pronunciation |
| --- | --- |
| 963 | Niner six tree |
| 2,600 | Two six zero zero |
| 96,000 | Niner six zero zero zero |